| Question(s): | 5 |  | Meeting, date: |  | Geneva, 28th October – 8th November 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 1 | Intended type of document (R-C-TD): |  | C |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |  |  |
| Title: | F/H.TPS-Arch: Inclusion of a captioning requirement |  |  |  |  |  |
| Contact: | Weiwei Yang Huawei Technologies China |  |  |  | Tel: +86 29 87674367 Fax: +86 29 87674151 Email: tommy@huawei.com |  |
| Contact: | Jing Xiao Huawei Technologies China |  |  |  | Tel: +86 755 36830146 Fax: +86 755 36830131 Email: lennard.xiao@huawei.com |  |
| Contact: | Christian Groves NTEC Australia Australia |  |  |  | Tel: +61 3 9391 3457 Fax: +61 3 9391 3457 Email: Christian.Groves@nteczone.com |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

- Introduction

This Contribution proposes updates to F/H.TPS-Arch in order to incorporate requirement Col002 on captioning.

- Discussion

F.TPS-Req lists many requirements that are applicable to telepresence systems. Appendix I to F.TPS-Req contains a requirements mapping showing how each of the requirements are covered by the Q.5/16 work items. The coverage of a number of requirements is listed as “?” indicating that they are not covered or not known to be covered.

Once such requirement is Col002:

Col002: A telepresence system can optionally support a captioning function.

Clause 7.1.1.5/[ITU-T H.TPS-AV] contains the “Embedded text” attribute that allows an endpoint to indicate that a capture contains embedded text, i.e. captioning information. So Col002 is supported by a Q.5/16 work item. When looking at [ITU-T F/H.TPS-Arch] there is no description of “text based” input being applicable to a telepresence system. Text communication and supplemental text information (such as captioning) is important functionality for accessibility reasons. CLUE also supports text captures as well as embedded text information in captures. Therefore the Contributors propose to incorporate text into draft ITU-T F/H.TPS-Arch to incorporate text functions into the telepresence system architecture.

- Proposal

It is proposed to accept the following updates.

Changes are marked up against TD-22a from the June 2013 Q.3/16 Rapporteurs’ meeting in Olso.

Draft new Recommendation F/H.TPS-Arch

Telepresence System Architecture

…

8.1.1.7	Telepresence media input/output control unit

The telepresence media input/output control unit is a logical unit that can select and control multiple input/output media streams that include main and auxiliary video, audio and text streams. Video and audio capture units, video and audio render units, PCs, text terminals and other audio endpoints are connected to the telepresence media input/output control unit.

8.1.1.8	Telepresence media processor

The telepresence media processor is used for processing video and audio steams to provide the required audio and visual experience. The main functions of telepresence media processor include:

- Geometric correction for multiple camera images to get better image mosaic effect.

- Colour correction for making luminance and chrominance of multiple camera images uniform.

- Geometric correction for multiple video stream rendering, that make the video outputs adapt to various types of screen and get better display performance, for example: geometric correction for arc projection screen.

- Colour correction for multiple video stream rendering for a better rendering performance.

- Pre-processing for audio signals captured by audio capture unit, such as audio de-noising and enhancement for multiple microphones, etc.

- Pre-processing for audio signals rendered by audio render unit, for example, improving audio quality of loudspeakers.

- Incorporation of text information (e.g. captioning) into video streams.

…

Changes are marked up against TD-26a from the June 2013 Q.3/16 Rapporteurs’ meeting in Olso.

Draft new Recommendation F.TPS-Reqs

Definitions, requirements, and use cases for Telepresence Systems

…

I.5	Collaboration functions

| Requirement | Covered | Details |
| --- | --- | --- |
| Col001 | Part | F/H.TPS-Arch discusses presentation streams but does not go into detail regarding the different collaboration means. |
| Col002 | Yes | [ITU-T F/H.TPS-Arch] clauses 8.1.1.7 and 8.1.1.8 discuss the support of text streams and captioning. [ITU-T H.TPS-AV] clause 7.1.1.5 contains an attribute to indicate that a capture contains embedded text. |

__________________