| Question(s): | 5 |  | Meeting, date: |  | Geneva, 28th October – 8th November 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 1 | Intended type of document (R-C-TD): |  | C |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |  |  |
| Title: | F/H.TPS-Arch: Inclusion of statistics requirement |  |  |  |  |  |
| Contact: | Weiwei Yang Huawei Technologies China |  |  |  | Tel: +86 29 87674367 Fax: +86 29 87674151 Email: tommy@huawei.com |  |
| Contact: | Jing Xiao Huawei Technologies China |  |  |  | Tel: +86 755 36830146 Fax: +86 755 36830131 Email: lennard.xiao@huawei.com |  |
| Contact: | Christian Groves NTEC Australia Australia |  |  |  | Tel: +61 3 9391 3457 Fax: +61 3 9391 3457 Email: Christian.Groves@nteczone.com |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

- Introduction

This Contribution proposes updates to F/H.TPS-Arch in order to incorporate requirement Other01 on statistics.

- Discussion

F.TPS-Req lists many requirements that are applicable to telepresence systems. Appendix I to F.TPS-Req contains a requirements mapping showing how each of the requirements are covered by the Q.5/16 work items. The coverage of a number of requirements is listed as “?” indicating that they are not covered or not known to be covered.

Once such requirement is Other1:

Other01: It is required that a  telepresence system be able to provide various statistical information which includes but is not limited to media type, maximum number of endpoints, duration, and data volume of each media type.

As mentioned in an accompanying contribution it is believed that such a statistics collection and reporting function is suitable for the management system to perform. As such the Contributors propose to add this function to the list of functions performed by the management system as listed in clause 6.1.5/[ITU-T F/H.TPS-Arch].

- Proposal

It is proposed to accept the following updates.

Changes are marked up against TD-22a from the June 2013 Q.3/16 Rapporteurs’ meeting in Olso.

Draft new Recommendation F/H.TPS-Arch

Telepresence System Architecture

…

6.1.5	Management System

A management system mainly contains control and management functions that provide specific telepresence services, such as device management, conference scheduling and user authentication, etc. It provides a statistic collection and reporting function for information related telepresence conference sessions and system usage.

…

Changes are marked up against TD-26a from the June 2013 Q.3/16 Rapporteurs’ meeting in Olso.

Draft new Recommendation F.TPS-Reqs

Definitions, requirements, and use cases for Telepresence Systems

…

I.6	Other functions

| Requirement | Covered | Details |
| --- | --- | --- |
| Other01 | Yes | Clause 6.1.5/[ITU-T F/H.TPS-Arch] contains a description of a management function that includes a statistic collection and reporting function. |
| Other02 | ? | There is no discussion in the TPS documents regarding this. |

__________________