| Question(s): | 5 |  | Meeting, date: |  | Geneva, 28th October – 8th November 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 1 | Intended type of document (R-C-TD): |  | C |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |  |  |
| Title: | F/H.TPS-Arch: Updates |  |  |  |  |  |
| Contact: | Weiwei Yang Huawei Technologies China |  |  |  | Tel: +86 29 87674367 Fax: +86 29 87674151 Email: tommy@huawei.com |  |
| Contact: | Jing Xiao Huawei Technologies China |  |  |  | Tel: +86 755 36830146 Fax: +86 755 36830131 Email: lennard.xiao@huawei.com |  |
| Contact: | Christian Groves NTEC Australia Australia |  |  |  | Tel: +61 3 9391 3457 Fax: +61 3 9391 3457 Email: Christian.Groves@nteczone.com |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

# Introduction

This Contribution discusses the editor’s notes in clauses 3.1.2, 8.1.1.3 and 8.2 and proposes actions to address them.

# Discussion

F/H.TPS-Arch currently contains several open editor’s notes. This contribution discusses several editor’s notes and proposes updates to address them. The editor’s notes are discussed below:

2.1 Terms

Clause 2.1 on terms defined elsewhere contains the following editor’s note:

{Editor’s note: The IETF Clue working group currently have an internet draft (draft-wenger-clue-definitions-xx)[http://datatracker.ietf.org/doc/draft-wenger-clue-definitions] defining Telepresence related terms. Rather than re-defining terms here it may be advantageous to reference the IETF terms.}

Since the inclusion of this editor’s note the definitions from <draft-wenger-clue-definitions> have been incorporated into <draft-ietf-clue-telepresence-requirements>. This draft includes definitions for:

Audio mixing: It is proposed to add this to F/H.TPS-Arch as it does mention audio mixing.

Conference: It is not proposed to adopt this definition as it is SIP specific.

Endpoint: It is not proposed to adopt this definition as F/H.TPS-Arch provides text in clause 6.1.1 describes endpoints and their characteristics.

Endpoint characteristics: It is not proposed to adopt this definition as F/H.TPS-Arch provides text in clause 6.1.1 describes endpoints and their characteristics.

Layout: It is proposed to add this to F/H.TPS-Arch as clause 10.1 describes specific layouts. It may be useful to add a generic description.

Left: It is not proposed to adopt this definition as F/H.TPS-Arch does not use it in the context described by the IETF draft.

Local: It is not proposed to adopt this definition as F/H.TPS-Arch does not use it in the context described by the IETF draft.

MCU: It is not proposed to adopt this definition as F/H.TPS-Arch provides text in clause 8.2 describing an MCU and its characteristics.

Media: It is not proposed to adopt this definition as the IETF draft limits the definition of media to that which can be sent via RTP.

Model: It is not proposed to adopt this definition as F/H.TPS-Arch does not use it in the context described by the IETF draft.

Remote: It is proposed to adopt this definition as F/H.TPS-Arch does this term.

Render: It is proposed to adopt this definition as F/H.TPS-Arch does this term.

Right: It is not proposed to adopt this definition as F/H.TPS-Arch does not use it in the context described by the IETF draft.

Telepresence: It is not proposed to adopt this definition as Q.5 had already developed a definition for this term.

2.2	Audio mixing

Clause 8.1.1.3 contains the following editor’s note:

{Editor’s note: Further contribution is required to discuss in more detail how the audio signals are combined and transmitted.}

It is proposed to add an example of how this is achieved. The Contributor’s believe that this is sufficient as audio mixing is a widely established process and is not particular to telepresence.

2.3 	MCU Adaptation

Clause 8.2 contains the following editor’s note:

- {Editor’s note: Adaptation for media stream of non-mixed mode - Further contribution is required to describe this functionality.}

The Contributor’s believe that the use of a “non-mixed mode” refers to the situation where adaptation is not necessary and thus not used. It should be possible for a MCU to detect cases where adaptation is not necessary. It is proposed to add text indicating this function.

# Proposal

It is proposed to accept the following updates. Changes are marked up against TD-22a from the June 2013 Q.3/16 Rapporteurs’ meeting in Olso.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.TPS-Reqs]	Recommendation ITU-T F.yyy (date), Definitions, requirements, and use cases for Telepresence Systems.

[H.TPS-AV]	Recommendation ITU-T H.yyy (date), Audio/Video Parameters for Telepresence systems.

[ITU-T H.245]	Recommendation ITU-T H.245 (05/2011), Control protocol for multimedia communication.

[ITU-T H.320]	Recommendation ITU-T H.320 (03/2004), Narrow-band visual telephone systems and terminal equipment.

[ITU-T H.323]	Recommendation ITU-T H.323 (12/2009), Packet-based multimedia communications systems.

[ITU-T Q.931]	Recommendation ITU-T Q.931 (05/1998), ISDN user-network interface layer 3 specification for basic call control.

[IETF RFC 5117]	IETF RFC 5117 (2008), RTP Topologies

[IETF CLUE REQ]	IETF RFC xxxx (xxxx), Requirements for Telepresence Multi-Streams. (Work in progress: draft-ietf-clue-telepresence-requirements-05)

…

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	Audio mixing [IETF CLUE-REQ]: refers to the accumulation of scaled audio signals to produce a single audio stream.  See RTP Topologies, [IETF RFC5117].

3.1.2	Layout [IETF CLUE-REQ]: How rendered media streams are spatially arranged with respect to each other on a single screen/mono audio telepresence endpoint, and how rendered media streams are arranged with respect to each other on a multiple screen/speaker telepresence endpoint.

Note: that audio as well as video is encompassed by the term layout--in other words, included is the placement of audio streams on speakers as well as video streams on video screens.

3.1.3	Remote [IETF CLUE-REQ]: Sender and/or receiver on the other side of the communication channel (depending on context); not Local.  A remote can be an Endpoint or an MCU.

3.1.4	Render [IETF CLUE-REQ]: the process of generating a representation from a media, such as displayed motion video or sound emitted from loudspeakers..

…

#### 8.1.1.3	Audio Capture Unit

An audio capture unit may comprise of multiple microphones in some spatial placement for capturing audio signals at the telepresence endpoint. This spatial placement would largely be dependent on the Telepresence endpoint configuration. For example: microphone placement may differ between single-row and multiple-row configuration. There may be no direct mapping between microphones and audio streams. Where spatial information is required the audio capture unit should support the carriage of this information.

Figure 8: Audio capture unit architecture

Microphones may connect through a media input/output control unit that can select and control multiple audio sources. Auxiliary audio streams from a PC or other devices also connect to endpoint through media input/output control unit. Audio signals from multiple microphones may be combined into one single audio stream for transmission. For example: the audio signal from each microphone may be coded as a separate channel using an applicable audio codec. This audio coding is then transported to the remote endpoint via a suitable transport such as RTP.

…

## 8.2	Multipoint Control Unit

The main functions of an MCU are basically identical to the definitions in [ITU-T H.323], as per the following descriptions.

Access functions:

- To support accesses of various types of endpoints that include: multi-screen telepresence endpoint, single-screen telepresence endpoint and audio endpoints.

- To support multi-image composition videoconferencing.

- To support single/multiple auxiliary media streams.

- To support MCU cascading connection. Multiple media channels should be supported and can be flexibly multiplexed in cascading connection mode.

- To assure QoS.

- To support automatic NAT traversal

- To support both IPv4/v6 connections.

Adaptation functions:

- Adaptation for media streams according to capabilities of endpoints, such as video transcoding, audio transcoding, etc.

- Adaptation for media streams according to endpoint characteristics, such as screen aspect ratio, etc.

- Utilising endpoint characteristics to determine cases where no adaptation of the media stream is necessary.

__________________