| Question(s): | 3 |  | Meeting, date: |  | Geneva, 28th October – 8th November 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 1 | Intended type of document (R-C-TD): |  | C |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |  |  |
| Title: | H.248.RTCPPROF: Updates |  |  |  |  |  |
| Contact: | Weiwei Yang Huawei Technologies China |  |  |  | Tel: +86 29 87674367 Fax: +86 29 87674151 Email: tommy@huawei.com |  |
| Contact: | Christian Groves NTEC Australia Australia |  |  |  | Tel: +61 3 9391 3457 Fax: +61 3 9391 3457 Email: Christian.Groves@nteczone.com |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

# Introduction

This Contribution proposes updates to H.248. RTCPPROF “Guidelines on the use of H.248 capabilities for performance monitoring in RTP networks in H.248 Profiles”.

# Discussion

H248. RTCPPROF is scheduled to be put forward for Consent at the 28th October – 8th November ITU-T SG16 meeting. The Contributors have performed a review of the draft Recommendation and have identified a number of editorial corrections that should be addressed. There are also a number of technical issues that need to be addressed. These have been noted via MS Word “comments”.

Note: Since performing the review draft-ietf-xrblock-rtcp-xr-jb has now been published as RFC 7005. This should also be reflected in the text.

# Proposed

It is proposed to accept the following updates. Changes are marked up against TD-18 from the June 2013 Q.3/16 Rapporteurs’ meeting in Olso.

Draft new Recommendation ITU-T H.248.xx (ex H.248.RTCPPROF)

Gateway control protocol: 
Guidelines on the use of H.248 capabilities for performance monitoring in RTP networks in H.248 Profiles

…

# 1	Scope

The scope of this Recommendation covers performance monitoring in RTP networks with the involvement of H.248 entities. Performance monitoring as such is a network service across multiple network entities which are located in user, control and/or management planes. The scope of this Recommendation is limited to the user/control/management plane interfaces of H.248 entities. The notion of performance monitoring is used to emphasize a call/session dependent service, which is basically in contrast to the legacy performance management capability, which is a pure management plane function [ITU-T M.3010, ITU-T M.60].

Performance monitoring could be considered as a network overlay function.

This Recommendation provides:

- an overview of the existing "entire H.248 toolkit" for RTCP extended report (XR) support, i.e., the set of available H.248 packages in that area;

- an overview of available H.248 packages for non-XR related performance metrics, typically subject of RTCP basic report (SR, RR) capabilities;

- example network use cases with different H.248 MG types (primarily physical-to-RTP gateways, media-agnostic IP-to-IP gateways, RTP-topology-aware RTP-to-RTP gateways);

- a collection of functional requirements in order to motivate associated profile guidelines;

- a classification of performance metrics (such as application-level versus transport-level metrics) in order to indicate applicability or not for H.248 MGs;

- the principal H.248 MG tasks of

  - measurement of RTP traffic,

  - reporting of measurement data via RTCP XR reports,

  - collection of RTCP XR carried measurement data,

  - loopback of RTP (in order to relocate a measurement point),

  - filtering of RTCP XR reports,

  - modification of RTCP XR reports and/or

  - reporting of measurement data via H.248 statistics to the MGC

  - control of such tasks via H.248 that could be addressed in a H.248 profile, i.e.

    - the identification of appropriate H.248 packages

    - the detailed specification of package usage

    - the example call-dependent procedures;

- the association to other related H.248.x Recommendations such as [ITU-T H.248.71], [ITU-T H.248.79], [ITU-T H.248.85] and [b-ITU-T H.248.RTPTOPO];

- control of measurement methods (such as interval, cumulative and alert type of measurements);

- usage of H.248.47-based conditional reporting [ITU-T H.248.47]; and

- other aspects as well.

This Recommendation also discusses the purpose of performance monitoring (and therefore possible motivation of a use case). For example:

- capture of Grade-of-Service (GoS) related data in order to evaluate network conditions

- capture of Quality-of-Service (QoS) and Quality-of-Experience (QoE) related data in order to evaluate service conditions (e.g. as part of service level agreements)

- capture of billing/charging related data

- capture of data as input for routing algorithms

- capture of data as input for network node selection algorithms

- capture of data as input for centralized servers as support of network operations

- capture of data as part of online tests

## 1.1	Framework and layout of this Recommendation

The scope and purposes may be summarized by the diagram in Figure 1, which illustrates the framework of this Recommendation.

Figure 1 – Scope, structuring principle and framework of this Recommendation

The primary audience of this Recommendation are authors of H.248 profile specifications, which aim to support a particular network performance monitoring use case, and the consideration of a specific location and/or type of an H.248 MG.

Performance monitoring support could affect three areas of an H.248 profile:

- basic protocol capabilities (such as connection model);

- packages and their detailed usage; and/or

- call-dependent procedures.

This Recommendation provides guidelines for these profile areas. Typically to define a H.248 profile there are several steps that are followed, according to Figure 1, given by

- requirements stage: firstly selecting use case(s) and the identification of required MG behaviour in terms of measurements, reporting, filtering, etc. capabilities;

- protocol stage: secondly, addressing the necessary templates of the indicated profile areas and recommended input; and

- profile stage: thirdly, incorporating "guidelines" in an actual profile specification and adapting the templates to the concrete specification setting.

This Recommendation is organized as follows:

- example use cases are collected in Appendix I;

- principal MG behaviour is defined in clause 8 by considering the set of requirements behind such MG roles (and the used prescriptive language for requirements is outlined in clause 5);

- an technology overview of existing H.248 packages is in scope of clause 6;

and finally

- guidelines for the three profile areas are subject of clause 9 (which follows the profile structuring of the profile template according to [ITU-T H.248.1]).

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.2]	Recommendation ITU-T H.248.2 (2005), Gateway control protocol: Facsimile, text conversation and call discrimination packages, including its Amendment 1 (01/2007).

[ITU-T H.248.8]	Recommendation ITU-T H.248.8(2005), Gateway control protocol: xxx.

[ITU-T H.248.30]	Recommendation ITU-T H.248.30 (2007), Gateway control protocol: RTCP extended performance metrics packages.

[ITU-T H.248.47]	Recommendation ITU-T H.248.47 (2008), Gateway control protocol: Statistic conditional reporting package.

[ITU-T H.248.48]	Recommendation ITU-T H.248.48 (2012), Gateway control protocol: RTCP XR Block Reporting Package.

[ITU-T H.248.58]	Recommendation ITU-T H.248.58 (2008), Gateway control protocol: Packages for application level H.248 statistics.

[ITU-T H.248.61]	Recommendation ITU-T H.248.61 (2009), Gateway control protocol: Packages for network level H.248 statistics.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (2010), Gateway control protocol: RTCP support packages.

[ITU-T H.248.76]	Recommendation ITU-T H.248.76 (2010), Gateway control protocol: Filter group package and guidelines.

[ITU-T H.248.79]	Recommendation ITU-T H.248.79 (2012), Gateway control protocol: Guidelines for Packet-Based Streams.

[ITU-T H.248.85]	Recommendation ITU-T H.248.85 (2013), Gateway control protocol: Usage of Loopback in H.248.

[ITU-T M.60]	Recommendation ITU-T M.60 (03/1993), Maintenance terminology and definitions.

[ITU-T M.3010]	Recommendation ITU-T M.3010 (02/2000), Principles for a telecommunications management network.

[ITU-T T.38]	Recommendation ITU-T T.38 (2010), Procedures for real-time Group 3 facsimile communication over IP networks.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 3611]	IETF RFC 3611 (2003), RTP Control Protocol Extended Reports (RTCP XR).

[IETF RFC 5093]	IETF RFC 5093 (xxx), xxx.

[IETF RFC 6776]	IETF RFC 6776 (10/2012), Measurement Identity and Information Reporting Using a Source Description (SDES) Item and an RTCP Extended Report (XR) Block.

[IETF RFC 6792]	IETF RFC 6792 (11/2012), Guidelines for Use of the RTP Monitoring Framework.

…

## 3.2	Terms defined in this Recommendation

3.2.1	application level metrics: a protocol stack based classification of a performance metric. Such a performance metric uses protocol information elements from the application layer and possibly from lower layer protocols (NOTE 1). Some application level metrics are furthermore coupled to the termination of the application protocol (NOTE 2) and/or the location of user equipment (NOTE 3).

NOTE 1 – Example: an H.248 IP-IP MG would typically be configured for a "media-aware" mode in order to have sufficient information about the carried application traffic with regards to the application level performance metric.

NOTE 2 – The termination of the application protocol correlates typically with RTP topology "RTP end system".

NOTE 3 – Example: the category of "QoE" based performance metrics as application level metrics.

3.2.2	collection point (CP) (adapted from clause 7.6.2/[ITU-T H.248.71]): A CP is the physical or logical point which records/collects remote measurement data, as received via a user plane reporting interface (here via RTCP report packets).

3.2.3	filtering point (FP) (adapted from filter definition of [ITU-T H.248.76]):

The location of policy rule enforcement point for filtering information. Such specific policy rules are often simply termed “filter rules”. In the context of this Recommendation,

- the filter rule conditions are primarily related to protocol control information elements related to RTP and RTCP header fields (e.g., payload/packet type, block type), but may also be based on carried measurement data; and

- the filter rule actions generally cover forwarding, blocking, counting or modification of data units at the levels of RTP/RTCP packets, reports down to individual measurement data.

NOTE 4 – Specification and syntax of such policy rules are out of scope of this Recommendation.

3.2.4	loopback point (LP) (adapted from [ITU-T H.248.85]): The location of a loopback function. In the context of this Recommendation, a loopback function comprises a copy and forward operation at the level of protocol data information, with primary scope on "IETF Media Loopback" (according to clause 3.2.3/[ITU-T H.248.85]).

NOTE 5 – A loopback function is different in comparison to a redirect function, which does not copy any protocol data.  The loopback function is called "loopback mirror" in [b-IETF RFC 6849]. The "H.248 Media Loopback" (see clause 3.2.2/[ITU-T H.248.85]) does relate to a redirect function only in contrast to the "IETF Media Loopback". Such a redirect function might be subject of consideration when used in performance monitoring solutions.

3.2.5	Subject to profile specification: When used this term indicates that the H.248 profile template section requires further specification by a profile specification.

3.2.6	transport level metrics: a protocol stack based classification of a performance metric. Such a performance metric could be realized solely on protocol information elements from the transport layer and lower layer protocols.

NOTE 6 – Example: an H.248 IP-IP MG in "media-agnostic, transport-aware" mode could still support measurements for transport level metrics

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations:

| 2G-PLMN | 2nd Generation Public Land Mobile Network |
| --- | --- |
| AMG | Access Media Gateway |
| APSI | (RTCP) Application-Specific Identifier (SDES item) |
| B2BRE | Back to Back RTP Endsystem |
| BGW | Border Gateway |
| BT | Block Type (RTCP XR) |
| CP | Collection Point |
| DLSR | xxx |
| DS0 | xxx |
| FP | Filtering Point |
| GoS | Grade of Service |
| GW | Gateway |
| IP | Internet Protocol |
| ISDN | Integrated Services Digital Network |
| LP | Loopback Point |
| LSR | xxx |
| Lx | Layer number |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MOS | Mean Opinion Score |
| MP | Measurement Point |
| N/A | Not Applicable |
| NE | Near-end |
| NGN | Next Generation Network |
| OP | Originating Point ("source of RTP/RTCP traffic") |
| PCI | Protocol Control Information |
| PSTN | Public Switched Telephone Network |
| PT | Packet Type |
| QoE | Quality of Experience |
| QoS | Quality of Service |
| RMG | Residential Media Gateway |
| RP | Reporting Point |
| RPH.248 | RP H.248 (control plane interface) |
| RPRTCP | RP RTCP (user plane interface) |
| RPSIP | RP SIP (control plane interface) |
| RR | Receiver Report (RTCP) |
| RTCP | RTP Control Protocol |
| RTP | Real-time Transport Protocol |
| RTPE | RTP Endsystem |
| RTPMT | RTP Media Translator |
| RTPTT | RTP Transport Translator |
| SDES | (RTCP) Source Description |
| SDO | Standard Development Organisation |
| SDU | xxx |
| SDP | Session Description Protocol |
| SIP | Session Initiation Protocol |
| SN | Sequence Number |
| SR | Sender Report (RTCP) |
| TDM | Time Division Multiplexed |
| TGW | Trunking Gateway |
| TS | Type of Service |
| UDP | User Datagram Protocol |
| UE | User Equipment |
| VoIP | VoIP |
| XR | eXtended Report (RTCP) |
| XNQ | (H.248) eXtended Network Quality (package) |

# 5	Conventions

## 5.1	Prescriptive language for requirements specification

This document provides a list of items (in clause 8), labelled as R-x/y, where x refers to the clause number and y a number within that clause. Such items use the following keywords with meanings as prescribed below:

- The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

- The keywords "is prohibited from" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

- The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required.  Thus this requirement need not be present to claim conformance.

- The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor's implementation must provide the option and the feature can be optionally enabled by the network operator/service provider.  Rather, it means the vendor may optionally provide the feature and still claim conformance with the specification.

# 6	Relation to other ITU-T H.248.x-series Recommendations

The purpose of this clause is to identify possible relations (or not) to other Recommendations and past work on H.248 profiles. An overview and background information is provided by Appendix IV. Below are the two in force H.248 Recommendations with support of RTCP extension reports and an expired work item for additional background information.

…

## 6.2	ITU-T Rec. H.248.48: RTCP XR Block Reporting Package

This Recommendation addresses the issues regarding H.248.30 and the work on H.248.xnq. H.248.48 represents a comprehensive measurement concept and obsoletes H.248.30 and the H.248.xnq work.

## 6.3	Expired ITU-T proposal H.248.xnq: Extended Network Quality Metrics Packages for Next Generation Networks

This expired work (see Appendix II) was developed in order to address the limitations of the [ITU-T H.248.30] defined measurement framework, as well as the IETF work on additional RTCP performance metrics. The work on the H.248 packages was completed, but the draft was not progressed or published by ITU-T in favour of a design change for a solution that is future proof and decoupled from IETF standardization development.

# 7	Classification of performance metrics

Performance metrics for RTP traffic could be structured in several categories. The following classification characteristics are useful for this Recommendation:

- protocol stack based ("metrics based on protocol information elements”);

- traffic direction based ("metrics based on bidirectional information of an RTP session or unidirectional only"); and

- network route based ("metrics based on network topology and RTP/RTCP traffic routes (such as metrics related to round trips, one-ways, loopbacks, etc.)").

…

# 8	Roles provided by MG

…

The following separate functions are identified for an MG with an (IP, IP) connection model (Figure 3):

- Generation of local measurements (MPL);

- Reporting of local measurements (RPL,H.248) via H.248 towards MGC;

- Reporting of local measurements (RPL,RTCP) via RTCP towards remote terminals, remote H.248 MGs, remote RTP endsystems, or other RTP entities;

- Collection of remote measurements (CPR,H.248);

- Reporting of remote measurements (RPR,H.248) towards MGC;

- Loopback of incoming RTP/RTCP traffic (LPRTP) back to a remote RTP endsystem [purpose: the remote RTP endsystem may then provide a local measurement point of a remote sink, i.e., loopback allows to relocate MPs];

- Filtering of remote measurements (FPOut) in an outgoing direction, from provider A towards B (which are transported via RTCP reports); and

- Filtering of remote measurements (FPIn) in ingress direction, from provider B (which are transport via RTCP reports).

## 8.1	H.248 MG as general measurement point (MP)

MG capability related requirements:

R-8.1/1:	The MG is required to support local performance measurements.

Technology-dependent requirements related to metric types which are particularly defined for RTP traffic:

NOTE 1 - Subsequent requirements consider "grouped performance metrics" at the top-level of H.248 packages or IETF RFC documents.

R-8.1/2:	The MG is required to execute measurements for the performance metrics as defined by the RTP package [ITU-T H.248.1].

R-8.1/3:	The MG is required to execute measurements for the performance metrics as defined by the RTP application data package [ITU-T H.248.58].

R-8.1/4:	The MG is required to execute measurements for the performance metrics as defined by the Received RTCP package [ITU-T H.248.71].

R-8.1/5:	The MG is required to execute measurements for the performance metrics as defined by RTCP XR block types 1 to 7 [IETF RFC 3611].

NOTE 2 – There are two reporting options via H.248 (RPH.248): [ITU-T H.248.30] or [ITU-T H.248.48], see clause 8.5.

R-8.1/6:	The MG is required to execute measurements for the performance metrics as defined by RTCP XR block type 8 [IETF RFC 5093].

R-8.1/7:	The MG is required to execute measurements for the performance metrics as defined by other RTCP XR block types.

…

### 8.2.1	Location-independent requirements

The following principal requirements are derived:

R-8.2.1/1:	The MG is required to support at least transport level measurements.

NOTE 1 – This is the very minimum requirement, applicable for almost all MG scenarios (related to RTP topologies, related to metric types according to clause 3.2.6).

R-8.2.1/2:	The MG is required to support transport level metrics, independent of the supported H.248 IP connection model ("all IP" or "IP-to-non-IP") and independent of RTP topologies.

## 8.2.2	Location and metric type specific restrictions

Some performance metrics are only applicable for particular locations in a network topology. Especially application level metrics are tied to the source/sink of application level traffic, the location of the final consumer of the service (e.g., the type of metrics related to quality of experience).

R-8.2.2/1:	The MG is prohibited to support application level metrics in  network locations where the support of the measurement is non-sensical due to possible violation of the metrics semantic.

NOTE 1 – This requirement is consistent with [IETF RFC 6792], see clause 3.2 "Location of RTP Monitors".

R-8.2.2/2:	The MG is required to notify the MGC of such a violation, if enforced by the MGC for such kind of measurements, by e.g. an error code.

…

### 8.4.1	Measurement data

R-8.4.1/1:	The MG is required to send RTCP basic reports (SR and/or RR).

R-8.4.1/2:	The MG is required to receive RTCP basic reports (SR and/or RR).

R-8.4.1/3:	The MG is recommended to send RTCP extension reports (XR; NOTE 1).

R-8.4.1/4:	The MG is recommended to receive RTCP extension reports (XR; NOTE 1).

NOTE 1 – This requirement could be further detailed to the block type (BT) level or below the BT level down to individual metrics.

### 8.4.2	Measurement configuration

R-8.4.2/1:	The MG is recommended to send the measurement identity related SDES item (such as the Application-Specific Identifier (APSI) according clause 3.1/[IETF RFC 6776]).

R-8.4.2/2:	The MG is recommended to receive measurement identity related SDES item (such as the Application-Specific Identifier (APSI) according clause 3.1/[IETF RFC 6776]).

R-8.4.2/3:	The MG is recommended to send measurement information according to XR block type 14 (clause 4/[IETF RFC 6776]).

R-8.4.2/4:	The MG is recommended to send measurement information according to XR block type 14 (clause 4/[IETF RFC 6776]).

## 8.5	H.248 MG as reporting point at H.248 interface (RPH.248)

### 8.5.1	Reporting content

This reporting capability is tightly coupled to the H.248 signalling capability according to individual H.248 packages:

R-8.5.1/1:	The MG is required to support H.248 statistics according to the RTP package [ITU-T H.248.1].

R-8.5.1/2:	The MG is required to support H.248 statistics according to the RTCP extended performance metrics package [ITU-T H.248.30].

R-8.5.1/3:	The MG is required to support H.248 statistics according to the RTCP XR Block Reporting package [ITU-T H.248.48].

R-8.5.1/4:	The MG is required to support H.248 statistics according to the RTP application data package [ITU-T H.248.58].

R-8.5.1/5:	The MG is required to support H.248 statistics according to the Received RTCP package [ITU-T H.248.71].

### 8.5.2	Reporting control

The principal H.248 control capabilities for statistics reporting are summarized in Appendix IV/[ITU-T H.248.1]. The following three major practices may be considered from H.248 command perspective:

R-8.5.2/1:	The MG is required to support H.248 statistics reporting at the end of communication phases only (i.e., related to SUBTRACT command).

R-8.5.2/2:	The MG is required to support H.248 statistics reporting during active communication phases on request by the MGC (i.e., related to AUDITVALUE command).

R-8.5.2/3:	The MG is required to support H.248 statistics reporting during active communication phases dependent on configured reporting conditions (i.e., related to [ITU-T H.248.47] and NOTIFY command).

## 8.6	H.248 MG as filtering point (for RTCP traffic) (FP)

Example scenarios with filtering support are illustrated in Appendix I, see use cases 2, 3, 4, 6 and 7. The specific filter capabilities are basically given by the particular filter rule conditions and filter rule actions (see clause 3.2.3). The level of conditions and actions is useful for the derivation of filter requirements.

Identification requirements, hierarchically ordered:

R-8.6/1:	The MG is required to support filter rule conditions related to the identification of RTP/RTCP traffic as such (e.g., related to the usual 5-tuple for an RTCP control flow (under the condition of separate IP transport connections for RTP and RTCP)).

R-8.6/2:	The MG is required to support filter rule conditions related to the identification of RTCP packets based on the RTCP packet type (PT) codepoint.

R-8.6/3:	The MG is required to support filter rule conditions related to the identification of RTCP report block structures based on the RTCP block type (BT) codepoint.

R-8.6/4:	The MG is required to support filter rule conditions related to the identification of an individual metric within an RTCP report block, based on the report block structure.

Action execution requirements:

a) action type "forwarding":

R-8.6/5:	The MG is required to support unmodified forwarding of information entities according to requirements R-8.6/1-4, as the default action.

b) action type "blocking":

R-8.6/6:	The MG is required to support blocking of the entire RTCP traffic.

NOTE 1 – This action may be subject of "abnormal traffic handling", but may violate end-to-end RTCP protocol applications.

R-8.6/7:	The MG is required to support blocking of specific RTCP packet types.

NOTE 2 – This action may relate to the discard of an entire RTCP packet or the removal of an individual RTCP packet unit in case of a compound RTCP packet.

R-8.6/8:	The MG is required to support blocking of specific RTCP block types.

NOTE 3 – The identified RTCP block would be entirely removed from an RTCP XR packet.

c) action type "modification":

R-8.6/9:	The MG is required to support the modification of an individual measurement value, i.e., the carried value of a specific metric type in an RTCP packet. Any concrete modification operation (such as a default value setting, value resetting, setting of a specific value) is out of scope of this Recommendation.

NOTE 4 – Such an action is typically requested by a network operator, which does not want to disclose its own measurement results to peering partners.

All above actions should be executed in a protocol correct manner, i.e., any modified RTP/L4/IP packet should be syntactically correct, which may imply the adaptation of length, padding and/or recalculation of checksum information.

## 8.7	H.248 MG as loopback point (for RTP/RTCP traffic) (LP)

There are multiple loopback options:

R-8.7/1:	The MG is required to support a LP in "native H.248" mode, i.e. due to a StreamMode property value LoopBack setting.

R-8.7/2:	The MG is required to support a LP according to [b-IETF RFC 6849] in "encapsulated packet loopback" mode.

R-8.7/3:	The MG is required to support a LP according to [b-IETF RFC 6849] in "direct loopback" mode.

R-8.7/4:	The MG is required to support a LP according to [b-IETF RFC 6849] in "media loopback" mode.

## 8.8	Support of specific RTP topologies

The relation between RTCP services and RTP topologies in H.248 entities is described in [b-ITU-T H.248.RTPTOPO]. Performance monitoring based of RTCP XR belongs to supplementary RTCP services which are basically an overlay function and hence independent of RTP topologies. Whereas performance monitoring using RTCP basic reports (such as SR, RR) would depend on RTP topologies.

R-8.8/1:	The MG is recommended to support RTP topology “RTP endsystem” (e.g. in use case 1 (see clause I.2)).

R-8.8/2:	The MG is recommended to support RTP topology “Back-to-back RTP endsystem” (e.g. in use case 6 (see clause I.7) with implicit filtering of RTCP measurement reports).

R-8.8/3:	The MG is recommended to support RTP topology “RTP transport translator” (e.g. in use case 2 (see clause I.3) with support of RTP-aware transport level metrics according RTCP basic reports).

R-8.8/4:	The MG is recommended to support RTP topology “RTP media translator” (e.g. in case of audio transcoding within an end-to-end RTP session).

# 9	H.248 profile specification guidelines

This clause provides guidelines for H.248 profile specifications. The structure follows the profile template according to Appendix III/[ITU-T H.248.1].

The template elements which are not applicable in this Recommendation are indicated by "Subject to profile specification".

Any profile guidelines are primarily dependent on the concerned network configuration and use case (see examples in Appendix I). The guidelines in this clause are therefore basically conditional. Two exemplary use cases are considered (as described in Appendix V), termed as capability set CSA and CSB. Capability set ‘A’ (CSA) is a trunking gateway performance monitoring scenario according use case 1 (clause I.2); and Capability set ‘B’ (CSB) is a border gateway performance monitoring scenario according use case 6 (clause I.7).

…

## 9.4	Connection Model

| Maximum number of contexts: | N/A |
| --- | --- |
| Maximum number of terminations per context: | N/A Examples: IF CSA THEN “2” (TGW). IF CSB THEN “2” (BGW). |
| Allowed termination type combinations in a context: | Context (one or more IP terminations) Examples: IF CSA THEN “(IP, phy) connection model”. IF CSB THEN “(IP, IP) connection model”. |

NOTE – The scope of this Recommendation is only applicable to H.248 gateways in IP networks.

…

### 9.7.3	Events Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations does not have implications on this Descriptor.

…

### 9.7.7	Statistics Descriptor

NOTE – H.248-based performance monitoring functions at RTP stream/terminations may demand for support of statistics in case of a RPH.248 requirement.

Examples:

Are statistics supported on terminations, streams or both?

| Statistics supported on: | Examples: IF CSA THEN “termination-level statistics only”. IF CSB THEN “both”. |
| --- | --- |

Are statistics to be reported?

| Statistics reported on Subtract: | IF CSA OR CSB THEN “YES” |  |
| --- | --- | --- |
| If yes | StatisticIDs reported: | IF CSA OR CSB THEN “ALL” (according package usage details by clause 5.14.3). |

…

### 9.7.10	Error Descriptor

Which H.248.8 and package defined error codes are supported?

Error Codes sent by the MGC:

| Supported H.248.8 Error Codes: | Subject to profile specification |
| --- | --- |
| Supported Error Codes defined in packages: | Subject to profile specification |

Error Codes sent by the MG:

| Supported H.248.8 Error Codes: | Subject to profile specification |
| --- | --- |
| Supported Error Codes defined in packages: | Subject to profile specification Examples: IF CSA OR CSB THEN “(additional) support of error codes 485, 486 and 487” (see clause 6.5/[ITU-T H.248.48]). |

…

#### 9.14.3.1	RTP Package

(if [ITU-T H.248.1], Annex E.12 approach)

Examples:

IF CSB THEN “usage details for the five statistics”:

| … |  |  |  |  |
| --- | --- | --- | --- | --- |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: | Termination/Stream Types Supported: |
| ps, (0x0004) | M | ADD, MOD, SUBTRACT, NOTIFY | ALL | IP |
| pr, (0x0005) | M |  | ALL | IP |
| pl, (0x0006) | M |  | ALL | IP |
| jit, (0x0007) | O Not required (NOTE 1) |  | - | - |
| delay, (0x0008) | M |  | ALL | IP |
| Error Codes | … |  |  |  |
| NOTE 1 – Assumption: support of RTCP XR packet delay variation metrics according [b-IETF RFC 6798], as part of the xrbr package (see clause 9.14.3.7). |  |  |  |  |

#### 9.14.3.2	Received RTCP Package

(if [ITU-T H.248.71] approach)

Examples:

IF CSB THEN “usage details for the five statistics”:

| … |  |  |  |  |
| --- | --- | --- | --- | --- |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: | Termination/Stream Types Supported: |
| rps, (0x0001) | M (NOTE 1) | ADD, MOD, SUBTRACT, NOTIFY | ALL | IP |
| ros, (0x0002) |  |  | ALL | IP |
| rpl, (0x0003) |  |  | ALL | IP |
| rcpl, (0x0004) |  |  | ALL | IP |
| rjit, (0x0005) |  |  | ALL | IP |
| Error Codes | … |  |  |  |
| NOTE 1 – Assumption: there may be two types of UEs in IP network domain Xd (see Figure I.6): a) UEs with support of RTCP basic report related performance metrics only, and b) UEs with additional support of RTCP extension report defined performance metrics. The statistics of the recrtcp package are only used for terminal of type (a). |  |  |  |  |

…

#### 9.14.3.8	RTCP Source Description Package

(possibly, if [ITU-T H.248.71] approach)

Examples:

IF CSB THEN “usage detail according following table …”:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| lssrc, (0x0001) | O (NOTE 1) | same as clause 9.14.3.7 | ALL |  | IP |
| rssrc, (0x0002) | M | same as clause 9.14.3.7 | ALL |  | IP |
| lcname, (0x0003) | M | same as clause 9.14.3.7 | ALL |  | IP |
| rcname, (0x0004) | M | same as clause 9.14.3.7 | ALL |  | IP |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE 1 – Local SSRC usage is conditional on the applied RTP topology (see Table V.2). |  |  |  |  |  |

…

#### 9.17.1.1	Enabling of MPs

Subject to profile specification.

Examples:

IF CSA OR CSB

THEN “measurements should always start immediately with creation of the termination or stream end point”,

THEN “enabling of MPs” should conceptually follow the procedures of clauses IV.2.1, IV.2.2 or IV.2.3 according to [ITU-T H.248.1].

#### 9.17.1.2	Disabling of MPs

Subject to profile specification.

Examples:

IF CSA OR CSB

THEN “the deactivation of measurements should be supported”,

THEN “deactivation of MPs” should conceptually follow the procedures of clauses IV.3.3 or IV.3.5 according to [ITU-T H.248.1].

…

##### 9.17.2.1.2	Resetting of statistics (RPH.248)

Subject to profile specification.

NOTE – The conceptual procedure is described in clause IV.4/[ITU-T H.248.1].

…

## I.2	Use case 1: H.248 MG with connection model (RTP, non-RTP), e.g. Trunking gateway

Figure I.1 illustrates an end-to-end media/bearer path between user equipment (UE) in a PSTN and VoIP network domain. The interworking of the two different user plane protocol stacks is provided by an H.248 physical-to-RTP media gateway (such as a trunking gateway (TGW) in this example). It may be noted that the TGW is located at the edge of the RTP domain, hence providing the "RTP endsystem" topology (according to [b-IETF RFC 5117]). However, the "RTP endsystem" is located at core network level, far away from the real user location ("the remote PSTN terminal UE B"). This observation is important concerning performance measurements with respect to user specific metrics (such as of type "QoE").

…

The performance monitoring network is engineered in such a way, that the reporting of measurement results towards the control plane is solely under responsibility of H.248 MGs ("and not of VoIP terminal equipment"). There is consequently a H.248-based reporting point (RPH.248). UE A therefore has to transfer the measurements of MPT and MPA via RTCP ("the RPRTCP of UE A ") back to the H.248 MG, which collects the data (via CP) and reports them typically at a later point in time (e.g., when the call is terminated) via RPH.248 as H.248 statistics.

Observation I.1.b:

No RPRTCP support is required by the H.248 MG because there is no usage of correspondent RTCP data by UE A.

It may be further recalled again that circuit-oriented UEs (like PSTN or ISDN terminals) do not support the generation of measurements for user plane traffic and their reporting ("hence, there is no MP and RP provided by UE B in Figure I.1").

## I.3	Use case 2: IP-IP H.248 MG and remote SIP UE – Single control plane RP, by MG

The end-to-end media/bearer path is now continuously RTP based (Figure I.2). The H.248 MG provides the role of a so called border gateway (BGW), characterized by an H.248 (IP, IP) connection model.

…

Observation I.2.a:

There is a single H.248 connection model (IP, IP), but a variety of different possible RTP topologies concerning the interconnection of RTP traffic between UEs A and B (see [b-ITU-T H.248.RTPTOPO]). The various RTP modes of operations as assigned to the BGW may impact measurements, filtering and/or reporting capabilities.

The evaluation of RTP topology dependencies is out of scope of this example. The H.248 MG provides MPs in A-to-B (1) and B-to-A (2.1) direction. The two MPs in B-to-A direction, (2.1) and (2.2) are necessary in order to estimate the impact of incoming RTP traffic from domain Xd+1 on domain Xd.

Observation I.2.b:

The number of MPs in a unidirectional traffic path may differ between the two traffic directions, from the perspective of a particular network operator and from the perspective of the MG location in the end-to-end RTP path.

The reporting framework, - in user and control plane -, is identical to use case #1.

Additional filtering capabilities:

- Network operator Xd wants to avoid the distribution of RTCP transferred measurement data outside his network. There is thus a FP in A-to-B direction located at the MG (5).

- There may be incoming RTCP reports from peering partner Xd+1, which could be filtered by the MG (6). Multiple filtering options might be imaginable: e.g., transparent forwarding, blocking of entire RTCP reports or early selected deletion of unwanted measurement data.

There are many conceivable variations of this use case. E.g., FP (6) could be augmented or replaced by a CP and RPH.248, e.g. in order to capture all incoming RTCP data already at the border of domain Xd.

## I.4	Use case 3: IP-IP H.248 MG and remote SIP UE – Single control plane RP, by UE

…

The H.248 MG is now requested to support RTCP-based reporting, see RPRTCP (3.1) and (3.2). The SIP UE collects the data and provides SIP-based reporting (together with the UE-local measurements (2.2)).

No H.248 Statistics are requested from the H.248 MG; the RPH.248 is hence lacking in this example.

Additional filtering capabilities:

- Any FP in egress direction is not required (5), because the SIP UEs could already suppress the distribution of RTCP measurement reports.

- The ingress situation is similar as in previous use case #3.

## I.5	Use case 4: IP-IP H.248 MG and remote SIP UE – Single location of MPs ("LP by MG")

The MP is normally at the same location as the projected observation point (as in previous examples). However, the MP could be geographically relocated from the observation point in case of an available transport means for transferring the concerned traffic data from the observation point to a remote location. "Loopbacking" is such a capability: the traffic is redirected at the loopback point (LP) in the reverse direction.

Observation I.4.a:

Loopbacking permits the geographical relocation of the MP, "anywhere" in the "loopback path". It may be noted that the relocated MP doesn't have to be at the location of the originating point (OP); it may also be in between the LP and OP section.

[b-IETF RFC 6849] defines a "media loopback" capability, which could be supported by H.248 entities (see [ITU-T H.248.85]). The notion of "media" is primarily related to all kinds of application data carried via RTP. Figure I.4 depicts a use case with an enabled "media loopback" at the H.248 BGW.

…

The H.248 BGW provides a loopback point (LP) for RTP traffic originating from UE A. The IETF "loopback source" and "loopback mirror" roles are assigned to UE A and BGW respectively in this example. The motivation for using an LP instead of a local MP at an H.248 MG is elaborated in more detail in clause I.9.

## I.6	Use case 5: Distributed control plane reporting with near-end measurements only (at the example of a Trunking gateway)

This is a variation of use cases #1 and #3, whereby control plane reporting is distributed between two RTP nodes (Figure I.5). The MPs are identical to use case #1, but each RTP node reports their own measurements, also known as near-end measurements (see also [ITU-T H.248.71]).

UE A provides a SIP-based reporting interface (RPSIP, e.g. based on [b-IETF RFC 6035]). The H.248 MG (here a TGW, but may also be a BGW) reports its near-end (NE) measurements (via RPH.248) in the form of H.248 statistics.

…

## I.7	Use case 6: IP-IP H.248 MG as peering node

…

Provider domain Xd  and the selected control plane reporting point in H.248 BGW (Nk) is of interest in use case 6. Measurement, reporting and filtering is similar to the previous use cases. The H.248 BGW (Nk) additionally provides the RPRTCP capability (3.1) and (3.2) for the reporting of MG-local measurements (1) and (2.1) respectively. UE A therefore also has all measurements results available: they could be used by the UE-embedded application control logic.

## I.8	Use case 7: Multiple IP network domains – segment-based measurements

Multiple IP network domains may be interconnected by H.248 BGWs (Figure I.7). Each network domain relates basically to a measurement segment from the perspective of an end-to-end media/bearer path. The primary scope of this use case is a H.248 BGW node Nk and provider domain Xd.

The fundamental challenge of this use case is the assumption that there are different network conditions in each segment, i.e., the Grade of Service (GoS) for IP traffic (here: RTP packets) diverges per network domain Xi. Each provider is interested in trying to figure out the GoS of their peering partners, besides the measurement of his own GoS values. This is in order to estimate the impact on the end-to-end QoS like the QoE conditions for users A and B.

Such an analysis is basically feasible, when the measurement segments are bounded by H.248 BGWs, the usage of RTCP XR and the H.248.48 based "performance monitoring suite" (e.g., plus H.248.71) in order to configure MPs, CPs, RPs, LPs and/or FPs.

…

## I.9	Use case 8: MP relocation by loopbacking

…

- MP relocation to "previous RTP node" (here node Nk-1), instead of RTP source endpoint (here: UEA)?

- Background: The relocated MP could be "somewhere" in the loopback path. There are thus two principal options in this use case (node Nk-1 or UEA).

- The option of node Nk-1 is actually out of scope of [b-IETF RFC 6849], but feasible in network scenarios with interim H.248 IP-IP gateways (as illustrated above);

- The "node Nk-1" option provides the following advantages:

  - mitigates the basic drawback of increased network transport capacity, and/or of the lack of measurement capability in the UE;

  - the loopback service may not at all be visible to the UE, when node Nk-1 acts as a "virtual loopback source" in above scenario.

Thus, as an overall conclusion: MP relocation by loopbacking is not a scalable option at network level (see also [b-IETF RFC 6849]), but might be beneficial and justified under specific conditions.

…

Appendix III

Example performance metric types

(This appendix does not form an integral part of this Recommendation)

## III.1	Background

Possible classification schemas for performance metrics for RTP traffic are described in clause 7. The protocol stack based method uses the terms application and transport level metrics (see clauses 3.2.1 and 3.2.6). This appendix provides some typical examples for each category.

There are examples which may only be metricsgenerated MG-locally and reported via H.248, metrics which may be reported via RTCP, and examples for both reporting options.

## III.2	Transport level metrics

### III.2.1	RTP-agnostic transport level metrics

Example metrics:

- all four statistics according to [ITU-T H.248.61] concerning octet and packet based IP traffic volume (for RTP sessions)

- packet delay variation [b-IETF RFC 6798]

H.248 Network package:

- statistics os and or, see clause E.11.4/[ITU-T H.248.1]

### III.2.2	RTP-aware transport level metrics

These kind of transport level metrics are RTP-aware due to consideration of RTP header information TS, SN and/or RTCP data.

Example metrics:

- Jitter buffer nominal delay [b-IETF rtcp-xr-jb]

- Jitter buffer maximum delay [b-IETF rtcp-xr-jb]

- Jitter buffer high water mark and jitter buffer low water mark [b-IETF rtcp-xr-jb]

- Sum of Burst Durations [b-IETF RFC 6958]

- Packets lost in bursts [b-IETF RFC 6958]

- Total packets expected in bursts [b-IETF RFC 6958]

- Number of bursts [b-IETF RFC 6958]

- Sum of Squares of Burst Durations [b-IETF RFC 6958]

- Round trip delay [IETF RFC 3611]

- Network round trip delay [b-IETF RFC 6843]

H.248 RTP package:

- All five statistics (ps, pr, pl, jit and delay), see clause E.12.4/[ITU-T H.248.1]

H.248 Received RTCP package:

- All five statistics (rps, ros, rpl, rcpl and rjit), see clause 7.4/[ITU-T H.248.71]

## III.3	Application level metrics

### III.3.1	RTP-agnostic application level metrics

Example metrics:

These examples are based on the pure application data stream at receiver side:

- Unimpaired seconds [b-IETF rtcp-xr-conceal]

- Concealed seconds [b-IETF rtcp-xr-conceal]

- Severely Concealed Seconds seconds [b-IETF rtcp-xr-conceal]

- Unimpaired seconds [b-IETF rtcp-xr-conceal]

The following example relates to fax-over-IP using [ITU-T T.38] in T.38-over-RTP transport mode and an H.248 IP termination enabled with the IP FAX package [ITU-T H.248.2]:

- Pages Transferred defined by H.248 statistic ipfax/pagestrans; is an application level metric due to data object 'page', and RTP-agnostic due to independence of RTP header information

Statistics of RTP application data package [ITU-T H.248.58] (again due to RTP payload relation only ("application data"), but independence of RTP header information):

- RTP payload octets sent and

- RTP payload octets received

### III.3.2	RTP-aware application level metrics

For these metrics the measurement function must be media format aware, based on the RTP payload type codepoint, in contrast to the examples of clause III.3.1.

Example metrics:

- MOS value (attributed by MOS type and calculation method) [b-IETF rtcp-xr-qoe]

- signal level [IETF RFC 3611]

- noise level [IETF RFC 3611]

- residual echo return loss [IETF RFC 3611]

- R factor [IETF RFC 3611]

- external R factor [IETF RFC 3611]

- end system delay [b-IETF RFC 6843] (virtual, internal round-trip delay in topology “RTP endsystem”; its an application level metric due to inclusion of encoding/decoding delays)

Appendix IV

Relation between "IETF RTCP XR" documents 
and ITU-T H.248.x work products

(This appendix does not form an integral part of this Recommendation)

## IV.1	Background

There were three major steps in the ITU-T concerning the support of "RTCP XR" based measurements and reporting. Figure IV.1 recalls the timeline and evolution of H.248.x capabilities for support of RTCP XR.

…

Figure IV.2 – Link between "IETF RTCP XR" documents and ITU-T H.248.x work products

Figure IV.2 indicates three major areas:

- Performance parameters (solely defined by IETF; metric definition based RTCP XR template);

- Performance measurement & reporting variants (for decomposed gateways in the RTP/RTCP bearer plane; defined by [ITU-T H.248.30], [b-ITU-T H.248.xnq] and [ITU-T H.248.48])

- Complementary framework & capabilities (primarily for H.248.48-based performance monitoring models; such as [ITU-T H.248.71], [ITU-T H.248.79], [ITU-T H.248.85] and [b-ITU-T H.248.RTPTOPO])

Figure IV.2 indicates the difficulties for service providers, network operators or manufactures in identifying the right technology for a particular deployment scenario ("which is finally given by an H.248 profile specification"). For instance, it is not evident that there are self-contained H.248.48 solutions, or whether always a combination together with the H.248.71 rtcpsdes package would be required.

The next sub-clause discusses typical capabilities in scope of H.248-based performance monitoring in RTP networks.

## IV.2	Capabilities versus H.248.x packages support

This section compares the different approaches of [ITU-T H.248.30], [b-ITU-T H.248.xnq] and [ITU-T H.248.48], their purposes, characteristics, but also principal limitations. The different H.248.x solutions satisfy different sets of requirements (or support different capability sets). The capabilities may be structured in multiple areas. Table IV.1 indicates the main categories:

Table IV.1 – "RTCP XR"-based Performance Monitoring: 
Differences H.248.30 vs H.248.xnq vs H.248.48 – Part 1

| Capability |  | (1) H.248.30 | (2) H.248.xnq | (3) H.248.48 | Comments |
| --- | --- | --- | --- | --- | --- |
| A) Supported RTCP XR metric types |  | BT = 1 - 7 (limited) | BT = 8 (limited) | BT = ”All” (NOTE 1) | A block type (BT) combines multiple performance metrics. |
| B) Supported RTP Topologies? |  |  |  |  | See also [b-ITU-T H.248.RTPTOPO]. |
| B.1 | RTP Endsystem (RTPE) | Yes | Yes | Yes |  |
| B.2 | Back-to-Back RTP Endsystem (B2BRE) | Yes | Yes | Yes |  |
| B.3 | RTP Translators (RTPTT, RTPMT) | No | No | Yes | “RTP translator" topology required for e.g. network peering |
| B.4 | RTP Mixer | Yes | Yes | Yes |  |
| C) H.248 MG as Reporting Point (RP) |  |  |  |  | via H.248 statistics … |
| C.1 | Only Near-End? | Yes | Yes | Yes |  |
| C.2 | Centralized RP for both directions? | No | No | Yes | ‚No' means that two MGs must be used, one for each direction |
| D) H.248 MG as Filtering Point (FP) |  | No | No | Yes | ‚No' because „not applicable" |
| E) H.248 MG types (Connection Model) |  |  |  |  |  |
| E.1 | Physical-to-RTP (e.g. H.248 RMG, AMG, TMG) | Yes | Yes | Yes |  |
| E.2 | IP-to-IP (incl. RTP-to-RTP) | No | No | Yes |  |
| NOTE 1 – All RTCP XR block types as registered with IANA, see [b-IANA RTCP XR]. |  |  |  |  |  |

The content of Table IV.1 reflects the network evolution path of the past decade, from anetwork architecture initially dominated by H.248 MGs at the border between an IP network and circuit-oriented networks (such as PSTN, ISDN, 2G-PLMN) towards the direction of "all-IP networks" with it's H.248 MGs as IP-IP gateway/router entities.

Table IV.2 provides some categories concerning technology status and profile availability.

Table IV.2 – "RTCP XR"-based Performance Monitoring: 
Differences H.248.30 vs H.248.xnq vs H.248.48 – Part 2

| Capability |  | (1) H.248.30 | (2) H.248.xnq | (3) H.248.48 | Comments |
| --- | --- | --- | --- | --- | --- |
| F) Part of H.248 Profile? |  |  |  |  |  |
| F.1 | Standardized Profile? | No | No | Not yet | [ITU-T H.248.48] is on the agenda for Border GWs . |
| F.2 | Other Profiles? | - | - | - | Out of scope of this Recommendation. |
| G) Non-H.248 Reporting? |  |  |  |  | Reporting of RTCP XR block types . |
| G.1 | SIP | Yes | No | planned |  |
| G.2 | H.323 | Yes | No | open |  |
| G.3 | MGCP | Yes | No | open |  |
| G.4 | Diameter | No | No | open |  |

Appendix V

Use case specific capability sets – Two examples

(This appendix does not form an integral part of this Recommendation)

## V.1	Overview

Profile content is mainly use case dependent as outlined in clause 1.1. In order to demonstrate profile specification guidelines in clause 9, this Recommendation considers two example use cases:

- Capability set ‘A’ (CSA): a trunking gateway performance monitoring scenario according to use case 1 (clause I.2); and

- Capability set ‘B’ (CSB): a border gateway performance monitoring scenario according to use case 6 (clause I.7).

Tables V.I and V.2 illustrate sets of requirements (based on clause 8) for profiling the two gateway types:

…

Bibliography

[b-IANA RTCP XR]	IANA Registry of RTCP XR Block Types at http://iana.org/assignments/rtcp-xr-block-types/rtcp-xr-block-types.xhtml

[b-ITU-T H.248.xnq]	Expired Draft Recommendation ITU-T H.248.xnq (2005), Gateway control protocol: Extended Network Quality Metrics Packages for Next Generation Networks (NGNs).
Contained in document AVD-2773:
WP2/16 Rapporteur Meeting (Geneva, 28 November - 2 December 2005), 
http://ftp3.itu.int/av-arch/avc-site/2005-2008/0511_Gen/AVD-2773.zip

[b-ITU-T H.248.RTPTOPO]	Draft Recommendation ITU-T H.248.RTPTOPO (2013), Gateway control protocol: RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations.

[b-ITU-T H.323]	Recommendation ITU-T H.323 (2009), Packet-based multimedia communications systems.

[b-IETF RFC 3261]	IETF RFC 3261 (2002), SIP: Session Initiation Protocol.

[b-IETF RFC 3435]	IETF RFC 3435 (2003), Media Gateway Control Protocol (MGCP) Version 1.0.

[b-IETF RFC 5117]	IETF RFC 5117 (xxxx), xxx.

[b-IETF RFC 6035]	IETF RFC 6035 (xxxx), xxx.

[b-IETF RFC 6798]	IETF RFC 6798 (11/2012), RTP Control Protocol (RTCP) Extended Report (XR) Block for Packet Delay Variation Metric Reporting.

[b-IETF RFC 6843]	IETF RFC 6843 (01/2013), RTP Control Protocol (RTCP) Extended Report (XR) Block for Delay Metric Reporting.

[b-IETF RFC 6849]	IETF RFC 6849 (02/2013), An Extension to the Session Description Protocol (SDP) and Real-time Transport Protocol (RTP) for Media Loopback.

[b-IETF rtcp-xr-qoe]	IETF draft-ietf-xrblock-rtcp-xr-qoe (201X), RTCP XR Blocks for QoE Metric Reporting.

[b-IETF rtcp-xr-jb]	IETF draft-ietf-xrblock-rtcp-xr-jb (201X), RTCP XR Report Block for Jitter Buffer Metric Reporting.

[b-IETF RFC 6958]	IETF draft-ietf-xrblock-rtcp-xr-burst-gap-loss (2013), RTP Control Protocol (RTCP) Extended Report (XR)t Block for Burst/Gap Loss metric Reporting.

[b-IETF rtcp-xr-conceal]	IETF draft-ietf-xrblock-rtcp-xr-conceal (201X), RTCP XR Report Block for Concealed Seconds metric Reporting.

__________________