| Question(s): | 3 |  | Meeting, date: |  | Geneva, 28th October – 8th November 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 1 | Intended type of document (R-C-TD): |  | C |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |  |  |
| Title: | H.248.RTPTOPO: Updates |  |  |  |  |  |
| Contact: | Weiwei Yang Huawei Technologies China |  |  |  | Tel: +86 29 87674367 Fax: +86 29 87674151 Email: tommy@huawei.com |  |
| Contact: | Christian Groves NTEC Australia Australia |  |  |  | Tel: +61 3 9391 3457 Fax: +61 3 9391 3457 Email: Christian.Groves@nteczone.com |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

# Introduction

This Contribution proposes updates to H.248.RTPTOPO “RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations”.

# Discussion

H248.RTPTOPO is scheduled to be put forward for Consent at the 28th October – 8th November ITU-T SG16 meeting. The Contributors have performed a review of the draft Recommendation and have identified a number of editorial corrections that should be addressed. There are also a number of technical issues that need to be addressed. These have been noted via MS Word “comments”.

# Proposed

It is proposed to accept the following updates. Changes are marked up against TD-17 from the June 2013 Q.3/16 Rapporteurs’ meeting in Olso.

__________________

Draft new Recommendation ITU-T H.248.xx (ex H.248.RTPTOPO)

Gateway control protocol: 
RTP Topology dependent RTCP Handling 
by H.248 Media Gateways with IP Terminations

AAP Summary

<Mandatory material – to be provided before Consent>

Summary

<Mandatory material>

# 1	Scope

The scope of this Recommendation is related to RTP Control Protocol (RTCP) handling by H.248 Media Gateways (MG) [ITU-T H.248.1]. The required MG behaviour is fairly clear for the majority of use cases. However, there are some scenarios which benefit from or even require explicit signalling indication for an unambiguous control of MG RTCP (but also RTP) traffic handling behaviour.

This Recommendation covers

- information about history and motivation behind RTP topologies and RTCP handling and the available H.248 tool landscape (clause 6);

- overview tables concerning "RTP topology versus RTCP handling" aspects and relevant H.248 connection models (clause 7);

- RTCP service aspects (clause 8);

- scope and limitations of RTP topology control (clause 9);

- an H.248 package for basic RTP topology control (clause 10); and

- package independent procedures for RTP topology control (clause 11).

This Recommendation may be useful in scenarios where an MG supports multiple RTP topologies and/or multiple RTP/RTCP services which may lead to ambiguous behaviour. In these scenarios a H.248 profile specification may use the package in this Recommendation to alleviate this behaviour.

# 2	References

[ITU-T G.799.3]	xxx.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.30]	Recommendation ITU-T H.248.30 (06/2008), Gateway control Protocol: IP NAPT traversal package.

[ITU-T H.248.48]	Recommendation ITU-T H.248.48 (02/2012), Gateway control Protocol: RTCP XR Block Reporting Package.

[ITU-T H.248.50]	Recommendation ITU-T H.248.50 (09/2010), Gateway control Protocol: NAT traversal toolkit packages.

[ITU-T H.248.57]	Recommendation ITU-T H.248.57 (06/2010), Gateway control Protocol: RTP control protocol package.

[ITU-T H.248.58]	Recommendation ITU-T H.248.58 (06/2008), Gateway control Protocol: Packages for application level H.248 statistics.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (02/2010), Gateway control Protocol: RTCP support packages.

[ITU-T Q.9]	xxx.

[ITU-T T.411]	xxx.

[ITU-T Y.1221]	xxx.

[IETF RFC 1889]	xxx

[IETF RFC 3550]	IETF RFC 3550 (07/2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 3551]	xxx.

[IETF RFC 3611]	IETF RFC 3611 (11/2003), RTP Control Protocol Extended Reports (RTCP XR).

[IETF RFC 3711]	xxx.

[IETF RFC 4585]	IETF RFC 4585 (07/2006), Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF).

[IETF RFC 4588]	IETF RFC 4588 (07/2006), RTP Retransmission Payload Format.

[IETF RFC 5117]	IETF RFC 5117 (01/2008), RTP Topologies.

[IETF RFC 5124]	IETF RFC 5124 (02/2008), Extended Secure RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/SAVPF).

[IETF RFC 5576]	IETF RFC 5576 (06/2009), Source-Specific Media Attributes in the Session Description Protocol (SDP).

[IETF RFC 5760]	IETF RFC 5760 (02/2010), RTP Control Protocol (RTCP) Extensions for Single-Source Multicast Sessions with Unicast Feedback.

[IETF RFC 5968]	IETF RFC 5968 (09/2010), Guidelines for Extending the RTP Control Protocol (RTCP)).

[IETF RFC 6051]	xxx.

[IETF RFC 6792]	IETF RFC 6792 (11/2012), Guidelines for Use of the RTP Monitoring Framework.

…

## 3.2	Terms defined in this Recommendation

3.2.1	Basic RTCP service: the RTCP services based on RTCP packet types SR (200), RR (201), SDES (202) and BYE (203).

NOTE 1 – These are RTCP services are considered to be the minimum of any RTP implementation according [IETF RFC 3550].

{Contributor’s note: RFC3550 doesn’t appear to specifically say this. It does also mention APP (204) application defined packets types. A translator would be expected to handle these also.}

3.2.2	Fully protocol unaware forwarding: Transparent forwarding, but the MG is unaware of the Lx protocol(s). The MG is thus not able to execute any kind of operation on the Lx-PDU.

NOTE 1 – Example for "fully RTP protocol unaware forwarding": the H.248 descriptor(s) does not contain any kind of signalling information (such as H.248 events, properties, signals, statistics or SDP elements) related to the indication of RTP traffic. This would be a fully L4+ agnostic stream setting.

…

3.2.4	RTP domain: A network domain with RTP entities which share a common RTP/RTCP-related identifier space. A RTP domain is characterized therefore by a domain wide (and temporal limited, NOTE 2) uniqueness of such protocol identifiers, which typically constitute unambiguous RTP traffic flows (e.g., protocol elements SSRC, CNAME, PT).

NOTE 2 – Temporal horizon of identifier values: [b-IETF RFC 6222] differentiates the identifier lifetime in short-term persistent and long-term persistent identifier value assignments.

NOTE 3 – An end-to-end RTP traffic flow could traverse multiple RTP domains, leading to the following observations:

- The end-to-end communication association would be comprised of multiple, concatenated RTP sessions.

- The edge of RTP domains is given by RTP entities with RTP topologies of type "RTP End system" or "Back-to-Back RTP End system".

…

3.2.8	RTCP service: A MG service tied to a particular RTCP packet type code point. The RTCP PT value may be thus considered as RTCP service identifier.

NOTE 6 – This is the coarsest granularity of an RTCP service definition, but sufficient for this Recommendation. A finer grain RTCP service level could take into account further RTCP packet embedded identifiers such as the block type (BT) code point in case of RTCP XR or the feedback message type (FMT) code point in case of RTCP FB.

NOTE 7 – Another classification approach could derive generic (because usually PT-independent) RTCP service categories such as related to performance monitoring, media encryption, codec rate control, congestion control, generic feedback, keep alive, etc.

…

3.2.10	Transparent forwarding: MG packet forwarding behaviour with the characteristic of Lx-PDU integrity. This is a unidirectional characteristic of a Lx-PDU flow.

NOTE 8 – This is normally implicit, basic behaviour, when LD/RD settings are identical: "identical media descriptions" imply "transparent forwarding" by the MG by the MG under the condition that the MG could derive the intended protocol layer Lx of transparent forwarding.

NOTE 9 – There would be then the characteristic of RTP packet integrity in the context of "RTP transparent forwarding". The MG might be RTP aware; e.g. support of RTP related statistics or event detection would not violate transparent forwarding behaviour.

NOTE 10 – Delayed transparent forwarding is a variant of transparent forward from a temporal perspective, which is characterized by transparent forwarding, but Lx-PDUs might be intentionally delayed in the outgoing direction. The traffic characteristic of the Lx-PDU flow would be then modified.
Examples:

- The application of IP traffic shaping (see clause 7.1.5/[ITU-T Y.1221]) might be one example in case of Lx equal to 'IP'.

- The application of RTP jitter elimination, reduction or adjustment (see [ITU-T G.799.3]) might be one example in case of Lx equal to 'RTP'

# 4	Abbreviations

This Recommendation uses the following abbreviations and acronyms:

| AGW | Access Gateway |
| --- | --- |
| APP | (RTCP) Application-defined packet type |
| AVP | (RTP) Audio Visual Profile |
| AVPF | (RTP) Audio Visual Profile with Feedback |
| B2BRE | Back-to-Back RTP End system |
| BGF | Border Gateway Function NOTE – synonym to Border Gateway and IP-to-IP Gateway in this Recommendation |
| BGW | Border Gateway |
| BR | (RTCP) ??? packet type |
| BT | (RTCP) Block Type |
| CNAME | Canonical Name |
| CSRC | Contributing Source |
| DCCP | xxx |
| FB | (RTCP) Feedback |
| FMT | (RTCP FB) Feedback Message Type |
| IP | Internet Protocol |
| LCD | Local Control Descriptor |
| LD | Local Descriptor |
| Lx | Layer x |
| MANE | Media Aware Network Element |
| MCU | Media Control Unit |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MSRP | Message Session Relay Protocol |
| MUX | Mulitplex |
| LD | Local Descriptor |
| NAPT | Network Address and Port Translation |
| NAT-T | NAT Traversal |
| PCI | Protocol Control Information |
| PDU | Protocol Data Unit |
| PP | Point to Point |
| PSTN | Public Switched Telephone Network |
| PT | (RTP) Payload Type; (RTCP) Packet Type |
| QoS | Quality of Service |
| RD | Remote Descriptor |
| RR | (RTCP) Receiver Report |
| RTCP | RTP Control Protocol |
| RTP | Real-time Transport Protocol |
| RTPASM | RTP Any Source Multicast |
| RTPASY | RTP Asymmetric Topologies |
| RTPE | RTP End system (or RTP endpoint) |
| RTPM | RTP Mixer |
| RTPMU | RTP Multicast |
| RTPMT | RTP Media Translator |
| RTPRTM | RTP RTCP-Terminating-MCU |
| RTPSSM | RTP Source Specific Multicast |
| RTPTF | RTP Transparent Forwarding |
| RTPTT | RTP Transport Translator |
| RTPVSM | RTP Video-Switch-MCU |
| SAVP | (RTP profile) Secure Audio Visual Profile |
| SAVPF | (RTP profile) Secure Audio Visual Profile with feedback |
| SBG | Session Border Gateway |
| SDES | Source Description (RTCP Packet) |
| SDP | Session Description Protocol |
| SEP | Stream End Point |
| SN | Sequence Number |
| SR | (RTCP) Sender Report |
| SRTP | Secure RTP |
| SSRC | Synchronization Source |
| TCP | Transmission Control Protocol |
| TF | Transparent Forwarding |
| TLS | Transport Layer Security |
| TR | Translator |
| TrGW | Trunking Gateway |
| TS | Time Space |
| UDP | User Datagram Protocol |
| UDPTL | UDP Transport Layer |
| VoIP | Voice over Internet Protocol |
| XR | (RTCP) eXtension Report |

…

# 6	Background to RTCP handling by H.248 MGs

The handling of RTCP packets by H.248 media gateways is often not straightforward as MG behaviour is typically not discussed by the applicable IETF RFCs. RTCP handling in general is tightly coupled to the RTP topology model, which drives the processing of RTP/RTCP traffic of a particular H.248 Context.

The aspect of RTP topologies is for many H.248 MG types not really relevant because the handling is obvious. For example: for all H.248 (IP, physical) connection models (as applied for VoIP MGs at the border towards circuit-switched networks). However, for H.248 (IP,IP) MGs the situation is slightly different.  In this case new RTCP services have been defined that increase the possible interactions between RTP topology and these services. This has led to possible ambiguity in the RTCP handling behaviour in the MG.

The problem also has an historical dimension: [IETF RFC 1889] and [IETF RFC 3550] were too vague on RTP topology and RTCP handling, which was the motivation to clarify on a more detailed level RTP topology models, published as [IETF RFC 5117]. [IETF RFC 5117] provides a finer grain topology classification and detailed topology description.

NOTE 1- It may be noted that the problem is not specific to H.248, it also concerns other gateway/policy control protocols, which are applied to IP-IP gateways or policy enforcement points. For example: [ITU-T H.248.48] considers different RTCP processing functions (e.g., local generation of RTCP reports, forwarding/filtering of RTCP reports). [ITU-T H.248.48] is applicable for multiple RTP topologies and could be categorized as a supplementary RTCP service (see clause 8.3.3). It has had to consider these interaction issues.

Generally RTP topology and RTCP handling may be summarized:

- Local RTCP handling by an H.248 MG is basically linked to the applied RTP topology, and the RTP topology is given by the Terminations/Streams in a H.248 Context.

- Without this Recommendation there are no explicit H.248 means defined to control RTP topology. Thus, H.248 profiles that do not utilise this Recommendation assume implicit behaviour ("which may lead to misinterpretations").

- Existing H.248 protocol elements for RTP/RTCP control are for other, different purposes, i.e.:

  - [ITU-T H.248.1] Annex E.12 "RTP Package": event and statistics

  - [ITU-T H.248.30] "RTCP extended performance metrics packages":
- RTCP XR Base Package,
- RTCP XR Burst Metrics Package

  - [ITU-T H.248.48] "RTCP XR Block Reporting Package": statistics

  - [ITU-T H.248.50] "RTP Keep Alive Request Package": "RTP signals"

NOTE 2 – Discussion: H.248.50 signal parameter "Keep Alive Packet Type" (from signal "Send keepalive packet") indicates some RTP/RTCP related packet types, which represent the semantic of RTP topology "RTP end system" (sender only). There might be then following controversial situation if the H.248 Stream endpoint were enabled with signal e.g. kar/skap/kapt/cn (indicating topology RTP end system) and also part of an overall RTP transport translator (given by a property setting of "rtpt/rtptopo = TR").

  - [ITU-T H.248.57] "RTCP Handling Package": L4 port allocation control

  - [ITU-T H.248.58] "RTP Application Data Package": statistics

  - [ITU-T H.248.71] "RTCP support packages":
- RTCP Source Description Package, 
- Received RTCP Package, 
- RTCP Feedback Message Package

The topic of correct RTCP handling by H.248 MGs, is important in order not to jeopardize end-to-end RTP-based services.

# 7	RTP topologies versus RTCP handling in H.248 MGs

This clause structures RTP Topologies versus RTCP Handling versus H.248 MG Context configurations. The functions related to RTCP handling are labelled with "F-x" according to clause 7.6. The RTP topologies are classified according to H.248 connection models which are determined by the number of terminations per Context, see clauses 7.1 to 7.4.

## 7.1	Single H.248 "IP" Termination per Context

See Table 1.

Table 1 – RTP Topologies vs RTCP Handling vs H.248 MG Models
– Single H.248 "IP" Termination per Context

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| I.1 | RTP End system (RTPE) (aka RTP Endpoint) | F-1,2,3,5 | (RTP, non-RTP) | VoIP Residential -, Access – or Trunking MG |

## 7.2	Two H.248 "IP" Termination per Context

### 7.2.1	RFC 5117 defined RTP topologies

See Table 2.

Table 2 – RTP Topologies vs RTCP Handling vs H.248 MG Models
– Two H.248 "IP" Terminations per Context

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| II.1 | Back-to-Back RTP End system (B2BRE) | F-1,2,3,4,5,6,7 | (RTP, RTP) | IP-to-IP MG located at the border between two RTP domains (e.g., SRTP-to-RTP interworking) |
| II.2 | RTP Transport Translator (RTPTT) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | IP-to-IP MG located within a single RTP domain, without any media processing |
| II.3 | RTP Media Translator (RTPMT) in general (NOTE 1) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | IP-to-IP MG located within a single RTP domain, with processing of the RTP payload |
| II.3.1 | RTP Media Translator without media format change (RTPMTlight) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | ditto, e.g., trans-packetization (changing packetization times), jitter reduction between two IP networks |
| II.3.2 | RTP Media Translator with media format change (RTPMTfull) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | ditto, e.g., audio transcoding |
| NOTE 1 – The RTPMT topology is sometimes further sub-classified here with II.3.1 & II.3.2 (see e.g. [b-IETF RFC 6679]). |  |  |  |  |

NOTE - Some media-level functions may be achieved by more than one RTP topology. For example, audio transcoding may be done in B2BRE or RTPMT topologies. However, there would be different RTCP handling functions.

### 7.2.2	Other RTP topologies

The high-level RTP topology models according to [IETF RFC 5117] have evolved since the publication of this RFC (in 2008) in terms of: more detailed information (e.g. semantic clarifications), evaluation whether the number of existing models might be reduced and the identification of new RTP topology models. This sub-clause provides descriptions of such new/extended models.

#### 7.2.2.1	RTP source translator topology

RTP source translators (RTPST) do not modify the media stream itself, but are concerned with RTP source parameters. RTP source parameters comprise of the following:

- RTP source identifier (parameter  SSRC),

- RTP source/endpoint persistent identifier (parameter CNAME),

- RTP source sequence number space (parameters SN, OSN) and

- RTP source time space (parameter TS).

RTP source translators could modify one or multiple RTP source related parameters.{Editor’s note (2013-01 meeting): RTCP handling should be described in more detail. Contributions are solicited.}

…

## 7.3	More than two H.248 "IP" Terminations per Context

See Table 3.

Table 3 – RTP Topologies versus RTCP Handling versus H.248 MG Models –
More than two H.248 "IP" Terminations per Context

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| III.1 | RTP Mixer (RTPM) | F-1,2,3,4,5,6 | (RTP, RTP, … RTP) | Media server |
| III.2 | RTP Video-Switch-MCU (RTPVSM) | F-1,2,3,4,5,6 | (RTP, RTP, … RTP) | Media server |
| III.3 | RTP RTCP-Terminating-MCU (RTPRTM) | F-1,2,3,4,5,6 | (RTP, RTP, … RTP) | Media server where each H.248 RTP Termination relates to the "RTP End system" topology |
| III.4 | RTP Multicast (RTPMU) | TBD (see clause 7.6) | TBD |  |
| III.4.1 | RTP Source Specific Multicast (RTPSSM) | TBD (see clause 7.6) | TBD |  |
| III.4.2 | RTP Any Source Multicast (RTPASM) | TBD (see clause 7.6) | TBD |  |

## 7.4	Others

See Table 4.

Table 4 – RTP Topologies vs RTCP Handling vs H.248 MG Models –
Others

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| IV.1 | Asymmetric topologies (RTPASY) | TBD (see clause 7.6) | (RTP, MUX, RTP … RTP) (TBD). | H.248 Contexts with internal H.248 MUX terminations … |
| IV.2 | Topology combinations | TBD (see clause 7.6) | (TBD). | Cascaded H.248 Contexts … |

## 7.5	Notes to the H.248 Connection Model

Names of terminations (TerminationID) may be arbitrarily chosen (see clause 6.2.2/[ITU-T H.248.1]), but termination naming schemes are usually given by H.248 profile specifications. Such H.248 profiles typically use the notation of "IP" for IP-based ephemeral terminations (e.g. in order not to exclude any possible IP applications, which might be concluded by a transport protocol or application protocol specific notation). This Recommendation uses "RTP" instead because there may be a difference between following connection models:

- (RTP, RTP)

  - MG is "RTP-aware", which enforces a transport-protocol aware mode of operation

  - MG may be media aware or media agnostic

- (IP, IP)

  - MG may be "RTP-agnostic", i.e., aware of the transport protocol (UDP), but unaware of higher protocol layers, thus RTP packets may transparently go through such an H.248 Context.

It could be noted that the termination name could carry an implicit semantic concerning configured RTP topologies and/or RTCP handling behaviour of a Context. However, this practise is not recommended. This Recommendation therefore recommends that the H.248.TerminationID should not be used to indicate a particular RTP topology.

## 7.6	Classes of RTCP related functions

The MG may be requested to support the following RTCP packets or RTCP related functions (Note: the list label is used in Tables 1 to 4):

F-1:	RTP node identifier: usage (or not) of MG-local CNAMEs (per MG?; per termination?);

F-2:	RTP source identifier: allocation (or not) of MG-local SSRCs per RTP session;

F-3:	RTCP protocol endpoint for basic services: origination/termination of RTCP packets (reports);

F-4:	RTCP packet forwarding: unmodified forwarding of RTCP packets (reports);

F-5:	RTCP packet modification: rewrite of RTCP packets;

F-6:	RTCP protocol endpoint for supplementary services: local generation of RTCP reports (such as for RTCP XR based applications);

F-7:	RTCP packet filtering: filtering of RTCP reports (e.g., either entire RTCP packet or just an individual RTCP report within a compound packet is filtered);

Additional functions may defined in the future.

## 7.7	Relation to modes of operations of IP-to-IP media gateways

The subject of "RTP topologies" affects all kinds of H.248 MGs, which are involved in RTP traffic processing, from PSTN residential/access/trunking MGs to media servers for large conferencing systems. Of particular interest is H.248 IP-to-IP media gateways (also known as border gateways, at the access/edge, core or interconnect/peering) network level due to RTP traffic processing related to NAPT, NAT-T, QoS support, policy control and/or media conversion functions, based on RTP topologies according to clause 7.2.

The basic packet processing behaviour by such MGs is called "mode of operation", leading to the aspect of possible interactions between RTP topologies and such modes of operations as defined for IP-to-IP media gateways.

There are already a number of such modes indentified for IP-to-IP media gateways, e.g. "media-format agnostic, transport protocol type aware", etc. For instance, [b-ETSI TR 183 068] provides an established categorization which could be summarized according to Figure 1.

Figure 1 (copy of Fig. G.3/[b-ETSI TR 183 068]): 
BGF modes - H.248 Local and Remote Descriptor: 
SDP "m="-line specification combinations

{Contributor’s note: G.729.1, H.264, V.152 may need to be added to the references.}

The modes of operation are related to the media description in the local and remote descriptors. The classification in Figure 1 is based on the SDP "m="-line only.

Recommendations for H.248 profiles for IP-to-IP gateways:

A particular profile may explicitly require support of a number of RTP topologies and a number of different modes of operations. There are following two situations:

- There are no further specification actions required in case of semantic unambiguity (e.g., no interactions are  identified). The correct RTP/RTCP processing behaviour could be clearly concluded by the MG from Context specifications, without explicit RTP topology control

- In case of possible interactions, unclear semantics and/or explicit RTP topology related behaviour, then it is recommended to consider explicit RTP topology control according to clause 10.

NOTE  – the profile may also need to provide further clarification on the derived MG behaviour in the above instance.

# 8	RTCP services

The purpose of this clause is to describe the handling of RTCP packets, dependent on the RTP topology and used RTCP services.

## 8.1	RTCP packet structure

Figure 2 illustrates the basic RTCP packet structure using an example of a compound RTCP packet.

Figure 2 - RTCP packet structure (Compound RTCP packet)

The RTCP packet structures need to be considered to identify key information elements (see next sub-clause).

## 8.2	Important RTCP information elements (from H.248 MG perspective)

Not every information element in RTCP packets is relevant from a H.248 perspective. However, there  are  some elements which demand careful consideration when processing RTP/RTCP traffic by H.248 MGs.

### 8.2.1	Information elements related to the identification of an RTP entity

There are primarily three information element types:

- SSRC (per originating RTP source),

- CSRC (for contributing RTP source), and

- SDES.

These elements define the spatial horizon of RTP media and RTCP control flows, i.e. the "end-to-end" significance of RTCP services. See for instance the detailed evaluation of example H.248 measurement architectures by [ITU-T H.248.48] and IETF monitoring architectures for RTP by [IETF RFC 6792].

### 8.2.2	Other relevant information elements

TBD

{Editor's note: there might be RTCP information elements of primary interests for RTP end-systems only …FFS.}

## 8.3	RTCP service levels

### 8.3.1	Basic RTCP services

Every RTP implementation must be compliant to [IETF RFC 3550] and thus basically support all the RTCP services defined by this RFC. They may be called basic RTCP services and cover the four RTCP packet types of SR (200), RR (201), SDES (202) and BYE (203).

NOTE - The code point for application defined RTCP packet types APP (204) is also introduced by this RFC, but the definition of application specific services is delegated to subsequent RFCs.

Basic RTCP services are indicated to the MG by an implicit or explicit request for RTCP port allocation (see [ITU-T H.248.57]) which leads to processing RTCP control flow traffic by an H.248 context.

### 8.3.2	RTP profile dependent RTCP services

RTP profiles may add additional RTCP services like e.g. the RTP profiles RTP/AVPF and RTP/SAVPF.

The use of RTP profile dependent RTCP services is typically indicated to the MG by the protocol field value in the SDP "m=" line, as part of the LD-/RD-embedded SDP media description.

…

# 9	Scope of RTP topology control

…

The scope of RTP topology control, - based on the H.248 rtpt package defined by this Recommendation -, is limited to basic (I) and RTP profile dependent (II) RTCP services.

Example:

The H.248 RTP stream in an H.248 IP-IP MG could be configured for RTP topology "Topo-Translator", which implies RTCP services such as

- no allocation of local SSRC values,

- no generation of RTCP RR reports, and

- forwarding of RTCP BYE packets.

The same H.248 RTP stream could additionally be used for H.248.48 type performance monitoring capabilities, which could lead to RTCP services such as

- allocation of local SSRC values for measurement points, or/and

- generation of RTCP XR reports.

The H.248.48 related supplementary RTCP services are not in contradiction to the RTP topology associated ones.

…

#### 10.6.3.1	Topology "Point-to-Point"

The MG shall provide an own, local CNAME.

The MG shall allocate local SSRC values per RTP session endpoint (which corresponds to a H.248 flow pair of RTP media and RTCP control traffic; often identical to the H.248 Stream endpoint (SEP)).

The MG shall originate/terminate RTCP packets of type:

- at least: "SDES", "SR", "BR" and "BYE"

[…]

…

#### 10.6.3.4	Topology "Mixer"

Not used.

…

# 11	Package-independent procedures for RTP topology control

## 11.1	Topology “RTP source translator”

### 11.1.1	Via SDP attribute "a=ssrc:"

The MGC shall use the SDP attribute "a=ssrc:" (see [IETF RFC 5576]) for the RTP source translator assignment of a particular Stream endpoint. Table 5 defines the allowed roles (i.e. SDP a=ssrc: value settings), given by the 4-tuple {Ta(LD), Ta(RD), Tb(LD), Tb(RD)}.

Table 5 – Indication of topology “RTP source translator”

| H.248 SEP |  | SDP attribute | “RTP source translator” |
| --- | --- | --- | --- |
| Ta | LD | a= ssrc: | wildcarded |
|  | RD | a= ssrc: | not sent |
| Tb | LD | a= ssrc: | not sent |
|  | RD | a= ssrc: | not sent |
| NOTE 1 – The semantic of "not sent" means that the MGC shall not include this SDP attribute in the H.248 Descriptor, despite the fact whether it is used on call signalling level. |  |  |  |

The RTP source translator function is requested to translate the RTP source value with following semantic:

- direction Ta to Tb: not any translation of RTP source parameter values;

- direction Tb to Ta: the received SSRC value in any kind of RTP packets is replaced by the locally used SSRC value for this H.248 SEP;

The MG shall reply with error code 449 in case of incorrect value settings.

…

Bibliography

[b-ETSI TS 183 068]	ETSI TS 183 068 v3.1.1, Telecommunications and Internet Converged Services and Protocols for Advanced Networks (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[b-IETF RFC 6184]	IETF RFC 6184 (2011), RTP Payload Format for H.264 Video.

[b-IETF RFC 6222]	IETF RFC 6222 (2011), Guidelines for Choosing RTP Control Protocol (RTCP) Canonical Names (CNAMEs).

[b-IETF RFC 6679]	IETF RFC 6679 (2012), Explicit Congestion Notification (ECN) for RTP over UDP.

[b-IETF rtp-howto]	IETF draft-ietf-payload-rtp-howto (2012), How to Write an RTP Payload Format.

[b-IETF rtp-topologies]	IETF draft-ietf-avtcore-rtp-topologies-update “RTP Topologies”

{Contributor’s note: This only appears in the living list appendix and would be removed at the time of Consent.}

…

### I.3.2	Example 2 “fully RTP protocol unaware forwarding”

Signalling according to Table I.3 should lead to a MG behaviour of “fully RTP protocol unaware forwarding”:

Table I.3 – Example 2

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=- - - - }, Remote { m=- - - - } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=- - - - }, Remote { m=- - - - } other Descriptors{<”RTP impacting elements Y/N?”>} …} | The so called BGW mode of operation (in TISPAN BGF, 3GPP IMS-AGW, 3GPP TrGW, MSF SBG) with the characteristic of: - media type agnostic - media format agnostic - transport protocol type agnostic and - L4 port agnostic. The “-“ notation was introduced by TISPAN in border gateway profiles. |

Discussion/conclusions of example 2:

- The MG would implicitly provide “fully RTP protocol unaware forwarding” if there would be not any other signalling elements related to RTP.

- The MGC knows whether the implicit signalling would be unambiguous or not (due to the indicated condition). The MGC may therefore provide an additional explicit indication in case of guaranteed transparent forwarding service requirements.

### I.3.3	Example 3 “additional RTCP awareness”

Example 3: same as example 2, but with additional property “rtcph/rsb = ON” (see [ITU-T H.248.57]) in the LocalControl Descriptor (LCD)

Discussion/conclusions of example 3:

- There’s an explicit indication of RTP bearer traffic, the MG becomes RTP aware.

- However, there are basically three meaningful RTP translator options (PP, TR and TF) with different impact on RTP packets, thus pure implicit signalling would be controversial.

- Unambiguous behaviour might be achieved by explicit assignment of PP or TR topology (with different RTCP handling, but common SSRC/CSRC modifications) in case of non-transparent forwarding.

- Transparent forwarding however could not be achieved via implicit signalling, requires rather the explicit assignment of the TF topology.

### I.3.4	Example 4 “fully specified SDP media description”

Example 4: same semantic aimed as with example 2, but fully specified SDP media description, rather than the “-“ option, see Table I.4:

Table I.4 – Example 4

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … }, Remote { m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … }, Remote { m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … } other Descriptors{<”RTP impacting elements Y/N?”>} …} | This option was added by 3GPP profiles (in comparison to TISPAN BGF). The MG has to compare all four SDP media descriptions, and if fully identical then to conclude a correspondent RTP topology behaviour. But which one exactly? |

Discussion/conclusions of example 4:

- This example should be functionally identical as example 2, i.e. leading to the same MG behaviour, but there are subtle difference between examples 2 and 4 when considering the dynamics of H.248 signalling.

- Problem: the final establishment of the H.248 stream might be subject of multiple signalling cycles (ADD, MOD, …). The MG normally already becomes RTP aware (even media aware) with the very first H.248 command request. But which topology? B2BRE, media translator, transport translator, transparent forwarding?

- The MG could of course wait till the stream is finally established, but that would be an unsatisfying situation for performance optimized implementations, which may want to achieve e.g. the minimization of call and bearer establishment delays.

- The explicit indication of the required RTP topology (MG, PP, TR or TF) in the first command request could solve the ambiguity.

…

### I.3.6	Example 6 “transparent forwarding of encrypted RTP traffic”

This use case relates to so-called end-to-end media security scenarios where an IP-IP MG is located in the middle of the end-to-end SRTP-enabled RTP session. There are H.248 profiles which use the “m=” line transport code point “RTP/SAVP” for the indication of SRTP traffic to the MG, see Table I.6:

…