| Question(s): | 3 |  | Meeting, date: |  | Geneva, 28th October – 8th November 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 1 | Intended type of document (R-C-TD): |  | C |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |  |  |
| Title: | H.248.TLSPROF: Updates |  |  |  |  |  |
| Contact: | Weiwei Yang Huawei Technologies China |  |  |  | Tel: +86 29 87674367 Fax: +86 29 87674151 Email: tommy@huawei.com |  |
| Contact: | Christian Groves NTEC Australia Australia |  |  |  | Tel: +61 3 9391 3457 Fax: +61 3 9391 3457 Email: Christian.Groves@nteczone.com |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

# Introduction

This Contribution proposes updates to H.248.TLSPROF "Guidelines on the use of H.248 capabilities for transport security in TLS networks in H.248 Profiles".

# Discussion

The Contributors have performed a review of the draft Recommendation and have identified a number of editorial corrections that should be addressed. There are also a number of technical issues that need to be addressed. These have been noted via MS Word “comments”.

# Proposed

It is proposed to accept the following updates. Changes are marked up against TD-15 from the June 2013 Q.3/16 Rapporteurs’ meeting in Olso.

Draft new ITU-T Recommendation H.248.TLSPROF

Gateway control protocol: 
Guidelines on the use of H.248 capabilities for transport security in TLS networks in H.248 Profiles

…

# 1	Scope

…

This Recommendation is organized as follows:

- examples of use cases are collected in Appendix II of [ITU-T H.248.TLS];

- principal requirements are subject of clauses  6 to 8, categorized in capabilities with regards to the mode of operation of a MG, TLS profile concepts and TLS protocol specific requirements;

and

- profile specification guidelines in clause 10 (which follows the profile structure of the H.248 profile template according to [ITU-T H.248.1]).

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.84]	Recommendation ITU-T H.248.84 (xxx), Gateway control protocol: xxx.

[ITU-T H.248.TLS]	Recommendation ITU-T H.248.TLS (xxx), Gateway control protocol: xxx.

[ITU-T X.509]	Recommendation ITU-T X.509 (2012), Information technology – Open systems interconnection – The Directory: Public-key and attribute certificate frameworks.

[IETF RFC 793]	IETF RFC 793 (1981), Transmission Control Protocol - DARPA Internet Program - Protocol Specification.

[IETF RFC 2712]	IETF RFC 2712 (1999), Addition of Kerberos Cipher Suites to Transport Layer Security (TLS).

[IETF RFC 3436]	IETF RFC 3436 (xxx), xxx.

[IETF RFC 4120]	IETF RFC 4120 (2005), The Kerberos Network Authentication Service (V5).

[IETF RFC 4279]	IETF RFC 4279 (2005), Pre-Shared Key Ciphersuites for Transport Layer Security (TLS).

[IETF RFC 4572]	IETF RFC 4572 (2006), Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP).

[IETF RFC 4583]	IETF RFC 4583 (2006), Session Description Protocol (SDP) Format for Binary Floor Control Protocol (BFCP) Streams.

[IETF RFC 5246]	IETF RFC 5246 (2006), The Transport Layer Security (TLS) Protocol Version 1.2.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

See TLS related terminology in [ITU-T H.248.TLS].

…

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| AN | Access Network |
| --- | --- |
| BFCP | Binary Floor Control Protocol |
| CA | Certification Authority |
| CN | Core Network |
| DB | Data Base |
| E2EA | xxx |
| IP | Internet Protocol |
| IWF | Interworking Function |
| Lx | Layer number |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| NAPT | Network Address and Port Translation |
| PSK | Pre-Shared Key |
| SCTP | Stream Control Transmission Protocol |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SHA | Secure Hash Algorithm |
| TCP | Transport Control Protocol |
| TLS | Transport Layer Security |
| UE | User Equipment |

…

# 6	Requirements for control the MG mode of operation

## 6.1	TLS transport mode

TLS is designed as a transport-independent protocol, just making the assumption of “reliable transport” of the underlying protocol (see clause 1/[IETF RFC 5246]). Support of one or multiple TLS transport mode(s) is required.

R-6.1/1: Transport mode “TLS-over-TCP”.

NOTE 1 – This requirement would be applicable for the very majority of use cases.

R-6.1/2: Transport mode “TLS-over-SCTP” [IETF RFC 3436]).

It may be noted that TLS protocol procedures need to be decoupled from protocol procedures of the underlying transport protocol.

## 6.2	MG mode of operation for case of TLS-over-TCP transport

…

R-6.2/3: Thus the presence of at least one TLS entity is a sufficient condition to require the TCP-proxy mode. Refer to Figure 2.

NOTE 1 - That there might be multiple entities present which require the TCP-proxy mode, e.g. in case both terminations of a Context are TLS-enabled, or if another application-aware interworking function performed by the MG.

NOTE 2 - The term “application” is widely used in TCP-related Contexts, but in this Recommendation it is used as a synonym for the term “media”.

…

The indication of the TCP-proxy mode is defined by [ITU-T H.248.84], the procedures to control a SEP in the TCP proxy modes are extended  by  [ITU-T H.248.TCP]).

…

## 7.1	Requirements to such a TLS profile

…

## 8.1	Selection of the TLS domain

A MG may be connected to multiple network domains with different TLS profiles. Figure 5 illustrates such a use case (with scope on TCP bearer traffic) using the example of an H.248 IP-IP MG located at the edge between access network (AN) and core network (CN) domains. The CN belongs to a trusted domain, hence transports unencrypted TCP traffic. Multiple different, TLS-enabled AN domains may be the result of multiple different access technologies, trust models, service provider models, etc.

…

### 8.2.1	TLS client/server role assignments

R-8.2.1/1: The MG is required to support both the TLS server and TLS client modes.

The TLS standards do not make any correlation between the initiation of the underlying transport connection and the TLS server/client mode. Thus this mode is decoupled from the (TCP) SDP “a=setup:” attribute, as used for TLS-based SDP indications (see clause 4/[IETF RFC 4572]).

R-8.2.1/2: The MGC is required to indicate the TLS server/client role to the MG.

NOTE – There is no existing TLS related SDP defined which represents TLS client/server semantic. [IETF RFC 4572] intentionally delegates such control information to the application (layer).

R-8.2.1/3: The MG is required to select the TLS client or TLS server role based on the MGC request.

### 8.2.2	Client/server role assignments to TLS transport protocols

R-8.2.2/1: Any IP transport protocol level client/server role assignment shall be independent of TLS server and TLS client mode.

…

##### 8.3.3.2.1	Applicability requirements

…

R-8.3.3.2.1/1: It is prohibited to use self-signed certificates when transport integrity can not be guaranteed.

R-8.3.3.2.1/2: Self-signed certificates can optionally be used at the H.248 interface when the integrity of the H.248 Control Association can be assured.

NOTE – This requirement is derived from “TLS-based SDP” usage from application control signalling, following the argumentation of clause 3.3/[IETF RFC 4572].

…

### 8.3.4	Pre-shared keys

…

R-8.3.4/5: In case the TLS-endpoint to be created is required to act as a TLS-server, the MGC can optionally provide an H.248 signalling element that indicates the PSK identity hint towards the MG.

### 8.3.5	Trusted third-party server

A third party server located in the same TLS-domain and trusted by both TLS-endpoints can be used to exchange keying information (Figure 8). For example the Kerberos Network Authentication Service as specified by [IETF RFC 4120] can be applied. In [IETF RFC 2712] additional TLS cipher suites are defined to support Kerberos based authentication.

How the TLS-endpoints interface with the trusted third party server and which information elements are exchanged between the TLS-point and the trusted third party server is out of scope of this Recommendation.

…

## 8.4	Renegotiation of security parameters

…

R-8.4/3: In case that during the renegotiation the authentication of the remote TLS-endpoint fails, the MG is requested to regard the remote endpoint as not authenticated and the related basic procedures shall apply.

R-8.4/4: In case the renegotiation fails, the MG is required to follow the basic error procedures.

## 8.5	Termination of the TLS/TCP session

### 8.5.1	Termination by remote TLS-endpoint

The termination of a TLS/TCP-session may be initiated by both TLS/TCP-endpoints. When terminating a session the initiating endpoint sends a TLS close_notify alert to the remote side.

R-8.5.1/1: On reception of a TLS close_notify alert the MG is required to close the TLS-session according to the procedures defined in [IETF RFC 5246] and notify the MGC indicating the reason for the TLS session termination.

…

### 8.5.3	Abnormal termination by local TLS-endpoint

R-8.5.3/1: In case the MG detects a condition which requires the TLS-session to be terminated, the procedures defined in [IETF RFC 5246] are required to be followed to terminate the TLS session and the MGC is required to be notified of the reason for the TLS session termination.

…

____________________

This use case is a variation of use case #1.4 from clause 6.1/[ITU-T H.248.TLS].