|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 0268 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | October 2013 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 2/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 0268 |  |  |  |  |  |
| Source: |  | Australia |  |  |  |
| Title: |  | H.323 Annex for ASSET work item updates |  |  |  |

# Summary

This document proposes changes to the H.323 Annex for ASSET work item created at the Rapporteurs Meeting in June 2013 in Oslo.

# Introduction

At the Rapporteurs Meeting in June 2013 in Oslo, AVD-4360 was presented and a new work item H.323 Annex for ASSET was agreed to be created. The proposed text was accepted as a baseline document. It was noted at that meeting.

[TD-08a]

This document contains a rough outline of the text that would comprise the new Annex to H.323 defining the Associative SET device type (ASSET).  It was agreed to create a new work item to define this new Annex to H.323.

# Discussion

The expert’s review has been noted and this contribution aims to further develop the rough outline and create the baseline text for the new Annex to H.323. It is proposed that this document form the baseline text for H.323 Annex for ASSET work item.

Annex ?

Associative simple endpoint type

(This annex forms an integral part of this Recommendation)

# ?.1	Introduction

An H.323 Associative SET device (ASSET) is a device within a H.323 network that is able to provide additional capabilities beyond those of any regular H.323 terminal devices. For instance, an H.323 device capable of only audio may connect with an H.323 Associative SET (ASSET) on another device to share a H.239 presentation within the context of a call initiated/received on the audio device or a file transfer application may be invoked on a separate device to send a file within the context of an on-going H.323 call.

## ?.2	Scope

The scope of this annex is to define ASSET functionality.

## ?.3	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.323]	Recommendation H.323 (2009), Packet-based multimedia communications systems.

[ITU-T H.225.0]	Recommendation H.225.0 (2009), Call signalling protocols and media stream packetization for packet-based multimedia communication systems.

[ITU-T H.235]	Recommendation H.235 (2003), Security and encryption for H-Series 
(H.323 and other H.245-based) multimedia terminals.

[ITU-T H.245]	           Recommendation H.245 (2011), Control protocol for Multimedia Communications.

[ITU-T H.460.1]	Recommendation H.460.1 (2002), Guidelines for the Use of the Generic Extensible Framework.

[ITU-T H.460.17]	Recommendation ITU-T H.460.17 (2005), Using H.225.0 call signalling connection as transport for H.323 RAS Messages.

[ITU-T H.460.18]	Recommendation ITU-T H.460.18 (2005), Traversal of H.323 signalling across network address translators and firewalls.

[ITU-T H.460.19]	Recommendation ITU-T H.460.19 (2005), Traversal of H.323 media across network address translators and firewalls.

[ITU-T H.460.23]	Recommendation ITU-T H.460.23 (2009), Network address translator and firewall device determination in ITU-T H.323 systems.

[ITU-T H.460.24]	Recommendation ITU-T H.460.24 (2009), Point to point media through network address translators and firewalls within ITU-T H.323 systems.

[ITU-T H.460.26]	Recommendation ITU-T H.460.26 (2009), Using ITU-T H.225.0 call signalling connection as transport for media.

[ITU-T H.46x.0]	     Recommendation H.46x.0 (201x), H.323 Associative Set (ASSET) Feature

## ?.4	Definitions

### ?.4.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

?.4.1.1 Simple Endpoint Type: As defined in Annex F.

### ?.4.2	Terms defined in this Recommendation

### ?.4.2.1	Application:  A real-time function which resides on an ASSET.

?.4.2.2 AssociationMode: The mode between the ASSET and other H.323 devices for the purpose of pairing and exchanging application information.

?.4.2.3 ApplicationMode: The mode between the ASSET and another ASSET for the purpose of invoking and running applications.

## ?.5	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

ASSET 		       Associative Simple Endpoint Type

FW			Firewall

NAT			Network Address Translator

RAS			Registration, Admission and Status

SET			Simple Endpoint Type

TCS			Terminal Capability Set

## ?.6	Requirements for Associative Simple Endpoint Type

The Associate Simple Endpoint Type (ASSET) shall advertise an H.225.0 endpointType of SET (bit 2 set) in all H.225.0 RAS and Call Signalling messages.

The ASSET shall have full H.225.0 RAS support to enable it to register to a gatekeeper and appear as a regular SET endpoint with an H.225.0 AliasAddress. The ASSET shall be reachable on the supplied H.225.0 AliasAddress.

There is no requirement for the ASSET to register to a gatekeeper however unless the device resides in a globally accessible location where NAT/FW are not a consideration, for instance on a public IP Address, it is recommended that the ASSET register to a gatekeeper. The ASSET shall support NAT/FW standards H.460.18/.19/.23/.24 and optionally H.460.17/.26 when registered.

In general the requirements for an ASSET can be divided into two parts: AssociationMode and ApplicationMode.

### ?.6.1 	AssociationMode requirements

AssociationMode is the mode in which an endpoint pairs with an ASSET and exchanges information about, and requests to invoke, applications that reside with the ASSET.

The ASSET shall be reachable by a supporting endpoint by using H.225.0 Call Signalling and shall operate with a conferenceGoal set to callIndependentSupplementaryService and shall only responds to a SETUP message set accordingly. Where the conferenceGoal does not match then it shall be evaluated as possibly being in ApplicationMode refer clause ?.6.2.

AssociationMode shall not support the full range of H.225.0 Call Signalling. It shall only support SETUP, CONNECT, FACILITY and RELEASECOMPLETE messages. Only the genericData field of the H323-UU-PDU is required. The h323-message-body and all other optional fields shall not be supported and be omitted. The h245Tunneling field shall be set to false and H.245 is not supported.

### ?.6.2 	ApplicationMode requirements

In ApplicationMode the ASSET shall operate as a regular H.323 endpoint with the exception for the requirement to support mandatory H.245 capabilities. The calling party shall support H.46x.0 as a neededFeature. If the feature is not present then the call shall be rejected with a RELEASECOMPLETE with the reason set to genericData.

The H.245 capabilities contained in the TerminalCapabilitySet (TCS) or FastConnect shall be limited to the capabilities required to perform the requested Application.

## ?.7	Procedures for Associative Simple Endpoint Type

Implementation of the Associative Simple Endpoint Type feature is defined in H.46x.0

_________________