|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 0299 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | October 2013 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 5/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 0299 |  |  |  |  |  |
| Source: |  | ZTE Corporation |  |  |  |
| Title: |  | H.TPS-AV: Proposes to update the description parameter |  |  |  |

1.	Introduction

This contribution proposes updates the description parameter.

2.	Discussion

There are three different mentions of description attribute in [IETF CLUE FW]: in the section   6.1.1. Media Capture Attributes, 6.2.1. Capture Scene attributes and 6.2.2. Capture Scene Entry attributes. The scene description in section 7.1.1.1 of the current H.TPS-AV is only defined to support the textual description for the overall scene.

In order to comprehensively define the description parameter aligning with CLUE, changes should be made.

3.	Proposal

It is proposed to incorporate the following changes in draft H.TPS-AV.

Changes are marked up against TD-25a_H_TPS_AV from the June ITU-T Q5/16 interim meeting.

[Begin Proposal]

### 7.1.1 General Parameters

#### 7.1.1.1 Description

| Identity: | Description |
| --- | --- |
| Description: | This parameter provides a human-readable description of the media captures, capture scenes and capture scene entries. Multiple languages may be included in the description. |
| Format: | List of Strings. |
| Possible Values: | A list of <description text> and <language tag> may be provided. |
| Default: | - |
| Reference: | See the 'Description' attributes in 6.1.1, 6.2.1, 6.2.2 in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | Describing the media captures, capture scenes and capture scene entries using text in multiple languages may assist in the consumer being able to make a more informed decision about which one to choose in the event that multiple contents are offered. |

[End Proposal]

_________________