|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 0301 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | October 2013 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 5/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 0301 |  |  |  |  |  |
| Source: |  | ZTE Corporation |  |  |  |
| Title: |  | Proposed initial input draft: “Signaling for Telepresence Systems” |  |  |  |

# 1.	Introduction

This contribution proposes to create the new work item “Signaling for Telepresence Systems” discussed at the last interim meeting in Oslo, as well as an initial baseline text for it.

# 2.	Discussion

The proposal to open a new work item to start the work of the signaling for H.323 based telepresence system was first proposed at the last rapporteur meeting in Oslo. And the new work item needs a baseline document supposed to be supported in this meeting. It was noted at the Q5/16 meeting report TD-10 (2013-06 Oslo).

| AVD-4458 | Proposed to start the work on Telepresence system signaling | ZTE Corporation |
| --- | --- | --- |

There was general agreement and support for this proposal.

Contributions on signalling topics are solicited.  Experts noted that some useful topics might be proposals for a signalling framework, RTP usage, protocol usage, potential signalling channels, and analysis of approaches to integration the CLUE advertisement/configure methodology with H.323 capability exchange and negotiation.

# 3.	Proposal

It is proposed to create the new work item and accept the following text as a start for the initial draft: “Signaling for Telepresence systems”.

[Begin Proposal]

CONTENTS

1 	Scope	3

2 	References	3

3 	Definitions	4

4 	Abbreviations and acronyms	4

5 	General framework	5

6 	Detailed procedures	5

6.1	Call setup	5

6.2	Basic capability exchange	5

6.2.1	Telepresence indication	5

6.2.2	Multiplex information	7

6.2	Open Logical Channel	8

6.3	Telepresence capability exchange	8

6.3.1 Use CLUE channel	8

6.3.2 Use the existing H.245 control channel	9

6.4	Reopen logical channel	10

6.4.1	Multiplex mapping	10

7 	Examples	11

7.1	Open a CLUE channel	11

7.2	Use the existing H.245 control channel	16

8 	Interworking	18

Draft new ITU-T Recommendation H.TPS-SIG

Signaling for Telepresence Systems

AAP Summary

<To be provided before Consent>

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1 	Scope

This recommendation describes the signaling procedures of H.323 based telepresence systems and also explores the solution for the key issues including the channel to carry the telepresence capability, the exchange of the telepresence indication and the multiplex information.

# 2 	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.323]			ITU-T Recommendation H.323 v7 (12/2009), Packet-based multimedia communications systems.

[ITU-T H.245]			ITU-T Recommendation H.245 v16 (05/2011), Control protocol for multimedia communication.

[ITU-T H.225.0]		ITU-T Recommendation H.225.0 v7 (12/2009), Call signalling protocols and media stream packetization for packet-based multimedia communication systems.

[IETF CLUE FRAMEWORK]	IETF draft-ietf-clue-framework-11, Framework for  Telepresence Multi-Streams.

[IETF CLUE SIGNALING] 	IETF draft-kyzivat-clue-signaling-05, CLUE Signaling.

# 3 	Definitions

3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	endpoint: An H.323 terminal, Gateway, or MCU. An endpoint can call and be called. It generates and/or terminates information streams.

3.1.2	H.245 control channel:  Reliable Channel used to carry the H.245 control information messages (following Rec. ITU-T H.245) between two H.323 endpoints.

3.1.3	channel: A channel is a unidirectional link between two endpoints.

3.1.4	logical channel: A logical channel is a unidirectional path or bidirectional path for the transmission of information.

3.1.5	capability: A terminal has a particular capability if it is able to encode and transmit or receive and decode that particular signal.

3.1.6	CLUE channel: A reliable, bidirectional, transport mechanism used to convey CLUE messages. A CLUE channel consists of one SCTP stream in each direction over a DTLS/SCTP session.

3.2	Terms defined in this Recommendation

3.2.1	basic capability: A set of capability especially related to codec capability.

3.2.2	telepresence capability: Additional to basic capability includes the description of the capture source, description of the capture scene, description of the encoding group and the simultaneous capability, etc.

3.2.3	telepresence indication: Characterises for Telepresence endpoint such as the endpoint type, the information for new channel created to transport the telepresence capability,etc.

3.2.4	multiplex information: The multiplex information includes the multiplex indication, the multiplex method and the multiplex mapping.

3.2.5	multiplex indication: The multiplex indication indicates whether or not the telepresence endpoint support streams multiplex transmission.

3.2.6	multiplex method: The multiplex method indicates the method that the telepresence endpoint exchanges the multiplex mapping information .

3.2.7	multiplex mapping: The multiplex mapping describes the mapping relation between the media capture streams and the transport streams.

3.2.8	legacy endpoint: A legacy endpoint is the H.323 conferencing endpoint without any telepresence extended capability.

# 4 	Abbreviations and acronyms

CLC		closeLogicalChannel

OLC		openLogicalChannel

OLCAck 	openLogicalChannelAck

TCS		TerminalCapabilitySet

TCSAck	TerminalCapabilitySetAck

# 5 	General framework

The video conferencing technology based on the current H.323 definition can only use the H.245 protocol to handle basic codec capability negotiation. It can’t negotiate some additional features for telepresence systems including media capture, capture scene, individual encoding and encoding group, etc. It also can’t control the multiple media streams to support streams multiplex transferring.

In order to solve these issues and compatibility with the legacy H.323 endpoint as well as SIP based telepresence system, it needs to make the secondary negotiation to separate the new telepresence capability negotiation from the existing basic capability negotiation.

As shown in figure 1, the signaling framework of telepresence systems need to add two parts based on the current Rec. ITU-T H.323 call signalling framewrok. One is Telepresence Capability Exchange and the other is Reopen Logical Channel. Telepresence Capability Exchange is the secondary capability negotiation for the new telepresence capability of the telepresence endpoint. Reopen Logical Channel is to reopen the logical channel according to the result of the secondary negotiation.

Figure 1 signaling framework of telepresence systems

# 6 	Detailed procedures

This section describes the detailed procedures of the telepresence call signaling framework.

## 6.1	Call setup

Call setup part uses the current procedure defined in H.323 and messages defined H.225.0.

## 6.2	Basic capability exchange

After call setup the two telepresence endpoints establish the call session and build H.245 control channel. Then an initial endpoint capability exchange should take place using the H.245 terminalCapabilitySet message via H.245 channel to exchange the basic capability, the telepresence indication and also the multiplex information if necessary.

### 6.2.1	Telepresence indication

The telepresence endpoints exchange the telepresence indication information so as to know the information about the opposite endpoint whether it is a telepresence endpoint or not and then to decide how to execute the following procedures.

The telepresence indication includes information exchange such as the endpoint type, the information of the new channel created to transport the telepresence capability.

The endpoint type represents the endpoint is a telepresence endpoint or not.

If a new channel is needed for the telepresence capability exchange such as CLUE channel, during this basic capability exchange the telepresence endpoint should carry the information for creating the new channel. The information includes transport protocol, IP address and port number, etc.

The missing of the telepresence endpoint indication represents the endpoint is not a telepresence endpoint.

The telepresence endpoint makes the telepresence indication in the capability exchange part. It can make the telepresence indication based on ITU-T H.245 terminalCapabilitySet message using the GenericParameter in the GenericCapability or GenericInformation structure.

As shown in figure 2, Endpoint 1 (calling endpoint) and Endpoint 2 (called endpoint) are both the telepresence endpoint.

- Endpoint 1 sends the TCS message contained the telepresence indication information to Endpoint 2 to make the telepresence indication.

- Endpoint 2 receives and parses the TCS message from Endpoint 1, and then knows that Endpoint 1 is a telepresence endpoint. Endpoint 2 sends terminalCapabilitySetAck message to Endpoint 1.

- Endpoint 2 sends the TCS message contained the telepresence indication to Endpoint 1 to make the telepresence indication.

- Endpoint 1 receives and parses the TCS message from Endpoint 2, and then knows that Endpoint 2 is a telepresence endpoint. Endpoint 1 sends terminalCapabilitySetAck message to Endpoint 2.

After the telepresence indication exchange, the telepresence endpoints can start the telepresence capability exchange according to the result of the telepresence indication exchange.

Figure 2 the telepresence indication in Capability Exchange

### 6.2.2	Multiplex information

If the telepresence endpoint selects to use the multiplex method to transfer media streams, the telepresence endpoint should exchange the multiplex information in the signaling each other, so as to know the information about the multiplex method.

The multiplex information includes the multiplex indication information, the multiplex method information and the multiplex mapping information.

The multiplex indication information indicates whether or not the telepresence endpoint adopt of the multiplex method.

The multiplex method information indicates the method that the telepresence endpoint exchanges the multiplex mapping information, such as static mapping, dynamic mapping and combination mapping mentioned in.

The multiplex mapping information describes the mapping relation between the media capture streams and the transport streams. This information can be exchanged after telepresence capability negotiation which will be discussed in the section 6.4.1.

The missing of the multiplex information represents that the multiplex transferring will not be used.

The telepresence endpoints can exchange the multiplex indication information and multiplex method information in the basic capability exchange part based on ITU-T H.245 terminalCapabilitySet message using the GenericParameter in the GenericCapability or GenericInformation structure.

As shown in figure 3 Endpoint 1 (calling endpoint) and Endpoint 2 (called endpoint) are both the telepresence endpoint.

- Endpoint 1 sends the TCS message contained the multiplex information to Endpoint 2 to make the multiplex indication.

- Endpoint 2 receives and parses the TCS message from Endpoint 1, and then knows that Endpoint 1 selects the multiplex transferring and the multiplex method. Endpoint 2 sends the TCSAck message to Endpoint 1.

- Endpoint 2 sends the TCS message contained the multiplex information to Endpoint 1 to make the multiplex indication.

- Endpoint 1 receives and parses the TCS message from Endpoint 2, and then knows that Endpoint 1 selects the multiplex transferring and the multiplex method. Endpoint 1 sends the TCSAck message to Endpoint 2.

After exchanges the multiplex information t, the telepresence endpoints can transport the media streams according to the result of the multiplex information exchange.

Figure 3 the multiplex indication in Capability Exchange

## 6.2	Open Logical Channel

After the basic capability exchange, the procedure of ITU-T H.245 open logical channel shall then be used to open the basic logical channel in order to send a single audio and/or video stream based on the negotiation result. Whether or not to send this initial media before telepresence capability exchange is implementation-dependent.

## 6.3	Telepresence capability exchange

After the endpoints know each other are telepresence endpoint, a telepresence capability exchange will be necessary to negotiate the new telepresence capability. The telepresence parameters include the description of the capture source, description of the capture scene, description of the encoding group and the simultaneous capability etc.

Two methods can be used to accomplish the telepresence capability exchange. One is to open a new channel such as IETF CLUE channel, to exchange the telepresence capability. The other is to use the existing H.245 control channel to exchange the telepresence capability.

### 6.3.1 Use CLUE channel

If a CLUE channel is opened to exchange the telepresence capability, the endpoints should get the information to create the CLUE channel first, and then create the CLUE channel through which to make the secondary telepresence capability exchange.

As shown in the figure 4, the overall procedures will be like this below.

- After the call setup procedures, the H.245 control channel between the two telepresence endpoints is established.

- The two endpoints exchange the basic capability set via the existing H.245 control channel. The capability exchange messages contain the information to create the CLUE channel including the transport protocol, IP address and port number etc.

- After the basic capability exchange, the two endpoints open logical channels, and get the information to create the new CLUE channel.

- The two endpoints create the CLUE channel through which to make the secondary capability negotiation.

- The two endpoints make the secondary capability negotiation to exchange the telepresence capability through the CLUE channel.

- The two endpoints reopen the bidirectional media logical channels for multiple media streams transferring based on the result of the secondary capability negotiation.

- The media streams transferred via the opened bidirectional media logical channels.

In this solution we can reuse the CLUE channel mechanism and also the advertisement/configure messages defined in IETF CLUE.

Figure 4 open a CLUE channel to exchange telepresence capability

### 6.3.2 Use the existing H.245 control channel

If the existing H.245 control channel to exchange the telepresence capability is used, the telepresence capability should be packaged in ITU-T H.245 terminalCapabilitySet message, and then the endpoints should make the secondary capability negotiation through H.245 Capability exchange messages such as terminalCapabilitySet, terminalCapabilitySetAck.

As shown in the figure 5, the overall procedures will be like this below.

- After the call set procedures, the H.245 control channel between the two telepresence endpoints is established.

- The two endpoints exchange the basic capability set via the existing H.245 control channel.

- After the basic capability exchange, the two endpoints open the logical channel.

- The two endpoints exchange the TCS message which is packaged with the telepresence capability to make the secondary telepresence capability negotiation via the existing H.245 control channel.

- The two endpoints reopen the logical channels for the multiple media streams to transfer based on the result of the secondary capability negotiation.

Figure 5 Use the existing H.245 control channel to exchange telepresence capability

There are two methods to package the telepresence capability in the terminalCapabilitySet message.

First is to treat the entire telepresence capability parameters based on XML format defined in IETF CLUE and also aligned in H.TPS- AV document as one string packaged using a suitable field in the terminalCapabilitySet message and transfer to the other endpoint.

Second is to package these telepresence capability parameters separately into different fields in the terminalCapabilitySet message according to the H.245 syntax structure.

If the endpoints use the existing H.245 control channel to exchange the telepresence capability there is no need for the endpoints to open a new channel and no more TCP ports are used.

## 6.4	Reopen logical channel

After the telepresence capability exchange, the telepresence endpoint should reopen the logical channel based on the result of the secondary capability negotiation.

### 6.4.1	Multiplex mapping

As mentioned in the section 6.2.2 the multiplex mapping information describes the mapping relation between the media capture streams and the transport streams.

If the telepresence endpoint selects to use multiplex transferring, it should carry the multiplex mapping information in the signaling message (statistic mapping), or in the media stream transport (dynamic mapping), or both (combination mapping).

In the dynamic mapping, the telepresence endpoint exchanges the multiplex mapping information through extended RTP header during the media stream transport. The dynamic mapping is not directly related with H.323 based protocol so we can refer the method from IETF.

In the statistic mapping, the telepresence endpoint carries the multiplex mapping information in the signaling message. After the basic capability exchange and the telepresence capability exchange, the endpoints can carry the multiplex mapping information which describes the relations between the media capture streams that the opposite endpoint requests and the RTP streams it prepares to transmit through the reopen logical channels as below.

The telepresence endpoint can use the genericInformation in openLogicalChannel message to describe the multiplex mapping information.

As shown in figure below, Endpoint 1 and Endpoint 2 are both the telepresence endpoint.

- Endpoint 1 and Endpoint 2 reopen the logical channel. Close the existing logical channel first, and then reopen the logical channel according to the result of the telepresence capability exchange.

- Endpoint 1 sends the OLC message contained the mapping relationship between the media capture streams that Endpoint 2 selected and the RTP streams.

- Endpoint 2 sends the OLCAck to Endpoint 1.

- Endpoint 2 sends the OLC message contained the mapping relationship between the media capture streams that Endpoint 1 selected and the RTP streams.

- Endpoint 1 sends the OLCAck to Endpoint 2.

Figure 6 the multiplex mapping in Reopen Logical Channel

# 7 	Examples

Below are two examples to describe the overall procedure of the telepresence call signaling flows between two telepresence endpoints. Case 1 is an example to open a CLUE channel to exchange the telepresence capability. Case 2 is an example to use the existing H.245 control channel to exchange the telepresence capability.

Endpoint 1 has 4 cameras and 3 displays. Endpoint 2 has 2 cameras and 2 displays.

## 7.1	Open a CLUE channel

The first example uses a new CLUE channel to exchange the telepresence capability. The figure below describes the whole procedures.

Figure 7 example for open a new control channel

## 7.2	Use the existing H.245 control channel

The second example uses the existing H.245 control channel to exchange the telepresence capability. The primary parts are the same with the first example except Telepresence Capability Exchange part. The figure below is only the different part compared with the front figure 7.

Figure 8 example for using the existing H.245 control channel

# 8 	Interworking

As the figure shown below, Endpoint 1 is a Telepresence endpoint and Endpoint 2 is a legacy endpoint.

- Endpoint 1 sends the TCS message contained the Telepresence indication information to Endpoint 2.

- Endpoint 2 can’t recognize the extension parameters, omit the telepresence indication and sends the TCSAck message to the calling endpoint.

- Endpoint 2 sends the TCS message without the telepresence indication to Endpoint 1.

- Endpoint 1 receives the TCS message, parses the message and knows that the opposite endpoint is not a telepresence endpoint. So the calling endpoint will not start the procedure owned by the telepresence including the telepresence capability exchange and the reopen logical channel parts. The calling endpoint sends the TCSAck message to the called endpoint.

- After the capability exchange the two endpoints open the logical channel and start to transfer the media streams.

Figure 9 Interworking with Legacy endpoint

[End Proposal]

_________________