| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4483 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.SEPLINK: challenging the Interlinkage design |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution discusses further interlinkage examples in order to check supported and unsupported use cases.

1	Introduction

The interlinkage package provides a fairly powerful tool concerning MG delegated autonomous behaviour. The range of property values is not really strictly limited, hence the package as such could be applied in scenarios beyond the ones currently discussed. The purpose of this contribution is to stress the current package design.

2	Discussion

There are at least three directions, which were not yet evaluated in more detail with respect to the question of useful interlinkage applications:

Interlinkage in context of

- self-referential transport protocol endpoints (see clause 2.1),

- tunneled (IP) protocol stacks (see clause 2.2) and

- extended (IP) protocol stacks (see clause 2.3).

2.1	Self-referential interlinkage?

2.1.1	TCP bidirectional closure

Use case:

An incoming TCP half-closure procedure shall immediately trigger an outgoing TCP half-closure procedure (besides the usual acknowledgement of the incoming procedure) in order to achieve a bidirectional TCP bearer connection release (see Figure 1).

Figure 1 – Example #1 of type "self-referential interlinkage": TCP connection release

Discussion:

Such an interlinkage is of type self-referential due to an a) intra-SEP interlinkage and b) idendity of source transport and interlinked transport endpoints (here TCP). Such an interlinkage type should be feasible in general, but questionable in case of concrete protocol TCP and the "half-release" procedure.

Table d.1 – Interlinkage description – Use case example #1

| <interlinked SEP> | <source transport EP> | <interlinked tranport EP> | <mode> | Comment |
| --- | --- | --- | --- | --- |
| "Ta" | "TCP" (at Ta/Si) | "TCP" (at Ta/Si) | "?" | Not yet any mode value for "unidirectional release" |

Conclusion for H.248.SEPLINK:

The current property parameter mode value "rel" relates to the semantic of a "usual connection release", which is a "bidirectional release". There isn't yet a mode value correspondent to "unidirectional connection release" semantic (see Table d.1). If there would be such additional mode value, then the tcpecc/ori property of H.248.TCP (half-closure vs. autonomous bi-directional closure) could be replaced by a seplink/linktopo configuration.

However, looking at the pros and cons, we don't propose an interlinkage extension because
a) TCP half-closure seems be to a fairly exotic case, and
b) the default property values of tcpecc/ori ("bidirectional") and seplink/linktopo 'rel' ("no interlinkage, thus unidirection") are in contradiction (in present draft package designs).

2.1.2	TCP: immediate release after establishment

Use case:

An incoming TCP establishment request (stimuli "TCP SYN") shall immediately trigger a TCP bearer connection release (see Figure 2). This is a protocol compliant reaction, inline with RFC 793. Such a reaction might be an alternative in cases where the MGC did initially "block" TCP establishment.

Figure 2 – Example #2 of type "self-referential interlinkage": TCP immediate release after establishment

Discussion:

Such an interlinkage is again of type self-referential. Such an interlinkage type should be feasible in general, but questionable when looking at current communication services supported by H.248 systems.

Table d.2 – Interlinkage description – Use case example #2

| <interlinked SEP> | <source transport EP> | <interlinked tranport EP> | <mode> | Comment |
| --- | --- | --- | --- | --- |
| "Ta" | "TCP" (at Ta/Si) | "TCP" (at Ta/Si) | "?" | Not yet any mode value "establishment trigger release" |

Conclusion for H.248.SEPLINK:

The current property parameter mode provides not yet any value for such a scenario. Protocol conform connection release as reaction of an incoming establishment request is a valid scenario in general, but not yet identified as a required use case for concrete IP services (such as TCP-based data services).

Hence, not any addition of a correspondent mode value proposed.

2.2	Interlinkage in tunneled (IP) protocol stacks

Use case:

Figure 3 illustrates a generic tunneled IP protocol stack. An incoming L4 protocol procedure (within the protocol stack segment realizing the "tunnel") shall immediately trigger an outgoing L4 protocol procedure above the "tunnel".

Figure 3 – Example #3 of type "interlinkage in tunneled (IP) protocol stacks"

Discussion:

This is a valid interlinkage scenario in general.

Table d.3 – Interlinkage description – Use case example #3

| <interlinked SEP> | <source transport EP> | <interlinked tranport EP> | <mode> | Comment |
| --- | --- | --- | --- | --- |
| "Ta" | "L4_low?" (at Ta/Si) | "L4_high?" (at Ta/Si) | "…" | Such an transport EP naming is not (yet) supported. FFS. |

Conclusion for H.248.SEPLINK:

Table d.3 highlights the problem. Tunnel may lead to "IP-over-IP", "TCP-over-TCP" or "X-over-X" in general. The present interlinkage package does not (yet) support to qualify the correspondent protocol endpoints. This topic is for further studies.

Thus, not any value extensions for linktopo parameters "sourceTransportEP" and "interlinkedTransportEP" proposed.

2.3	Interlinkage in extended (IP) protocol stacks

Use case:

Figure 4 illustrates an interworking scenario in H.248 WebRTC gateways for the WebRTC data channel. The WebRTC data channel stack "SCTP/DTLS/UDP" is converted to native IP data transport "TCP". E.g., the WebRTC data channel establishment shall autonomously trigger the establishment of the TCP connection.

Figure 4 – Example #4 of type "interlinkage in extended (IP) protocol stacks"

Discussion:

This is a valid interlinkage scenario in general.

Table d.4 – Interlinkage description – Use case example #3

| <interlinked SEP> | <source transport EP> | <interlinked tranport EP> | <mode> | Comment |
| --- | --- | --- | --- | --- |
| "Tb" | "SCTP" (at Ta/Si) | "TCP" (at Tb/Si) | "est" | Would be a valid, unambi-guous interlinkage configuration. |

Conclusion for H.248.SEPLINK:

The concrete WebRTC use case would be already supported, under the condition that value "SCTP" is supported (see discussion in AVD-4482).

3	Conclusions

Purpose of this contribution is to challenge current package design in order to emphasize present limitations. Generally we think that the package as such should be as flexible as possible, - and the limitations on concrete use cases would be then under responsibility of H.248 profile specifications. However, there are already some scenarios, as discussed, which would be beyond the defined capabilities of seplink package version 1.

Such use cases are for further studies, and may be scope of future revisions of H.248.SEPLINK.

Furthermore: the illustration of interlinkage scenarios might be useful for first readers of this Recommendation. A correspondent new Appendix is proposed to be added to H.248.SEPLINK, see proposal.

4	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4477).

Change proposal #1:

[Begin Proposed Correction]

Appendix I

Illustration of interlinkage capabilities

(This appendix does not form an integral part of this Recommendation)

This Appendix illustrates the interlinkage concept, the principal interlinkage types and indicates some use cases which are beyond current interlinkage capabilities.

## I.1	Illustration of concept

Figure I.1 summarizes the used terminology and the principal elements of the seplink/linktopo property.

Figure I.1 – Stream endpoint assigned property linktopo and their parameters

The linktopo property is always associated with a specific stream endpoint (here SEP "Ta/Si").

## I.2	Illustration of supported use cases

## Three principal interlinkage types may be differentiated.

### I.2.1	Use case "Inter-SEP interlinkage"

The interlinkage across two terminations is called "inter-SEP interlinkage" (Figure I.2).

Figure I.2 – Use case "Inter-SEP interlinkage"

Table I.1 summarizes the principal interlinkage description:

Table I.1 – Interlinkage description – Use case "Inter-SEP interlinkage"

| <interlinked SEP> | <source transport EP> | <interlinked tranport EP> | <mode> | Comment |
| --- | --- | --- | --- | --- |
| "Tb" | "EPn+1" (at Ta/Si) | "EPn" (at Tb/Si) | "…" |  |

### I.2.2	Use case "Intra-SEP interlinkage"

There are two variants of intra-SEP interlinkage:

### I.2.2.1	External driven "Intra-SEP interlinkage"

The interlinked transport endpoint may be located at the same SEP as the source transport endpoint, called "intra-SEP interlinkage" (Figure I.3).

Figure I.3 – Use case External driven "Intra-SEP interlinkage"

Table I.2 summarizes the principal interlinkage description:

Table I.2 – Interlinkage description – Use case "Intra-SEP interlinkage"

| <interlinked SEP> | <source transport EP> | <interlinked tranport EP> | <mode> | Comment |
| --- | --- | --- | --- | --- |
| "Ta" | "EPn" (at Ta/Si) | "EPnn+1" (at Ta/Si) | "…" |  |

#### I.2.2.2	Internal driven "Intra-SEP interlinkage"

There is following difference between the previous external driven intra-SEP interlinkage case and the interna-driven one:

An MG-internal stimuli (see Figure I.4), - either orignating in the MGC (0a) (i.e., related to H.248 signalling) or in the MG itself due to a previous interlinkage scenario (0b) -, triggers an external outgoing procedure (1). The internal stimuli (2) will be generated, as usual, when the source transport endpoint is successfully transitioning its state.

Figure I.4 – Use case Internal driven "Intra-SEP interlinkage"

The interlinkage description is identical to the external driven "Intra-SEP interlinkage" (i.e., according to Table I.2). The history before is just a different scenario.

### I.2.3	Use case "Multi-SEP interlinkages"

A single stimuli may be used to trigger multiple outgoing bearer procedures (of the same or different type). The interlinked transport endpoints may be located at the same or different SEP(s). Figure I.5 illustrates such an example.

Figure I.5 – Use case "Multi -SEP interlinkage"

The linktopo property "value" represents a list structure (Table I.3):

Table I.3 – Interlinkage description – Use case "Multi-SEP interlinkage"

| <interlinked SEP> | <source transport EP> | <interlinked tranport EP> | <mode> | Comment |
| --- | --- | --- | --- | --- |
| "Tb" | "EPn+1" (at Ta/Si) | "EPn" (at Tb/Si) | "…" |  |
| "Tc" | "EPn+1" (at Ta/Si) | "EPn" (at Tc/Si) | "…" |  |

## I.3	Controversial use cases

### I.3.1	Use cases with "self-referential interlinkage"

The case of intra-SEP interlinkage with identical source transport and interlinked transport endpoints is called self-referential interlinkage.

Such an interlinkage seems to be controversial in general concerning the question which incoming bearer control procedures trigger immediate outgoing control procedures? What would be correspondent mode values?

There were not any useful self-referential interlinkage scenarios identified for seplink package version 1.

### I.3.2	Use cases  with "tunneled (IP) protocol stacks"

The usage of tunneling is e.g. a characteristic of many IP access network technologies.The notion of tunnel refers to hierarchical protocol stacks of type e.g. "X-over-IP-over-X-over-IP",

Such an interlinkage seems to be controversial in general concerning the unambiguous identification of source transport and/or interlinked transport endpoints due to the existing supported value range by seplink package version 1. E.g., the indication of "TCP" (in case of a TCP-over-TCP stack) would be ambiguous. Such scenarios are for further studies.

### I.3.3	Use cases  with "extended (IP) protocol stacks"

Extended protocol stacks are also beyond the layered protocol architectures according to basic reference models, but the interlinkage is not necessarily between identical protocol types (as in above tunneled scenarios).

The support of such use cases is already in scope of seplink package version 1, but conditional. The usage depends on the unambiguous identification of source transport and/or interlinked transport endpoints.

[End Proposed Correction]

____________________