| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4484 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP: update of summary and scope sections |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides updates for scope and summary sections.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4473).

Change proposal:

[Begin Proposed Correction]

AAP Summary

This Recommendation provides multiple H.248 packages for support of TCP, complemented by models, package-less considerations and signalling flows.

Summary

TCP is a connection-oriented IP transport protocol, which leads to specific support for H.248-controlled TCP bearer endpoints in H.248 media gateways (MG). Such support covers external bearer control procedures related to the establishment and release of TCP connections, as well as the MG internal interworking of TCP packets. This Recommendation provides multiple H.248 packages for support of TCP, complemented by models, package-less considerations and signalling flows.

# 1	Scope

In scope of this Recommendation are H.248 IP media gateway TCP-based bearer interfaces. Such an H.248 MG provides various functions for processing of TCP/IP packets, TCP payload data and is involved in functions related to TCP protocol control information (based on TCP header flags). This Recommendation focuses primarily on the aspect of TCP connection control, which comprises the establishment and release of TCP bearer connections.

The Recommendation considers

- connection models with a single or multiple TCP bearer connection endpoints within an H.248 context;

- establishment, through-connection and release behaviour in case of end-to-end TCP bearer connections across two H.248 TCP terminations ("a so-called H.248 TCP media stream");

- different H.248 control models.

Further, this Recommendation provides information on existing H.248 tools with respect to other TCP support functions.

## 1.1	Applicability statements

Table 1 summarizes all possible TCP-based interfaces of H.248 entities, - under the assumption of an underlying IP network -, and their relevance for this Recommendation.

Table 1 – Principal TCP-based interfaces of H.248 entities 
and their relevance for this Recommendation

| TCP-based transport | H.248 entity | Relevance |
| --- | --- | --- |
| Call control interface (e.g., SIP) | MGC | Out of scope of this Recommendation. |
| Gateway control interface (H.248) | MGC, MG | Out of scope of this Recommendation. Possible H.248 transport modes are indicated by [ITU-T H.248.67]. Usage of a TCP-based H.248 transport mode would be typically specified by an H.248 profile (as part of clause 6.12 in the profile definition template (see Appendix III in [ITU-T H.248.1])). |
| Bearer interface | MG | Within the scope of this Recommendation. |

## 1.2	Package-less TCP connection control

Figure 1 summarizes the methods for H.248-based TCP connection control (at the MG bearer interface). The general approach (besides SDP elements) is based on package-defined H.248 properties by this Recommendation. However, in simple and limited network environments the package-less option may be sufficient.

Figure 1 – Overview: Methods for H.248-based TCP connection control

Usage of dedicated SDP elements is principally not excluded in package-based TCP connection control. However, such SDP elements should be avoided which could semantically overlap or lead to ambiguities with H.248.

[End Proposed Correction]

____________________