| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4485 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP: Move clause 7 "Relation to similar H.248 packages" into an Appendix |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to move the entire clause 7 to a new Appendix V.

Motivation

Support of TCP by Recommendation H.248.TCP is not a self-contained, isolated technology. There is some history of TCP work and other Recommendations on connection-oriented bearer technologies which provide a frame for H.248.TCP. Purpose of clause 7 is to provide such an inventory and information for first readers. This is useful information.

However, the text as such isn't normative, rather subject of an Appendix.

Furthermore:

- there are still some pending editor's notes (which are resolved by this contribution) and

- some references may need to be moved then from clause 2 to the Bibliography (still to be done).

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4473).

Change proposal #1:

[Begin Proposed Correction]

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix V

Relation to similar H.248 packages

(This appendix does not form an integral part of this Recommendation.)

## V.1	Overview of TCP related packages

There are some existing packages which are either defined for TCP or could be theoretically reused for TCP, see Figure V.1.

Figure V.1 – Overview of TCP related packages

{Editor's note (2013-10 meeting): above Figure is not yet consistent with the text and need to be updated …(to be done before the 2014-06 SG16 plenary)}

The specific relation to this Recommendation is described below.

## V.2	ITU-T Q.1950

This Recommendation introduces 11 H.248 packages for usage in so-called Bearer-Independent Call Control type of network architectures. Four out of the eleven could be principally applied (with extensions) for TCP networks:

### V.2.1	Package 'gb' (generic bearer connection):

TCP is a connection-oriented protocol which implies a connection state model (see Figure 6 in [IETF RFC 793]) in the bearer connection endpoints. This is a common characteristic with the gb package (Table V.1):

Table V.1 – Q.1950-defined H.248 packages – gb package

| Package name | Purpose |
| --- | --- |
| Generic bearer connection package | The gb package provides control elements for such stateful bearer technologies, which implies basic stimuli for the bearer control signalling protocol as well as a basic connection state concept. The bearer connection endpoint models supports also incoming and outgoing bearer control protocol procedures, which is e.g. required for the modelling of active/passive open (or close) procedures or client/server behaviour. |
| Possible relation to this Recommendation: |  |
| The TCP connection state model could be basically mapped on the underlying gb package state model. The gb package could be used for the majority of TCP connection control procedures. Open/differences: - TCP explicitly supports simultaneous bearer connection establishment and release, which would be so called collision scenarios in case of the gb package. - TCP bearer connection release: unidirectional concept (called half close), whereas the gb package considers bi-directionality only. |  |

### V.2.2	Package 'bcp' (bearer characteristics):

The notion of bearer characteristics relates to a particular (or multiple) protocol layer(s) in the bearer plane (see Table V.2):

Table V.2 – Q.1950-defined H.248 packages – bcp package

| Package name | Purpose |
| --- | --- |
| Bearer characteristics package | The existing bcp package supports an initial set of bearer technologies such as RTP/IP for IP-based networks. However, bearer type "TCP" is not supported. |
| Possible relation to this Recommendation |  |
| Usage of the gb package would imply the need for an explicit bearer type indication such as supported by the bcp package. Open/differences: - A new version or an extension package with additional codepoints would be required (at least bearer type "TCP", but possibly also "TLS", "TLS/TCP", "DTLS", "SCTP", "DCCP"). - Instead of the Property-based bearer type indication (via bcp package) at LCD level exists the alternative of the SDP-based bearer type indication (via "m=" line element proto) at LD/RD level. - Use case TLS-over-TCP: the establishment/release of the TCP connection and the TLS session could be basically controlled via the the gb package. The bearer connection control procedures at TCP and TLS level could be basically temporarily decoupled, which would imply the modification of the bcp/BNCChar property value, a scenario originally out of scope of the bcp package. |  |

### V.2.3	Package 'bnct' (bearer network connection cut through):

See Table V.3:

Table V. – Q.1950-defined H.248 packages – bnct package

| Package name | Purpose |
| --- | --- |
| Bearer network connection cut through package | The MG can indicate whether the cut through will occur "early" or "late", which are effectively related to bearer control protocol signalling events. Early relates to the bearer cutting-through on the establishment. Late refers to cutting-through on the confirmation. |
| Possible relation to this Recommendation |  |
| The aspect of "cut-through" control is basically relevant for TCP as well, perhaps even more important due to a) the assured transport characteristic of TCP (e.g., handling of acknowledgments, loss, retransmission) and H.248 MG TCP modes (such as relay, merge or proxy). Questions for clarification: - The explicit cut-through semantic might be also an implicit capability of TCP modes. - Unclear is the interaction with StreamMode property settings? Understanding: The "BNC Cut through" package doesn't actually control cut through, i.e. from clause A.4.5/[ITU-T Q.1950]: The MGC can ask the MG as to when the cut through of a particular bearer will occur, through the use of the BNCCT property. The MG can indicate whether the cut through will occur "early" or "late". Early relates to the bearer cutting-through on the establishment. Late refers to cutting-through on the confirmation. The BNCCT property in this package does not actually order the cut through nor does it indicate when the Cut-through has occurred. Furthermore, clause 4.4.10/[ITU-T Q.1950] BNC-cut-through-capability: Used by the bearer interworking functio (BIWF) to inform the call state machine (CSM) of the bearer cut-through capability (i.e. commits resources on the receipt of a Bearer SetupReq or confirm). The transactions in [ITU-T Q.1950] show that the usage depends on the BICC call setup direction i.e. forward direction. See also clause 7.2.1.2.2/[b-ITU-T Q.1902.4]. There’s also no interaction with H.248 StreamMode settings. See [b-ITU-T Q.Supp.32] for call flows showing the interaction. There’s also more of a discussion on the use of the Cut-through capability in clause 5.3.2 step 9. Conclusion: No relation to this Recommendation. |  |

### V.2.4	Package 'ri' (reuse idle):

See Table V.4:

Table V.4 – Q.1950-defined H.248 packages – ri package

| Package name | Purpose |
| --- | --- |
| Reuse idle package | The ri/RII property is used by the MG to indicate to the MGC that an idle bearer is to be reused. |
| Possible relation to this Recommendation: |  |
| Reuse of existing TCP connections is a supported concept, such as in SIP networks by [IETF RFC 4145.] See also clause 13.6/[ITU-T H.248.84]. Open/differences: - Possible models of MG-level TCP resources, e.g.: - resource component "TCP local source/destination transport address"; - resource component "TCP local statefull bearer connection endpoint" which would imply the TCP connection state machine; - resource component "end-to-end TCP bearer connection", based on the n-tuple of local and remote transport source/destination addresses and connection state; - others? - The semantic of reuse idle bearer would be then dependent on the agreed TCP resource model. It might be even transparent for the MG. Understanding: Reuse of idle procedures are a BICC Network option. Annex B/[b-ITU-T Q.1902.4] describes the overall procedures. MLPP (multi-level precedence and preemption) is a service that supports the reuse of IDLE bearers (i.e [b-ITU-T Q.850], cause 9). [b-IETF RFC 4411] discusses the use of this cause with respect to SIP and basically says that it’s not valid in an IP environment. The Reuse idle package is used within the framework of a call control that supports an application (i.e. MLPP) that has re-use logic. The RII property is used in conjunction with the BNC-ID etc to choose an existing bearer (i.e. bearer independent situations). Whereas the SDP "a=connection" attribute is really a flag to use an existing bearer because you don't want to release it rather than selecting an idle TCP connection i.e. can be used in “bearer dependent” situations. Conclusion: Whilst it is possible to reuse a TCP connection, there is not any real relation of this Q.1950 package to this Recommendation. |  |

## V.3	ITU-T H.248.67

### V.3.1	Package 'trm' (GCP transport mode indication):

See Table V.5:

Table V.5 – H.248.67-defined H.248 package – trm package

| Package name | Purpose |
| --- | --- |
| GCP transport mode indication package | Related to the H.248 Control Association, which may use TCP-based transport modes. |
| Possible relation to this Recommendation |  |
| None: [ITU-T H.248.67] and this Recommendation are both diametrically opposed. TCP usage for the H.248 signalling interface is out of scope of this Recommendation. |  |

## V.4	ITU-T H.248.84

The TCP enabled SEP will be activated through the indication of the TCP-proxy mode as defined in [ITU-T H.248.84]. This Recommendation only defines properties that are needed on top of [ITU-T H.248.84] for the control of a TCP enabled SEP.

This Recommendation does not define TCP-related statistics and events which are already covered by [ITU-T H.248.84]. Whenever needed the statistics and events of the related package as defined by [ITU-T H.248.84] must be used.

### V.4.1	Package-independent procedures for NAT-T with TCP bearers:

See Table V.6:

Table V.6 – H.248.84-defined package-independent procedures for NAT-T with TCP bearers

| Procedures | Purpose |
| --- | --- |
| Clause 13/[ITU-T H.248.84] | a) Indication of IP transport protocol 'TCP' b) Indication of 'TCP mode' for H.248 MG |
| Possible relation to this Recommendation |  |
| To a): - Normative for SDP-based bearer type indication "TCP". To b): - The TCP merge mode could be used as NAT traversal service. The TCP modes as such could interact with TCP connection control. E.g., the tcpcc package would be not required (and should be not used) for TCP connection establishment when NAT-T is used (because it provides already end-to-end TCP connection establishment (Note: TCP connection release is different). |  |

### V.4.2	Package 'tcphp' (TCP hole punching):

See Table V.7:

Table V.7 – H.248.84-defined H.248 package – tcphp package

| Package name | Purpose |
| --- | --- |
| TCP hole punching package | NAT traversal support service for TCP. |
| Possible relation to this Recommendation |  |
| Orthogonal to this Recommendation; the tcphp package could be used in conjunction with this Recommendation. |  |

### V.4.3	Package 'tcptv' (TCP traffic volume metrics):

See Table V.8:

Table V.8 – H.248.84-defined H.248 package – tcptv package

| Package name | Purpose |
| --- | --- |
| TCP traffic volume metrics package | Support of octet and packet count statistics for TCP bearer protocol. |
| Possible relation to this Recommendation |  |
| Performance monitoring is an orthogonal application; hence, the H.248 statistics could be basically used in conjunction with this Recommendation. |  |

### V.4.4	Package 'tcpccm' (TCP connection control metrics):

See Table V.9:

Table V.9 – H.248.84-defined H.248 package – tcpccm package

| Package name | Purpose |
| --- | --- |
| TCP connection control metrics package | Statistics related to the establishment process of the TCP bearer connections. |
| Possible relation to this Recommendation |  |
| Basically all three statistics could be used. |  |

### V.4.5	Package 'tcpcqm' (TCP connection quality metrics):

See Table V.10:

Table V.10 – H.248.84-defined H.248 package – tcpcqm package

| Package name | Purpose |
| --- | --- |
| TCP connection quality metrics package | Additional statistic related to the establishment process of TCP bearer connections. |
| Possible relation to this Recommendation |  |
| The round-trip time based statistic may be used as well. |  |

### V.4.6	Inventory of TCP functions and their possible impact on H.248 TCP gateways:

See Table V.11:

Table V.11 – H.248.84– Inventory of TCP functions

| Inventory in: | Purpose: |
| --- | --- |
| Appendix II/[ITU-T H.248.84] | - TCP Functions versus H.248 TCP Modes of Operation, in ten areas: - Basic Capabilities - Topology Hiding - Protocol Encryption - Security - Application Data Inactivity Detection - Interactions with other policy rules - Performance measurements & statistics" - Connection keep alive support - Add-on's to TCP - IP layer operations |
| Possible relation to this Recommendation: |  |
| This informative analysis is principally valid as well. |  |

## V.5	ITU-T H.248.43 and ITU-T H.248.79

[ITU-T H.248.43] and [ITU-T H.248.79] are related to the filtering of TCP traffic. See overview in clause 9/[ITU-T H.248.79].

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Bibliography

[b-ITU-T H.248.86]	Recommendation ITU-T H.248.86 (2013), Gateway control protocol: H.248 Support for deep packet inspection.

[b-ITU-T Q.850]	Recommendation ITU-T Q.850 (1998), Usage of cause and location in the Digital Subscriber Signalling System No. 1 and the Signalling System No. 7 ISDN user part.

[b-ITU-T Q.1902.4]	Recommendation ITU-T Q.1902.4 (2001), Bearer independent call control protocol (Capability Set 2): Basic call procedures.

[b-ITU-T Q.2630.1]	Recommendation ITU-T Q.2630.1 (1999), AAL type 2 signalling protocol.

[b-ITU-T Q.Sup31]	Technical Report TRQ.2141.0 (12/2000): Signalling requirements for the support of narrow-band services over broadband transport technologies - Capability set 2 (CS-2).

[b-ITU-T Q.Sup31]	Technical Report TRQ.2141.1 (11/2002): Signalling requirements for the support of narrowband services via broadband transport technologies - CS-2 signalling flows.

[b-ETSI TS 183 068]	ETSI TS 183 068 v3.1.1, Telecommunications and Internet Converged Services and Protocols for Advanced Networks (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[b-IETF RFC 4411]	IETF RFC 4411 (2006), Extending the Session Initiation Protocol (SIP) Reason Header for Preemption Events.

[b-IETF RFC 5925]	IETF RFC 5925 (2010), The TCP Authentication Option.

[b-IETF RFC 6528]	IETF RFC 6528 (2012), Defending against Sequence Number Attacks.

[b-Bellovin1989]	Morris, R. (1989), "Security Problems in the TCP/IP Protocol Suite", Computer Communications Review, Vol. 19, No. 2, pp. 32 - 48.

[b-CPNI-TCP]	Centre for the Protection of National Infrastructure (CPNI) (2009), "Security Assessment of the Transmission Control Protocol (TCP)", 
URL: http://www.gont.com.ar/papers/tn-03-09-security-assessment-TCP.pdf

[End Proposed Correction]

____________________