| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4486 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP: completion of TCP basic connection control package (clause 8 & Appendix III) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes changes based on the resolution of all final comments and editior's notes in clause 8.

1	Introduction

H.248.TCP is technically already fairly stable, apart from some final issues to be fxed.

2	Discussion

2.1	Blocking establishment requests

There are two comments:

- clause 8.1.1: comment to codepoint "tcpbcc/bcep = blocked" => "Implicit overwrite would violate H.248.1, thus not allowed"

- clause 8.1.1: Editor's note:

{Editor's note (2013-06 meeting): following items were identified and need to be resolved:

C1: the property semantics are focusing on the ingress direction (tbc); i.e., when property not relevant in outgoing direction, then explicit text should be added (note 2013-10: the outgoing direction was/is considered due to the sepplink option).

C2: Signal EstBNC should implicitly overwrite the property value, i.e., replacing blocked by unblocked. Correspondent updates should be added.

Comment (2013-10): You cannot have a signal that changes the value of a property.

Given that the Signal would have to be sent we don’t think it’s an issue to send a property “unblocking” the termination and sending the establishment.

The interaction between sending the Estab or Release signal whilst in state “blocked” would have to be described.

Also what is the effect if the SEP has a TCP connection state of Established and the blocking property is suddenly set to “blocked”.

Solution might be: signal usage implies parallel, explicit property value modification-

NOTE – The semantics of this property needs to be considered from use cases with combined tcpbcc and tcpecc usage. (E.g., scenario Appendix IV.3.2 where SEP T2(S1) would be in "blocked" state, etc.)}

Resolution:

Following assumptions are made:

- MGC must explicitly unblock (sent "tcpbcc/bcep = unblocked"). E.g., when requesting establishment by "tcpbcc/EstBNC"

- The blocking property takes only effect in H.248 connection states Idle & Blocked, but not in state Established. I.e., When in state Established, then the TCP connection must be firstly released (=> state Idle) before new incoming TCP connection establishment requests could be blocked.

- When in state Blocked, then correspondent H.248 command requests (such as signal EstBNC) shall be replied by an error code.

2.2	Package procedures

{Editor's note (2013-01): description of procedures needs to be updated …}

Resolution: review and update proposals

2.3	TCP endpoint role

Clause 8.6.2:

{Editor's note (2013-10 meeting): the "TCP endpoint role" would be coupled with "TCP endpoints", which are related to the two MG IWFs of a) TCP-to-TCP as "TCP proxy" and b) TCP-to-non-TCP. However, more important is whether the MG has to provide a "TCP OPEN" procedure or not. Present situation:
I)TCP to non-TCP: yes, either TCP active or passive OPEN;
II) TCP to TCP in TCP relay: no; 
III) TCP to TCP in TCP merge: yes, but only TCP passive OPEN; 
IV) TCP to TCP in TCP proxy: yes, either TCP active or passive OPEN;
The TCP role is then given by the TCP OPEN type.
The entire clause 8.6 should be revisited and possibly revised again..}

Resolution: correspondent text and table inserted

2.4	TCP endpoint role

Clause 8.6.2 – electronic comment:

8.6.3 mentions the results of the establishment for client and server roles including that g/cause can be used to indicate errors. In either 8.6.1 or 8.6.2 we should mention that a H.248 command where the MGC changes the state to IDLE should also include setting the g/cause and tcpbcc/bncchange events.

Resolution: Paragraph added at the end of clause 8.6.2.

2.5	StreamMode

Clause 8.6.4.1:

{Editor's note (2012-09 meeting): Consensus required on this behaviour.
Comment (2013-06 meeting): the principle problem relates to the fact that TCP allows to piggyback control and data information, i.e., an individual TCP/IP packet may not always be classified solely as part of the TCP data flow (-> H.248 media flow) or TCP control flow (-> H.248 control flow).
Reminder: H.248.1/7.1.7.1.1 StreamMode affects only H.248 media flow, but not H.248 control flow traffic.
Comment (2013-10 meeting): the problem as such is not specific to TCP, hence a more general discussion opened (see C-265) and anchored to H.248.79 (because relevant for other packet-oriented protocols). The semantical behaviour in Table 19 seems to be OK, whether the entire "TCP packet" is MG internally forwarded/processed or only protocol control information may need to be detailed.).}

Resolution: the proposed behaviour by Table 19 was already accepted. A central place for documenting StreamMode effects on packet bearer technologies wasn't agreed (yet) (as proposed by C-265). This might be unfortunate because we'll face the same interaction issues in H.248.SCTP (StreamMode versus SCTP chunk types which are representing procotol control information, see AVD-4583).

Here: Editor's note deleted.

3	Proposal

There are a number of changes in clause 8.

Further Appendix III is considered too because counterpart of clause 8. However, there are not any technical changes in Appendix III, apart from some minor editorial Visio related corrections in the Figures.

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4473).

Change proposal #1:

[Begin Proposed Correction]

# 8	TCP basic connection control package

|  | Package Name: | TCP basic connection control package |
| --- | --- | --- |
|  | Package ID: | tcpbcc (0x####) |
|  | Description: | This package provides the functionality to establish and release a TCP bearer connection. NOTE – The package design follows conceptually the generic bearer connection (gb) package according to clause A.6/[ITU-T Q.1950] by using an event-/signal-based approach for connection establishment/release control. Exceptions: a) bearer modification is not required for TCP; b) bearer release: uses a package-defined codepoint instead of element g/cause. |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

### 8.1.1	Bearer connection establishment blocking

|  | Property Name: | Bearer connection establishment blocking |
| --- | --- | --- |
|  | Property ID: | bceb (0x0001) |
|  | Description: | This property defines the MG behaviour before and during TCP bearer connection establishment. NOTE 1: See Annex A concerning a state model based illustration. NOTE 2: This capability might be relevant for the prevention of other TCP processing behaviour such as "TCP relay" or "TCP merge" modes (see [ITU-T H.248.84]). |
|  | Type: | Enumeration |
|  | Possible values: | See Table 15 Table 15 – Bearer connection establishment preparedness (of TCP endpoints) Value; MG behaviour. "blocked" (0x0001); - Bearer resources shall be prepared (i.e., resources for TCP endpoint shall be reserved and allocated to H.248 termination/SEP); - TCP role assignment: still open, not yet decided; - TCP connection state = "CLOSED" (Figure A.1); - H.248 counterpart state = "Blocked" (Figure A.3); - Any incoming TCP packet shall be silently discarded (e.g., TCP-SYN requests shall be not acknowledged), there is no state transitioning from "CLOSED" (NOTE 3). - any outgoing TCP bearer connection control procedures shall be not affected (i.e., also not due tointernal interlinkage (see clause 9)). - H.248 command requests which may not be executed in state Blocked shall be replied by an error code.. "unblocked" (0x0002); - MG shall be ready for incoming or/and outgoing TCP bearer connection control procedures; - TCP role assignment: relates to an (initial) TCP server role effectively ("TCP Passive Open") (NOTE 4); - TCP connection state: transitioning from "CLOSED" to "LISTEN" (Figure A.1); - H.248 counterpart state = "Idle" (Figure A.3); - TCP bearer connection control options: - Incoming TCP-SYN: TCP server role confirmed, 3-way handshake proceeded; - Signal EstBNC (clause 8.3.1): start 3-way handshake by sending of TCP-SYN; TCP client role assigned.. NOTE 3 - This semantic seems to be similar to SDP codepoint "a=setup:holdconn", see [IETF RFC 4145]. NOTE 4 - This semantic could be correlated with SDP codepoint "a=setup:actpass", see [IETF RFC 4145].. |
|  | Default: | "Unblocked" (due to be consistent with default behaviour as in [ITU-T H.248.84]). NOTE - H.248 profile specifications could redefine other default values. Examples: H.248 profiles for - MGs with support of "TCP proxy" mode only may choose default value "blocked" due to fully separated establishment procedures towards both remote TCP endpoints; - MGs with additional support of "TCP relay" and "TCP merge" modes may choose default value "unblocked" due to in order not to miss any incoming TCP connection establishment requests ("early media"). |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 8.2	Events

### 8.2.1	TCP connection state change ("BNC change")

|  | Event Name: | TCP connection state change |
| --- | --- | --- |
|  | Event ID: | BNCChange (0x0001) |
|  | Description: | This event occurs whenever a change to a (TCP) bearer network connection occurs. For example a bearer has been established or a bearer has been released. The event is related to a simplified state transitioning model, - in contrast to [IETF RFC 793] -, which is described in Annex A. NOTE – The event design is aligned with event gb/BNCChange according clause A.6.2.1/[ITU-T Q.1950] by using an event-/signal-based approach for connection establishment/release control. |

#### 8.2.1.1	EventsDescriptor parameters

##### 8.2.1.1.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. |
|  | Type: | Sub-list of Enumeration |
|  | Optional: | Yes |
|  | Possible values: | Table 16 defines the protocol semantic per value: Table 16 – Semantic of parameter Type Value; Name; Final state (Figure A.2); Semantic. Est [0x01]; Bearer Established; Established; Notify MGC successful TCP bearer establishment.. Rel [0x05]; Bearer Released; Idle; Notify MGC successful TCP bearer release.. NOTE 3 – The numerical codepoint assignments are aligned with [ITU-T Q.1950].. |
|  | Default: | Detect ALL |

#### 8.2.1.2	ObservedEventsDescriptor parameters

##### 8.2.1.2.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | This is used to indicate what change has occurred to the TCP bearer connection. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | Est [0x01] Bearer Established Rel [0x05] Bearer Released |
|  | Default: | None |

## 8.3	Signals

The signals are related to a simplified state transitioning model, in contrast to [IETF RFC 793], which is described in Annex A.

### 8.3.1	Establish (BNC)

|  | Signal Name: | Establish BNC |
| --- | --- | --- |
|  | Signal ID: | EstBNC (0x0001) |
|  | Description: | This signal triggers the bearer control function to start bearer establishment (i.e., this signal is used to initiate the TCP bearer connection establishment: the MG takes the TCP client role, sends a TCP SYN segment and completes the TCP active OPEN ("three-way handshake" procedures). This signal takes only effect in H.248 connection state Idle (and would be replied by an error code when in H.248 connection states Blocked or Established). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 8.3.1.1	Additional parameters

None.

### 8.3.2	Release (BNC)

|  | Signal Name: | Release BNC |
| --- | --- | --- |
|  | Signal ID: | RelBNC (0x0002) |
|  | Description: | This signal triggers the bearer control function to send bearer release (i.e., this signal is used to initiate the TCP closure procedure(s): the MG sends a TCP FIN segment and completes the associated handshake procedure). This signal takes only effect in H.248 connection state Established (and would be replied by an error code when in H.248 connection states Blocked or Idle). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 8.3.2.1	Additional parameters

None.

## 8.4	Statistics

None.

## 8.5	Error Codes

None

## 8.6	Procedures

### 8.6.1	TCP endpoint creation

The MGC is responsible for creating a TCP connection enabled SEP in the MG. This is indicated via the H.248 interface through the use of the SDP "m=" line according the rules as specified in clause 13.4 in [ITU-T H.248.84].

The MG shall reserve "TCP bearer resources" (such as a "TCP connection state machine") and allocates them to the SEP. The TCP endpoint role (client/server) is not (yet) determined.

NOTE 1 – This functionality is synonymous to the "bearer type indication" semantic as defined by the BNC Characteristics property (BNCChar) of the bearer characteristics package (BCP), see clause A.3.1.1/[ITU-T Q.1950].

### 8.6.2	TCP endpoint role assignment

The TCP endpoint role, - client or server -, is primarily relevant during the establishment phase of communication (of perspective of H.248 gateways). However, a TCP endpoint role depends whether a TCP-enabled H.248 termination/SEP provides an actual TCP endpoint representation, which again depends to the TCP mode of operation. Table x1 provides a summary:

Table x.1 – State models versus TCP modes of operation

|  |  | TCP mode |  | TCP endpoint role |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| a) |  | TCP to non-TCP interworking |  | either TCP ACTIVE or PASSIVE OPEN |  | Because complete protocol termination of TCP. |
| b) |  | TCP to TCP in TCP relay mode |  | none |  | Because TCP bearer connection establishment packets transparently forwarded (stateless). |  |
| c) | TCP to TCP in TCP merge mode | only TCP PASSIVE OPEN | Because of an end-to-end TCP simultaneous open emulation. |
| d) | TCP to TCP in TCP proxy mode | either TCP ACTIVE or PASSIVE OPEN | Because of an end-to-end TCP simultaneous open emulation. |

The TCP endpoint role is then given by the TCP OPEN type, i.e. the MGC shall basically prepare  the MG for an incoming or outgoing TCP bearer connection establishment (clause 8.6.3) procedure. This incoming or outgoing establishment leads to an implicit TCP endpoint role assignment. Table 17 summarizes MGC actions and their results in TCP endpoint role assignments:

Table 17 – TCP endpoint role assignment (at establishment phase)

| No. | TCP role | H.248 indication | MG behaviour: |
| --- | --- | --- | --- |
| 1 | Not yet assigned | H.248 SEP state "Blocked", either due to explicit signalling of bceb property or correspondent default value | Any incoming TCP packet shall be silently discarded. |
| 2 | TCP server | H.248 SEP state "Idle" by "unblocking" the SEP | Expect incoming TCP SYN, subsequent completion of 3-way handshake. |
| 3 | TCP client | H.248 SEP state "Idle" plus signal EstBNC enabled. | Start of 3-way handshake for TCP connection establishment. |
| 4 |  |  |  |

The MGC may arm events tcpbcc/BNCChange and g/cause (in H.248 connection state Idle) in order to be notified about successful or unsuccessful TCP bearer connection establishment.

### 8.6.3	TCP bearer connection establishment

The incoming and outgoing side of TCP bearer connection protocol procedures is given by the assigned TCP role (see above clause 8.6.2) by the MGC.

The MG shall initiate the TCP connection setup according to the indicated role and the TCP connection setup handshake is performed by the MG without any further involvement of the MGC. The end of the TCP bearer connection establishment phase may be summarized according Table 18:

Table 18 – TCP bearer establishment - Results

| TCP role | Result of TCP bearer connection establishment | MGC notification? |
| --- | --- | --- |
| TCP client (i.e., outgoing establishment side) | Successful | MGC dependent on subscription of event and parameter value tcpbcc/BNCChange{Type=[Est]} . |
|  | Unsuccessful | Unsuccessful establishment may be indicated to the MGC via a correspondent ServiceChange procedure or event g/cause. |
| TCP server (i.e., incoming establishment side) | Successful | As per client side. |
|  | Unsuccessful | As per client side. |

### 8.6.4	TCP application data transfer

#### 8.6.4.1	MG external

The TCP bearer connection is ready for application data transfer when remote and local TCP endpoints are both transitioned to TCP connection state "ESTAB".

The StreamMode property of the LocalControl Descriptor affects the application data flow rather than the TCP control information used to establish or close the TCP connection. The requested MG behaviour is defined by Table 19:

Table 19 – Impact of StreamMode on TCP bearer traffic at external MG interface

| Traffic direction | StreamMode settings | MG behaviour: |
| --- | --- | --- |
| Outgoing | Sending enabled (i.e., values SendOnly, SendRecv, LoopBack) | All TCP packets sent |
|  | Sending disabled (i.e., values RecvOnly, Inactive) | Only TCP packets with active usage of TCP SYN, FIN, RST and PSH information are sent, independent of optional piggybacked data are sent. |
| Incoming | Receiving enabled (i.e., values RecvOnly, SendRecv, LoopBack) | All TCP packets accepted |
|  | Receiving disabled (i.e., values SendOnly, Inactive) | Only TCP packets with active usage of TCP SYN, FIN, RST and PSH information are accepted, independent of optional piggybacked data are sent. |

TCP provides the application the capability to affect the data application transfer through the PSH- and the URG-flag. The usage of these flags is application specific and therefore must be specified in the application specific procedures.

#### 8.6.4.2	MG internal

This clause is only relevant for H.248 contexts with TCP SEPP(s), as outlined in clause 6.2.3.

Thus application data may be received at a SEP in time where the other SEP of the context is not yet able to process or send the application data. The behaviour of the MG in such a situation is implementation and/or application dependent. If application independent or unambiguous behaviour would be required, then the interlinkage capability as profiled by the tcpecc package (see clause 9.1.1) should be used.

### 8.6.5	TCP bearer connection release

#### 8.6.5.1	Characteristic of TCP bearer connection release

This package supports bidirectional TCP bearer connection release only. Consequently, two TCP half-close procedures shall be performed, resulting in a four-way handshake (see e.g. clause III.4). The transitioning from H.248 state Established to Idle of the local TCP bearer connection endpoint is only done after the successful processing of both TCP half-close procedures.

#### 8.6.5.2	Assignment of incoming or outgoing TCP bearer connection release

##### 8.6.5.2.1	Outgoing side

The local TCP bearer connection endpoint shall be explicitly prepared for outgoing TCP bearer connection release via signal RelBNC by the MGC.

##### 8.6.5.2.2	Incoming side

The local TCP bearer connection endpoint shall be able to process incoming TCP bearer connection release requests (TCP-FIN packet) at any point in time (whilst in H.248 state Established).

#### 8.6.5.3	MG bearer plane procedures

##### 8.6.5.3.1	MGC initiated release (outgoing side)

The RelBNC signal is used to initiate Bearer Release signalling (Table 20).

Table 20 – MG bearer plane procedures (outgoing side)

| MG bearer plane procedure | Result | MGC notification? |
| --- | --- | --- |
| Starts first TCP half-close by sending TCP-FIN. Awaits TCP-FIN-ACK and TCP-FIN of second TCP half-close. Subsequent confirmation by a TCP-FIN-ACK. Transition to connection state H.248 Idle (TCP 'CLOSED'). | Successful | The MGC default assumption is a "successfully released bearer connection" however it may be explicitly notified via the optionally BNCChange event, ObservedEventsDescriptor parameter codepoint "Type=Rel". |
|  | Unsuccessful | There are following options: - Via event g/cause (NOTE 1) - Via error code. - Via event adid/ipstop (see [ITU-T H.248.40]) (NOTE 2) |
| NOTE 1 - Not all generic codepoints are applicable for TCP. NOTE 2 - Background: there could be still media activity in case of unsuccessful bearer release. Hence, this event could principally be used to indicated a "hanging TCP connection". |  |  |

See Appendix III.4.2 for example traffic flows.

##### 8.6.5.3.2	Bearer initiated release (incoming side)

The MG waits for an incoming TCP-FIN packet as the start event for TCP bearer connection release (Table 21).

Table 21 – MG bearer plane procedures (incoming side)

| MG bearer plane procedure | Result | MGC notification? |
| --- | --- | --- |
| Awaits first TCP half-close by detection of incoming TCP-FIN. Subsequent acknowledgment as well as start of second TCP half-close procedures. Transition to connection state H.248 Idle (TCP 'CLOSED') after reception of TCP FIN-ACK. | Successful | See Table 20 |
|  | Unsuccessful | There are following options: - Via event g/cause - Via event adid/ipstop |

See Appendix III.4.1 for example traffic flows.

### 8.6.6	Path MTU Discovery

Path MTU discovery (PMTUD) is normally started after successful establishment of TCP bearer connections, see [IETF RFC 1191] (TCP-over-IPv4), [IETF RFC 1981] (TCP-over-IPv6) and [IETF RFC 2923] (TCP specific problems with PMTUD). It is expected that bearer plane PMTUD procedures are completely hidden to the MGC.

PMTUD procedures may result in error conditions, such as received ICMP errors from remote side. Whether such situations should lead to error notifications of the MGC by the MG is for further studies.

### 8.6.7	Interaction of TCP-retransmissions with traffic control functions

Retransmissions of unacknowledged TCP segment leads to an inherent increase of the consumed transport capacity. There might be therefore possible side effects and interactions with traffic control functions, such as traffic policing or traffic shaping.

#### 8.6.7.1	Interaction with traffic policing: IP byterate policing according to [ITU-T H.248.53]

Such a traffic policer entity is enforced at the ingress side of a TCP SEP. The traffic policer function shall consider all incoming TCP packets, without distinction between first sent or retransmitted TCP data.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix III

Illustration of the TCP basic connection control package protocol semantics

(This appendix does not form an integral part of this Recommendation.)

## III.1	Overview

The tcpbcc package (clause 8) defines the basic TCP bearer connection control with scope on support for establishment and release. The illustrated use cases are abstracted examples.

## III.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TCP) is highlighted besides the H.248 interface.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TCP bearer connection endpoint.

## III.3	Establishment of TCP bearer connections

### III.3.1	Successful establishment – Terminating side

See Figure III.1, termed as use case (E.1):

Figure III.1 – Successful establishment – Terminating side

The MGC explicitly blocks incoming TCP traffic handling (1). The MG discards any incoming TCP traffic (2). The MGC unblocks the SEP (3) and subscribes for state transitioning related to TCP bearer connection establishment (4). There is an incoming TCP bearer connection establishment procedure, given by a TCP three-way handshake (5, 6, 7). The TCP-ACK (7) leads to an H.248 state transition from Idle to Established, which again leads to the notification of the MGC (8).

### III.3.2	Successful establishment – Originating side

See Figure III.2, use case (E.2):

Figure III.2 – Successful establishment – Originating side

This scenario is similar with previous one, with exeption of

- MGC triggers outgoing TCP bearer connection establishment by signal EstBNC (4); and

- a corresponding TCP three-way handshake (5, 6, 7) in opposite direction.

### III.3.3	Unsuccessful establishment

Figure III.3 illustrates principle unsuccessful establishment scenarios:

Figure III.3 – Unsuccessful establishment

The TCP three-way handshake is basically not completed, either in incoming (see scenario (E.1), clause III.3.1) or outgoing (see scenario (E.2), clause III.3.2) direction. The H.248 state remains in Idle. The MGC may explicitly supervise the establishment process (1), see also clause 8.6.5.3.1.

The MG is able to detect unsuccessful establishment in this example, e.g., due to received TCP protocol control information (2).

## III.4	Release of TCP bearer connections

### III.4.1	Successful release – Terminating side

See Figure III.4, termed as use case (R.1):

NOTE: The event subscription (Rel) [1] may be sent before Established state (this comment is valid not only for this example).

Figure III.4 – Successful release – Terminating side

The MGC wants to be notified of successfully release TCP bearer connection (1). The MG receives a TCP-FIN (2) which starts a TCP bearer connection release by the remote TCP endpoint. The MG acknowledges the release requests (3) and initiates the closure of the other TCP traffic direction (4). The second TCP half-closure procedures is completed by (5), which leads to an H.248 state transition from Established to Idle, and the notification of the MGC (6).

### III.4.2	Successful release – Originating side

See Figure III.5, use cases (R.2) illustrates an outgoing, bidirectional TCP bearer connection release:

Figure III.5 – Successful release – Originating side

### III.4.3	Unsuccessful release

An unsuccessful TCP bearer connection  release procedure (Figure III.6) would imply still that the MG remains in H.248 state Established and that the MGC could possibly suspect a yet established TCP bearer connection. There are multiple options how such protocol deadlocks could be resolved. The situation is cleared in this example just by the subtraction of the termination by the MGC (1).

Figure III.6 – Unsuccessful release

[End Proposed Correction]

____________________