| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4488 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP: completion of clause 11 (package less procedures) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution intends to finalize clause 11 by resolving the final editor's notes.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4473).

Change proposal:

[Begin Proposed Correction]

# 11	Package-less TCP control

## 11.1	Package-less TCP control in other H.248.x-Recommendations

Pure SDP is used for the establishment of TCP bearer connection control by following Recommendations:

- [ITU-T H.248.69]: for MSRP-over-TCP bearers in MSRP switch configurations;

- [ITU-T H.248.84], clause 13: TCP merge and TCP relay mode configurations.

H.248 profiles shall avoid potential semantical interactions by using the (H.248) Property-based control method in case of expected amgibuity.

## 11.2	Principal H.248 control steps for communication establishment

### 11.2.1	Overview

Figure 10 makes an attempt in indicating all possible control steps (from H.248 perspective) concerning the establishment of TCP connections in environments demanding for NAT traversal support:

Figure 10 – Principal H.248 control steps for establishing a native TCP bearer

It is apparent that some steps might be combined (e.g., 1 and 2) and that specific steps might be optional (e.g., 3, 5, 7).

Table 24 provides a preliminary inventory of possible control elements from H.248 perspective. The column "H.248/SDP element" refers to "package-less TCP control".

Table 24 – H.248 control steps and possible control elements

| H.248 control step |  | H.248 property | H.248/SDP element | Comments |
| --- | --- | --- | --- | --- |
| Establishment phase: "IP transport connection establishment" |  |  |  |  |
| 1 | Creation SEP / SEPP | ADD.req/MOD.req command |  |  |
| 2 | Bearer type (L4/L3) indication to SEP | Indirect: package elements used from TCP-related H.248 packages | "m="-line <proto> (clause 13.4 / [ITU-T H.248.84, H.248.69]) | SDP may be ambiguous due to "protocol stack" concept (NOTE 1). 2nd issue: bearer type is a LCD level characteristic, but not LD/RD related! |
| 3 | Block / Unblock L4 Bearer Control Procedures at SEP | Property: tcpbcc/bceb | "a=setup:holdconn" | High level semantic. |
| 4 | Start IP Transport Connection Establishment Procedure ("TCP Open") | TCP proxy mode: Event: tcpbcc/BNCChange Signal: tcpbcc/EstBNC | TCP proxy mode: not possible due to [ITU-T H.248.84] "a=setup:" semantics for LD/RD usage | Conditional, dependent on required TCP mode.TCP proxy mode needs to be supported, in TCP ACTIVE and PASSIVE OPEN. |
| 5 | Local L4/L3 NAT traversal support | [ITU-T H.248.37] | "a=setup:" for [ITU-T H.248.84] NAT-T | Complementary due to different NAT-T methods |
| 6 | Successfully established IP Transport Connection (State "EstablishedTCP") | Event: tcpbcc/BNCChange | — | MGC notification might be optional. |
| 7 | Local L4+ NAT traversal support | [ITU-T H.248.78] | e.g., "a=msrp-cema" (?) | NOTE 2 |
| Communication phase |  |  |  |  |
| 8 | Active communication phase (i.e., transfer of application data) | - |  |  |
| NOTE 1 – The SDP "m=" line <proto> element allows the indication of a) a single protocol or b) a protocol stack (i.e., multiple protocol layers). The protocol stack based value is inherently ambiguous in case of the requirement in indicating a particular protocol layer out of the stack. This is a well-known issue of this element. NOTE 2 – The SDP based indication seems to be principally deficient ("application protocol specific SDP"), whereas [ITU-T H.248.78] supports the flexibility of different application protocols. |  |  |  |  |

### 11.2.2	Step 1: TCP-enabled termination/stream endpoint creation

Not SDP related, based on H.248 Command request.

### 11.2.3	Step 2: Bearer type indication "TCP"

According the SDP-based method as defined by clause 8.6.1.

Conditional approach, dependent on:

- C1: unambiguous indication of transport protocol "TCP" by protocol stack based SDP element "m="-line <proto>.

- C2: consistent LD/RD signalling (due to the fact of a LCD level characteristic) (NOTE – TCP is a bidirectional IP transport protocol, hence the SDP "m="-line <proto> needs to be consistent concerning the L4 indication of 'TCP'.).

- C3: could imply additional TCP bearer connection establishment (step 4) when using [ITU-T H.248.69].

### 11.2.4	Step 3: Block / Unblock TCP bearer control procedures at SEP

Not supported by SDP, thus property tcpbcc/bceb required.

NOTE – [IETF RFC 4145] SDP codepoint "a=setup:holdconn" is intended to hold TCP bearer connection establishment, however, it is semantically unclear how an H.248 MG should process any incoming TCP traffic.

### 11.2.5	Step 4: Start TCP connection establishment procedure ("TCP Open")

NOTE – Unconditional, general approach: signal tcpbcc/EstBNC or event tcpbcc/BNCChange{Type=Est} for triggering of outgoing or incoming TCP bearer connection establishment procedures.

The semantic of [IETF RFC 4145] SDP "a=setup" attribute could be used to derive the semantic of a "TCP active OPEN" or "TCP passive OPEN" procedure. However, this method may lead to ambiguities due to the "semantically overloaded" situation across multiple IETF RFCs.

Conditional approach, dependent on:

- C1: immediate establishment start only;

- C2: correct mapping on LD/RD;

- C3: settings shall be consistent with and not violate [ITU-T H.248.84], Table 1;

- C4: SDP element not be applied on other protocol layers (e.g, TLS, DTLS).

It is recommended to use the property-based control when one of these conditions would be relevant in a dedicated H.248 profile.

### 11.2.6	Step 5: Local L4/L3 NAT traversal support

Basically controlled via [ITU-T H.248.37], [ITU-T H.248.50] or/and [ITU-T H.248.84], but could be complemented by SDP-based signalling e.g. in case of "ICE for TCP" [b-IETF RFC 6544].

### 11.2.7	Step 6: Reporting successfully established TCP connection

Not possible via SDP, thus event tcpbcc/BNCChange required.

### 11.2.8	Step 7: Local L4+ NAT traversal support

Not possible via SDP for general L4+ NAT-T, thus [ITU-T H.248.78] required.

NOTE – Some application-specific L4+ NAT-T support functions could be associated with application-specific SDP information, however, this is out of scope of this Recommendation.

### 11.2.9	Step 8: Active communication phase

Control of functions during the active communication phase (such as performance measurements or application data inactivity detection for TCP) as such is out of scope of the SDP.

## 11.3	Release of TCP connection

Not possible via SDP. The hard removal of a TCP-enabled stream endpoint via an H.248 MODify.req or SUBtract.req command would lead to a complete release of MG level TCP resources. Usage of signal tcpbcc/RelBNC allows to keep these resources as long as the H.248 SEP should be not removed.

[End Proposed Correction]

____________________