| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4489 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.57: RTCP port allocation rules in context of RTP transport multiplexing - Discussion |  |  |  |
| Purpose: | Discussion |  |  |  |

Summary

This contribution discusses RTCP port allocation rules in context of RTP transport multiplexing and potential impact on H.248.57.

1	Introduction

H.248.57 is the central Recommendation concerning normative RTCP port allocation rules for H.248 gateways. This contribution discusses the impacts of additional support of RTP transport multiplexing (see H.248.RTPMUX), which leads basically to an extension of existing RTCP port allocation rules. Unfortunately, this extension is not straightforward, hence resolution proposals are required and discussed.

Parallel contribution AVD-4490 provides concrete change proposals against H.248.57. A revision of H.248.57 would be necessary.

2	Discussion

## 2.1	Reminder – H.248 bearer connection endpoints

There are four H.248 RTP bearer connection endpoints, - and RTCP port allocation rules are not necessarily the same for all four connection endpoints. RTCP port allocation rules are content of the H.248 Local and Remote Descriptors. Figure 1 (= copy of Fig. 2/H.248.1) recalls the H.248 conventions for connection endpoint naming and the scope of the H.248 descriptors LD and RD:

Figure 1 – Reminder – H.248.1 conventions: H.248 Local & Remote Descriptors

Figure 1 indicates also the pecularity of H.248 with respect to the disparity of four connection endpoints but only two H.248 descriptors for endpoint configuration. We need to keep this in mind because RTP transport multiplexing may be used principally at each connection endpoint individually.

Examples:

- The primary scope of the H.248 Local Descriptor is the local destination connection endpoint, but is also used (implictly) for local source connection endpoint settings (because symmetry assumptions are dominating). Leading to the question how RTCP port allocation would be controlled when both local connection endpoints would need different RTP transport multiplexing support?

- Or, information about the remote source connection endpoint is actually not available by the H.248 MG (because normally not needed). H.248 takes typically the assumption of symmetry at remote side … but might be an open door for security attacks against H.248 MGs.

Anyway, this contribution is not requesting any new syntax or/and semantics related to the H.248 LD and RD, but we need to be aware of this aspect when looking at RTCP port allocation.

The four principal connection endpoint options for RTP transport multiplexing are summarized in Figure 2. Figure 2 represents an extension of the generic connection endpoint conventions (Fig. 1) for concrete H.248 bearer type "RTP|RTCP".

Figure 2 – Reminder – H.248.1 conventions: RTCP Port Allocation

Figure 2 is furthermore reminding RTP traffic handling scenarios by H.248 IP-IP media gateways beyond the typical deployment scenarios in NGN/IMS, such as

- BGW without local NA(P)T (such as in H.248.64 IPR mode): e.g., no port translation could imply not any explicit allocation of local RTCP ports at all;

- BGW vs RTP topologies (see H.248.88): RTCP handling is typically dependent on the particular RTP topology;

- BGW with ANAT (RFC 4091) support: there might be double reservations of RTP/RTCP port resources at the very initial phase of bearer connection establishment.

Anyway, this contribution is not further discussing such scenarios, rather focusing on the "half call" model only by limiting our scope on H.248 (RTP enabled) terminations / stream endpoints, operated as RTP endsystem (e.g., the H.248.88 B2BRE mode in case of a BGW).

## 2.2	Reminder – Permutations of multiplexed and unmultiplexed options

Figure 3 summarizes the principal permutations of multiplexed and unmultiplexed options in case of RTP transport multiplexing. Only one traffic direction is considered. IP host X or Y could be a bearer connection endpoint located in an H.248 MG.

Figure 3 indicates whether a single or two IP transport connection endpoints are used for the RTP media and RTCP control flow components (i.e., RTP transport multiplexed mode or RTP transport unmultiplexed mode).

Figure 3 – Reminder – connection endpoints: 
permutations of multiplexed and unmultiplexed options

There are four permutations due to unidirectional consideration only (in Figure 3). There would be in total 16 principal options in case of a bidirectional picture of the RTP/RTCP session.

There are only a small number of relevant options in practise due to symmetry assumptions. There are basically two symmetry variants:

- Direction Symmetry (related the traffic direction between local (L) and remote (R) sites):

IF "ingress direction (R to L)" AND "egress direction (L to R)" use the SAME multiplexing method (i.e., multiplexed or unmultiplexed).

- Location Symmetry:

IF "local location" AND "remote location" use the SAME multiplexing method (i.e., multiplexed or unmultiplexed (with respect to one traffic direction).

The discussion and consideration of the various permutations in context of RTP transport multiplexing might be rather subject of H.248.RTPMUX. This contribution is only looking at the RTCP port allocation aspect from perspective of RTP transport multiplexed or unmultiplexed traffic.

What are the means for RTCP port allocation control from H.248 perspective? See next subclause…

## 2.3	Reminder – H.248 signalling tools for RTCP port allocation control

There are presently four explicit signalling elements which are used for RTCP port allocation control at H.248 interfaces (see summary in Table 1). Two were introduced by H.248 packages (rtcph, gm) in the early phase of H.248 border gateway development, and two were defined by the IETF for SDP.

Table 1 – H.248 signalling tools for RTCP port allocation control

| Signalling element |  | RTCP port allocation rule | Reference |
| --- | --- | --- | --- |
| 1 | H.248 property "rtcp/rsb" | PRTCP = PRTP + 1 | H.248.57 |
| 2 | SDP attribute "a=rtcp" | PRTCP = "a=rtcp:value" | RFC 3605 |
| 3 | SDP attribute "a=rtcp-mux" | PRTCP = PRTP | RFC 5761 |
| 4 | H.248 properties "gm/esps", "gm/lsp" | PRTCP = "gm/lsp = value" | H.248.43 |

Table 1 indicates the semantic of individual RTCP port allocation rule per element. It is obvious that there are interactions and contradictions when multiple of these elements are used in parallel. The purpose of H.248.57 is to define unambiguous RTCP port allocation rules for such cases (as part of the rtcp package specification).

We'd like to focus on the first three elements in the following.

## 2.4	Reminder – H.248 signalling tools versus H.248 descriptor level

The elements from Table 1 are defined/used at different descriptor levels: LCD versus LD/RD, see Figure 4. The descriptor level again relates to the scope on affected bearer connection endpoints (as introduced in clause 2.1):

Figure 4 – Reminder – RTCP port allocation control via three "H.248 tools"
(consideration of a single H.248 SEP ("half call model"))

When trying to sort out potential RTCP port allocation interaction issues when using a combination of the three elements "rtcph/rsb", "a=rtcp" and "a=rtcp-mux", it is helpful to use firstly abstracted element values at boolean level of "TRUE", "FALSE", "don't care" and "not sent" (see also Figure 4).

## 2.5	Reminder – History behind Table 1/H.248.57

The introduction of support for RTP transport multiplexing affects primarily Table 1/H.248.57 because acting as summary of all normative RTCP port allocation rules, taking into account rule interactions. Figure 5 recalls the status of existing H.248.57, based on the two elements "rtcph/rsb" and "a=rtcp":

Figure 5 – Reminder – History behind Table 1/H.248.57

It has to be noted that Table 1/H.248.57 did already consider "semantical controversy" (see NOTE 2) by allowing the "SDP semantic of ignore", which is normally not allowed in H.248 (and handled by error codes):

- IF H.248 "rtcph/rsb" property value 'OFF', then the sent SDP attribute "a=rtcp" by the MGC shall be ignored by the MG. [= NOTE 2 in Table 1/H.248.57]

The task to do is now to extend Table 1/H.248.57 by adding the rules and interaction resolution proposals for 3rd element "a=rtcp-mux".

## 2.3	Adding RTP transport multiplexing to Table 1/H.248.57

Firstly, a new column ""a=rtcp-mux" sent to MG?" is added to the Table 1/H.248.57.  Secondly we need to consider the combination of usage of that SDP element in H.248 descriptors LD, RD or both. We've splitted the final extended Table 1/H.248.57 in five pieces for discussion here. Below Table 2a provides the first part:

Table 2a – Extension of Table 1/H.248.57 – Part 1/5

Table 2a provides the combinations without any usage of "a=rtcp". We've applied following assumptions in these combinations:

- If "rtcph/rsb" is omitted, then the provisioned default value applies, which is either value "ON" or "OFF".

NOTE 1 – The aspect behind is the fact that there are H.248 profiles for RTP MGs without any usage of the rtcph package (such as for a trunking gateway). Thus, we need to define the semantic of "omitted". Assumption (R1) reflects the implicit behaviour of such H.248 RTP supporting gateways (like the TISPAN RGW, AGW and TGW). See also Appendix I.

The combined usage of "rsb = ON" AND "a=rtcp-mux" is inherently in contradiction (because the "PRTCP = PRTP + 1" allocation rule clashes with "PRTCP = PRTP"), following resolution is proposed.

- SDP attribute "a=rtcp-mux" overrules H.248 "rtcph/rsb" property value 'ON' .

NOTE 2 – This rule is motivated by the fact in ensuring backward compatibility with H.248 profiles, which support already the rtcph package (see also Appendix II). Because when codepoint "rsb = ON" would overrule SDP attribute "a=rtcp-mux", then the capability of RTP transport multiplexing couldn't be added to such profiles.

The next tables consider combinations with additional usage of "a=rtcp". E.g., Tables 2b, 2c and 2d need to resolve the interaction between the two SDP elements. Following assumption is applied:

- SDP attribute "a=rtcp-mux" overrules SDP attribute "a=rtcp".

NOTE 3 – The rationale for this proposal is motivated in Appendix III.

Table 2b – Extension of Table 1/H.248.57 – Part 2/5

Table 2c – Extension of Table 1/H.248.57 – Part 3/5

Table 2d – Extension of Table 1/H.248.57 – Part 4/5

All controversial cases for rsb = "ON" in above Tables 2a, 2b, 2c and 2d may be resolved when accepting resolutions R1, R2 and R3.

Table 2e – Extension of Table 1/H.248.57 – Part 5/5

Finally, we consider the cases for rsb = "OFF" (Table 2e), which needs furthermore to resolve the inherent semantical contradiction "RTCP control flow occurrence: No AND Yes!?" ('No' due rsb codepoint, 'Yes' due the SDP attribute usage).

Following resolution is applied, motivated by (R0), i.e., similar handling of SDP attributes versus H.248 rsb property:

- If H.248 "rtcph/rsb" property value 'OFF', then the sent SDP attribute "a=rtcp-mux" by the MGC shall be ignored by the MG in case of contradiction.

NOTE 4 – The H.248 property at LCD level is higher weighted than SDP attributes at LD/RD level.

3	Summary of analysis

## 3.1	Overview of applied rules for resolving interaction issues

Table 3 provides a summary of all rules for interaction resolution:

Table 3 – Overview of applied rules for resolving interaction issues

| Interaction resolution rule |  | Status |
| --- | --- | --- |
| R0 | If H.248 "rtcph/rsb" property value 'OFF', then the sent SDP attribute "a=rtcp" by the MGC shall be ignored by the MG. | Existing (= NOTE 2 in Table 1/H.248.57) |
| R1 | If "rtcph/rsb" is omitted, then the provisioned default value applies, which is either value "ON" or "OFF". | New proposal |
| R2 | SDP attribute "a=rtcp-mux" overrules H.248 "rtcph/rsb" property value 'ON' . | New proposal |
| R3 | SDP attribute "a=rtcp-mux" overrules SDP attribute "a=rtcp". | New proposal |
| R4 | If H.248 "rtcph/rsb" property value 'OFF' overrules SDP attribute "a=rtcp-mux", i.e., then the sent SDP attribute "a=rtcp-mux" by the MGC shall be ignored by the MG in case of contradiction. | New proposal |

Now, we need to proof that the five interaction resolution rules R0 to R4 are not contradictory in itself:

- R1 is disjoint to all four other rules R0, R2, R3 and R4. (OK)

- R0 & R4 are disjoint. (OK)

- R2 is disjoint against R0 & R4. (OK)

- R3 is independent of all "rsb" related rules R0, R1, R2 and R4. (OK)

Thus, the application of these rules allows the unambiguous integration of RTP transport multiplexing support.

## 3.2	Summary of applied rationales

There are of course alternatives to the four new proposed interaction resolution rules, such as

- limitation of usage (i.e., the MGC does NOT sent …);

- other precedence / ignore principles;

- H.248 descriptor scope limitations …;

- mutual exclusion rule for contradictory SDP elements;

- other semantic of 'omitt' (for rsb); or

- symmetry assumptions.

We did investigate such alternatives, but finally coming to proposals R1 to R4, primarily due to

- backward compatibility reasons;

- consideration of H.248 profiles with and without rtcph package; and

- equal handling of RD/LD level SDP attributes versus LCD level properties.

4	Proposals

H.248.57 should be revised by considering following items:

- extension with SDP attribute "a=rtcp-mux";

- no new syntax required for rtcph package,

- but new semantics in terms of package procedures,

- primarily by adding a new table (according to Tables 2a to 2e) similar as Table 1; and

- adding a new Annex on "H.248 profiles with and without support of RTCP handling package".

Parallel contribution AVD-4490 indicates correspondent change proposals.

Appendix I – Rationale behind rtcph package-less H.248 profiles

The H.248 profiles for VoRTP gateways (such as residential, access and trunking gateways for PSTN emulation subsystems) doesn't contain the rtcph package at all.

Strictly, the interaction resolution rules R0, R2 and R4 could be interpreted differently ("SDP attribute would be applied").

However, there shouldn't be any issue in reality because such H.248 VoRTP profiles use all the implicit RTCP port allocation behaviour according RFC 3550, i.e., the "N+1" allocation rule (when RTCP is used at all).

Thus, such H.248 profiles provide implicitly "rtcph/rsb property value 'ON'" behaviour.

Further, these referred H.248 VoRTP profiles (in their current versions) do not (yet) support these SDP attributes related to RTCP port allocation (to our knowledge).

Any upversioning of these profiles by additional support of "a=rtcp" or/and "a=rtcp-mux" could and should be then resolved by referring to H.248.57.

Appendix II – Rationale behind resolution of
combinations "rsb" property and SDP "a=rtcp-mux"

The semantic of the "rsb" property could be essentially summarized as

- prohibit RTCP traffic (rsb="OFF")

- "consecutive port number" rule (rsb="ON"), IF NOT "a=rtcp:…" defines another number …

Usage of SDP "a=rtcp-mux" in addition leads to principal contradictions, see Fig. II.1. The figure indicates also possible resolutions. Finally we selected the resolutions rules R2 and R4 for these combinations.

Figure II.1 – Contradictions for combinations "rsb" property and SDP "a=rtcp-mux"

Appendix III – Rationale behind resolution of
combinations with SDP "a=rtcp" and SDP "a=rtcp-mux"

Figure III.1 summarizes problem statement and selected resolution for these combinations:

Figure III.1 – Combination SDP "a=rtcp" and SDP "a=rtcp-mux" – Problem statement and resolution

The solution is based on the interpretation of usage at SIP/SDP level:

An SDP Offer with both SDP attributes "a=rtcp" and SDP "a=rtcp-mux" actually represents a semantic of "select 1-out-of-2 options" (but not both).

On a first glance, such a semantic seems to be not required at the H.248 interface, because the MGC as master could make already the decision.

However, we finally concluded on rule R2 ("a=rtcp-mux overrules a=rtcp ") because may be justified with RFC 5761, Sec 5.1.1, which specifies for SIP/SDP signaling:

"If the answer does not contain an "a=rtcp-mux" attribute, the offerer MUST NOT multiplex RTP and RTCP packets on a single port.  Instead, it should send and receive RTCP on a port allocated according to the usual port-selection rules (either the port pair, or a signalled port if the "a=rtcp:" attribute [10] is also included).  This will occur when talking to a peer that does not understand the "a=rtcp-mux" attribute."

____________________