| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4490 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.57: RTCP port allocation rules in context of RTP transport multiplexing - Change proposal for a revised H.248.57 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes changes for H.248.57 in order to support additionally RTP transport multiplexing.

Discussion

The motivation and proposed solution is discussed in parallel contribution AVD-4489.

Conclusion

If there is agreement that support of RTP transport multiplexing should be incorporated in H.248.57 (in order to solve the problem of interaction issues concerning RTCP port allocation rules), then a Revision of this Recommendation seems to be necessary. An Amendment seems to be insufficient (to be discussed).

Proposal

There would be following changes:

- extension of scope (clause 1), indication of RTP transport multiplexing support;

- additional references RFC 5671 and H.248.RTPMUX (clause 2);

- reference to term " RTP transport multiplexing " (new clause 3.2)

- RTCP port allocation rules in new clause 7 ("in order not to overload clauses 6.6.x")

- addition of H.248 profile related information (new Annex A)

It is proposed to accept the marked up changes below. Changes are marked up against ITU-T Rec. H.248.57 (03/2013).

Change proposals:

Recommendation ITU-T H.248.57

Gateway control protocol: RTP control protocol package

# 1	Scope

This Recommendation contains functionality to describe the use of the real-time transport protocol control protocol (RTCP) in ITU-T H.248-controlled media gateways. RTCP is used for instance to monitor the quality of service and to convey information about the participants in an on-going real-time transport protocol (RTP) session.

[ITU-T H.248.1] assumes that when an RTP stream is specified in a local or remote descriptor, that an RTCP flow may be established according to the rules defined in section 11 of [IETF RFC 3550]. Additional ITU-T H.248/RTCP behaviour is defined in clause E.12.5 of [ITU-T H.248.1]. However, some media gateways may not instantiate an RTCP flow. For interoperability and for some applications such as firewall and network address translation (NAT), it is important for the MGC to be certain of the media gateway behaviour with respect to port allocation for RTCP. The "RTCP Handling Package" defined in this Recommendation provides a property to control this RTCP flow allocation.

Revision 1 of this Recommendation supports in addition RTCP port allocation rules in context of RTP transport multiplexing (see [ITU-T H.248.57]).

## 1.1	Overview

RTCP handling may comprise the following functions:

1)	Indication of the existence of an RTCP flow of an RTP session (RTP with or without RTCP).

2)	Indication of endpoint identifiers on RTP level that are relevant for both RTP and RTCP (e.g., SSRC).

In addition, if an RTCP flow exists:

3)	Resource management of endpoint identifiers on layer 4 (UDP ports; i.e., reservation, allocation and indication of a specific allocation rule) for RTCP flows.

4)	Resource management of endpoint identifiers on RTP level (e.g., CNAME) for RTCP flows.

5)	Resource management of other resources required for RTCP flows (e.g., reservation and allocation of transport capacity).

6)	Indication of specific modes of operation for RTCP (e.g., layer 4 multiplexing of RTP and RTCP flows, or multiplexing mode of RTCP reports in a single RTCP packet).

Orthogonal to the above functions:

7)	Mapping scheme of RTP and RTCP flows on H.248 streams.

This Recommendation deals with functions 1 and 3 only. Other functions are out of the scope of this Recommendation.

In addition, this Recommendation assumes that regarding function 7, RTP and RTCP flows are always mapped into a single ITU-T H.248 stream. Other mapping schemes are out of the scope of this Recommendation and may make this Recommendation's procedures irrelevant.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.
<http://www.itu.int/rec/T-REC-H.248.1>

[ITU-T H.248.RTPMUX]	Draft Recommendation ITU-T H.248.RTPMUX (2015?), Gateway control protocol: H.248 support for RTP multiplexing.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.
<http://www.ietf.org/rfc/rfc3550.txt>

[IETF RFC 3605]	IETF RFC 3605 (2003), Real Time Control Protocol (RTCP) attribute in Session Description Protocol (SDP).
<http://www.ietf.org/rfc/rfc3605.txt>

[IETF RFC 4566]	IETF RFC 4566 (2006), SDP: Session Description Protocol.
<http://www.ietf.org/rfc/rfc4566.txt>

[IETF RFC 5761]	IETF RFC 5761 (04/2010), Multiplexing RTP Data and Control Packets on a Single Port.
<http://www.ietf.org/rfc/rfc5761.txt>

# 3	Definitions

## 3.1	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.1.1	RTCP flow: The sending and reception of RTCP packets as described by section 6 of [IETF RFC 3550]. The RTCP flow relates to an "(ITU-T H.248) control flow" (see clause 3.9 of [ITU-T H.248.1]).

NOTE – An RTCP flow comprises all RTCP traffic on an RTP session. An RTCP flow may be further separated into RTCP sub-flows due to different RTCP report types. An RTCP sub-flow is identified by a specific RTCP packet type codepoint.

3.1.2	RTP flow: The sending and reception of RTP packets of an RTP session. The RTP flow relates to an "ITU-T H.248 media flow" (see [ITU-T H.248.1]).

3.1.3	RTP session: An RTP session comprises a single RTP flow and an optional RTCP flow.

## 3.2	Terms defined elsewhere

3.2.1	RTP / RTCP transport multiplexing (briefly "RTP transport multiplexing") [H.248.RTPMUX]: a single IP transport (L4) port for RTP and RTCP packets

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

AVP	Audio Visual Profile

LD	Local Descriptor

MG	Media Gateway

MGC	Media Gateway Controller

NAT	Network Address Translation

RD	Remote Descriptor

RTCP	Real-time Transport protocol Control Protocol

RTP	Real-time Transport Protocol

SSRC	Synchronization Source

# 5	Conventions

None.

# 6	RTCP Handling Package

|  | Package name: | RTCP Handling Package |
| --- | --- | --- |
|  | Package ID: | rtcph (0x00b5) |
|  | Description: | This package allows the MGC to indicate to the MG per stream whether or not to establish an RTCP control flow when an RTP media flow (e.g., for RTP profiles such as RTP/AVP, etc.) is created. |
|  | Version: | 1 |
|  | Extends: | none |

## 6.1	Properties

### 6.1.1	RTCP Allocation Specific Behaviour

|  | Property name: | RTCP Allocation Specific Behaviour |
| --- | --- | --- |
|  | Property ID: | rsb (0x0009) |
|  | Description: | This property indicates whether or not an RTCP flow and an associated port is automatically associated with an RTP flow. |
|  | Type: | Boolean |
|  | Possible values: | ON (a RTCP flow shall be allocated or de-allocated automatically with the creation or deletion of the RTP flow); OFF (a RTCP flow shall not be allocated or de-allocated automatically with the creation or deletion of the RTP flow) |
|  | Default: | Provisioned |
|  | Defined in: | Local control |
|  | Characteristics: | Read/Write |

## 6.2	Events

None.

## 6.3	Signals

None.

## 6.4	Statistics

None.

## 6.5	Error Codes

None.

## 6.6	Procedures

### 6.6.1	Reservation and allocation of layer 4 ports for RTP and RTCP

The port number allocation rules, as defined by [IETF RFC 3550], are used as the starting point.

#### 6.6.1.1	Overview – Four RTCP transport addresses

Figure 1 shows a bidirectional RTP session with RTCP in each direction. There are then four traffic flows. The RTP-based media flow and RTCP-based control flow sharing the same IP addresses (see Note as regards possible exceptions), but using normally different L4 (UDP) ports. These four RTCP ports are shown in the diagram.

NOTE – The SDP attribute "a=rtcp:" allows also the explicit specification of an IP network address in addition to the layer 4 port. This may then lead to different IP addresses for RTP and RTCP flows. Figure 1 illustrates the particular case of identical IP addresses.

Figure 1 – Connection endpoint naming conventions – the four
RTCP ports of a bidirectional RTP/RTCP session

#### 6.6.1.2	Specific behaviour "activated"

If the MGC sets the rtcph/rsb property to "ON", when the MG is requested to allocate/de-allocate a local destination port for an RTP stream, a consecutive port for the reception of the associated RTCP flow is automatically allocated/de-allocated.

#### 6.6.1.3	Specific behaviour "deactivated"

If the MGC sets the rtcph/rsb property to "OFF", then only a single port is allocated to the RTP stream and no RTCP flow is assumed to exist for the stream.

#### 6.6.1.4	Precedence of rtcph/rsb property over other port allocation mechanisms

The setting of the rtcph/rsb property in general takes precedence over any values set in the local and remote descriptors with regard to port allocation for RTP streams. However, there is one exception: if the rtcph/rsb property is set to ON and a specific RTCP transport address is provided in the local and/or remote descriptor, e.g., using SDP "a=rtcp:" attribute as defined in [IETF RFC 3605], the RTCP packets should use the transport addresses indicated by the local/remote descriptors.

##### 6.6.1.4.1	Notes to SDP attribute "a=rtcp"

In the scope of this Recommendation, it is noted that:

–	the SDP attribute "a=rtcp:" allows also the explicit specification of an IP network address in addition to the layer 4 port; and that

–	this SDP attribute may be principally also used in the H.248 LD besides its usage in the ITU-T H.248 RD.

Note that both the IP address and the port number appearing in the "a=rtcp:" attribute may be underspecified. In this case, the MG is free to allocate any transport address to the RTCP flow (i.e., not necessarily using the same IP address as the RTP flow).

#### 6.6.1.5	Summary of basic rules according to [IETF RFC 3550] and [IETF RFC 3605]

Table 1 provides a summary of the port allocation rules for RTCP.

Table 1 – Allocation rules for RTCP ports with [IETF RFC 3550]
(with and without RFC 3605 attribute)

| H.248 property value | "a=rtcp" sent to MG? | Local endpoints |  | Remote endpoints |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LD(A, P) | LS(A, P) | RD(A, P) | RS(A, P) |
| rsb = "ON" | No | Consecutive port to the one indicated by the local descript. | MG (Note 1) | Consecutive port to the one indicated by the remote descript. | Out of scope |
|  | Yes in H.248 LD | "a=rtcp" port | MG (Note 1) | – | – |
|  | Yes in H.248 RD | – | – | "a=rtcp:" port | Out of scope |
| rsb = "OFF" | No | No port allocated | No port allocated | No port allocated | Out of scope |
|  | Yes in H.248 LD | No port allocated (Note 2) | No port allocated (Note 2) | No port allocated (Note 2) | Out of scope |
|  | Yes in H.248 RD | No port allocated (Note 2) | No port allocated (Note 2) | No port allocated (Note 2) | Out of scope |
| NOTE 1 – The management of LS(A) and LS(P) resources is under control of the media gateway. The MG shall apply the "consecutive" port allocation rule. NOTE 2 – This is a semantic contradiction, however the MG shall ignore the "a=rtcp:" attribute. |  |  |  |  |  |

#### 6.6.1.6	Additional rules due to "number of port" qualifier according to [IETF RFC 4566]

[IETF RFC 4566] defines the "number of port" qualifier for the SDP "m=" line used to specify multiple transport ports. The RTCP port allocation rules (Note) are also provided by [IETF RFC 4566] in the case of RTP-based media. The "number of port" qualifier indicates the number of RTP/RTCP flow pairs for a single H.248 stream.

NOTE – Section 5.14 of [IETF RFC 4566] states that "In such a case, the ports used depend on the transport protocol. For RTP, the default is that only the even-numbered ports are used for data with the corresponding one-higher odd ports used for the RTCP belonging to the RTP session, and the <number of ports> denoting the number of RTP sessions " ..." If non-contiguous ports are required, they must be signalled using a separate attribute (for example, "a=rtcp:" as defined in …)".

RTCP port values shall be thus odd numbers, also in case of "a=rtcp" usage. This Recommendation does suppose a single SDP "a=rtcp" attribute per SDP media description ("m=" line) when [IETF RFC 3605] is used.

Table 2 provides a summary of these rules in conjunction with the rsb property.

Table 2 – Allocation rules for RTCP ports for RTP applications
using SDP "number of port" qualifier

| H.248 property value | "a=rtcp" sent to MG? | Local endpoints |  | Remote endpoints |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LD(A, P) | LS(A, P) | RD(A, P) | RS(A, P) |
| rsb = "ON" | No | Consecutive port to the one indicated by the local descript. (for the first pair) plus rule "contiguous (odd) for each further RTP/RTCP pair" | See Table 1 | Consecutive port to the one indicated by the remote descript. (for the first pair) plus rule "contiguous (odd) for each further RTP/RTCP pair" | See Table 1 |
|  | Yes in H.248 LD | "a=rtcp:" port (for the first RTP/RTCP pair) plus rule "contiguous (odd) for each further RTP/RTCP pair" (Note 1) | See Table 1 | – | – |
|  | Yes in H.248 RD | – | – | "a=rtcp:" port (for the first RTP/RTCP pair) plus rule "contiguous (odd) for each further RTP/RTCP pair" (Note 2) | See Table 1 |
| rsb = "OFF" | No | See Table 1 | See Table 1 | See Table 1 | See Table 1 |
|  | Yes in H.248 LD | See Table 1 | See Table 1 | See Table 1 | See Table 1 |
|  | Yes in H.248 RD | See Table 1 | See Table 1 | See Table 1 | See Table 1 |
| NOTE 1 – If both 'number of ports' and "a=rtcp:" are indicated in the LD, the RTCP port of the first pair is allocated according to the "a=rtcp:" and the RTP port of the next pair takes the one-higher value to the RTCP port of the last pair. The RTCP port of the next pair takes the consecutive higher number to the RTP port. NOTE 2 – If both 'number of ports' and "a=rtcp:" are indicated in the RD, the RTCP port of the first pair is allocated according to the "a=rtcp:" and the RTP port of the next pair takes the one-higher value to the RTCP port of the last pair. The RTCP port of the next pair takes the consecutive higher number to the RTP port. |  |  |  |  |  |

### 6.6.2	RTP sessions with or without an RTCP flow (existence of RTCP flow)

RTCP is fundamentally optional for RTP.

#### 6.6.2.1	Indication of the existence of RTCP per H.248 stream or termination

As the use of RTCP is optional, the default behaviour of an MG as to the use of RTCP flows is not described in [ITU-T H.248.1]. The default behaviour could be, e.g., defined in an ITU-T H.248 profile specification (see, e.g., template clauses for connection model, stream descriptor or SDP information elements in Appendix III of [ITU-T H.248.1]).

The usage protocol elements (e.g., such as the ITU-T H.248 rtcph/rsb property, or ITU-T H.248 statistics on RTCP basis, or RTCP-specific SDP information elements according to [b-IETF RFC 3556], [IETF RFC 3605] or [b-IETF RFC 3890]) in ITU-T H.248 commands, which provide an indication of RTCP, shall over-rule the default behaviour.

#### 6.6.2.2	RTCP-less RTP sessions

No UDP port (or other resources) for RTCP will be reserved.

#### 6.6.2.3	RTP sessions with RTCP

Handling of RTCP shall follow the guidelines of this Recommendation.

### 6.6.3	Mapping of RTP and RTCP flows on ITU-T H.248 streams

#### 6.6.3.1	Mapping schemes

##### 6.6.3.1.1	Single ITU-T H.248 stream for RTP and RTCP

The default assumption is that an RTP/RTCP session is mapped on a single ITU-T H.248 stream.

NOTE – This is based on clause E.12 of [ITU-T H.248.1] (in case of usage for ITU-T H.248 RTP terminations) and the reference on RTP profile "RTP/AVP" according to [IETF RFC 3550]. The default media description (e.g., by SDP information elements) for media types/formats for media transport "RTP/AVP" does not explicitly specify RTCP. RTCP is implicitly part of the same ITU-T H.248 stream as for RTP per default.

##### 6.6.3.1.2	Individual ITU-T H.248 streams for RTP and RTCP

An individual ITU-T H.248 stream, solely for RTCP traffic, could be used in principle. But it is so far not possible to provide sufficient information within the ITU-T H.248 stream descriptor for "comprehensive" specification of RTCP flows.

Such an individual ITU-T H.248 stream could be characterized by the 2-tuples of IP address and port (which is not really specific for RTCP).

#### 6.6.3.2	Mapping control

Mapping control is subject of the usage and capabilities of the ITU-T H.248 stream descriptor. There are no protocol means presently available for the individual stream mapping mode.

Property rtcph/rsb shall not be used to influence the mapping between RTP and RTCP flows on ITU-T H.248 streams.

# 7	Additional support of RTP transport multiplexing

## 7.1	Introduction

RTP transport multiplexing (see [ITU-T H.248.RTPMUX]) is indicated by the MGC towards MG via the SDP attribute "a=rtcp-mux" [IETF RFC 5761] in the H.248 LD and/or RD. RTP transport multiplexing may be used in addition to the existing capabilities of the RTCP handling package (clause 6).

## 7.2	Backward compatibility

In order to ensure backward compatibility in H.248 profiles with existing support of the RTCP handling package, possible interactions between RTCP port allocation rules according to [IETF RFC 3550], [IETF RFC 3605]  and [IETF RFC 5761] must be considered.

Table 3 provides a summary of all rules for interaction resolution:

Table 3 – Overview of applied rules for resolving interaction issues

| Interaction resolution rule |  | Comment |
| --- | --- | --- |
| R0 |  | If H.248 "rtcph/rsb" property value 'OFF', then the sent SDP attribute "a=rtcp" by the MGC shall be ignored by the MG. |  | Existing rule, see NOTE 2/Table 1 |
| R1 |  | If "rtcph/rsb" is omitted, then the provisioned default value applies, which is either value "ON" or "OFF". |  | See Annex A for more information. |
| R2 |  | SDP attribute "a=rtcp-mux" overrules H.248 "rtcph/rsb" property value 'ON'. |  | If not, RTCP transport multiplexing couldn't be supported. |
| R3 |  | SDP attribute "a=rtcp-mux" overrules SDP attribute "a=rtcp". |  | In line with [IETF RFC 5761]. |
| R4 |  | If H.248 "rtcph/rsb" property value 'OFF' overrules SDP attribute "a=rtcp-mux", i.e., then the sent SDP attribute "a=rtcp-mux" by the MGC shall be ignored by the MG in case of contradiction. |  | In order to safe basic rtcph semantics. |

## 7.3	Summary of rules according to [IETF RFC 3550], [IETF RFC 3605] and [IETF RFC 5671]

See Table 4/1 to 4/5:

Table 4/1 – Rules according to [IETF RFC 3550], [IETF RFC 3605] and [IETF RFC 5671] – Part 1/5

| H.248 rsb property value | "a=rtcp" sent to MG? | "a=rtcp-mux" sent to MG? | Local endpoints |  | Remote endpoints |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LD(A, P) | LS(A, P) | RD(A, P) | RS(A, P) |
| omitted |  |  | If rtcph/rsb is omitted, then the provisioned default value applies, which is either "ON" or "OFF". Thus, then corresponding rows below apply. |  |  |  |
| rsb = "ON" | No | No | Consecutive port to the one indicated by the local descript. m-line, LD(A)RTCP = LD(A)RTP LD(P)RTCP = LD(P)RTP + 1 | MG (Note 1), where LS(A)RTCP = LS(A)RTP LS(P)RTCP = LS(P)RTP + 1 Typically also: LS(A,P)RTP = LD(A,P)RTP LS(A,P)RTCP = LD(A,P)RTCP | Consecutive port to the one indicated by the remote descript. m-line, RD(A)RTCP = RD(A)RTP RD(P)RTCP = RD(P)RTP + 1 | Out of scope |
|  |  | Yes in H.248 LD | LD(A,P)RTCP = LD(A,P)RTP | MG (Note 1), where LS(A,P)RTCP = LS(A,P)RTP Typically also: LS(A,P)RTCP = LD(A,P)RTCP = LD(A,P)RTP = LS(A,P)RTP | Consecutive port to the one indicated by the remote descript. m-line, RD(A)RTCP = RD(A)RTP RD(P)RTCP = RD(P)RTP + 1 | Out of scope |
|  |  | Yes in H.248 RD | LD(A)RTCP = LD(A)RTP LD(P)RTCP = LD(P)RTP + 1 | MG (Note 1), where LS(A)RTCP = LS(A)RTP LS(P)RTCP = LS(P)RTP + 1 Typically also: LS(A,P)RTP = LD(A,P)RTP LS(A,P)RTCP = LD(A,P)RTCP | RD(A,P)RTCP = RD(A,P)RTP | Out of scope |
|  |  | Yes in H.248 LD & RD | LD(A,P)RTCP = LD(A,P)RTP | MG (Note 1), where LS(A,P)RTCP = LS(A,P)RTP Typically also: LS(A,P)RTCP = LD(A,P)RTCP = LD(A,P)RTP = LS(A,P)RTP | RD(A,P)RTCP = RD(A,P)RTP | Out of scope |

Table 4/2 – Rules according to [IETF RFC 3550], [IETF RFC 3605] and [IETF RFC 5671] – Part 2/5

| H.248 rsb property value | "a=rtcp" sent to MG? | "a=rtcp-mux" sent to MG? | Local endpoints |  | Remote endpoints |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LD(A, P) | LS(A, P) | RD(A, P) | RS(A, P) |
| rsb = "ON" | Yes in H.248 LD | No | LD(A)RTCP = LD(A)RTP LD(P)RTCP = "a=rtcp" value | MG (Note 1), where LS(A)RTCP = LS(A)RTP Typically also: LS(A,P)RTCP = LD(A,P)RTCP | RD(A)RTCP = RD(A)RTP RD(P)RTCP = RD(P)RTP + 1 | Out of scope |
|  |  |  |  | Yes in H.248 LD |  | LD(A,P)RTCP = LD(A,P)RTP (rule R2 ("a=rtcp-mux" overrules "a=rtcp")) |  | MG (Note 1), where LS(A,P)RTCP = LS(A,P)RTP Typically also: LS(A,P)RTCP = LD(A,P)RTCP = LD(A,P)RTP = LS(A,P)RTP |  | RD(A)RTCP = RD(A)RTP RD(P)RTCP = RD(P)RTP + 1 |  | Out of scope |  |
|  |  |  |  | Yes in H.248 RD |  | LD(A)RTCP = LD(A)RTP LD(P)RTCP = "a=rtcp" value |  | MG (Note 1), where LS(A)RTCP = LS(A)RTP Typically also: LS(P)RTCP = "a=rtcp" port LS(A,P)RTCP = LD(A,P)RTCP |  | RD(A,P)RTCP = RD(A,P)RTP |  | Out of scope |  |
|  |  |  |  | Yes in H.248 LD & RD |  | LD(A,P)RTCP = LD(A,P)RTP (rule R2 ("a=rtcp-mux" overrules "a=rtcp")) |  | MG (Note 1), where LS(A,P)RTCP = LS(A,P)RTP Typically also: LS(A,P)RTCP = LD(A,P)RTCP = LD(A,P)RTP = LS(A,P)RTP |  | RD(A,P)RTCP = RD(A,P)RTP |  | Out of scope |  |

Table 4/3 – Rules according to [IETF RFC 3550], [IETF RFC 3605] and [IETF RFC 5671] – Part 3/5

| H.248 rsb property value | "a=rtcp" sent to MG? | "a=rtcp-mux" sent to MG? | Local endpoints |  | Remote endpoints |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LD(A, P) | LS(A, P) | RD(A, P) | RS(A, P) |
| rsb = "ON" |  | Yes in H.248 RD |  | No |  | LD(A)RTCP = LD(A)RTP LD(P)RTCP = LD(P)RTP + 1 |  | MG (Note 1), where LS(A)RTCP = LS(A)RTP LS(P)RTCP = LS(P)RTP + 1 Typically also: LS(A,P)RTP = LD(A,P)RTP LS(A,P)RTCP = LD(A,P)RTCP |  | RD(A)RTCP = RD(A)RTP RD(P)RTCP = "a=rtcp:" port |  | Out of scope |  |
|  |  |  |  | Yes in H.248 LD |  | LD(A,P)RTCP = LD(A,P)RTP |  | MG (Note 1), where LS(A,P)RTCP = LS(A,P)RTP Typically also: LS(A,P)RTCP = LD(A,P)RTCP = LD(A,P)RTP = LS(A,P)RTP |  | RD(A)RTCP = RD(A)RTP RD(P)RTCP = "a=rtcp:" port |  | Out of scope |  |
|  |  |  |  | Yes in H.248 RD |  | LD(A)RTCP = LD(A)RTP LD(P)RTCP = LD(P)RTP + 1 |  | MG (Note 1), where LS(A)RTCP = LS(A)RTP LS(P)RTCP = LS(P)RTP + 1 Typically also: LS(A,P)RTP = LD(A,P)RTP LS(A,P)RTCP = LD(A,P)RTCP |  | RD(A,P)RTCP = RD(A,P)RTP (rule R2 ("a=rtcp-mux" overrules "a=rtcp")) |  | Out of scope |  |
|  |  |  |  | Yes in H.248 LD & RD |  | LD(A,P)RTCP = LD(A,P)RTP |  | MG (Note 1), where LS(A,P)RTCP = LS(A,P)RTP Typically also: LS(A,P)RTCP = LD(A,P)RTCP = LD(A,P)RTP = LS(A,P)RTP |  | RD(A,P)RTCP = RD(A,P)RTP (rule R2 ("a=rtcp-mux" overrules "a=rtcp")) |  | Out of scope |  |

Table 4/4 – Rules according to [IETF RFC 3550], [IETF RFC 3605] and [IETF RFC 5671] – Part 4/5

| H.248 rsb property value | "a=rtcp" sent to MG? | "a=rtcp-mux" sent to MG? | Local endpoints |  | Remote endpoints |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LD(A, P) | LS(A, P) | RD(A, P) | RS(A, P) |
| rsb = "ON" |  | Yes in H.248 LD & RD |  | No |  | LD(A)RTCP = LD(A)RTP LD(P)RTCP = "a=rtcp:" port |  | MG (Note 1), where LS(A)RTCP = LS(A)RTP Typically also: LS(A,P)RTCP = LD(A,P)RTCP |  | RD(A)RTCP = RD(A)RTP RD(P)RTCP = "a=rtcp:" port |  | Out of scope |  |
|  |  |  |  | Yes in H.248 LD |  | LD(A,P)RTCP = LD(A,P)RTP (rule R2 ("a=rtcp-mux" overrules "a=rtcp")) |  | MG (Note 1), where LS(A,P)RTCP = LS(A,P)RTP Typically also: LS(A,P)RTCP = LD(A,P)RTCP = LD(A,P)RTP = LS(A,P)RTP |  | RD(A)RTCP = RD(A)RTP RD(P)RTCP = "a=rtcp:" port |  | Out of scope |  |
|  |  |  |  | Yes in H.248 RD |  | LD(A)RTCP = LD(A)RTP LD(P)RTCP = "a=rtcp:" port |  | MG (Note 1), where LS(A)RTCP = LS(A)RTP Typically also: LS(A,P)RTCP = LD(A,P)RTCP |  | RD(A,P)RTCP = RD(A,P)RTP (rule R2 ("a=rtcp-mux" overrules "a=rtcp")) |  | Out of scope |  |
|  |  |  |  | Yes in H.248 LD & RD |  | LD(A,P)RTCP = LD(A,P)RTP (rule R2 ("a=rtcp-mux" overrules "a=rtcp")) |  | MG (Note 1), where LS(A,P)RTCP = LS(A,P)RTP Typically also: LS(A,P)RTCP = LD(A,P)RTCP = LD(A,P)RTP = LS(A,P)RTP |  | RD(A,P)RTCP = RD(A,P)RTP (rule R2 ("a=rtcp-mux" overrules "a=rtcp")) |  | Out of scope |  |

Table 4/5 – Rules according to [IETF RFC 3550], [IETF RFC 3605] and [IETF RFC 5671] – Part 5/5

| H.248 rsb property value | "a=rtcp" sent to MG? | "a=rtcp-mux" sent to MG? | Local endpoints |  | Remote endpoints |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LD(A, P) | LS(A, P) | RD(A, P) | RS(A, P) |
| rsb = "OFF" | No | No | No port allocated | No port allocated | No port allocated | Out of scope |
|  |  | Yes in H.248 LD | No port allocated (Note 3) | No port allocated (Note 3) | No port allocated (Note 3) | Out of scope |
|  |  | Yes in H.248 RD | No port allocated (Note 3) | No port allocated (Note 3) | No port allocated (Note 3) | Out of scope |
|  |  | Yes in H.248 LD & RD | No port allocated (Note 3) | No port allocated (Note 3) | No port allocated (Note 3) | Out of scope |
|  | Yes in H.248 LD | No | No port allocated (Note 2) | No port allocated (Note 2) | No port allocated (Note 2) | Out of scope |
|  |  |  |  | Yes in H.248 LD |  | No RTCP control flow at all, thus: No port allocated (Note 3) (rule R2 ("rsb=OFF" overrules SDP attribute "a=rtcp-mux")) |  | No port allocated (Note 3) |  | No port allocated (Note 2) |  | Out of scope |  |
|  |  | Yes in H.248 RD | No port allocated (Note 2) | No port allocated (Note 2) | No port allocated (Note 3) | Out of scope |
|  |  | Yes in H.248 LD & RD | No port allocated (Notes 2, 3) | No port allocated (Notes 2, 3) | No port allocated (Note 3) |  |
|  | Yes in H.248 RD | No | No port allocated | No port allocated | No port allocated (Note 2) | Out of scope |
|  |  | Yes in H.248 LD | No port allocated (Note 3) | No port allocated (Note 3) | No port allocated (Note 2) | Out of scope |
|  |  |  |  | Yes in H.248 RD |  | No port allocated |  | No port allocated |  | No RTCP control flow at all, thus: No port allocated (Notes 2, 3) (rule R2 ("rsb=OFF" overrules SDP attribute "a=rtcp-mux")) |  | Out of scope |  |
|  |  | Yes in H.248 LD & RD | No port allocated (Note 3) | No port allocated (Note 3) | No port allocated (Notes 2, 3) | Out of scope |
|  | Yes in H.248 LD & RD | No | No port allocated (Note 2) | No port allocated (Note 2) | No port allocated (Note 2) | Out of scope |
|  |  | Yes in H.248 LD | No port allocated (Notes 2, 3) | No port allocated (Notes 2, 3) | No port allocated (Note 2) | Out of scope |
|  |  | Yes in H.248 RD | No port allocated (Note 2) | No port allocated (Note 2) | No port allocated (Notes 2, 3) | Out of scope |
|  |  | Yes in H.248 LD & RD | No port allocated (Notes 2, 3) | No port allocated (Notes 2, 3) | No port allocated (Notes 2, 3) | Out of scope |

The common legend for Tables 4/…:

NOTE 1 –The management of LS(A) and LS(P) resources is under control of the media gateway unless the MGC explicitly adds "gm/esas = ON" and/or "gm/lsps = ON" and signales corresponding local source address and/or port. In such a case there might be an inconsistency if" a=rtcp:x" is present in Local and if "gm/esps = ON" and "gm/lsp=y" with "x ≠ y" are present in LocalControl. Here this potential inconsistency is not further investigated. NOTE 2 – This is a semantic contradiction, however the MG shall ignore the "a=rtcp:" attribute. NOTE 3 – This is a semantic contradiction, however the MG shall ignore the "a=rtcp-mux" attribute.

Annex A

H.248 profiles with and without support of RTCP Handling Package

(This annex forms an integral part of this Recommendation.)

## A.1	Background

## There are H.248 profiles with and without support of the RTCP handling package for ITU-T H.248 media gateways with RTP bearer traffic. Only such MG types are in scope of this Recommendation.

## A.2	H.248 profiles without support of RTCP Handling Package

## Historically, all H.248 profiles related to MG types with scope on "physical-to-RTP" interworking, - such as PSTN/ISDN residential, access and trunking gateways -, were defined without usage of the RTCP handling package. An RTCP port was always and implicitly allocated based on the "N+1" rule, independent whether RTCP was finally used or not.

## But the RTCP handling package could be also added to such H.248 profiles, as well as SDP elements for RTCP port allocation control.

## This Recommendations takes into account such kind of possible profile upgrading scenarios.

## A.3	H.248 profiles with support of RTCP Handling Package

The RTCP handling package was produced when the first H.248 profiles for IP-IP MGs were defined. Such MGs support multiple modes of operation concerning bearer traffic handling, particularily the so called media-agnostic (RTP unaware) mode created the demand for an explicit signalling element concerning the reservation and allocation of two IP transport connection endpoints for the two flow components of the H.248 stream.

This Recommendations takes into account such kind of possible profile upgrading scenarios.

Bibliography

[b-IETF RFC 3551]		IETF RFC 3551 (2003), RTP Profile for Audio and Video Conferences with Minimal Control.
<http://www.ietf.org/rfc/rfc3551.txt>

[b-IETF RFC 3556]		IETF RFC 3556 (2003), Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth.
<http://www.ietf.org/rfc/rfc3556.txt>

[b-IETF RFC 3890]		IETF RFC 3890 (2004), A Transport Independent Bandwidth Modifier for the Session Description Protocol (SDP).
<http://www.ietf.org/rfc/rfc3890.txt>

____________________