| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4493 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: completion of TLS basic session control package (clause 8 & Appendix IV) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes changes based on the resolution of all final comments and editior's notes in clause 8 plus correspondent updates in Appendix IV.

1	Introduction

H.248.TLS is technically already fairly stable, apart from some final issues to be fxed.

2	Discussion

2.1	Blocking establishment requests

There is following main comment:

- clause 8.1.1: Editor's note:

{Editor's note (2013-06 meeting): following items (same as in H.248.TCP) were identified and need to be resolved:

C1: the property semantics are focusing on the ingress direction (tbc); i.e., when property not relevant in outgoing direction, then explicit text should be added.

C2: Signal EstBNC should implicitly overwrite the property value, i.e., replacing blocked by unblocked. Correspondent updates should be added. 
Comment (2013-10): You cannot have a signal that changes the value of a property.

Given that the Signal would have to be sent we don’t think it’s an issue to send a property “unblocking” the termination and sending the establishment.

The interaction between sending the Estab or Release signal whilst in state “blocked” would have to be described.

Also what is the effect if the SEP has a TLS connection state of Established and the blocking property is suddenly set to “blocked”.}

Resolution:

Following assumptions are made, - as in case for H.248.TCP (see AVD-4486):

- MGC must explicitly unblock (sent "tlsbsc/bcep = unblocked"). E.g., when requesting establishment by "tlsbsc/EstBNC"

- The blocking property takes only effect in H.248 connection states Idle & Blocked, but not in state Established. I.e., When in state Established, then the TLS session must be firstly released (=> state Idle) before new incoming TLS session establishment requests could be blocked.

- When in state Blocked, then correspondent H.248 command requests (such as signal EstBNC) shall be replied by an error code.

2.2	TLS endpoint role

Clause 8.6.2:

{Editor's note (2013-10 meeting): the "TLS endpoint role" concept should be replaced by originating and terminating TLS session control procedures.}

Resolution: aligned with H.248.TCP resolution

2.3	Unsuccessful establishment

Clause 8.6.2 – Table 5 – TLS security session establishment - Results:

{Editor's note (2013-11): error code is not possible … because bearer control procedures are still ongoing when H.248 command.reply is already sent. Either a Notify or ServiceChange based method …}

Resolution: aligned with H.248.TCP resolution

2.4	TLS session release

Clause 8.6.5: there are a number of electronic comments, as well as an open check concerning potential overlappings with clause 10.

Resolution: correspondent updates are proposed.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4474).

Change proposal #1:

[Begin Proposed Correction]

# 8	TLS basic session control package

|  | Package Name: | TLS basic session control package |
| --- | --- | --- |
|  | Package ID: | tlsbsc (0x####) |
|  | Description: | This package provides the functionality to establish and /release a TLS (or DTLS, see [ITU-T H.248.DTLS]) based security session. Furthermore it includes the capability to block the start of TLS session establishment procedures at the MG bearer interface. NOTE 1 – The package design follows similar connection-oriented bearer (or media) protocols, such as the H.248 tcpbcc package [ITU-T H.248.TCP]. Terminology: "(D)TLS security session" correlates to "TCP bearer connection" and the "Q.1950 BNC" concept. NOTE 2 – The differences between DTLS and TLS are subject of [ITU-T H.248.DTLS]. |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

### 8.1.1	Security session establishment blocking

|  | Property Name: | Security session establishment blocking |
| --- | --- | --- |
|  | Property ID: | bceb (0x0001) NOTE 1 - Identifier aligned with H.248 tcpbcc package. |
|  | Description: | This property defines the MG behaviour before and during TLS session establishment. NOTE 2 - See Annex A concerning a state model based illustration. |
|  | Type: | Enumeration |
|  | Possible values: | See Table 2 Table 2 – Security session establishment preparedness (of TLS endpoints) Value; MG behaviour. "blocked" (0x0001); - Bearer resources shall be prepared (i.e., resources for TLS endpoint shall be reserved and allocated to H.248 termination/SEP); - TLS role assignment: still open, not yet decided; - TLS session still closed; - H.248 counterpart state = "Blocked" (Figure A.1); - Any incoming TLS packet shall be silently discarded (e.g., TLS ClientHello requests shall be not acknowledged). - Any outgoing TLS session control procedures shall be not affected (i.e., also not due tointernal interlinkage (see clause 9)). - H.248 command requests which may not be executed in state Blocked shall be replied by an error code.. "unblocked" (0x0002); - MG shall be ready for incoming or/and outgoing TLS security session control procedures; - TLS role assignment: relates to an (initial) TLS server role effectively; - TLS session state: transitioning from closed to …; - H.248 counterpart state = "Idle" (Figure A.1); - TLS security session control options: - Incoming TLS ClientHello: TLS server role confirmed; - Signal tlsbsc/EstBNC: start of TLS session establishment by sending of TLS ClientHello; TLS client role assigned. |
|  | Default: | "Unblocked" NOTE - H.248 profile specifications could redefine other default values. |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 8.2	Events

### 8.2.1	TLS session state change ("BNC change")

|  | Event Name: | TLS session state change |
| --- | --- | --- |
|  | Event ID: | BNCChange (0x0001) |
|  | Description: | This event occurs whenever a change to a (TLS) bearer network connection occurs. For example a bearer (i.e., a TLS security session) has been established or a bearer has been released. The event is related to a simplified state transitioning model, which is described in Annex A. |

#### 8.2.1.1	EventsDescriptor parameters

##### 8.2.1.1.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. This is used to request the MG to notify it of a particular bearer event. |
|  | Type: | Sublist of Enumeration |
|  | Optional: | Yes |
|  | Possible values: | Table 3 defines the possible values. Table 3 – Parameter values Value; Name; Final state (Annex A.3); Semantic. Est, [0x01]; Bearer Established; Established; Notify MGC of successful TLS security session establishment.. Rel, [0x05]; Bearer Released; Idle; Notify MGC of successful TLS security session release.. |
|  | Default: | Type=[Est,Rel] The MGC would be then automatically notified about all state transitions when subscribing to event tlsbsc/BNCChange. |

#### 8.2.1.2	ObservedEventsDescriptor parameters

##### 8.2.1.2.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | This is used to indicate what change has occurred to the TLS bearer. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | Est [0x01] Bearer Established Rel [0x05] Bearer Release |
|  | Default: | None |

## 8.3	Signals

### 8.3.1	Establish (BNC)

|  | Signal Name: | Establish BNC |
| --- | --- | --- |
|  | Signal ID: | EstBNC (0x0001) |
|  | Description: | This signal triggers the bearer control function to send bearer establishment (i.e., this signal is used to establish the TLS security session: the MG takes the TLS client role and initiates the start of a TLS session establishment by sending a TLS ClientHello request). This signal takes only effect in H.248 connection state Idle (and would be replied by an error code when in H.248 connection states Blocked or Established). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 8.3.1.1	Additional parameters

None.

### 8.3.2	Release (BNC)

|  | Signal Name: | Release BNC |
| --- | --- | --- |
|  | Signal ID: | RelBNC (0x0002) |
|  | Description: | This signal triggers the bearer control function to send bearer release (i.e., this signal is used to close the TLS security session: the MG sends a TLS alert close_notify request). This signal takes only effect in H.248 connection state Established (and would be replied by an error code when in H.248 connection states Blocked or Idle). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 8.3.2.1	Additional parameters

None.

## 8.4	Statistics

None.

## 8.5	Error Codes

None.

## 8.6	Procedures

### 8.6.1	TLS endpoint creation

The MGC is responsible for creating a TLS session enabled SEP in the MG. This may be indicated on the H.248 interface through the use of the SDP "m=" element <proto>.

The MG shall reserve "TLS bearer resources" (such as a "TLS encryption/decryption function") and allocates them to the SEP. The TLS endpoint role (client/server) is not (yet) determined.

NOTE 1 – This functionality is synonym to the "bearer type indication" semantic as defined by the BNC Characteristics property (BNCChar) of the bearer characteristics package (BCP), see clause A.3.1.1/[ITU-T Q.1950].

### 8.6.2	TLS endpoint role assignment

The TLS endpoint role, client or server, is primarily relevant during the establishment phase of communication (of perspective of H.248 gateways). The TLS endpoint role assignment shall prepare the MG for an incoming or outgoing TLS security session establishment (clause 8.6.3) procedure. Table 4 summarizes MGC actions and their results in TLS role assignments.

Table 4 – TLS endpoint role assignment (at establishment phase)

| No. | TCP role | H.248 indication | MG behaviour |
| --- | --- | --- | --- |
| 1 | Not yet assigned | H.248 SEP state "Blocked", either due to explicit signalling of bceb property or correspondent default value | Any incoming TLS PDU shall be silently discarded. |
| 2 | TLS server | H.248 SEP state "Idle" by "unblocking" the SEP | Expect incoming (TLS) ClientHello, subsequent completion of the establishment related handshake procedure. |
| 3 | TLS client | H.248 SEP state "Idle" of SEP plus signal EstBNC enabled. | Start of TLS session establishment related handshake procedure. |

The MGC may arm events tlsbsc/BNCChange and g/cause (in H.248 connection state Idle) in order to be notified about successful or unsuccessful TLS security session establishment.

### 8.6.3	TLS security session establishment

The incoming and outgoing side of TLS security session protocol procedures is given by the assigned TLS role (see above clause 8.6.2) by the MGC.

The MG shall initiate the TLS security session setup according to the indicated role and the TLS security session setup handshake is performed by the MG without any further involvement of the MGC. The end of the TLS security session establishment phase may be summarized according Table 5:

Table 5 – TLS security session establishment - Results

| TLS role | Result of TLS security session establishment | MGC notification? |
| --- | --- | --- |
| Client (i.e., outgoing establishment side) | Successful | MGC dependent on subscription of event and parameter value tlsbsc/BNCChange{Type=[Est]} . |
|  | Unsuccessful | Unsuccessful establishment may be indicated to the MGC via a correspondent ServiceChange procedure or event g/cause. |
| Server (i.e., incoming establishment side) | Successful | As per client side. |
|  | Unsuccessful | As per client side. |

### 8.6.4	TLS application data transfer

#### 8.6.4.1	MG external

The TLS session is ready for application data transfer when remote and local TLS endpoints have both completed the TLS handshake procedures for session establishment.

The StreamMode property of the LocalControl Descriptor affects the application data flow rather than the TLS control information used to establish or close the TLS session. The requested MG behaviour is defined by Table 6:

Table 6 – Impact of StreamMode on TLS bearer traffic at external MG interface

| Traffic direction | StreamMode settings | MG behaviour |
| --- | --- | --- |
| Outgoing | Sending enabled (i.e., values SendOnly, SendRecv, LoopBack) | All TLS PDUs sent |
|  | Sending disbled (i.e., values RecvOnly, Inactive) | Only TLS PDUs related to: - TLS handshake protocol - TLS cipher change protocol and - TLS alert protocol but not native application data over TLS record protocol are sent. |
| Incoming | Receiving enabled (i.e., values RecvOnly, SendRecv, LoopBack) | All TLS PDUs received |
|  | Receiving disbled (i.e., values SendOnly, Inactive) | Only TLS PDUs related to: - TLS handshake protocol - TLS cipher change protocol and - TLS alert protocol but not native application data over TLS record protocol are sent. |

#### 8.6.4.2	MG internal

This clause is only relevant for H.248 contexts with TLS SEPP(s).

Thus application data may be received at a SEP in time where the other SEP of the context is not yet able to process or send the application data. The behaviour of the MG in such a situation is implementation and/or application dependent. If application independent or unambiguous behaviour would be required, then the interlinkage capability as profiled by the tlsesc package (see clause 9.1) should be used.

### 8.6.5	TLS session release

The trigger for a TLS session termination might come from different sources, see Figure 7:

Figure 7 – Principle stimuli for triggering TLS session release

- Bearer control protocol – Security session: The TLS session is terminated by the remote TLS endpoint by sending a (TLS) close_notify alert or a fatal alert;

- Bearer control protocol – Transport connection: The underlying L4 transport connection is terminated;

- TLS protocol: A non-recoverable error condition has been detected in the TLS function, e.g. the authentication of a received TLS segment has failed;

- The MGC indicates the closure of the TLS session by the signal tlsbsc/RelBNC;

- The release of the security session of the partner SEP triggers the release autonomously in the MG (Note: not supported by base package tlsbsc, related to inter-linkage property tlsesc/sepplink, see clause 9.1.1);

- In case the MG acts in an application aware mode, the application part in the MG may indicate the release of the TLS session.

#### 8.6.5.1	Bearer control protocol – Security session: TLS alert protocol

NOTE – The defined behaviour in this clause is consistent with the TLS session maintenance package according to clause 10. It has to be further noted that there might be H.248 profiles with and without this TLS session maintenance package.

In case a TLS alert is received that leads to the termination of the TLS session, then the behaviour of the TLS protocol is as specified in [IETF RFC 5246], i.e. in case of a received TLS close_notify the TLS endpoint shall reply a TLS close_notify on its own and the TLS session is closed.

Impact on underlying IP transport connection:

- The MG shall not autonomously initiate the release of the (if connection-oriented) IP transport connection as well.

NOTE - extension package tlsesc allows to tightly couple bearer control procedures for L4 and TLS, follow protocol layers interlinkage, as referred byclause 9.1.2.

- If requested the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}. Handling of the TLS-specific AlertDescription as received in the TLS alert message is for further studies.

Impact on H.248 SEP and possibly SEPP:

- The SEP as such still exists after a successful TLS session release (state Idle). Removal of the SEP follows usual H.248 SUBtract.req or MODify.req commands.

- The SEPP interlinkage capability (as referred by clause 9.1.1) allows the trigger of a TLS session release at the partner SEPP.

#### 8.6.5.2	Bearer control protocol – Transport connection: L4 bearer release

In case the underlying IP transport connection is released, e.g. due to a received TCP-FIN or TCP-RST in case of TCP transport, then

- default mode (base package): the MGC shall be notified (by the MG) about the L4 state change (e.g., either by subscription to a correspondent event, an error or ServiceChange indication) and provide instructions for further proceeding with the TLS layer.

- enhanced mode (extension package): the MG may autonomously and immediately initiate the closure of the TLS session (dependent on used protocol layers interlinkage, as referred by clause 9.1.2). The release procedures of the IP transport connection shall be delayed in order to complete firstly successfully a TLS session release procedure (see also clause V.4.2.2.1).

NOTE - extension package tlsesc indicates the option of coupled bearer control procedures for L4 and TLS, see clause 9.1.2.

If requested, the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}.

#### 8.6.5.3		TLS protocol: non recoverable error condition detected

NOTE – The defined behaviour in this clause is consistent with the TLS session maintenance package according to clause 10. It has to be further noted that there might be H.248 profiles with and without this TLS session maintenance package.

In case an error condition is detected locally in the TLS protocol layer which results in the termination of the TLS session, the procedures as defined in [IETF RFC 5246] apply. A TLS alert shall be sent to the remote TLS endpoint with the AlertLevel set to "fatal" and the AlertDescription is provided accordingly.

If requested the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}, when H.248 TLS state transitioned from Established to Idle.

#### 8.6.5.4	H.248: MGC driven TLS session release

In case the H.248 signal tlsbsc/RelBNC is received, the TLS session closure shall be initiated by the MG by sending a TLS close_notify alert towards remote TLS entity.

#### 8.6.5.5	MG autonomous release initiated on partner SEP

This capability is not supported by the base package, it is subject of the extension package tlsesc indicated SEPP interlinkage capability, see clause 9.1.1.

#### 8.6.5.6	Release indication received from TLS application

In case the MG operates in a TLS-application aware mode, an error condition in the application aware entity might be detected that requires the release of the TLS session. In this situation the TLS session shall be closed autonomously by sending a TLS alert (close_notify).

If requested the MGC shall be notified by the event tlsbsc/BNCChange{Type=Rel}, when H.248 TLS state transitioned from Established to Idle.

The underlying IP transport shall be not affected (base package behaviour).

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix IV

Illustration of protocol semantics 
of the TLS basic session control package

(This appendix does not form an integral part of this Recommendation.)

## IV.1	Overview

The tlsbsc package (clause 8) defines the basic TLS security session control with scope on support for establishment and release. The illustrated use cases are abstracted examples.

## IV.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TLS) is highlighted besides the H.248 interface.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TLS session endpoint.

## IV.3	Establishment of TLS security sessions

### IV.3.1	Successful establishment – Terminating side

See Figure IV.1, termed as use case (E.1):

Figure IV.1 – Successful establishment – Terminating side

More details for this example:

- The MGC blocks the start of TLS session establishment (1).

- Early incoming TLS messages are discarded by the MG (2).

- The MGC unblocks the TLS SEP (3).

- TLS session establishment (5 to 7); MG acting as TLS server.

- Optional notification of MGC concerning available TLS session for application data transfer (8), if subscription to event by MGC (4).

### IV.3.2	Successful establishment – Originating side

See Figure IV.2, use case (E.2).

Figure IV.2 – Successful establishment – Originating side

More details for this example:

- Steps 1 to 3 as in clause IV.3.1

- TLS session establishment (5 to 7), triggered by MGC (4) (please note previous explicit "unblocking" step 3); MG acting as TLS client.

- Optional notification of MGC concerning available TLS session for application data transfer (9), if subscription to event by MGC (8).

### IV.3.3	Unsuccessful establishment

The TLS security session negotiation handshake is basically not completed.

## IV.4	Release of TLS security sessions

### IV.4.1	Successful release – Terminating side

See Figure IV.4, termed as use case (R.1).

Figure IV.4 – Successful release – Terminating side

More details for this example:

- MG receives an incoming TLS close_notify alert (2), the indication for TLS session release.

- The MG acknowledges the release request (3).

- Optional notification of MGC concerning successfully released TLS session (4), if subscription to event by MGC (1).

### IV.4.2	Successful release – Originating side

See Figure IV.5, use cases (R.2) illustrates an outgoing TLS session release:

Figure IV.5 – Successful release – Originating side

More details for this example:

- MGC triggers the TLS session release (1).

- MG sends an outgoing TLS close_notify alert (2), (the indication for TLS session release) towards remote TLS endpoint X.

- Successful release (3).

- Optional notification of MGC concerning successfully released TLS session (5), if subscribed to the event by MGC (4).

### IV.4.3	Unsuccessful release

An unsuccessful TLS session release procedure would imply still that the MG remains in H.248 state Established and that the MGC could possibly suspect a yet established TLS session. There are multiple options how such protocol deadlocks could be resolved. The situation could be e.g. cleared by the subtraction of the termination by the MGC.

[End Proposed Correction]

____________________