| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4495 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: completion of TLS capability negotiation package (clause 10) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution tries to fix the remaining open items related to the negotiation process of a TLS profile during TLS session establishment. There are two parallel related contributions:

AVD-4498 on the aspect of TLS session resumption

AVD-4545 on modelling of data object "TLS profile" (illustrates a structural model and indicates behaviour modeling aspects).

Discussion

- Clause 10.1.2:

{Editor's note (2012-02): the scope needs to be clarified, i.e., either Root or LocalControl level per H.248 profile, or both. Contributions are solicited. => (2013-06): might be addressed in clause 10.6.2}

{Editor's note (2013-06 meeting): following items were identified and need to be resolved:

C1: clarify possible interaction between non-root and root termiantion usage, or define exclusive approach, etc).
This comment is relevant for all package properties.}

{Comment to clause 10 as such (2013-06 meeting): there are

- two negotiation methods ("profile" vs "individual") and

- two termination levels ("Root" vs "non-Root")

There are four combinations, and the mutual exclusion should be revisited from that perspective.}

Resolution:

Mutual exclusion conditions introduced for first package version in order to simplify the package specification. Limitations may be reopened again in a future package version.

It has to be noted that such restrictions would not prevent any existing use case (e.g., 3GPP Rel-12 work item eMEDIASEC is limited to a single "TLS domain profile" and limited in MGC capabilities concerning individual TLS profile parameter control.

- Clause 10.1.3:

{Editor's note (2013-11 meeting): discussion

comment: "RFC5246 indicates that TLS_RSA_WITH_AES_128_CBC_SHA is mandatory to implement should we specify a default per version?"

consideration "The package should be orthogonal to TLS versions. Otherwise: TLS version specific defaults?"}

Resolution:

We should not specify a default per version in our opinion due to 
a) the version aspect and
b) TLS evolution as such, i.e. RFC 5246 just reflects a snap shot ("IETF WG TLS is still a very high active group, TLS is revisited and suggested to be enhanced in many directions").

- Clause 10.6.3.2:

RFC5246 indicates “The cipher suite list, passed from the client to the server in the 
   ClientHello message, contains the combinations of cryptographic
   algorithms supported by the client in order of the client's
   preference (favorite choice first).” Should the MG follow the same behaviour? Perhaps an AuditCap returns the MGs preferential list and a AuditVal returns the order that the property was set. i.e. The MGC preference.

Resolution:

That's a good catch and related to the lifetime of a "TLS profile" object from the creation date of a TLS-enabled termination over modification dates due to initial TLS session negotiation and optional subsequent TLS session resumptions (see also parallel contributions).
We'd like to suggest to split H.248 procedures in 
a) auditing the TLS MG profile (at root level) and
b) auditing the TLS endpoint profile (at non-root level).
Such a change may not fully satisfy all aspects of the comment, but should be a resolution in the right direction.

- Clause 10.6.5.9:

{Editor's note (2012-09 meeting): Current assumption is that no explicit MGC control of the renegotiation is required, e.g. trigger renegotiation through a signal.
However, to be considered: "If the MGC provides the property tlscn/rp, then the MG will use this value instead of the provisioned default TLD Domain profile's "rp" value }

Resolution:

The procedure should be enhanced by adding this information.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4474).

Change proposal:

[Begin Proposed Correction]

# 10	TLS capability negotiation package

|  | Package Name: | TLS capability negotiation package |
| --- | --- | --- |
|  | Package ID: | tlscn (0x####) |
|  | Description: | When this package is not used by an H.248 profile, then the MG will autonomously negotiate TLS protocol configurations with the peer TLS node. The aimed negotiation behaviour would be then provisioned in the MG (but the managed objects used does not necessarily correspond with the H.248 signalling objects of this package). This package provides tools for the MGC to control bearer-level TLS negotiations between the MG and the remote TLS endpoint. The MGC could impact TLS negotiations in terms of: - TLS protocol parameter values (such as TLS version); - Quality/goals of negotiation results; - Time limits for negotiations (e.g. due to realtime aspects of the overall application/service); or/and - Handling of unsuccessful negotiations. There are two options for negotiation of TLS protocol configurations: - MGC may indicate a TLS profile (or multiple) as such; - MGC may control individual TLS protocol parameters. Both methods shall be mutually exclusive in this package version. |
|  | Version: | 1 |
|  | Extends: | None. |

## 10.1	Properties

### 10.1.1	TLS Domain Profile Identifier

|  | Property Name: | TLS Domain Profile Identifier |
| --- | --- | --- |
|  | Property ID: | dpid (0x0001) |
|  | Description: | This property identifies the TLS domain profile that shall be ideally used by the MG to setup the TLS security session. The actually applied, so called TLS endpoint profile is the match result of locally supported TLS capabilities by the MG (termed as TLS MG profile) with requested TLS domain profiles. It should be noted that the normal case would be aligned TLS profile capabilities, i.e.: The applied "TLS endpoint profile" = requested "TLS domain profile". |
|  | Type: | Sub-list of String |
|  | Possible values: | Any string as agreed between MGC and MG. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

### 10.1.2	TLS Versions

|  | Property Name: | TLS Versions |
| --- | --- | --- |
|  | Property ID: | tlsv (0x0002) |
|  | Description: | This property indicates which TLS versions should be negotiated with the remote TLS endpoint. |
|  | Type: | Sub-list of String |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the TLS version as defined in the TLS protocol version related IETF RFCs (e.g. [IETF RFC 5246, Appendix A.1]). The hexadecimal representation of the major version is coded in the first two characters, while the hexadecimal representation of the minor version is coded in the last two characters of the string. The hexadecimal representation of the versions omits the "0x", and if needed, a leading "0" is added for padding. Over-decadic digits are to be represented by lower case characters ("a".."f"). |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

### 10.1.3	Cipher Suites

|  | Property Name: | Chipher Suites |
| --- | --- | --- |
|  | Property ID: | cs (0x0003) |
|  | Description: | This property indicates the cipher suites and their precedence (in decending order of preference) for the negotiation with the remote TLS endpoint. |
|  | Type: | Sublist of String |
|  | Possible values: | Each string consists of 4 hexadecimal digits and represents the cipher suite in accordance to the TLS Cipher Suite Registry of IANA (http://www.iana.org/assignments/tls-parameters/tls-parameters.xml). The first value of the value pair as defined by the TLS Cipher Suite Registry of IANA shall be coded into the first two characters, and the second value shall be coded into the last two characters of the string. Thus each value in its hexadecimal representation shall be converted into the double-hexdigit string. The "0x" shall be omitted and if needed, a "0" is used for padding. Over-decadic digits are to be represented by lower case characters ("a".."f"). |
|  | Default: | Provisioned NOTE – A particular TLS protocol version may define default value(s). Thus, an H.248 profile which defines a default version for TLS could also specify default value(s) for this property. |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

### 10.1.4	Compression Methods

|  | Property Name: | Compression Methods |
| --- | --- | --- |
|  | Property ID: | cm (0x0004) |
|  | Description: | This property declares the list of compression methods and their preference (in descending order) for the negotiation with the remote TLS-endpoint. |
|  | Type: | Sub-list of unsigned Integer |
|  | Possible values: | The values correspond to the values allocated by IANA for TLS Compression Method Identifiers (see http://www.iana.org/assignments/comp-meth-ids/comp-meth-ids.xml). |
|  | Default: | “CompressionMethod.null” |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

### 10.1.5	Support for Session Resumption

|  | Property Name: | Support for Session Resumption |
| --- | --- | --- |
|  | Property ID: | ssr (0x0005) |
|  | Description: | This property declares the support for the TLS session resumption. |
|  | Type: | Boolean |
|  | Possible values: | False Session Resumption shall not be supported True Session Resumption shall be supported |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

### 10.1.6	Support for Renegotiation of the Security Context

|  | Property Name: | Support of Renegotiation of the Security Context |
| --- | --- | --- |
|  | Property ID: | srsc (0x0006) |
|  | Description: | This property declares whether the renegotiation of the security context shall be supported. |
|  | Type: | Boolean |
|  | Possible values: | False Renegotiation shall not be supported True Renegotiation shall be supported |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

### 10.1.7	Renegotiation Period

|  | Property Name: | Renegotiation Period |
| --- | --- | --- |
|  | Property ID: | rp (0x0007) |
|  | Description: | This property declares the period in seconds after which a renegotiation of the security context shall be initiated by the MG. The value 0 indicates that the MG shall not initiate a renegotiation. |
|  | Type: | Unsigned Integer |
|  | Possible values: | Any non-negative value |
|  | Default: | 0, unless provisioned otherwise |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) |
|  | Characteristics: | Read/Write |

### 10.1.8	Client Authentication Required

|  | Property Name: | Client Authentication Required |
| --- | --- | --- |
|  | Property ID: | car (0x0008) |
|  | Description: | This property declares if the MG – if acting as a TLS server – shall authenticate the remote TLS client. |
|  | Type: | Boolean |
|  | Possible values: | False In case the MG acts as a TLS server, the client shall not be authenticated. True In case the MG acts as a TLS server, the client shall be authenticated. |
|  | Default: | True, unless provisioned otherwise |
|  | Defined in: | LocalControl (ephemeral IP terminations), TerminationState (Root termination) Both methods shall be mutually exclusive in this package version. |
|  | Characteristics: | Read/Write |

## 10.2	Events

None.

## 10.3	Signals

None.

## 10.4	Statistics

None.

## 10.5	Error codes

None.

## 10.6	Procedures

### 10.6.1	Principles of TLS capability negotiations – overview and scope

## The behaviour of TLS capability negotiations is principally influenced by following dimensions:

- Package usage or package-less operation (see clause 10.6.2);

When package is used, then there are

- two methods ("profile" vs "individual"), and

## two termination levels ("Root" vs "non-Root")

## concerning the indication of a requested TLS domain profile by the MGC to the MG (see clause 10.6.3).

## There is consequently a high flexibility in supporting many TLS capability negotiation variants (from H.248 perspective), however, the allowed applicable procedures are limited in tlscn package version 1 in order to simplify the operation of an H.248 profile using this package. Such limitations may be reopened again in a future version of this package.

### 10.6.2	General default behaviour (MGC does not influence TLS capability negotiations)

Package tlscn is optional for H.248 profile specifications with TLS support. When the package is not used, then the MG shall independently negotiate TLS configuration(s) with remote TLS endpoints.

Such MG-autonomous TLS capability negotiations shall be compliant to [IETF RFC 5246]. The negotiations would be based on the TLS MG profile and possibly preprovisioned negotiation policies.

### 10.6.3	MGC-controlled TLS capability negotiations: Options

There are two options for MGC-controlled TLS capability negotiations, with the aim to determine the TLS domain profile instance, which is to be used by the MG to form the TLS endpoint profile (by also taking into account the TLS MG profile), see Table 9.

Table 9 – Variants of MGC-controlled TLS capability negotiations

| Method | H.248 elements | Protocol procedures, see: |
| --- | --- | --- |
| a) TLS profile approach | Property: tlscn/dpid | clause 10.6.4 |
| b) Individual TLS protocol parameter control | Properties: tlscn/tlsv tlscn/cs tlscn/cm tlscn/ssr tlscn/srsc tlscn/rp tlscn/car | clause 10.6.5 |

Both methods shall be mutually exclusive in this package version. Thus, an H.248 profile with tlscn v1 package support should select one method.

### 10.6.4	Determining TLS related capabilities (TLS MG profile)

### The supported TLS capabilities are subject of the "TLS MG profile". The TLS MG profile is a characteristic at MG level. The auditing of a TLS MG profile shall be therefor limited on the Root termination only.

#### 10.6.4.1	Auditing supported TLS protocol profiles

The MGC may access supported TLS protocol profile(s) by auditing property tlscn/dpid. The identifier of a particular TLS protocol profile relates to a reference on a TLS protocol profile description, which might be for instance explicitly part of an H.248 profile definition or documented anywhere else.

#### 10.6.4.2	Auditing individual supported TLS protocol capabilities

The MGC may determine the properties of the TLS MG profile by auditing the value of Supported TLS Versions (tlsv), Supported Cipher Suites (scs), Supported Compression Methods (scm) and Support for Session Resumption (ssr).

The Supported TLS Versions property provides the TLS versions supported by the MG without any specific sequence.

The Supported Cipher Suite property shall include the cipher suite TLS_NULL_WITH_NULL_NULL ("0000"), which is used as the initial cipher suite during the first handshake for the TLS session, but which must not be used for a cipher suite negotiation.

The list of supported cipher suites and the list of supported compression methods are provided without any specific sequence.

TLS session resumption may be used to avoid the need for a negotiation of the new security context. The Support for Session Resumption property provides the MGC with an indication whether the MG supports this feature.

### 10.6.5	Indication of TLS profile(s) to MG by MGC

The MGC may request TLS domain profile(s) support from the MG via property dpid, leading to following MG behaviour:

- Built intersection between locally supported TLS capabilities (the "TLS MG profile") with indicated TLS domain profile(s), resulting in a TLS endpoint profile (see also clause 14);

- The TLS endpoint profile shall be the baseline for TLS negotiations with the remote TLS entity (during TLS security session establishment/resumption procedures).

### 10.6.6	Indication of individual TLS protocol capabilities to MG by the MGC

In this case the TLD Domain profile instance to be used by the MG is (partially) defined by the MGC by the actually signalled tlscn/x properties with "x != dpid". Those tlscn/x properties, which are not explicitly signalled by the MGC assume their values as provisioned in MG's TLS MG profile.

#### 10.6.6.1	TLS protocol parameter "TLS version"

The list of TLS versions used for the TLS procedures of the session is determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. This list is supposed to be provisioned (ie., the property value for dpid) on the MG as typically default configuration.

NOTE – The TLS endpoint profile again (see also clause 14) relates to the intersection/match of TLS MG profile (as provisioned on MG) and individual explicitly signalled tlscn/x properties with "x != dpid".

In case the MGC signals the property TLS Versions (tlsv), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

In case the MG acts as a TLS client, the highest TLS versions as defined by the specific TLS endpoint profile are included into the initial TLS handshake message (ClientHello). In case the MG acts as a TLS server, the MG shall select the highest TLS version that is present in the list of TLS versions offered from the TLS client and in the TLS versions as defined by the TLS endpoint profile. If there is no match between both lists the TLS handshake is regarded as failed.

Unsuccessful TLS negotiations should be reported by error code #449 (Unsupported or Unknown Parameter or Property Value).

#### 10.6.6.2	TLS protocol parameter "cipher suite"

The prioritized list of cipher suites used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. The TLS domain profile may specify different cipher suites for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account.

In case the MGC signals property Chipher Suites (cs), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The MG will negotiate the cipher suite according to the TLS endpoint profile. In case the MG is acting as a TLS client, the cipher suites are included into the first TLS-handshake message (ClientHello) in the same sequence as defined by the TLS endpoint profile. In case the MG is acting as a TLS server, the Cipher Suites property is used to negotiate the cipher suite with the client according to the procedure that is applicable for the negotiated TLS version.

#### 10.6.6.3	TLS protocol parameter "compression method"

The list of compression methods used for the TLS handshakes of the session are determined by the intersection of the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid) and the TLS MG profile. The TLS domain profile may specify different compression methods for TLS clients and TLS server, thus the property TLS endpoint role (er) must be taken into account.

In case the MGC signals property Compression Methods (cm), the MG shall use this list instead. In this case the MGC must ensure the consistency of the list between the requirements of the TLS domain and the capability of the MG.

The negotiation of the compression methods follows the same principle as the negotiation of the cipher suite. The compression methods as defined by the TLS endpoint profile provides the MG with the list of methods to be used for the negotiation as defined by the procedure that is applicable for the negotiated TLS version.

#### 10.6.6.4	TLS protocol procedure "client authentication"

The MG's TLS server implementation shall support client authentication.

In case the TLS endpoint in the MG is configured as a TLS server, the client authentication indication is taken from the TLS domain profile as given by the property TLS-Domain-Profile Identifier (dpid).

In case the MGC signals property Client Authentication Required (car), the MG shall use this value instead. In this case the MGC must ensure the consistency of the value with the requirements of the TLS domain.

#### 10.6.6.5	TLS protocol procedure "self-signed client certificates"

In case the TLS endpoint in the MG is configured as a TLS server and client authentication is required, the type of client authentication is implicitly defined by the SDP attribute "a=fingerprint". If present in the Remote Descriptor, the client certificate shall be self-signed and shall be verified against the fingerprint.

#### 10.6.6.6	TLS protocol procedure "signed client certificates"

In case the TLS endpoint in the MG is configured as a TLS server, client authentication is required and no SDP fingerprint attribute is present, the client certificate will be verified against a list of root certificates as defined for the TLS domain profile. This list of root certificates is supposed to be configured per dpid on the MG.

#### 10.6.6.7	TLS protocol procedure "server authentication"

In case the TLS endpoint in the MG is configured as a TLS server, the MG shall provide the server certificate to the client as defined by the TLS domain profile.

#### 10.6.6.8	TLS protocol procedure "session resumption"

Use cases for TLS session resumption (with H.248 gateways) are for further studies.

#### 10.6.6.9	TLS protocol procedure "renegotiation support"

The TLS implementation in the MG shall provide support for renegotiation.

Whether renegotiation for a TLS session shall be enabled is determined by the TLS domain profile. When the MGC provides the property tlscn/rp, then the MG shall use this value instead of the provisioned default TLD Domain profile's "rp" value.

In case the MGC signals property Support of Renegotiation of the Security Context (srsc), the MG shall use this value instead.

The behaviour of the renegotiation of the security context is defined by the TLS endpoint profile, which indicates if renegotiation shall be supported or not, taking into account property Renegotiation Period (rp).

In case the TLS endpoint profile disables renegotiation, the MG shall never initiate a renegotiation. If receiving a renegotiation request from the remote TLS endpoint, the MG shall respond with a TLS no_renegotiation alert, as defined by the procedure that is applicable by the negotiated TLS version.

If renegotiation shall be supported, the MG shall accept and process a renegotiation request from the remote TLS endpoint. The initiation of the renegotiation procedure by the MG depends on the value of the Renegotiation Period property. If this property set to value "0", no renegotiation shall be initiated. If set to any other non-zero value, a timer with the duration of the Renegotiation Period is started each time a TLS handshake completed successfully. On expiration of the timer the MG shall trigger the renegotiation as defined by the procedure that is applicable by the negotiated TLS version. In case the MG receives a trigger for the renegotiation from the remote TLS endpoint while this timer is running, the MG shall cancel and restart this timer.

### 10.6.7	Auditing the actual TLS configuration (TLS endpoint profile)

The actual agreed TLS configuration between the local and remote TLS endpoint referes to the current value settings of a TLS endpoint profile and may be audited by the MGC via an AuditValue procedure. Such kind of auditing shall be limited on non-Root termination level.

[End Proposed Correction]

____________________