| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4496 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: completion of TLS session maintenance package (clause 11) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides final editorial updates in order to complete this package.

Discussion

There's one remaining, actually pure editorial task open, as indicated in clause 11.6 by

{Editor's note (2013-11): it was requested to expand the description by explicit text in all sub-clauses starting with "As per … clause …".}

Thus, contribution provides a proposed for expanded text.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4474).

Change proposal:

[Begin Proposed Correction]

# 11	TLS session maintenance package

|  | Package Name: | TLS session maintenance package |
| --- | --- | --- |
|  | Package ID: | tlsm (0x####) |
|  | Description: | This package defines capabilities for support of TLS alerts, which are exchanged between the remote and MG local TLS endpoint, following the TLS alert protocol. NOTE: Appendix V provides some background information concerning TLS alert protocol handling by H.248 gateways. This package version is limited to the so-called TLS error alerts, as defined by clause 7.2.2/[IETF RFC 5246]. The package provides capabilities for MGC triggered sending of TLS alerts by the MG, as well as a reporting mechanism to notify the MGC about received TLS alerts by the MG. |
|  | Version: | 1 |
|  | Extends: | None. |

## 11.1	Properties

None.

## 11.2	Events

### 11.2.1	TLS Error Alert

|  | Event Name: | TLS error alert |
| --- | --- | --- |
|  | Event ID: | mgea (0x0001) |
|  | Description: | This event indicates an abnormal condition for the TLS layer that does not lead to the immediate termination of the TLS session. This is typically an TLS alert with severity level "warning", but could be also an received "fatal" alert (see also Appendix V.2). The abnormal condition may be detected on the local or on the remote TLS endpoint. The event is limited to TLS error alerts and excludes the TLS closure alert event. The event shall be set at stream level. |

#### 11.2.1.1	EventsDescriptor parameters

None

#### 11.2.1.2	ObservedEventsDescriptor parameters

##### 11.2.1.2.1	Bearer level Alert Initiator

|  | Parameter Name: | Bearer level alert initiator |
| --- | --- | --- |
|  | Parameter ID: | blai (0x0001) |
|  | Description: | Indicates on which TLS endpoint the abnormal condition was detected. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | "local" (0x0001) Abnormal condition originated and is detected by the MG local TLS endpoint. "remote" (0x0002) Abnormal condition originated at remote TLS endpoint side, then reported to the the MG local TLS endpoint via TLS alert protocol by the remote TLS endpoint. |
|  | Default: | NA |

##### 11.2.1.2.2	Error Alert Type

|  | Parameter Name: | Error alert type |
| --- | --- | --- |
|  | Parameter ID: | eat (0x0002) |
|  | Description: | Indicates the alert description for the abnormal condition by referencing the TLS error alert type. The codepoints registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml). |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for TLS error alert codepoints, which are registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml). |
|  | Default: | N/A |

## 11.3	Signals

### 11.3.1	MGC triggered TLS error alert

|  | Signal Name: | MGC triggered TLS error alert |
| --- | --- | --- |
|  | Signal ID: | mgcea (0x0004) |
|  | Description: | This signal is used to indicate an abnormal condition for the TLS-session. The severity of the condition determines whether the session will be closed or whether it may continue. If the al parameter indicates "fatal", the session shall be terminated. NOTE - the TLS "close_notify(0)" alert is excluded because it does not belong to the TLS error alert category. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 11.3.1.1	Additional parameters

##### 11.3.1.1.1	Alert Level

|  | Parameter Name: | Alert Level |
| --- | --- | --- |
|  | Parameter ID: | al (0x0001) |
|  | Description: | Indicates the TLS alert level as defined in [IETF RFC 5246] , i.e., either a warning or fatal alert severity. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for "AlertLevel", refer to [IETF RFC 5246]. |
|  | Default: | NA |

##### 11.3.1.1.2	Alert Description

|  | Parameter Name: | Alert Description |
| --- | --- | --- |
|  | Parameter ID: | ad (0x0002) |
|  | Description: | Indicates the TLS alert description level as defined in [IETF RFC 5246]. |
|  | Type: | Unsigned Integer |
|  | Optional: | No |
|  | Possible values: | One of the values as defined for TLS error alert codepoints, which are registered by IANA within the TLS Alert Registry (see http://www.iana.org/assignments/tls-parameters/tls-parameters.xml). |
|  | Default: | NA |

## 11.4	Statistics

None.

## 11.5	Error codes

None.

## 11.6	Procedures

Generally, there are TLS alert specific H.248 procedures due to the nature of the TLS alert protocol, see Appendix V.

### 11.6.1	TLS warning error alerts

#### 11.6.1.1	General behaviour

The TLS session should be kept active (and not released) when TLS alerts with warning severity are exchanged between local and remote TLS endpoints.

#### 11.6.1.2	MG received TLS error alerts from remote TLS endpoint

The MGC may subscribe to be notified about TLS warning error alerts via event mgea. Whenever the MG receives an incoming TLS error alert at warning level, it shall notify theMGC by:

- mapping the TLS error alert type to H.248 observed events parameter mgea/eat equal to "IANA registered codepoint"; and

- tagging observed event parameter mgea/blai equal to "remote".

#### 11.6.1.3	MG locally generated TLS error alerts and sent to remote TLS endpoint

The MGC may subscribe to be notified about TLS warning error alerts via event mgea. Whenever there are such MG locally generated TLS error alerts, an outgoing TLS error alert at warning level is sent and the MG shall notify theMGC by:

- mapping the TLS error alert type to H.248 observed events parameter mgea/eat equal to "IANA registered codepoint"; and

- tagging observed event parameter mgea/blai equal to "local".

#### 11.6.1.4	MGC triggered TLS error alert, sent by the MG to remote TLS endpoint

When the MGC provides signal mgcea, then the MG shall generate a TLS alert message based on the information as contained in signal parameters al and ad, and sent the TLS alert towards the remote TLS endpoint.

### 11.6.2	TLS fatal error alerts

#### 11.6.2.1	General behaviour

Whenever a TLS fatal error alert is exchanged between local and remote TLS endpoints, then there will be always a subsequent, immediate release procedure of the TLS session (see also Appendix V.2).

#### 11.6.2.2	MG received TLS fatal error alerts from remote TLS endpoint

The MGC may subscribe to be notified about TLS fatal error alerts via event mgea. Whenever the MG receives an incoming TLS error alert at fatal level, it shall notify theMGC by:

- mapping the TLS error alert type to H.248 observed events parameter mgea/eat equal to "IANA registered codepoint"; and

- tagging observed event parameter mgea/blai equal to "remote".

The subsequently expected TLS closure_notify alert indicates a TLS session release request. The received TLS closure_notify alert would then trigger the TLS basic session control package (tlsbsc) procedures according clause 8.6.5.1.

#### 11.6.2.3	MG locally generated TLS fatal error alerts and sent to remote TLS endpoint

The MGC may subscribe to be notified about TLS fatal error alerts via event mgea. Whenever there are such MG locally generated TLS error alerts, an outgoing TLS error alert at fatal level is sent and the MG shall notify theMGC by:

- mapping the TLS error alert type to H.248 observed events parameter mgea/eat equal to "IANA registered codepoint"; and

- tagging observed event parameter mgea/blai equal to "local".

The MG shall send a subsequent TLS closure_notify alert in order to initiate an outgoing TLS session release procedure. The outgoing TLS session release procedure shall be either triggered by the MGC (see clause 8.6.5.4) or autonomously started by the MG itself (see clause 9.6.2).

#### 11.6.2.4	MGC triggered TLS fatal error alert, sent by the MG to remote TLS endpoint

When the MGC provides signal mgcea, then the MG shall generate a TLS alert message based on the information as contained in signal parameters al and ad, and sent the TLS alert towards the remote TLS endpoint.

Furthermore, the MG shall sent a subsequent TLS closure_notify alert in order to initiate an outgoing TLS session release procedure. The outgoing TLS session release procedure shall be either triggered by the MGC (see clause 8.6.5.4) or autonomously started by the MG itself (see clause 9.6.2).

### 11.6.3	Lower layer issues

There might be issues in the protocol layers below the TLS layer.

Example use case:

- Established IP bearer path, i.e. there is an active TLS session;

- The underlying IP transport connection (e.g., TCP or SCTP transport connection) is released from the remote side, but without any TLS closure alert indication;

- Hence, there would be still an established TLS session endpoint, but a released L4 bearer connection from a local MG perspective;

- And therefore the TLS session would be interrupted, no TLS alert could be sent to the remote side.

Procedures for the maintenance of lower layer issues are out of scope of this package, rather subject of exsting H.248 error handling capabilities.

[End Proposed Correction]

____________________