| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4497 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: completion of TLS traffic volume metrics package (clause 12) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

The technical package elements, - the set of H.248 statistics -, were developed over the past meetings and were stable since the last meeting. Because an explicit data model (Annex B) allows the specification of unambiguous semantics. However, the package procedural section wasn't yet adapted. This contribution tries to close that gap.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4474).

Change proposal:

[Begin Proposed Correction]

# 12	TLS traffic volume metrics package

|  | Package Name: | TLS traffic volume metrics package |
| --- | --- | --- |
|  | Package ID: | tlstv (0x####) |
|  | Description: | This package is used to support explicit statistics for the TLS protocol related to various protocol data unit types (such as octets, TLS fragments, etc.). The used data model is defined by Annex B, which is consistent with the TLS protocol itself. The statistics are applicable as long as the MG is aware that the H.248 Stream carries TLS traffic. |
|  | Version: | 1 |
|  | Extends: | None. |

## 12.1	Properties

None.

## 12.2	Events

None.

## 12.3	Signals

None.

## 12.4	Statistics

### 12.4.1	Number of Received Application Data TLS-Fragments

|  | Statistic Name: | Number of Received Application Data TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | recadfrag (0x0001) |
|  | Description: | Provides the number of application data TLS fragments received on the Termination or Stream since the statistic has been set. Only application data TLS fragments are counted which are passed from the TLS record layer to the application layer. A received TLS-PDU is regarded as application data if the TLS header field "type" of the received TLS-PDU has been set to "application_data(23)". At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either (i.e., stream or termination level) |

### 12.4.2	Number of Sent Application Data TLS-Fragments

|  | Statistic Name: | Number of Sent Application Data TLS-Fragments |
| --- | --- | --- |
|  | Statistic ID: | sentadfrag (0x0002) |
|  | Description: | Provides the number of application data TLS fragments sent from the Termination or Stream since the statistic has been set. A TLS fragment is regarded as application data if the TLS header field "type" of the resulting TLS-PDU is set to "application_data(23)". At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.3	Number of Received TLS Message Fragments

|  | Statistic Name: | Number of Received TLS Message Fragments |
| --- | --- | --- |
|  | Statistic ID: | rectmfrag (0x0003) |
|  | Description: | Provides the number of TLS message fragments received on the Termination or Stream since the statistic has been set. Only TLS message fragments are counted which are passed from the TLS record layer to the change_cipher_spec protocol, the alert protocol or the handshake protocol. A received TLS-PDU is regarded as an TLS message if the TLS header field "type" of the received TLS-PDU has been set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either (i.e., stream or termination level) |

### 12.4.4	Number of Sent TLS Message Fragments

|  | Statistic Name: | Number of Sent TLS Message Fragments |
| --- | --- | --- |
|  | Statistic ID: | senttmfrag (0x0004) |
|  | Description: | Provides the number of TLS message fragments sent from the Termination or Stream since the statistic has been set. A TLS fragment is regarded as TLS message if the TLS header field "type" of the resulting TLS-PDU is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.5	Number of Received Application Data Octets

|  | Statistic Name: | Number of Received Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | recado (0x0005) |
|  | Description: | This statistic logs the number of application data octets that are delivered to the application layer above the TLS record layer. Received application data messages from the remote TLS endpoint will have the TLS header field "type" set to "application_data(23)" and are decrypted, the authentication tag is removed, and the data are decompressed before this statistics value is updated. Other TLS messages are not subject of this statistics. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.6	Number of Sent Application Data Octets

|  | Statistic Name: | Number of Sent Application Data Octets |
| --- | --- | --- |
|  | Statistic ID: | sentado (0x0006) |
|  | Description: | This statistic logs the number of octets received from the application layer. The octets are counted before the application data are compressed, authenticated and encrypted. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.7	Number of Received TLS Message Octets

|  | Statistic Name: | Number of Received TLS Message Octets |
| --- | --- | --- |
|  | Statistic ID: | rectmo (0x0007) |
|  | Description: | This statistic logs the number of TLS message octets that are delivered to the change_cipher_spec protocol, the alert protocol or to the handshake protocol above the TLS record layer. Received TLS messages from the remote TLS endpoint will have the TLS header field "type" set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The TLS messages and are decrypted, the authentication tag is removed, and the data are decompressed before this statistics value is updated. Application data messages are not subject of this statistics. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.8	Number of Sent TLS Message Octets

|  | Statistic Name: | Number of Sent TLS Message Octets |
| --- | --- | --- |
|  | Statistic ID: | senttmo (0x0008) |
|  | Description: | This statistic logs the number of octets received from the change_cipher_spec protocol, the alert protocol or the handshake protocol. The octets are counted before the TLS messages are compressed, authenticated and encrypted. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.10	Number of Sent Application Data TLS-PDU Octets

|  | Statistic Name: | Number of Sent Application Data TLS-PDU Octets |
| --- | --- | --- |
|  | Statistic ID: | sentadpduo (0x000a) |
|  | Description: | This statistic provides the number of TLS PDU octets, where the TLS header field "type" is set to "application_data(23)", sent from the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the egress Application Data TLS PDUs of all TLS flows of an H.248 Stream, excluding all lower layer overhead. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.9	Number of Received Application Data TLS-PDU Octets

|  | Statistic Name: | Number of Received Application Data TLS-PDU Octets |
| --- | --- | --- |
|  | Statistic ID: | recadpduo (0x0009) |
|  | Type: | Double |
|  | Description: | This Statistic provides the number of TLS PDU octets, where the TLS header field "type" is set to "application_data(23)", received on the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the ingress Application Data TLS PDUs of all TLS flows of an H.248 Stream excluding all lower layer overhead. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.12	Number of Sent TLS Message PDU Octets

|  | Statistic Name: | Number of Sent TLS Message PDU Octets |
| --- | --- | --- |
|  | Statistic ID: | senttmpduo (0x000c) |
|  | Description: | This statistic provides the number of TLS PDU octets where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The statistic comprises TLS PDU octets sent from the Termination or Stream since the Termination or Stream has existed and the statistic has been set. The octets represent the egress TLS Message PDUs of all TLS flows of an H.248 Stream, excluding all lower layer overhead. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.11	Number of Received TLS Message PDU Octets

|  | Statistic Name: | Number of Received TLS Message PDU Octets |
| --- | --- | --- |
|  | Statistic ID: | rectmpduo (0x000b) |
|  | Type: | Double |
|  | Description: | This Statistic provides the number of TLS PDU octets where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The statistic comprises TLS PDU octets received on the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the ingress TLS Message PDUs of all TLS flows of an H.248 Stream excluding all lower layer overhead. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.14	Number of Sent Application Data Octets, Protected and Compressed

|  | Statistic Name: | Number of Sent Application Data Octets, Protected and Compressed |
| --- | --- | --- |
|  | Statistic ID: | sentadpco (0x000e) |
|  | Description: | This statistic provides the number of octets of protected and compressed application data TLS fragments in egress direction. It corresponds to TLSCiphertext.fragment (refer to the presentation language of RFC 5246) of a TLS PDU where the TLS header field "type" is set to "application_data(23)". The octets represent the egress protected and compressed Application Data of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.13	Number of Received Application Data Octets, Protected and Compressed

|  | Statistic Name: | Number of Received Application Data Octets, Protected and Compressed |
| --- | --- | --- |
|  | Statistic ID: | recadpco (0x000d) |
|  | Description: | This statistic provides the number of octets of protected and compressed application data TLS fragments in ingress direction. It corresponds to TLSCiphertext.fragment (refer to the presentation language of RFC 5246) of a TLS PDU where the TLS header field "type" is set to "application_data(23)". The octets represent the ingress protected and compressed Application Data of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.16	Number of Sent TLS Message Octets, Protected and Compressed

|  | Statistic Name: | Number of Sent TLS Message Octets, Protected and Compressed |
| --- | --- | --- |
|  | Statistic ID: | senttmpco (0x0010) |
|  | Description: | This statistic provides the number of octets of protected and compressed TLS messages in egress direction. It corresponds to TLSCiphertext.fragment (refer to the presentation language of RFC 5246) of a TLS PDU where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The octets represent the egress protected and compressed TLS messags of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.15	Number of Received TLS Message Octets, Protected and Compressed

|  | Statistic Name: | Number of Received TLS Message Octets, Protected and Compressed |
| --- | --- | --- |
|  | Statistic ID: | rectmpco (0x000f) |
|  | Description: | This statistic provides the number of octets of protected and compressed TLS messages in ingress direction. It corresponds to TLSCiphertext.fragment (refer to the presentation language of RFC 5246) of a TLS PDU where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The octets represent the ingress protected and compressed TLS message of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.18	Number of Octets of Compressed Application Data Fragments in egress Direction

|  | Statistic Name: | Number of Octets of Compressed Application Data Fragments in egress Direction |
| --- | --- | --- |
|  | Statistic ID: | sentadcfo (0x0012) |
|  | Description: | This statistic provides the number of octets of compressed application data fragments in egress direction. It corresponds to TLSCompressed.fragment (refer to the presentation language of RFC 5246) for a TLS PDU where the TLS header field "type" is set to "application_data(23)". The octets represent the egress compressed Application Data of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.17	Number of Octets of Compressed Application Data Fragments in ingress Direction

|  | Statistic Name: | Number of Octets of Compressed Application Data Fragments in ingress Direction |
| --- | --- | --- |
|  | Statistic ID: | recadcfo (0x0011) |
|  | Description: | This statistic provides the number of octets of compressed application data fragments in ingress direction. It corresponds to TLSCompressed.fragment (refer to the presentation language of RFC 5246) for a TLS PDU where the TLS header field "type" is set to "application_data(23)". The octets represent the ingress compressed Application Data of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.20	Number of Octets of Compressed TLS Message Fragments in egress Direction

|  | Statistic Name: | Number of Octets of Compressed TLS Message Fragments in egress Direction |
| --- | --- | --- |
|  | Statistic ID: | senttmcfo (0x0014) |
|  | Description: | This statistic provides the number of octets of compressed TLS message fragments in egress direction. It corresponds to TLSCompressed.fragment (refer to the presentation language of RFC 5246) for a TLS PDU where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The octets represent the egress compressed TLS Messages of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the egress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 12.4.19	Number of Octets of Compressed TLS Message Fragments in ingress Direction

|  | Statistic Name: | Number of Octets of Compressed TLS Message Fragments in ingress Direction |
| --- | --- | --- |
|  | Statistic ID: | rectmcfo (0x0013) |
|  | Description: | This statistic provides the number of octets of compressed TLS message fragments in ingress direction. It corresponds to TLSCompressed.fragment (refer to the presentation language of RFC 5246) for a TLS PDU where the TLS header field "type" is set to one of the following values: - change_cipher_spec(20) - alert(21) - handshake(22) The octets represent the ingress compressed TLS Message of all TLS flows of an H.248 Stream. At the Termination level, it is equal to the sum of the ingress TLS flows over all Streams. This statistic is only applicable when TLS is terminated locally (e.g., in case of TLS-to-non-TLS interworking). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

## 12.5	Error codes

None.

## 12.6	Procedures

### 12.6.1	General information

### 12.6.1.1	TLS data model

### The underlying data model is contained in Annex B, which defines as well relevant measurement points (MP)..

#### 12.6.1.2	Overview of performance metrics

### Table x1 summarizes all ten categories of performance metrics which result in twenty statistics due to support in both traffic directions:

Table x1 – Overview of performance metrics – All H.248 statistics

| No. | Category of H.248 statistics | Direction |  | Clause |  | StatisticsID |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Number of Application Data TLS-Fragments |  | Received |  | 12.4.1 |  | recadfrag |  |
| 2 |  |  | Sent |  | 12.4.2 |  | sentadfrag |  |
| 3 | Number of TLS Message Fragments |  | Received |  | 12.4.3 |  | rectmfrag |  |
| 4 |  |  | Sent |  | 12.4.4 |  | senttmfrag |  |
| 5 | Number of Application Data Octets |  | Received |  | 12.4.5 |  | recado |  |
| 6 |  |  | Sent |  | 12.4.6 |  | sentado |  |
| 7 | Number of TLS Message Octets |  | Received |  | 12.4.7 |  | rectmo |  |
| 8 |  |  | Sent |  | 12.4.8 |  | senttmo |  |
| 9 | Number of Application Data TLS-PDU Octets |  | Received |  | 12.4.9 |  | recadpduo |  |
| 10 |  |  | Sent |  | 12.4.10 |  | sentadpduo |  |
| 11 | Number of TLS Message PDU Octets |  | Received |  | 12.4.11 |  | rectmpduo |  |
| 12 |  |  | Sent |  | 12.4.12 |  | senttmpduo |  |
| 13 | Number of Application Data Octets, Protected and Compressed |  | Received |  | 12.4.13 |  | recadpco |  |
| 14 |  |  | Sent |  | 12.4.14 |  | sentadpco |  |
| 15 | Number of TLS Message Octets, Protected and Compressed |  | Received |  | 12.4.15 |  | rectmpco |  |
| 16 |  |  | Sent |  | 12.4.16 |  | senttmpco |  |
| 17 | Number of Octets of Compressed Application Data Fragments |  | Received |  | 12.4.17 |  | recadcfo |  |
| 18 |  |  | Sent |  | 12.4.18 |  | sentadcfo |  |
| 19 | Number of Octets of Compressed TLS Message Fragments |  | Received |  | 12.4.19 |  | rectmcfo |  |
| 20 |  | Sent | 12.4.20 | senttmcfo |

Table x2 indicates the internal measurement points (given by Figure B.1) and whether a TLS protocol termination would be required:

Table x2 – Internal measurement points (MP), traffic types and conditional usage

| Category of H.248 statistics |  | StatisticsID |  | Relates to |  | Measurement point |  | Presentation Language of RFC 5246 |  | TLS protocol termination at SEP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Application Data TLS-Fragments | recadfrag | Application Data | Application Data | TLSPlaintext. fragment | Yes |
|  | sentadfrag |  |  |  |  |
| Number of TLS Message Fragments | rectmfrag | TLS message | TLS message | TLSPlaintext. fragment | Yes |
|  | senttmfrag |  |  |  |  |
| Number of Application Data Octets | recado | Application Data | Application Data | - | No |
|  | sentado |  |  |  |  |
| Number of TLS Message Octets | rectmo | TLS message | TLS-RL PDU | - | No |
|  | senttmo |  |  |  |  |
| Number of Application Data TLS-PDU Octets | recadpduo | Application Data | TLS-RL PDU | TLSCiphertext | No |
|  | sentadpduo |  |  |  |  |
| Number of TLS Message PDU Octets | rectmpduo | TLS message | TLS-RL PDU | TLSCiphertext | No |
|  | senttmpduo |  |  |  |  |
| Number of Application Data Octets, Protected and Compressed | recadpco | Application Data | TLS-RL PDU | TLSCiphertext.fragment | No |
|  | sentadpco |  |  |  |  |
| Number of TLS Message Octets, Protected and Compressed | rectmpco | TLS message | TLS-RL PDU | TLSCiphertext.fragment | No |
|  | senttmpco |  |  |  |  |
| Number of Octets of Compressed Application Data Fragments | recadcfo | Application Data | TLS-RL PDU | TLSCompressed.fragment | Yes |
|  | sentadcfo |  |  |  |  |
| Number of Octets of Compressed TLS Message Fragments | rectmcfo | TLS message | TLS-RL PDU | TLSCompressed.fragment | Yes |
|  | senttmcfo |  |  |  |  |

#### 12.6.1.3	Profile selection guidelines

### It is expected that a particular H.248 profile with support of the tlstv package will only support a subset from the defined set of statistics. Example:

### A profile interested in application level measurements only would just use the statistics related to measurement point "Application Data".

### A profile with support of "TLS transparent forwarding" only may support just the statistics which does not need a TLS protocol termination in the MG.

### A profile interested in "bitrate"-based traffic volume measurement data would just use the octet-based H.248 statistics.

### A profile interested in TCP traffic volume with and without TLS-based transport security (e.g., comparing data-over-TLS-over-TCP with data-over-TCP) may need to gather measurements at measurment point "TLS-RL PDU" for detailed performance evaluations.

### etc.

#### 12.6.1.4	Measurement principles

The usual H.248 principles apply:

- consideration of all successfully received/sent data units to/from its H.248 TLS Stream/Termination;

- aggregation of all TLS flows of an H.248 Stream/termination.

### 12.6.2	Number of Application Data TLS-Fragments

Statistics tlstv/recadfrag and tlstv/sentadfrag provide a fragment-oriented based volume measurements between TLS and application protocol layer.

### 12.6.3	Number of TLS Message Fragments

Statistics tlstv/rectmfrag and tlstv/senttmfrag provide a fragment-oriented based volume measurements between TLS record protocol and TLS handshake/cipher/alert protocol layer.

### 12.6.4	Number of Application Data Octets

Statistics tlstv/recado and tlstv/sentado provide octet-oriented based volume measurements between TLS and application protocol layer.

### 12.6.5	Number of TLS Message Octets

Statistics tlstv/recmo and tlstv/sentmo provide octet-oriented based volume measurements between TLS record protocol and TLS handshake/cipher/alert protocol layer.

### 12.6.6	Number of Application Data TLS-PDU Octets

Statistics tlstv/recadpduo and tlstv/sentadpduo provide octet-oriented based volume measurements between L4 transport and TLS protocol layer.

### 12.6.7	Number of TLS Message PDU Octets

Statistics tlstv/rectmpduo and tlstv/senttmpduo provide octet-oriented based volume measurements between L4 transport and TLS protocol layer.

### 12.6.8	Number of Application Data Octets, Protected and Compressed

Statistics tlstv/recadpco and tlstv/sentadpco provide octet-oriented based volume measurements between L4 transport and TLS protocol layer.

### 12.6.9	Number of TLS Message Octets, Protected and Compressed

Statistics tlstv/rectmpco and tlstv/senttmpco provide octet-oriented based volume measurements between L4 transport and TLS protocol layer.

### 12.6.10	Number of Octets of Compressed Application Data Fragments

Statistics tlstv/recadcfo and tlstv/sentadcfo provide fragment-oriented based volume measurements between L4 transport and TLS protocol layer.

### 12.6.11	Number of Octets of Compressed TLS Message Fragments

Statistics tlstv/rectmcfo and tlstv/senttmcfo provide fragment-oriented based volume measurements between L4 transport and TLS protocol layer.

[End Proposed Correction]

____________________