| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4505 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.78 (Rev): completion of "MG located Bearer Level ALG" package (clause 8) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to the mgbalg package in order to get a completed revision of H.248.78.

Discussion

The proposed changes are firstly driven by the open editor's notes:

- clause 8.1.3:

{Editor's note (2013-10): a default value should be provided.}

Resolution: proposal of "S [0x01]" as default due to "The translation of source address information only is e.g. a very common ALG scenario."

- clause 8.1.4:

{Editor's note (2013-10): there was a comment that there are two sources in case of "raip = B". There might be then a scenario where one source is "IP" and the other source is "PR". Such combination were not yet considered. To be checked. Property type change?}

Resolution: the property value supports already a list format. We think that we should allow any combination of values in case of "raip = B".

- clause 8.1.4:

{Editor's note (2013-10): a default value should be provided.}

Resolution: proposal of "SD" as default, i.e. a single value to the proposed raip default value.

- clause 8.6.6 - examples:

{Editor's note (2013-10): in particular examples with "raip" & "saip" property usage should be provided.}

Resolution: two examples are described.

Secondly, we had some discussions whether it is obvious how the B-ALG is actually effective, i.e., the questions of

- uni- vs bidirectional B-ALG service?

- directionality, i.e., effect of a unidirectional B-ALG service?

and

- which termination gets the B-ALG configuration (i.e., the four mgbalg properties)?

Based on that discussion we've concluded to move the examples to a new Appendix with some introductory text concerning the overal B-ALG concept, and to add a new procedural clause 8.6.1.

The B-ALG concept is consistent with the behaviour of MG-local L3/L4 NAT functions, which is extensively illustrated in ETSI TR 183 068, Annex H. A correspondent link is provided in the new text.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against input draft to this meeting (see AVD-4472).

Change proposal #1:

[Begin Proposed Correction]

# 8	MG located Bearer Level ALG

|  | Package name: | MG located Bearer Level ALG |
| --- | --- | --- |
|  | Package ID: | mgbalg (0x0###) |
|  | Description: | The actual ALG function with respect to the replacement of address information at application layer is executed by the MGC when using package mcbalg (clause 7). This package moves that function down to the MG itself. The MGC is only involved in enabling that function and (if required) providing address information to be inserted. |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

### 8.1.1	Protocol type bearer level ALG

|  | Property name: | Protocol type bearer level ALG |
| --- | --- | --- |
|  | Property ID: | ptbalg (0x0001) |
|  | Description: | This property enables/disables the MG located Bearer Level ALG based on the protocol type. |
|  | Type: | Boolean |
|  | Possible values: | ON to enable MG located bearer level ALG OFF to disable MG located bearer level ALG |
|  | Default: | OFF |
|  | Defined in: | LocalControl on an individual stream or TerminationState on the root termination. The variants of non-root and root termination level shall be mutually exclusive. An H.248 profile should indicate the supported variant. |
|  | Characteristics: | Read/write |

### 8.1.2	Upper layer protocol filter

|  | Property name: | Upper layer protocol filter |
| --- | --- | --- |
|  | Property ID: | ulpf (0x0002) |
|  | Description: | This property specifies the stream's upper layer protocols type that the filtering is applied to. The term "upper layer" denotes the protocol layer above the layer 4 transport protocol, i.e., an "upper layer" protocol is a "L4+" protocol, e.g., HTTP, MSRP, or RTSP. |
|  | Type: | Sub-list of Integer |
|  | Possible values: | A single value or a sub-list. A value representing an application protocol, using the well known or registered port number assigned to that protocol by IANA (http://www.iana.org/assignments/port-numbers). For example: 80 World Wide Web HTTP 554 Real Time Streaming Protocol (RTSP) 2855 MSRP |
|  | Default: | 0, indicating that the protocol is determined via the local and remote descriptors. |
|  | Defined in: | LocalControl on an individual stream or TerminationState on the root termination. The variants of non-root and root termination level shall be mutually exclusive. An H.248 profile should indicate the supported variant. |
|  | Characteristics: | Read/write |

### 8.1.3	Relevant address information part

|  | Property name: | Relevant address information part |
| --- | --- | --- |
|  | Property ID: | raip (0x0003) |
|  | Description: | Communication addresses in Lx-PDUs (protocol data unit at layer Lx; here primarily layers concerned above L3) contain typically address information of the traffic source and/or traffic sink. The ALG function could be required for the replacement of source or/and sink information, dependent typically on the particular translation behaviour of remote NAT devices in the IP bearer path. The translation of source address information only is e.g. a very common ALG scenario. This property allows to indicate the relevant address information part. |
|  | Type: | Enumeration |
|  | Possible values: | S [0x01] Source address information D [0x02] Destination address information B [0x03] Consideration of both (source and destination address information) |
|  | Default: | S [0x01] |
|  | Defined in: | LocalControl on an individual stream or TerminationState on the root termination. The variants of non-root and root termination level shall be mutually exclusive. An H.248 profile should indicate the supported variant. |
|  | Characteristics: | Read/write |

### 8.1.4	Indication of "source of replaced address information"

|  | Property name: | Source of replaced address information part |
| --- | --- | --- |
|  | Property ID: | saip (0x0004) |
|  | Description: | The MG needs to know which address information has to be used for the ALG service. There are basically multiple options. This property allows the MGC to indicate the used source(s) of replaced address information. |
|  | Type: | Sublist of Enumeration |
|  | Possible values: | SD [0x01] Contained in H.248 Stream Descriptor PR [0x02] Provisioned in MG IP [0x03] IP (transport) address information The sublist contains a single or two values. When raip is equal to S or D (i.e., single replaced address information), then the first list value applies (if two values were wrongly signaled). When raip is equal to B (i.e., doubled replaced address information), then the first list value applies source address information and the second one to destination address information. Any combination of values is possible. |
|  | Default: | SD NOTE – An H.248 profile with a defined default of "raip = B" may consider for this property a default of "saip = "SD,SD"". |
|  | Defined in: | LocalControl on an individual stream or TerminationState on the root termination. The variants of non-root and root termination level shall be mutually exclusive. An H.248 profile should indicate the supported variant. |
|  | Characteristics: | Read/write |

## 8.2	Events

None.

## 8.3	Signals

None.

## 8.4	Statistics

None.

## 8.5	Error codes

None.

## 8.6	Procedures

### 8.6.1	General characteristics of the MG located bearer level ALG service

#### 8.6.1.1	Directionality

The MG located bearer level ALG service affects only one traffic direction, thus belongs to the category of unidirectional address translation functions (as opposed to bidirectional ones). Appendix I.1 illustrates the concept.

#### 8.6.1.2	Context aspects

The MG located bearer level ALG is enabled at the termination in outgoing direction (see also Appendix I.1).

### 8.6.2	Enabling the MG located bearer level ALG service

When a MG located bearer level ALG service is required for incoming packet traffic, the MGC shall set the property protocol type bearer level ALG (mgbalg/ptbalg) to "ON".

### 8.6.3	Indication of affected application protocol(s)

The bearer level ALG function is normally application specific, i.e., enabled for selected application(s). The MGC should use property upper layer protocol filter (mgbalg/ulpf) to indicate the application(s).

### 8.6.4	Indication of relevant address information

The MG located bearer level ALG service needs to know whether source or/and destination address information should be replaced. The MGC should provide a correspondent indication via property relevant address information part (mgbalg/raip), else, a default property value shall be provisioned.

### 8.6.5	Indication of replaced address information

The MG needs to know which address information has to be used for the ALG service. There are basically three options:

Table 1 – Principle options for the indication of replaced address information

| Variant | Source of translated address information | Indicated by MGC via: |
| --- | --- | --- |
| 1 | Contained in H.248 Stream Descriptor (NOTE 1). E.g., as part of the SDP media description, other than "c=" and/or "m=" line address information. Such as related to (general or application specific) SDP attributes. | Explicitly by MGC. Example: The SDP "a=path:" attribute (clause 8.2/[IETF RFC 4975]), used by application protocol ‘MSRP’. |
| 2 | Provisioned in MG. | Not indicated by MGC. The MG configured for that variant via management plane actions. |
| 3 | IP L3 (and L4) source (transport) address information of incoming IP packet (NOTE 2). | Default option, i.e., when neither variant 1 nor 2 is used. |
| NOTE 1 – Depending on the value of the property mgbalg/raip this address information is located in the Local descriptor (if mgbalg/raip=S), or in the Remote descriptor (if mgbalg/raip=D) or in both (if mgbal/raip=B). NOTE 2 – I.e., numerical IP address information. |  |  |

The MGC should provide a correspondent indication via property source of replaced address information part (mgbalg/saip), else, a default property value shall be provisioned.

### 8.6.6	B-ALG: L4+ protocol specific translation details

The translation of address information at L4+ level may be not defined in a generic manner because dependent on the particular L4+ protocol. The protocol dependent application details are therefore out of scope of this Recommendation (as in case of the mcbalg package, see e.g. clause 7.6.4). Such information could be e.g. part of an H.248 profile specification.

### 8.6.7	Examples

### 8.6.7.1	Examples for MSRP-based instant messaging service

Use case:

MSRP is used in session-mode, leading to the transport of MSRP messages via TCP/IP packets in the bearer plane (in contrary to the page-mode, which uses the SIP MESSAGE method in the signalling plane).

Two example B-ALG configurations for MSRP are described (inclusive example H.248 signalling) in Appendix I.2.1 and I.2.2.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix I

MG located bearer level ALG service – 
Illustration and signalling examples

(This appendix does not form an integral part of this Recommendation)

Clause 8 defines the B-ALG variant of a MG located bearer level ALG service. This Appendix illustrates the concept and provides example H.248 signalling.

## I.1	Illustration of concept

The configuration of a particular MG located bearer level ALG service is subject of four properties (as defined by the mgbalg package). Figure I.1 illustrates the resulting action when the package properties would be associated to a specific context (i.e., part of LocalControl descriptor).

Figure I.1 – Concept of MG located bearer level ALG service

The B-ALG configuration is part of the Stream descriptor of termination TB in Figure 1. The resulting B-ALG service is thus effective in traffic direction from X to Y.

Background information:

The B-ALG represents a NAT function at protocol layer L4+ ("L4+ NAT"). The behaviour of a L4+ NAT function should be consistent with the usual L3/L4 NAT functions, from H.248 perspective. Such H.248 context models for L3/L4 NAT are e.g. illustrated in Annex H in [b-ETSI TR 183 068].

Essentially, the effect of a L3/L4 NAT function becomes visible at the MG external bearer interface in outgoing direction. Thus, any local, unidirectional NAT in X-to-Y direction is controlled at termination TB (and not at TA), because the replaced address information is subject of the Stream descriptor of TB in case of L3/L4 NAT.

The same principle applies for local L4+ NAT, i.e., the B-ALG in this Recommendation.

The relevant address information for translation is controlled by the raip property (either source or/and destination address information). Figure I.1 indicates a raip value of 'B(oth)'.

There are three options concerning the source of replaced address information (see saip property values in Figure I.1).

## I.2	Illustration of example use cases

## Some B-ALG services are illustrated at the example of MSRP traffic (see introduction in clause 6 and Figure 1 concerning L4+ address information elements).

### I.2.1	Use case "Unidirectional B-ALG with source address information translation"

This example considers a unidirectional B-ALG for MSRP, taking the assumption that the originating MSRP client (here X) could already insert the correct L4+ destination address information (i.e., the MSRP header "To-Path" parameter value is already valid from end-to-end perspective). However, the L4+ source address information is not correct of incoming MSRP messages, thus a correspondent B-ALG service needs to be enforced in the MG (see Figure I.2).

Figure I.2 – Use case "Unidirectional B-ALG with source address 
information translation" – MSRP bearer traffic

The TCP/IP header is used as source of replaced address information (see example H.248 syntax in Table I.1). The translated MSRP "From-Path" element would then contain a numerical IP address value (note: a named IP address format would imply a reverse DNS lookup).

Table I.1 – Example command encoding– B-ALG configuration by MGC

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = … { Context = … { Add = ip/$ { ; Termination TB, SEP S1 .Media { Stream = 1 { LocalControl { mgbalg/ptbalg = ON ; B-ALG enabled mgbalg/ulpf = 2855 ; for MSRP traffic mgbalg/raip = S ; for source address info. mgbalg/saip = IP ; from TCP/IP header info. } ... Local { v=0 c=IN IP6 <IP_addr> m=message <port> TCP/MSRP – ; NOTE 1 }, Remote { v=0 c=IN IP6 <IP_addr> m=message <port> TCP/MSRP – }…} | NOTE 1 – The "m=" line <proto> provides an application-aware indication due to the enforced B-ALG function (for application 'MSRP'). |

### I.2.2	Use case "Bidirectional B-ALG with source & destination address information translation"

This example considers a bidirectional B-ALG for MSRP with the need to adapt L4+ source and destination address information. Hence, both MSRP header elements "To-Path" and "From-Path" needs to be modified by the MG. The replaced address information is provided by the MGC to the MG via the (MSRP-related) SDP attribute "a=path:…", which are subject of the LD and RD.

Figure I.3 depicts the bidirectional B-ALG service, as composed by two unidirectional B-ALG functions associated to the two terminations TA and TB.

Figure I.3 – Use case "Bidirectional B-ALG with source & destination 
address information translation " – MSRP bearer traffic

Example H.248 syntax is abstracted in Table I.2.

Table I.2 – Example command encoding– B-ALG configuration by MGC

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = … { Context = … { Add = ip/$ { ; Termination TB, SEP S1 .Media { Stream = 1 { LocalControl { mgbalg/ptbalg = ON ; B-ALG enabled mgbalg/ulpf = 2855 ; for MSRP traffic mgbalg/raip = B ; for both address info. mgbalg/saip = "SD,SD" ; from Stream Descriptor } ... Local { v=0 c=IN IP6 <IP_addr> ; NOTE 1 m=message <port> TCP/MSRP – ; NOTE 1 a=path:… ; NOTE 2 }, Remote { v=0 c=IN IP6 <IP_addr> ; NOTE 3 m=message <port> TCP/MSRP – ; NOTE 3 a=path:… ; NOTE 4 }…} Add = ip/$ { ; Termination TA, SEP S1 .Media { Stream = 1 { LocalControl { mgbalg/ptbalg = ON ; B-ALG enabled mgbalg/ulpf = 2855 ; for MSRP traffic mgbalg/raip = B ; for both address info. mgbalg/saip = "SD,SD" ; from Stream Descriptor } ... Local { v=0 c=IN IP6 <IP_addr> ; NOTE 5 m=message <port> TCP/MSRP – ; NOTE 5 a=path:… ; NOTE 6 }, Remote { v=0 c=IN IP6 <IP_addr> ; NOTE 7 m=message <port> TCP/MSRP – ; NOTE 7 a=path:… ; NOTE 8 }…} | Configuration of unidirectional B-ALG in X-to-Y direction (i.e., Termination TB: - The local L3/L4 source address information is subject of the correspondent LD SDP "c="-/"m="-line contents (NOTE 1). - The local L4+ source address information is subject of LD SDP attribute "a=path:…" (NOTE 2). - The local L3/L4 destination address information is subject of the correspondent RD SDP "c="-/"m="-line contents (NOTE 3). - The local L4+ destination address information is subject of RD SDP attribute "a=path:…" (NOTE 4). Configuration of unidirectional B-ALG in Y-to-X direction (i.e., Termination TA: - The local L3/L4 source address information is subject of the correspondent LD SDP "c="-/"m="-line contents (NOTE 5). - The local L4+ source address information is subject of LD SDP attribute "a=path:…" (NOTE 6). - The local L3/L4 destination address information is subject of the correspondent RD SDP "c="-/"m="-line contents (NOTE 7). - The local L4+ destination address information is subject of RD SDP attribute "a=path:…" (NOTE 8). |

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Bibliography

[b-ETSI TR 183 068]	ETSI 183 068 V3.1.1 (2009-08), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[b-ITU-T H.248.64]		Recommendation ITU-T H.248.64 (03/2013), Gateway control protocol: IP router packages.

[b-IETF RFC 2663]		IETF RFC 2663 (1999), IP Network Address Translator (NAT) Terminology and Considerations.

[End Proposed Correction]

____________________