| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4510 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva (CH), 10 – 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.SCTP: use case(s) - input proposal |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes input material for clause "use cases", similar as for bearer types "TCP" and "TLS" (see H.248.TCP, H.248.TLS).

Discussion

We'd like to highlight following aspects:

- SCTP transport modes: we've described the three known transport modes in clause 6.1, despite the fact that presently only one is in discussion from H.248 profile perspective ("the WebRTC related mode"). However, an H.248 WebRTC media gateway may be requested for different kind of WebRTC data channel interworking, which could lead to additional transport mode indication at H.248 MG level.
 clause 6 should be complete concering possible SCTP transport modes, whereas the first release of this Recommendation could focus on H.248 protocol capabilities by consideration of firstly required SCTP transport modes.

- SCTP bearer connection use cases: similar comment as above, clause 6.2 indicates all (?) theoretical interworking scenarios with scope on H.248 (IP, IP) connection models.
There are also use case for (IP, IP, …IP) connection models, - e.g., an H.248 media server providing a conference point for WebRTC data channel services -, … but this is FFS.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against the output draft from last meeting (see TD 113 (WP 1/16)).

Change proposal #1:

[Begin Proposed Correction]

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.DTLS]	Draft Recommendation ITU-T H.248.DTLS (XX/2014), Gateway control protocol: H.248 support for control of transport security using DTLS.

[IETF RFC 4960]	IETF RFC 4960 (2007), Stream Control Transmission Protocol.

[IETF RFC 5061]	IETF RFC 5061 (2007), Stream Control Transmission Protocol (SCTP) Dynamic Address Reconfiguration.

[IETF RFC 6525]	IETF RFC 6525 (2012), Stream Control Transmission Protocol (SCTP) Stream Reconfiguration.

{Editor's note (2013-10): following references might be needed

- draft-ietf-mmusic-sctp-sdp

- draft-ietf-tsvwg-sctp-dtls-encaps

- draft-marcon-msrp-over-webrtc-data-channels

- draft-ietf-rtcweb-data-protocol

- H.248.DTLS

- others

}

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 6	Use case descriptions

Basic use cases may be identified from the perspective of

- H.248 MG type: IP-to-IP gateway (see clause 6.2) or IP-to-non-IP gateway (see clause 6.3) in case of usual two-party communication services; and

- Multiparty service types (see clause 6.4).

It may be noted that the various network use cases may be abstracted by a single bearer connection model, see clause 7.2.

## 6.1	Use cases related to SCTP transport modes

The SCTP protocol could be used in various transport protocol stack variants. Multiple SCTP transport modes are possible (Figure x1).

Figure x1 – SCTP transport modes

This Recommendation is not dependent on any SCTP transport mode, but focuses on native SCTP/IP as example.

## 6.2	Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.1	High-level use case categories

The notion "IP-to-IP" (briefly "IP-IP") indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])).

Each H.248 stream endpoint (SEP) may be associated with a non-SCTP or different SCTP protocol stack variants, leading to various bearer network connection use cases (see Figure x2):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end SCTP bearer connection, there are two sub-variants

  - Use case #1.1.1: the end-to-end SCTP bearer connection is divided in two separate SCTP associations ("SCTP proxy mode");

  - Use case #1.1.2: the end-to-end SCTP bearer connection relates to a single SCTP association ("SCTP relay mode");

- Use case #1.2: FIXTHIS;

- Use case #1.3: FIXTHIS;

- Use case #1.4: FIXTHIS;

- Use case #2.1: FIXTHIS;

- Use case #2.2: FIXTHIS;

- Use case #2.3: FIXTHIS;

- Use case #3.1: FIXTHIS;

- Use case #3.2: FIXTHIS;

Figure x2 – Bearer connection network use cases with H.248 IP-IP gateways

All these use cases are basically of type "application-agnostic".

### 6.2.2	Use case variations

#### 6.2.2.1	"SCTP-to-SCTP transparent forwarding"

This scenario is a variation of use case #1.1.2 and requires the MG behaviour of SCTP transparent forwarding.

#### 6.2.2.2	"SCTP-to-SCTP application aware processing"

This scenario is a variation of use case #1.1.1, #2.1 or #3.1. The MG may be required to be application aware in case of   FIXTHIS  .

## 6.3	Bearer connection network use cases with H.248 "SCTP to non-SCTP" gateways

Relates to use cases #1.4, #2.3 and #3.2.

Example: WebRTC data channel interworking as illustrated in Appendix I.3/[ITU-T H.248.DTLS]; all that examples relate to use case #3.2.

[End Proposed Correction]

____________________