| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4522 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia Pty Ltd |  |  |  |
| Title: | AMS Signalling Syntax |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

Previously, the experts in Q2/16 decided to adopt JSON as the syntax notation for H.325/AMS.  This document is an initial draft that formally defines that syntax using notation as described in IETF draft draft-newton-json-content-rules.  It also describes the construction of messages and fundamental messaging procedures.

Within H.325, the terms “Container” and “application” refer to specific functions within the H.325 architecture.  However, these entities are the origination and/or termination point for messages and, as such, are referred to generically as “endpoints” in this document.

The Service Node is not classified as an “endpoint”.  The Service Node is primarily viewed as a functional entity with which an endpoint registers and uses to convey signalling messages to remote endpoints.  A Service Node may exist, however, which does not handle endpoint registrations and merely serves to convey signalling messages.  The latter type of Service Node is referred to as a “relay”.

As a matter of convention, text in italics are message primitives.  Text in bold are message elements.  Values associated with JSON members are shown in quotes.

# Message syntax and fundamental messaging procedures

## Base message syntax

This section includes elements that are common to all messages (e.g., the base H.325 message structure) and commonly used definitions like UUID.

root
{

sessionID,

?metaSessionID,

sender,

recipient,

?sequence, ; Shall be present, except when sending an explicit acknowledgement

?ackSequence,

messageType,

?body

}

sessionID "sessionID" [ 1*2 UUID ]

metaSessionID "metaSessionID" [ 1*2 UUID ]

sender "sender"

{

?names,

aliases,

?instanceURI

}

recipient "recipient"

{

?names,

aliases,

?instanceURI ; Present when directing requests to specific instances

}

sequence_range : integer 1..4294967295

sequence "sequence" sequence_range

ackSequence "ackSequence" sequence_range

messageType "messageType" ( initiateMessage /

engagedMessage /

terminateMessage /

informationMessage )

body "body" ( plaintextBody / encryptedBody )

plaintextBody

{

"action" : string /^.{1,32}$/,

?genericData,

?content: any ; Content is defined below for each message

}

encryptedBody : string

; Basic message primitives

initiateMessage : string /^initiate$/

engagedMessage : string /^engaged$/

terminateMessage: string /^terminate$/

informationMessage: string /^information$/

; UUIDs are defined in RFC 4122

UUID : string /^[a-z0-9]{8}-[a-z0-9]{4}-[a-z0-9]{4}-[a-z0-9]{4}-[a-z0-9]{12}$/

OID : string /^[0-9]+(\.[0-9]+)*$/

entityID "entityID" UUID

timeToLive "timeToLive" : integer 0..4294967295

aliases "aliases" [ 1*: uri ]

language "language" : string ; Defined in RFC 5646

languages "languages" [ 1*: string ] ; Defined in RFC 5646

name "name" : string /^.{1,128}$/

nameAndLanguage

{

name,

?language

}

names "names" [ 1* nameAndLanguage ]

entity

{

"type" : uri,

?"icon" : uri,

?names

}

entities "entities" [ 1* entity ]

enterprise "enterprise" : OID

vendorInfo

{

?enterprise,

"vendor" : string /^.{1,64}/,

"product" : string /^.{1,64}/,

"version" : string /^.{1.32}/,

?genericData

}

genericData "genericData" [ 1* genericItem ]

genericItem

{

id : uri,

?value: any

}

instanceURI "instanceURI" : uri

transport "transport" ( transportTCP /

transportTLS /

transportUDP /

transportDTLS /

transportSCTP )

; Transport options

transportTCP : string /^TCP$/

transportTLS : string /^TLS$/

transportTLS : string /^UDP$/

transportTLS : string /^DTLS$/

transportTLS : string /^SCTP$/

port : integer 0..65535

ipAddress "ipAddress"

{

"address" : ip4,

"port" : port

}

ip6Address "ip6Address"

{

"address" : ip6,

"port" : port

}

transportAddress "transportAddress" :

{

transport,

ipAddress / ip6Address

}

transportAddresses "transportAddresses" [ 1* transportAddress ]

## Registration syntax

This section defines some the common syntactical elements used as a part of the registration process.

password "password" : string

authMethodPassword : string /^password$/

authMethod "authMethod" authMethodPassword

hashalg "hashalg" : string /^.{1,32}$/

nonce "nonce" : string

alternateSNs "alternateSNs" [ 1* : fqdn ]

### Initiating registration

When an endpoint registers with the Service Node, it will send an initiate message with the string “register” in the action member of body. The content member may contain the register object.

register

{

entityID,

?entities,

?vendorInfo,

?languages,

registrationCredentials

}

registrationCredentials "credentials"

{

"username" : string /^.{1,128}$/,

?password,

?hashalg

}

If the service node requires no specific credentials, then the registration might be accepted without credentials or with only the username provided. If a password or other form of authorization is required and not provided, the registration request would be rejected by sending a terminate message.

The entityID is important to ensure that the Service Node can positively identify an endpoint when it attempts a registration (e.g., following a reboot), this avoiding “phantom” registrations or confusion as to which Container is a current and active Container.  This value should remain constant through reboots and software updates.

### Rejecting a registration

When rejecting a registration, the Service Node will send a terminate message with the string “registrationReject” in the action member of body.  The content member contains the registrationReject object.

registrationReject

{

?entities,

?vendorInfo,

registrationRejectReason,

?registrationCredentialRequirements

}

registrationCredentialRequirements "credentials"

{

authMethod,

?hashAlg,

?nonce

}

registrationRejectReason "reason" ( authRegistrationRejectReason /

unspecifiedRegistrationRejectReason )

; Registration rejection reasons

authRegistrationRejectReason : string /^auth$/

unspecifiedRegistrationRejectReason : string /^unspecified$/

### Accepting a registrations

Upon successfully registering with the Service Node, the Service Node sends an engaged message to the registering endpoint.  The body of the message contains the string “registered” in the action member of body. The content member may contain the registered object.

registered

{

?entities,

?vendorInfo,

?natBindTest,

?alternateSNs,

?languages,

timeToLive

}

natBindTest "natBindTest" [ 1* : fqdn ]

If an endpoint is instructed to perform a NAT bind test, as indicated via the presence of a populated natBindTest member the content of the engaged message, the endpoint is expected to perform a NAT bind test and report through a registration refresh procedure.

### Refreshing a registration

From time-to-time, it might be possible that a registered entity loses connectivity with the Service Node to which it is registered (e.g., broken TCP connection) or its IP address changes.  When this happens, the registered entity sends an information message with the body containing the action value “refresh”.  The content member may contain the registrationRefresh object.

registrationRefresh

{

natResults

}

natResults

{

TBD

}

If a Service Node receives such an information message for a registration that does not exist or has expired, the Service Node may send a terminate message to signal that the registration is not valid.  The endpoint would then re-register with the Service Node.

Prior to sending a message to refresh the registration when an IP address associated with the endpoint change, the endpoint shall perform a NAT bind test and report the results in the refresh message.

### Terminating a registration

Either an endpoint or the Service Node may terminate a registration.  This is done by sending a terminate message to the Service Node or Endpoint with the action value set to “unregister”.  The content member will contain the unregister object.

unregister

{

unregisterReason

}

unregisterReason "reason" ( unspecifiedUnregisterReason /

maintenanceUnregisterReason /

shutdownUnregisterReason )

; Unregister reasons

unspecifiedUnregisterReason : string /^unspecified$/

maintenanceUnregisterReason : string /^maintenance$/

shutdownUnregisterReason : string /^shutdown$/

## Acknowledging messages

Nearly all H.325 messages require either passive acknowledgement (e.g., sending an engaged message in response to an initiate message) or active acknowledgement.  An active acknowledgement message is required when there is no other message the recipient is otherwise required to transmit (e.g., an acknowledgement to a registration update).

Active acknowledgement messages are defined as information messages with the string “ack” in the action member of body.  Additionally, the sequence member shall not be present and the message sequence number counter shall not be incremented internally.  The content member is undefined presently.

When acknowledging a message either actively or passively, the responding entity shall include the acksSequence member with the value set to the largest message sequence number for which the message serves as an acknowledgement.  The recipient of the active or passive acknowledgement shall assume that all messages having a message sequence number less than or equal to the value of ackSequence are acknowledged.

It is the responsibility of the message sender to re-transmit any messages not acknowledged and for which acknowledgement is required.

With the exception of the active acknowledgement message as defined in this section, receiving entities shall process messages sequentially and shall wait for messages to be retransmitted in the event that a message is lost, reordering messages as required to ensure sequential processing.

NOTE – since an active or passive acknowledgement message can be used to acknowledge multiple messages at once, it is possible for an endpoint to send multiple information messages, for example, and for the recipient to send a single message acknowledging the series of messages.  It is recommended that endpoints respond to all incoming messages without delay, but a single reply might be appropriate in the case an endpoint receives a number of messages in rapid succession or when messages get queued due to the loss of a message that later gets retransmitted.

## Updating instance information for active sessions

When an endpoint registers with a different Service Node, it may be assigned new user identifying information, such as names, aliases, or instanceURI.  If any of those data elements change when an endpoint is actively engaged in a communication session with another endpoint, the endpoint shall send an information message containing the new information in the sender member and having a body member that has the action value “refresh”.  All unacknowledged messages that are retransmitted and future messages sent from the peer endpoint shall use the updated endpoint information to ensure proper message delivery.

## Association syntax

This section defines some the common syntactical elements used as a part of the Container/Application association process.

entityNames "entityNames" [ 1* nameAndLanguage ]

associationToken "associationToken" UUID

associationStatus "status" ( associationIdle /

associationActive /

associationBusy,

associationOffline )

; Association Status Values

associationIdle : string /^idle$/

associationActive : string /^active$/

associationBusy : string /^busy$/

associationOffline : string /^offline$/

Some associations may be long-lived and persist even through reboots.  Such long-lived associations may utilize the same SessionID value, but endpoints might maintain active sessions following a reboot.  Even if the session state is preserved, a prolonged downtime might mean that the associated endpoint has decided that enough time has passed that it should assume the association to be terminated.  Therefore, it may be necessary to re-establish associations following a reboot and it is recommended that endpoints do make an attempt to re-establish previously held associations.  Note, however, that it is very likely that a new association token will be assigned when using a new session to re-establish an association; this is expected behaviour.

What the user sees on a user interface when an associated endpoint appears to have failed must be the same as if the associated endpoint sent an “offline” indication.  There is a difference between an associated endpoint going offline and terminating an association.  An association that is terminated should result in the associated device disappearing from the user’s interface entirely, not merely marked as offline or otherwise temporarily unavailable.

### Initiating an association

When an application or Container wishes to establish an association, it will send an initiate message with the string “associate” in the action member of body.  The content member contains the associate object.

associate

{

entityID,

?entities,

?vendorInfo,

?languages,

?entityNames,

?timeToLive,

associationStatus

}

The entityID is important to ensure that the peer can differentiate one Container or application from another.  The entityID value should persist through reboots or software updates.

### Rejecting an association

When rejecting an associate request, the endpoint will send a terminate message with the string “associateReject” in the action member of body.  The content member contains the associateReject object.

associateReject

{

?entities,

?vendorInfo,

associateRejectReason

}

associateRejectReason "reason" ( userAssociateRejectReason /

unspecifiedAssociateRejectReason )

; Registration rejection reasons

userAssociateRejectReason : string /^user$/

unspecifiedAssociateRejectReason : string /^unspecified$/

### Accepting an association

When an application or Container wishes to accept an association request, it will send an engaged message with the string “associated” in the action member of body.  The content member contains the associated object.

associated

{

entityID,

?entities,

?vendorInfo,

?languages,

?entityNames,

timeToLive,

associationToken,

associationStatus

}

The entityID is important to ensure that the peer can differentiate one Container or application from another.  The entityID value should persist through reboots or software updates.

### Refreshing an association

Each association has a finite time-to-live associated with it.  Prior to the expiration time of an association, the Container must attempt to refresh the association.

Endpoints associated with each other must also send a refresh message anytime the status of the endpoint changes.  Transmission of a refresh message for the purpose of updating the endpoint status shall reset the time-to-live counter.

To refresh an association, an endpoint sends an information message with the body containing the action value “refresh”.  The content member must contain the associationRefresh object.

associationRefresh

{

associationToken,

associationStatus

}

### Terminating an association or the associated signalling session

To terminate an association, either endpoint sends its associated endpoint a terminate message with the action value set to “disassociate”.  The content member will contain the disassociate object.

disassociate

{

associationToken,

disassociateReason

}

disassociateReason "reason" ( unspecifiedDisassociateReason /

userDisassociateReason /

offlineDisassociateReason )

; Unregister reasons

unspecifiedDisassociateReason : string /^unspecified$/

userDisassociateReason : string /^user$/

policyDisassociateReason : string /^policy$/

offlineDisassociateReason : string /^offline$/

Note that terminating an association may be permanent or temporary.

A temporary termination occurs when a device crashes or otherwise powers off, communication issues exist and the peer is presumed dead, or when a signalling session is terminated with an explicit reason of “offline”.  An attempt is made to re-established associations that are temporarily terminated when the endpoint is reactivated.  Temporary terminations give the user the appearance that the associated device is offline, but remains visible to the user.  This endpoint behaviour for temporary terminations should appear to the user to be the same as if a “refresh” message was sent indicating “offline”.

A permanent termination is one where the user (or the endpoint through policy) explicitly terminates the association.  This is done by sending a terminate message for any other reason than “offline”.  For such terminations, the associations should disappear from the user’s interface.

### Re-Establishing temporarily terminated associations

A device that signalled the “offline” reason or a device that reboots and loses the established signalling session shall re-establish any previously established associations by sending an initiate message to create a new association.  This requires that endpoint maintains only limited information about the endpoint with which an association was established (e.g., the remote endpoint’s alias address and, for applications, the instanceURI).

In the event that both endpoints are rebooted, there is a possibility that the association becomes difficult to recover automatically, since the Container might be assigned a new instanceURI.  The instanceURI is used by the applications to help ensure that association requests are directed to the proper Container.  The Container, however, does not utilize the instanceURI when re-establishing associations, since it is expected that each application will have a unique URI.

If an application is unable to contact a Container with which it previously had an association using the last-known instanceURI, it shall not attempt to send a message to the user’s alias as that might result in an attempt to associate with the wrong Container.

A Container must attempt to re-establish associations following a reboot by sending an initiate message to the alias address of the application indicating a request to associate.  Additionally, a Container should periodically attempt to re-establish associations that were lost, including those temporary terminations that were explicitly signalled.  A Container that defines application sets wherein only certainly applications are intended to be available at a given time may elect to re-establish associations only when a given application set is active.

To smooth the automatic re-establishment of associations, endpoints should maintain information about each entityID the user had previously authorized for association and that were not permanently terminated.  For such authorized associations, a new association should be accepted without prompting the user.  An application may want to expire such automatic authorizations after some lengthy period of time (perhaps configurable by the user) or allow the user to remove previously authorized associations manually.

A user should also be given the option to have applications automatically accept associations from a Container for a specified user, regardless of the entityID value.  In that way, if a user utilizes a different Container, it would be authorized for association automatically if the user so chooses.  Note, however, this behaviour assumes an implicit trust exists for the network to prevent users from assuming the identity of another user.  If there are no such guarantees made by the network, applications should not provide this option and should only utilize the entityID when authorizing automated associations.

_______________________