| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4524 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia Pty Ltd |  |  |  |
| Title: | Common NAT/FW Procedures for H.300-series packet-based multimedia systems |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution is to define a common simple NAT/FW bind test procedure for H.300-series packet-based multimedia systems to be used as input to formulate appropriate H.32x protocol NAT traversal strategies. It may also be used to open pinholes in NAT/FW for media.

Introduction

In H.323 systems, classic STUN (RFC 3489) used in conjunction with H.460.23 provides an excellent method to detect the behaviour of a NAT/FW device. The behaviour can then be used as input into NAT Strategy calculations as defined in H.460.24.

H.460.24 Annex B has been shown to be a very omnivorous in achieving direct IP connectivity between devices that reside behind different NAT/FW devices. Once both devices have been tested and detected as being suitable, the method uses an ordered controlled progression of UDP messages to yield direct media connectivity.

NAT/FW traversal can be a very complex topic. With the movement towards simplifying signalling and media flow establishment procedures NAT/FW procedures should not be overlooked.

Discussion

By simplifying RFC 3489 and using the core logic of H.460.24 Annex B a very simple NAT/FW solution is possible. This would involve a simple NAT binding test procedure and protocol specific NAT strategy implementation procedure.

Conclusion

Attached is a proposal draft for a new work item for a media binding test procedures.

Media binding test procedures in H.300-series packet-based multimedia systems

# Scope

This procedure may be used to test the binding behaviour of a NAT/FW device. The results of which may be used as input to formulate NAT/FW traversal strategies. The procedure may also be used, where instructed, to open pinholes in the NAT/FW device for the purpose of establishing media flows.

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[IETF RFC 3489]	IETF RFC 3489 (2003), STUN – Simple Traversal of User Datagram Protocol (UDP) Through Network Address Translators (NATs).

# Definitions

## Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

## Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1 Cone Binding: The behaviour of a NAT/FW device where a packet sent from the same IP and port on the inside of the NAT/FW to two or more different IP and Port on the outside of the NAT/FW device uses the same IP and Port (binding) on the outside of the NAT/FW device.

3.2.2 Symmetric Binding: The behaviour of a NAT/FW device where a packet sent from the same IP and Port on the inside of the NAT/FW to two or more different IP and Port on the outside of the NAT/FW device uses different IP and Port (binding) on the outside of the NAT/FW device.

3.2.3 Bind Test Server: A server on the public internet to assist in bind testing.

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

BTS            Bind Test Server

FW	Firewall

NAT	Network Address Translator

RTCP	Real-time Transport Control Protocol

RTP	Real-time Transport Protocol

STUN	Simple Traversal of User datagram protocol through Network address translators

UDP	User Datagram Protocol

# Overview

This recommendation implements a small subset of RFC3489 for the purpose of determining general NAT/FW behaviour for the purpose of implementing appropriate protocol specific NAT traversal strategies.

Not every NAT/FW device behaves in the same way when sending packets from the inside of the NAT/FW to the outside. RFC 3489 clause 5 specifies a number of NAT/Firewall behaviour variants. These variants can be grouped into 2 broad categories based on their binding behaviour. Table 1 details the variants and their binding behaviour.

Table 1  Mapping RFC3489 variants

| RFC3489 Variant | Binding behaviour |
| --- | --- |
| Full Cone | Cone |
| Restricted Cone |  |
| Port Restricted Cone |  |
| Symmetric | Symmetric |
| Symmetric Firewall |  |

## Cone Binding

When sending packets from a single IP address and port behind the NAT/FW to two or more IP/Port outside the NAT/FW the same IP/Port on the outside of the NAT/FW device is used.

There are 3 main binding security variations:

### Full Cone

A packet from any IP address and port on the outside of the NAT/FW will be accepted on an open binding.

### Restricted Cone

A packet will only be accepted from outside of the NAT/FW on an open binding if a packet has first been sent using that binding to the detected source IP address.

### Port Restricted Cone

A packet will only be accepted from outside of the NAT/FW on an open binding if a packet has first been sent using that binding to the detected source IP address and port.

## Symmetric Binding

When sending packets from a single IP/Port behind the NAT/FW to two or more IP/Port outside the NAT/FW, different IP/Port on the outside of the NAT/FW device is used. All bindings are unique and media must originate and terminate on the same IP/Port on both sides of the NAT/FW device.

# Message format

## Bind Message Header

All Bind Messages shall contain a Bind Message Header as defined in Figure 1.

The Header shall contain:

Type:  Message Type as defined in Table 1.

Length: Length in bits of the Bind Message body (refer clause 6.2) or 0 if no body

| 0 1 2 3 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 |  |
| --- | --- |
| Type | Length |

Figure 1  Bind Message Header

Table 1  Bind Message Type

| Type | Value |
| --- | --- |
| bindRequest | 0x0001 |
| bindResponse | 0x0101 |

## Bind Message Body

A Bind Message Body if present shall contain 1 or more Bind Address Elements (refer Figure 2).

The Element shall contain.

Type: The Address Type as defined in Table 2.

Family: The IP family: Default to 0x01 for IPv4.

Port: The Port number

Address: IP address

| 0 1 2 3 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 |  |  |
| --- | --- | --- |
| Type | Family | Port |
| Address |  |  |

Figure 2  Bind Address Element

Table 2  Address Type

| Message Type | Value |
| --- | --- |
| mappedAddress | 0x0001 |
| nextServer | 0x0005 |

# Bind Testing Procedures

A Bind Test Server (BTS) is stateless device or application that resides on the outside of the NAT/FW that listens on an IP address and port and can be used to perform bind tests. The following procedure requires 2 BTS.

A device behind the NAT/FW device shall send a small number of UDP packets from the allocated local IP Address and port to an IP address and port of a BTS.  The UDP packet shall contain the Bind Message header (refer Figure1) with the type set to bindRequest (as defined in table 1).

The supplying of the IP address and port of the BTS to the endpoint is beyond the scope of this document.

The BTS shall respond with a bindResponse message that shall contain the bind message header and

- A Bind Address element with type mappedAddress containing the apparent source address of the bindRequest. This will indicate the mapped address on the outside of the NAT/FW device.

- A Bind Address element with type nextServer containing an IP Address and port to send the subsequent bindRequest messages to. This address shall be used for the next bindRequest message.

The endpoint shall send a second bindRequest from the same local IP Address and port to the IP and port supplied in the nextSever element of the bindResponse.

The BTS shall respond with a second bindResponse.  Once received, the endpoint shall not send any further bindRequest.

Where no bindResponse is received from either bindRequest within a reasonable time, the NAT/FW shall be considered to be blocked. In this case no media flows are possible.

Bind Tests should be carried out periodically to confirm the binding behaviour of the NAT/FW device. Tests should be carried out at intervals greater than the binding timeout of the NAT/FW device to ensure existing bindings do not affect the test results.

# Interpreting the Bind Test results

Where the mappedAddress of the first bindResponse is the same as the mappedAddress of the second, this shall indicate that the NAT/FW is using the same binding in the NAT/FW and shall be considered as being a Cone Binding.  Where the mappedAddress are different it shall be considered as being Symmetric Binding.

Where the mappedAddress of both bindResponse are the same as the local IP and Port then it may be considered that there is no NAT/FW device present.

# Opening pinholes for media

The BTS as defined in clause 7 may also be used, where instructed, to open pinholes in the NAT/FW for symmetric media flows. In this case only one bindRequest is initiated from the local media IP Address and port and the mappedAddress of the reply bindResponse may be used to advertise to the remote device the open pinhole in the NAT/FW device to establish media flows.

__________________