| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4527 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia Pty Ltd |  |  |  |
| Title: | Updates to H.asset |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution is to update changes recommended in the meeting report in Geneva last November and to propose changes and updates to harmonise the activities on H.325

Discussion

Meeting report TD-102 Study Group16 meeting Geneva Oct 28 – Nov8  2013

C.267-H.ASSET: work item updates [Australia]

There were a number of editorial changes made to this contribution, the output of which appeared as TD 133/WP1.  It was agreed to accept this text as the new baseline text for the Recommendation.

Additional comments were made and the editor was requested to make changes accordingly.  Those include:

- A suggestion to return the list of registered applications when the association is confirmed, rather than having another round trip

- Remove the words "for further study" from the security considerations section, since it was not clear what was for further study; certainly, there will be further study on the entire specification

Those additional changes would appear in the document at the next meeting.

It is proposal to harmonise the activities between H.asset and H.325.

Conclusion

Using the output document TD-133 from the last meeting in Geneva the following changes have been made:

Meeting report:

Clause 9: applicationList field added to AssociationResponse to remove the requirement for an applistRequest message on association.

Clause 11  “For further study” removed.

Harmonisation with H.325 work.

Add Sequence and ackSequence to the AssetMessage ASN.1

Add Clause 12 to document harmonisation with H.325 work:

Add associateUpdate and associateTerminate ASN.1 fields to ApplicationIE to match similar functionality proposed in H.325.

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.Asset Ed. 0.1 | First version |
| H.Asset Ed. 0.2 | Output version of the SG16 meeting (Geneva, 28 Oct. – 8 Nov. 2013) |

Draft New Recommendation H.Asset

H.323 associative set (ASSET) feature

# Scope

This Recommendation describes the procedures for using H.225.0 Call Signalling as a method for the association of H.323 systems for the purpose of remote querying and invoking of applications to realise functionality beyond the capability of one single device.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.323]	Recommendation H.323 (2009), Packet-based multimedia communications systems.

[ITU-T H.225.0]	Recommendation H.225.0 (2009), Call signalling protocols and media stream packetization for packet-based multimedia communication systems.

[ITU-T H.235]	Recommendation H.235 (2003), Security and encryption for H-Series 
(H.323 and other H.245-based) multimedia terminals.

[ITU-T H.245]	Recommendation H.245 (2011), Control protocol for Multimedia Communications.

[ITU-T H.460.1]	Recommendation H.460.1 (2002), Guidelines for the Use of the Generic Extensible Framework.

# Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	application: A real-time function which resides on an Associative Simple Endpoint Type.

## 3.2	Terms defined in this Recommendation

3.2.1 pairing:  A unique association between an endpoint and an ASSET.

3.2.2 association token: A Globally Unique Identifier used to validate/authenticate ASSET pairings.

3.2.3 association life: The amount of time in seconds an association token shall remain active without any activity.

3.2.4 invoke token: A Globally Unique Identifier used to identity an application in the context of a call to invoke/terminate.

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| ASN.1 | Abstract Syntax Notation No. 1 |
| --- | --- |
| ASSET | Associative Simple Endpoint Type as defined in H.323 Annex x |
| GEF | Generic Extensible Framework |
| OID | Object Identifier |
| UI | User Interface |

# Feature description

This Recommendation defines a procedure wherein H.323 devices may associate with an ASSET for the purpose of discovering and utilising functionality on that device that otherwise would not be available to the H.323 device.

# Informative description

This Recommendation outlines the mechanism to use the H.225.0 call signalling connection as a method for a H.323 device to associate with an ASSET for the purpose of discovering applications on that ASSET and invoking then during the context of a call on the H.323 device.

# Capability advertisement

An ASSET shall have an H.225.0 endpointType of SET (bit 2 set) as defined in H.323 Annex x.

Endpoints and ASSETs capable of supporting this Recommendation shall advertise this capability via the generic extensibility framework (GEF) defined in [ITU-T H.323] and [ITU-T H.460.1].

An endpoint calling another endpoint shall advertise the ASSET feature via the supportedFeatures field in the Setup message. An endpoint that supports this Recommendation shall respond via the supportedFeatures field in the Connect Message. All messaging shall be conveyed in the ASSET Application parameter (refer Table 2)

An ASSET calling another ASSET within the context of an existing call shall advertise the ASSET feature via the neededFeatures field in the Setup message. An endpoint that supports this Recommendation shall respond via the neededFeatures field in the Connect Message. All messaging shall be conveyed in the ASSET Application parameter (refer Table 2)

All calls between endpoints and ASSETs shall set the conferenceGoal to callIndependentSupplementaryService. The ASSET feature shall not be advertised. One or more ASSET Associate parameters (refer Table 3) shall be included in the genericData field of the H323-UU-PDU. All other optional fields shall be omitted.

Table 1 defines the ASSET feature in this Recommendation.

Table 1 – Indication of ASSET feature

| Feature name: | ASSET Feature |
| --- | --- |
| Feature description: | H.323 Associative Simple Endpoint (ASSET) Feature |
| Feature identifier type: | OID |
| Feature identifier value: | 0.0.8.46x.0 |

Table 2 – ASSET Application parameter

| Parameter name: | ASSET Application |
| --- | --- |
| Parameter description: | ASSET Application Message |
| Parameter identifier type: | OID |
| Parameter identifier value: | 0.0.8.46x.0.1 |
| Parameter type: | octetString ASN.1 Encoded ASSETMessages (Refer Annex A) |
| Parameter cardinality: | One and only one |

Table 3 – ASSET Associate parameter

| Parameter name: | ASSET Associate |
| --- | --- |
| Parameter description: | ASSET Associate Message |
| Parameter identifier type: | OID |
| Parameter identifier value: | 0.0.8.46x.0.0 |
| Parameter type: | octetString ASN.1 Encoded ASSETMessage (Refer Annex A) |
| Parameter cardinality: | One or more |

Each AssetMessage shall be assigned an incremental sequence value. Where the AssetMessage is a response, the ackSequence field AssetMessage shall be present and contain the originating message sequence number.

# Pairing an endpoint with an ASSET

Endpoints wishing to pair shall place a call to the ASSET containing the ASSETMessage with the application field set to associateRequest. All other fields shall be omitted. The associateRequest shall contain a proposed association life.

How an endpoint discovers the  alias address of the ASSET to place the call and what value the association life to set to is beyond the scope of this Recommendation.

Upon receiving the associateRequest the ASSET may prompt the user to approve or decline the request. All subsequent messages shall be ignored until the pairing has been confirmed or declined.

Figures 1 and 2 below provide indicative call flows for endpoint and ASSET pairing

Figure 1 –Pairing rejection call flows

ASSET

Endpoint

Setup Non-Call(associateRequest)

Connect (associateConfirm)

ReleaseComplete

Figure 2 –Pairing confirmation and application listing call flows

Where pairing is rejected, the ReleaseComplete message shall be sent with reason set to genericDataReason.

Where the pairing has been approved, an association token (Globally Unique Identifier) shall be generated. This is transmitted back to the Endpoint via a Connect message containing an ASSETMessage with the application set to associateResponse. All other fields of the ASSETMessage shall be omitted.

The associateResponse shall contain:

- associateToken: The locally generated association token for the pairing

- timeToLive: association time to live.

- applicationList: Information on the applications supported by the ASSET and their current status.

and may include:-

- lifeTime: The time the association shall be valid without an associationUpdate.

The association timeToLive signalled shall be the lower of the endpoint and the ASSET. If there has been no activity for the given time, the association token shall no longer be valid for the ASSET.

The associationToken field in the ASSETMessage shall be presented in all future communication between the endpoint and the ASSET.

The applicationList field shall contain a SEQUENCE of ApplicationListItem.

Each ApplicationListItem shall contain:-

- applicationId: Unique object identifier (OID) for the application. (Refer Annex B).

and may include:-

- display: An ApplicationDisplay contains human readable name of the application and the language.

- avatar:  A URL path to an image to display for the application.

- status: An ApplicationState choice indicating the current availability of the application.

The Endpoint may request status information by sending an appListRequest to the ASSET. The ASSET shall respond with an appListResponse containing updated application status information.

The ASSET may at any time update the endpoint of any status change events by sending an unsolicited appListResponse.

From time to time and before timeToLive has expired, the endpoint shall send an associationUpdate to the ASSET to update its association. Where present, the association life shall also be incremented by the lifeTime value.

Where the ASSET has not received an associationUpdate before the expiry of the timeToLive then the endpoint shall be deemed to have gone offline.

Where the ASSET receives an associationUpate from the endpoint that is offline, the ASSET shall respond with an appListResponse to update the endpoint on current application status.

Where the ASSET has not received an associationUpdate before the expiry of the association life then the association shall be deemed to have expired and a fresh association is required.

Where the ASSET receives an associationUpate from the endpoint where the association is invalid or expired, the ASSET shall respond with a ReleaseComplete message with reason set to genericDataReason.

The endpoint may notify the ASSET to terminate temporarily or permanently the association. The endpoint shall send an associateTerminate with the AssociateTerminateReason set to either:

- offLine: The endpoint is going offline and association is temporarily inactive.

- userTerminate: The association is permanently terminated.

The ASSET may update the endpoint of any status change events by sending an unsolicited appListResponse.

# Invoking Application

The callIdentifer field of the ASSETMessage shall be present for all messages that are within the context of a call, as illustrated in Figure 3.

Figure 3 – Call and application invocation call flows

## Advertising and negotiating ASSET support during a call

The Setup message shall contain an applicationList.  The applicationList shall contain the applications that are locally available for the call.

The remote endpoint shall reply with its own applicationList containing the applications that are locally supported and available for the call.

Upon receiving an applicationList the endpoints shall generate a commonly supported application list and send a callIndication to each ASSET with applications available for the call to indicate that a call is in progress and that the identified applications may be invoked for the call.

The callAppList shall contain:-

- applicationId: unique object identifier (OID) for the application (Refer Annex B)

- aliasAddress: alias of the original call.

During a call the endpoints may update the local application list by sending a replacement applicationList.

## Invoking an application during a call

An application may be invoked by the endpoint or the ASSET supporting the application.

An endpoint may invoke one or many applications concurrently.

An endpoint shall start an application by sending a preInvokeRequest to the local ASSET supporting the required application.

The ASSET shall respond with a preInvokeResponse. Where the application is available for use the state field shall be set to available. Where the application is not available the state field shall be set to invokeFail and include the reason.

Where the preInvokeResponse returns available, the endpoint shall send an invokeRequest to the remote endpoint which is then forwarded to the ASSET containing the application. The application shall respond with an invokeResponse. Where the request is declined the invokeResponse shall be set to declined and contain the reason. Where the request is approved, the invokeResponse shall be set to approved and contain the necessary information for the remote ASSET it initiate the call.

The approved field shall contain:-

- applicationId: Unique object identifier (OID) for the application to invoke

- aliasAddress: The SEQUENCE of aliases the application can be contacted on.

- invokeToken: The ASSET generated invoke token to be used to start the application.

Where both endpoints attempt to invoke the same application at the same time, the two invoke token shall be binary compared from the most significant to the least significant octet where the lower value shall hold ascendancy and used by the party to initiate the call.

## Starting an application

An ASSET may start an application. To start an application the ASSET shall originate an invokeRequest to the local endpoint which is then forwarded on and the reply invokeResponse shall be forwarded back to the ASSET.

Where an endpoint invokes the application as per Clause 9.2, and after receiving the invokeResponse an invoke shall be sent to instruct the ASSET to start the application.

To start an application, the ASSET places a call to the H.225.0 Alias Address supplied in the aliasAddress field in either the invoke or invokeResponse. The Setup message shall include an invokeStartList containing the applications to start.

### Notifying an application has started

Upon successful call establishment the ASSET shall send to the endpoint an invokeNotify with the state field set to associated. Where the call is unsuccessful the state field shall be set to h2250Error.

## Terminating an application

To terminate an application an ASSET simply ends the call. Where an endpoint terminates the application a stopRequest shall be sent to the local ASSET to end the call.

The ASSET shall notify the endpoint that the application has terminated via a stopNotify.

## Hanging up a call

When the original call terminates, all running applications associated with that call shall also terminate and resources released. This is done by sending a callRelease to all associated ASSETs.

# Security considerations

All endpoint/ASSET pairing have a unique association token which may require approval from the ASSET before being issued. The association token must be present before an ASSET shall action any request from an endpoint.

An application shall not be invoked unless it has been issued a unique invoke token. The invoke token shall be a unique value linking an application to a call and shall be present in the Setup message. If a valid invoke token is not present the call shall be rejected with a Release Complete.

The endpoint may receive an invokeRequest to invoke an application and prompt the user to allow the ASSET to receive that request. Likewise the ASSET may receive an invoke or Setup message and prompt the user for approval before allowing an application to start.

# Interworking H.asset  with H.235.

{editors note: This section is a living section to be updated in accordance with updates to H.325}

## H.325 Message Header

| H.325 Field | H.2225.0 Equivalent |  |
| --- | --- | --- |
| sessionID | callIdentifer (the replying H.325 sessionID component omitted) |  |
| metaSessionID | not required |
| sender | AliasAddress |  |
| recipient | AliasAddress |  |
| sequence | H.asset AssetMessage sequence field |
| ackSequence | H.asset AssetMessage ackSequence field |
| messageType | H.asset AssetMessage Refer clause 12.2 |
| messageBody | H.asset AssetMessage Refer clause 12.2 |

Table 1 – H.325 Message Header / H.323 Mapping

## Message Mapping

| H.323 Message | H.asset ApplicationIE | H.325 messageType | H.325 messageBody |
| --- | --- | --- | --- |
| ReleaseComplete |  | information | ack |

Table 2 –H.323 H.asset / H.325 Common Mappings

### Association

| H.323 Message |  | H.asset ApplicationIE |  | H.325 messageType |  | H.325 messageBody |
| --- | --- | --- | --- | --- | --- | --- |
| Setup (Non-Call) |  | associateRequest |  | initiate |  | associate |
| Connect |  | associateResponse |  | engaged |  | associated |
| ReleaseComplete |  |  |  | terminate |  | associationReject |
| Setup (Non-Call) | associateUpdate | information | refresh |
| Setup (Non-Call) |  | associateTerminate |  | terminate |  | disassociate |

Table 3 –H.323 H.asset / H.325 Association Mappings

### Application

Annex A

ASN.1 Code for H.Asset

ASSET {itu-t(0) recommendation(0) h(8) 46x (0) } DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

AliasAddress,

GenericIdentifier,

GloballyUniqueID,

CallIdentifier,

TimeToLive

FROM H323-MESSAGES; -- See Rec. ITU-T H.225.0

--

-- ASSETMessages

--

ASSETMessages ::= SEQUENCE OF ASSETMessage

--

-- ASSETMessage

--

ASSETMessage ::= SEQUENCE

{

sequence       INTEGER (1..4294967295),

application      ApplicationIE,

ackSequence     INTEGER (1..4294967295)      OPTIONAL,

associateToken   GloballyUniqueID              OPTIONAL,

callIdentifier   CallIdentifier                OPTIONAL,

...

}

--

-- Application IE

--

ApplicationIE ::= CHOICE

{

associateRequest    AssociateRequest,

associateResponse   AssociateResponse,

associateUpdate     NULL,

associateTerminate  AssociateTerminateReason,

appListRequest      NULL,

appListResponse     SEQUENCE OF Applicationlistitem,

applicationlist     SEQUENCE OF Applicationlistitem,

callIndication      SEQUENCE OF ApplicationAvailable,

preInvokeRequest    Application,

preInvokeResponse   Applicationlistitem,

invokeRequest       ApplicationInvokeRequest,

invokeResponse      ApplicationInvokeResponse,

invoke              ApplicationInvoke,

invokeStartList     SEQUENCE OF ApplicationStart,

invokeNotify        ApplicationStatus,

stopRequest         SEQUENCE OF Application,

stopNotify          SEQUENCE OF Application,

callRelease         NULL,

...

}

--

-- AssociateRequest

--

AssociateRequest ::= SEQUENCE

{

timeToLive      TimeToLive,   -- seconds

...

}

--

-- AssociateTerminateReason

--

AssociateTerminateReason ::= CHOICE

{

offline         NULL,

userTerminate   NULL,

…

}

-- AssociateResponse

--

AssociateResponse ::= SEQUENCE

{

associateToken  GloballyUniqueID,

timeToLive      TimeToLive,   -- seconds

applicationList SEQUENCE OF Applicationlistitem,

lifeTime  TimeToLive     OPTIONAL,   -- seconds

...

}

--

-- Applicationlistitem

--

Applicationlistitem::= SEQUENCE

{

applicationId      GenericIdentifier,

display            SEQUENCE OF ApplicationDisplay   OPTIONAL,

avatar             IA5String                        OPTIONAL,

state              ApplicationState                 OPTIONAL,

...

}

--

-- ApplicationDisplay

--

ApplicationDisplay ::= SEQUENCE

{

language      IA5String                 OPTIONAL,    -- RFC4646 language tag

display       BMPString(SIZE(1..128))

}

--

-- ApplicationState

--

ApplicationState ::= CHOICE

{

available      NULL,

unavailable    NULL,

inuse          NULL,   -- In use by another call

associated     NULL,   -- In use by this call

invokeFail     InvokeFailReason,

...

}

--

-- InvokeFailReason

--

InvokeFailReason ::= CHOICE

{

unavailable    NULL,

inuse          NULL,   -- In use by another call

declined       NULL,

h2250Error     NULL,

...

}

--

-- Application

--

Application ::= SEQUENCE

{

applicationId      GenericIdentifier,

...

}

--

-- ApplicationAvailable

--

ApplicationAvailable ::= SEQUENCE

{

applicationId      GenericIdentifier,

aliasAddress       SEQUENCE OF AliasAddress,

...

}

--

-- ApplicationInvokeRequest

--

ApplicationInvokeRequest ::= CHOICE

{

applicationId      GenericIdentifier,

...

}

--

-- ApplicationInvokeResponse

--

ApplicationInvokeResponse ::= CHOICE

{

approved   ApplicationInvoke,

declined   InvokeFailReason,

...

}

--

-- ApplicationInvoke

--

ApplicationInvoke ::= SEQUENCE

{

applicationId      GenericIdentifier,

invokeToken        GloballyUniqueID,

aliasAddress       SEQUENCE OF AliasAddress,

...

}

--

-- ApplicationStart

--

ApplicationStart ::= SEQUENCE

{

applicationId      GenericIdentifier,

invokeToken        GloballyUniqueID,

...

}

END

Annex B
OIDs Defined for this Recommendation

Table B.1 – OIDs Defined for this Recommendation

| OID | Clause Reference |
| --- | --- |
| { itu-t (0) recommendation (0) h (8) 46x baseline(0) } | 7 |
| { itu-t (0) recommendation (0) h (8) 46x baseline(0) associate(0) } | 7 |
| { itu-t (0) recommendation (0) h (8) 46x baseline(0) application(1) } | 7 |

__________________