| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4528 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2, 5/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia Pty Ltd |  |  |  |
| Title: | Amendment to H.245 MediaTransportType to support SCTP over UDP |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This proposal is to amend the H.245 ASN.1 to include support for the transport of media via SCTP carried over a UDP transport (RFC6951)

# Introduction

It was discussed in the last SG16 Meeting on the use of RFC6951 UDP Encapsulation of Stream Control Transmission Protocol (SCTP) Packets for End-Host to End-Host Communication for the transport of various reliable and unreliable media over a single unreliable UDP transport.

# Discussion

This contribution is an attempt to address an issue with multimedia communication and transmission of media over a reliable transport in a NAT/FW environment. Currently such transport is almost impossible and all current H.323 NAT/FW solutions require the use of unreliable UDP. SCTP (RFC4960) supports the transmission of both reliable and unreliable data and when encapsulated in UDP (RFC6951) provides a method by which media may be transmitted reliably even if the underlying transport is unreliable. SCTP also provides a method to mix multiple streams of reliable and unreliable data over a single media transport.

RFC6951 opens a wide range of opportunities for the transmission of new types of media services and may have use cases in the area of E-health and the transport of real time data and with the work being carried out in telepresence systems in Q5/16.

# Conclusion

Attached is a proposal to amend H.245 ASN.1 to include support for SCTP carried over UDP in the Media Transport type field which is transmitted in the OpenLogicalChannel and to include a choice of whether the media stream is reliable or unreliable and to assign the Stream Identifier (RFC4960 cl. 3.3.1) within the SCTP stream.

NOTE: This contribution is only an update the H.245 ASN.1 to support the opportunity to implement RFC6951 in H.323 systems.

[Begin]

MediaTransportType ::= CHOICE

{

ip-UDP						NULL,

ip-TCP						NULL,

atm-AAL5-UNIDIR				NULL, -- virtual circuits used as unidirectional

atm-AAL5-BIDIR					NULL, -- virtual circuits used as bidirectional

...,

atm-AAL5-compressed			SEQUENCE

{

variable-delta				BOOLEAN,

...

},

ip-UDP-SCTP                      CHOICE

{

reliable                     INTEGER (0..65535), -- Stream Identifier

unreliable                   INTEGER (0..65535)  -- Stream Identifier

}

}

[End]

__________________