| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4531 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co. Ltd. |  |  |  |
| Title: | H.TPS-AV: Considerations for Telepresence parameters to be signalled |  |  |  |
| Purpose: | Discussion |  |  |  |

Summary

This contribution provides an initial analysis of the parameters identified in H.TPS-AV as to which need to be signalled from one Telepresence system to another to achieve interoperability.

# Introduction

Draft Rec. H.TPS-AV [1] lists many parameters which collectively define a high-performance Telepresence system. With the creation of the new work item H.TPS-SIG "Signalling for telepresence-enabled conferencing" it is timely to consider which of these parameters need to be signalled between Telepresence systems to provide inter-operability.

# Discussion

As a starting point, the tables below list all of the parameters specified in H.TPS-AV with an initial view (Yes/No) of whether they need to be signalled at session initiation and/or during the session, along with any other remarks.

7.1	Capture Related Parameters

7.1.1	General Parameters

| Parameter | Need for signalling at session initiation | Need for signalling during session | Remarks |
| --- | --- | --- | --- |
| Captured Media Type | Y | Y |  |
| Scene Description | Y | Y |  |
| Capture Scene Entry Description | Y | Y |  |
| Scene Switch Policy | Y | N |  |
| Language | Y | N |  |
| Priority | Y | Y? | May not need to be signalled during session |
| Embedded text | Y | Y |  |
| Related to | Y | Y |  |
| Presentation | Y | Y |  |
| Composed | Y | Y |  |
| Auto switched | Y | Y |  |
| Capture description | Y | Y |  |
| Area scale | Y | N |  |
| Mobility of capture | Y | N |  |
| View | Y | Y |  |
| Overall maximum bit-rate | Y | N |  |
| Overall maximum macroblocks per second | Y | N |  |
| Max capture encodings | Y | N |  |

7.1.2	Visual parameters

| Parameter | Need for signalling at session initiation | Need for signalling during session | Remarks |
| --- | --- | --- | --- |
| Video Colour Gamut | Y | N |  |
| Video LumaBitDepth | Y | N |  |
| Video ChromaBitDepth | Y | N |  |
| Effective Resolution | N? | N | May need to be signalled at session initiation |
| Number of video captures | Y | N |  |
| Capture area | Y | Y |  |
| Capture point | Y | Y |  |
| Point on line of capture | Y | Y |  |
| Maximum video bitrate | Y | N |  |
| Maximum H.264 macroblocks per second | Y | N |  |
| Maximum video resolution width | Y | N |  |
| Maximum video resolution height | Y | N |  |
| Maximum video framerate | Y | N |  |

7.1.3	Audio parameters

| Parameter | Need for signalling at session initiation | Need for signalling during session | Remarks |
| --- | --- | --- | --- |
| Number of audio captures | Y | N |  |
| Audio capture spatial information | Y | Y |  |
| Audio channel format | Y | N |  |
| Maximum audio bitrate | Y | N |  |
| Audio Level | Y | N |  |
| Sending Loudness Rating | Y? | N | May not need to be signalled |
| Weighted Microphone-Loudspeaker Coupling Loss | N | N |  |

7.1.4	Delay parameters

| Parameter | Need for signalling at session initiation | Need for signalling during session | Remarks |
| --- | --- | --- | --- |
| End to End Video Delay | N | N |  |
| End to End Audio Delay | N | N |  |
| Audio / Video Synchronization | N | N |  |

7.2	Render related Parameters

7.2.1	Video parameters

7.2.2	Audio parameters

| Parameter | Need for signalling at session initiation | Need for signalling during session | Remarks |
| --- | --- | --- | --- |
| Receiving Loudness Rating | N | N |  |

7.3	Telepresence system environment parameters

7.3.1	Colour and lighting parameters

| Parameter | Need for signalling at session initiation | Need for signalling during session | Remarks |
| --- | --- | --- | --- |
| Illuminant | Y | N? | May need to be signalled if lighting changes during session |
| Colour Rendering Index | N | N |  |
| Colour temperature | N | N |  |

7.3.2	Acoustic Parameters

| Parameter | Need for signalling at session initiation | Need for signalling during session | Remarks |
| --- | --- | --- | --- |
| Reverberation time | N | N |  |
| Ambient noise | N | N |  |
| Sound insulation | N | N |  |

Further points to be elaborated include whether an additional column is necessary to identify how the parameters should signalled (i.e. with what kind of protocol and when), and whether the above list is exhaustive. For example, should we include other parameters that are needed to be signalled for a conventional multimedia conference or just focus on those that are needed to provide the telepresence experience?

# Proposal

It is proposed that the above table be used as a starting point for discussion on parameters to signalled for Telepresence.

# References

[1] Draft Rec. H.TPS-AV "Audio/video parameters for telepresence systems", TD 89R1 (WP 1/16), November 2013

__________________