| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4563 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.RTPMUX: Behaviour and directionality of different grouping semantics |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution from Ericsson provides examples and further behavioural description of the different grouping semantics and further insight on its usage in H.248 vs. other H.248 mechanisms.

# Discussion

The behavioural description of the different grouping semantics needs further elaboration, especially to distinguish when the grouping semantics are needed and when the behaviour can be achieved by other existing H.248 mechanisms. Some further analysis on the use of the directionality is beneficial.

Some of these issues were captured already in the last meeting in editor’s notes. The editor’s notes are also addressed by this contribution.

# Proposal

The contribution proposes the following changes.

2.1 Addition of references

[Begin Correction]

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (09/2005), Gateway Control Protocol: Version 3

[ITU-T H.248.57]	Recommendation ITU-T H.248.57 (03/2013), Gateway control protocol: RTP control protocol package

[ITU-T H.248.65]	Recommendation ITU-T H.248.65 (03/2009), Gateway control protocol: Support of the resource reservation protocol

[IETF RFC 3524]	IETF RFC 3524 (04/2003), Mapping of Media Streams to Resource Reservation Flows

[IETF RFC 3550]	IETF RFC 3550 (07/2003), RTP: A Transport Protocol for Real-Time Applications

[IETF RFC 4566]	IETF RFC 4566 (07/2006), SDP: Session Description Protocol

[IETF RFC 4588]	IETF RFC 4588 (07/2006), RTP Retransmission Payload Format

[IETF RFC 5583]	IETF RFC 5583 (07/2009), Signaling Media Decoding Dependency in the Session Description Protocol (SDP)

[IETF RFC 5761]	IETF RFC 5761 (04/2010), Multiplexing RTP Data and Control Packets on a Single Port

[IETF RFC 5888]	IETF RFC 5888 (2010), The Session Description Protocol (SDP) Grouping Framework

[IETF RFC 5956]	IETF RFC 5956 (09/2010), Forward Error Correction Grouping Semantics in the Session Description Protocol

[IETF draft taxonomy]	IETF draft-lennox-raiarea-rtp-grouping-taxonomy (201X), A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources

[End Correction]

2.2 Stream list with single element in the grouping semantics

The ABNF specification for the StreamID List within the groupse property allows also for a single value, which is not meaningful in the context of grouping semantics, as at a minimum two streams should be involved. However a single value may be preferable in the AuditCapability reply.

[Begin Correction]

### 9.1.1	Group Semantics

|  | Property Name: | Group Semantics |
| --- | --- | --- |
|  | Property ID: | groupse (0x0001) |
|  | Description: | This property allows a MGC to indicate which media streams to group and for which purpose they are grouped. This allows the MG to allocate resources accordingly. For example: providing a common address/port in response to a wildcarded request from the MGC in case multiplexing is supported. The property allows a MGC to assign a group to a single stream directional or both directions via the optional “Direction” element. When not included a default of “bothway” is assumed. Several sets of grouped streams are allowed per Termination. |
|  | Type: | Sub-list of String |
|  | Possible values: | Each element of the sub-list follows the group-attribute syntax in [IETF RCF 5888], here represented using type “Group”: Group = Semantics 1*WSP StreamIDList[1*WSP Direction] ; (NOTE 1) WSP = SP/HTAB StreamIDList = StreamID *(WSP StreamID) StreamID = UINT16 Semantics = token token-char = %x21 / %x23-27 / %x2A-2B / %x2D-2E / %x30-39/ %x41-5A / %x5E-7E token = 1*(token-char) Direction = Bothtoken / Sendtoken / Receivetoken Bothtoken = “B” Sendtoken = “S” Receivetoken = “R” (Note 2) As defined by [ITU-T H.248.1 Annex B]. Note 1: The syntax functionally duplicates the Grouping attribute syntax from [IETF RFC 5888] with the optional addition of a direction parameter. Note 2: This syntax definition allows for a Stream ID list containing one single value. This case is only allowed in a reply to an AuditCapabilities command, where the StreamId shall be set to 0. |
|  | Default: | None. |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |

[End Correction]

[Begin Correction]

#### 9.6.1.2	Via auditing

An MGC may audit the Packages Descriptor to determine if a MG supports media grouping. If the response contains the “Media Grouping Package” (mgroup) it can assume that the MG support media grouping. The MGC may perform an H.248 AuditCapability request command on the sgroupse and/or rgroupse properties to determine which semantics are supported by the MG. In this case the MG shall respond with the supported semantic tags and the Stream ID set to 0.

An MGC controlling a MG that supports the Media Grouping package can instruct the MG to group two or more sent and/or received media streams. Each media stream (including the sent and received directions) is described in its own Stream Descriptor and all are included in the same Media Descriptor.

The MGC can indicate to the MG:

- Which sent media streams to group

- Which received media streams to groupFor which purpose the media streams are grouped.

In general the mgroup properties can be used together with the H.248 Reserved Group concept (see clause 7.1.7/[ITU-T H.248.1] as the ReserveGroup property applies to alternatives related to a particular H.248 stream. The mgroup package relates multiple H.248 streams. If ReserveGroup is used with the mgroup package properties, the MGC should ensure that any semantic is applicable to each of reserved groups on the applicable stream.

[End Correction]

2.3 Removal of obsolete note

The note below was introduced on the assumption that the streams would be tagged as per SDP mid attributes. However, now it has been agreed to use the stream IDs instead. Therefore the note is no longer valid.

[Begin Correction]

### 9.6.2	Mapping of group attributes in H.248

An MGC that sends or receives SDP media grouping related attributes in an SDP Offer/Answer dialogue should map those attributes as follows when sending the command to the MG:

- Each m-block is mapped into one Stream Descriptor.

- The “a=mid” attribute of each m-block is dropped in the Local/Remote Descriptor of the corresponding Stream Descriptor, instead the StreamID is used for the index in the group

- The “a=group” attribute is mapped to the mgroup/groupse property and included in the TerminationState Descriptor. The content of the property, a sub-list of string, should be follow a copy of the “a=group” attribute. The semantic is retained and the mid value is replaced with the corresponding StreamID.

-

[End Correction]

2.4 Directionality of the LS semantics

The following proposal addresses the directionality of the LS semantics and suggests some text to align its usage in H.248 with the SDP negotiation.

[Begin Correction]

#### 9.6.3.1 Lip Synchronisation (LS)

The “Lip Synchronisation” semantic is defined in [IETF RFC 5888]. As per clause 7[IETF RFC 5888], the MG must synchronise the play out of H.248 Streams listed together in the same “LS” marked group.

9.6.3.1.1 Directionality

In most scenario where LS is used both the video and audio would be bi-directional. However there are some scenarios such as synchronization of source audio and simultaneous translation where lip sync may only be applied in one direction. [IETF RFC 5888] does not define an SDP mechanism to negotiate an unidirectional use of the LS semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. The mgroup/groupse property allows for defining an unidirectional use of the LS semantics also if the grouped streams are all bi-directional. However, as this use does not have an SDP counterpart between SDP peers, it is discouraged.

[End Correction]

2.5 Behaviour and directionality of the FID semantics

Several examples are described in this proposal to address the editor’s note and distinguish when the FID semantics is needed in H.248 and when it can be emulated with other mechanisms. It also addresses the directionality of this semantics.

[Begin Correction]

#### 9.6.3.2 Flow Identifier (FID)

The “Flow Identifier” semantic is defined in [IETF RFC 5888]. The use of the flow identifier with H.248 is in some cases redundant due to the H.248 connection model and H.248 usage of SDP. H.248 has the StreamID identifier which uniquely identifies media flows. Each H.248 stream may utilise the Reserve Group/Reserve Value (see 7.1.8.2.3/[ITU-T H.248.1]) construct which enables multiple configurations to be set for a H.248 stream. This allows a single H.248 stream to have multiple sets of characteristics of which one set is active at a particular point in time.

The following example described in 8.4.1/[IETF RFC 5888] can be emulated with the use of the Reserve Group mechanism:

Example 1

Two audio streams are negotiated between two peers with different audio codecs. Only one is active at a time while the other one is muted. The peers indicate the same IP address for receiving both streams but different ports.

The following example also described in 8.4.1/[IETF RFC 5888] can not be emulated with the use of the Reserve Group/Reserve Value constructs and therefore requires the use of the H.248 mgroup/groupse property:

Example 2

Three audio streams are negotiated between two peers. The first two are bi-directional and use different audio codecs. Only one of these two is active at a time while the other one is muted. The third is unidirectional and has two payload types, one corresponding to each of the codecs used by the other two streams. This third stream is always carrying a copy of the active stream, either of the first or the second stream. Both active streams are received at the same IP address at different ports. The third stream is received at a different IP address.

NOTE: Reserve Group/Reserve Value can apply to alternatives using the same or different IP addresses or ports.

Another example of use of the FID semantics that can not be emulated with the ReserveGroup/Reserve Value contructs is the one described in 8.7/[IETF RFC 4588]:

Example 3

Two SDP peers negotiate one audio and one video stream. In addition they negotiate another audio and another video streams used for retransmission. The audio source and retransmission streams are grouped with the FID semantics and the video source and retransmission streams are grouped with the FID semantics.

9.6.3.2.1 Directionality

[IETF RFC 5888] does not define a mechanism to negotiate an unidirectional use of the FID semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. The mgroup/groupse property allows for defining a unidirectional use of the FID semantics also if all the grouped streams are bi-directional. However, as this use does not have an SDP counterpart between SDP peers, it is discouraged.

[End Correction]

2.6 Behaviour and directionality of the SRF semantics

The following proposal addresses  the relevance of the SRF semantics in H.248 and the combination with the tools defined in H.248.65. Further description of the use of H.248.65 signals and events in combination with the SRF semantics is provided in a separate contribution addressing the stream representative topic. It also addresses the directionality of this semantics.

[Begin Correction]

#### 9.6.3.3 Single Reservation flow (SRF)

The “Single Reservation flow” semantic is defined in [IETF RFC 3524]. It indicates that media (described by m=lines) should be mapped to the same resource reservation flow. [ITU-T H.248.65] defines a package for the support of RSVP. The initiation of a resource reservation request is typically explicitly requested through the use of a H.248 signal. Any identification of flow is part of the parameters to the signal.

If the wanted behaviour is the allocation and subsequent policy enforcement of a predefined total bandwidth for a number of combined streams, the use of the SRF semantics is required as opposed to allocate individual bandwidths for each stream. The use of [ITU-T H.248.65] may also be applicable in combination with the mgroup package. In that case sent or received RSVP messages will refer to all grouped media combined. For that reason, signals and events of the rsvp package should be sent or activated in the stream representative when they are conveying RSVP messages related to a grouped media streams (see clauses 9.6.5.1 and 9.6.5.2).

Contributor’s note: there is another Ericsson contribution adding text to these clauses.

9.6.3.3.1 Directionality

[IETF RFC 3524] does not define a mechanism to negotiate an unidirectional use of the SRF semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. Thus it must be assumed that the negotiation of multiple streams grouped with the SRF semantics between peers is bi-directional if the grouped streams are also bi-directional.

However, this semantics is commonly associated to RSVP to reserve the required bandwidth. RSVP is unidirectional in nature. Thus the SRF semantics may require two RSVP sessions. The use of RSVP in a MG requires [ITU-T H.248.65] which has the inherent mechanisms to determine the applicable direction, e.g. sending of an RSVP Path message with the H.248 rsvp/path signal is related to the outgoing stream while sending an RSVP  Resv message with the H.248 rsvp/resv signal is related to the incoming stream.

If the mgroup/groupse property is used with the direction parameter together with the SRF semantics, the sent or received RSVP messages should apply or not to all grouped streams depending on the direction. For example, if the MGC sends mgroup/groupse = “SRF 1 2 R” to the MG, an RSVP Resv message sent by that termination will refer to both streams 1 and 2 combined and the MGC should set the rsvp/resv signal in the group representative stream, but an RSVP Path message will refer only to the individual stream where the signal rsvp/path is set.

[End Correction]

2.7 Emulation of the ANAT semantics with the reserveGroup construct

The existing draft already states that the ANAT grouping semantics is redundant in H.248. This proposal describes more in detail how to emulate it with the ReserveGroup construct.

[Begin Correction]

#### 9.6.3.4 Alternative Network Address Types (ANAT)

The “Alternative Network Address Types” semantic is defined in [b-IETF RFC 4091]. The ANAT semantic allows an SDP offerer to indicate that multiple network address types may be used for a single logical media stream. This semantic is redundant in H.248 as it offers multiple methods for indicating or requesting address types. A MGC may use “over specification” (see 7.1.8.2.2/[ITU-T H.248.1) to provide a list of network address types for a particular H.248 Stream. A MG will then chose an appropriate network address type.

An MGC that wants to initiate an SDP offer with the ANAT semantics can reserve resources for both transport types using two property groups. Both property groups will contain the same information except for the IP address information (e.g. the c= attribute) which will be partially wildcarded, and will request an IPv4 address in one of the groups and an IPv6 address in the other group. The MGC will set ReserveGroup to true. Thereafter it will send the SDP offer.

The MGC that receives the SDP offer with the ANAT semantics may be able to determine by itself which of the transport types to use, e.g. by certain call or traffic distribution criteria. In that case it will only indicate one address type when adding the termination in the MG. The MGC may also defer that decision to the MG. In that case it will send two property groups, like the initiating MGC, but in this case, it will set ReserveGroup to false. The MG will only allocate resources for one of the transport types and indicate the chosen one to the MGC in the ADD reply.

The initiating MGC, when receiving the SDP answer which contains the chosen transport type, may send a MOD to the MG, this time excluding the property group with the transport type that was not chosen.

[End Correction]

2.8 FEC-ER semantics

As already captured in an editor’s note, when a session involves more than one source and one repair stream it may not be possible using simply the a=rtpmap attribute to identify to which source each repair streams corresponds, especially if the relationship between source and repair streams is not 1 to 1. The following proposal addresses the editor’s note and provides further description as to when the SRF semantics are needed in H.248.

It also addresses the directionality of this semantics.

[Begin Correction]

#### 9.6.3.6 Forward Error Correction FR (FEC-FR)

The “Forward Error Correction FR” semantic is defined in [IETF RFC 5956]. This is used to associate source and repair flows for the purposes of forward error correction (FEC). If this semantic is received the MG should utilise the source and repairs H.248 Streams listed together in the same “FEC-FR” marked group for the purposes of FEC.

The MGC should indicate whether a flow is a source or repair flow through the use of coding related parameters (i.e. a=rtpmap:111 1d-interleaved-parityfec/90000 and a=fmtp:111 L=10; D=10; repair-window=400000).

If the seesion consists only of one source stream and one repair stream, the MG can unambiguously identify by means of the above attribute, which is the source and which the corresponding repair stream. The mgroup/groupse property may be omitted in this case. However, if there are multiple source streams and/or multiple repair streams, it may not be possible to unambiguously identify to which source each repair stream corresponds. Furthermore, the forward error correction mechanism allows for different types of correction schemes: n repair streams may be related to m source streams. In all these cases the SRF semantics is required to indicate the association between repair and source streams.

9.6.3.6.1 Directionality

[IETF RFC 5956] does not define a mechanism to negotiate an unidirectional use of the FEC-FR semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. The mgroup/groupse property allows for defining a unidirectional use of the FEC-FR semantics also if all the grouped streams are bi-directional. However, as this use does not have an SDP counterpart between SDP peers, it is discouraged.

Thus, the preferred method to specify the use of forward error correction only in one direction is with the use of StreamMode (SendOnly or RecvOnly) or the SDP direction attribute (a=sendonly or a=recvonly) in the repair stream(s).

[End Correction]

2.9 Removal of the CS semantics

The CS semantics seem to have been abandoned. The mentioned draft has expired and the FLUTE RFC 3926 does not contain it any more. It is proposed to remove this clause.

[Begin Correction]

[End Correction]

2.10 Directionality of the DDP semantics

The following proposal addresses the directionality of the DDP semantics and suggests some text to align its usage in H.248 with the SDP negotiation.

[Begin Correction]

#### 9.6.3.8 Decoding Dependency (DDP)

The “Decoding Dependency” semantic is defined in [IETF RFC 5583]. This semantic may be used with H.248 as well as other SDP attributes (e.g. a=depend) to describe decoding dependencies for layered codings.

[IETF RFC 5583] does not define a mechanism to negotiate an unidirectional use of the DDP semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. The mgroup/groupse property allows for defining a uni-directional use of the DDP semantics also if all the grouped streams are bi-directional. However, as this use does not have an SDP counterpart between SDP peers, it is discouraged.

[End Correction]

2.11 Directionality of the BUNDLE semantics

The following proposal addresses the directionality of the BUNDLE semantics.

[Begin Correction]

9.6.3.9 Multiplexing Negotiation Using Session Description Protocol (BUNDLE)

The “BUNDLE” semantic is defined in [IETF draft-ietf-mmusic-sdp-bundle-negotiation]. It used to describe that two or more RTP streams use a single 5-tuple (i.e. multiplex multiple RTP streams onto a single 5-tuple). As [IETF draft-ietf-mmusic-sdp-bundle-negotiation] only describes the use of BUNDLE for IP transports it shall not be used with Streams using non-IP transports.

As described in clause 8 it is possible to request that a MG use a multiplexed RTP stream however there are a number of deficiencies.

Firstly the MGC is unaware if the MG supports RTP multiplexing. In order to determine if the MG supports RTP multiplexing the MGC may audit the media grouping package to determine if the “BUNDLE” semantic is supported (see clause 10.6).

Secondly there is no way to wildcard CHOOSE the address/port information from several H.248 Streams and request that a single transport connection (i.e. multiplexed RTP stream) be used. The media grouping package contains the “sent group semantics” and “received group semantics” properties that in conjunction with the BUNDLE attribute allows the MGC to indicate that two or more H.248 Streams are multiplexed together. MGs that support the media grouping package should check the “sent group semantics” and “received group semantics” properties when processing wildcarding requests for multiple H.248 Streams. If the H.248 Streams match the association given by these properties then the same value should be returned for those H.248 Streams.

See clause 10 for more information on the use of the BUNDLE semantics and how to support RTP multiplexing in H.248.

9.6.3.9.1 Directionality

BUNDLE is symmetric in nature. If it is used, it applies to source and destination.

-In practice, however, the required behaviour from the MG specific to this semantic, is related to the Local Descriptor, i.e. to the received media. The use of the same address and port for multiple streams in the Remote Descriptor is usually not problematic in the MG, also when BUNDLE is not used.

Therefore the use of the direction parameter in the mgroup/groupse property is superfluous in case of the BUNDLE semantics.

[End Correction]

2.12  Applicability of specific error codes

It is not clear if the two examples at the end of the clause are referring to the third bullet, or to the following paragraph, i.e. when a more specific error is returned. Since the examples do not propose an specific error code, it is assumed that error 473 is still applicable in these examples and therefore the last two paragraphs are swaped.

[Begin Correction]

### 9.6.4	Error handling

In the following error cases the MG should return an error code 473 “Conflicting Property Values”:

- If any of the stream identities included in the groupse property does not exist in the termination.

- If the stream identity in the grouprep property exists already in the termination.

- Depending on the grouping semantics if there is a mismatch between the mgroup/groupse property and the Local or Remote Descriptor.

For example: An error may be generated if the MGC uses the BUNDLE media grouping and specifies a StreamID of a H.248 Stream whose Local Descriptor does not have the same address/port as the other H.248 Streams in the group (and is not updated in the current command). Another example is if an error may be generated if MGC uses the “LS” semantic and specifies the StreamIDs of H.248 streams where the MG cannot maintain lip sync.

A more specific error code may be returned if available.  The underlying reasons for generating an error may depend on the semantics associated with the group.

[End Correction]

2.13 Bibliography list

Since the ANAT semantics is not needed in H.248, it is proposed to add the relevant RFC as informative only

[Begin Correction]

Bibliography

[b-IETF RFC 4091]	IETF RFC 4091 (07/2005), The Alternative Network Address Types (ANAT) Semantics for the Session Description Protocol (SDP) Grouping Framework

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title

[End Correction]

__________________