| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4566 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.TCP: TCP role assumption, blocking semantics and interlinkage |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution from Ericsson proposes changes in the H.248.TCP draft in relation to the TCP role assumption and the scope of the ‘Bearer connection establishment blocking’ property.

# Introduction

The use of the term “TCP role assignment” is used throughout the draft in spite of the fact that there is no explicit role assignment by the MGC. There is only an implicit assignment for the client role together with the initiation of the establishment procedure if this is ordered by the MGC. In no case there is a role assignment prior to the initiation of the establishment procedure that could serve as preparation.

The semantics of the bceb property have been discussed during the last two meetings, as to whether or not they affect outgoing establishment procedures, also in the light of the new H.248.SEPLINK draft.

# Discussion

In the starting phases of the H.248.TCP draft Q3 considered the option of MGC explicitly assigning a role to the TCP endpoint (client or server). This role assignment could set the TCP endpoint in preparedness to receive a TCP SYN (server role) or to initiate an outgoing establishment procedure, i.e. send a TCP SYN (server role). This option has been later on abandoned in favour of considering that the TCP endpoint role is assumed as a consequence of either an incoming establishment procedure (server role) or an outgoing establishment procedure (client role). Although since then, the text has been further elaborated to try to be consistent with this approach, there are still descriptions that seem to be based on the existence of a role assignment or may mislead the reader into this belief. This contribution proposes changes to make the text consistent throughout the draft with the following facts:

a) there is no explicit role assignment

b) the role is assumed (adopted) as a consequence of incoming or outgoing establishment procedures

c) the MGC has the ability to trigger outgoing procedures and to allow or prevent initiation of outgoing procedures triggered by procedures in the associated SEP [H.248.SEPLINK] or by procedures in another layer of the same SEP [H.248.SEPLINK]

d) the MGC has the ability to prevent incoming procedures

Furthermore, since the MGC has always control of outgoing procedures, via explicit use of the EstBNC signal or by allowing interlinkage procedures [H.248.SEPLINK] it is unnecessary to let these procedures be controlled additionally by the bceb property. The use of the bceb property should only affect the incoming procedures.

# Conclusion

It is proposed to adopt following changes in order to address the issues subject of the discussion above.

[Begin Correction]

### 8.1.1	Incoming Bearer connection establishment blocking

|  | Property Name: | Incoming bearer connection establishment blocking |
| --- | --- | --- |
|  | Property ID: | bceb (0x0001) |
|  | Description: | This property defines the MG availability to accept or reject incoming TCP bearer connection establishment procedures. NOTE 1: See Annex A concerning a state model based illustration. NOTE 2: This capability might be relevant for the prevention of other TCP processing behaviour such as "TCP relay" or "TCP merge" modes (see [ITU-T H.248.84]). |
|  | Type: | Enumeration |
|  | Possible values: | See Table 15 Table 15 – Bearer connection establishment preparedness (of TCP endpoints) Value; MG behaviour. "blocked" (0x0001); - Bearer resources are prepared (i.e., resources for TCP endpoint are reserved and allocated to H.248 termination/SEP); - TCP role: not determined; prevents assumption of server role. - TCP connection state = "CLOSED" (Figure A.1); - H.248 counterpart state = "Blocked" (Figure A.3); - Any incoming TCP packet shall be silently discarded (e.g., TCP-SYN requests shall not be acknowledged), there is no state transitioning from "CLOSED" (NOTE 3). - Any outgoing TCP bearer connection control procedures are not affected: - Signal EstBNC (clause 8.3.1): start 3-way handshake by sending of TCP-SYN; TCP client role is effectively assigned. - Start 3-way handshake by sending of TCP-SYN triggered by an interlinkage procedure [H.248.SEPLINK]; SEP adopts TCP client role. - Signal EstBNC (clause 8.3.1) implicitly overwrites the value to "unblocked".. "unblocked" (0x0002); - MG shall be ready for incoming TCP bearer connection control procedures; - TCP role: not determined but prepares for a TCP server role effectively ("TCP Passive Open") (NOTE 4); - Incoming TCP-SYN: TCP server role is confirmed, 3-way handshake proceeded; - Any outgoing TCP bearer connection control procedures are not affected: - Signal EstBNC (clause 8.3.1): start 3-way handshake by sending of TCP-SYN; TCP client role is effectively assigned. - Start 3-way handshake by sending of TCP-SYN triggered by an interlinkage procedure [H.248.SEPLINK]; SEP adopts TCP client role. - TCP connection state: transitioning from "CLOSED" to "LISTEN" (Figure A.1); - H.248 counterpart state = "Idle" (Figure A.3);. NOTE 3 - This semantic is analogous to SDP codepoint "a=setup:holdconn", see [IETF RFC 4145]. NOTE 4 - This semantic could be correlated with SDP codepoint "a=setup:actpass", see [IETF RFC 4145].. |
|  | Default: | "Unblocked" (due to be consistent with default behaviour as in [ITU-T H.248.84]). NOTE - H.248 profile specifications could redefine other default values. Examples: H.248 profiles for - MGs with support of "TCP proxy" mode only may choose default value "blocked" due to fully separated establishment procedures towards both remote TCP endpoints; - MGs with additional support of "TCP relay" and "TCP merge" modes may choose default value "unblocked" due to in order not to miss any incoming TCP connection establishment requests ("early media"). |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

[End Correction]

[Begin Correction]

### 8.6.2	TCP endpoint role assumption

{Editor's note (2013-10 meeting): the "TCP endpoint role" would be coupled with "TCP endpoints", which are related to the two MG IWFs of a) TCP-to-TCP as "TCP proxy" and b) TCP-to-non-TCP. However, more important is whether the MG has to provide a "TCP OPEN" procedure or not. Present situation:
I)TCP to non-TCP: yes, either TCP active or passive OPEN;
II) TCP to TCP in TCP relay: no; 
III) TCP to TCP in TCP merge: yes, but only TCP passive OPEN; 
IV) TCP to TCP in TCP proxy: yes, either TCP active or passive OPEN;
The TCP role is then given by the TCP OPEN type.
The entire clause 8.6 should be revisited and possibly revised again..}

The TCP role, - client or server -, is primarily relevant during the establishment phase of communication (of perspective of H.248 gateways). The MGC does not explicitly assign the SEP a TCP role. It is an incoming or outgoing establishment which leads to the assumption of a TCP role. The SEP is basically prepared for an incoming or outgoing establishment since the moment it is created with an appropriate media descriptor. The MGC may prevent the assumption of a server role by setting the bceb property to “blocked”. The MGC may effectively determine the SEP to assume the client role by sending the EstBNC signal. The SEP may also assume the client role as a consequence of interlinkage procedures [H.248.SEPLINK]. If these procedures are allowed the SEP may assume a client role as a consequence of e.g. an incoming procedure in the associated SEP. Table 17 summarizes MGC actions and their results in TCP role adopted by the SEP:

Table 17 – TCP endpoint role assumption (at establishment phase)

|  | H.248 indication | MG behaviour: | TCP role assumed by SEP |
| --- | --- | --- | --- |
| 1 | 1) H.248 SEP state "blocked", due to explicit signalling of bceb property | Any incoming TCP packet shall be silently discarded. | Not yet determined |
| 2 | 2) H.248 SEP state "unblocked" either due to explicit signalling of bceb property or correspondent default value | Incoming TCP SYN leads to subsequent completion of 3-way handshake. | Server role |
| 3 | H.248 signal EstBNC. Value of bceb property not relevant. | Start of 3-way handshake for TCP connection establishment. | Client role |
| 4 | Interlinkage procedures allowed according to [H.248.SEPLINK]. Value of bcep property not relevant (although this or similar properties may be relevant at the incoming side of the interlinkage). | An interlinkage establishment procedure[H.248.SEPLINK], e.g. an incoming TCP establishment procedure in the associated SEP leads to the start of 3-way handshake for TCP connection establishment. | Client role |

[End Correction]

[Begin Correction]

### 8.6.3	TCP bearer connection establishment

If the SEP assumes the client role, as a result of an EstBNC signal or of an interlinkage procedure [H.248.SEPLINK], the MG shall initiate and complete the TCP connection setup handshake without any further involvement of the MGC.

If the SEP assumes the server role, as a result of an incoming TCP SYN, and being allowed to do so by the MGC by having the bceb property set to “unblocked”, the MG shall complete the TCP connection setup handshake without any further involvement of the MGC.

The end of the TCP bearer connection establishment phase may be summarized according Table 18:

Table 18 – TCP bearer establishment - Results

| TCP role | Result of TCP bearer connection establishment | MGC notification? |
| --- | --- | --- |
| TCP client (i.e., outgoing establishment side) | Successful | MGC dependent on subscription of event and parameter value tcpbcc/BNCChange{Type=[Est]} . |
|  | Unsuccessful | Unsuccessful establishment may be indicated to the MGC via a correspondent error code or event g/cause. |
| TCP server (i.e., incoming establishment side) | Successful | As per client side. |
|  | Unsuccessful | As per client side. |

[End Correction]

__________________