| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-17 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, CH, 10 - 14 March, 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.SEPLINK |  |  |  |
| Title: | H.248.SEPLINK "Gateway control protocol: Stream endpoint interlinkage package" (New): Output draft Ed. 0.4 |  |  |  |
| Purpose: | Meeting output (Draft Recommendation) |  |  |  |

Summary

This document contains the editor's output draft for draft new H.248.SEPLINK " Gateway control protocol: Stream endpoint interlinkage package" (Ed. 0.4).

__________________

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.SEPLINK Ed. 0.1 | 2013-10 (Geneva): contribution C-264 (and C-258) Location: http://www.itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-C-0264 |
| H.248.SEPLINK Ed. 0.2 | Output (TD 122/WP1) from Geneva meeting (2013-10) Location: http://www.itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-131028-TD-WP1-0122 |
| H.248.SEPLINK Ed. 0.3 | Input (AVD-4477) to WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/AVD-4477.zip |
| H.248.SEPLINK Ed. 0.4 | Output (TD-17) from WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/TD-17.zip |
| H.248.SEPLINK Ed. 0.5 |  |
| H.248.SEPLINK Ed. 0.6 |  |
| H.248.SEPLINK Ed. 0.7 |  |
| H.248.SEPLINK Ed. 0.8 |  |
| H.248.SEPLINK Ed. 0.9 |  |
| H.248.SEPLINK Ed. 0.10 |  |

CONTENTS

Page

1 Scope 6 1.1 Applicability statements 7 2 References 7 3 Definitions 7 3.1 Terms defined elsewhere 7 3.2 Terms defined in this Recommendation 8 4 Abbreviations and acronyms 8 5 Conventions 8 6 Motivation, use cases and models 8 7 Stream endpoint interlinkage package 13 7.1 Properties 13 7.1.1 Interlinkage topology 13 7.2 Events 14 7.3 Signals 14 7.4 Statistics 14 7.5 Error Codes 14 7.5.1 Incorrect intra-SEP interlinkage 15 7.6 Procedures 15 7.6.1 Configuration of "interlinkage topologies" 15 7.6.2 Relation to H.248 Stream Mode: 16 7.6.3 Relation to H.248 Topology descriptor 16 7.6.4 Error cases: 17 7.6.5 Examples: 17 Appendix I Illustration of interlinkage capabilities 19 I.1 Illustration of concept 19 I.2 Illustration of supported use cases 20 I.2.1 Use case "Inter-SEP interlinkage" 20 I.2.2 Use case "Intra-SEP interlinkage" 21 I.2.3 Use case "Multi-SEP interlinkages" 22 I.3 Controversial use cases 23 I.3.1 Use cases with "self-referential interlinkage" 23 I.3.2 Use cases with "tunneled (IP) protocol stacks" 23 I.3.3 Use cases with "extended (IP) protocol stacks" 23 Appendix II SDP for gateway control - IANA considerations 23 II.1 Purpose 24 II.2 SDP codepoints related to SDP "m=" line <proto> element 24 II.3 SDP codepoints related to SDP attributes 24 Bibliography 24

List of Tables

Page

Table I.1 – Interlinkage description – Use case "Inter-SEP interlinkage" 21 Table I.2 – Interlinkage description – Use case "Intra-SEP interlinkage" 21 Table I.3 – Interlinkage description – Use case "Multi-SEP interlinkage" 23 Table II.1 – SDP codepoints related to SDP "m=" line <proto> element 24

List of Figures

Page

Figure 1 – Two-termination context with multiple connection-oriented protocols in the SEP-specific protocol stack (segments) 9 Figure 2 – Principle interlinkage options (here: two-termination context) from perspective of a single SEP (here: SEP-A) 10 Figure 3 – Interlinkage example for a three-termination context 11 Figure 4 – Example of a "TLS/TCP to SCTP" connection model – Vertical interlinkage configuration at SEP-A 17 Figure 5 – Example of a "TLS/TCP to SCTP" connection model – Horizontal interlinkage configuration between SEP-A "TCP" and SEP-B "SCTP" 18 Figure 6 – Example of a "TLS/TCP to SCTP" connection model – Horizontal interlinkage configuration between SEP-B "SCTP" and SEP-A "TLS" 19 Figure I.1 – Stream endpoint assigned property linktopo and their parameters 20 Figure I.2 – Use case "Inter-SEP interlinkage" 20 Figure I.3 – Use case External driven "Intra-SEP interlinkage" 21 Figure I.4 – Use case Internal driven "Intra-SEP interlinkage" 22 Figure I.5 – Use case "Multi -SEP interlinkage" 22

Draft new Recommendation ITU-T H.248.xx (ex H.248.SEPLINK)

Gateway Control Protocol:
Stream endpoint interlinkage package

AAP Summary

<To be provided before Consent>

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

The protocol stack of an H.248 media gateway bearer interface may contain principally connectionless and connection-oriented protocols. The concerned protocol layer(s) from H.248 perspective, i.e., which are in scope of the H.248 Stream Descriptors, are typically a network layer (L3) and higher. Connection-oriented protocols are known from L2 onwards in general, and in case of IP from L4 and higher concerning with respect to basic reference models for layered protocol architectures.

Connection-oriented protocols are characterized by explicit bearer connection control procedures, related to the establishment, modification and/or release of a bearer connection. Normally, the MG as slave is triggered by the master MGC for a particular bearer connection control procedure, and then autonomously executing such a procedure at the bearer interface related to a particual H.248 termination (T) / stream endpoint (SEP).

The end-to-end bearer plane connectivity implies typically scenarios there multiple SEPs are interconnected in an H.248 context. SEP-individual bearer control procedures are then often interlinked; e.g. an incoming bearer connection establishment procedure (at one SEP) could lead to an outgoing bearer connection establishment procedure (at a partner SEP), - at the same or at a different protocol layer.

An MGC may wish to individually control each of these procedures (so called MGC-strictly controlled mode). But there are many use cases where the MGC could benefit from a delegation of "interlinkage work" down to the MG itself.

Such kind of behaviour is the purpose of this Recommendation.

An H.248 package, called stream endpoint interlinkage package (text codepoint 'seplink'), is defined in a rather generic manner, which represents a fairly flexible tool for a wide range of interlinkage options.

The supported interlinkage

- is limited to a single context,

- is direction dependent,

and allows the interlinkage

- of protocol endpoints, at the same or different layers, in different SEPs ("inter-SEP interlinkage"), and

- of protocol endpoints at different layers within an individual SEP ("intra-SEP interlinkage"),

and

- allows multiple interlinkage configurations could be assigned to an invidiual SEP.

## 1.1	Applicability statements

The capability of "interlinkage" relates to the MG internal forwarding of protocol control information (PCI) between different SEPs and/or different protocol layers. A particular layered protocol architecture is thus associated with a SEP.

The interlinkage capability is basically limited on protocol stack segments, which may be unambiguously described via the mechanism according clause 7.1.1.

The defined interlinkage capability is basically generic, i.e., applicable for all kind of connection-oriented bearer protocols. However, the focus of the initial package version is tightly coupled with protocols related to IP.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.TCP]	Draft Recommendation ITU-T H.248.TCP (201X), Gateway control protocol: TCP support packages.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	stream endpoint (SEP) [clause 3.2.10/ITU-T H.248.1]: relates to a stream associated to a Termination. A SEP is identified by the tuple of StreamID and TerminationID values.

3.1.2	stream endpoint pair (SEPP) [clause 3.2.11/ITU-T H.248.1]: two associated Stream Endpoints (SEP) of two Terminations within the same Context. The two SEPs share thus the same StreamID value. There is always a point-to-point topology for Stream Endpoint Pair configurations (e.g., there is no further associated SEP on a third Termination etc).

3.2.3	stream endpoint tuple (SEPT) [clause 3.2.1/b-ITU-T H.SuppTerm]: the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share thus the same StreamID value. The stream topology is given by the topology descriptor settings.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	stream endpoint tuple (SEPT): the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share thus the same StreamID value. The stream topology is given by the topology descriptor settings.

3.2.1	interlinkage: the MGC empowered interconnection of two protocol endpoints. The two protocol endpoints are associated to a single stream endpoint (SEP) or different SEPs within the same context. The MG is enabled for autonomously forwarding of indicated stimuli from the source protocol endpoint to the destination endpoint. The stimuli relates to abstracted control information (and not any real Lx protocol control information).

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| EP | End Point |
| --- | --- |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| SCTP | Stream Control Transmission Protocol |
| SDP | Session Description Protocol |
| SEP | Stream End Point |
| SEPP | Stream End Point Pair |
| SEPT | Stream End Point Tuple |
| T | Termination |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |

# 5	Conventions

None.

# 6	Motivation, use cases and models

H.248 controlled Media Gateways may terminate multiple connection-oriented transport layer protocols, like TCP, TLS, SCTP.

This is outlined in the following Figure 1 where in a given H.248 context each Stream Endpoint (SEP) is supposed to host at least one or more connection-oriented transport layer endpoints.

Figure 1 – Two-termination context with multiple connection-oriented protocols 
in the SEP-specific protocol stack (segments)

The interlinkage concept is constituted by following characteristics:

- the interlinkage scope is limited to a single context;

- an interlinkage configuration is defined via an H.248 Property, which itself is linked to a particular stream endpoint/termination;

- a specific external, incoming bearer control procedure (e.g., establishment event) is immediately triggering the execution of an external, outgoing bearer control procedure, at the same or another termination in the context;

- a single interlinkage configuration acts in an unidirectional manner; and

- multiple interlinkage configurations in parallel may be defined for a single SEP.

Thus, three principal interlinkage types may be differentiated (see also illustration in Appendix I.2):

- inter-SEP interlinkage (when source and interlinked transport endpoints are located at different terminations);

- intra-SEP interlinkage (when source and interlinked transport endpoints are located at the same termination); and

- multi-SEP interlinkage (when a single stimuli is used to trigger multiple outgoing bearer procedures).

The following Figure 2 outlines the various interlinkage options enabling an MG autonomous establishment/release of a connection/session-oriented partner transport protocol endpoint (again at the example of a stream endpoint pair (SEPP)):

Figure 2 – Principle interlinkage options (here: two-termination context) 
from perspective of a single SEP (here: SEP-A)

Example interlinkage configurations (in Figure 2):

- Intra-SEP interlinkage from lower to higher protocol layer: within one SEP a lower layer connection oriented transport protocol endpoint (EP) triggers an upper layer connection oriented transport protocol EP. 
Note that this doesn’t mean that lower layer service access point is used by the upper layer transport protocol directly, i.e. there might reside other connectionless protocol layers in between.

- Intra-SEP interlinkage from higher to lower protocol layer: within one SEP an upper layer connection oriented transport protocol EP triggers a lower layer connection oriented transport protocol EP. 
Note that this doesn’t mean that lower layer service access point is used by the upper layer transport protocol directly, i.e. there might reside other connectionless protocol layers in between.

- Inter-SEP interlinkage: Given a SEP pair SEP-A triggers the connection oriented transport protocol of the peer SEP-B. Either the same or a different transport protocol can be interlinked, thus for example SEP-A TCP endpoint could trigger a SEP-B TCP endpoint, or a SEP-A TCP endpoint could trigger e.g. a SEP-B SCTP endpoint.

The first three interlinkage configurations are part of SEP-A. When the 3rd interlinkage configuration (3) should be complemented by interlinkage in reverse direction, then following, fourth interlinkage configuration needs to be associated with SEP-B:

- Inter-SEP interlinkage: Given a SEP pair SEP-A receives a trigger regarding a connection oriented transport protocol from its peer SEP-B. Either the same or different transport protocol can be interlinked, thus for example SEP-B TCP endpoint could trigger a SEP-A TCP endpoint, or a SEP-B TCP endpoint could trigger e.g. a SEP-A SCTP endpoint.

There are following principal limitations:

- Inter-SEP interlinkage (also known as "horizontal protocol layer interlinkage") is only permitted between SEPs belong to the same H.248 stream.

- Intra-SEP interlinkage (also known as "vertical protocol layer interlinkage") is only permitted within a SEP, i.e. between its supported transport protocol layers.

- In horizontal protocol layer interlinkage only external, incoming procedures are permitted as stimuli.

- The triggered procedure is always an outgoing procedure

- Establishment procedures within one SEP shall follow an upwards order, i.e the initiation of an establishment procedure shall only start when the establishment procedure at the lower layer is completed.

- Release procedures within one SEP should generally follow a downwards order, i.e. the inititation of a release procedure should only start when the release procedure at the upper later is completed. However, depending on the protocol rules, it may be possible to omit an explicit release procedure at the upper layer altogether.

NOTE – The last two limitations are due to basic communication principles in layered protocol architectures (see [ITU-T X.200]: an (N+1)-layer is based on the service of the (N)-layer connection).

A H.248 context may contain more than two terminations; where more than two terminations host SEPs belonging to the same H.248 stream. In this case the SEP interlinkage capabilitiy needs to be able to address a dedicated stream endpoint tuple (SEPT).

For example, following interlinkage scheme should be supported in case of an H.248 context containing three SEPs (same H.248 streamID).

Figure 3 – Interlinkage example for a three-termination context

Whereas SEPP (SEP-A, SEP-B) can trigger each other regarding the identified transport protocol endpoints, the SEP-A connection oriented transport protocol EP may trigger SEP-C connection oriented transport protocol EP in addition. However SEPP (SEP-B, SEP-C) doesn’t have any interlinkage, whereas also SEP-C connection oriented transport protocol EP cannot trigger SEP-A connection oriented transport protocol EP in any way.

# 7	Stream endpoint interlinkage package

|  | Package Name: | Stream endpoint interlinkage package |
| --- | --- | --- |
|  | Package ID: | seplink (0x####) |
|  | Description: | This package provides the functionality to interlink connection/session oriented transport protocol endpoints. If two transport protocol endpoints are interlinked the establishment and/or release of a connection/session at the source transport protocol endpoint will trigger an MG-autonomous establishment and/or release of the interlinked transport protocol endpoint. |
|  | Version: | 1 |
|  | Extends: | None |

## 7.1	Properties

### 7.1.1	Interlinkage topology

|  | Property Name: | Interlinkage topology |
| --- | --- | --- |
|  | Property ID: | linktopo (0x0001) |
|  | Description: | This property defines the MG interlinkage behaviour regarding connection/session oriented transport endpoints. Interlinkage can be specified between the connection oriented transport layers of a given SEP or between any connection oriented transport layers of a given SEP pair. Interlinkage defines a undirectional information flow ("stimuli") from a "SEP-local transport EP" towards another "SEP-local or –remote transport EP". |
|  | Type: | Sub-list of Strings |
|  | Possible values: | Each string instance of the sub-list is coded according to the following ABNF syntax: linktopo = interlinkedSEP COLON sourceTransportEP COLON interlinkedTransportEP COLON mode interlinkedSEP = TerminationID ; Note 1 sourceTransportEP = proto ; Note 2 interlinkedTransportEP = proto ; Note 2 mode = modetypes *(COMMA modtypes) modetypes = “*” / “est” / “rel” ; * is all mode types ; est is Establishment ; rel is Release Note 1: TerminationID, COLON, COMMA are as per Annex B.2/[ITU-T H.248.1]. For binary usage of H.248.1 the binary TerminationID octets should be encoded using a hexadecimal octet coding (as per Annex B.3/[H.248.1]. The wildcarding field shall be omitted. The only wildcarding allowed is the use of ALL or CHOOSE which shall be indicated using “*” and “$” respectively. Note 2: proto is as per clause 9/[IETF RFC 4566]. {Editor’s note (2014-03 meeting): Further updates may be needed to be able to identify a level in the “proto” hierarchy. Other approaches to the one specified here may be needed.} Where: interlinkedSEP: Is the TerminationID of the interlinked SEP where the transport signalling is terminated. The source inter-linked SEP is the Termination where the linktopo property is set. This may be set to the same value as the source SEP TerminationID in which case it refers to Figure 2 Options 1 and 2. sourceTransportEP: Is the transport protocol endpoint which is the source of the signalling. interlinkedTransportEP: Is the transport protocol endpoint which is the destination of the signalling. In Figure 2 options 1 and 2 the sourceTransportEP and interlinkedTransportEP shall be set to different transport protocols. Mode: Specifies whether the inter-linkage is related to connection/session establishment and/or release procedures. NOTE: Detailed agreements regarding the admitted <proto> values shall be defined in the particular H.248 profile specifications, typically in the profile clauses on mandatory/optional support of SDP. |
|  | Default: | Empty list |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 7.2	Events

None.

## 7.3	Signals

None.

## 7.4	Statistics

None.

## 7.5	Error Codes

None

### 7.5.1	Incorrect intra-SEP interlinkage

{Editor’s note (2014-03 meeting): the error code was moved from H.248.TLS, former tlsesc package, but still need to generalized. The primary cause of incorrect intra-SEP interlinkage is related to the complementary connection concepts "connection-oriented" and "connection-less" (see ITU-T X.200: "connection-mode" and "connection-less mode"). The connection-less protocols lacking inherently any bearer connection control procedures, thus, a connection-less protocol layer as
a) source transport endpoint: there would be never any stimuli, hence, never an interlinkage triggered; and
b) interlinked transport endpoint: a received stimuli would just silently disappear because there are not any procedures which could be triggered. }

|  | Error Code #: | <number> |  |
| --- | --- | --- | --- |
|  | Name: | Incorrect intra-SEP interlinkage |  |
|  | Definition: | Vertical, protocol layers interlinkage (as based on a intra-SEP interlinkage configuration) is principally limited in case of "TLS-over-L4" protocol stack segments, particularily when the TLS used IP transport protocol would be connection-oriented (such as TCP). The MGC shall only use intra-SEP interlinkage configurations as defined by clauses 9.6.2.4 and 9.6.3.4. Other intra-SEP interlinkage configurations would violate protocol concepts and shall be replied by this error code. |  |
|  | Error text in the Error Descriptor: |  | Incorrect intra-SEP interlinkage configuration. |
|  | Comment: | None. |  |

## 7.6	Procedures

### 7.6.1	Configuration of "interlinkage topologies"

#### 7.6.1.1	SEP identification:

a) Source of stimuli:

The source SEP transport protocol endpoint, i.e. the connection/session oriented transport protocol endpoint, which triggers autonomous procedures at another connection/session oriented transport protocol endpoint, is specified as follows:

- The TerminationID and StreamID is derived from the termination/stream the LocalControl descriptor is related to containing the seplink/linktopo property value. The name of the source SEP is thus implicitly given.

- The transport protocol is explicitly specified in the seplink/linktopo property value (parameter "sourceTransportEP").

b) Sink of stimuli:

The destination SEP transport protocol endpoint, i.e. the connection/session oriented transport protocol endpoint, which receives a trigger to start autonomous procedures from another connection/session oriented transport protocol endpoint, is specified as follows:

- The TerminationID is explicitly specified in the seplink/linktopo property value (parameter "interlinkedSEP").

- The StreamID is derived from the termination/stream the LocalControl descriptor is related to containing the seplink/linktopo property value.

- The transport protocol is explicitly specified in the seplink/linktopo property value (parameter "interlinkedTransportEP").

#### 7.6.1.2	Traffic directions:

The seplink/linktopo property defines a unidirectional behavior, that is it specifies the interlinkage behavior of a source (identified by the TerminationID and StreamID the LocalControl descriptor is related to) and an interlinked SEP (identified by the TerminationID; same StreamID as source).

In case a bidirectional interlinkage between a SEPP/SEPT is expected, each SEP’s Local Descriptor must contain a corresponding interlinkage topology.

#### 7.6.1.3	Wildcard usage:

The interlinked SEP can be specified using the ALL wildcard

"seplink/linktopo = ["*:TCP:TCP:est,rel"]".

In this case the specified interlinkage behavior is valid to all matching SEPPs (i.e. only horizontal interlinkage, vertical interlinkage is specifically excluded, that is ALL wildcard excludes own TerminationID). In case a new SEP is added into the H.248 context later on, the ALL wildcarded interlinkage topology is applied as well to the new SEP.

The interlinked SEP can be specified using the CHOOSE wildcard

"seplink/linktopo = ["$:TCP:TCP:est,rel"]".

The CHOOSE wildcard matches the TerminationID that the MG assigns in the first Add Command that uses a CHOOSE wildcard in the same action. This allows to add a new termination/SEP, and to modify the existing interlinkage topologies (using a modify command) within the same action.

Transaction = 1 {

Context = 1 {

Add = ip/1/$/$ {

Media {

Stream = 1 {

LocalControl { seplink/linktopo = ["ip/1/1/1:TCP:TCP,est,rel"] }}}}

Modify = ip/1/1/1 {

Media {

Stream = 1 {

LocalControl { seplink/linktopo = ["$:TCP:TCP:est,rel"] }}}}

### 7.6.2	Relation to H.248 Stream Mode:

A specified interlinkage topology is independent of the StreamMode property of the LocalControl Descriptor. The StreamMode property affects the application data flow rather than the transport protocol control information used to establish or close the corresponding transport connection.

### 7.6.3	Relation to H.248 Topology descriptor

The effectiveness or non-effectiveness of a specified horizontal interlinkage topology depends on the context level topology descriptor, i.e. whether the corresponding flow direction in the topology descriptor for a particular SEPP (or in case of wildcard ALL usage: for all matching SEPPs) has been enabled or not.

### 7.6.4	Error cases:

In case a new interlinkage topology is specified refering to an unknown termination identifier, then the MG shall reject this using Error #430 "Unknown TerminationID".

In case a new interlinkage topology is specified refering to a valid termination identifier but this termination doesn’t contain an SEP for this specific stream identifier, then the MG shall reject this using Error #581 "Does Not Exist".

In case a new interlinkage topology is specified refering to a transport protocol not being part of the SEP’s m-line transport protocol, then the MG shall reject this using Error #581 "Does Not Exist".

### 7.6.5	Examples:

#### 7.6.5.1	Example of a "TLS/TCP to SCTP" connection model

i) Example of intra-SEP interlinkage:

Given an SEP-A (T-A) uses transport layers TLS/TCP. A TCP connection setup shall autonomously trigger a setup of the TLS session and a TLS session release shall autonomously trigger a release of the TCP connection.

H.248 command request:

Modify = T-A {
  Media {
    Stream = 2 {
      LocalControl {
        seplink/linktopo = ["T-A:TCP:TLS:est","T-A:TLS:TCP:rel"]
} } } }

The resulting interlinkage looks as follows (Fig. 4):

Figure 4 – Example of a "TLS/TCP to SCTP" connection model 
– Vertical interlinkage configuration at SEP-A

ii) Example of inter-SEP interlinkage:

In addition SEP-A’s transport layer TCP can be interlinked with SEP-B’s transport layer SCTP (i.e. SEP-A’s TCP connection establishment/release shall autonomously trigger SEP-B’s SCTP connection establishment/release).

Note: SEP-A’s TCP connection establishment/release can be either stimulated by using the corresponding H.248 TCP package capabilities ( e.g., [ITU-T H.248.TCP] properties tcpbcc/EstBNC (outgoing establishment procedure (TCP client role)) or tcpbcc/RelBNC) or by receiving TCP connection establishment/release request from the far-end TCP endpoint.

H.248 command request:

Modify = T-A {
  Media {
    Stream = 2 {
      LocalControl {
        seplink/linktopo = ["T-A:TCP:TLS:est", "T-A:TLS:TCP:rel",
                            "T-B:TCP:SCTP:est,rel"]
} } } }

Figure 5 – Example of a "TLS/TCP to SCTP" connection model – Horizontal 
interlinkage configuration between SEP-A "TCP" and SEP-B "SCTP"

iii) Example of bidirectional inter-SEP interlinkage:

To make sure that SCTP connection establishment at SEP-B will also trigger SEP-A TCP connection establishment and that SCTP connection release at SEP-B will also trigger SEP-A TLS session release, the seplink/linktopo property has to be applied as follows in SEP-B’s LocalControl Descriptor:

H.248 command request:

Modify = T-B {
  Media {
    Stream = 2 {
      LocalControl {
        seplink/linktopo = ["T-A:SCTP:TCP:est", "T-A:SCTP:TLS:rel"]
} } } }

Figure 6 – Example of a "TLS/TCP to SCTP" connection model – Horizontal 
interlinkage configuration between SEP-B "SCTP" and SEP-A "TLS"

Appendix I

Illustration of interlinkage capabilities

(This appendix does not form an integral part of this Recommendation)

This Appendix illustrates the interlinkage concept, the principal interlinkage types and indicates some use cases which are beyond current interlinkage capabilities.

## I.1	Illustration of concept

Figure I.1 summarizes the used terminology and the principal elements of the seplink/linktopo property.

Figure I.1 – Stream endpoint assigned property linktopo and their parameters

The linktopo property is always associated with a specific stream endpoint (here SEP "Ta/Si").

## I.2	Illustration of supported use cases

## Three principal interlinkage types may be differentiated.

### I.2.1	Use case "Inter-SEP interlinkage"

The interlinkage across two terminations is called "inter-SEP interlinkage" (Figure I.2).

Figure I.2 – Use case "Inter-SEP interlinkage"

Table I.1 summarizes the principal interlinkage description:

Table I.1 – Interlinkage description – Use case "Inter-SEP interlinkage"

| <interlinked SEP> | <source transport EP> | <interlinked tranport EP> | <mode> | Comment |
| --- | --- | --- | --- | --- |
| "Tb" | "EPn+1" (at Ta/Si) | "EPn" (at Tb/Si) | "…" |  |

### I.2.2	Use case "Intra-SEP interlinkage"

There are two variants of intra-SEP interlinkage:

### I.2.2.1	External driven "Intra-SEP interlinkage"

The interlinked transport endpoint may be located at the same SEP as the source transport endpoint, called "intra-SEP interlinkage" (Figure I.3).

Figure I.3 – Use case External driven "Intra-SEP interlinkage"

Table I.2 summarizes the principal interlinkage description:

Table I.2 – Interlinkage description – Use case "Intra-SEP interlinkage"

| <interlinked SEP> | <source transport EP> | <interlinked tranport EP> | <mode> | Comment |
| --- | --- | --- | --- | --- |
| "Ta" | "EPn" (at Ta/Si) | "EPnn+1" (at Ta/Si) | "…" |  |

#### I.2.2.2	Internal driven "Intra-SEP interlinkage"

There is following difference between the previous external driven intra-SEP interlinkage case and the interna-driven one:

An MG-internal stimuli (see Figure I.4), - either orignating in the MGC (0a) (i.e., related to H.248 signalling) or in the MG itself due to a previous interlinkage scenario (0b) -, triggers an external outgoing procedure (1). The internal stimuli (2) will be generated, as usual, when the source transport endpoint is successfully transitioning its state.

Figure I.4 – Use case Internal driven "Intra-SEP interlinkage"

The interlinkage description is identical to the external driven "Intra-SEP interlinkage" (i.e., according to Table I.2). The history before is just a different scenario.

### I.2.3	Use case "Multi-SEP interlinkages"

A single stimuli may be used to trigger multiple outgoing bearer procedures (of the same or different type). The interlinked transport endpoints may be located at the same or different SEP(s). Figure I.5 illustrates such an example.

Figure I.5 – Use case "Multi -SEP interlinkage"

The linktopo property "value" represents a list structure (Table I.3):

Table I.3 – Interlinkage description – Use case "Multi-SEP interlinkage"

| <interlinked SEP> | <source transport EP> | <interlinked tranport EP> | <mode> | Comment |
| --- | --- | --- | --- | --- |
| "Tb" | "EPn+1" (at Ta/Si) | "EPn" (at Tb/Si) | "…" |  |
| "Tc" | "EPn+1" (at Ta/Si) | "EPn" (at Tc/Si) | "…" |  |

## I.3	Controversial use cases

### I.3.1	Use cases with "self-referential interlinkage"

The case of intra-SEP interlinkage with identical source transport and interlinked transport endpoints is called self-referential interlinkage.

Such an interlinkage seems to be controversial in general concerning the question which incoming bearer control procedures trigger immediate outgoing control procedures? What would be correspondent mode values?

There were not any useful self-referential interlinkage scenarios identified for seplink package version 1.

### I.3.2	Use cases with "tunneled (IP) protocol stacks"

The usage of tunneling is e.g. a characteristic of many IP access network technologies.The notion of tunnel refers to hierarchical protocol stacks of type e.g. "X-over-IP-over-X-over-IP",

Such an interlinkage seems to be controversial in general concerning the unambiguous identification of source transport and/or interlinked transport endpoints due to the existing supported value range by seplink package version 1. E.g., the indication of "TCP" (in case of a TCP-over-TCP stack) would be ambiguous. Such scenarios are for further studies.

### I.3.3	Use cases with "extended (IP) protocol stacks"

Extended protocol stacks are also beyond the layered protocol architectures according to basic reference models, but the interlinkage is not necessarily between identical protocol types (as in above tunneled scenarios).

The support of such use cases is already in scope of seplink package version 1, but conditional. The usage depends on the unambiguous identification of source transport and/or interlinked transport endpoints.

Appendix II

SDP for gateway control - IANA considerations

(This appendix does not form an integral part of this Recommendation)

{Editor's note (2014-03): this clause is a placeholder and dependent on the final definition of the linktopo property in clause 7.1.1. The clause will be removed again when finally not needed.}

## II.1	Purpose

The H.248 gateway control protocol reuses normally the SDP codepoints as defined by the IETF or other organizations. However, gateway control may demand for additional SDP codepoints. The ones idendified by this Recommendation are collected in this clause and are subject of registration with IANA.

## II.2	SDP codepoints related to SDP "m=" line <proto> element

This Recommendation identifies following bearer type indication requirements (Table II.1):

Table II.1 – SDP codepoints related to SDP "m=" line <proto> element

| Type | SDP name (value) | Use case | IANA status |
| --- | --- | --- | --- |
| proto | FFS | Indication of "source transport endpoint" and "interlinked transport endpoint" in interlinkage specifications (based on H.248 property seplink/linkopo) | To be checked |

Thus, not any additional SDP codepoints needs to be registered with IANA.

## II.3	SDP codepoints related to SDP attributes

None (because SDP attributes are not used by this H.248 package).

Bibliography

[b-ITU-T H.SuppTerm]	Supplement ITU-T H.Sup.Term (20xx), Gateway Control Protocol: ITU-T H.248.x-series - Supplement on common H.248 terminology

____________________