| Rapporteur Meeting of Questions 1,2, 3, 5 and 21/16 |  |  |  | TD-21 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.66 |  |  |  |
| Title: | H.248.66 (ex H.248.RTSP) "Packages for RTSP and H.248 interworking" (New): output draft |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the Editor's output draft for H.248.66 (ex H.248.RTSP) "Packages for RTSP and H.248 interworking" (New).

CONTENTS

Page

1 Scope 7 2 Reference 7 3 Definitions 8 3.1 Terms defined elsewhere 8 3.2 Terms defined in this Recommendation 8 4 Abbreviations and acronyms 9 5 Conventions 9 6 RTSP and H.248 interworking 10 6.1 Method interworking 10 6.1.1 OPTIONS 10 6.1.2 DESCRIBE 10 6.1.3 SETUP 10 6.1.4 PLAY 10 6.1.5 PLAY_NOTIFY 11 6.1.6 PAUSE 11 6.1.7 TEARDOWN 12 6.1.8 GET_PARAMETER 12 6.1.9 SET_PARAMETER 12 6.1.10 REDIRECT 12 6.1.11 Extension methods 12 6.2 Embedded (interleaved) binary data 12 6.3 Status / error code interworking 12 6.3.1 Success 1xx 13 6.3.2 Success 2xx 13 6.3.3 Redirection 3xx 13 6.3.4 Client error 4xx 13 6.3.5 Server error 5xx 13 6.4 Header field definitions 13 6.4.1 Non-mapped headers 14 6.4.2 Mapped headers 14 7 Media resource identification package 17 7.1 Properties 17 7.1.1 Specification 17 7.2 Events 17 7.3 Signals 17 7.4 Statistics 17 7.5 Error codes 17 7.6 Procedures 18 8 Range format support package 18 8.1 Properties 18 8.1.1 Format types 18 8.2 Events 19 8.3 Signals 19 8.4 Statistics 19 8.5 Error codes 19 8.6 Procedures 19 9 Media resource description expiry package 20 9.1 Properties 20 9.1.1 Date and time 20 9.2 Events 21 9.3 Signals 21 9.4 Statistics 21 9.5 Error codes 21 9.6 Procedures 21 10 Media block size package 21 10.1 Properties 22 10.1.1 Size 22 10.2 Events 22 10.3 Signals 22 10.4 Statistics 22 10.5 Error codes 22 10.6 Procedures 22 11 RTSP media resource syntax package 23 11.1 Properties 23 11.2 Events 23 11.3 Signals 23 11.4 Statistics 23 11.5 Error codes 23 11.6 Procedures 23 11.6.1 General structure 23 11.6.2 Provisioned segment specifications 24 12 RTSP play package 25 12.1 Properties 25 12.2 Events 25 12.2.1 End-of-stream notification event 25 12.2.2 Media properties and range change event 27 12.2.3 Scale Change Event 29 12.3 Signals 32 12.3.1 Play 32 12.4 Statistics 35 12.5 Error codes 35 12.6 Procedures 35 13 Signal pause package 36 13.1 Properties 37 13.2 Events 37 13.3 Signals 37 13.3.1 Pause 37 13.4 Statistics 39 13.5 Error codes 39 13.5.1 Unable to pause the playout of the signal 39 13.6 Procedures 39 13.6.1 Pausing signal playout 39 13.6.2 Requesting pause information 40 13.6.3 Restarting after a pause 40 14 Data delivery speed adjustment package 40 14.1 Properties 41 14.2 Events 41 14.3 Signals 41 14.3.1 Speed 41 14.4 Statistics 43 14.5 Error codes 43 14.5.1 Unable to adjust the data delivery speed of the signal 43 14.6 Procedures 43 14.6.1 Adjusting data delivery speed 43 14.6.2 Requesting range information 44 14.6.3 Normal data delivery 45 15 Playback relative scale adjustment package 45 15.1 Properties 45 15.2 Events 45 15.3 Signals 45 15.3.1 Scale 45 15.4 Statistics 48 15.5 Error codes 48 15.5.1 Unable to adjust the playback relative scale of the signal 48 15.6 Procedures 49 15.6.1 Adjust playback relative scale 49 15.6.2 Requesting range information 50 15.6.3 Normal data delivery 50 16 RTP information package 50 16.1 Properties 50 16.1.1 Synchronization source 50 16.1.2 Sequence number 51 16.1.3 RTP timestamp 51 16.2 Events 51 16.3 Signals 51 16.4 Statistics 51 16.5 Error codes 51 16.6 Procedures 52 17 RTP interleaving package 52 17.1 Properties 52 17.2 Events 52 17.2.1 Outgoing data 52 17.3 Signals 53 17.3.1 Incoming data 53 17.4 Statistics 54 17.5 Error codes 54 17.6 Procedures 54 Bibliography 55

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 1/H.248.66 – RTSP and H.248 interworking architecture 7

Draft new ITU-T Recommendation H.248.66 (ex H.248.RTSP)

Gateway control protocol: Packages for RTSP and H.248 interworking

AAP Summary

The Real Time Streaming protocol (RTSP) is used to control the playout of media resources. A MGC may be required to support the RTSP protocol and request an H.248 controlled MG to playout media resources and provide other associated functionality. H.248.66 provides guidance on mapping Real Time Streaming Protocol (RTSP) information elements and procedures to H.248.

Summary

The Real Time Streaming protocol (RTSP) is used to control the playout of media resources. A MGC may be required to support the RTSP protocol and request an H.248 controlled MG to playout media resources and provide other associated functionality. H.248.66 provides guidance on mapping Real Time Streaming Protocol (RTSP) Methods, Error/Status Codes and Header Fields to H.248 protocol information elements, packages and procedures. Several packages are defined to enhance interoperability between RTSP and H.248, however the functionalities defined in these packages may be used for other applications/services.1

# 1 Scope

This Recommendation provides guidance on mapping Real Time Streaming Protocol (RTSP) Methods, Error/Status Codes and Header Fields to H.248 protocol information elements, packages and procedures. Several packages are defined to enhance interoperability between RTSP and H.248, however the functionalities defined in these packages may be used for other applications/services.

RTSP operates between User Equipment (UE) and a Media Server. This Recommendation assumes that the Media Server is functionally decomposed into a Media Control Function (MCF) and a Media Delivery Function (MDF). These functions can be realized in a Media Gateway Controller (MGC) and a Media Gateway (MG). The Figure 1 illustrates the theoretical architecture in scope.

NOTE 1 - There may be other network entities between the UE and MGC (MCF), or the UE and the MG (MDF).

MGC (MCF)

MG (MDF)

H.248

RTSP

RTP

UE

Figure 1/H.248.66 – RTSP and H.248 interworking architecture

NOTE 2 - Alternate architectures are for further study.

# 2	Reference

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway Control Protocol: Version 3

[ITU-T H.248.8]	Recommendation ITU-T H.248.8 (03/2013), Gateway control protocol: Error code and service change reason description

[ITU-T H.248.9]	Recommendation ITU-T H.248.9 (12/2009), Gateway control protocol: Advanced media server packages

[ITU-T H.248.10]	Recommendation ITU-T H.248.10 (07/2001), Gateway control protocol: Media gateway resource congestion handling package

[ITU-T H.248.11]	Recommendation ITU-T H.248.11 (03/2013), Gateway control protocol: Media gateway overload control package

[ITU-T H.248.77]	Recommendation ITU-T H.248.77 (09/2010), Gateway control protocol: SRTP package and procedures

[IETF RFC 2326]	IETF RFC 2326 (04/1998), Real Time Streaming Protocol (RTSP)

[IETF RFC 3550]	IETF RFC 3550 (07/2003), RTP: A Transport Protocol for Real-Time Applications

[IETF RFC 3986]	IETF RFC 3986 (01/2005), Uniform Resource Identifier (URI): Generic Syntax

[IETF RFC 3987]	IETF RFC 3987 (01/2005), Internationalized Resource Identifiers (IRIs)

[IETF RFC 4566]	IETF RFC 4566 (07/2006), SDP: Session Description Protocol

[IETF RFC 5234]	IETF RFC 5234 (01/2008), Augmented BNF for Syntax Specifications: ABNF

[IETF RFC 5322]	IETF RFC 5322 (10/2008), Internet Message Format.

[IETF RFC RTSP2]	draft-ietf-mmusic-rfc2326bis-40.txt, Real Time Streaming Protocol 2.0 (RTSP)

# 3	Definitions

## 3.1	Terms defined elsewhere

None.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Add.req: H.248.1 Add command request

3.2.2	AuditCapabilities.req: H.248.1 AuditCapabilities command request

3.2.3	AuditValue.req: H.248.1 AuditValue command request

3.2.4	Modify.req: H.248.1 Modify command request

3.2.5	Move.req: H.248.1 Move command request

3.2.6	Notify.req: H.248.1 Notify command request

3.2.7	Playback: Is the process of delivering media, otherwise known as "Media Delivery" in RTSP.

3.2.8	ServiceChange.req:	H.248.1 ServiceChange command request.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| ABNF | Augmented Backus–Naur form |
| --- | --- |
| HTTP | Hyper Text Transfer Protocol |
| IP | Internet Protocol |
| IRI | Internationalized Resource Identifier |
| MCF | Media Control Function |
| MDF | Media Delivery Function |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| NPT | Normal Play Time |
| PDU | Protocol Data Unit |
| RTP | Real-Time Transport Protocol |
| RTCP | RTP Control Protocol |
| RTSP | Real Time Streaming Protocol |
| SDP | Session Description Protocol |
| SMPTE | Society of Motion Picture and Television Engineers |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |
| TTL | Time To Live |
| UDP | User Datagram Protocol |
| UE | User Equipment |
| UTC | Coordinated Universal Time |
| URI | Universal Resource Identifier |

# 5	Conventions

None

# 6	RTSP and H.248 interworking

## 6.1	Method interworking

### 6.1.1	OPTIONS

The MGC should be able to provide a response to an RTSP OPTIONS method based on the RTSP service capabilities. As the RTSP OPTIONS method may be used as a keep alive the MGC may issue an appropriate H.248 command (e.g. an empty AuditValue.req) to the MG to maintain a keep alive with the MG for a particular termination.

### 6.1.2	DESCRIBE

On receipt of a RTSP DESCRIBE method the MGC may request the presentation/session description from the MG if it does not already have this information. The MGC should add a Termination to a Context using an Add.req and set the Media Resource Specification (mri/spec) property to the URI received in the DESCRIBE. The MGC should also CHOOSE wildcard the LocalControl Descriptor to determine the presentation/session description information for the media resource of interest.

### 6.1.3	SETUP

Using the information in the RTSP SETUP method the MGC can use an Add.req or Modify.req to set the remote address/port in the Remote Descriptor and request the server port from the Local Descriptor. The MGC shall ensure that the "Specification" property of the Media Resource Identifier package (mri/spec) is set for the particular stream/termination to ensure that the correct media resource is specified.

Session information can either be provided by the MGC or from the SDP session information from the MG. If a Session identifier is provided by the client then the MGC can use the H.248 stream concept to bundle the session.

In the case that a SETUP contains multiple transport options these may be provided to the MG using multiple groups and allowing the MG to choose the applicable group. The MGC may also request the acceptable range formats by performing a CHOOSE wildcard on the Format Types (rfs/ft) property.

A RTSP SETUP method may result in the transport parameters being changed whilst a media resource is being played. In this case the MGC may need to reissue a new RTSP Play (rtspp/play) signal whilst CHOOSE wildcarding the Play Range (pr) parameter to determine the playout time at which the change in transport parameters occurred. In addition the MGC may need to perform a CHOOSE wildcard on the RTP Information including Synchronization Source (rtpinfo/ssrc), Sequence Number (rtpinfo/seq) and RTP Timestamp (rtpinfo/ts) properties.

On RTSP SETUP failure or session timeout the MGC is responsible subtracting the applicable H.248 terminations.

See clause 13.3/[IETF RFC RTSP2] for more information on SETUP procedures.

### 6.1.4	PLAY

The RTSP PLAY method relates directly to sending a H.248 signal. The playout of a particular media resource may be achieved by issuing a RTSP Play (rtspp/play) signal in a Modify.req. The MG shall play the media resource indicated by the Media Resource Identifier (mri) parameter carried in the signal. If provided this overrides the mri/spec property.

The RTSP PLAY method may include time range information in the Range Header, if included this should be sent to the MG in the Play Range (pr) parameter.

The MGC may need to determine the time range formats that are supported by the MG for the particular media resource for use in a RTSP PLAY response, in this case the MGC shall CHOOSE wildcard the Range Format (rf) parameter.

The MGC may need to determine the time range information regarding the playout of the media resource used in a RTSP PLAY response, in this case the MGC shall CHOOSE wildcard the Play Range (pr) parameter.

If the RTSP PLAY method contains a Scale header the appropriate value should be included in the Play Scale (ps) parameter. If the RTSP PLAY method contains a Speed header the appropriate value/s should be included in the Data Speed (ds) parameter. If the RTSP PLAY method contains a Seek-Style header the appropriate value should be included in the Seek Behaviour (sb) parameter.

In addition the MGC may need to perform a CHOOSE wildcard on the RTP Information properties including Synchronization Source (rtpinfo/ssrc), Sequence Number (rtpinfo/seq) and RTP Time (rtpinfo/ts).

The MGC may also set a NotifyCompletion event in order to ascertain when the playout of the signal has ceased.

If a RTSP PLAY method is received for a media resource which is currently playing out, several alternatives exist. If the time range information in the RTSP PLAY method Range header has changed compared to the playing out signal then the MGC should reissue an rtspp/play signal with the update time information (and any other relevant parameters). This will have the effect of restarting the signal at a different time location. If the RTSP PLAY method contains changed "Speed" or "Scale" information then the MGC should issue the Speed (ddsa/spd) and/or Scale (prsa/scl) signal respectively.

In order to be backwards compatible to RTSP version 1 [IETF RFC 2326] a MGC can provide PLAY METHOD queuing as H.248 provides a signal completion mechanism that can be used to indicate when a play request is finished. Signals lists can also be used to ensure sequenced playout.

See clause 13.4/[IETF RFC RTSP2] for more information on PLAY procedures.

### 6.1.5	PLAY_NOTIFY

The use of RTSP PLAY_NOTIFY is triggered by the notification of the different media play state events, which are defined in the RTSP Play Package. The MGC can customize the events when sending a RTSP Play signal. Three types of PLAY_NOTIFY requests correspond to the notification of the three events, i.e. End-of-Stream Notification Event (rtspp/eos), Media Properties and Range Change Event (rtspp/mprc), Scale Change Event (rtspp/sc).

See clause 13.5/[IETF RFC RTSP2] for more information on PLAY_NOTIFY procedures.

### 6.1.6	PAUSE

The RTSP PAUSE method is achieved through the use of the "Signal Pause (sp/pause)" signal. On receipt of a RTSP PAUSE method the MGC should issue the sp/pause signal with the applicable parameters. In order to determine the pause point information the MGC should perform CHOOSE wildcard on the "Pause Range (pr)" parameter. That MGC may specify the Time format to be used by setting the "Range Format (rf)" parameter.

On reception of the sp/pause signal the MG shall pause the relevant signals. To resume playout of the signal from the pause point the MGC shall remove the sp/pause signal from the Signals Descriptor. If the MGC wishes to resume playout from another time point then the procedures for the RTSP PLAY method should be followed.

See clause 13.6/[IETF RFC RTSP2] for more information on PAUSE procedures.

### 6.1.7	TEARDOWN

The RTSP TEARDOWN method can be directly mapped to:

- A H.248 Subtract.req command where an entire session is removed.

- Removal of a H.248 stream via a H.248 Modify command where only a single media is to be removed.

- Stopping a "RTSP Play signal (rtspp/play)" through removal from a Signals Descriptor will also result in resources being freed.

See clause 13.7/[IETF RFC RTSP2] for more information on TEARDOWN procedures.

### 6.1.8	GET_PARAMETER

The RTSP GET_PARAMETER method is mapped to the H.248 AuditValue.req message in order to determine property values.

See clause 13.8/[IETF RFC RTSP2] for more information on GET_PARAMETER procedures.

### 6.1.9	SET_PARAMETER

The RTSP SET_PARAMETER method is mapped to a Modify.req command to that modifies a property/parameter in an applicable H.248 descriptor.

See clause 13.9/[IETF RFC RTSP2] for more information on SET_PARAMETER procedures.

### 6.1.10	REDIRECT

A RTSP REDIRECT method may be initiated by the MGC based on service information from a MG. This information may be as a result of overload indications such as those defined in [ITU-T H.248.10] and [ITU-T H.248.11] or as a result of H.248 ServiceChange procedures.

### 6.1.11	Extension methods

Extension methods defined outside the core [IETF RFC RTSP2] specification may be supported by existing H.248 procedures / code points. For example the "Announce" method [b-IETF RTSPANN] may use existing H.248 NotifyCompletion mechanisms to indicate the end of a stream of media.

## 6.2	Embedded (interleaved) binary data

Routing media data through an MGC is not recommended for performance reasons however the use of H.248 signals and events allow the tunnelling of data. If a MGC receives an indication that Interleaving should be used then it should set the "RTSP Interleaving Package Outgoing Data (rtpint/od)" event on the MG with the channel number that should be used to encode the data block. The MGC shall then be ready to receive media data from the MG to forward to the RTSP client. If the MGC receives media data (e.g. RTCP reports) it shall forward this information to the MG via the "RTSP Interleaving Incoming Data (rtpint/id)" signal.

Note: The interleaving mechanism supported by [IETF RFC RTSP2] allows a maximum PDU size of 65535 bytes. If interleaving is used across H.248 consideration should be given to the maximum PDU size of the underlying H.248 transport. PDU fragmentation may be needed. This is out of scope of this Recommendation.

## 6.3	Status / error code interworking

H.248 has a full range of status codes which map to HTTP and RTSP status codes. These codes are defined in [ITU-T H.248.8]. Mappings to various status codes are discussed below:

### 6.3.1	Success 1xx

A success status code 1xx may be generated as a result of a H.248 response message that does not contain an error.

### 6.3.2	Success 2xx

A success status code 2xx may be generated as a result of a H.248 response message that does not contain an error.

### 6.3.3	Redirection 3xx

H.248 has ServiceChange messages and overload mechanisms that enables the MGC to determine MG (resource) availability in order to generate 3xx responses.

### 6.3.4	Client error 4xx

The following error codes should be handled at the MGC level: 400, 401, 402, 403, 404, 405, 406, 407, 408, 411, 413, 415, 451, 452, 454, 455, 456, 459, 460, 462, 463, 464, 465, 466, 470, 471and 472.

The following error codes may be mapped to H.248 error codes in the following manner:

- 404 Not Found: May be issued as a result of H.248 error codes related to the playout of an announcement i.e. 606, 611 etc.

- 410 Gone: May be issued as a result of H.248 error codes related to the playout of an announcement. There are several H.248 error codes in the 600-611 series that relate errors with respect to segments related to URIs. However there is no specific error code that is equivalent to the definition in 17.4.10/[IETF RFC RTSP2]. If the MGC is able to determine through another means that the resource is gone it may generate 410 otherwise 404 should be used as per 17.4.10/[IETF RFC RTSP2].

- 453 Not Enough Bandwidth: May be issued as a result of H.248 error codes i.e. 526.

- 457 Invalid Range: May be handled by the MGC if it knows the values supported by the MG otherwise may be mapped from H.248 error codes i.e. 449.

- 458 Parameter Is Read-Only: May be handled by the MGC if it knows the values supported by the MG otherwise may be mapped from H.248 error codes i.e. 534.

- 461 Unsupported Transport: May be handled by the MGC if it knows the values supported by the MG otherwise may be mapped from H.248 error codes i.e. 445, 461.

### 6.3.5	Server error 5xx

The MGC is typically able to issue 5xx status codes independent of the MG. However a 5xx error may be generated due to a condition on an MG. For example: if a MG has indicated that it is overloaded through the use of [ITU-T H.248.10] or [ITU-T H.248.11] the MGC may respond with "503 Service Unavailable" as the underlying resources related to the RTSP request may not be available even though the MGC itself is not overloaded. Likewise a MGC may respond with "551 Option not supported" or "501 Not Implemented" if a MG does not support the functionality associated with the request even though the MGC may support a RTSP protocol function or option.

## 6.4	Header field definitions

The mapping of the header fields to H.248 information elements is dependent on the RTSP method they are associated with. Some cases RTSP headers are not mapped to H.248 information elements as they are used solely by the MGC.

### 6.4.1	Non-mapped headers

The following headers can be considered to be used by the MGC only and do not have a direct mapping to H.248 information elements:

| Accept-Credentials | Content-Type | Proxy-Supported |
| --- | --- | --- |
| Accept-Encoding | CSeq | Public |
| Accept-Language | Date | Referrer |
| Allow | From | Request-Status |
| Authentication-Info | If-Match | Require |
| Authorization | If-Modified-Since | Server |
| Cache-Control | If-None-Match | Session |
| Connection | Last-Modified | Supported |
| Connection-Credentials | Notify-Reason | Timestamp |
| Content-Base | Pipelined-Requests | User-Agent |
| Content-Encoding | Proxy-Authenticate | Via |
| Content-Language | Proxy-Authentication-Info | WWW-Authenticate |
| Content-Length | Proxy-Authorization |  |
| Content-Location | Proxy-Require |  |

### 6.4.2	Mapped headers

The following RTSP Headers may be mapped to H.248:

#### 6.4.2.1	Accept

The MGC should know the content types that are acceptable for the RTSP method response. However if the MGC is unaware of the content types it may alternatively perform a H.248 AuditCapabilities.req on the MG to determine the allowable application types.

#### 6.4.2.2	Accept-ranges

The Accept-Ranges information may be determined by performing a CHOOSE wildcard on the "Range Format (rf)" parameter in a "RTSP Play (rtspp/play)", "Signal Pause (sp/pause)", "Data Delivery Speed Adjustment Speed (ddsa/spd)" and/or "Playback Relative Scale Adjustment Scale (prsa/scl)" signals.

#### 6.4.2.3	Bandwidth

The Bandwidth header may be mapped to "Bandwidth parameter (b=)" in SDP carried by H.248.

#### 6.4.2.4	Blocksize

The block size header may be mapped to the "Block Size (mbs/size)" property.

#### 6.4.2.5	Expires

If the MGC requires media resource expiry information it should set the URI of the applicable resource on the MG via the Media Resource Identifier (mri/spec) property. The MGC may either CHOOSE wildcard via Add.req or Modify.req or perform an AuditValue.req on the Media Resource Description Expiry Date and Time (mrde/dt) property.

#### 6.4.2.6	Location

It is expected that the MGC will use this for any re-directs. As discussed in 6.1.10 this may be based on information provided by the MG.

#### 6.4.2.7	Media-properties

The MGC may use this information to control the play state when issuing a RTSP play signal to the MG. The Media properties change that the MG requires can be acquired by the notification of the Media Properties (mp) parameter in RTSP Play package.

#### 6.4.2.8	Media-range

It is expected that the MGC will use this for the indication of the available media ranges. See clause 12.2 for the detection of the change that the MG requires.

#### 6.4.2.9	Mtag

If the MGC requires the information for populating the mtag header, then it should use "SDP a=mtag:?" in a H.248 Add.req or Modify.req to determine the value from the MG.

Range is supported in H.248 depending on the METHOD and functionality required. See text in clauses 12.3.1.1.4, 13.3.1.1.4, 14.3.1.1.4, and 15.3.1.1.4 on how range maps to the "Play Range (plyrng)", "Pause Range (pr)" and "Range (rng)" parameters.

#### 6.4.2.10	Range

Range is supported in H.248 depending on the METHOD and functionality required. See text in clauses 12.3.1.1.4, 13.3.1.1.4, 14.3.1.1.4, and 15.3.1.1.4 on how range maps to the “Play Range (plyrng)”, “Pause Range (pr)” and “Range (rng)” parameters.

#### 6.4.2.11	Retry-after

The MGC may provide this information based on provisioned values and MG outage information from H.248 ServiceChange.req commands or Notify.req commands with MG overload indications (e.g. through the use of [ITU-T H.248.10] or [ITU-T H.248.11]).

NOTE - The inclusion of a specific ServiceChange Outage Time is for further study and may be included in subsequent [ITU-T H.248.1] versions.

#### 6.4.2.12	RTP-info

The ssrc, seq and rtptime values may be determined by performing a CHOOSE wildcard via an Add.req or Modify.req on the RTP Information Synchronisation Source (rtpinfo/ssrc), Sequence Number (rtpinfo/seq ) and RTP Timestamp (rtpinfo/ts) properties respectively.

#### 6.4.2.13	Scale

The scale header maps to the on the Playback Scale (ps) parameter in a "RTSP Play (rtspp/play)" and or a Playback Scale (ps) parameter in the "Playback Relative Scale Adjustment Scale (prsa/scl)" signal. See clause 12.6 and 15.6 for procedural information.

#### 6.4.2.14	Seek-style

The Seek-Style header maps to the Seek Behaviour (sb) parameter in a "RTSP Play (rtspp/play)" signal. See clause 12.6 for procedural information.

#### 6.4.2.15	Speed

The speed header maps to the Data Speed (ds) parameter in a "RTSP Play (rtspp/play)" and or the Data Speed (ds) parameter in the "Data Delivery Speed Adjustment Speed (ddsa/spd)" signal. See clause 12.6 and 14.6 for procedural information.

#### 6.4.2.16	Teriminate-Reason

Typically the MGC may provide the information in the header to a user without MG involvement. However the "time" parameter may be populated as a result of the MGC receiving a ServiceChange command with a "Graceful" with a ServiceChangeDelay parameter.

#### 6.4.2.17	Transport

The MGC may interact with the MG via Add, Modify and Move commands to determine/set the transport parameters in the local and remote descriptors. H.248 supports over specification to allow for preferential lists of parameters.

General parameters:

Unicast | multicast: The MGC may provide this to the MG via SDP in H.248 Add.req, Modify.req or Move.req.  An IPv4 address range 224.0.0.0 to 239.255.255.255 or an IPv6 address starting with FF indicates a Multicast address. If a multicast address is used it is the responsibility of the MG to allocate a unicast IP address and join the multicast group through an applicable protocol, e.g. IGMP. Likewise at subtraction it is the responsibility of the MG to remove the unicast address from the multicast group.

If the MG is unable to allocate an IP address error code 445 "Unsupported or Unknown Property" is returned.

Layers: Multicast layers are supported by the SDP c=line (for an IP multicast address).

Destination: The MGC may provide this to the MG via SDP in H.248 Add.req, Modify.req or Move.req.

Source: It is expected that the MGC will provide this to the MG via SDP in H.248 Add.req, Modify.req or Move.req.

Mode: It is expected that the MGC will be able to map a mode to the relevant H.248 request.

MIKEY: [ITU-T H.248.77] discusses the use of secure RTP in split MGC/MGs.

Interleaved: Interleaving is supported by the RTSP Interleaving (rtspint) Package.

Ttl: Multicast TTL is supported by the SDP c= line.

RTP parameters:

The synchronisation source may be specified through the use of Synchronisation Source Property in clause 16.1.1. RTCP-mux may be specified via an SDP attribute in the H.248 Local and Remote descriptors.

Connection oriented transport parameters:

The values associated with parameters "setup" and "connection" have no direct information element in the H.248 protocol. However the MGC may provide an equivalent interworking. For example: for "Connection" the MGC may utilise an existing H.248 Stream rather than assigning a new one with a new connection address.

#### 6.4.2.18	Unsupported

It is expected the MGC will handle this. There may be some interaction with the MG via the H.248 error codes i.e. 440, 443-446.

# 7	Media resource identification package

|  | Package Name: | Media Resource Identification |
| --- | --- | --- |
|  | Package ID: | mri (0x00d5) |
|  | Description: | This package allows the MGC to identify a Media Resource that is operated on by the current or subsequent command. |
|  | Version: | 1 |
|  | Extends: | None |

## 7.1	Properties

### 7.1.1	Specification

|  | Property Name: | Specification |
| --- | --- | --- |
|  | Property ID: | spec (0x0001) |
|  | Description: | This property allows the MGC to specify a Media Resource which it currently or subsequently performs an operation on. |
|  | Type: | String |
|  | Possible values: | As per RTSPMediaResourceSpec of RTSP Media Resource Syntax (mrs) package. Optionally AnnouncementSpec in clause 6.2.5/[ ITU-T H.248.9] may be supported in addition. |
|  | Default: | None |
|  | Defined in: | TerminationState or LocalControl |
|  | Characteristics: | ReadWrite |

## 7.2	Events

None

## 7.3	Signals

None

## 7.4	Statistics

None

## 7.5	Error codes

None

## 7.6	Procedures

A MGC may determine the Media Resource syntax that is supported by a MG/Termination through auditing Packages. If the "RTSP Media Resource Syntax (mrs)" package is supported by the MG, the MGC can assume that the RTSP URI/IRI syntax may be used to describe a Media Resource. If the "Basic Announcement Syntax (bannsyx)" package is supported, the announcement syntax may be also used to describe a Media Resource.

However the MGC through auditing Packages is unable to determine the individual properties of a Media Resource. If the MGC requires additional information such as range, expiry, etc., or wants to adjust detailed parameters such as scale, speed, etc., of a particular Media Resource, it shall firstly set the Specification property of the Media Resource Identification package (mri/spec) to the Media Resource that will be used in its subsequent operating instruction to the MG regarding that Media Resource.

When the MGC performs an operation on a particular Media Resource via sending an Add, Modify or Move command request, it shall set the mri/spec property in this command request to identify the Media Resource. The MG shall then associate the Media Resource Identifier indicated by this property with that Media Resource, and respond with information about the Media Resource to the MGC via a corresponding command reply.

Subsequently, the MGC may use additional packages/properties with above Media Resource identifier to determine information or adjust parameters regarding that particular Media Resource. Whilst the Media Resource Identifier is provided a non-"None" value by the MGC in a request, then all responses from the MG for that request shall pertain to the Media Resource identified by that value.

The mri/spec property may be set on a Stream (of a Termination) level, an Individual Termination level or the Root Termination level. This allows the operation regarding the Media Resource to be scoped according to usage. If the mri/spec property is set on the Termination/Stream level, any subsequent operation should also pertain to the instance of this property on the Termination/Stream level.

If the mri/spec property is set on the higher (e.g. Root Termination) level and subsequent operation pertains to the instance of this property on lower (e.g. Individual Termination) level, the MG should process it based on the setting of the this property on the lower level.

# 8	Range format support package

|  | Package Name: | Range Format Support |
| --- | --- | --- |
|  | Package ID: | rfs (0x00d6) |
|  | Description: | This package allows the MGC and the MG to exchange which range format/s they respectively support for the particular media resources that have range information associated with them. |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

### 8.1.1	Format types

|  | Property Name: | Format Types |
| --- | --- | --- |
|  | Property ID: | ft (0x0001) |
|  | Description: | This property describes the range formats that may be supported. |
|  | Type: | Sub-list of String |
|  | Possible values: | NPT NPT format is supported SMPTE SMPTE format with the default (29.97fps) frame rate is supported SMPTE-30 SMPTE format with the 30fps frame rate is supported SMPTE-25 SMPTE format with the 25fps frame rate is supported UTC UTC (Clock) format is supported Frame Frame number format is supported Byte Byte number format is supported Bit Bit number format is supported Word Word number format is supported Sentence Sentence number format is supported Paragraph Paragraph number format is supported None No range information is associated with the Media Resource NOTE -The use of type String allows the use of extension tokens without the need to open the Package. |
|  | Default: | Provisioned |
|  | Defined in: | TerminationState or LocalControl |
|  | Characteristics: | ReadWrite |

## 8.2	Events

None

## 8.3	Signals

None

## 8.4	Statistics

None

## 8.5	Error codes

None

## 8.6	Procedures

Media Resources may have range information associated with them. This range information may be in different formats such as: NPT (Normal Play Time), SMPTE (Society of Motion Picture and Television Engineers) with typical frame rate, UTC (Coordinated Universal Time), Frame, Byte, Bit, Word, Sentence and Paragraph.

- NPT indicates the stream absolute position relative to the beginning of the presentation (see clause 4.4.2/[IETF RFC RTSP2]).

- SMTPE expresses time relative to the start of the clip for frame-level access accuracy (see clause 4.4.1/[IETF RFC RTSP2]).

- UTC (clock) is used to express a absolute time (see clause 4.4.3/[IETF RFC RTSP2]).

- Frame, Byte and Bit indicate the position respectively in frame, byte and bit number of the presentation/clip.

- Word, Sentence and Paragraph indicate the position respectively in word, sentence and paragraph units of the text/script.

When the MGC wishes to exchange the range format support capabilities about a particular media resource with the MG, it shall set the Format Type property of the Range Format Support package (rfs/ft) in the Add, Move or Modify command request regarding to that media resource. The MGC shall set the value of the rfs/ft property to a list of the range formats it requests (i.e. it supports). The MG shall then determine the list of range formats to be used by the media resource, and respond via the rfs/ft property in the corresponding command reply to the MGC. The MG may determine the range format support by determining the intersection between the one requested by the MGC and the one supported by the MG itself.

When the MGC wishes to know what range format a particular media resource supports from the MG, it shall either set the rfs/ft property with "CHOOSE ($)" wildcard value in the Add, Move or Modify command request, or perform an AuditCapabilities command request of the rfs/ft property regarding to that media resource. The MG shall then respond with the rfs/ft property containing a list of range formats supported by the media resource in the corresponding command reply to the MGC.

Where a MGC wishes to exchange or know the range format support capabilities about a particular instance of media resource, it shall set the Specification property of the Media Resource Identifier package (mri/spec) with the rfs/ft property in the above Add, Modify or Move command request.

# 9	Media resource description expiry package

|  | Package Name: | Media Resource Description Expiry |
| --- | --- | --- |
|  | Package ID: | mrde (0x00d7) |
|  | Description: | This package allows the MGC to determine when the description of a particular media resource should be considered stale. |
|  | Version: | 1 |
|  | Extends: | None |

## 9.1	Properties

### 9.1.1	Date and time

|  | Property Name: | Date and Time |
| --- | --- | --- |
|  | Property ID: | dt (0x0001) |
|  | Description: | This property indicates the Date and Time of the expiry of the media resource description. |
|  | Type: | String |
|  | Possible values: | The format is afull date and time as defined bysection 3.3/[IETF RFC 5322]. See clause 18.22/[IETF RFC RTSP2] for more details. |
|  | Default: | Provisioned |
|  | Defined in: | TerminationState or LocalControl |
|  | Characteristics: | ReadOnly |

## 9.2	Events

None

## 9.3	Signals

None

## 9.4	Statistics

None

## 9.5	Error codes

None

## 9.6	Procedures

Media resources may be described through the use of a "presentation" description which may be sent to (and through) various network entities. These "presentation" descriptions commonly use the Session Description Protocol (SDP) [IETF RFC 4566] as a means to describe the characteristics of the media resource. When these "presentation" descriptions pass through the network they may be cached by intermediate network entities or the endpoint. However the descriptions of the media resources may change with time due to changes of provisioning or dynamic changes of low level information and thus the original "presentation" description becomes stale. By indicating the expiry information associated with the "presentation" description it allows the network entities to be aware how long the media resource description is valid for. See clause 18.22/[IETF RFC RTSP2] for more information how an expiry information may be used.

As it is the MG that contains the media resources, the MGC may not be aware of the expiry information associated with a particular media resource.

When the MGC wishes to know the expiry information associated with a particular media resource from the MG, it shall either set the Date and Time property of the Media Resource Description Expiry package (mrde/dt) property with "CHOOSE ($)" wildcard value in the Add, Move or Modify command request, or perform AuditValue command request of the mrde/dt property regarding to that media resource. The MG shall then respond via the mrde/dt property with the expiry information related to that media resource in the corresponding command reply to the MGC.

Where a MGC wishes to know the expiry information associated with a particular instance of media resource, it shall synchronously set the Specification property of the Media Resource Identifier package (mri/spec) with the mrde/dt property in the above Add, Modify or Move command request.

If the mrde/dt property is set on a Termination/Stream without the Media Resource Identifier and there are multiple Media Resources on the Termination/Stream, the MG shall respond with error code 472 "Required Information Missing".

# 10	Media block size package

|  | Package Name: | Media Block Size |
| --- | --- | --- |
|  | Package ID: | mbs (0x00d8) |
|  | Description: | This package allows the MGC to request the MG to use a particular media packet size when delivering media from a Termination/Stream. |
|  | Version: | 1 |
|  | Extends: | None |

## 10.1	Properties

### 10.1.1	Size

|  | Property Name: | Size |
| --- | --- | --- |
|  | Property ID: | size (0x0001) |
|  | Description: | This property allows the MGC to request a particular media packet size. The media packet size does not include lower-layer headers such as IP, UDP, or RTP. |
|  | Type: | Integer |
|  | Possible values: | Any positive number of Octets |
|  | Default: | Provisioned dependent on media format/type |
|  | Defined in: | TerminationState or LocalControl |
|  | Characteristics: | ReadWrite |

## 10.2	Events

None

## 10.3	Signals

None

## 10.4	Statistics

None

## 10.5	Error codes

None

## 10.6	Procedures

A MGC may be requested by a client entity to provide a particular media packet size. The media packet size shall be provided to the MG via the use of the Size property of the Media Block Size (mbs/size) package. The MG may freely use a media packet size which is lower than the one requested. The MG may truncate this media packet size to the closest multiple of the minimum, media-specific block size, or override it with a media-specific size if necessary.  No H.248 error response is returned if the value is overridden. Then the MG shall return the actual media packet size it uses to the MGC. See clause 18.10/[IETF RFC RTSP2] for further information.

Where a MGC requests that all media on a particular Termination use a specific media packet size the mbs/size property should be provided in the TerminationState descriptor. Where only a particular Stream should use a specific media packet size the mbs/size property should be provided in the LocalControl descriptor.

If a MGC performs a "CHOOSE ($)" wildcard on the mbs/size property, the MG should provide the media packet size value. However if it is unable to do so due to the media description (i.e. Local and Remote) being insufficient, error code 472 "Required Information Missing" should be returned.

# 11	RTSP media resource syntax package

|  | Package Name: | RTSP Media Resource Syntax |
| --- | --- | --- |
|  | Package ID: | mrs (0x00d9) |
|  | Description: | This package exists only to indicate that the MG is capable of processing the RTSP URI syntax described herein. An MGC learns that the capability is supported by auditing the packages supported by the Termination on which RTSP playout is to be performed and verifying that this package is listed. NOTE - The syntax of "RTSPMediaResourceSpec" described below is aligned with the [ITU-T H.248.9] "Basic Announcement Syntax" package syntax for provisioned segments described by "announcementSpec". This is in order that generic packages such as the "Signal Pause" Package may have applicability to various Media Resource syntaxes. |
|  | Version: | 1 |
|  | Extends: | None |

## 11.1	Properties

None

## 11.2	Events

None

## 11.3	Signals

None

## 11.4	Statistics

None

## 11.5	Error codes

None

## 11.6	Procedures

### 11.6.1	General structure

A RTSP Media Resource is identified through the use of an URI that whose syntax is restricted as compared to [IETF RFC 3986]. Clause 3.2/[IETF RFC 3986] describes the use of URIs and IRIs with respect to RTSP. The clauses below describe the encoding of the RTSP URIs and IRIs which respect to H.248 Signal parameters and Properties. IRIs can be mapped to URI using the procedures defined in clause 3/[IETF RFC 3987].

#### 11.6.1.1	ASN.1 encoding

In the ASN.1 encoding, the general signal parameter syntax is ultimately constrained by the Value production of Annex A/[ITU-T H.248.1]. Parameter values are double-wrapped with an inner BER encoding applied first to aid interpretation of the parameter, followed by an outer BER encoding as an OCTET STRING. The general structure of the basic RTSP Media Resource specification syntax for purposes of the inner encoding is expressed as follows:

RTSPMediaResourceSpec ::= IA5String

The details of the string structure are as specified in the remainder of clause 11.6: they are equally applicable to text and ASN.1 encoding.

#### 11.6.1.2	Text encoding

In the text encoding, the detailed signal parameter syntax is ultimately constrained by the VALUE production of Annex B/[ITU-T H.248.1]. The ABNF [IETF RFC 5234] description of the gross structure of an announcement specification is as follows:

RTSPMediaResourceSpec = DQUOTE segSpec DQUOTE
	  ; DQUOTE and COMMA are as defined in Annex B/H.248.1.

segSpec = keyword "=" "<" spec ">" ; angle brackets as delimiters

keyword = "sid"          ; provisioned segment identifier

spec    = provSegSpec    ; provisioned segment identifier

The quotedString form of VALUE is required for RTSPMediaResourceSpec because a segSpec can contain restricted characters (e.g., =, <, > as shown above), and because successive segSpecs are comma-separated. However, the VALUE production requires escapes for the following:

- all control characters (%x00-%x1F and %x7F) except TAB (%x09) ;

- the DQUOTE character (%x22).

Escaping within URIs must be performed as "percent-encoding" described in [IETF RFC 3986]. Escaping within stand-alone voice variable specifications uses the same mechanism as [IETF RFC 3986], but applies only to the characters listed above, the percent sign "%" (which is used as an escape character), and the closing angle bracket ">" (which terminates a variable value). (This is currently a non-issue, since neither "%" as a non-escape character nor ">" will be found within any variable value defined in this Recommendation.)

Keywords in the text encoded syntax are case-insensitive. Case sensitivity within URIs is defined by the applicable standards.

### 11.6.2	Provisioned segment specifications

A provisioned segment specification consists of a URI formed under the rules of [IETF RFC 3986]. This Recommendation supports three URI schemes:

- the rtsp: scheme, the rtsps: scheme, the rtspu: The rtsp schemes are used to indicate resources for the primary purpose of streaming delivery of the resource to a client. The difference of the schemes is with regards to the RTSP protocol itself. i.e. "rtsps" relates to its transport of TLS, "rtspu" relates to its transport over UDP and "rtsp" relates to its transport over TCP.

In accordance with [IETF RFC 3986], the following characters must be escaped within all URIs:

- reserved characters within the individual URI schemes.

- the space character;

- characters used as delimiters or for escaping: "<", ">", "#", "%", and <">;

- characters subject to unwanted transformations or subject to misinterpretation: "{", "}", "|", "\", "^", "[", "]", and "`".

#### 11.6.2.1	Text encoding

This clause provides a detailed description of the provSegSpec production which is referred to in 6.2.5.1.2 / [ITU-T H.248.9].

provSegSpec = rtspurl

rtspurl = RTSP-URI

; See clause 20.2.1/[IETF RFC RTSP2] for the

; syntax related to URIs using the rtsp, rtsps and rtspu schemes.

# 12	RTSP play package

|  | Package Name: | RTSP Play |
| --- | --- | --- |
|  | Package ID: | rtspp (0x00da) |
|  | Description: | This Package is used to playout a particular media resource with the specified parameters. |
|  | Version: | 1 |
|  | Extends: | None |

## 12.1	Properties

None

## 12.2	Events

### 12.2.1	End-of-stream notification event

|  | Event Name: | End-of-Stream Notification |
| --- | --- | --- |
|  | Event ID: | eos (0x0001) |
|  | Description: | This event enables the MGC to be notified when the playout of a media resource has ended or is nearing completion. |

#### 12.2.1.1	EventsDescriptor parameters

None

#### 12.2.1.2	ObservedEventsDescriptor parameters

##### 12.2.1.2.1	End range

|  | Parameter Name: | End Range |
| --- | --- | --- |
|  | Parameter ID: | er (0x0001) |
|  | Description: | This parameter indicates the point in the stream or streams where delivery is/are ending with the timescale that has been used by the client in the PLAY request being fulfilled. |
|  | Type: | String |
|  | Optional: | No |
|  | Possible values: | Range = ranges-spec The ABNF syntax is defined in clause 20.2.3/[IETF RFC RTSP2]. NOTE - The above syntax omits "Range" HCOLON as it is unnecessary for H.248 encoding. The syntax also allows the use of a "Quoted String" form. If this is used the double quote character should be represented by \%x22. |
|  | Default: | None |

##### 12.2.1.2.2	RTP synchronisation source

|  | Parameter Name: | RTP Synchronization Source |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This property allows the MGC to receive the synchronisation source information if RTP is used as media transport. See clause 13.5.1/[IETF RFC RTSP2]. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | ssrc = 8 * HEXDIG Encoded as per Annex B.2/[ITU-T H.248.1]. |
|  | Default: | None |

##### 12.2.1.2.3	RTP sequence number

|  | Parameter Name: | RTP Sequence Number |
| --- | --- | --- |
|  | Parameter ID: | seq (0x0003) |
|  | Description: | This property allows the MGC to receive the last sequence number that applies to the RTP stream. See clause 13.5.1/[IETF RFC RTSP2]. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 12.2.1.2.4	RTP timestamp

|  | Parameter Name: | RTP Timestamp |
| --- | --- | --- |
|  | Parameter ID: | ts (0x0004) |
|  | Description: | This property allows the MGC to receive the RTP timestamp value corresponding to the end time that applies to the RTP stream. See clause 13.5.1/[IETF RFC RTSP2]. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | Unsigned Integer |
|  | Default: | None |

##### 12.2.1.2.5	Media resource identifier

|  | Parameter Name: | Media Resource Identifier |
| --- | --- | --- |
|  | Parameter ID: | mri (0x0005) |
|  | Description: | The identifier of the media resource that is streaming. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per RTSPMediaResourceSpec of RTSP Media Resource Syntax (rmrs) package. |
|  | Default: | None |

##### 12.2.1.2.6	Request ID

|  | Parameter Name: | Request ID |
| --- | --- | --- |
|  | Parameter ID: | rid (0x0006) |
|  | Description: | This parameter indicates to which play request the event is associated. See clause 7.1.11/[ITU-T H.248.1]. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 1-4294967295 |
|  | Default: | None |

### 12.2.2	Media properties and range change event

|  | Event Name: | Media Properties and Range Change |
| --- | --- | --- |
|  | Event ID: | mprc (0x0002) |
|  | Description: | This event enables the MGC to be notified when the media properties (see clause 18.29/[IETF RFC RTSP2]) or the media range (see clause 18.30/[IETF RFC RTSP2]) requires change. |

#### 12.2.2.1	EventsDescriptor parameters

##### 12.2.2.1.1	Time progressing interval

|  | Parameter Name: | Time Progressing Interval |
| --- | --- | --- |
|  | Parameter ID: | tpi (0x0001) |
|  | Description: | Specifies the time interval for mprc event notifications in the case of time progressing media. See clause 13.5.2/[IETF RFC RTSP2]. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 0 upwards in seconds. |
|  | Default: | 300 seconds. |

#### 12.2.2.2	ObservedEventsDescriptor parameters

##### 12.2.2.2.1	Media properties

|  | Parameter Name: | Media Properties |
| --- | --- | --- |
|  | Parameter ID: | mp (0x0001) |
|  | Description: | Indicates the media properties change that the MG requires. |
|  | Type: | String |
|  | Optional: | No |
|  | Possible values: | As per media-prop-list in clause 20.2.3/[IETF RFC RTSP2] |
|  | Default: | None |

##### 12.2.2.2.2	Media range

|  | Parameter Name: | Media Range |
| --- | --- | --- |
|  | Parameter ID: | mr (0x0002) |
|  | Description: | This parameter indicates the media range change that the MG requires. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per ranges-list in clause 20.2.3/[IETF RFC RTSP2] |
|  | Default: | None |

##### 12.2.2.2.3	Media resource identifier

|  | Parameter Name: | Media Resource Identifier |
| --- | --- | --- |
|  | Parameter ID: | mri (0x0003) |
|  | Description: | The identifier of the media resource that is streaming. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per RTSPMediaResourceSpec of RTSP Media Resource Syntax (rmrs) package. |
|  | Default: | None |

##### 12.2.2.2.4	Request ID

|  | Parameter Name: | Request ID |
| --- | --- | --- |
|  | Parameter ID: | rid (0x0004) |
|  | Description: | This parameter indicates to which play request the event is associated. See clause 7.1.11/[ITU-T H.248.1]. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 1-4294967295 |
|  | Default: | None |

### 12.2.3	Scale Change Event

|  | Event Name: | Scale Change |
| --- | --- | --- |
|  | Event ID: | sc (0x0003) |
|  | Description: | This event enables the MGC to be notified when the MG detects that it will be unable to continue to provide content at the current scale. See clause 13.5.3/[IETF RFC RTSP2]. |

#### 12.2.3.1	EventsDescriptor parameters

##### 12.2.3.1.1	Enable scale change

|  | Parameter Name: | Enable Scale Change |
| --- | --- | --- |
|  | Parameter ID: | esc (0x0001) |
|  | Description: | This parameter enables or disables the autonomous scale change capability when scale adjustment is desired. |
|  | Type: | Boolean |
|  | Optional: | Yes |
|  | Possible values: | OFF (0x0000): Autonomous scale change is not allowed ON (0x0001): Autonomous scale change is enabled |
|  | Default: | OFF |

##### 12.2.3.1.2	Media resource identifier

|  | Parameter Name: | Media Resource Identifier |
| --- | --- | --- |
|  | Parameter ID: | mri (0x0002) |
|  | Description: | The identifier of the media resource that is streaming. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per RTSPMediaResourceSpec of RTSP Media Resource Syntax (rmrs) package. |
|  | Default: | None |

##### 12.2.3.1.3	Request ID

|  | Parameter Name: | Request ID |
| --- | --- | --- |
|  | Parameter ID: | rid (0x0003) |
|  | Description: | This parameter indicates to which play request the event is associated. See clause 7.1.11/[ITU-T H.248.1]. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 1-4294967295 |
|  | Default: | None |

#### 12.2.3.2	ObservedEventsDescriptor parameters

##### 12.2.3.2.1	Scale change

|  | Parameter Name: | Scale Change |
| --- | --- | --- |
|  | Parameter ID: | sc (0x0001) |
|  | Description: | If the "Enable Scale Change" (esc) parameter is set to ON, this parameter indicates the scale that the MG has adopted after the change; if the esc parameter is set to OFF, it indicates the scale that the MG wants to change to. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | Scale Change = FLOAT POS-FLOAT = 1*12DIGIT ["." 1*9DIGIT] FLOAT = ["-"] POS-FLOAT The ABNF syntax is defined in clause 20.2.3/[IETF RFC RTSP2]. |
|  | Default: | 1 |

##### 12.2.3.2.2	Scale range

|  | Parameter Name: | Scale Range |
| --- | --- | --- |
|  | Parameter ID: | sr (0x0002) |
|  | Description: | This parameter indicates the media time at which the change took effect with the timescale that has been used by the client in the PLAY request being fulfilled. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | Range = ranges-spec The ABNF syntax is defined in clause 20.2.3/[IETF RFC RTSP2]. NOTE - The above syntax omits "Range" HCOLON as it is unnecessary for H.248 encoding. The syntax also allows the use of a "Quoted String" form. If this is used the double quote character should be represented by \%x22. |
|  | Default: | None |

##### 12.2.3.2.3	RTP synchronisation source

|  | Parameter Name: | RTP Synchronization Source |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0003) |
|  | Description: | This property allows the MGC to receive the synchronisation source information if RTP is used as media transport. See clause 13.5.1/[IETF RFC RTSP2]. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | ssrc = 8 * HEXDIG Encoded as per Annex B.2/[ITU-T H.248.1]. |
|  | Default: | None |

##### 12.2.3.2.4	RTP sequence number

|  | Parameter Name: | RTP Sequence Number |
| --- | --- | --- |
|  | Parameter ID: | seq (0x0004) |
|  | Description: | This property allows the MGC to receive the sequence number corresponding to the point of change that applies to the RTP stream. See clause 13.5.1/[IETF RFC RTSP2]. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 12.2.3.2.5	RTP timestamp

|  | Parameter Name: | RTP Timestamp |
| --- | --- | --- |
|  | Parameter ID: | ts (0x0005) |
|  | Description: | This property allows the MGC to receive the RTP timestamp value corresponding to the point of change that applies to the RTP stream. See clause 13.5.1/[IETF RFC RTSP2]. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | Unsigned Integer |
|  | Default: | None |

##### 12.2.3.2.6	Media resource identifier

|  | Parameter Name: | Media Resource Identifier |
| --- | --- | --- |
|  | Parameter ID: | mri (0x0006) |
|  | Description: | The identifier of the media resource that is to be streamed. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per RTSPMediaResourceSpec of RTSP Media Resource Syntax (rmrs) package. |
|  | Default: | None |

##### 12.2.3.2.7	Request ID

|  | Parameter Name: | Request ID |
| --- | --- | --- |
|  | Parameter ID: | rid (0x0007) |
|  | Description: | This parameter indicates to which play request the event is associated. See clause 7.1.11/[ITU-T H.248.1]. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 1-4294967295 |
|  | Default: | None |

## 12.3	Signals

### 12.3.1	Play

|  | Signal Name: | Play |
| --- | --- | --- |
|  | Signal ID: | play (0x0001) |
|  | Description: | Plays a RTSP Media Resource. |
|  | SignalType: | Brief |
|  | Duration: | Not applicable to Brief signals |

#### 12.3.1.1	Additional parameters

##### 12.3.1.1.1	Media Resource identifier

|  | Parameter Name: | Media Resource Identifier |
| --- | --- | --- |
|  | Parameter ID: | mri (0x0001) |
|  | Description: | The identifier of the media resource that is to be streamed. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per RTSPMediaResourceSpec of RTSP Media Resource Syntax (rmrs) package. |
|  | Default: | None |

##### 12.3.1.1.2	Playback scale

|  | Parameter Name: | Playback Scale |
| --- | --- | --- |
|  | Parameter ID: | ps (0x0002) |
|  | Description: | The playback relative scale of media resource specifiable as a positive (faster) or negative (slower) variation from the normal playback speed. For more information on playback scale see clause 18.44/[IETF RFC RTSP2]. NOTE - The functionality of the Playback Scale parameter is otherwise known as "play speed". |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | Playback Scale = FLOAT POS-FLOAT = 1*12DIGIT ["." 1*9DIGIT] FLOAT = ["-"] POS-FLOAT The ABNF syntax is defined in clause 20.2.3/[IETF RFC RTSP2]. |
|  | Default: | 1 |

##### 12.3.1.1.3	Direction

|  | Parameter Name: | Direction |
| --- | --- | --- |
|  | Parameter ID: | di (0x0003) |
|  | Description: | The direction parameter can be used to indicate the direction that the RTSP media stream is to be sent. |
|  | Type: | Enumeration |
|  | Optional: | Yes NOTE - The implementation of this parameter is optional in ITU-T H.248.1v3 implementations as signal direction is supported as part of the core protocol. |
|  | Possible values: | Ext (0x01) :indicates that the RTSP media stream is sent from the MG to an external point Int (0x02): indicates that the RTSP media stream is played into the MG to the other terminations Both (0x03): indicates internal and external behaviour |
|  | Default: | Ext (0x01) |

##### 12.3.1.1.4	Play range

|  | Parameter Name: | Play Range |
| --- | --- | --- |
|  | Parameter ID: | pr (0x0004) |
|  | Description: | This parameter is used to indicate the playout range information of the media resource associated with the signal. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | Range = ranges-spec The ABNF syntax is defined in clause 20.2.3/[IETF RFC RTSP2]. NOTE - The above syntax omits "Range" HCOLON as it is unnecessary for H.248 encoding. The syntax also allows the use of a "Quoted String" form. If this is used the double quote character should be represented by \%x22. |
|  | Default: | None |

##### 12.3.1.1.5	Range format

|  | Parameter Name: | Range Format |
| --- | --- | --- |
|  | Parameter ID: | rf (0x0005) |
|  | Description: | This parameter is used to indicate the range format used in the range information related to the URI parameter in this signal. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per the Format Type property of the Range Format Support package (rfs/ft). NOTE -The use of type String allows the use of extension tokens without the need to open the Package. |
|  | Default: | NPT |

##### 12.3.1.1.6	Data speed

|  | Parameter Name: | Data Speed |
| --- | --- | --- |
|  | Parameter ID: | ds (0x0006) |
|  | Description: | This parameter allows the MGC to instruct the MG to deliver the media resource at a specified data speed. The bandwidth required to deliver the media resource may need to be altered. For more information on data delivery speed see clause 18.50/[IETF RFC RTSP2]. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | Speed = lower-bound MINUS upper-bound The ABNF syntax is defined in clause 20.2.3/[IETF RFC RTSP2]. NOTE: The above syntax omits "Range" HCOLON as it is unnecessary for H.248 encoding. |
|  | Default: | 1-1 |

##### 12.3.1.1.7	Media properties

|  | Parameter Name: | Media Properties |
| --- | --- | --- |
|  | Parameter ID: | mp (0x0007) |
|  | Description: | Indicates the media properties that the MGC requires. May be used with wildcard CHOOSE in order to determine the media properties from the MG. |
|  | Type: | String |
|  | Optional: | No |
|  | Possible values: | As per media-prop-list in clause 20.2.3/[IETF RFC RTSP2] |
|  | Default: | None |

##### 12.3.1.1.8	Seek behaviour

|  | Parameter Name: | Seek Behaviour |
| --- | --- | --- |
|  | Parameter ID: | sb (0x0008) |
|  | Description: | This parameter can be used to indicate the seek preference of where to start the media stream when performing a random access. See more details of seek behaviours in clause 18.47/[IETF RFC RTSP2]. |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | RAP (0x0001) indicates to the MG to locate and start play from the closest previous random access point of the media stream FP (0x0002) indicates to the MG to start delivery with the media unit that has a playout time first prior to the requested time Next (0x0003) indicates to the MG to start from the next media unit after the provided start time CoRap (0x0004) is a variant of the RAP behaviour. See clause 18.45/[IETF RFC RTSP2] for a definition of the behaviour. |
|  | Default: | RAP (0x0001) |

## 12.4	Statistics

None

## 12.5	Error codes

None

## 12.6	Procedures

The MGC invokes the Play signal of the RTSP Play package (rtspp/play) with at least the Media Resource Identifier (mri) parameter set to play out the specified RTSP media resource. RTSP media resource playout is subject to termination by Events or new Signals descriptor settings in the normal way. The MGC can use the standard signal NotifyCompletion capability to determine when and why playout has ended.

The MGC may also specify the range of the RTSP media resource that should be played through the use of the Play Range (pr) parameter. The MG shall then play the content indicated by the pr parameter from the MGC, and return the actual range of the RTSP media resource it is playing via the pr parameter to the MGC. The MGC or the MG may use the Range Format (rf) parameter to indicate the format of the information in the pr parameter. When the MGC sends a Play signal with the Play Range (pr) parameter to perform a random access to the media, the MGC may use the Seek Behaviour (sb) parameter to specify the access and seek preference of where to start media delivery. If the MGC requires a specific seek style then it shall send on one seek value. If the MGC has a preference for a certain style it should send a list of values with the preferred value in the first list position. Where a list is provided the MG shall  return the actual seek policy of the media it adopted to the MGC.

The MGC may set an initial playback relative scale and/or data delivery speed through the use of the Playback Scale (ps) and Data Speed (ds) respectively. In order to control the media play state, the MGC may also specify (or request) the Media Properties (mp) parameter to indicate the Media properties, including the Random Access, Retention, and Content Modifications of the media resource. See clause 18.29/[IETF RFC RTSP2] for the detailed properties.

If the MGC needs the range information associated with the RTSP media resource, it should "CHOOSE ($)" wildcard the pr parameter. The MG shall then return the range of the RTSP media resource via the pr parameter. If the MGC requires specified range format, it shall use the rf parameter.

The MGC indicates the related events on the MG to detect the changes of the media play state, when the change is detected, the MG would notify the event to the MGC. There are three types of events are specified:

- The End-of-Stream Notification Event (eos) is used to notify the MGC that the media has ended or will be complete in the near future. The Signal Completion Event can be used to indicate the termination of the play signal, but the eos event may occur before the end of the media to enable the MGC to acquire  associated parameters, including the End Range of the media and the RTP related information if RTP is used.

- The MGC orders the Media Properties and Range Change Event to acquire the changes of the media properties and media range that the MG requires, which is carried by the Media Properties (mp) and Media Range (mr) parameters. The Time Progressing Interval (tpi) parameter provides the time interval of the notification. Based on the reported mprc Event, the MGC may issue an update play signal to specify the new properties.

- The Scale Change Event enables the MGC to be notified when the MG detects that a scale adjustment is needed. The MGC may use the Media Resource Identifier (mri) and Request ID (rid) parameters to specify the instance that the event applies. When setting this event on the MG, the MGC may enable the autonomous scale change capability via Enable Scale Change (esc) parameter. If the esc parameter is set to 'ON', the MG changes the scale value autonomously and notifies the new scale value and media range time where the change took effect to the MGC. If the esc parameter is set to 'OFF', the MG cannot make the change without the MGC's indication. The MG should notify the MGC of the scale value that it wants to change to, which may then specify the adjusted scale value via an update RTSP play signal. In both of the above two cases, RTP related parameters should also be notified if RTP is used.

In the case that the MGC uses multiple RTSP play signals on the termination it should use a Signal RequestID. If used the MG shall return the requestID with the event to indicate the signal request that is associated. Unlike SignalCompletion the signal id is not needed because it will always be linked to the play RTSP signal.

# 13	Signal pause package

|  | Package Name: | Signal Pause |
| --- | --- | --- |
|  | Package ID: | sp (0x00db) |
|  | Description: | This package allows the MGC to request the MG to pause the playout of a particular signal. This is a generic package that may be applied to a diverse range of signals. |
|  | Version: | 1 |
|  | Extends: | None |

## 13.1	Properties

None

## 13.2	Events

None

## 13.3	Signals

### 13.3.1	Pause

|  | Signal Name: | Pause |
| --- | --- | --- |
|  | Signal ID: | pause (0x0001) |
|  | Description: | This signal allows the MGC to request a MG to pause the playout of signals. |
|  | SignalType: | On/Off |
|  | Duration: | Not applicable to On/Off signals |

#### 13.3.1.1	Additional parameters

##### 13.3.1.1.1	Signal identity

|  | Parameter Name: | Signal Identity |
| --- | --- | --- |
|  | Parameter ID: | sigid (0x0001) |
|  | Description: | The Signal Identity parameter allows a MGC to pause the playout of a Signal with the indicated signal identity. |
|  | Type: | Octet String |
|  | Optional: | Yes |
|  | Possible values: | For text encoding a Package Name / Signal ID encoded as per pkgdName in Annex B.2/[ITU-T H.248.1]. For binary encoding a Package Name / Signal ID encoded as per SignalName in Annex A.2/[ ITU-T H.248.1]. |
|  | Default: | None |

##### 13.3.1.1.2	Signal list identity

|  | Parameter Name: | Signal List Identity |
| --- | --- | --- |
|  | Parameter ID: | listid (0x0002) |
|  | Description: | The Signal List Identity parameter allows a MGC to pause the playout of a Signal List with the indicated signal list identity. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 0 to 65535 |
|  | Default: | None |

##### 13.3.1.1.3	Media resource identifier

|  | Parameter Name: | Media Resource Identifier |
| --- | --- | --- |
|  | Parameter ID: | mri (0x0003) |
|  | Description: | The Media Resource Identifier parameter allows the MGC to indicate the media resource that a pause should be applied to. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per RTSPMediaResourceSpec of RTSP Media Resource Syntax (mrs) package. Optionally AnnouncementSpec in clause 6.2.5/[ ITU-T H.248.9] may be supported in addition. |
|  | Default: | None |

##### 13.3.1.1.4	Pause range

|  | Parameter Name: | Pause Range |
| --- | --- | --- |
|  | Parameter ID: | pr (0x0004) |
|  | Description: | This parameter allows the MGC to request the MG to return the point at which the playout of a signal was interrupted and what part of the signal remains unplayed. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | Range = ranges-spec The ABNF syntax is defined in clause 20.2.3/[IETF RFC RTSP2]. NOTE -The above syntax omits "Range" HCOLON as it is unnecessary for H.248 encoding. The syntax also allows the use of a "Quoted String" form. If this is used the double quote character should be represented by \%x22. |
|  | Default: | None |

##### 13.3.1.1.5	Range format

|  | Parameter Name: | Range Format |
| --- | --- | --- |
|  | Parameter ID: | rf (0x0005) |
|  | Description: | This parameter indicates the range information format that shall be used with this signal. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per the Format Type property of the Range Format Support package (rfs/ft). NOTE - The use of type String allows the use of extension tokens without the need to open the Package. |
|  | Default: | NPT |

## 13.4	Statistics

None

## 13.5	Error codes

### 13.5.1	Unable to pause the playout of the signal

|  | Error Code #: | 475 |  |
| --- | --- | --- | --- |
|  | Name: | Unable to pause the playout of the signal |  |
|  | Definition: | The MG has been unable to pause the playout of the signal referenced by the Pause signal of the Signal Pause package. |  |
|  | Error Text in the Error Descriptor: |  | The signal identity of the applicable signal encoded as per clause 13.3.1.1.1. |
|  | Comment: | The signal referenced by the Pause signal continues to play out. |  |

## 13.6	Procedures

### 13.6.1	Pausing signal playout

A MGC may be requested by a client entity to halt the delivery of a media component in such a way that the media is paused at a particular point in order for the media delivery to start again from that point at some time in the future.

The use of Stream mode set to "RecvOnly" fulfils the objective of stopping media flow but does not maintain this playout state information. Stopping a signal by turning a signal off or replacing the Signals descriptor has the effect of stopping the media flow but does not maintain the playout state information either.

In order to halt media delivery (which was started as an result of the operating of certain Signal) and maintain the pause point information, the MGC shall send the Pause signal of the Signal Pause Package (sp/pause) to the MG. The MGC shall include existing Signals/SignalLists as well as the sp/pause signal in the Signal Descriptor in order to maintain the playout (albeit paused) of the Signal. This makes use of the H.248 capability where multiple Signals are played simultaneously (see clause 7.1.11/[ITU-T H.248.1]).

If the sp/pause signal is set by the MGC at a Termination or Stream level without the Signal Identity (sigid), Signal List Identity (listid) or Media Resource Identifier (mri) parameter then the MG shall pause the playout of all signals on that Termination or Stream.

If the sp/pause signal is set by the MGC at a Termination or Stream level with the sigid or listid parameter then the MG shall pause the playout of the signal indicated by the sigid parameter or the entire signal list indicated by the listid parameter on that Termination or Stream.

If the sp/pause signal is set by the MGC with the mri parameter then the MG shall pause the playout of the signal(s) associated with the media resource.

If the sp/pause signal is set by the MGC with the mri parameter and the sigid or listid parameter then the MG shall pause the playout of the signal(s) associated with the media resource and indicated by the sigid or listid parameter.

If the MG detects a mismatch between the sigid/listid parameter and the Signal/SignalList playout on that Termination/Stream or associated with the media resource indicated by the mri parameter then it should return error code 473 "Conflicting Property Values".

Note 1: In some cases there may be more than one instance of a Signal Identity or Signal List Identity for a Termination/Stream in these cases the MGC should use the mri parameter to uniquely identify the signal instance.

Note 2: A MGC may determine the Media Resource Resource syntax that is supported by a MG/Termination through auditing packages. If the "RTSP Media Resource Syntax (mrs)" package is supported the MG can assume that the RTSP URI/IRI syntax may be used to describe a media resource. If the "Basic Announcement Syntax (bannsyx)" Package is supported announcement syntax may be also used to describe a media resource.

The MG should not release media resources associated with the signal(s) that are paused. However the MGC may stop the playing Signal(s) if it determines it is necessary. In this case the MGC should remove the playing Signal(s) as well as the sp/pause signal.

If the MG is unable to pause the signal(s) indicated by the sp/pause signal then it should respond with error code 475 "Unable to pause the playout of the signal". The referenced signal(s) should continue to play out.

### 13.6.2	Requesting pause information

An MGC may be requested to provide information associated with the pause point and the range regarding the media yet to playout. This information can then be used in later communication to restart the playout from the pause point or at a point later in the remaining media range. To request the pause information the MGC should do a "CHOOSE ($)" wildcard on the Pause Range (pr) parameter. The MG shall then return the range information in the format indicated by the Range Format (rf) parameter. See clause 13.6/[IETF RFC RTSP2] for an example use of Pause Point information. If multiple media resources/streams are in use, this may need to multiple pause point information being returned. If the sp/pause signal affects multiple media resources and the pr parameter is requested then the MG should return the common range information for all the media resources. If the MG is unable to determine which pr parameter values should be returned, it should issue error code 473 "Conflicting Property Values".

### 13.6.3	Restarting after a pause

When the MGC wants resume the playout of the signal(s) from the pause point it should remove the relevant instance of the sp/pause signal from the Signal Descriptor. The MG shall then continue to play the signal(s) related to the removed sp/pause signal from the saved pause point.

The MGC may also request that playout should resume from another point, in this case the MGC should remove the sp/pause signal and the signal(s) related to the removed sp/pause signal and re-issue the signal(s) with new range information.

# 14	Data delivery speed adjustment package

|  | Package Name: | Data Delivery Speed Adjustment |
| --- | --- | --- |
|  | Package ID: | ddsa (0x00dc) |
|  | Description: | This package allows the MGC to request the MG to alter the data delivery speed of a particular signal. This is a generic package that may be applied to a diverse range of signals related to media playout. |
|  | Version: | 1 |
|  | Extends: | None |

## 14.1	Properties

None

## 14.2	Events

None

## 14.3	Signals

### 14.3.1	Speed

|  | Signal Name: | Speed |
| --- | --- | --- |
|  | Signal ID: | spd (0x0001) |
|  | Description: | This signal allows the MGC to alter the data delivery speed of a particular signal. |
|  | SignalType: | On/Off |
|  | Duration: | Not applicable to on/off signals |

#### 14.3.1.1	Additional parameters

##### 14.3.1.1.1	Signal identity

|  | Parameter Name: | Signal Identity |
| --- | --- | --- |
|  | Parameter ID: | sigid (0x0001) |
|  | Description: | The Signal Identity parameter allows a MGC to alter the data delivery speed to the playout of a Signal with the indicated signal identity. |
|  | Type: | Octet String |
|  | Optional: | Yes |
|  | Possible values: | For text encoding a Package Name / Signal ID encoded as per pkgdName in Annex B.2/[ITU-T H.248.1]. For binary encoding a Package Name / Signal ID encoded as per SignalName in Annex A.2/[ ITU-T H.248.1]. |
|  | Default: | None |

##### 14.3.1.1.2	Signal list identity

|  | Parameter Name: | Signal List Identity |
| --- | --- | --- |
|  | Parameter ID: | listid (0x0002) |
|  | Description: | The Signal List Identity parameter allows a MGC to alter the data delivery speed to the playout of a Signal List with the indicated identity. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 0 to 65535 |
|  | Default: | None |

##### 14.3.1.1.3	Media resource identifier

|  | Parameter Name: | Media Resource Identifier |
| --- | --- | --- |
|  | Parameter ID: | mri (0x0003) |
|  | Description: | The Media Resource Identifier parameter allows the MGC to indicate the media resource whose data delivery speed should be altered. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per RTSPMediaResourceSpec of RTSP Media Resource Syntax (mrs) package. Optionally AnnouncementSpec in clause 6.2.5/[ ITU-T H.248.9] may be supported in addition. |
|  | Default: | None |

##### 14.3.1.1.4	Range

|  | Parameter Name: | Range |
| --- | --- | --- |
|  | Parameter ID: | rng (0x0004) |
|  | Description: | This parameter is used to indicate the adjusting range information of the media resource associated with the signal. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | Range = ranges-spec The ABNF syntax is defined in clause 20.2.3/[IETF RFC RTSP2]. NOTE - The above syntax omits "Range" HCOLON as it is unnecessary for H.248 encoding. The syntax also allows the use of a "Quoted String" form. If this is used the double quote character should be represented by \%x22. |
|  | Default: | None |

##### 14.3.1.1.5	Range format

|  | Parameter Name: | Range Format |
| --- | --- | --- |
|  | Parameter ID: | rf (0x0005) |
|  | Description: | This parameter indicates the range information format that shall be used with this signal. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per the Format Type property of the Range Format Support package (rfs/ft). NOTE - The use of type String allows the use of extension tokens without the need to open the Package. |
|  | Default: | NPT |

##### 14.3.1.1.6	Data speed

|  | Parameter Name: | Data Speed |
| --- | --- | --- |
|  | Parameter ID: | ds (0x0006) |
|  | Description: | This parameter allows the MGC to instruct the MG to deliver the media resource at an adjusted data speed. The bandwidth required to deliver the media resource may need to be altered. For more information on data delivery speed see clause 18.50/[IETF RFC RTSP2]. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | Speed = lower-bound MINUS upper-bound The ABNF syntax is defined in clause 20.2.3/[IETF RFC RTSP2]. NOTE - The above syntax omits "Range" HCOLON as it is unnecessary for H.248 encoding. |
|  | Default: | 1-1 |

## 14.4	Statistics

None

## 14.5	Error codes

### 14.5.1	Unable to adjust the data delivery speed of the signal

Error Code #: 476

Name: Unable to adjust the data delivery speed of the signal

Definition: The MG has been unable to adjust the data delivery speed of the signal referenced by the Speed signal of the Data Delivery Speed Adjustment package.

Error Text in the Error Descriptor: The signal identity of the applicable signal encoded as per clause 14.3.1.1.1.

Comment: The signal referenced by the Speed signal continues to play out.

## 14.6	Procedures

### 14.6.1	Adjusting data delivery speed

A MGC may be requested by a client entity to adjust the data delivery speed (i.e. delivery bandwidth) of a playing media resource associated with a particular signal.

The MGC may audit packages to determine if data delivery speed adjustment is supported by the MG.

If the MGC initiates the playout of a new signal with an altered data delivery speed, it shall use an appropriate Play signal (e.g. rtspp/play) that contains a Data Speed parameter. Likewise if the MGC requires a range change and a data delivery speed adjustment is shall use an appropriate Play signal.

An MGC shall use the Data Delivery Speed Adjustment (ddsa) package when it wants to adjust the data delivery speed of a now playing signal.

In order to change the data delivery speed of certain Signal, the MGC shall send the Speed signal of Data Delivery Speed Adjustment package (ddsa/spd) to the MG. The MGC shall include existing Signals/SignalLists as well as the ddsa/spd signal in the Signal Descriptor in order to maintain the playout (albeit with an altered data delivery speed) of the Signal. This makes use of the H.248 capability where multiple Signals are played simultaneously (see clause 7.1.11/[ITU-T H.248.1]).

If the ddsa/spd signal is set by the MGC at a Termination or Stream level without Signal Identity (sigid), Signal List Identity (listid) or Media Resource Identifier (mri) parameter then the MG shall alter the data delivery speed of all signals on that Termination or Stream.

If the ddsa/spd signal is set by the MGC at a Termination or Stream level with the sigid or listid parameter then the MG shall alter the data delivery speed of the signal indicated by the sigid parameter or the entire signal list indicated by the listid parameter on that Termination or Stream.

If the ddsa/spd signal is set by the MGC with the mri parameter then the MG shall alter the data delivery speed of the signal identified or the signal(s) associated with the media resource.

If the ddsa/spd signal is set by the MGC with the mri parameter and the sigid or listid parameter then the MG shall alter the data delivery speed of the signal(s) associated with the media resource and indicated by the sigid or listid parameter.

If the MG detects a mismatch between the sigid/listid parameter and the Signal/SignalList playout on that Termination/Stream or associated with the media resource indicated by the mri parameter then it should return error code 473 "Conflicting Property Values".

NOTE 1 - In some cases there may be more than one instance of a Signal Identity or Signal List Identity for a Termination/Stream in these cases the MGC should use the mri parameter to uniquely identify the signal instance.

NOTE 2 - A MGC may determine that Media Resource Resource syntax that is supported by a MG/Termination through auditing packages. If the "RTSP Media Resource Syntax (mrs)" package is supported the MG can assume that the RTSP URI/IRI syntax may be used to describe a media resource. If the "Basic Announcement Syntax (bannsyx)" Package is supported announcement syntax may be also used to describe a media resource.

The MG shall alter the data delivery speed according to the Data Speed (ds) parameter from the MGC, and may respond with the actual data delivery speed it is using via the ds parameter to the MGC. See clause 18.50/[IETF RFC RTSP2] for an example of how Speed adjustment is used. If the MG is unable to support the ds parameter value provided by the MGC, it may return error code 449 "Unsupported or Unknown Parameter or Property Value" or in the case the MG supports an alternate value it may return this value via the ds parameter in the response.

The MGC may overwrite the ddsa/spd signal with an updated ds parameter in which case the MG shall apply the new data delivery speed.

If the MG is unable to adjust the data delivery speed indicated by the ddsa/spd signal then it should respond with error code 476 "Unable to adjust the data delivery speed of signal". The referenced signal should continue to play out in a current manner.

### 14.6.2	Requesting range information

An MGC may be requested to provide range information associated with the data speed adjusted signal playout. To request the range information the MGC should do a "CHOOSE ($)" wildcard on the Range (rng) parameter. The MG shall then return the time range information in the format indicated by the Range Format (rf) parameter. See clause 13.4/[IETF RFC RTSP2] for an example use of range information. If multiple media resources/streams are in use, this may lead to multiple range information being returned. If the ddsa/spd signal affects multiple media resources and the rng parameter is requested then the MG should return the common range information for all the media resource. If the MG is unable to determine which rng parameter values should be returned, it should issue error code 473 "Conflicting Property Values".

### 14.6.3	Normal data delivery

When the MGC wants to revert back to the normal data delivery speed, it should remove the relevant instance of the ddsa/spd signal from the Signal Descriptor. The MG shall then continue to play the signal(s) related to the removed ddsa/spd at a normal data delivery speed.

# 15	Playback relative scale adjustment package

|  | Package Name: | Playback Relative Scale Adjustment |
| --- | --- | --- |
|  | Package ID: | prsa (0x00dd) |
|  | Description: | This package allows the MGC to request the MG to alter the playback relative scale of a particular signal. "Relative Scale" is otherwise known as "play speed". This is a generic package that may be applied to a diverse range of signals related to media playout. |
|  | Version: | 1 |
|  | Extends: | None |

## 15.1	Properties

None

## 15.2	Events

None

## 15.3	Signals

### 15.3.1	Scale

|  | Signal Name: | Scale |
| --- | --- | --- |
|  | Signal ID: | scl (0x0001) |
|  | Description: | This signal allows the MGC to alter the playback relative scale of a particular signal. |
|  | SignalType: | On / Off |
|  | Duration: | Not applicable to on / off signals |

#### 15.3.1.1	Additional parameters

##### 15.3.1.1.1	Signal identity

|  | Parameter Name: | Signal Identity |
| --- | --- | --- |
|  | Parameter ID: | sigid (0x0001) |
|  | Description: | The Signal Identity parameter allows a MGC to alter the playback relative scale of a Signal with the indicated signal identity. |
|  | Type: | Octet String |
|  | Optional: | Yes |
|  | Possible values: | For text encoding a Package Name / Signal ID encoded as per pkgdName in Annex B.2/[ITU-T H.248.1]. For binary encoding a Package Name / Signal ID encoded as per SignalName in Annex A.2/[ ITU-T H.248.1]. |
|  | Default: | None |

##### 15.3.1.1.2	Signal list identity

|  | Parameter Name: | Signal List Identity |
| --- | --- | --- |
|  | Parameter ID: | listid (0x0002) |
|  | Description: | The Signal List Identity parameter allows a MGC to alter the playback relative scale of a Signal List with the indicated identity. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 0 to 65535 |
|  | Default: | None |

##### 15.3.1.1.3	Media resource identifier

|  | Parameter Name: | Media Resource Identifier |
| --- | --- | --- |
|  | Parameter ID: | mri (0x0003) |
|  | Description: | The Media Resource Identifier parameter allows the MGC to indicate the media resource whose playback relative scale should be altered. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per RTSPMediaResourceSpec of RTSP Media Resource Syntax (mrs) package. Optionally AnnouncementSpec in clause 6.2.5/[ ITU-T H.248.9] may be supported in addition. |
|  | Default: | None |

##### 15.3.1.1.4	Range

|  | Parameter Name: | Range |
| --- | --- | --- |
|  | Parameter ID: | rng (0x0004) |
|  | Description: | This parameter is used to indicate the adjusting range information of the media resource associated with the signal. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | Range = ranges-spec The ABNF syntax is defined in clause 20.2.3/[IETF RFC RTSP2]. NOTE - The above syntax omits "Range" HCOLON as it is unnecessary for H.248 encoding. The syntax also allows the use of a "Quoted String" form. If this is used the double quote character should be represented by \%x22. |
|  | Default: | None |

##### 15.3.1.1.5	Range format

|  | Parameter Name: | Range Format |
| --- | --- | --- |
|  | Parameter ID: | rf (0x0005) |
|  | Description: | This parameter indicates the range information format that shall be used with this signal. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | As per the Format Type property of the Range Format Support package (rfs/ft). NOTE - The use of type String allows the use of extension tokens without the need to open the Package. |
|  | Default: | NPT |

##### 15.3.1.1.6	Playback scale

|  | Parameter Name: | Playback Scale |
| --- | --- | --- |
|  | Parameter ID: | ps (0x0006) |
|  | Description: | This parameter allows the MGC to instruct the MG to deliver the media resource at an adjusted playback relative scale. The playback relative scale of a signal is specifiable as a positive (faster) or negative (slower) variation from the normal playback speed. For more information on playback scale see clause 18.44/[IETF RFC RTSP2]. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | Playback Scale = FLOAT POS-FLOAT = 1*12DIGIT ["." 1*9DIGIT] FLOAT = ["-"] POS-FLOAT The ABNF syntax is defined in clause 20.2.3/[IETF RFC RTSP2].A ratio of the normal playback speed. |
|  | Default: | 1 |

##### 15.3.1.1.7	Playback scale delta

|  | Parameter Name: | Playback Scale Delta |
| --- | --- | --- |
|  | Parameter ID: | psd (0x0007) |
|  | Description: | As per the Playback Scale parameter this parameters allows the MGC to instruct the MG to deliver the media resource at an adjusted playback relative scale. The scale delta provided by this parameter is relative to the current playback speed. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | Playback Scale = FLOAT POS-FLOAT = 1*12DIGIT ["." 1*9DIGIT] FLOAT = ["-"] POS-FLOAT The ABNF syntax is defined in clause 20.2.3/[IETF RFC RTSP2]. A ratio of the current playback speed. |
|  | Default: | 1 |

##### 15.3.1.1.8	Restart

|  | Parameter Name: | Restart |
| --- | --- | --- |
|  | Parameter ID: | rs (0x0008) |
|  | Description: | This parameter can be used to indicate the behaviour when the playout of a rewind operation reaches the beginning of the media or a fast forward operation reaches the end of the media. |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | ST (0x01): indicates to the MG to stop playing media. This will result in Scale signal completion. RS (0x02): indicates to the MG to play the media from the beginning LB (0x03): indicates the MG to loop back to the end of the media and go on rewinding, or in the case of fast forward loop back to the start of the media and go on fast forwarding. |
|  | Default: | ST (0x01) |

## 15.4	Statistics

None

## 15.5	Error codes

### 15.5.1	Unable to adjust the playback relative scale of the signal

|  | Error Code #: | 477 |  |
| --- | --- | --- | --- |
|  | Name: | Unable to adjust the playback relative scale of the signal |  |
|  | Definition: | The MG has been unable to adjust the playback relative scale of the signal referenced by the Scale signal of the Playback Relative Scale Adjustment package. |  |
|  | Error Text in the Error Descriptor: |  | The signal identity of the applicable signal encoded as per clause 15.3.1.1.1. |
|  | Comment: | The signal referenced by the Scale signal continues to play out. |  |

## 15.6	Procedures

### 15.6.1	Adjust playback relative scale

A MGC may be requested by a client entity to adjust the playback relative scale (i.e. play speed) of a playing media resource associated with a particular signal.

The MGC may audit packages to determine if playback relative scale adjustment is supported by the MG.

If the MGC initiates the playout of a new signal with an altered playback relative scale, it shall use an appropriate Play signal (e.g. rtspp/play) that contains a Playback Scale parameter. Likewise if the MGC requires a range change and a playback relative scale adjustment is shall use an appropriate Play signal.

An MGC shall use the Playback Relative Scale Adjustment (prsa) package when it wants to adjust the playback relative scale of a now playing signal.

In order to change the playback relative scale of a certain Signal the MGC shall send the Scale signal of the Playback Relative Scale Adjustment package (prsa/scl) to the MG. The MGC shall include existing Signals/SignalLists as well as the prsa/scl signal in the Signal Descriptor in order to maintain the playout (albeit with an altered playback relative scale) of the Signal. This makes use of the H.248 capability where multiple Signals are played simultaneously (see clause 7.1.11/[ ITU-T H.248.1]).

If the prsa/scl signal is set by the MGC at a Termination or Stream level without Signal Identity (sigid), Signal List Identity (listid) or Media Resource Identifier (mri) parameter then the MG shall alter the playback relative scale of all signals on that Termination or Stream.

If the prsa/scl signal is set by the MGC at a Termination or Stream level with the sigid or listid parameter then the MG shall alter the playback relative scale of the signal indicated by the sigid parameter or the entire signal list indicated by the listid parameter on that Termination or Stream.

If the prsa/scl signal is set by the MGC with the mri parameter then the MG shall alter the playback relative scale of the signal identified or the signal(s) associated with the media resource.

If the prsa/scl signal is set by the MGC with the mri parameter and the sigid or listid parameter then the MG shall alter the playback relative scale of the signal(s) associated with the media resource and indicated by the sigid or listid parameter.

If the MG detects a mismatch between the sigid/listid parameter and the Signal/SignalList playout on that Termination/Stream or associated with the media resource indicated by the mri parameter then it should return error code 473 "Conflicting Property Values".

NOTE 1 - In some cases there may be more than one instance of a Signal Identity or Signal List Identity for a Termination/Stream in these cases the MGC should use the mri parameter to uniquely identify the signal instance.

NOTE 2 - A MGC may determine that Media Resource Resource syntax that is supported by a MG/Termination through auditing packages. If the "RTSP Media Resource Syntax (mrs)" package is supported the MG can assume that the RTSP URI/IRI syntax may be used to describe a media resource. If the "Basic Announcement Syntax (bannsyx)" Package is supported announcement syntax may be also used to describe a media resource.

The MG shall alter the playback relative scale according to the Playback Scale (ps) or the Playback Scale Delta (psd) parameter from the MGC, and may respond with the actual playback relative scale it is using via the ps or psd parameter to the MGC. The MGC shall not use both parameters in a signal. See clause 18.46/[IETF RFC RTSP2] for an example of how Scale adjustment is used. If the MG is unable to support the ps parameter value provided by the MGC, it may return error code 449 "Unsupported or Unknown Parameter or Property Value" or in the case the MG supports an alternate value it may return this value via the ps parameter in the response.

When the MGC instructs the MG to perform a rewind or fast forward operation it may send the Restart (rs) parameter to specify the restart behaviour. If the playback reaches the very beginning or end of the media stream, the MG will deal with the media stream depending on the indication of the rs parameter.

NOTE - A rewind operation is indicated by sending the Scale signal with a negative value for the Playback Scale (ps) or Playback Scale Delta (psd) parameter.

The MGC may overwrite the prsa/scl signal with an updated ps parameter in which case the MG shall apply the new playback relative scale.

If the MG is unable to adjust the playback relative scale indicated by the prsa/scl signal then it should respond with error code 477 "Unable to adjust the playback relative scale of the signal". The referenced signal should continue to play out in the current manner.

### 15.6.2	Requesting range information

The procedures of clause 14.6.2 apply with ddsa/spd changed to prsa/scl.

### 15.6.3	Normal data delivery

The procedures of clause 14.6.3 apply with ddsa/spd changed to prsa/scl.

# 16	RTP information package

|  | Package Name: | RTP Information |
| --- | --- | --- |
|  | Package ID: | rtpinfo (0x00de) |
|  | Description: | This package allows the MGC to request information regarding an RTP stream from a MG. |
|  | Version: | 1 |
|  | Extends: | None |

## 16.1	Properties

### 16.1.1	Synchronization source

|  | Property Name: | Synchronization Source |
| --- | --- | --- |
|  | Property ID: | ssrc (0x0001) |
|  | Description: | This property allows the MGC to request the synchronisation source that applies to the RTP stream. See "Synchronisation Source" clause 5.1/[IETF RFC 3550]. |
|  | Type: | String |
|  | Possible values: | ssrc = 8 * HEXDIG Encoded as per Annex B.2/[ITU-T H.248.1]. |
|  | Default: | None |
|  | Defined in: | LocalControl |
|  | Characteristics: | ReadOnly |

### 16.1.2	Sequence number

|  | Property Name: | Sequence Number |
| --- | --- | --- |
|  | Property ID: | seq (0x0002) |
|  | Description: | This property allows the MGC to request the sequence number of the first packet that applies to the RTP stream. See "Sequence Number" clause 5.1/[IETF RFC 3550] for more information. |
|  | Type: | Integer |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |
|  | Defined in: | LocalControl |
|  | Characteristics: | ReadOnly |

### 16.1.3	RTP timestamp

|  | Property Name: | RTP Timestamp |
| --- | --- | --- |
|  | Property ID: | ts (0x0003) |
|  | Description: | This property allows the MGC to request the RTP timestamp value corresponding to the start time that applies to the RTP stream. See "Timestamp" clause 5.1/[IETF RFC 3550] for more information. |
|  | Type: | String |
|  | Possible values: | Unsigned Integer |
|  | Default: | None |
|  | Defined in: | LocalControl |
|  | Characteristics: | ReadOnly |

## 16.2	Events

None

## 16.3	Signals

None

## 16.4	Statistics

None

## 16.5	Error codes

None

## 16.6	Procedures

An MGC may be required to provide RTP related information regarding a particular media stream to a client. A MGC may request the following information by issuing a "CHOOSE ($)" wildcard on the relevant property on the relevant stream:

- Synchronisation Source (rtpinfo/ssrc)

- Sequence Number (rtpinfo/seq)

- Timestamp (rtpinfo/ts)

The information returned by the MG as a result is as per clause 18.45/[IETF RFC RTSP2]. If the MG does not have sufficient information regarding the details of the stream (i.e. lacking information in the Local and Remote descriptors), the MG may return error code 472 "Required Information Missing".

If the MGC requires that the RTP information be scoped against a particular media resource (e.g. URI) then it should set the Specification property of the Media Resource Identifier package (mri/spec) to that particular media resource. The MG shall then use this media resource as an input to provide the relevant RTP information.

# 17	RTP interleaving package

|  | Package Name: | RTP Interleaving |
| --- | --- | --- |
|  | Package ID: | rtpint (0x00df) |
|  | Description: | This package allows the MGC to request that media stream packets be routed through the MGC instead of being transported directly between the MG and a remote endpoint of a particular media connection. |
|  | Version: | 1 |
|  | Extends: | None |

## 17.1	Properties

None

## 17.2	Events

### 17.2.1	Outgoing data

|  | Event Name: | Outgoing Data |
| --- | --- | --- |
|  | Event ID: | od (0x0001) |
|  | Description: | This event indicates to the MG that outgoing media stream packets should be sent to the MGC rather than sent directly to the remote endpoint of a particular media connection. |

#### 17.2.1.1	EventsDescriptor parameters

##### 17.2.1.1.1	Channel number

|  | Parameter Name: | Channel Number |
| --- | --- | --- |
|  | Parameter ID: | cn (0x0001) |
|  | Description: | This parameter indicates the Channel Number/s that should be used when generating the interleaved data blocks. See "Interleaved" in clause 18.54/[IETF RFC RTSP2]. |
|  | Type: | String |
|  | Optional: | No |
|  | Possible values: | cn = channel [ "-" channel ] channel = 1*3DIGIT; 0 to 255 |
|  | Default: | None |

#### 17.2.1.2	ObservedEventsDescriptor parameters

##### 17.2.1.2.1	Data block

|  | Parameter Name: |  | Data Block |
| --- | --- | --- | --- |
|  | Parameter ID: |  | db (0x0001) |
|  | Description: |  | This parameter contains the data block sent from the MG to the MGC. |
|  | Type: |  | Octet String |
|  | Optional: |  | No |
|  | Possible values: | The octet string is first encoded as per clause 14/[IETF RFC RTSP2], i.e. 0 1 2 3 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ "$" = 24 ¦ Channel ID ¦ Length in bytes ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ : Length number of bytes of binary data : +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ For Text Encoded H.248, clause B.3/[ITU-T H.248.1] is then applied. |  |
|  | Default: |  | None |

##### 17.2.1.2.2	Channel number

|  | Parameter Name: | Channel Number |
| --- | --- | --- |
|  | Parameter ID: | cn (0x0002) |
|  | Description: | This parameter indicates the Channel Number/s that actually be used when generating the interleaved data blocks. See "Interleaved" in clause 18.54/[IETF RFC RTSP2]. |
|  | Type: | String |
|  | Optional: | Yes |
|  | Possible values: | cn = channel [ "-" channel ] channel = 1*3DIGIT; 0 to 255 |
|  | Default: | None |

## 17.3	Signals

### 17.3.1	Incoming data

|  | Signal Name: | Incoming Data |
| --- | --- | --- |
|  | Signal ID: | id (0x0001) |
|  | Description: | This signal indicates to the MG that incoming media stream packets should be received via the MGC rather than received directly from the local endpoint of a particular media connection. |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable to Brief signals |

#### 17.3.1.1	Additional parameters

##### 17.3.1.1.1	Data block

|  | Parameter Name: | Data Block |
| --- | --- | --- |
|  | Parameter ID: | db (0x0001) |
|  | Description: | This parameter contains the data block received via the MGC to the MG. |
|  | Type: | Octet String |
|  | Optional: | No |
|  | Possible values: | The octet string is first encoded as per clause 14/[IETF RFC RTSP2]. See 17.2.1.2.1 for an example. For Text Encoded H.248, clause B.3/[ITU-T H.248.1] is then applied. |
|  | Default: | None |

##### 17.3.1.1.2	Channel number

|  | Parameter Name: | Channel Number |
| --- | --- | --- |
|  | Parameter ID: | cn (0x0002) |
|  | Description: | This parameter indicates the Channel Number/s that should be used when parsing the interleaved data blocks. See "Interleaved" in clause 18.54/[IETF RFC RTSP2]. |
|  | Type: | String |
|  | Optional: | No |
|  | Possible values: | cn = channel [ "-" channel ] channel = 1*3DIGIT; 0 to 255 |
|  | Default: | None |

## 17.4	Statistics

None

## 17.5	Error codes

None

## 17.6	Procedures

Using H.248 to transport Media Stream Data via an MGC with the peer endpoint of a media connection incurs considerable processing overhead. Therefore it is not recommended that this method be used. A MGC may audit packages in order to determine if a MG supports this functionality.

When the MGC needs to indicate the MG that the a media stream (e.g. RTP data) should be embedded into the control stream (i.e. H.248 message) for transport, it shall set the Outgoing Data event of RTP Interleaving package (rtpint/od) and the Incoming Data signal of RTP Interleaving package (rtpint/id) respectively for the media stream data outgoing from and incoming to the MG.

When the MGC has determined that media stream data outgoing from a Termination of the MG should be sent to the MGC rather than sent directly to the remote end of the media connection, it shall set the rtpint/od event on the Termination. If there are multiple streams associated with the Termination, the MGC shall set the event with the StreamID of the stream that the embedding applies to. The MGC shall also set the Channel Number (cn) parameter to indicate the channels number that should be used by the MG in the generating/encoding the interleaved data block.

On reception of the rtpint/od event the MG shall send all related outgoing media stream data (i.e. RTP and RTCP packets) to the MGC via the ObservedEvent instead of sending it directly to the remote endpoint of a particular media connection. The MG shall generate/encode the related outgoing media stream data into the Data Block (db) parameter using the Channel Number indicated by the cn parameter from the MGC according to clause 14/[IETF RFC RTSP2]. The MG shall also return the actual used Channel Number via cn parameter to the MGC if it is different to the one requested by the MGC.

If the rtpint/od event is removed then media stream data sending reverts back to normal.

When the MGC has determined that media stream data incoming to a Termination of the MG should be received via the MGC rather than received directly from the local endpoint of a particular media connection, it should send the rtpint/id signal to the Termination. Where there are multiple streams associated with the Termination, the MGC shall set the signal with the StreamID of the stream that the embedding applies to. The MGC shall use the Data Block (db) parameter to contain the related incoming media stream data, and if necessary the Channel Number (cn) parameter to indicate the channels number should be used by the MG in the parsing/decoding the interleaved data block. The MG shall then parse/decode the related incoming media stream data with the Channel Number indicated by the cn parameter form the MGC according to clause 14/[IETF RFC RTSP2].

If the rtpint/id signal is removed then media data reception reverts back to normal.

Normally both the incoming and the outgoing media stream data are embedded into the control stream to transport or not. However there may be some exceptions.

Bibliography

[b-IETF RTSPANN]	IETF draft-ietf-mmusic-rtsp-announce-01 (2005), RTSP Announce Method

____________________