| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-30a |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | Q5/16 | Geneva, 10 - 14 March 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Rapporteur Q5/16 |  |  |  |
| Title: | H.TPS-SIG "Signalling for telepresence-enabled conferencing" (New): Output Draft |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the output draft (H.TPS-SIG Ed. 0.2) for H.TPS-SIG "Signalling for telepresence-enabled conferencing" (New).

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.TPS-SIG Ed. 0.1 | Initial baseline text. Output from the Geneva October 2013 meeting. It incorporates aspects of C.301, C.381, and C.420. |
| H.TPS-SIG Ed. 0.2 | Input text to the Geneva Meeting, March 2014. It is a copy of TD-120 from the Geneva, October 2013 meeting with the change marks accepted and editorial cleanup. |
| H.TPS-SIG Ed. 0.3 | Output text to the Geneva Meeting, March 2014. It incorporates: AVD-4559, AVD-4560, AVD-4577, AVD-4578 and AVD-4579. |

Living List Items:

- C.301 proposes a specific approach to telepresence signaling which could be a useful reference.  More contributions on this and other approaches are solicited.  Analysis of the various possible approaches would be helpful for progressing this work item.

- Multiplexing of RTP is an active work item in the IETF, and it is unlikely that the approach taken there will be consistent with existing ITU-T recommendations.  Q5/16 experts agreed in the Geneva October 2013 meeting that multiplexing is likely a more appropriate topic for Q2/16.

- Q5/16 might be able to directly re-use the CLUE messages.  This is by no means certain, but if the binding of CLUE media captures to SDP offers and RTP streams is done through token identifiers, it is possible.  Q5/16 experts agreed in the Geneva October meeting that re-use of the CLUE signalling messages is highly desirable.

- Carriage of H.TPS-AV parameters in SIP systems is desirable.

CONTENTS

Page

1 Scope 4 2 References 4 3 Definitions 5 3.1 Terms defined elsewhere 5 3.2 Terms defined in this Recommendation 5 4 Abbreviations and acronyms 5 5 Conventions 5 6 Framework 6 6.1 Phase A – Call setup 7 6.2 Phase B - Initial communication and capability exchange 7 6.3 Phase C - Establishment of audiovisual communication 7 6.4 Phase D - Call services 8 6.5 Phase E - Call termination 8 7 Signalling support for CLUE 8 7.1 Carriage of H.TPS-AV parameters 8 7.2 Carriage of CLUE messages in H.323 systems 8 7.2.1 CLUE capability negotiation 8 7.2.2 CLUE Messaging 9 7.2.3 Media correlation 10 8 Signalling Procedures for H.245 based systems 11 8.1 Registration 11 8.2 Call session setup 11 8.3 Capability exchange 11 8.4 Media stream transport and control 11 8.5 Open Logical Channel 11 8.6 Call session release 11 8.7 Deregistration 11 9 Multipoint control 11 10 Security 12 11 Interworking 12 Annex A <Annex Title> 12 Appendix I Example Telepresence Signalling Flows 12 I.1 Telepresence session using an multiple stage media establishment and SCTP channel for CLUE messages 12 Bibliography 14

List of Tables

Page

No table of figures entries found.

List of Figures

Page

No table of figures entries found.

Draft new ITU-T H.TPS-SIG

Signalling for telepresence-enabled conferencing

AAP Summary

[To be provided before Consent]

Summary

This contribution describes signalling for telepresence-enabled conferencing.

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# Scope

This Recommendation describes the signalling procedures which support telepresence in conferencing systems. It covers signalling related to point-to-point and multipoint call processes in H.245 based telepresence systems, including registration, call session establishment, capability exchange, and negotiation of media stream transport.  It also covers signaling of H.TPS-AV parameters in both SIP and H.323 based systems.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.TPS-Reqs]	Recommendation ITU-T F.yyy (date), Definitions, Requirements, and use cases for Telepresence Systems.

[F/H.TPS-Arch]	Recommendation ITU-T F/H.TPS-Arch (date), Telepresence System Architecture.

[H.TPS-AV]	Recommendation ITU-T H.yyy (date), Audio/Video Parameters for Telepresence systems.

[ITU-T H.323]	Recommendation ITU-T H.323 v7 (12/2009), Packet-based multimedia communications systems.

[ITU-T H.245]	Recommendation ITU-T H.245 v16 (05/2011), Control protocol for multimedia communication.

[ITU-T H.225.0]	Recommendation ITU-T H.225.0 v7 (12/2009), Call signalling protocols and media stream packetization for packet-based multimedia communication systems.

[ITU-T H.460.1]	Recommendation ITU-T H.460.1 (03/2002), Guidelines for the use of the generic extensible framework.

[IETF CLUE DM]	IETF draft-ietf-clue-data-model-schema-03, An XML Schema for the CLUE data model.

[IETF CLUE FW]	IETF draft-ietf-clue-framework-14, Framework for  Telepresence Multi-Streams.

[IETF CLUE SIG] 	IETF draft-kyzivat-clue-signaling-07, CLUE Signaling.

# Definitions

<Check in the ITU-T Terms and definitions database on the public website whether the term is already defined in another Recommendation. It may be more consistent to refer to such a definition rather than redefine it>

## Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	Capture [IETF CLUE FW]: <optional quoted definition>

3.1.2	Capture Encoding [IETF CLUE FW]: <optional quoted definition>

## Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| Ack | Acknowledgement |
| --- | --- |
| CLUE | ControLling mUltiple streams for tElepresence |
| MCU | Multipoint Control Unit |
| Nack | Negative Acknowledgement |
| OID | Object Identifier |
| PC | Personal Computer |
| RAS | Registration, Admission and Status |
| STS | Simultaneous Transmission Set |
| XML | Extensible Markup Language |

# Conventions

Information elements related to ITU-T H-series Recommendations are indicated in bold.

Information elements related to IETF Recommendations are indicated using italics.

# Framework

A telepresence system is characterised by a set of functions, devices and network elements which are able to capture, deliver, manage and render multiple high quality interactive audio and video signals in a Telepresence conference. An appropriate number of devices (e.g. cameras, screens, loudspeakers, microphones, codecs, MCUs, PCs) and environmental characteristics are used to establish Telepresence. Such functions, elements and characteristics are described in [ITU-T F/H.TPS-Arch]. In order to support a telepresence experience, information related to the functions and characteristics must be negotiated and transmitted between the network elements.

This Recommendation utilises the CLUE framework [IETF CLUE FW] to signal information related multiple media captures in order that the network elements may provide a telepresence experience.

Furthermore additional information is provided by native [ITU-T H.245] signalling that provides further additional enhancements.

{Editor’s note: The above statement is a placeholder. There needs to be further investigation how the additional elements from H.TPS-AV are supported.}

A H.323 telepresence session follows the same basic call signalling procedures as outlined in clause 8/[ITU-T H.323] but with some additional behaviour to negotiate the use of CLUE signalling and to link the CLUE information to H.245 capabilities. These aspects are briefly outlined in the clauses below.

The call flow below shows a high level basic call flow using each of the call phases. Depending on the application, one or more phases may be used multiple times. For example Phase B and C may be used to negotiate a normal H.323 media following by a negotiation of CLUE controlled media.

Figure 1 – Overview of Call Phases

## Phase A – Call setup

RAS and H.225.0 call setup messages (e.g. Setup/Alerting) contain a CLUEControlCapability feature to indicate support of a CLUE signalling. Clause 7.9.4/[ITU-T H.323] provides details of the negotiation procedures. This allows an endpoint to negotiate the support of CLUE signalling for further call phases. If the peer does not support this feature then CLUE will not be used any further. If the peer supports the CLUEControlCapability feature then the use of CLUE will integrated into the Phases B, C, D and E. The phases below assume the support this feature.

## Phase B - Initial communication and capability exchange

In the knowledge that a peer supports CLUE signalling the endpoint issues a H.245 terminalCapabilitySet message with its supported configurations. In addition, in order to link the particular configurations with particularly CLUE capture encodings a CLUE Encoding Correlation containing the relevant encodingID is appended to each configuration.

The endpoint then uses CLUE messages to perform the procedures of [IETF CLUE SIG]. The first part of the exchange is to negotiate the CLUE protocol version and any supported extensions. The second part is the Advertisement of the endpoint capabilities using the CLUE data as outlined in [IETF CLUE DM]. The encodingIDs used in the CLUE Advertisement provide a link to the H.245 capabilities with the same encodingID.

The terminalCapabilitySet configurations need to align with the simultaneous transmission sets (STS) in the CLUE data.

On reception of the CLUE Advertisement the peer endpoint will select which captures and encodings it requires based on the information in the Advertisement message and the capability information from the terminalCapabilitySet message. The peer endpoint will then send a CLUE Configure message indicating which Captures and Encodings that it has selected to receive.

The peer endpoint also replies with a terminalCapabilitySetAck indicating that it has received the terminal capability set configurations.

{Editor’s note: There are two options under study to support the signalling of CLUE messages: H.245 generic messages and/or a SCTP channel.}

## Phase C - Establishment of audiovisual communication

For those H.245 configurations that are not labelled with an encodingID the endpoint may initiate [ITU-T H.245] open logical channel procedures to open media streams on the receipt of a terminalCapabilitySetAck message.

For those configurations that are labelled with an encodingID the endpoint only opens media streams for the configurations that are labelled with encodingIDs received in the CLUE Configure message. The H.245 OpenLogicalChannel message contains the CLUE Correlation capability with the encodingID related to the capture encoding of the media stream. This allows the receiving endpoint to identify which media stream relates to a particular capture description. On receipt of the media stream the receiver endpoint may then apply processing to the media based on the CLUE capture description.

During the session the endpoints may issue updated CLUE Advertisement or Configure messages resulting in messages being sent containing the updated CLUE data. Updated terminalCapabilitySet messages may also be required as a result. This may lead to open logical channel procedures to add, re-configure or release logical channels.

## Phase D - Call services

An endpoint that wishes to change its bandwidth should consider any impacts to the CLUE data that it has signalled. If the bandwidth request requires addition bandwidth to what has been advertised being associated with a particular encodingID this may require updated CLUE messages and terminalCapabilitySet messages.

The CLUEControlCapability may be used in multipoint conferencing scenarios. In these scenarios a multipoint controller (MC) receives the CLUE messages and terminalCapabiltySet messages from multiple endpoints and uses the received information to create its own set of CLUE data which is then sent to the endpoints. As per phase C this information is used to establish the applicable media streams.

## Phase E - Call termination

In addition to the normal call termination procedures, endpoints may indicate that either a capture encoding is no longer available or that it does not wish to receive particular capture encodings any more. This is communicated through the use of CLUE messages and if applicable terminalCapabiltySet messages. This may result in the closure of one or more logical channels without the need to terminate the call.

# Signalling support for CLUE

## Carriage of H.TPS-AV parameters

{Editor’s note: This section shall contain information regarding the carriage of H.TPS-AV parameters in both SIP and H.245-based systems.  This might be accomplished by registering XML parameters defined by Q5/16 with IANA}

## Carriage of CLUE messages in H.323 systems

This section describes the various elements required for the negotiation, correlation and carriage of CLUE information.

### CLUE capability negotiation

This clause describes the methods for negotiating the use of CLUE in H.323 systems.

{Editor’s note: Consideration needs to be given if a method is needed to indicate which CLUE signalling method (H.245 generic message or SCTP) is supported.}

#### H.225 Generic Feature “CLUE control capability”

This clause describes a generic feature “CLUE control capability” utilising the generic extensible framework (GEF) [ITU-T H.460.1] to negotiate the use CLUE protocol messages within H.245.

Table 1 – CLUE control capability

| Feature name | CLUEControlCapability |
| --- | --- |
| Feature Description: | This features indicates the use of CLUE for providing additional information about the advertised media streams. |
| Feature identifier type: | OID |
| Feature identifier value: | {itu-t(0) recommendation(0) h(8) h.tps-sig (?) CLUEControlCapability(1)}. |

In order to negotiate the use of CLUE within [ITU-T H.225.0] an indication needs to be transmitted to the remote endpoint. This capability is sent to the remote end via feature negotiation as indicated by clause 7.9/[ITU-T H.323].

The remote endpoint upon receiving the CLUEControlCapability then decides whether to support CLUE in its capabilities announcement.

#### H.245 CLUE control capability

{Editor’s note: this clause is to define a method/elements for carrying an indication to initiate CLUE messages.}

### CLUE Messaging

### This clause describes the methods for the carriage of CLUE messages within a H.323 system.

### {Editor’s note: The two options below are included below for further study and contribution.}

### SCTP Transport

### {Editor’s note: This section will provide more detail on the information elements used to establish an SCTP association and the negotiation of CLUE for the association. It will also covers issues related to SCTP transport options, security, firewall support etc,}

### H.245 Generic Message Transport

[ITU-T H.245] GenericMessages are used to convey CLUE request and response messages between endpoints.

#### Signalling of CLUE messages

For H.245 each CLUE message shall consist of a GenericRequest or a GenericResponse containing a GenericMessage.messageIdentifier with the OID { itu-t(0) recommendation(0) h(8) CLUESignalling(2) }, and a subMessageIdentifier. The tables below describe the subMessageIdentifier and associated messageContent syntax.

Table 2 describes the CLUE messages subMessageIdentifiers values.

Table 2 – CLUESignalling subMessageIdentifier values

| subMessageIdentifier | Message name | Message type (for H.245) |
| --- | --- | --- |
| 1 | CLUERequest | GenericRequest |
| 2 | CLUEResponse | GenericResponse |

Table 3 describes the messageContent

Table 3 – GenericParameters used in CLUESignalling messageContent sequences

| Parameter identifier | Parameter name | Parameter value | Parameter value type |
| --- | --- | --- | --- |
| 0 | reserved | 0 | unsignedMin |
| 1 | CLUEsyntax | OctetString | As per [IETF CLUE DM] |

The CLUERequest shall contain XML encoded CLUE request messages (Advertisement, Configure, Supported, Required) as per the syntax of [IETF CLUE DM].

The CLUEResponse shall contain XML encoded CLUE response messages (Ack, Nack, Error) as per the syntax of [IETF CLUE DM].

An endpoint utilising CLUE shall use H.245 generic messages for CLUE request and response messages as appropriate to communicate CLUE information. Procedures related to the use of CLUE in different call/session phases are described in clause 8.

{Editor’s note: Signalling in CLUE is still an open in the IETF. Topics such as version and error handling may have an impact on the messages.}

### Media correlation

In order to provide a linkage between CLUE captures and encodings for the resultant media streams a correlation identity is required. CLUE messages carry an “encodingID” attribute (see clause 16.3/[IETF CLUE DATA]) which is linked to a particular Capture for this purpose.

In order to provide a linkage between a CLUE capture and the media in a logical channel the “encodingID” is included in a TerminalCapabilitySet configurations describing the applicable logical channel where the captured media is contained.

#### CLUE Correlation Capability

Table 4 – CLUE Correlation Capability

| Capability name | CLUE Correlation Capability |
| --- | --- |
| Capability class | Control protocol |
| Capability identifier type | Standard |
| Capability identifier value | {itu-t(0) recommendation(0) h(8) h.tps-sig (?) CLUECorrelationCapability(2)}. |
| maxBitRate | This field shall not be included. |
| Collapsing | This field contains the encodingID parameter. |
| nonCollapsing | This field shall not be included. |
| nonCollapsingRaw | This field shall not be included. |
| transport | This field shall not be included. |

Table 5 – Encoding ID parameter

| Parameter name | encodingID |
| --- | --- |
| Parameter description | This parameter allows an endpoint to correlate a terminalCapabilitySet configuration with a particular CLUE encoding. This parameter also allows an endpoint to correlate a logical channel with a particular CLUE capture encoding. It contains the “encodingID” associated with the capture whose contents the logical channel contains. This ensures a linkage between a configured CLUE capture and encoding to the resultant media stream. |
| Parameter identifier value | 1 |
| Parameter status | Mandatory |
| Parameter type | octetString |
| Supersedes | – |

The CLUE Correlation Capability (containing the encodingID parameter) is communicated to the remote endpoint to correlate terminalCapabilitySet configurations for CLUE encodings. It is also used to correlate a particular H.245 logical channel number with a particular capture/encodingID associated via a EncodingID.

# Signalling Procedures for H.245 based systems

{Editor’s note: Many of the procedures will mirror the CLUE signalling draft. This draft is still in a state of flux with several major issues yet to be clarified, e.g. interaction between the CLUE channel and established media, version handling, media correlation etc.}

## Registration

{Editor’s note: The intention of this section is to describe how the support of telepresence may be indicated through RAS}

## Call session setup

{Editor’s note: The intention of this section is to describe how a telepresence enabled conference is set up. E.g. how the CLUE Signalling messages relate in an overall call flow.}

## Capability exchange

{Editor’s note: The intention of this section is to specify telepresence capabilities.}

## Media stream transport and control

{Editor’s note: The intention of this section is to detail the interaction between CLUE signalling messages and logical channels whilst the session has been established. This will cover issues such as protocol state co-ordination etc. Multiplexing of RTP should be handled in a different recommendation}

## Open Logical Channel

{Editor’s note: The intention of this section is to detail the interaction between CLUE signaling and OLC procedures.}

## Call session release

{Editor’s note: The intention of this section is to describe the call release process where CLUE signalling messages are used. i.e. the need of the CLUE state machine to be closed gracefully}

## Deregistration

{Editor’s note: The intention of this section is to describe if there are any consideration with regards to the registration process particular to telepresence sessions.}

# Multipoint control

{Editor’s note: The intention of this section is to describe any behaviour specific to MCUs with regards to telepresence support. CLUE itself being a point to point protocol simplifies protocol handing. The support of multipoint is still an open area of discussion in CLUE, i.e. the method of support of switched and composed captures is still to be agreed.}

# Security

{Editor’s note: The intention of this section is to describe security considerations around the support of telepresence (in particular CLUE signalling messages). The IETF CLUE work assumes the use of a DTLS/SCTP/IP transport. Therefore for CLUE signalling in H.series based conferences to be equivalent in functionality a secure method of signalling H.245 messages is needed.}

# Interworking

{Editor’s note: The intention of this section is to describe considerations / procedures related to interworking H.series and SIP based telepresence session and interworking with non-telepresence enabled endpoints.}

Annex A

<Annex Title>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

Example Telepresence Signalling Flows

(This appendix does not form an integral part of this Recommendation.)

{Editor’s note: This section is expected to capture example calls flows for various signalling procedures in telepresence sessions.}

## I.1	Telepresence session using an multiple stage media establishment and SCTP channel for CLUE messages

The figure 1 shows a call flow between the two H.323-based telepresence endpoints  using a phased media establishment and SCTP transport of CLUE messages.

Figure I.1 Call flow – staged media establishment and CLUE over SCTP

The registration and the call setup procedures follow the H.323 definitions. After these there is two-round signalling exchange to introduce the telepresence support into H.323-based signaling.

The first round exchange uses the H.245-based the terminalCapabilitySet message to exchange the codec information and telepresence indication information and the openLogicalChannel message to open the basic media channel and also the CLUE channel.

The terminalCapabilitySet message is to negotiate the basic media capability. The receive capability in the terminalCapabilitySet message can be used to carry the basic media capability for the basic capability negotiation. The transmit capability in the terminalCapabilitySet message can be used to carry the individual encoding information. The terminalCapabilitySet message also carries the telepresence indication information to indicate that the endpoint is the telepresence endpoint and CLUE-capable. The GenericCapability can be used to carry the indication information.

{Editor’s Notes: the use of the send and receive capabilities in the terminal capability set messge needs to be clarified. The need to define capability sets specific to CLUE need to be further studied.}

The openLogicalChannel and openLogicalChannelACK messages are to open the basic media channel according to the result of the basic media capability negotiation. Whether or not to transmit the basic media, for example audio-only, is optional and implementation dependent. The openLogicalChannel message can be used to open the CLUE channel after the exchange of the telepresence indication information. The CLUE channel should be opened according to the collection of the information like transmission protocol, channel address (IP/port) to create the CLUE channel.

The result of the first round exchange establishes the basic media channel and the CLUE channel.

The second round exchange uses the CLUE messages to negotiate the telepresence capabilities, the H.245 request Mode messages to request the media data that the receiver needs and the H.245 open Logical Channel operations to update the media channels used to transmit the real telepresence media streams selected by the receiver.

The CLUE message is used to negotiate the telepresence capabilities via the established CLUE channel. How to use the CLUE message should follow the CLUE protocol defined by IETF.

The H.245 request Mode messages are used to request the media data the receiver would like to receive offered in the transmit capabilities from the transmitter.

Once again the H.245 open logical channel operations are used to open the media logical channels bidirectionally according to the result of the second round negotiation. Then the telepresence media streams can be transmitted through these media logical channels.

Bibliography

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title

____________________