| Question(s): | 1, 2, 3, 5 |  | Meeting, date: |  | Geneva, 10-14 March 2014 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: |  | 1 |  |  |
| Source: | Rapporteur Q3/16 |  |  |  |  |  |
| Title: | LS/ r on Q.3910 “Parameters for monitoring NGN protocols” |  |  |  |  |  |
| LIAISON STATEMENT |  |  |  |  |  |  |
| For action to: |  | ITU-T Q13/11 |  |  |  |  |
| For comment to: |  |  |  |  |  |  |
| For information to: |  |  |  |  |  |  |
| Approval: |  | Rapporteur meeting of Question 1, 2, 3 and 5 /16 (Geneva, 10 -14 March) |  |  |  |  |
| Deadline: |  | N/A |  |  |  |  |
| Contact: | Christian Groves Huawei Australia |  |  |  |  | Tel: +61 3 9391 3457 Fax: Email: christian.groves@nteczone.com |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

ITU-T Q3/16 would like to thank Q13/11 for their reply to our liaison on ITU-T Q.3910. Q3/16 has considered the answers and has some additional comments and questions. These are indicated (in brown) below.

Q3/16 is appreciative for your cooperation.

…

# 7	RTP/RTCP monitoring and measurement parameters

## 7.1	General

RTP/RTCP performance, availability and utilization mostly depend on the IP network. RTP/RTCP node status, faults and quality, availability, and utilization indicators are detailed in Tables 1, 2, 3 and 4, respectively.

The 'Usage' column indicates the categories applicable to each parameter.

The 'Interval' column indicates the duration of each parameter.

The 'Units' column is used to indicate the registration items applied at the interval.

The 'Mandatory' column indicates those parameters which must be provided.

[…]

Comment [Q.3/16]:

It looks like that the RTP/RTCP node in Q.3910 relates to an RTP endsystem topology (RFC 5117) located in user equipment only. Is this correct?

RTP/RTCP may be also processed (e.g. terminated) in an H.248 MG (see also H.248.RTPTOPO). There seems to be then a clash between the two Q.3910 node concepts of RTP/RTCP node and H.248 node.

[End comment.]

Answer；No, RTP/RTCP node includes not only user equipment but also all node RTP across. Other word can be considered to make it clearer.

Reply (Q.3/16): 
1) Terminology: There is no existing definition for RTP/RTCP node! Q.3910 should refer to RFC5117 and replace RTP/RTCP node by the considered RTP topologies.
2) H.248 references: SG16 has published the following Q.3910 related Recommendations, which both seem to be relevant:

| H.248.87 | Guidelines on the use of ITU-T H.248 capabilities for performance monitoring in RTP networks in ITU-T H.248 Profiles |
| --- | --- |
| H.248.88 | RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations |

…

# 9	H.248/MEGACO monitoring and measurement parameters

## 9.2	Parameters for H.248/MEGACO node status

Comment [Q.3/16]:

Node related tables: Should there be individual MGC and MG tables?
Destination node? I.e., there’s also a source node?

[End comment.]

Answer: there is no difference between source and destination, so the table is only of the node. 
Reply (Q.3/16): Has the text of Q.3910 been updated accordingly?

The following parameters give the most probable H.248/MEGACO failure reasons. In some cases, other reasons might apply. The comparison of several parameters might give additional information.

–	Item 9.1 indicates the duration of unavailability of the destination node.

Comment [Q.3/16]:

What are the semantics of “unavailability”? 
E.g., related to the H.248 ServiceChange state (“Out of Service”) or/and an X.731 OAM state?

[End comment.]

Answer: related to the H.248 ServiceChange state 
Reply (Q.3/16): Has the text of Q.3910 been updated accordingly?

–	Item 9.4 indicates the total number of messages discarded due to the transmission error.

Comment [Q.3/16]:

Is the “messages discarded” from an MGC or MG perspective? Is there any relation to a H.248.8 error code?
It is assumed that “Message” equals a “H.248 message”?

[End comment.]

Answer: we only consider the number of loss without error code.

yes. it is“H.248 message”

Reply (Q.3/16): Note- H.248, - as every other signalling protocol -, relies on an assured transport service. Thus, any lost H.248 message would be retransmitted again.

Table 9 – H.248/MEGACO node status parameters

| Item | Description of parameters | Units | Usage | Interval | Mandatory |
| --- | --- | --- | --- | --- | --- |
| 9.1 | Duration of the destination node's unavailability | second/node | F, R, N | 60 min | Yes |
| 9.2 | Start of 9.1 | event/node | F, R | On occurrence | Yes |
| 9.3 | End of 9.1 | event/node | F, R | On occurrence | Yes |
| 9.4 | Total number of messages discarded due to the transmission error | message/node | N, F | 60 min | Yes |

## 9.3	Parameters for H.248/MEGACO node availability

–	Item 10.1 indicates the H.248/MEGACO node availability rate.

–	Items 10.2 and 10.3 indicate the duration of availability and unavailability of the interfaces.

–	Items 10.4 and 10.5 indicate the total number of bytes the interface received and sent.

Table 10 – H.248/MEGACO availability parameters

| Item | Description of parameters | Units | Usage | Interval | Mandatory |
| --- | --- | --- | --- | --- | --- |
| 10.1 | Local MEGACO availability rate | 1/node | T, F, N | 60 min | No |
| 10.2 | Duration of local availability | second/node | T, N | 60 min | No |
| 10.3 | Duration of local unavailability | second/node | F, R | 60 min | Yes |
| 10.4 | Total number of bytes received | bytes/node | T, N | 30 min | Yes |
| 10.5 | Total number of bytes sent | bytes/node | T, N | 30 min | Yes |
| 10.6 | Start of local unavailability | event/node | F, R | On occurrence | Yes |
| 10.7 | End of local unavailability | event/node | F, R | On occurrence | Yes |

Comment [Q.3/16]:

Can you provide more detail on the concept of local availability from a MGC/MG/H.248 perspective?

[End comment.]

Answer: “availability” means the node can deal with H.248 messages.
Reply (Q.3/16): Thus, "Local MEGACO availability" should be replaced by e.g "H.248 node availability" and the definition of "availability" referred to:(http://www.itu.int/ITU-R/index.asp?redirect=true&category=information&link=terminology-database&lang=en&adsearch=&SearchTerminology=&sector=&language=all&part=abbreviationterm&kind=anywhere ).

## 9.4	Parameters for H.248/MEGACO Control Association utilization

Comment [Q.3/16]:

H.248 defines a hierarchical PDU concept of Message, Transaction, Action and Command. It is unclear whether this clause relates to the H.248 Message or H.248 Transaction level?
[The next comments supposes the Transaction level.]

[End comment.]

Answer: yes 
Reply (Q.3/16): Has the text of Q.3910 been updated accordingly?

–	Items 11.1 and 11.2 indicate the total number of pending messages sent and received.

Comment [Q.3/16]:

“pending message” = H.248 TransactionPending unit?

[End comment.]

Answer: yes 
Reply (Q.3/16): Has the text of Q.3910 been updated accordingly?

–	Items 11.3 and 11.4 indicate the total number of requests sent and received.

Comment [Q.3/16]:

“request” = H.248 TransactionRequest unit?

[End comment.]

Answer: yes 
Reply (Q.3/16): Has the text of Q.3910 been updated accordingly?

–	Items 11.5 and 11.6 indicate the total number of replies sent and received.

Comment [Q.3/16]:

“reply” = H.248 TransactionReply unit?

[End comment.]

Answer: yes 
Reply (Q.3/16): Has the text of Q.3910 been updated accordingly?

–	Item 11.7 indicates the total number of responses not received. This can be calculated from Items 11.1, 11.2 and 11.6.

Table 11 – H.248/MEGACO utilization parameters

| Item | Description of parameters | Units | Usage | Interval | Mandatory |
| --- | --- | --- | --- | --- | --- |
| 11.1 | Total number of pending sent | message/node | T, N | 15 min | Yes |
| 11.2 | Total number of pending received | message/node | T, N | 15 min | Yes |
| 11.3 | Total number of requests sent | message/node | T, N | 15 min | Yes |
| 11.4 | Total number of requests received | message/node | T, N | 15 min | No |
| 11.5 | Total number of replies sent | message/node | T, N | 15 min | No |
| 11.6 | Total number of replies received | message/node | T, N | 15 min | Yes |
| 11.7 | Total number of requests that did not receive responses | message/node | F, R | 15 min | Yes |

## 9.5	Parameters for the quality of the H.248/MEGACO service

Comment [Q.3/16]:

This clause makes the assumption that all H.248 request/reply signalling cycles are initiated by the MGC (due to master-slave relationship).

However, there are also “H.248 services” which are triggered by the MG (e.g., related to Notification and some ServiceChange procedures).

[End comment.]

Answer: yes. Message from MG TO MGC can be statistic. 
Reply (Q.3/16): See H.248.1 Appendix IV concerning the various options on how statistics could be accessed. An H.248.47-based reporting via Notification is just one option. The above comment was related to MG driven ServiceChange procedures and event-notifications in general.
Has the text of Q.3910 been updated to consider this?

–	Item 12.1 indicates the total number of MGC messages sent.

–	Item 12.2 indicates the total number of MG messages received.

–	Items 12.3 and 12.4 indicate the total number of requests sent and received.

–	Items 12.5 and 12.6 indicate the one way delay of requests and replies.

Table 12 – Quality of H.248/MEGACO service parameters

| Item | Description of parameters | Units | Usage | Interval | Mandatory |
| --- | --- | --- | --- | --- | --- |
| 12.1 | Total number of MGC messages sent | message/node | T, N | 30 min | Yes |
| 12.2 | Total number of MG messages received | message/node | T, N | 30 min | Yes |
| 12.3 | Total number of requests sent | message/node | T, N | 30 min | Yes |
| 12.4 | Total number of requests received | message/node | T, N | 30 min | Yes |
| 12.5 | One way delay of requests | second/node | Q | 5 min | Yes |
| 12.6 | One way delay of replies | second/node | Q | 5 min | Yes |

______________