| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 216 (PLEN) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Sapporo, 30 June - 11 July 2014 |
| TD |  |  |
| Source: | Editor H.248.89 |  |
| Title: | Consent: H.248.89 (ex H.248.TCP) "Gateway control protocol: TCP support packages" (New) |  |

This document contains draft new H.248.89 (ex H.248.TCP) "Gateway control protocol: TCP support packages" for Consent.

NOTE - A.5 justification TD: TD 206/Plen.

AAP Summary

This Recommendation provides H.248 packages for support of the TCP, complemented by models, considerations of package mode operations and signalling flows.

CONTENTS

Page

1 Scope 9 1.1 Applicability statements 9 1.2 Package-less TCP connection control 10 2 References 10 3 Definitions 12 3.1 Terms defined elsewhere 12 3.2 Terms defined in this Recommendation 12 4 Abbreviations and acronyms 12 5 Conventions 14 5.1 Naming of stream endpoints and terminations 14 5.2 TCP bearer 14 5.3 TCP bearer control 14 5.4 TCP endpoint 15 6 Motivation use cases and models 16 6.1 Bearer connection network use cases 16 6.2 Bearer connection model 17 6.2.1 Model for a context with a single TCP termination 17 6.2.2 Model for a context with two TCP terminations 17 6.2.3 Model with focus on context-internal interworking function 18 7 TCP basic connection control package 19 7.1 Properties 20 7.1.1 Incoming bearer connection establishment blocking 20 7.1.2 Oneway Release Indicator 21 7.2 Events 22 7.2.1 TCP connection state change ("BNC change") 22 7.3 Signals 23 7.3.1 Establish (BNC) 23 7.3.2 Release (BNC) 23 7.4 Statistics 24 7.5 Error Codes 24 7.6 Procedures 24 7.6.1 TCP endpoint creation 24 7.6.2 TCP endpoint role assumption 24 7.6.3 TCP bearer connection establishment 25 7.6.4 TCP application data transfer 26 7.6.5 TCP bearer connection release 27 7.6.6 Path MTU Discovery 28 7.6.7 Interaction of TCP-retransmissions with traffic control functions 28 8 TCP-specific stream endpoint interlinkage procedures 28 8.1 Introduction 28 8.2 Procedures 28 8.2.1 TCP bearer connection establishment 28 8.2.2 TCP bearer connection release 29 9 TCP retransmission metrics package 30 9.1 Properties 30 9.2 Events 30 9.3 Signals 30 9.4 Statistics 31 9.4.1 Retransmitted TCP Octets Sent 31 9.4.2 Retransmitted TCP Packets Sent 31 9.5 Error codes 31 9.6 Procedures 31 9.6.1 Overview – "TCP retransmission traffic" and connection models 31 9.6.2 Egress TCP traffic – Statistic "Retransmitted TCP Packets Sent" 32 9.6.3 Egress TCP traffic – Statistic "Retransmitted TCP Octets Sent" 32 9.6.4 Derived statistics 32 10 Package-less TCP control 32 10.1 Package-less TCP control in other H.248.x-Recommendations 32 10.2 Principal H.248 control steps for communication establishment 33 10.2.1 Overview 33 10.2.2 Step 1: TCP-enabled termination/stream endpoint creation 34 10.2.3 Step 2: Bearer type indication "TCP" 34 10.2.4 Step 3: Block / Unblock TCP bearer control procedures at SEP 35 10.2.5 Step 4: Start TCP connection establishment procedure ("TCP Open") 35 10.2.6 Step 5: Local L4/L3 NAT traversal support 35 10.2.7 Step 6: Reporting successfully established TCP connection 36 10.2.8 Step 7: Local L4+ NAT traversal support 36 10.2.9 Step 8: Active communication phase 36 10.3 Release of TCP connection 36 11 Security considerations 36 Annex A State modelling for TCP bearer connection endpoints 38 A.1 Introduction and purpose 38 A.2 Original state model for TCP bearer connection endpoints 38 A.3 Simplified state model for H.248-based TCP basic connection control 39 A.4 State models versus TCP modes of operation 41 Appendix I Sample use-cases of TCP connection control 42 I.1 Use case #1: WebRTC to NGN/IMS interworking function with termination of transport security by MG 42 Appendix II Example call flows 43 II.1 Overview 43 II.1.1 Mode type: application agnostic TCP-proxy 43 II.2 Basic TCP bearer connection control 43 II.2.1 Network configuration 43 II.2.2 Establishment 44 II.2.3 Release 46 II.3 Extended TCP bearer connection control 47 II.3.1 Establishment 47 II.3.2 Release 48 Appendix III Illustration of the TCP basic connection control package protocol semantics 50 III.1 Overview 50 III.2 Conventions 50 III.3 Establishment of TCP bearer connections 50 III.3.1 Successful establishment – Terminating side 50 III.3.2 Successful establishment – Originating side 51 III.3.3 Unsuccessful establishment 52 III.4 Release of TCP bearer connections 53 III.4.1 Successful release – Terminating side 53 III.4.2 Successful release – Originating side 54 III.4.3 Unsuccessful release 54 Appendix IV Illustration of the TCP-specific interlinkage procedures 55 IV.1 Overview 55 IV.2 Conventions 55 IV.3 Usage of stream endpoint interlinkage capability 55 IV.3.1 Property "seplink/linktopo" not used for interlinkage 55 IV.3.2 Property "seplink/linktopo" enabled for establishment and release 57 IV.4 Usage of Oneway Release Indicator property 58 IV.4.1 Property "ori = FALSE" 58 IV.4.2 Property "ori = TRUE" 59 IV.5 Combined consideration of both properties 60 IV.5.1 Property settings: "ori = FALSE" and "seplink/linktopo" enabled for release 60 IV.5.2 Property settings: "ori = TRUE" and "seplink/linktopo" enabled for release 60 IV.5.3 Signalling examples 61 Appendix V Relation to similar H.248 packages 63 V.1 Overview of TCP related packages 63 V.2 ITU-T Q.1950 64 V.2.1 Package 'gb' (generic bearer connection): 64 V.2.2 Package 'bcp' (bearer characteristics): 64 V.2.3 Package 'bnct' (bearer network connection cut through): 65 V.2.4 Package 'ri' (reuse idle): 65 V.3 ITU-T H.248.67 66 V.3.1 Package 'trm' (GCP transport mode indication): 66 V.4 ITU-T H.248.84 67 V.4.1 Package-independent procedures for NAT-T with TCP bearers: 67 V.4.2 Package 'tcphp' (TCP hole punching): 67 V.4.3 Package 'tcptv' (TCP traffic volume metrics): 67 V.4.4 Package 'tcpccm' (TCP connection control metrics): 68 V.4.5 Package 'tcpcqm' (TCP connection quality metrics): 68 V.4.6 Inventory of TCP functions and their possible impact on H.248 TCP gateways: 68 V.5 ITU-T H.248.43 and ITU-T H.248.79 69 Bibliography 69

List of Tables

Page

Table 1 – Principal TCP-based interfaces of H.248 entities and their relevance for this Recommendation 9 Table 2 – Use cases and TCP connection control aspects 17 Table 3 – Bearer connection establishment preparedness (of TCP endpoints) 20 Table 4 – Semantic of property ori 22 Table 5 – Semantic of parameter Type 23 Table 6 – TCP endpoint role assumption (at establishment phase) 25 Table 7 – State models versus TCP modes of operation 25 Table 8 – TCP bearer establishment - Results 26 Table 9 – Impact of StreamMode on TCP bearer traffic at external MG interface 27 Table 10 – MG bearer plane procedures (outgoing side) 28 Table 11 – MG bearer plane procedures (incoming side) 28 Table 12 – TCP bearer connection establishment – Interlinkage enabled 29 Table 13 – TCP bearer connection release – Interlinkage and oneway release indicator enabled 30 Table 14 – H.248 control steps and possible control elements 34 Table A.1 – State models versus TCP modes of operation 41 Table V.1 – Q.1950-defined H.248 packages – gb package 64 Table V.2 – Q.1950-defined H.248 packages – bcp package 64 Table V.3 – Q.1950-defined H.248 packages – bnct package 65 Table V.4 – Q.1950-defined H.248 packages – ri package 65 Table V.5 – H.248.67-defined H.248 package – trm package 66 Table V.6 – H.248.84-defined package-independent procedures for NAT-T with TCP bearers 67 Table V.7 – H.248.84-defined H.248 package – tcphp package 67 Table V.8 – H.248.84-defined H.248 package – tcptv package 67 Table V.9 – H.248.84-defined H.248 package – tcpccm package 68 Table V.10 – H.248.84-defined H.248 package – tcpcqm package 68 Table V.11 – H.248.84– Inventory of TCP functions 68

List of Figures

Page

Figure 1 – Overview: Methods for H.248-based TCP connection control 10 Figure 2 – Conventions for TCP bearer control 15 Figure 3 – Conventions for TCP endpoint types 15 Figure 4 – Bearer connection network use cases with TCP stream endpoints 17 Figure 5 – Two-termination context with a single TCP termination only 18 Figure 6 – Two- TCP termination context 18 Figure 7 – Two-termination context with context-internal interworking function 19 Figure 8 – "TCP retransmission traffic" in (TCP, TCP) connection models 32 Figure 9 – Principal H.248 control steps for establishing a native TCP bearer 34 Figure A.1 – TCP connection state diagram [IETF RFC 793] 39 Figure A.2 – Simplified state model for H.248-based TCP basic connection control 40 Figure A.3 – Extended state model for H.248-based TCP basic connection control 41 Figure I.1 – Use-case #1: WebRTC to IMS interworking function 42 Figure II.1 – Configuration "application agnostic TCP-proxy" 43 Figure II.2 – Traffic flow example for an MGC strictly controlled TCP bearer connection establishment using the TCP-proxy mode 45 Figure II.3 – Traffic flow example for an incoming TCP bearer connection release, explicitly propagated by the MGC in outgoing direction 46 Figure II.4 – Traffic flow example for outgoing TCP bearer connection release procedures towards both remote TCP endpoints X and Y 47 Figure II.5 – Traffic flow example for an extended TCP bearer connection establishment 48 Figure II.6 – Traffic flow example for an extended TCP bearer connection release 49 Figure III.1 – Successful establishment – Terminating side 50 Figure III.2 – Successful establishment – Originating side 51 Figure III.3 – Unsuccessful establishment 52 Figure III.4 – Successful release – Terminating side 53 Figure III.5 – Successful release – Originating side 54 Figure III.6 – Unsuccessful release 54 Figure IV.1 – Property "seplink/linktopo" not used for interlinkage 56 Figure IV.2 – Signalling example for TCP bearer connection establishment, property "seplink/linktopo" not used for interlinkage 56 Figure IV.3 – Property "seplink/linktopo" enabled for establishment and release at SEP T1(S1) 57 Figure IV.4 – Signalling example for TCP bearer connection establishment, property "seplink/linktopo" 58 Figure IV.5 – Property "ori = FALSE" 59 Figure IV.6 – Property "ori = TRUE" 59 Figure IV.7 – Property settings: "ori = FALSE" and "seplink/linktopo" enabled for release 60 Figure IV.8 – Property settings: "ori = TRUE" and "seplink/linktopo" enabled for release 61 Figure IV.9 – Signalling example for TCP bearer connection release and example value combinations for both properties 62 Figure V.1 – Overview of TCP related packages 63

Draft new Recommendation ITU-T H.248.89

Gateway Control Protocol:
TCP support packages

Summary

TCP is a connection-oriented IP transport protocol, which leads to specific requirements on H.248 support for the control of bearer procedures related to the establishment and release of TCP connections, as well as the MG internal interworking of TCP packets. This Recommendation provides H.248 packages for support of TCP, complemented by models, considerations of package mode operations and signalling flows.

# 1	Scope

In scope of this Recommendation are H.248 IP media gateway TCP-based bearer interfaces. Such an H.248 MG provides various functions for processing of TCP/IP packets, TCP payload data and is involved in functions related to TCP protocol control information (based on TCP header flags). This Recommendation focuses primarily on the aspect of TCP connection control, which comprises the establishment and release of TCP bearer connections.

The Recommendation considers

- connection models with a single or multiple TCP bearer connection endpoints within an H.248 context;

- establishment, through-connection and release behaviour in case of end-to-end TCP bearer connections across two H.248 TCP terminations ("a so-called H.248 TCP media stream");

- different H.248 control models.

Further, this Recommendation provides information on existing H.248 tools with respect to other TCP support functions.

## 1.1	Applicability statements

Table 1 summarizes all possible TCP-based interfaces of H.248 entities, - under the assumption of an underlying IP network -, and their relevance for this Recommendation.

Table 1 – Principal TCP-based interfaces of H.248 entities 
and their relevance for this Recommendation

| TCP-based transport | H.248 entity | Relevance |
| --- | --- | --- |
| Call control interface (e.g., SIP) | MGC | Out of scope of this Recommendation. |
| Gateway control interface (H.248) | MGC, MG | Out of scope of this Recommendation. Possible H.248 transport modes are indicated by [ITU-T H.248.67]. Usage of a TCP-based H.248 transport mode would be typically specified by an H.248 profile (as part of clause 6.12 in the profile definition template (see Appendix III in [ITU-T H.248.1])). |
| Bearer interface | MG | Within the scope of this Recommendation. |

## 1.2	Package-less TCP connection control

Figure 1 summarizes the methods for H.248-based TCP connection control (at the MG bearer interface). The required approach for TCP connection control concerning the TCP proxy mode and TCP to non-TCP interworking is based on package-defined H.248 elements introduced by this Recommendation. However, the package-less option should be used in the stateless TCP interworking configurations TCP relay and TCP merge mode (see clause 10.1). Furthermore the package-less option is required for transparent forwarding of TCP traffic. TCP transparent forwarding covers both TCP protocol control information ("the TCP header") and TCP payload data, and is typically characteristic of the application-agnostic TCP relay mode and application-agnostic TCP merge mode (after TCP connection establishment).

Figure 1 – Overview: Methods for H.248-based TCP connection control

The usage of some SDP elements is appropriate in package-based TCP connection control as long as their semantics do not contradict the H.248 elements. The bearer type indication 'TCP' is always required (see 10.2.3). The SDP "a=setup" attribute is also used in the package-based approach.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.37]	Recommendation ITU-T H.248.37 (06/2008), Gateway control protocol: IP NAPT traversal package.

[ITU-T H.248.40]	Recommendation ITU-T H.248.40 (03/2013), Gateway control protocol: Application data inactivity detection package.

[ITU-T H.248.43]	Recommendation ITU-T H.248.43 (06/2008), Gateway control protocol: Packages for gate management and gate control.

[ITU-T H.248.53]	Recommendation ITU-T H.248.53 (03/2009), Gateway control protocol: Traffic management packages.

[ITU-T H.248.64]	Recommendation ITU-T H.248.64 (03/2013), Gateway control protocol: IP router packages.

[ITU-T H.248.67]	Recommendation ITU-T H.248.67 (12/2009), Gateway control protocol: Transport mode indication package.

[ITU-T H.248.69]	Recommendation ITU-T H.248.69 (03/2009), Gateway control protocol: Packages for interworking between MSRP and H.248.

[ITU-T H.248.78]	Recommendation ITU-T H.248.78 (03/2013), Gateway control protocol: Bearer-level application level gateway.

[ITU-T H.248.79]	Recommendation ITU-T H.248.79 (02/2012), Gateway control protocol: Guidelines for packet-based streams.

[ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control protocol: NAT-traversal for peer-to-peer services.

[ITU-T H.248.88]	Recommendation ITU-T H.248.88 (11/2013), Gateway control protocol: RTP topology dependent RTCP handling by H.248 Media Gateways with IP Terminations.

[ITU-T H.248.90]	Recommendation ITU-T H.248.90 (07/2014), Gateway control protocol: H.248 packages for control of transport security using TLS.

[ITU-T H.248.92]	Recommendation ITU-T H.248.92 (07/2014), Gateway control protocol: Stream endpoint interlinkage package.

[ITU-T Q.1950]	Recommendation ITU-T Q.1950 (12/2002), Bearer independent call bearer control protocol.

[ITU-T X.200]	Recommendation ITU-T X.200 (07/1994), Information technology - Open Systems Interconnection - Basic Reference Model: The basic model.

[IETF RFC 793]	IETF RFC 793 (1981), Transmission Control Protocol.

[IETF RFC 1191]	IETF RFC 1191 (1990), Path MTU Discovery.

[IETF RFC 1981]	IETF RFC 1981 (1990), Path MTU Discovery for IP version 6.

[IETF RFC 4022]	IETF RFC 4022 (2005), Management Information Base for the Transmission Control Protocol (TCP).

[IETF RFC 4145]	IETF RFC 4145 (2005), TCP-Based Media Transport in the Session Description Protocol (SDP).

[IETF RFC 4898]	IETF RFC 4898 (2007), TCP Extended Statistics MIB.

[IETF RFC 4976]	IETF RFC 4976 (2007), Relay Extensions for the Message Session Relay Protocol (MSRP).

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	TCP packet [ITU-T H.248.84]: IP datagram (also known as IP packet) carrying a (single) TCP segment in the payload.

NOTE – Such a L4/L3 PDU is also known as TCP/IP packet (briefly TCP packet).

3.1.2	Transparent forwarding [ITU-T H.248.88]: MG packet forwarding behaviour with the characteristic of Lx-PDU integrity. This is a unidirectional characteristic of a Lx-PDU flow.

3.1.3	transport (TCP) proxy (translator) mode [ITU-T H.248.84], see clause I.4.1 in [b-ETSI TR 183 068]: mode (also known as Back-to-Back TCP Endpoint (B2BTE) mode): stateful forwarding of TCP packets in terms of full protocol termination. The end-to-end TCP connection is partitioned in two TCP connection legs by the BGF. Each H.248 Stream endpoint provides a stateful TCP connection state machine.

NOTE 1 – The term proxy mode is similar as used for HTTP proxy, FTP proxy, SIP proxy, etc.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	TCP proxy types: the basic TCP proxy mode is defined in clause 3.1.3. The following TCP proxy types are useful for the various TCP services as supported by H.248 MGs.

The TCP proxy may be an

- Application-aware TCP proxy (the MG knows the protocol (stack) carried by TCP packets);

- Application-agnostic TCP proxy (the MG is unaware of the TCP payload content).

A TCP proxy may implement the full TCP connection states (stateful) or a sub-set of those states (light).

Example: "application-agnostic, stateful TCP proxy" mode.

3.2.2	TCP transparent forwarding: MG packet forwarding behaviour with the characteristic of TCP-PDU integrity (NOTEs 1, 2). This is a unidirectional characteristic of a TCP-PDU flow.

NOTE 1 – A TCP-PDU relates to a TCP packet (see [ITU-T H.248.84]), which comprises TCP protcol control information (TCP header) and TCP payload data.

NOTE 2 – Definition based on clause 3.1.2, i.e., the characteristic of PDU integrity comprises the properties of bit integrity and data integrity (see also clauses 3.2.3, 3.1.1 and 3.1.2 in [ITU-T H.248.88]).

NOTE 3 – There is the characteristic of TCP packet integrity in the context of "TCP transparent forwarding". The MG might be TCP aware; e.g. support of TCP related statistics or event detection would not violate transparent forwarding behaviour.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| B2BIH | Back-to-Back IP Host |
| --- | --- |

| B2BTE | Back-to-Back TCP Endpoint |
| --- | --- |
| BCP | Bearer Control Protocol |
| BGF | Border Gateway Function |
| BICC | Bearer Independent Call Control |
| BNC | Backbone Network Connection |
| DTLS | Datagram Transport Layer Security |
| FFS | For further studies |
| FTP | File Transfer Protocol |
| GCP | Gateway Control Protocol |
| HTTP | Hypertext Transfer Protocol |
| ICE | Interactive Connectivity Establishment |
| ICMP | Internet Control Message Protocol |
| IMS | IP multimedia subsystem |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |

| IPR | IP Router |
| --- | --- |
| IWF | Interworking Function |
| LCD | Local Control Descriptor |

| LD | Local descriptor |
| --- | --- |

| Lx | (Protocol)Layer number |
| --- | --- |

| MG | Media gateway |
| --- | --- |
| MGC | Media gateway controller |

| MIB | Management Information Base |
| --- | --- |

| MPTCP | Multipath TCP |
| --- | --- |
| MSRP | Message Session Relay Protocol |
| MTU | Maximum Transmission Unit |
| NAT | Network address translation |
| NAT-T | NAT traversal |
| NGN | Next generation network |

| NPT | Network Prefix Translation |
| --- | --- |

| PMTUD | Path MTU discovery |
| --- | --- |
| RD | Remote descriptor |

| RTP | Real-Time Transport Protocol |
| --- | --- |

| SCTP | Stream Control Transmission Protocol |
| --- | --- |
| SDP | Session description protocol |
| SEP | (H.248) Stream endpoint |
| SEPP | SEP Pair |
| SIP | Session initiation protocol |
| TCP | Transmission control protocol |
| TLS | Transport Layer Security |
| UDP | User datagram protocol |
| UE | User equipment |
| W3C | World Wide Web Consortium |
| WEBRTC | Real-Time Communication in WEB-browsers (as work item in W3C) |

# 5	Conventions

## 5.1	Naming of stream endpoints and terminations

The notations "Tx" and "Tx(Sy)" are used for naming of terminations and stream endpoints (SEP) respectively, with

- "Tx" as TerminationID value;

- "Sy" as StreamID value and

- numerical variables x and y.

## 5.2	TCP bearer

In this Recommendation the term TCP bearer connection refers to a TCP connection in the network user plane. There might be also a TCP connection between MGC and MG in case of a TCP-based H.248 transport mode [ITU-T H.248.67]. This connection is not is scope of this Recommendation.

NOTE 1 – This relates to the so called backbone network connection (BNC) concept in [ITU-T Q.1950], which is referred by clause 7.

## 5.3	TCP bearer control

There are connection control information flows for TCP bearer connections in the network user plane (e.g., IP media/data/bearer plane) and inH.248 signalling. Figure 2 outlines the used conventions for the differentiation of both control flows:

Figure 2 – Conventions for TCP bearer control

## 5.4	TCP endpoint

The notion of endpoint represents different concepts, see Figure 3:

Figure 3 – Conventions for TCP endpoint types

Usage in

- H.248 control : H.248 terminations/stream endpoints with TCP processing are denoted as TCP-enabled termination or stream endpoint (SEP) respectively;

- user plane (TCP): a TCP bearer connection endpoint represents an "(N)-connection-endpoint" according [ITU-T X.200]. This concept comprises a terminator (i.e., TCP protocol termination) plus a service access point (i.e., L4+ access).

The Stream Endpoint (SEP) subject of the packages and procedures of this Recommendation may act as a TCP bearer connection endpoint or not:

- The TCP basic connection control package (clause 7) assumes a SEP that is a TCP bearer connection endpoint.

- The interlinkage procedures described in clause 8 using tools specified in [ITU-T H.248.92] assume a SEP or SEP pair that is (are) TCP bearer connection endpoint(s).

- The use of TCP retransmission metrics package (clause 9) is applicable both for SEPs that are TCP bearer connection endpoints and SEPs which are not.

# 6	Motivation use cases and models

## 6.1	Bearer connection network use cases

There are three fundamental use cases from the perspective of H.248 MGs and the TCP bearer connection control (see Figure 4):

- Use case #1.1: An H.248 IP-IP MG located in the middle of an end-to-end TCP connection; there are two interconnected H.248 stream endpoints (SEP), - a TCP stream endpoint pair (TCP SEPP) -, within the H.248 context; two cases may be distinguished from an overall network perspective:

- Use case #1.1.1: there is a single, continuous end-to-end TCP connection, any interim H.248 IP-IP MG does not fully terminate the TCP protocol; this use case relates to a TCP relay or merge mode behaviour of the MG (see [ITU-T H.248.84]);

- Use case #1.1.2: the end-to-end TCP connection is divided in two separate TCP connection segments, the H.248 IP-IP MG provides complete TCP protocol terminations towards the remote TCP bearer connection endpoints; this use case relates to the TCP proxy mode behaviour of the MG;

- Use case #1.2: a TCP to non-TCP interworking scenario; the H.248 MG acts as a single TCP endpoint.

The following use cases are for further studies:

- Use case #2: multipath TCP (MPTCP) and H.248 MG involvement.

- Use case #3: multiple TCP bearer connection endpoint models such as applied by media servers (e.g., an [ITU-T H.248.69] based MSRP switch).

Figure 4 illustrates the primary network use cases.

Figure 4 – Bearer connection network use cases with TCP stream endpoints

The TCP connection control aspect is a primary scope of this Recommendation, some general conclusions are described in Table 2:

Table 2 – Use cases and TCP connection control aspects

| Use case | TCP connection control aspects |
| --- | --- |
| 1.1.1 | TCP connection establishment (and release) is true end-to-end, meaning that the TCP connection control procedures at the two H.248 SEPs are tightly coupled. E.g., an incoming TCP connection establishment request (i.e. a TCP SYN packet) from remote endpoint X would lead to an immediate forwarding of that "bearer connection control request" towards remote endpoint Y. This relates to a TCP relay and merge mode scenario. |
| 1.1.2 | TCP connection establishment (and release) is decoupled from end-to-end perspective. This looks like a back-to-back TCP endpoint as given by TCP proxy mode. The TCP bearer connection control procedures are independent for each SEP. |
| 1.2 | The H.248 MG provides an IP host function for a single TCP connection endpoint. |

## 6.2	Bearer connection model

All protocol elements and procedures described in this Recommendation are described in a connection model with up to two ITU-T H.248 terminations (note: there might be one or multiple stream endpoint pairs (SEPP) in case of two terminations). However they can be applied in connection models with more than two H.248 terminations. In addition, no assumptions are made regarding either the lower layer protocols beneath the TCP layer (IPv4 or IPv6) or the upper layer protocols being carried by the TCP payload. This allows the use of the Recommendation's procedures in various connection models and use cases such as illustrated in the sub-clauses.

### 6.2.1	Model for a context with a single TCP termination

Figure 5 details the generic connection-model where a TCP-enabled termination is connected to a single non-TCP enabled termination.

Figure 5 – Two-termination context with  a single TCP termination only

This model relates to use case #1.2.

### 6.2.2	Model for a context with two TCP terminations

Figure 6 represents the general model for use case category #1.1. The type of interworking function (IWF) determines the particular MG behaviour concerning TCP handling (see next sub-clause 6.2.3).

Figure 6 – Two- TCP termination context

### 6.2.3	Model with focus on context-internal interworking function

Figure 7 depicts interworking functions within a two termination context with scope on example interworking functions.

Figure 7 – Two-termination context with context-internal interworking function

The IWF is principally a context-level function, associated to a particular SEPP (here: SEPs T1(S1) and T2(S1)). In scope of this Recommendation are TCP bearer traffic processing functions (inclusive L4 IWFs), such as:

- Cut-through characteristic: when the two SEPs are interconnected for enabled traffic flow.

- BCP (for TCP) information of one SEP as possible stimuli for procedures at a partner SEP, or not. For example: an incoming TCP connection release triggers an outgoing TCP connection release.

- TCP transparent forwarding: Only the application-agnostic TCP relay mode (i.e., without any L4+-IWF) supports TCP transparent forwarding. Both, the TCP merge and TCP proxy mode modify TCP header information, which contradicts TCP transparent forwarding. However, a TCP merge mode usually behaves transparently in the data transfer phase.

L3 IWF is not in scope of this Recommendation.  However, TCP modes have ususally a dependency on L3 IWF modes:

- The full protocol termination as part of the TCP proxy mode implies two separate, addressable IP transport endpoints, which are only subject of the B2BIH mode.

- The TCP relay mode and TCP merge modes may use an underlying IP Router mode, a B2BIH mode of L3 IWF or a NAT-less mode according to [b-ETSI TR 183 068].

L4+ IWF is not in scope of this Recommendation. However, L4+ IWF may require or be dependent on certain types of L4 IWF:

- Transparent forwarding of TCP-SDUs (i.e., TCP payload data): transparent forwarding is principally feasible for all TCP modes unless a specific L4+ IWF impacts bit integrity.

- MSRP switch [ITU-T H.248.69]: this Recommendation could have an interaction with [ITU-T H.248.69] because the MSRP switch relates to an "application-aware, stateful TCP proxy" type.

- MSRP relay [IETF RFC 4976]: the MSRP relay function could be provided by an H.248 gateway as part of an application-aware TCP relay or application-aware, lightweight TCP proxy. [ITU-T H.248.69] itself does not provide explicit information concerning an "MGC controlled MSRP relay".

- Bearer-level ALG [ITU-T H.248.78]: a B-ALG service could be enabled for TCP traffic, which would imply the processing of TCP-SDUs (i.e., the TCP payload data). A B-ALG is in principle feasible for all kind of TCP modes (because processing the unstructured octet stream is carried in the TCP payload, which does not require the protocol termination of the TCP). Thus, a B-ALG inherently is characterized by non-transparent TCP forwarding.

- Transport level security [ITU-T H.248.90]: TLS transparent forwarding may be supported by all TCP modes, but TLS/TCP-to-TCP interworking implies an application-aware TCP proxy mode.

# 7	TCP basic connection control package

|  | Package Name: | TCP basic connection control package |
| --- | --- | --- |
|  | Package ID: | tcpbcc (0x0115) |
|  | Description: | This package provides the functionality to establish and release a TCP bearer connection. NOTE – The package design follows conceptually the generic bearer connection (gb) package according to clause A.6/[ITU-T Q.1950] by using an event-/signal-based approach for connection establishment/release control. Exceptions: a) bearer modification is not required for TCP; b) bearer release: uses a package-defined codepoint instead of element g/cause. Furthermore, the package supports bi- and unidirectional TCP bearer connection release procedures. |
|  | Version: | 1 |
|  | Extends: | None |

## 7.1	Properties

### 7.1.1	Incoming bearer connection establishment blocking

|  | Property Name: | Incoming bearer connection establishment blocking |
| --- | --- | --- |
|  | Property ID: | bceb (0x0001) |
|  | Description: | This property defines the MG availability to accept or reject incoming TCP bearer connection establishment procedures. NOTE 1: See Annex A concerning a state model based illustration. NOTE 2: This capability might be relevant for the prevention of other TCP processing behaviour such as "TCP relay" or "TCP merge" modes (see [ITU-T H.248.84]). |
|  | Type: | Enumeration |
|  | Possible values: | See Table 3 Table 3 – Bearer connection establishment preparedness (of TCP endpoints) Value; MG behaviour. "blocked" (0x0001); - Bearer resources are prepared (i.e., resources for TCP endpoint are reserved and allocated to H.248 termination/SEP); - TCP role: not determined; prevents assumption of server role; - TCP connection state = "CLOSED" (Figure A.1); - H.248 counterpart state = "Blocked" (Figure A.3); - Any incoming TCP packet shall be silently discarded (e.g., TCP-SYN requests shall not be acknowledged), there is no state transitioning from "CLOSED" (NOTE 3). - Any outgoing TCP bearer connection control procedures are not affected. - Signal EstBNC (clause 7.3.1): start 3-way handshake by sending of TCP-SYN; TCP client role is effectively assigned. - Start 3-way handshake by sending of TCP-SYN triggered by an interlinkage procedure [ITU-T H.248.92]; SEP adopts TCP client role.. "unblocked" (0x0002); - MG shall be ready for incoming TCP bearer connection control procedures; - TCP role: not determined but prepares for a TCP server role effectively ("TCP Passive Open") (NOTE 4); - Incoming TCP-SYN: TCP server role is confirmed, 3-way handshake proceeded; - Any outgoing TCP bearer connection control procedures are not affected: - Signal EstBNC (clause 7.3.1): start 3-way handshake by sending of TCP-SYN; TCP client role is effectively assigned. - Start 3-way handshake by sending of TCP-SYN triggered by an interlinkage procedure [ITU-T H.248.92]; SEP adopts TCP client role. - TCP connection state: transitioning from "CLOSED" to "LISTEN" (Figure A.1); - H.248 counterpart state = "Idle" (Figure A.3);. NOTE 3 - This semantic is analogous to SDP codepoint "a=setup:holdconn", see [IETF RFC 4145]. NOTE 4 - This semantic could be correlated with SDP codepoint "a=setup:actpass", see [IETF RFC 4145].. |
|  | Default: | "Unblocked" (in order to be consistent with the default behaviour as in [ITU-T H.248.84]). NOTE - H.248 profile specifications could redefine other default values. Examples: H.248 profiles for - MGs with support of "TCP proxy" mode only may choose default value "blocked" due to fully separated establishment procedures towards both remote TCP endpoints; - MGs with additional support of "TCP relay" and "TCP merge" modes may choose default value "unblocked" in order not to miss any incoming TCP connection establishment requests ("early media"). |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 7.1.2	Oneway Release Indicator

|  | Property Name: | Oneway Release Indicator |
| --- | --- | --- |
|  | Property ID: | ori (0x0002) |
|  | Description: | This property defines the behaviour in the MG in case a TCP-FIN is received for an established TCP connection. It instructs the MG whether or not to send a TCP-FIN. |
|  | Type: | Boolean |
|  | Possible values: | Table 4 defines the protocol semantic per value. Table 4 – Semantic of property ori Value; MG semantic for TCP bearer connection. False; Indicate the duplex closure of the TCP bearer connection by sending a TCP-FIN autonomously.. True; Keep the TCP bearer connection in the half-closed state, i.e. the MG shall not generate the TCP-FIN autonomously. The MGC explicitly releases the TCP bearer connection via signal tcpbcc/RelBNC.. |
|  | Default: | False |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 7.2	Events

### 7.2.1	TCP connection state change ("BNC change")

|  | Event Name: | TCP connection state change |
| --- | --- | --- |
|  | Event ID: | BNCChange (0x0001) |
|  | Description: | This event occurs whenever a change to a (TCP) bearer network connection occurs. For example a bearer has been established or a bearer has been released. The event is related to a simplified state transitioning model, - in contrast to [IETF RFC 793] -, which is described in Annex A. NOTE – The event design is aligned with event gb/BNCChange according clause A.6.2.1/[ITU-T Q.1950] by using an event-/signal-based approach for connection establishment/release control. |

#### 7.2.1.1	EventsDescriptor parameters

##### 7.2.1.1.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. |
|  | Type: | Sub-list of Enumeration |
|  | Optional: | Yes |
|  | Possible values: | Table 5 defines the protocol semantic per value: Table 5 – Semantic of parameter Type Value; Name; Final state (Figure A.2); Semantic. Est [0x01]; Bearer Established; Established; Notify MGC successful TCP bearer establishment.. Rel [0x05]; Bearer Released; Idle; Notify MGC successful TCP bearer release.. NOTE – The numerical codepoint assignments are aligned with [ITU-T Q.1950].. |
|  | Default: | Est, Rel |

#### 7.2.1.2	ObservedEventsDescriptor parameters

##### 7.2.1.2.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | This is used to indicate what change has occurred to the TCP bearer connection. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | Est [0x01] Bearer Established Rel [0x05] Bearer Released |
|  | Default: | None |

## 7.3	Signals

The signals are related to a simplified state transitioning model, in contrast to [IETF RFC 793], which is described in Annex A.

### 7.3.1	Establish (BNC)

|  | Signal Name: | Establish BNC |
| --- | --- | --- |
|  | Signal ID: | EstBNC (0x0001) |
|  | Description: | This signal triggers the bearer control function to start bearer establishment (i.e., this signal is used to initiate the TCP bearer connection establishment: the MG takes the TCP client role, sends a TCP SYN segment and completes the TCP active OPEN ("three-way handshake" procedures). This signal only takes effect in H.248 connection states Idle and Blocked (and would be replied by an error code when in H.248 connection state Established). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 7.3.1.1	Additional parameters

None.

### 7.3.2	Release (BNC)

|  | Signal Name: | Release BNC |
| --- | --- | --- |
|  | Signal ID: | RelBNC (0x0002) |
|  | Description: | This signal triggers the bearer control function to send bearer release (i.e., this signal is used to initiate the TCP closure procedure(s): the MG sends a TCP FIN segment and completes the associated handshake procedure). This signal only takes effect in H.248 connection state Established (and would be replied by an error code when in H.248 connection states Blocked or Idle). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 7.3.2.1	Additional parameters

None.

## 7.4	Statistics

None.

## 7.5	Error Codes

None

## 7.6	Procedures

### 7.6.1	TCP endpoint creation

The MGC is responsible for creating a TCP connection enabled SEP in the MG. This is indicated via the H.248 interface through the use of the SDP "m=" line according the rules as specified in clause 13.4 in [ITU-T H.248.84].

The MG shall reserve "TCP bearer resources" (such as a "TCP connection state machine") and allocates them to the SEP. The TCP endpoint role (client/server) is not determined yet.

NOTE 1 – This functionality is synonymous to the "bearer type indication" semantic as defined by the BNC Characteristics property (BNCChar) of the bearer characteristics package (BCP), see clause A.3.1.1/[ITU-T Q.1950].

### 7.6.2	TCP endpoint role assumption

The TCP role, client or server, is primarily relevant during the establishment phase of communication (of perspective of H.248 gateways). The MGC does not explicitly assign the SEP a TCP role. It is an incoming or outgoing establishment which leads to the assumption of a TCP role. The SEP is basically prepared for an incoming or outgoing establishment since the moment it is created with an appropriate media descriptor.

The MGC may prevent the assumption of a server role by setting the bceb property to "blocked". The MGC may effectively request the SEP to assume the client role by sending the EstBNC signal. The SEP may also assume the client role as a consequence of interlinkage procedures [ITU-T H.248.92]. If these procedures are allowed the SEP may assume a client role as a consequence of e.g. an incoming procedure in the associated SEP. Table 6 summarizes MGC actions and their results in terms of TCP role adopted by the SEP:

Table 6 – TCP endpoint role assumption (at establishment phase)

| No. | H.248 indication | MG behaviour: | TCP role |
| --- | --- | --- | --- |
| 1 | 1) H.248 SEP state "blocked", due to explicit signalling of bceb property. | Any incoming TCP packet shall be silently discarded. | Not yet determined |
| 2 | 2) H.248 SEP state "unblocked" either due to explicit signalling of bceb property or correspondent default value. | Incoming TCP SYN leads to subsequent completion of 3-way handshake. | Server role |
| 3 | H.248 signal EstBNC. Value of bceb property not relevant. | Start of 3-way handshake for TCP connection establishment. | Client role |
| 4 | Interlinkage procedures allowed according to [ITU-T H.248.92]. Value of bcep property not relevant (although this or similar properties may be relevant at the incoming side of the interlinkage). | An interlinkage establishment procedure [ITU-T H.248.92], e.g. an incoming TCP establishment procedure in the associated SEP leads to the start of 3-way handshake for TCP connection establishment. | Client role |

Furthermore, a TCP endpoint role assumption is only meaningful if the TCP-enabled H.248 termination/SEP provides an actual TCP endpoint representation, which depends on the TCP mode of operation. Table 7 provides a summary:

Table 7 – State models versus TCP modes of operation

|  | TCP mode | TCP endpoint role | Comments |
| --- | --- | --- | --- |
| a) | TCP to non-TCP interworking | either TCP ACTIVE or PASSIVE OPEN | Due to complete protocol termination of TCP. |
| b) | TCP to TCP in TCP relay mode | none | Due to TCP bearer connection establishment packets transparently forwarded (stateless). |
| c) | TCP to TCP in TCP merge mode | only TCP PASSIVE OPEN | Due to an end-to-end TCP simultaneous open emulation. |
| d) | TCP to TCP in TCP proxy mode | either TCP ACTIVE or PASSIVE OPEN | Due to an end-to-end TCP simultaneous open emulation. |

### 7.6.3	TCP bearer connection establishment

If the SEP assumes the client role, as a result of an EstBNC signal or of an interlinkage procedure [ITU-T H.248.92], the MG shall initiate and complete the TCP connection setup (3-way handshake) without any further involvement of the MGC.

If the SEP assumes the server role, as a result of an incoming TCP SYN, and being allowed to do so by the MGC by having the bceb property set to "unblocked", the MG shall complete the TCP connection setup handshake without any further involvement of the MGC.

The MGC may arm events tcpbcc/BNCChange and g/cause (in H.248 connection state Idle) in order to be notified about successful or unsuccessful TCP bearer connection establishment.

The end of the TCP bearer connection establishment phase is summarized according to Table 8:

Table 8 – TCP bearer establishment - Results

| TCP role | Result of TCP bearer connection establishment | MGC notification? |
| --- | --- | --- |
| TCP client (i.e., outgoing establishment side) | Successful | Successful establishment may be notified to the MGC depending on subscription of event and parameter value tcpbcc/BNCChange{Type=[Est]}. |
|  | Unsuccessful | Unsuccessful establishment may be indicated to the MGC via a corresponding ServiceChange procedure or event g/cause. |
| TCP server (i.e., incoming establishment side) | Successful | As per client side. |
|  | Unsuccessful | As per client side. |

### 7.6.4	TCP application data transfer

#### 7.6.4.1	MG external

The TCP bearer connection is ready for application data transfer when remote and local TCP endpoints are both transitioned to TCP connection state "ESTAB".

The StreamMode property of the LocalControl Descriptor affects the application data flow rather than the TCP control information used to establish or close the TCP connection. The requested MG behaviour is defined by Table 9:

Table 9 – Impact of StreamMode on TCP bearer traffic at external MG interface

| Traffic direction | StreamMode settings | MG behaviour: |
| --- | --- | --- |
| Outgoing | Sending enabled (i.e., values SendOnly, SendRecv, LoopBack) | All TCP packets sent |
|  | Sending disabled (i.e., values RecvOnly, Inactive) | Only TCP packets with active usage of TCP SYN, FIN, RST and PSH information are sent. Optional piggybacked data is sent. |
| Incoming | Receiving enabled (i.e., values RecvOnly, SendRecv, LoopBack) | All TCP packets accepted |
|  | Receiving disabled (i.e., values SendOnly, Inactive) | Only TCP packets with active usage of TCP SYN, FIN, RST and PSH information are accepted. Optional piggybacked data is accepted. |

TCP provides the application the capability to affect the data application transfer through the PSH- and the URG-flag. The usage of these flags is application specific and therefore must be specified in the application specific procedures.

#### 7.6.4.2	MG internal

This clause is only relevant for H.248 contexts with TCP SEPP(s), as outlined in clause 6.2.3.

Application data may be received at a SEP in time where the other SEP of the context is not yet able to process or send the application data. The behaviour of the MG in such a situation is implementation and/or application dependent.

The interlinkage capability as profiled by clause 8 may be used to align establishment procedures in both SEPs. This may increase the likelihood that application data received in one SEP is not lost.

### 7.6.5	TCP bearer connection release

#### 7.6.5.1	Characteristic of TCP bearer connection release

This package supports bidirectional TCP bearer connection release only. Consequently, two TCP half-close procedures shall be performed, resulting in a four-way handshake (see e.g. clause III.4). The transitioning from H.248 state Established to Idle of the local TCP bearer connection endpoint is only done after the successful processing of both TCP half-close procedures.

#### 7.6.5.2	Incoming or outgoing TCP bearer connection release procedure

The TCP bearer connection endpoint shall perform an outgoing TCP bearer connection release procedures when indicated via signal RelBNC by the MGC.

The local TCP bearer connection endpoint shall be able to process incoming TCP bearer connection release requests (TCP-FIN packet) at any point in time (whilst in H.248 state Established).

#### 7.6.5.3	MG bearer plane procedures

##### 7.6.5.3.1	MGC initiated release (outgoing side)

In order to initiate an outgoing TCP bearer connection release the local TCP bearer connection endpoint requires an explicit indication from the MGC via signal RelBNC by the MGC (Table 10).

Table 10 – MG bearer plane procedures (outgoing side)

| MG bearer plane procedure | Result | MGC notification? |
| --- | --- | --- |
| Starts first TCP half-close by sending TCP-FIN. Awaits TCP-FIN-ACK and TCP-FIN of second TCP half-close. Subsequent confirmation by a TCP-FIN-ACK. Transition to connection state H.248 Idle (TCP 'CLOSED'). | Successful | The MGC default assumption is a "successfully released bearer connection" however it may be explicitly notified via the optionally BNCChange event, ObservedEventsDescriptor parameter codepoint "Type=Rel". |
|  | Unsuccessful | There are following options: - Via event g/cause (NOTE 1) - Via error code - Via event adid/ipstop (see [ITU-T H.248.40]) (NOTE 2) |
| NOTE 1 - Not all generic codepoints are applicable for TCP. NOTE 2 - Background: there could be still media activity in case of unsuccessful bearer release. Hence, this event could principally be used to indicated a "hanging TCP connection". |  |  |

See Appendix III.4.2 for example traffic flows.

##### 7.6.5.3.2	Bearer initiated release (incoming side)

The local TCP bearer connection endpoint shall be able to process incoming TCP bearer connection release requests (TCP-FIN packet) at any point in time (whilst in H.248 state Established). The TCP-FIN packet is the start event for TCP bearer connection release (Table 11).

Table 11 – MG bearer plane procedures (incoming side)

| MG bearer plane procedure | Result | MGC notification? |
| --- | --- | --- |
| Awaits first TCP half-close by detection of incoming TCP-FIN. Subsequent acknowledgment as well as start of second TCP half-close procedures. Transition to connection state H.248 Idle (TCP 'CLOSED') after reception of TCP FIN-ACK. | Successful | See Table 5 |
|  | Unsuccessful | There are the following options: - Via event g/cause - Via event adid/ipstop |

See Appendix III.4.1 for example traffic flows.

### 7.6.6	Path MTU Discovery

Path MTU discovery (PMTUD) is normally started after successful establishment of TCP bearer connections, see [IETF RFC 1191] (TCP-over-IPv4), [IETF RFC 1981] (TCP-over-IPv6) and [b-IETF RFC 2923] (TCP specific problems with PMTUD). It is expected that bearer plane PMTUD procedures are completely hidden to the MGC.

PMTUD procedures may result in error conditions, such as received ICMP errors from remote side. Whether such situations should lead to error notifications of the MGC by the MG is for further studies.

### 7.6.7	Interaction of TCP-retransmissions with traffic control functions

Retransmissions of unacknowledged TCP segment leads to an inherent increase of the consumed transport capacity. There might be therefore possible side effects and interactions with traffic control functions, such as traffic policing or traffic shaping.

#### 7.6.7.1	Interaction with traffic policing: IP byterate policing according to [ITU-T H.248.53]

Such a traffic policer entity is enforced at the ingress side of a TCP SEP. The traffic policer function shall consider all incoming TCP packets, without distinction between first sent or retransmitted TCP data.

# 8	TCP-specific stream endpoint interlinkage procedures

## 8.1	Introduction

Property interlinkage topology (linktopo) from [ITU-T H.248.92], clause 7.1.1 may be used for extended TCP connection control procedures at establishment or release phase. This clause defines TCP-specific stream endpoint interlinkage procedures.

With regards to TCP bearer connection release: the oneway release indicator (property tcpbcc/ori, see clause 7.1.2) may interact with "release interlinkage". All combinations are considered in the sub-clauses below.

## 8.2	Procedures

### 8.2.1	TCP bearer connection establishment

#### 8.2.1.1	Stream endpoint pair interlinkage disabled

If property seplink/linktopo is not used (or disabled per default), then the incoming TCP bearer connection establishment procedure shall be identical to base package tcpbcc procedures.

NOTE – Clause IV.3.1 provides signalling examples.

#### 8.2.1.2	Stream endpoint pair interlinkage enabled

The interlinkage property seplink/linktopo may be enabled for TCP establishment (see example H.248 syntax in Table 12), then the incoming TCP bearer connection establishment request shall be propagated to the partner SEP, triggering there an outgoing TCP-SYN.

Table 12 – TCP bearer connection establishment – Interlinkage enabled

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = 1 { Context = 1 { Add = ip/1/$/$ { ; Termination T1 Media { Stream = 1 { ; SEP S1 LocalControl { seplink/linktopo = ["<interlinkedSEP>:TCP:TCP:est"] }}…} | Interlinkage (for stream S1) is enabled from termination T1 to termination T2 (given by property parameter value "interlinkedSEP"). |

NOTE – Clause IV.3.2 provides signalling examples.

### 8.2.2	TCP bearer connection release

Only relevant for incoming TCP bearer connection release requests from MG external bearer interface. Outgoing procedures are not affected.

#### 8.2.2.1	Stream endpoint pair interlinkage disabled and Oneway release indicator disabled

If parameter <release> in property seplink/linktopo is not used (or disabled per default) and the property ori is set to False:

- the incoming TCP bearer connection release procedure shall be not propagated to the partner SEP;

- the incoming TCP bearer connection release procedure shall lead to a bidirectional TCP bearer connection release, hence identical to base package tcpbcc procedures.

#### 8.2.2.2	Stream endpoint pair interlinkage enabled and Oneway release indicator disabled

The interlinkage property seplink/linktopo is enabled for TCP release (see Table 13) and the property ori is set to False:

- the incoming TCP bearer connection release request shall be propagated to the partner SEP, triggering there an outgoing TCP-FIN.

- the incoming TCP bearer connection release procedure shall led to a bidirectional TCP bearer connection release, hence identical to the tcpbcc package procedures.

NOTE – Appendix IV.5.3 provides signalling examples.

#### 8.2.2.3	Stream endpoint pair interlinkage disabled and Oneway release indicator enabled

If parameter <release> in property seplink/linktopo is not used (or disabled per default) and the property ori is set to True:

- the incoming TCP bearer connection release procedure shall not be propagated to the partner SEP.

- the incoming TCP bearer connection release procedure shall lead to a one TCP bearer connection release (so called TCP half closure).

#### 8.2.2.4	Stream endpoint pair interlinkage enabled and Oneway release indicator enabled

If the interlinkage property seplink/linktopo is enabled for TCP release (see Table 13), and the ori property is set to True,

- the incoming TCP bearer connection release request shall be propagated to the partner SEP, triggering there an outgoing TCP-FIN.

- the incoming TCP bearer connection release procedure shall lead to a one TCP bearer connection release (so called TCP half closure).

Table 13 – TCP bearer connection release – Interlinkage and 
oneway release indicator enabled

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:54321 Transaction = 1 { Context = 1 { Add = ip/1/$/$ { ; Termination T1 Media { Stream = 1 { ; SEP S1 LocalControl { seplink/linktopo = ["<interlinkedSEP>:TCP:TCP:rel"] tcpbcc/ori = ON }}…} | Both release related capabilities are enabled. |

NOTE – Appendix IV.5.3 provides signalling examples.

# 9	TCP retransmission metrics package

|  | Package Name: | TCP retransmission metrics package |
| --- | --- | --- |
|  | Package ID: | tcprm (0x0116) |
|  | Description: | This package is used to support explicit octet and packet count statistics for the TCP bearer protocol, related to TCP retransmission traffic. The statistics are applicable for all TCP modes of operation, as long as the MG is aware that the H.248 Stream carries TCP traffic. |
|  | Version: | 1 |
|  | Extends: | None. |

## 9.1	Properties

None.

## 9.2	Events

None.

## 9.3	Signals

None.

## 9.4	Statistics

### 9.4.1	Retransmitted TCP Octets Sent

|  | Statistic Name: | Retransmitted TCP Octets Sent |
| --- | --- | --- |
|  | Statistic ID: | tcpros (0x0001) |
|  | Description: | This Statistic provides the number of retransmitted octets sent on the Termination or Stream since the Termination or Stream has existed and the Statistic has been set. The octets represent the egress retransmitted TCP packets of all TCP flows of an H.248 Stream. It is the total number of octets (i.e., including TCP header) retransmitted in TCP packets. At the Termination level, it is equal to the sum of the egress TCP flows over all Streams. NOTE – The semantic of this TCP metric is similar with the managed object tcpEStatsPerfOctetsRetrans of the TCP MIB [IETF RFC 4898]. |
|  | Type: | Unsigned Integer |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 9.4.2	Retransmitted TCP Packets Sent

|  | Statistic Name: | Retransmitted TCP Packets Sent |
| --- | --- | --- |
|  | Statistic ID: | tcprps (0x0002) |
|  | Description: | Provides the number of retransmitted packets sent on the Termination or Stream since the statistic has been set. The packets represent the egress retransmitted TCP packets of all TCP flows of an H.248 Stream. It is the total number of TCP packets, inclusive of TCP connection establishment, data transfer and connection release phases. At the Termination level, it is equal to the sum of the egress TCP flows over all Streams. NOTE – The semantic of this TCP metric is similar with the managed object tcpRetransSegs of the TCP MIB [IETF RFC 4022]. |
|  | Type: | Unsigned Integer |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

## 9.5	Error codes

None.

## 9.6	Procedures

### 9.6.1	Overview – "TCP retransmission traffic" and connection models

Connection models with a single TCP-enabled termination/SEP are uncontroversial. The scope here is therefore on connection models with more than one termination/SEP. Figure 8 illustrates an example with a TCP-enabled stream endpoint pair (SEPP). H.248 tcprm statistics are activated on SEPT2(S1), thus only the traffic direction from X to Y is relevant.

Figure 8 – "TCP retransmission traffic" in (TCP, TCP) connection models

There are two fundamental configurations for a TCP-enabled SEPP:

- TCP merge or TCP relay mode: TCP retransmission traffic originates in X only; or

- TCP proxy mode: TCP retransmission traffic originates in SEPT2(S1) only.

However, the tcprm statistics are applicable in both scenarios.

### 9.6.2	Egress TCP traffic – Statistic "Retransmitted TCP Packets Sent"

Every outgoing, retransmitted TCP packet, sent from a H.248 TCP Stream/Termination, is counted by Statistic tcprm/tcprps.

### 9.6.3	Egress TCP traffic – Statistic "Retransmitted TCP Octets Sent"

The measurement represents the volume of TCP retransmission of all TCP flows of an H.248 Stream, i.e. across all outgoing, retransmitted TCP packets according to Statistic tcprm/tcpros.

### 9.6.4	Derived statistics

The H.248 statistics of this package together with other TCP related statistics (such as from [ITU-T H.248.84] allow the derivation of other TCP performance metrics. This clause provides one example.

#### 9.6.4.1	Performance metric "TCP efficiency"

This metric is defined in clause 4.2 in [b-IETF RFC 6349] according following equation:

Metric TCP efficiency for an H.248 stream/termination could be calculated (by the MGC), using the two H.248 statistics TCP Octets Sent (tcptv/tcpos, see clause 10.4.1/[ITU-T H.248.84]) and Retransmitted TCP Octets Sent (tcprm/tcpros).

# 10	Package-less TCP control

## 10.1	Package-less TCP control in other H.248.x-Recommendations

Pure SDP is used for the establishment of TCP bearer connection control by following Recommendations:

- [ITU-T H.248.69]: for MSRP-over-TCP bearers in MSRP switch configurations;

- [ITU-T H.248.84], clause 13: TCP merge and TCP relay mode configurations.

The (H.248) Property-based control method (according to clauses 7 to 9) is applied for stateful TCP handling by the MG, which covers TCP to non-TCP interworking (i.e., one SEP realizes a TCP endsystem mode), TCP-to-TCP interworking in TCP proxy mode and context configurations with more than two TCP-enabled H.248 terminations which are interconnected.

## 10.2	Principal H.248 control steps for communication establishment

### 10.2.1	Overview

Figure 9 indicates possible H.248 control steps concerning the establishment of TCP connections in environments demanding NAT traversal support:

Figure 9 – Principal H.248 control steps for establishing a native TCP bearer

It is apparent that some steps might be combined (e.g., 1 and 2) and that specific steps might be optional (e.g., 3, 5, 7).

Table 14 provides an inventory of possible control elements from H.248 perspective. The column "H.248/SDP element" refers to "package-less TCP control".

Table 14 – H.248 control steps and possible control elements

| H.248 control step |  | H.248 property | H.248/SDP element | Comments |
| --- | --- | --- | --- | --- |
| Establishment phase: "IP transport connection establishment" |  |  |  |  |
| 1 | Creation SEP / SEPP | ADD.req/MOD.req command |  |  |
| 2 | Bearer type (L4/L3) indication to SEP | Indirect: package elements used from TCP-related H.248 packages | "m="-line <proto> (clause 13.4 / [ITU-T H.248.84, H.248.69]) | SDP may be ambiguous due to "protocol stack" concept (NOTE 1). 2nd issue: bearer type is a LCD level characteristic, but not LD/RD related! |
| 3 | Block / Unblock L4 Bearer Control Procedures at SEP | Property: tcpbcc/bceb | "a=setup:holdconn" | High level semantic. |
| 4 | Start IP Transport Connection Establishment Procedure ("TCP Open") | TCP proxy mode: Event: tcpbcc/BNCChange Signal: tcpbcc/EstBNC | TCP proxy mode: not possible due to [ITU-T H.248.84] "a=setup:" semantics for LD/RD usage | Conditional, dependent on required TCP mode.TCP proxy mode needs to be supported, in TCP ACTIVE and PASSIVE OPEN. |
| 5 | Local L4/L3 NAT traversal support | [ITU-T H.248.37] | "a=setup:" for [ITU-T H.248.84] NAT-T | Complementary due to different NAT-T methods |
| 6 | Successfully established IP Transport Connection (State "EstablishedTCP") | Event: tcpbcc/BNCChange | — | MGC notification might be optional. |
| 7 | Local L4+ NAT traversal support | [ITU-T H.248.78] | e.g., "a=msrp-cema" | NOTE 2 |
| Communication phase |  |  |  |  |
| 8 | Active communication phase (i.e., transfer of application data) | - |  |  |
| NOTE 1 – The SDP "m=" line <proto> element allows the indication of a) a single protocol or b) a protocol stack (i.e., multiple protocol layers). The protocol stack based value is inherently ambiguous in case of the requirement in indicating a particular protocol layer out of the stack. This is a well-known issue of this element. NOTE 2 – The SDP based indication seems to be principally deficient ("application protocol specific SDP"), whereas [ITU-T H.248.78] supports the flexibility of different application protocols. |  |  |  |  |

### 10.2.2	Step 1: TCP-enabled termination/stream endpoint creation

Not SDP related, based on H.248 Command request.

### 10.2.3	Step 2: Bearer type indication "TCP"

The 'TCP' bearer type must be included according to the SDP-based method as defined by clause 7.6.1. This is applicable for both the package-less and the package-based approaches.

The SDP "m="-line <proto> element may or may not contain upper layer protocol information (above the TCP layer), leading to application-aware or application-agnostic indications.

TCP provides a bidirectional communication service, hence the SDP "m="-line <proto> values need to be consistent in the LD/RD concerning the L4 indication of 'TCP'.

### 10.2.4	Step 3: Block / Unblock TCP bearer control procedures at SEP

Block / Unblock TCP bearer control procedures at SEP are not supported by SDP, thus property tcpbcc/bceb required.

NOTE – [IETF RFC 4145] SDP codepoint "a=setup:holdconn" is intended to hold TCP bearer connection establishment, however, it is semantically unclear how an H.248 MG should process any incoming TCP traffic.

### 10.2.5	Step 4: Start TCP connection establishment procedure ("TCP Open")

When the SEP performs full TCP protocol termination (TCP proxy mode and TCP to non-TCP interworking) the [IETF RFC 4145] SDP "a=setup" attribute can not be used to derive the semantic of a "TCP active OPEN" or "TCP passive OPEN" procedure since it is mandatory to set the attribute to "actpass" as the indication of the TCP mode. Thus the use of this method (usage of values "active" and "passive" in the "a=setup" attribute) in cases where TCP protocol termination is required (as package-less approach) is deprecated. Furthermore:

- In a TCP active OPEN that method does not allow delayed establishment.

- If the SDP "m="-line <proto> element also contains other protocol layers (e.g, TLS, DTLS), the "a=setup" attribute may also trigger or influence the connection establishment at the upper layer.

The use of the signal tcpbcc/EstBNC triggers an immediate TCP bearer connection establishment. The use of this signal also implies TCP protocol termination, which is not applicable to a TCP relay and TCP merge mode.

The TCP proxy mode configuration implies a packaged-based approach and if the SDP "a=setup" attribute is used in the LD/RD then the SDP "a=setup" value settings shall be according to [ITU-T H.248.84], Table 1. Likewise for TCP to non-TCP interworking.

#### 10.2.5.1	Potential benefits of a package based approach for bearer establishment

The package based approach offers services which might be of interest for specific TCP-based communication services and network solutions:

- Capability to monitor the TCP bearer establishment or lack of it with the tcpbcc/BNCChange event;

- Capability to control, e.g. delay, the timing of a TCP bearer establishment procedure with the tcpbcc/EstBNC signal and tcpbcc/bceb property;

- Capability to block incoming TCP bearer establishment requests;

- Capability to decouple TCP bearer establishment procedures and upper layer bearer establishment procedures.

### 10.2.6	Step 5: Local L4/L3 NAT traversal support

Local L4/L3 NAT traversal support is basically controlled via [ITU-T H.248.37], [ITU-T H.248.50] or/and [ITU-T H.248.84], but could be complemented by SDP-based signalling e.g. in case of "ICE for TCP" [b-IETF RFC 6544].

### 10.2.7	Step 6: Reporting successfully established TCP connection

Reporting successfully established TCP connection is not possible via SDP, thus event tcpbcc/BNCChange is required.

### 10.2.8	Step 7: Local L4+ NAT traversal support

Local L4+ NAT traversal support is not possible via SDP for general L4+ NAT-T, thus [ITU-T H.248.78] is required.

NOTE – Some application-specific L4+ NAT-T support functions could be associated with application-specific SDP information, however, this is out of scope of this Recommendation.

### 10.2.9	Step 8: Active communication phase

Control of functions during the active communication phase (such as performance measurements or application data inactivity detection for TCP) is out of scope of the SDP.

## 10.3	Release of TCP connection

The release of a TCP connection is not possible via SDP. The hard removal of a TCP-enabled stream endpoint via an H.248 MODify.req or SUBtract.req command would lead to a complete release of MG level TCP resources but may not always ensure a correct and complete TCP closure handshake with the remote TCP bearer connection endpoint. Usage of signal tcpbcc/RelBNC removes the TCP resources but keeps other resources associated with the SEP. This represents a benefit of the package based approach in the TCP proxy case, especially in TCP to non-TCP scenarios.

# 11	Security considerations

The primary focus of this Recommendation is the H.248-based control of establishment and release of TCP bearer connections. Especially the establishment phase is critical from security perspective due to a plethora of well known attack scenarios, see e.g. [b-Bellovin1989] and [b-CPNI-TCP] and the references cited by these publications.

The IETF was (and is still active) developing extensions and practices in order to enhance security for the TCP, e.g., [b-IETF RFC 5925] or [b-IETF RFC 6528].

The H.248 MG could be consequently subject of a TCP security threat itself (particularily when terminating the TCP protocol, i.e., "TCP endpoint" or "TCP proxy" mode), or forward TCP traffic related to potential attacks of downstream network equipment.

In order to minimize any risk,

- the MGC could actively

- monitor the TCP bearer connection establishment by requesting notification about successfully established TCP-enabled terminations/stream endpoints;

- delay/block TCP data transfer as long as the TCP bearer connection establishment process is ongoing;

- the MG could be used for countering security attacks by enforcing policy rules on TCP/IP packets according e.g.,

- the filtering guidelines by clause 9.1.2.1 of [ITU-T H.248.79]; or

- a more advanced policing using DPI [b-ITU-T H.248.86].

Any concrete considerations with regards to the overall security architecture, trust models, network protection, etc. should be subject of the applied H.248 profile specification.

Annex A

State modelling for 
TCP bearer connection endpoints

(This annex forms an integral part of this Recommendation.)

## A.1	Introduction and purpose

The TCP basic connection control package (clause 8) is tightly coupled to a state model, because the event (tcpbcc/BNCChange) and signal (tcpbcc/EstBNC) are related to state transitioning behaviour. The detailed user plane state model for TCP is given by [IETF RFC 793]. However, a simplified view is sufficient from perspective of H.248 gateway control procedures.

The underlying state model of package tcpbcc is described by this Annex.

## A.2	Original state model for TCP bearer connection endpoints

The principle model for TCP is comprised of 12 states and defined by Figure 6/[IETF RFC 793], see Figure A.1.

Figure A.1 – TCP connection state diagram [IETF RFC 793]

## A.3	Simplified state model for H.248-based TCP basic connection control

The TCP basic connection control package makes a couple of assumptions, primarily:

- Bearer establishment:

- neglects TCP simultaneous open;

- interim states of 3-way handshake are not needed;

- Bearer release:

- one-way closure not considered;

- all interim states between the transitioning from TCP states ESTAB to CLOSED are not needed.

The resulting simplified model is depicted by Figure A.2:

Figure A.2 – Simplified state model for H.248-based TCP basic connection control

There are two remaining states, called Idle and Established in the H.248 state model. Signal tcpbcc/EstBNC is used for triggering state changes by the MGC, event tcpbcc/BNCChange indicates successfully completed state transitions to the MGC. In more detail, transitioning completed:

- to state Established when TCP-ACK (as third element of TCP three-way handshakes)

- received for incoming TCP bearer connection establishment procedure; and

- sent for outgoing TCP bearer connection establishment procedure

- to state Idle when TCP-FIN-ACK (as fourth element of two TCP half-closures)

- received for incoming TCP bearer connection release procedure; and

- sent for outgoing TCP bearer connection release procedure.

The MGC may block the establishment of an incoming TCP bearer connection request, e.g., whilst TCP role assignment is yet unclear or during ongoing call control signalling procedures, etc. This results in a third state Blocked from a H.248 perspective, see Figure A.3.

Figure A.3 – Extended state model for H.248-based TCP basic connection control

Comment to the notion of "blocking": The concept of blocking/unblocking of bearer connection procedures is a known concept from BICC (see e.g. [b-ITU-T Q.2630.1]).

## A.4	State models versus TCP modes of operation

Support of different TCP modes of operations is primarily driven by resource related cost factors and performance aspects. The amount of required MG resources nearly correlates with the number of considered TCP connection states. Table A.1 provides an overview about state modeling versus TCP modes.

Table A.1 – State models versus TCP modes of operation

| State model | TCP relay | TCP merge | TCP proxy (light) | TCP proxy (full) |
| --- | --- | --- | --- | --- |
| Signalling plane (H.248 model) | See Fig. A.2 or A.3 |  |  |  |
| Bearer plane (considered TCP connection states) | 0 ["stateless"] | 2 [NOTE 1] | 2 ≤ … < 12 [NOTE 2] | 12 [see Fig. A.1] |
| NOTE 1 – The MG roughly needs to keep track of transitioning from TCP state "CLOSED" to "ESTAB". NOTE 2 – The lightweight TCP proxy mode is not further detailed. |  |  |  |  |

Appendix I

Sample use-cases of TCP connection control

(This appendix does not form an integral part of this Recommendation.)

## I.1	Use case #1: WebRTC to NGN/IMS interworking function with termination of transport security by MG

This use case shows an MG which provides a transport layer interworking function between a WebRTC-based data application and a UE located in a NGN or IMS network. This is an example for "TCP to non-TCP" interworking (according to use case #1.2 in clause 6.1).

Background:

Web-based Real-Time Communication Services (WebRTC) include non-media (data) applications, which are using a datagram connection for data transport (see [b-IETF rtcweb-data]). The selected protocol solution results in a hierarchical protocol layering (data-over-SCTP-over-DTLS-over-ICE/UDP), driven by NAT traversal complexity, multiplexing and security related aspects.

The considered data applications (by WebRTC) would use legacy TCP transport in non-WebRTC IP network environments. [b-ITU-T H.248.WEBRTC] considers several variants of H.248 WebRTC gateways which are positioned for WebRTC interworking scenarios.

The setup of termination T2 is controlled by the TCP connection control package while the setup of termination T1 is out of scope of this Recommendation.

Figure I.1 – Use-case #1: WebRTC to IMS interworking function

Appendix II

Example call flows

(This appendix does not form an integral part of this Recommendation.)

## II.1	Overview

The example call flows illustrate the typical behaviour for basic TCP connection establishment and release. The establishment behaviour is related to the TCP mode of operation.

### II.1.1	Mode type: application agnostic TCP-proxy

The MGC needs to assign the TCP mode of operation to an H.248 context. This is fundamentally based on SDP information elements according to [ITU-T H.248.84], clause 13.5.

The MGC could use also additional explicit H.248 signalling elements for the SEP individual assignment of TCP client/server roles, via the indication of incoming or outgoing TCP bearer connection establishment procedures, using the tcpbcc package capabilities as defined by this Recommendation, see example in clause II.2.2.

## II.2	Basic TCP bearer connection control

### II.2.1	Network configuration

The call flow is based on the network configuration as shown in Figure II.1:

Figure II.1 – Configuration "application agnostic TCP-proxy"

The use case shows an MG with two SEPs (labelled as T1(S1) and T2(S1)) configured in an application agnostic TCP-proxy mode. The configuration and the call flow are characterized as follows:

- MGC strict control of the TCP bearer endpoints on both terminations

- Stream endpoint T1(S1): TCP-server, i.e., providing incoming TCP bearer establishment procedures ("TCP passive open" according [IETF RFC 793])

- Stream endpoint T2(S1): TCP-client, i.e., providing outgoing TCP bearer establishment procedures ("TCP active open" according [IETF RFC 793])

- TCP-connection for both terminations are setup in parallel

- TCP-connection released by remote TCP-endpoint of termination T1; application data may still flow from UE-Y to UE-X.

### II.2.2	Establishment

Figure II.2 illustrates a traffic flow example concerning the establishment of an end-to-end TCP connection between UE-X and UE-Y, which relates to two TCP connection segments from H.248 MG perspective (due to TCP proxy mode).

The example traffic flow indicates parts of application control signalling (here SIP/SDP with scope on SDP Offer/Answer information), gateway control signalling (H.248) and bearer plane traffic (here TCP packets for connection establishment).

The MGC follows an early reservation strategy of MG resources, thus initiates the creation of a "TCP-TCP" context in the MG already during still ongoing capability negotiations at application control level. In order to prevent accidental TCP state transitioning in the MG due to "too early TCP media", the MGC decides to block explicitly TCP bearer connection establishment (via property tcpbcc/bceb).

As soon as application control signalling allows, the MGC enforces incoming (at SEP T1(S1)) and outgoing (at SEP T2(S1)) TCP bearer connection establishment procedures.

The MGC requests to be notified of successfully established TCP bearer connection segments by setting the tcpbcc/BNCchange event in this example.

The signalling scenario is termed as "MGC strictly controlled" due to the fact that only the TCP basic connection control package is used. The interlinkage capability (according to [ITU-T H.248.92]) as described in clause 9 would tightly couple both TCP connection control procedures, see clause IV.3.2.

Figure II.2 – Traffic flow example for an MGC strictly controlled TCP bearer connection establishment using the TCP-proxy mode

### II.2.3	Release

Figure II.3 illustrates the traffic flow for the use case when the MGC does not know which remote TCP endpoint initiates TCP CLOSE procedures. Further, the default case of a bidirectional TCP bearer connection release is described. The MGC subscribes to event tcpbcc/BNCChange in order to be notified about release activities. It may be reminded that the notification is only issued when the local SEP has successfully transitioned to H.248 connection state IDLE.

Figure II.3 – Traffic flow example for an incoming TCP bearer connection release, explicitly propagated by the MGC in outgoing direction

Figure II.4 provides another release example: the MGC enforces a TCP bearer release by initiating parallel TCP bearer connection release procedures towards both remote TCP endpoints X and Y:

Figure II.4 – Traffic flow example for outgoing TCP bearer connection release procedures towards both remote TCP endpoints X and Y

## II.3	Extended TCP bearer connection control

### II.3.1	Establishment

This use case applies to the same configuration as the previous use case depicted in Figure II.2. There is no necessity for explicit TCP bearer establishment control by the MGC in this example, because e.g. of lacking NAT traversal support, or application specific, or security dedicated requirements. The direction of TCP bearer establishment is also unimportant from MGC point of view. The MGC only wants to wait with TCP establishment as long as application control level capability negotiations become stable, hence the MGC initially blocks possible establishment. Later on, TCP bearer establishment is unblocked and the interlinkage at the concerned stream endpoint pair is enabled, in both directions in the example of Figure II.5.

The notification of successful TCP bearer establishment is again an optional feature, up to the MGC level application/service control logic.

Figure II.5 – Traffic flow example for an extended TCP bearer connection establishment

### II.3.2	Release

Figure II.6 provides an example for an MG autonomous release of the TCP bearer connection. The MGC enables interlinkage in both traffic directions in this example, e.g. because the MGC does not know which remote TCP endpoint is initiating a TCP release.

Figure II.6 – Traffic flow example for an extended TCP bearer connection release

Appendix III

Illustration of the TCP basic connection control package protocol semantics

(This appendix does not form an integral part of this Recommendation.)

## III.1	Overview

The tcpbcc package (clause 7) defines the basic TCP bearer connection control with scope on support for establishment and release. The illustrated use cases are abstracted examples.

## III.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TCP) is highlighted besides the H.248 interface.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TCP bearer connection endpoint.

## III.3	Establishment of TCP bearer connections

### III.3.1	Successful establishment – Terminating side

See Figure III.1, termed as use case (E.1):

Figure III.1 – Successful establishment – Terminating side

The MGC explicitly blocks incoming TCP traffic handling (1). The MG discards any incoming TCP traffic (2). The MGC unblocks the SEP (3) and subscribes for state transitioning related to TCP bearer connection establishment (4). There is an incoming TCP bearer connection establishment procedure, given by a TCP three-way handshake (5, 6, 7). The TCP-ACK (7) leads to an H.248 state transition from Idle to Established, which again leads to the notification of the MGC (8).

### III.3.2	Successful establishment – Originating side

See Figure III.2, use case (E.2).

Figure III.2 – Successful establishment – Originating side

This scenario is similar to the previous one, with the exception of the following behaviour:

- MGC triggers outgoing TCP bearer connection establishment by signal EstBNC (4); and

- a corresponding TCP three-way handshake (5, 6, 7) in the opposite direction.

### III.3.3	Unsuccessful establishment

Figure III.3 illustrates principle unsuccessful establishment scenarios.

Figure III.3 – Unsuccessful establishment

The TCP three-way handshake is basically not completed, either in the incoming (see scenario (E.1), clause III.3.1) or outgoing (see scenario (E.2), clause III.3.2) direction. The H.248 state remains in Idle. The MGC may explicitly supervise the establishment process (1), see also clause 7.6.5.3.1.

The MG is able to detect unsuccessful establishment in this example, e.g., due to received TCP protocol control information (2).

## III.4	Release of TCP bearer connections

### III.4.1	Successful release – Terminating side

See Figure III.4, termed as use case (R.1).

NOTE: The event subscription (Rel) [1] may be sent before Established state (this comment is valid not only for this example).

Figure III.4 – Successful release – Terminating side

The MGC wants to be notified of successfully release TCP bearer connection (1). The MG receives a TCP-FIN (2) which starts a TCP bearer connection release by the remote TCP endpoint. The MG acknowledges the release requests (3) and initiates the closure of the other TCP traffic direction (4). The second TCP half-closure procedures is completed by (5), which leads to an H.248 state transition from Established to Idle, and the notification of the MGC (6).

### III.4.2	Successful release – Originating side

See Figure III.5, use cases (R.2) illustrates an outgoing, bidirectional TCP bearer connection release.

Figure III.5 – Successful release – Originating side

### III.4.3	Unsuccessful release

An unsuccessful TCP bearer connection  release procedure (Figure III.6) implies that the MG still remains in H.248 state Established and that the MGC could possibly suspect a yet to be established TCP bearer connection. There are multiple options how such protocol deadlocks could be resolved. The situation is cleared in this example by the subtraction of the termination by the MGC (1).

Figure III.6 – Unsuccessful release

Appendix IV

Illustration of the TCP-specific interlinkage procedures

(This appendix does not form an integral part of this Recommendation.)

## IV.1	Overview

Clause 8 describes TCP-specific stream endpoint interlinkage procedures based on [ITU-T H.248.92].  The effect of the interlinkage capability is illustrated in clause IV.3 ([ITU-T H.248.92] property seplink/linktopo applied on TCP; only the inter-SEP interlinkage variant is used). Clause IV.4 illustrates the application of the oneway release indicator (property tcpbcc/ori). Clause IV.5 describes examples of combined usage of both capabilities.

The illustrated use cases represent just a few examples. An exhaustive consideration of all possible combinations is out of scope.

## IV.2	Conventions

An H.248 context with a single SEPP, given by SEPs T1(S1) and T2(S1), is considered. The two   properties (seplink/linktopo and tcpbcc/ori) are related to individual SEPs (as opposed to Context-level properties); however, the scope is on SEP T1(S1) property settings only.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) are not discussed, but could be for instance related to state changes of the local TCP bearer connection endpoint(s) (as defined by the tcpbcc package).

## IV.3	Usage of stream endpoint interlinkage capability

Property seplink/linktopo is defined in clause 7.1.1/[ITU-T H.248.92] and TCP-specific interlinkage procedures are defined in clause 8. Interlinkage may be used for TCP bearer connection establishment and release procedures.

### IV.3.1	Property "seplink/linktopo" not used for interlinkage

There will be decoupled SEPs in case of unused interlinkage (abstracted by semantic FALSE in Figure IV.1), i.e., reflects the semantic of the tcpbcc package. There are two separate TCP bearer connection segments, - the two segments related to TCP connections "X  MG (SEPT1(S1))" and "Y  MG (SEPT2(S1))" -, from the perspective of the MG.

Figure IV.1 – Property "seplink/linktopo" not used for interlinkage

Here: an incoming TCP bearer connection establishment request (via TCP-SYN) or release request (via TCP-FIN) at SEP T1(S1) does not affect SEP T2(S1).

Figure IV.2 illustrates correspondent example TCP bearer establishment signalling scenarios:

Figure IV.2 – Signalling example for TCP bearer connection establishment, property "seplink/linktopo" not used for interlinkage

It may be emphasized that the seplink/linktopo setting at SEP T2(S1) does not have any impact in these scenarios.

If an incoming TCP bearer connection establishment request from remote X should also lead to establishment procedures towards remote TCP endpoint Y, then the MGC needs to be involved (see E.1 in the example).

### IV.3.2	Property "seplink/linktopo" enabled for establishment and release

A property value setting of seplink/linktopo equal to "<interlinkedSEP>:TCP:TCP:est,rel" leads to a forwarding of the TCP bearer connection control request within a SEPP (see Figure IV.3.).

Figure IV.3 – Property "seplink/linktopo" enabled for establishment and release at SEP T1(S1)

The TCP open and closure procedures, when initiated by remote TCP endpoints (X or Y), could now have an end-to-end scope. E.g., the three-way handshake for TCP bearer connection establishment or a TCP half-closure for oneway TCP bearer connection release would be led by remote TCP endpoints (X, Y) and forwarded via the SEPP, under the condition that both seplink/linktopo properties (of SEPT1(S1) and SEPT2(S1)) would be enabled for interlinkage.

Figure IV.4 illustrates correspondent signalling scenarios at the example of TCP bearer establishment:

NOTE – It should be noted that the first received TCP-SYN at T1(S1) is not MG-internally forwarded as a "TCP/IP packet", rather an internal, abstracted stimuli is generated which triggers the outgoing TCP-SYN at T2(S1).

Figure IV.4 – Signalling example for TCP bearer connection establishment, 
property "seplink/linktopo"

It may be emphasized that the seplink/linktopo setting at SEP T2(S1) does not have any impact in these scenarios (because the property only considers incoming procedures at SEP T1(S1)).

## IV.4	Usage of Oneway Release Indicator property

Property tcpbcc/ori is defined in clause 7.1.2. The property affects only TCP bearer connection release procedures.

### IV.4.1	Property "ori = FALSE"

See Figure IV.5. Semantic FALSE implies base package behaviour, i.e., a complete TCP bearer connection release.

Figure IV.5 – Property "ori = FALSE"

Here: an incoming TCP bearer connection release request (via TCP-FIN) at SEPT1(S1) triggers an immediate TCP half-closure of the reverse direction towards X, but does not affect SEPT2(S1).

### IV.4.2	Property "ori = TRUE"

Semantic TRUE results in a one-way TCP bearer connection release, see Figure IV.6.

Figure IV.6 – Property "ori = TRUE"

## IV.5	Combined consideration of both properties

Only TCP bearer connection release (due to ori property) needs to be considered.

### IV.5.1	Property settings: "ori = FALSE" and "seplink/linktopo" enabled for release

See Figure IV.7. The ori property codepoint leads to a complete TCP bearer connection release between MG and remote TCP endpoint X, and the seplink/linktopo property codepoint results in an initiation of a TCP half-closure procedure from SEPT2(S1) towards remote TCP endpoint Y. Whether the entire TCP bearer connection with Y will be completely released depends on remote TCP endpoint Y.

Figure IV.7 – Property settings: "ori = FALSE" and "seplink/linktopo" enabled for release

### IV.5.2	Property settings: "ori = TRUE" and "seplink/linktopo" enabled for release

See Figure IV.8. The ori property codepoint leads to an oneway TCP bearer connection release only between MG and remote TCP endpoint X, and the seplink/linktopo property codepoint results again in an initiation of a TCP half-closure procedure from SEPT2(S1) towards remote TCP endpoint Y. Whether the entire TCP bearer connection with Y will be completely released depends on remote TCP endpoint Y.

Figure IV.8 – Property settings: "ori = TRUE" and "seplink/linktopo" enabled for release

### IV.5.3	Signalling examples

Figure IV.9 summarizes the impact of both properties (Note: only enabled seplink/linktopo depicted; unused interlinkage does not provide any new information).

Figure IV.9 – Signalling example for TCP bearer connection release and example value combinations for both properties

Use case (C.1): the incoming TCP bearer connection release request from remote TCP endpoint X is propagated on the other side to remote TCP endpoint Y, and the tcpbcc package behaviour of a bidirectional TCP connection release towards X.

Use case (C.2): Property ori enforces a TCP half-closure in the direction to X, and seplink/linktopo the start of TCP bearer connection release towards Y.

Appendix V

Relation to similar H.248 packages

(This appendix does not form an integral part of this Recommendation.)

## V.1	Overview of TCP related packages

There are some existing packages which are either defined for TCP or could be theoretically reused for TCP, see Figure V.1.

Figure V.1 – Overview of TCP related packages

The specific relation to this Recommendation is described below.

## V.2	ITU-T Q.1950

This Recommendation introduces 11 H.248 packages for usage in so-called Bearer-Independent Call Control type of network architectures. Two out of the eleven could be principally applied (with extensions) for TCP networks:

### V.2.1	Package 'gb' (generic bearer connection):

TCP is a connection-oriented protocol which implies a connection state model (see Figure 6 in [IETF RFC 793]) in the bearer connection endpoints. This is a common characteristic with the gb package (Table V.1):

Table V.1 – Q.1950-defined H.248 packages – gb package

| Package name | Purpose |
| --- | --- |
| Generic bearer connection package | The gb package provides control elements for such stateful bearer technologies, which implies basic stimuli for the bearer control signalling protocol as well as a basic connection state concept. The bearer connection endpoint models supports also incoming and outgoing bearer control protocol procedures, which is e.g. required for the modelling of active/passive open (or close) procedures or client/server behaviour. |
| Possible relation to this Recommendation: |  |
| The TCP connection state model could be basically mapped on the underlying gb package state model. The gb package could be used for the majority of TCP connection control procedures. Open/differences: - TCP explicitly supports simultaneous bearer connection establishment and release, which would be so called collision scenarios in case of the gb package. - TCP bearer connection release: unidirectional concept (called half close), whereas the gb package considers bi-directionality only. |  |

### V.2.2	Package 'bcp' (bearer characteristics):

The notion of bearer characteristics relates to a particular (or multiple) protocol layer(s) in the bearer plane (see Table V.2):

Table V.2 – Q.1950-defined H.248 packages – bcp package

| Package name | Purpose |
| --- | --- |
| Bearer characteristics package | The existing bcp package supports an initial set of bearer technologies such as RTP/IP for IP-based networks. However, bearer type "TCP" is not supported. |
| Possible relation to this Recommendation |  |
| Usage of the gb package would imply the need for an explicit bearer type indication such as supported by the bcp package. Open/differences: - A new version or an extension package with additional codepoints would be required (at least bearer type "TCP", but possibly also "TLS", "TLS/TCP", "DTLS", "SCTP", "DCCP"). - Instead of the Property-based bearer type indication (via bcp package) at LCD level exists the alternative of the SDP-based bearer type indication (via "m=" line element proto) at LD/RD level. - Use case TLS-over-TCP: the establishment/release of the TCP connection and the TLS session could be basically controlled via the the gb package. The bearer connection control procedures at TCP and TLS level could be basically temporarily decoupled, which would imply the modification of the bcp/BNCChar property value, a scenario originally out of scope of the bcp package. |  |

### V.2.3	Package 'bnct' (bearer network connection cut through):

See Table V.3:

Table V.3 – Q.1950-defined H.248 packages – bnct package

| Package name | Purpose |
| --- | --- |
| Bearer network connection cut through package | The MG can indicate whether the cut through will occur "early" or "late", which are effectively related to bearer control protocol signalling events. Early relates to the bearer cutting-through on the establishment. Late refers to cutting-through on the confirmation. |
| Possible relation to this Recommendation |  |
| The aspect of "cut-through" control is basically relevant for TCP as well, perhaps even more important due to a) the assured transport characteristic of TCP (e.g., handling of acknowledgments, loss, retransmission) and H.248 MG TCP modes (such as relay, merge or proxy). Questions for clarification: - The explicit cut-through semantic might be also an implicit capability of TCP modes. - Unclear is the interaction with StreamMode property settings? Understanding: The "BNC Cut through" package doesn't actually control cut through, i.e. from clause A.4.5/[ITU-T Q.1950]: The MGC can ask the MG as to when the cut through of a particular bearer will occur, through the use of the BNCCT property. The MG can indicate whether the cut through will occur "early" or "late". Early relates to the bearer cutting-through on the establishment. Late refers to cutting-through on the confirmation. The BNCCT property in this package does not actually order the cut through nor does it indicate when the Cut-through has occurred. Furthermore, clause 4.4.10/[ITU-T Q.1950] BNC-cut-through-capability: Used by the bearer interworking functio (BIWF) to inform the call state machine (CSM) of the bearer cut-through capability (i.e. commits resources on the receipt of a Bearer SetupReq or confirm). The transactions in [ITU-T Q.1950] show that the usage depends on the BICC call setup direction i.e. forward direction. See also clause 7.2.1.2.2/[b-ITU-T Q.1902.4]. There’s also no interaction with H.248 StreamMode settings. See [b-ITU-T Q.Supp.32] for call flows showing the interaction. There’s also more of a discussion on the use of the Cut-through capability in clause 5.3.2 step 9. Conclusion: No relation to this Recommendation. |  |

### V.2.4	Package 'ri' (reuse idle):

See Table V.4:

Table V.4 – Q.1950-defined H.248 packages – ri package

| Package name | Purpose |
| --- | --- |
| Reuse idle package | The ri/RII property is used by the MG to indicate to the MGC that an idle bearer is to be reused. |
| Possible relation to this Recommendation: |  |
| Reuse of existing TCP connections is a supported concept, such as in SIP networks by [IETF RFC 4145.] See also clause 13.6/[ITU-T H.248.84]. Open/differences: - Possible models of MG-level TCP resources, e.g.: - resource component "TCP local source/destination transport address"; - resource component "TCP local stateful bearer connection endpoint" which would imply the TCP connection state machine; - resource component "end-to-end TCP bearer connection", based on the n-tuple of local and remote transport source/destination addresses and connection state; - others? - The semantic of reuse idle bearer would be then dependent on the agreed TCP resource model. It might be even transparent for the MG. Understanding: Reuse of idle procedures are a BICC Network option. Annex B/[b-ITU-T Q.1902.4] describes the overall procedures. MLPP (multi-level precedence and preemption) is a service that supports the reuse of IDLE bearers (i.e [b-ITU-T Q.850], cause 9). [b-IETF RFC 4411] discusses the use of this cause with respect to SIP and basically says that it’s not valid in an IP environment. The Reuse idle package is used within the framework of a call control that supports an application (i.e. MLPP) that has re-use logic. The RII property is used in conjunction with the BNC-ID etc to choose an existing bearer (i.e. bearer independent situations). Whereas the SDP "a=connection" attribute is really a flag to use an existing bearer because you don't want to release it rather than selecting an idle TCP connection i.e. can be used in “bearer dependent” situations. Conclusion: Whilst it is possible to reuse a TCP connection, there is not any real relation of this Q.1950 package to this Recommendation. |  |

## V.3	ITU-T H.248.67

### V.3.1	Package 'trm' (GCP transport mode indication):

See Table V.5:

Table V.5 – H.248.67-defined H.248 package – trm package

| Package name | Purpose |
| --- | --- |
| GCP transport mode indication package | Related to the H.248 Control Association, which may use TCP-based transport modes. |
| Possible relation to this Recommendation |  |
| None: [ITU-T H.248.67] and this Recommendation are both diametrically opposed. TCP usage for the H.248 signalling interface is out of scope of this Recommendation. |  |

## V.4	ITU-T H.248.84

The TCP enabled SEP will be activated through the indication of the TCP-proxy mode as defined in [ITU-T H.248.84]. This Recommendation only defines properties that are needed on top of [ITU-T H.248.84] for the control of a TCP enabled SEP.

This Recommendation does not define TCP-related statistics and events which are already covered by [ITU-T H.248.84]. Whenever needed the statistics and events of the related package as defined by [ITU-T H.248.84] must be used.

### V.4.1	Package-independent procedures for NAT-T with TCP bearers:

See Table V.6:

Table V.6 – H.248.84-defined package-independent procedures for NAT-T with TCP bearers

| Procedures | Purpose |
| --- | --- |
| Clause 13/[ITU-T H.248.84] | a) Indication of IP transport protocol 'TCP' b) Indication of 'TCP mode' for H.248 MG |
| Possible relation to this Recommendation |  |
| To a): - Normative for SDP-based bearer type indication "TCP". To b): - The TCP merge mode could be used as NAT traversal service. The TCP modes as such could interact with TCP connection control. E.g., the tcpcc package would be not required (and should be not used) for TCP connection establishment when NAT-T is used (because it provides already end-to-end TCP connection establishment (Note: TCP connection release is different). |  |

### V.4.2	Package 'tcphp' (TCP hole punching):

See Table V.7:

Table V.7 – H.248.84-defined H.248 package – tcphp package

| Package name | Purpose |
| --- | --- |
| TCP hole punching package | NAT traversal support service for TCP. |
| Possible relation to this Recommendation |  |
| Orthogonal to this Recommendation; the tcphp package could be used in conjunction with this Recommendation. |  |

### V.4.3	Package 'tcptv' (TCP traffic volume metrics):

See Table V.8:

Table V.8 – H.248.84-defined H.248 package – tcptv package

| Package name | Purpose |
| --- | --- |
| TCP traffic volume metrics package | Support of octet and packet count statistics for TCP bearer protocol. |
| Possible relation to this Recommendation |  |
| Performance monitoring is an orthogonal application; hence, the H.248 statistics could be basically used in conjunction with this Recommendation. |  |

### V.4.4	Package 'tcpccm' (TCP connection control metrics):

See Table V.9:

Table V.9 – H.248.84-defined H.248 package – tcpccm package

| Package name | Purpose |
| --- | --- |
| TCP connection control metrics package | Statistics related to the establishment process of the TCP bearer connections. |
| Possible relation to this Recommendation |  |
| Basically all three statistics could be used. |  |

### V.4.5	Package 'tcpcqm' (TCP connection quality metrics):

See Table V.10.

Table V.10 – H.248.84-defined H.248 package – tcpcqm package

| Package name | Purpose |
| --- | --- |
| TCP connection quality metrics package | Additional statistic related to the establishment process of TCP bearer connections. |
| Possible relation to this Recommendation |  |
| The round-trip time based statistic may be used as well. |  |

### V.4.6	Inventory of TCP functions and their possible impact on H.248 TCP gateways:

See Table V.11.

Table V.11 – H.248.84– Inventory of TCP functions

| Inventory in: | Purpose: |
| --- | --- |
| Appendix II/[ITU-T H.248.84] | - TCP Functions versus H.248 TCP Modes of Operation, in ten areas: - Basic Capabilities - Topology Hiding - Protocol Encryption - Security - Application Data Inactivity Detection - Interactions with other policy rules - Performance measurements & statistics" - Connection keep alive support - Add-on's to TCP - IP layer operations |
| Possible relation to this Recommendation: |  |
| This informative analysis is principally valid as well. |  |

## V.5	ITU-T H.248.43 and ITU-T H.248.79

[ITU-T H.248.43] and [ITU-T H.248.79] are related to the filtering of TCP traffic. See overview in clause 9/[ITU-T H.248.79].

Bibliography

[b-ITU-T H.248.86]	Recommendation ITU-T H.248.86 (2013), Gateway control protocol: H.248 support for deep packet inspection.

[b-ITU-T H.248.WEBRTC]	Draft Recommendation ITU-T H.248.WEBRTC (2014), Guidelines on the use of H.248 capabilities for web-based real-time communication services in H.248 profiles.

[b-ITU-T Q.850]	Recommendation ITU-T Q.850 (1998), Usage of cause and location in the Digital Subscriber Signalling System No. 1 and the Signalling System No. 7 ISDN user part.

[b-ITU-T Q.1902.4]	Recommendation ITU-T Q.1902.4 (2001), Bearer independent call control protocol (Capability Set 2): Basic call procedures.

[b-ITU-T Q.2630.1]	Recommendation ITU-T Q.2630.1 (1999), AAL type 2 signalling protocol.

[b-ETSI TR 183 068]	ETSI TR 183 068 v3.1.1, Telecommunications and Internet Converged Services and Protocols for Advanced Networks (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[b-IETF RFC 2923]	IETF RFC 2923 (1990), TCP Problems with Path MTU Discovery.

[b-IETF RFC 4411]	IETF RFC 4411 (2006), Extending the Session Initiation Protocol (SIP) Reason Header for Preemption Events.

[b-IETF RFC 5925]	IETF RFC 5925 (2010), The TCP Authentication Option.

[b-IETF RFC 6349]	IETF RFC 6349 (08/2011), Framework for TCP Throughput Testing.

[b-IETF RFC 6528]	IETF RFC 6528 (2012), Defending against Sequence Number Attacks.

[b-IETF RFC 6544]	IETF RFC 6544 (2012), TCP Candidates with Interactive Connectivity Establishment (ICE).

[b-Bellovin1989]	Morris, R. (1989), "Security Problems in the TCP/IP Protocol Suite", Computer Communications Review, Vol. 19, No. 2, pp. 32 - 48.

[b-CPNI-TCP]	Centre for the Protection of National Infrastructure (CPNI) (2009), "Security Assessment of the Transmission Control Protocol (TCP)", 
<http://www.gont.com.ar/papers/tn-03-09-security-assessment-TCP.pdf>

____________________