| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 181 (WP 1/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Sapporo, 30 June-11 July 2014 |
| TD |  |  |
| Source: | Editor H.248.STGROUP |  |
| Title: | H.248.STGROUP “Gateway Control Protocol: H.248 Stream grouping and aggregation” (New): Output draft (Sapporo, Japan, 30 June – 11 July 2014) |  |

1.	Introduction

This document contains the Editor’s output draft for H.248.STGROUP “Gateway Control Protocol: H.248 Stream grouping and aggregation” (New).

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.STGROUP Ed. 0.1 | Initial baseline text for H.248.STGROUP. Based on the output TD-22 from the RGM in Geneva, 10 - 14 March 2014 and C.523r1, C.582r1 and C.459 from the Sapporo, July 2014 ITU-T SG16 meeting. |

CONTENTS

Page

1 Scope 4 2 References 5 3 Definitions 5 3.1 Terms defined elsewhere 5 3.2 Terms defined in this Recommendation 6 4 Abbreviations and acronyms 6 5 Conventions 7 6 H.248.1 defined stream grouping and aggregation 7 6.1 Reserve Group 7 6.2 Stream multiplexing using transport address information 7 7 Further issues with existing H.248 stream grouping mechanisms 9 7.1 Scope of Signals / Events and Statistics 9 7.2 Scope of Topology 10 8 Media Grouping package 10 8.1 Properties 10 8.1.1 Group Semantics 10 8.1.2 Stream Aggregation 11 8.2 Events 12 8.3 Signals 12 8.4 Statistics 12 8.5 Error Codes 12 8.6 Procedures 12 8.6.1 Support of media grouping 12 8.6.2 Mapping of group attributes in H.248 12 8.6.3 MG Behaviour 13 8.6.4 Error handling 17 8.6.5 Usage of stream aggregation 18 8.6.6 Topology and aggregation streams 22 Bibliography 24

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 1 – Example single and multiplexed RTP streams 8 Figure 2 – Multiplexed Stream 18 Figure 3 – Signals on an aggregation stream with FID semantic 19 Figure 4 – Events on an aggregation stream with SRF semantic 20 Figure 5 – Events on an aggregation stream with ANAT semantic 21 Figure 6 – Statistics on an aggregation stream with ANAT semantic 22 Figure 7 – Interworking between grouped streams and single stream 23 Figure 8 – Example of a multiple stream aggregation into a single stream 23 Figure 9 – Example of invalid direct stream and aggregation stream connection between two terminations 24

Draft new Recommendation ITU-T H.248.xx (ex H.248.STGROUP)

Gateway Control Protocol: 
H.248 Stream grouping and aggregation

AAP Summary

<Mandatory material to be provided before Consent>

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

As described by the H.248 connection model (clause 6/[ITU-T H.248.1) a termination sources and/or sinks one or more streams. In a multimedia conference, a termination can be multimedia and sources or sinks multiple media streams. The H.248 protocol uses the stream concept to model individual media flows. That is typically there is one media (and optional control flow) per H.248 stream. The characteristics of each stream is described through the use of the local control, local and remote descriptors. Each termination contains a group of streams. H.248 makes no assumptions as to the relationship between the streams on a termination other than indicating that streams exiting a termination shall be synchronised with each other (clause 7.1.4/ITU-T H.248.1).

Certain network technologies may require a media gateway (MG) to have knowledge of how two or more streams relate to each other beyond simply knowing that they shall be synchronised. Where there is a multiplicity of streams there may be a different relation between different sets of streams. For the purposes of this Recommendation the relation between streams is defined as the “grouping semantic”. Depending on the grouping semantic the MG may be required to apply certain processing to the media related to the streams. For example: one semantic may indicate that the media is to be duplicated on separate streams, another semantic may indicate media is only sent on one of two streams at a time. Some grouping semantics require that media flows from multiple streams are aggregated into a single flow.

H.248.1 does not support a means to describe grouping semantics. This Recommendation defines the “media grouping package” that allows a media gateway controller (MGC)  to group media from two or more H.248 streams according to a particular semantic and optionally to aggregate the media from the grouped streams according to the semantic.

{Editor’s note (2013-06): the package proposal isn’t yet complete, the current Media Grouping package in sub-clause 10 is only giving a general way to indicate the group characteristics of multiple media streams, however the following aspects are identified during the meeting for further investigation .

o IP endpoint transport multiplexing. Is it only IP or other transports?

Contributions are solicited.}

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.7]	Recommendation ITU-T H.248.7 (2004), Gateway Control Protocol: Generic Announcement package.

[ITU-T H.248.47]	Recommendation ITU-T H.248.47 (2008), Gateway Control Protocol: Statistic conditional reporting package.

[ITU-T H.248.58]	Recommendation ITU-T H.248.58 (2008), Gateway Control Protocol: Packages for application level H.248 statistics.

[ITU-T H.248.61]	Recommendation ITU-T H.248.61 (2013), Gateway Control Protocol: Packages for network level ITU-T H.248 statistics.

[ITU-T H.248.65]	Recommendation ITU-T H.248.65 (2009), Gateway control protocol: Support of the resource reservation protocol.

[IETF RFC 3524]	IETF RFC 3524 (2003), Mapping of Media Streams to Resource Reservation Flows.

[IETF RFC 4566]	IETF RFC 4566 (2006), SDP: Session Description Protocol.

[IETF RFC 4588]	IETF RFC 4588 (2006), RTP Retransmission Payload Format.

[IETF RFC 5583]	IETF RFC 5583 (2009), Signaling Media Decoding Dependency in the Session Description Protocol (SDP).

[IETF RFC 5888]	IETF RFC 5888 (2010), The Session Description Protocol (SDP) Grouping Framework.

[IETF RFC 5956]	IETF RFC 5956 (2010), Forward Error Correction Grouping Semantics in the Session Description Protocol.

[IETF draft-ietf-mmusic-sdp-bundle-negotiation]	IETF draft-ietf-mmusic-sdp-bundle-negotiation (201X), Negotiating Media Multiplexing Using the Session Description Protocol (SDP).

{Editor’s note: Title has changed to change}

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1		media sink [IETF XYZ]: FIXTHIS

3.1.2		media source [IETF XYZ]: FIXTHIS

3.1.4		H.248 media stream [ITU-T H.248.1]: FIXTHIS

3.1.5		Stream Semantics: [IETF RFC 5888] the purpose for which the media streams are grouped.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Aggregation: the process of combining two or more source streams on entering the H.248 context in the MG into one single stream, called aggregation stream. The way the source streams are combined is dependent on the grouping semantics defined for the source streams. It may consist of selecting one of the source streams, applying a reparation procedure to one of the source streams based on the information from another source stream, etc.

3.2.2	Aggregation stream: a stream that sources media to two or more streams and/ or sinks media from two or more streams which are related to each other with a certain grouping semantics. The aggregation stream exists only in the interior of the context. Within one context a stream may be defined as an aggregation stream in one termination and as a normal stream in another termination.

3.2.3	Deaggregation: the inverse process of  aggregation. The way source streams are generated from an aggregation stream is dependent of the grouping semantics. It may consist on delivering media to only one of them at a time, providing a replica, generating repairing or redundancy information, etc.

3.2.4	Source stream: each one of the streams external to the context that sources media to an aggregation stream when entering the context or which receives media from the aggregation stream when leaving the context. Also, each of the streams defined as belonging to a media group.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| AMR | Adaptive Multi Rate |
| --- | --- |
| ANAT | Alternative Network Address Types |
| DDP | Decoding Dependency |
| ID | Identity |
| FEC | Forward Error Correction |
| FEC-FR | Forward Error Correction Flow Repair |
| ID | Identity |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| LD | Local descriptor |
| LS | Lip Synchronization |
| MG | Media gateway |
| MGC | Media gateway controller |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NGN | Next generation network |
| PCM | Pulse Code Modulation |
| RD | Remote descriptor |
| RSVP | Resource Reservation Protocol |
| RTCP | RTP Control Protocol |
| RTP | Real Time Protocol |
| SDP | Session description protocol |
| SRF | Single Reservation Flow |
| SEP | (H.248) Stream endpoint |
| SEPP | (H.248) Stream endpoint pair |
| SIP | Session initiation protocol |
| UDP | User datagram protocol |
| WEBRTC | Real-Time Communication in WEB-browsers (as work item in W3C) |

# 5	Conventions

None.

{Editor’s note: Further work needs to occur on the definitions of / conventions for the terms for aggregation / grouping and multiplexing.}

# 6	H.248.1 defined stream grouping and aggregation

This clause describes the existing H.248.1 mechanisms related to grouping of streams.

## 6.1	Reserve Group

Within a single stream H.248.1 allows multiple sets of local and/or remote parameters to be specified. This is accomplished through the use of the ReserveGroup attribute (clause 7.1.7.1.2/[ITU-T H.248.1]). This allows a MGC to request that a MG reserve resources for functions described in each group. Media packets may be sent or received according to any of the groups although only a single alternative may be active at any given time. The Reserve Group concept does not describe any grouping semantic. Clause 8.6.3 describes several grouping semantics where the reserve group concept may be used.

## 6.2	Stream multiplexing using transport address information

{Editor’s note: How the multiplexing clause relates to stream grouping needs further explanation.}

Typically a single media stream is assigned to a single H.248 Stream endpoint by specifying a unique address/port combination in the Local Descriptor per H.248 Stream on a Termination along with the associated media properties. In general to indicate the use of a multiplexed media stream where multiple streams are sent using the same transport address, the MGC requests the same address/port in the Local Descriptor for all H.248 Streams that are to make up the multiplexed media stream.

For text based H.248 SDP is used for each Local Descriptor. In order to request to the same address/port the following conditions must be must:

- The value of the c-line must be the same for each Local Descriptor

- The value of m-line [IETF RFC 4566] <port> field is the same for each Local Descriptor.

A MGC may fully specify the address/port information for multiple H.248 Streams in a single Termination in an Add.req command. However in many cases the MGC will wildcard “choose” this information, in order for the MG to provide the address/port. In the case where wildcarding is used the MGC can:

- request the address/port information for one of the H.248 Streams involved in the multiplex and then use the same address/port information in subsequent Mod.req/s for additional H.248 Streams, or

- request address/port information for all the H.248 Streams involved in the multiplex and then select the address/port from one of the H.248 Streams and then send a subsequent Modify.request to modify the address/port information. This has the disadvantage over option a) that some resources may have been allocated to the previous address/port and these allocations would have to be changed.

Figure 1 shows example syntax for a Termination (T1) utilising two RTP Streams and Termination (T2) utilising one RTP Stream for two H.248 Streams.

| Two H.248 Streams, Two RTP Streams | Two H.248 Streams, One RTP Stream |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10003 { Context = 1 { Add = T1 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2223 RTP/AVP 34 } } } } } } | MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T2 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2222 RTP/AVP 34 } } } } } } |

Figure 1 – Example single and multiplexed RTP streams

On reception of an Add, Modify or Move request command with several Local Descriptors from the same Termination containing the same address/port information the MG shall treat the associated H.248 Streams as a multiplexed media stream. Multiplexing is applied according to the type or profile of the stream. For example, in the case that a media profile RTP/AVP is given RTP multiplexing is applied. Each media stream component of the overall multiplexed RTP Stream is coded according to the information Local Descriptor / H.248 Stream for that particular media.

RTP multiplexing is described by [IETF RFC xxxx].

{Editor’s note: a specific reference to the RTP multiplexing procedures as defined by the IETF AVT and MMUSIC groups is expected to be added.}

This multiplexed media stream may contain more than one media type.

Multiple multiplexed media streams are also possible on a Termination.

Individual components of the media multiplex can be modified by changing the Local and/or Remote Descriptors of the related H.248 Stream. If the address/port information of a H.248 Stream is changed to be different from the other components of the multiplexed media stream then the media defined by the H.248 Stream will longer be part of the multiplex and it will be its own media stream.

Clause 8.6.3 describes several grouping semantics where the use of transport address for multiplexing may be used.

# 7	Further issues with existing H.248 stream grouping mechanisms

This clause describes issues associated with the stream grouping described in clause 6.

## 7.1	Scope of Signals / Events and Statistics

Signals, Events and Statistics may be set at either a Termination or Stream level.  Generally when set at a Termination level the scope of this elements applies to all of the H.248 Streams on the Termination. For example: the octets sent statistic would be the sum of octets sent by each H.248 Stream and by corollary each media stream.

When these elements are set at a Stream level the scope of the element is given by the StreamID. In the case of the octets sent statistic it would contain only the octets sent by the H.248 Stream given by the StreamID.

When applying grouping to media Streams the above behaviour should be considered. The grouped Streams may consitute a sub-set of the total H.248 Streams in a Termination. For example: StreamID(1) could be a single stream, StreamIDs(2&3) could be grouped media Streams. This leads to complications regarding the scope of how Signals, Events and Statistics are applied. For example: A Termination level octets sent Statistic would correctly give the number of octets sent by the Termination however there is no way to request an octets sent Statistic related to the grouped media streams. The MGC would have to request a statistic for StreamID(2) and another statistic for StreamID(3) and add them together.

This issue is also applicable to Signals and Events. This is no way to indicate that a Signal or Event is only applicable to those grouped streams

The Media Grouping package defined in this Recommendation introduces an aggregation stream concept that allows signals, events and statistics to be set against an aggregation stream. What effect these elements have is based on the semantics associated with the aggregation.

## 7.2	Scope of Topology

For some of the media grouping semantics a MG may be required to interconnect a group of media streams at one interface with a single stream or a group of a different number of streams at another interface. This type of connectivity cannot be described with the existing topology descriptor.

The Media Grouping package defined in this Recommendation introduces an aggregation stream concept. This allows an MGC to indicate a topology for the aggregation stream.

# 8	Media Grouping package

|  | Package Name: | Media Grouping |
| --- | --- | --- |
|  | Package ID: | mgroup (0x????) |
|  | Description: | This package provides a general way for the MGC to instruct the MG to group media streams defined in different stream descriptors within the same media descriptor. |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

### 8.1.1	Group Semantics

|  | Property Name: | Group Semantics |
| --- | --- | --- |
|  | Property ID: | groupse (0x0001) |
|  | Description: | This property allows a MGC to indicate which media streams to group and for which purpose they are grouped. This allows the MG to allocate resources accordingly. For example: providing a common address/port in response to a wildcarded request from the MGC in case multiplexing is supported. In addition the optional “Direction” element allows to indicate whether the grouping is applicable in both directions or in one particular direction. When not included a default of “bothway” is assumed. Several sets of grouped streams are allowed per Termination. |
|  | Type: | Sub-list of String |
|  | Possible values: | Each element of the sub-list follows the group-attribute syntax in [IETF RCF 5888], here represented using type “Group”: Group = Semantics 1*WSP StreamIDList[1*WSP Direction] ; (NOTE 1) WSP = SP/HTAB StreamIDList = StreamID *(WSP StreamID) StreamID = UINT16 Semantics = token token-char = %x21 / %x23-27 / %x2A-2B / %x2D-2E / %x30-39/ %x41-5A / %x5E-7E token = 1*(token-char) Direction = Bothtoken / Sendtoken / Receivetoken Bothtoken = “B” Sendtoken = “S” Receivetoken = “R” (NOTE 2) As defined by [ITU-T H.248.1 Annex B]. Note 1: The syntax functionally duplicates the Grouping attribute syntax from [IETF RFC 5888] with the optional addition of a direction parameter. Note 2: This syntax definition allows for a Stream ID list containing one single value. This case is only allowed in a reply to an AuditCapabilities command, where the StreamId shall be set to 0. |
|  | Default: | None. |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |

### 8.1.2	Stream Aggregation

|  | Property Name: | Stream Aggregation |
| --- | --- | --- |
|  | Property ID: | stragg (0x0002) |
|  | Description: | This property indicates that the stream that this property is set on is a aggregation of the source streams specified in the property. The source streams must be grouped according to the grouping semantics indicated by the groupse property. |
|  | Type: | Sub-list of integer |
|  | Possible values: | Each value in the list is a Stream ID. |
|  | Default: | None |
|  | Defined in: | Local Control |
|  | Characteristics: | Read/Write |

## 8.2	Events

None.

## 8.3	Signals

None.

## 8.4	Statistics

None.

## 8.5	Error Codes

None.

## 8.6	Procedures

### 8.6.1	Support of media grouping

Usage and support of media grouping may be pre-agreed in a profile specification or may be discovered via the auditing capability.

An MGC may audit the Packages Descriptor to determine if a MG supports media grouping. If the response contains the “Media Grouping Package” (mgroup) it can assume that the MG support media grouping. The MGC may perform an H.248 AuditCapability request command on the groupse property to determine which semantics are supported by the MG. In this case the MG shall respond with the supported semantic tags and the Stream ID set to 0.

An MGC controlling a MG that supports the Media Grouping package can instruct the MG to group two or more sent and/or received media streams. Each media stream (including the sent and received directions) is described in its own Stream Descriptor and all are included in the same Media Descriptor.

The MGC can indicate to the MG:

- Which sent media streams to group

- Which received media streams to group and for which purpose the media streams are grouped.

In general the mgroup properties can be used together with the H.248 Reserved Group concept (see clause 7.1.7/[ITU-T H.248.1] as the ReserveGroup property applies to alternatives related to a particular H.248 stream. The mgroup package relates multiple H.248 streams. If ReserveGroup is used with the mgroup package properties, the MGC should ensure that any semantic is applicable to each of reserved groups on the applicable stream.

### 8.6.2	Mapping of group attributes in H.248

An MGC that sends or receives SDP media grouping related attributes in an SDP Offer/Answer dialogue should map those attributes as follows when sending the command to the MG:

- Each m-block is mapped into one Stream Descriptor.

- The “a=mid” attribute of each m-block is dropped in the Local/Remote Descriptor of the corresponding Stream Descriptor, instead the StreamID is used for the index in the group.

- The “a=group” attribute is mapped to the mgroup/groupse property and included in the TerminationState Descriptor. The content of the property, a sub-list of string, should be follow a copy of the “a=group” attribute. The semantic is retained and the mid value is replaced with the corresponding StreamID.

Example:

An MGC sends an SDP Offer with the following media description (see Table 1).

Table 1 – Example command encoding– (SIP) SDP Offer

| (SIP) SDP encoding | Comments |
| --- | --- |
| v=0 o=Alice 289083124 2890844526 IN IP4 124.124.124.222 c=IN IP4 124.124.124.222 a=group:LS 1 2 m=audio 30002 RTP/AVP 97 a=mid 1 a=rtpmap:97 AMR/8000 a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2; \ mode-change-neighbor = 1; mode-change-capability=2; max-red=0 m=video 52738 RTP/AVP 99 a=mid 2 b=TIAS:256000 b=AS:270 a=rtpmap:99 H264/90000 a=fmtp:99 profile-level-id=428014; a=sendrecv | It describes a session containing one video stream and one audio stream. They use the “LS” [IETF RF 5888] grouping semantics to indicate that the play-out of both media must be synchronized. |

The MGC maps this description into the following Media Description (Table 2).

Table 2 – Example command encoding – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Media { TerminationState { mgroup/groupse = ["LS 1 2"]}, Stream = 1 { Remote { c=IN IP4 124.124.124.222 m=audio 30002 RTP/AVP 97 a=rtpmap:97 AMR/8000 a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2; \ mode-change-neighbor = 1; mode-change-capability=2; max-red=0 } }, Stream = 2 { Remote { c=IN IP4 124.124.124.222 m=video 52738 RTP/AVP 99 b=TIAS:256000 b=AS:270 a=rtpmap:99 H264/90000 a=fmtp:99 profile-level-id=428014; a=sendrecv } } } | NOTE - A similar mapping would apply to the Local Descriptor. The Local Descriptor is not shown but it is also part of the Media Descriptor and will be sent to the MG. |

### 8.6.3	MG Behaviour

The behaviour of the MG with respect to the grouping is largely dependent on the semantic associated with the grouping, e.g. the value of semantic in the mgroup/groupse properties.

The SDP parameter IANA registry (http://www.iana.org/assignments/sdp-parameters/) contains a list of defined SDP group semantics. In general the MG should adopt the behaviour defined by the RFC (or other standard) that defines the semantic taking into consideration H.248 protocol behaviour. The following sub-clauses discuss several semantics.

#### 8.6.3.1	Lip Synchronisation (LS)

The “Lip Synchronisation” semantic is defined in [IETF RFC 5888]. As per clause 7/[IETF RFC 5888], the MG must synchronise the play out of H.248 Streams listed together in the same “LS” marked group.

##### 8.6.3.1.1	Directionality

In most scenario where LS is used both the video and audio would be bi-directional. However there are some scenarios such as synchronization of source audio and simultaneous translation, here lip sync may only be applied in one direction. [IETF RFC 5888] does not define an SDP mechanism to negotiate an unidirectional use of the LS semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. The mgroup/groupse property allows for defining a unidirectional use of the LS semantics also if the grouped streams are all bi-directional. However, as this use does not have an SDP counterpart between SDP peers, it is discouraged.

#### 8.6.3.2	Flow Identifier (FID)

The “Flow Identifier” semantic is defined in [IETF RFC 5888]. The use of the flow identifier with H.248 is in some cases redundant due to the H.248 connection model and H.248 usage of SDP. H.248 has the StreamID identifier which uniquely identifies media flows. Each H.248 stream may utilise the Reserve Group/Reserve Value (see 7.1.8.2.3/[ITU-T H.248.1]) construct which enables multiple configurations to be set for a H.248 stream. This allows a single H.248 stream to have multiple sets of characteristics of which one set is active at a particular point in time.

The following example described in 8.4.1/[IETF RFC 5888] can be emulated with the use of the Reserve Group mechanism:

Example 1

Two audio streams are negotiated between two peers with different audio codecs. Only one is active at a time while the other one is muted. The peers indicate the same IP address for receiving both streams but different ports.

The following example also described in 8.4.1/[IETF RFC 5888] cannot be emulated with the use of the Reserve Group/Reserve Value constructs and therefore requires the use of the H.248 mgroup/groupse property:

Example 2

Three audio streams are negotiated between two peers. The first two are bi-directional and use different audio codecs. Only one of these two is active at a time while the other one is muted. The third is unidirectional and has two payload types, one corresponding to each of the codecs used by the other two streams. This third stream always carries a copy of the active stream, either of the first or the second stream. Both active streams are received at the same IP address at different ports. The third stream is received at a different IP address.

NOTE: Reserve Group/Reserve Value can apply to alternatives using the same or different IP addresses or ports.

Another example use of the FID semantics that cannot be emulated with the ReserveGroup/Reserve Value constructs is the one described in 8.7/[IETF RFC 4588]:

Example 3

Two SDP peers negotiate one audio and one video stream. In addition they negotiate another audio and another video streams used for retransmission. The audio source and retransmission streams are grouped with the FID semantics and the video source and retransmission streams are grouped with the FID semantics.

##### 8.6.3.2.1	Directionality

[IETF RFC 5888] does not define a mechanism to negotiate an unidirectional use of the FID semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. The mgroup/groupse property allows for defining a unidirectional use of the FID semantics also if all the grouped streams are bi-directional. However, as this use does not have an SDP counterpart between SDP peers, it is discouraged.

#### 8.6.3.3	Single Reservation flow (SRF)

The “Single Reservation flow” semantic is defined in [IETF RFC 3524]. It indicates that media (described by m=lines) should be mapped to the same resource reservation flow. [ITU-T H.248.65] defines a package for the support of RSVP. The initiation of a resource reservation request is typically explicitly requested through the use of a H.248 signal. Any identification of flow is part of the parameters to the signal.

If the wanted behaviour is the allocation and subsequent policy enforcement of a predefined total bandwidth for a number of combined streams, the use of the SRF semantics is required as opposed to allocate individual bandwidths for each stream. The use of [ITU-T H.248.65] may also be applicable in combination with the mgroup package. In that case sent or received RSVP messages will refer to all grouped media combined. For that reason, signals and events of the rsvp package should be sent or activated in the aggregation stream when they are conveying RSVP messages related to a grouped media streams (see clauses 8.6.5.1 and 8.6.5.2).

##### 8.6.3.3.1	Directionality

[IETF RFC 3524] does not define a mechanism to negotiate a unidirectional use of the SRF semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. Thus it must be assumed that the negotiation of multiple streams grouped with the SRF semantics between peers is bi-directional if the grouped streams are also bi-directional.

However, this semantic is commonly associated with RSVP to reserve the required bandwidth. RSVP is unidirectional in nature. Thus the SRF semantics may require two RSVP sessions. The use of RSVP in a MG requires [ITU-T H.248.65] which has the inherent mechanisms to determine the applicable direction, e.g. sending of an RSVP Path message with the H.248 rsvp/path signal is related to the outgoing stream while sending an RSVP Resv message with the H.248 rsvp/resv signal is related to the incoming stream.

If the mgroup/groupse property is used with the direction parameter together with the SRF semantics, the sent or received RSVP messages should apply or not to all grouped streams depending on the direction. For example, if the MGC sends mgroup/groupse = “SRF 1 2 R” to the MG, an RSVP Resv message sent by that termination will refer to both streams 1 and 2 combined and the MGC should set the rsvp/resv signal in the aggregation stream, but an RSVP Path message will refer only to the individual stream where the signal rsvp/path is set..

#### 8.6.3.4	Alternative Network Address Types (ANAT)

The “Alternative Network Address Types” semantic is defined in [b-IETF RFC 4091]. The ANAT semantic allows an SDP offerer to indicate that multiple network address types may be used for a single logical media stream. This semantic is redundant in H.248 as it offers multiple methods for indicating or requesting address types. A MGC may use “over specification” (see 7.1.8.2.2/[ITU-T H.248.1) to provide a list of network address types for a particular H.248 Stream. A MG will then chose an appropriate network address type.

An MGC that wants to initiate an SDP offer with the ANAT semantics can reserve resources for both transport types using two property groups. Both property groups will contain the same information except for the IP address information (e.g. the c= attribute) which will be partially wildcarded, and will request an IPv4 address in one of the groups and an IPv6 address in the other group. The MGC will set ReserveGroup to true. Thereafter it will send the SDP offer.

The MGC that receives the SDP offer with the ANAT semantics may be able to determine by itself which of the transport types to use, e.g. by certain call or traffic distribution criteria. In that case it will only indicate one address type when adding the termination in the MG. The MGC may also delegate that decision to the MG. In that case it will send two property groups, like the initiating MGC, but in this case, it will set ReserveGroup to false. The MG will only allocate resources for one of the transport types and indicate the chosen one to the MGC in the ADD reply.

The initiating MGC, when receiving the SDP answer which contains the chosen transport type, may send a H.248 Modify command to the MG, this time excluding the property group with the transport type that was not chosen.

#### 8.6.3.5	Forward Error Correction (FEC)

The “Forward Error Correction” semantic is defined in [IETF RFC 5956]. The use of “FEC” has been deprecated and should not be used with H.248.

#### 8.6.3.6	Forward Error Correction FR (FEC-FR)

The “Forward Error Correction FR” semantic is defined in [IETF RFC 5956]. This is used to associate source and repair flows for the purposes of forward error correction (FEC). If this semantic is received the MG should utilise the source and repairs H.248 Streams listed together in the same “FEC-FR” marked group for the purposes of FEC.

The MGC should indicate whether a flow is a source or repair flow through the use of coding related parameters (i.e. a=rtpmap:111 1d-interleaved-parityfec/90000 and a=fmtp:111 L=10; D=10; repair-window=400000).

If the session consists only of one source stream and one repair stream, the MG can unambiguously identify by means of the above attribute, which is the source and which the corresponding repair stream. The mgroup/groupse property may be omitted in this case. However, if there are multiple source streams and/or multiple repair streams, it may not be possible to unambiguously identify to which source each repair stream corresponds. Furthermore, the forward error correction mechanism allows for different types of correction schemes: n repair streams may be related to m source streams. In all these cases the SRF semantics is required to indicate the association between repair and source streams.

#### 8.6.3.6.1	Directionality

[IETF RFC 5956] does not define a mechanism to negotiate an unidirectional use of the FEC-FR semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. The mgroup/groupse property allows for defining a unidirectional use of the FEC-FR semantics also if all the grouped streams are bi-directional. However, as this use does not have an SDP counterpart between SDP peers, it is discouraged.

Thus, the preferred method to specify the use of forward error correction only in one direction is with the use of StreamMode (SendOnly or RecvOnly) or the SDP direction attribute (a=sendonly or a=recvonly) in the repair stream(s).

#### 8.6.3.7	Decoding Dependency (DDP)

The “Decoding Dependency” semantic is defined in [IETF RFC 5583]. This semantic may be used with H.248 as well as other SDP attributes (e.g. a=depend) to describe decoding dependencies for layered codings.

[IETF RFC 5583] does not define a mechanism to negotiate a unidirectional use of the DDP semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. The mgroup/groupse property allows for defining a uni-directional use of the DDP semantics also if all the grouped streams are bi-directional. However, as this use does not have an SDP counterpart between SDP peers, it is discouraged.

#### 8.6.3.8	Multiplexing Negotiation Using Session Description Protocol (BUNDLE)

The “BUNDLE” semantic is defined in [IETF draft-ietf-mmusic-sdp-bundle-negotiation]. It used to describe that two or more RTP streams use a single 5-tuple (i.e. multiplex multiple RTP streams onto a single 5-tuple). As [IETF draft-ietf-mmusic-sdp-bundle-negotiation] only describes the use of BUNDLE for IP transports it shall not be used with Streams using non-IP transports.

As described in clause 7 it is possible to request that a MG use a multiplexed RTP stream however there are a number of deficiencies.

Firstly the MGC is unaware if the MG supports RTP multiplexing. In order to determine if the MG supports RTP multiplexing the MGC may audit the media grouping package to determine if the “BUNDLE” semantic is supported (see clause 8.6.1).

Secondly there is no way to wildcard CHOOSE the address/port information from several H.248 Streams and request that a single transport connection (i.e. multiplexed RTP stream) be used. The media grouping package contains the “group semantics” property that, when including the BUNDLE semantic allows the MGC to indicate that two or more H.248 Streams are multiplexed together. MGs that support the media grouping package should check the “group semantics” property when processing wildcarding requests for multiple H.248 Streams. If the H.248 Streams match the association given by this property then the same value should be returned for those H.248 Streams.

##### 8.6.3.8.1	Directionality

BUNDLE is symmetric in nature. If it is used, it applies to source and destination.

In practice, however, the required behaviour from the MG specific to this semantic, is related to the Local Descriptor, i.e. to the received media. The use of the same address and port for multiple streams in the Remote Descriptor is usually not problematic in the MG, also when BUNDLE is not used.

Therefore the use of the direction parameter in the mgroup/groupse property is superfluous in case of the BUNDLE semantics.

### 8.6.4	Error handling

In the following error cases the MG should return an error code 473 “Conflicting Property Values”:

- If any of the stream identities included in the groupse property does not exist in the termination.

- If the stragg property is set without a value in the groupse property indicating the semantic associated with the aggregation stream, i.e. a value in the groupse property with the same streamID list as in the aggregation.

- Depending on the grouping semantics if there is a mismatch between the mgroup/groupse property and the Local or Remote Descriptor.

For example: An error may be generated if the MGC uses the BUNDLE media grouping and specifies a StreamID of a H.248 Stream whose Local Descriptor does not have the same address/port as the other H.248 Streams in the group (and is not updated in the current command). Another example is if an error may be generated if MGC uses the “LS” semantic and specifies the StreamIDs of H.248 streams where the MG cannot maintain lip sync. A more specific error code may be returned if available. The underlying reasons for generating an error may depend on the semantic associated with the group.

### 8.6.5	Usage of stream aggregation

In some grouping scenarios multiple media streams on entering the context may be aggregated into one media stream and, in the other direction, may result from the reverse operation of splitting one media stream into several media streams. Clause 6.2/[ITU-T H.248.1] describes the multiplexing of Terminations through the use of a multiplex termination. However it does not support multiplexing of streams within a Termination.

The mgroup package defines the concept of an Aggregation Stream. By setting the stragg property on an aggregation stream indicating the source streams it allows stream aggregation to be modelled internally in the MG. Media is then aggregated /de-aggregated to/from the source streams to the aggregation stream according to the applicable grouping semantics.

Figure 2 illustrates the concept.

Figure 2 – Multiplexed Stream

In figure 2 media streams labelled media 1, 2, and 3 received as described by the Stream descriptors Sid1, Sid2, Sid3 will be aggregated according to the groupse semantic for those streams. This aggregation will be sent via stream Sid4 to Termination Tb. It is then sent externally to the media gateway as described by the Stream descriptor for Stream Sid4 on Termination Tb.  The MG may apply any interworking/adaptation etc. functions that would normally apply when the same media stream is set on two Terminations. The aggregation stream may also remain isolated to one Termination in a Context.

The behaviour of the aggregation stream is well determined by the grouping semantics and the media descriptors of the corresponding source streams. Thus the local control, local, and remote descriptors are not needed in the aggregation stream. If set the values of the properties should be compatible with those defined to the source streams and in no case should override any value set in those streams.

When entering or leaving the context, the source streams have a send and receive relationship with the aggregation stream. Thus the stream mode does not need to be set in the aggregation stream. If set, it will be ignored by the MG.

A MGC may apply signals, events, statistics and topology to the aggregation stream as per normal streams. The behaviour is discussed in more detail in the sub-clauses below.

#### 8.6.5.1	Signals and aggregation streams

A signal applied to the aggregation stream will have the effect of being played out on one or more source streams. Which source streams the signal is played out over depends on the grouping semantic associated with the aggregation. Signals may still be specified to be played on the individual source streams. In this case the playout is isolated to that particular source stream.

Figure 3 illustrates an example scenario using the Flow Identification semantic.

Figure 3 – Signals on an aggregation stream with FID semantic

Example 1:

Termination Ta has 4 streams including an aggregation stream. It has two audio streams with identities 1 and 2 which use the same codec. They are grouped with the FID (Flow Identification) semantics [IETF RFC 5888]. Thus they carry identical content. One is a replica of the other, usually sent to a different destination address. The termination has in addition other media stream with identity 3, which is not part of the media group. Termination Ta has an aggregation stream with streams 1 and 2 as the source. Requesting an an/apv [ITU-T H.248.7] signal on stream 4 to play an audio announcement should result in the announcement being sent synchronously to streams 1 and 2.

Example 2:

Termination Ta has 4 streams, including an aggregation stream. It has two audio streams with identities 1 and 2 which use different codecs. They are grouped with the FID semantics [IETD RFC 5888].  At a time only one of the streams carries data in each direction while the other one is muted in that direction. This is dependent on the codec choice by the end user equipment which can change dynamically. Termination Ta has an aggregation 4 stream with streams 1 and 2 as the source. Requesting an an/apv [ITU-T H.248.7] signal on stream 4 to play an audio announcement should result in the announcement being sent in the stream (1 or 2) which carries media at that moment.

Example 3:

A termination Ta has one video stream and two audio streams with identities 1, 2 and 3 grouped with the SRF (Single Reservation Flow) semantics [IETF RFC 3524]. Thus resource reservation using RSVP and policy enforcing is applied to the complete group. The MGC defines an aggregation stream 4. In order to send RSVP messages related to all grouped streams, the MGC should set the signals of the rsvp package [ITU-T H.248.65], e.g. rsvp/path on the aggregation stream 4.

Note: RSVP sessions are normally defined by the triplet destination address, protocol ID, destination port. For the identification of the session related to grouped media see [IETF RFC 3524].

#### 8.6.5.2	Events and aggregation streams

An event applied to the aggregation stream will have the effect of detecting events on the source streams as a result of stimulus usually from outside the context. How the event is reported is based on the grouping semantic associated with the aggregation stream. Events may still be specified on the source streams. In this case detection is isolated to that particular source stream.

Where an event is set at the termination level on a termination that has an aggregation stream the effect will be that it is only set against the aggregation streams and stream not involved in an aggregation.

Example 1:

Figure 4 – Events on an aggregation stream with SRF semantic

Figure 4 shows an example where a Context has a Termination Ta with one video stream and two audio streams with identities 1, 2 and 3 grouped with the SRF (Single Reservation Flow) semantics [IETF RFC 3524]. Thus resource reservation using RSVP and policy enforcement is applied to the complete group. The MGC has defined an aggregation stream with stream identity 4. In order to receive RSVP messages related to all grouped streams, the MGC should request the events of the rsvp package [ITU-T H.248.65], e.g. rsvp/pathr event on stream 4. The detection of a RSVP path message related to stream1, 2 and 3 results in the notification of a single ObservedEvent from the aggregation stream.

Note: RSVP sessions are normally defined by the triplet destination address, protocol ID, destination port. For the identification of the session related to grouped media see [IETF RFC 3524].

Example 2:

Figure 5 – Events on an aggregation stream with ANAT semantic

Figure 5 shows an example where a Context has a termination Ta. It has two audio streams which use different codecs. They are grouped with the FID (Flow Identification) semantics [IETF RFC 5888]. Thus, at any point in time, only one of them carries data while the other one is muted. The termination has in addition other media stream with identity 3, not part of the media group. The MGC has defined an aggregation stream 4. The MGC requests the MG to collect data volume related statistics on the aggregation stream. The MGC uses also the conditional reporting functionality [ITU-T H.248.47] and activates therefore the scr/cr event on the aggregation stream and refers to the relevant statistics in the si parameter.

#### 8.6.5.3	Statistics and aggregation streams

When statistics are set against an aggregation stream, the statistics are collected as if they had been set against all the source streams. For example: if the rtp/pr “RTP packets received” statistic is set on an aggregation stream it will count the sum of RTP packets received from all the source streams.

When statistics are set at the termination level on a termination that has an aggregation stream the effect will be that it is only set against the aggregation streams and streams not involved in an aggregation.

Example 1:

Figure 6 – Statistics on an aggregation stream with ANAT semantic

Figure 6 shows an example where a Context has a termination Ta that handles three audio streams with identities 1, 2 and 3 grouped with the BUNDLE semantic. The termination has in addition another media stream with identity 4, not part of the media group. The MGC has defined an aggregation stream with identity 5. The MGC wishes to collect statistics on the data volumes sent and received in the bundled group at different levels (network, transport, application). It uses the Network Package os and or [ITU-T H.248.1], the IP Layer Octets Counts Statistics Package ipos, ipor, ipps and ippr [ITU-T H.248.61] and the RTP Application Data Package payloados and payloador [ITU-T H.248.58] statistics. It requests these statistics by including them in the statisticsDescriptor of the streamDescriptor of stream 5. The MG should collect statistics in each of the three source streams and sum up the values.

### 8.6.6	Topology and aggregation streams

An aggregation stream allows the MG to transfer the media from/to several source media streams into/from one single stream or a different set of source media streams

The requested behaviour from the MG depends on the grouping semantics but it will usually involve some dynamic switching of the connectivity between the terminations included in the topology command based on external conditions. This dynamic switching will not be controlled by the MGC.

Example 1:

Figure 7 illustrates a scenario where a context connects two terminations. One of them has three audio streams: stream with identity 1 (sendrecv) uses PCM A-law, stream with identity 2 (sendrecv) uses AMR and stream with identity 3 (sendonly) uses PCM A-law. The three streams are grouped with the FID [IETF RFC 5888] semantics. The other termination has one single stream with two payload types, PCM A-law and AMR.

Figure 7 – Interworking between grouped streams and single stream

In order to connect the set of streams at Ta with the single stream at Tb, the MGC defines stream identity 4 as an aggregation stream on termination Ta. It then sets stream identity 4 on termination Tb. Terminations Ta and Tb are then by default bothway connected through stream 4 (Figure 9).

Figure 8 – Example of a multiple stream aggregation into a single stream

NOTE - the direction attributes shown in the figure correspond to the MG terminations’ views.

P1 (Figure 7) can change the codec dynamically to send data. According to the FID semantics, when PCM encoded audio is received externally in Tb, Ta forwards it to P2 in stream 1 and as a replica in stream 3 –it may be with a different IP destination address if specified like that in the Remote Descriptor- and stream 2 remains muted. When AMR encoded audio is received in T1, T2 forwards it to P2 in stream 2 while stream 1 and stream 2 remain muted.

When PCM encoded audio is received externally in Ta in stream 1 it is forwarded to Tb via stream4. When AMR encoded audio is received externally in Ta stream 2 it is forwarded to Tb via stream 4.

It is possible for a stream to be connected to other Terminations directly of via an aggregation stream.

Figure 9 – Example of invalid direct stream and aggregation stream connection between two terminations

It is not possible for a normal stream to be directly connected to other terminations in a Context and be connected at the same time to the same termination through an aggregation stream. Figure 9 shows an example of such a case where two terminations are connected via a normal stream (stream identity 1) and an aggregation stream (stream identity 4).

There may be multiple aggregation streams on a Termination. A stream cannot be source of more than one  aggregation streams if the aggregation streams are connected between the same terminations, i.e. it is not possible for a stream to be connected to another termination through more than one aggregation stream.

However a stream can be source of multiple aggregation streams if at most one (either the source or one of the aggregation streams) is connected to another specific termination. This can be the case e.g. with aggregation streams defined for statistics gathering, sending signals or detecting events, which can have an isolated topology from other terminations.

The MGC can modify topology related to an aggregation stream by using its identity in a topology descriptor. For example using figure 8 the topology may be set Topology(Ta,Tb,oneway,Sid4), Topology(Tb,Ta, Oneway, Sid4), Topology(Ta,Tb, isolate, Sid4,) or Topology(Ta,Tb,bothway, Sid4).

Bibliography

[b-IETF RFC 4091]	IETF RFC 4091 (07/2005), The Alternative Network Address Types (ANAT) Semantics for the Session Description Protocol (SDP) Grouping Framework.

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title.

_______________________