| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 241 R1 (PLEN/16) |
|  |  | English only Original: English |
| Question(s): | 5/16 | Sapporo, Japan, 30 June – 11 July 2014 |
| TD |  |  |
| Source: | Editor H.420 |  |
| Title: | Consent: H.420 (ex F/H.TPS-Arch) “Telepresence system architecture” (New) |  |

1.	Introduction

This TD contains the draft new H.420 (ex F/H.TPS-Arch) “Telepresence system architecture”, which is proposed for Consent at this SG16 meeting.

AAP Summary

H.420 describes a high level telepresence system architecture describing how telepresence endpoints, multi-point control units, call and resource controllers, gateways, and management systems are used in a communication session to provide telepresence. It also provides guidance on telepresence room design.

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Recommendation 6 4 Abbreviations and acronyms 6 5 Conventions 7 5.1 Requirement terminology 7 5.2 Icons used 8 6 High level architecture 9 6.1 Description of sub-system components 10 6.1.1 Telepresence endpoint 12 6.1.2 Multipoint control unit 13 6.1.3 Call and resource controller 13 6.1.4 Gateway 13 6.1.5 Management system 13 6.1.6 Legacy conferencing endpoints 13 6.1.7 Network 13 7 Conference control functions 14 7.1 Media control functions 14 7.1.1 Chair control 14 7.1.2 Media rendering 15 7.2 Meeting arrangement 16 7.3 Endpoint management 16 8 Sub-system architectures 17 8.1 Endpoint architecture 17 8.1.1 Endpoint architecture description 18 8.2 Multipoint control unit 23 8.3 Call and resource controller 24 8.4 Gateway 25 9 H.323 architecture 25 9.1 Mapping telepresence on a H.323 architecture 25 9.1.1 Peer-to-peer communication 27 9.1.2 Multi-point communication 28 9.2 Call process 28 10 Telepresence room design 29 10.1 Room layouts 29 10.1.1 Single row seating three-screen telepresence system 30 10.1.2 Multi row seating three-screen telepresence system 30 10.1.3 Multi row seating four-screen telepresence system 31 10.1.4 Single row seating six-screen telepresence system 32 10.2 Room characteristics 34 10.2.1 Room dimensions and shape 34 10.2.2 Wall, floor and ceiling surfaces 35 10.2.3 Furniture 35 10.2.4 Lighting 36 10.2.5 Acoustics 36 10.2.6 Heating ventilation and air conditioning (HVAC) 36 10.2.7 Power requirements 37 10.2.8 Network connectivity 37 Bibliography 38

List of Tables

Page

Table 1: Telepresence system call control interfaces 11 Table 2: Telepresence system management interfaces 11 Table 3: Telepresence system media interfaces 11

List of Figures

Page

Figure 1: Icons used in this Recommendation 8 Figure 2: High level telepresence architecture network model 9 Figure 3: Telepresence system functional components 10 Figure 4: Endpoint architecture 17 Figure 5: Video capture unit architecture 18 Figure 6: Camera models: converging camera model (left), parallel camera model (middle), con-optical centre camera model (right) 19 Figure 7: Video render unit architecture 20 Figure 8: Audio capture unit architecture 21 Figure 9: Audio render unit architecture 22 Figure 10: H.323 telepresence network model 25 Figure 11: H.323 telepresence system architecture 25 Figure 12: H.323 telepresence peer-to-peer communication 27 Figure 13: H.323 telepresence multi-point communication 28 Figure 14: H.323 telepresence call processes 29 Figure 15: Three-screen telepresence room layout (single row) 30 Figure 16: Three-screen telepresence room layout (multi-row) 31 Figure 17: Four-screen telepresence room layout (Multi row) 32 Figure 18: Screen and camera layout for six-screen telepresence 33 Figure 19: Six-screen telepresence room layout 34

Draft new Recommendation H.420 (ex F/H.TPS-Arch)

Telepresence system architecture

Summary

This Recommendation describes a high-level telepresence system architecture. It describes sub-system components: telepresence endpoints, multi-point control units, call and resource controllers, gateways, management systems and how these are used in a communication session to provide telepresence. It also provides guidance on telepresence room design in terms of room layout and room characteristics.

# 1	Scope

This Recommendation first describes a telepresence system in a high level technology agnostic manner taking into consideration the use cases and requirements defined in [ITU-T F.734]. This is then followed by a more in depth description of the components. The later parts of this Recommendation then describe how [ITU-T H.323] relates to a telepresence system. It then details aspects regarding telepresence room design.

Unless explicitly stated in the text, the interfaces depicted in the figures and functions in this Recommendation are logical and thus do not necessarily represent a particular physical implementation.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.734]	Recommendation ITU-T F.734 (2014), Definitions, requirements, and use cases for Telepresence Systems.

[ITU-T H.243]	Recommendation ITU-T H.243 (10/2005), Procedures for establishing communication between three or more audiovisual terminals using digital channels up to 1920 kbit/s.

[ITU-T H.245]	Recommendation ITU-T H.245 (05/2011), Control protocol for multimedia communication.

[ITU-T H.320]	Recommendation ITU-T H.320 (03/2004), Narrow-band visual telephone systems and terminal equipment.

[ITU-T H.323]	Recommendation ITU-T H.323 (12/2009), Packet-based multimedia communications systems.

[ISO 8995]		ISO 8995 (2002), Lighting of indoor work places.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	Audio mixing [b-IETF RFC 7262]: refers to the accumulation of scaled audio signals to produce a single audio stream.  See RTP Topologies, [b-IETF RFC5117].

3.1.2	Layout [b-IETF RFC 7262]: How rendered media streams are spatially arranged with respect to each other on a telepresence endpoint with a single screen and a single loudspeaker, and how rendered media streams are arranged with respect to each other on a telepresence endpoint with multiple screens or loudspeakers.

NOTE - Audio as well as video is encompassed by the term layout--in other words, included is the placement of audio streams on speakers as well as video streams on video screens.

3.1.3	Remote [b-IETF RFC 7262]: Sender and/or receiver on the other side of the communication channel (depending on context); i.e., not Local.  A remote can be an endpoint or an MCU.

3.1.4	Render [b-IETF RFC 7262]: the process of generating a representation from a media, such as displayed motion video or sound emitted from loudspeakers.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	endpoint: For the purposes of this Recommendation an endpoint is a device representing the logical point of final termination through receiving, decoding and rendering, and/or point of origin through capturing,  encoding, and sending of media streams, e.g. a H.323 terminal.

3.2.2	telepresence media codec: This term (and function) is used to describe any media (i.e. audio or video) codec that would be applicable for use in a telepresence system.

3.2.3	telepresence system: A set of functions, devices and network elements which are able to capture, deliver, manage and render multiple high quality interactive audio and video signals in a telepresence conference. An appropriate number of devices (e.g. cameras, screens, loudspeakers, microphones, codecs, MCUs, PCs) and environmental characteristics are used to establish telepresence.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| ACU | Audio Capture Unit |
| --- | --- |
| ARU | Audio Render Unit |
| CRC | Call and Resource Controller |
| GK | Gatekeeper |
| GW | Gateway |
| HDTV | High Definition Television |
| HVAC | Heating, Ventilation and Air Conditioning |
| IP | Internet Protocol |
| ISDN | Integrated Services Digital Network |
| IU | Interactive Unit |
| LAN | Local Area Network |
| LCD | Liquid Crystal Display |
| MCU | Multipoint Control Unit |
| NAT | Network Address Translation |
| NU | Network Unit |
| PC | Personal Computer |
| PDP | Plasma Display Panel |
| PLMN | Public Land Mobile Network |
| PSTN | Public Switched Telephone Network |
| QoE | Quality of Experience |
| QoS | Quality of Service |
| RAS | Registration, Admission and Status |
| RTP | Real-Time Protocol |
| RTCP | RTP Control Protocol |
| SIP | Session Initiation Protocol |
| TCU | Telepresence Control Unit |
| TMC | Telepresence Media Codec |
| TMIO | Telepresence Media Input / Output control unit |
| TMP | Telepresence Media Processor |
| VCU | Video Capture Unit |
| VRU | Video Render Unit |
| WAN | Wide Area Network |

# 5	Conventions

## 5.1	Requirement terminology

In this Recommendation the following conventions are used:

- "Shall" indicates a mandatory requirement.

- "Should" indicates a suggested but optional course of action.

- "May" indicates an optional course of action rather than a recommendation that something take place.

## 5.2	Icons used

Figure 1 shows the meaning of icons used in this Recommendation.

Figure 1: Icons used in this Recommendation

# 6	High level architecture

Figure 2: High level telepresence architecture network model

Figure 2 shows a high-level telepresence architecture including: telepresence endpoints, legacy video conferencing endpoints, legacy audio endpoints, legacy non-H.323 endpoints including H.320 and SIP endpoints, Multipoint Control Units (MCU), Call and Resource Controllers (CRC), Gateways (GW), the management system, etc. There may be multiple forms of telepresence endpoints, such as three-screen endpoints and single-screen endpoints. The architecture supports inter-operation of heterogeneous telepresence endpoints and inter-operation between telepresence endpoints and legacy video conferencing and audio endpoints.

There may be multiple MCUs and CRCs in the architecture.

The architecture may have multiple GWs that connect to other types of networks, such as fixed and mobile telephone networks.

## 6.1	Description of sub-system components

A telepresence system is made up of several functional components. These components are illustrated in Figure 3.

Figure 3: Telepresence system functional components

Figure 3 shows telepresence system functional components and the call control and media interfaces between them. All interfaces are assumed to be bi-directional. There may be multiple instantiations of the components. In this case there may also be addition call control or media interfaces between them. For example there may be a call control interface between call and resource controllers. The exception is the management system. Whilst there may be multiple systems this is treated as a monolithic system in this architecture.

The interfaces are numbered according to Tables 1 through 3.

The possible Call Control interfaces are illustrated in Table 1.

The possible management interfaces are illustrated in Table 2.

The possible media interfaces are illustrated in Table 3.

Table 1: Telepresence system call control interfaces

| Number | Between Components |  |
| --- | --- | --- |
| C1 | Telepresence Endpoint | Call and Resource Controller |
| C2 | Telepresence Endpoint | MCU |
| C3 | Telepresence Endpoint | Gateway |
| C4 | Telepresence Endpoint | Legacy Endpoint |
| C5 | Telepresence Endpoint | Telepresence Endpoint |
| C6 | Legacy Endpoint | Call and Resource Controller |
| C7 | Legacy Endpoint | MCU |
| C8 | Legacy Endpoint | Gateway |
| C9 | Legacy Endpoint | Legacy Endpoint |
| C10 | Call and Resource Controller | MCU |
| C11 | Call and Resource Controller | Gateway |
| C12 | Call and Resource Controller | Call and Resource Controller |
| C13 | MCU | Gateway |
| C14 | MCU | MCU |

Table 2: Telepresence system management interfaces

| Number | Between Components |  |
| --- | --- | --- |
| MT1 | Call and Resource Controller | Management System |
| MT2 | MCU | Management System |
| MT3 | Gateway | Management System |
| MT4 | Telepresence Endpoint | Management System |
| MT5 | Legacy Endpoint | Management System |

Table 3: Telepresence system media interfaces

| Number | Between Components |  |
| --- | --- | --- |
| M1 | Telepresence Endpoint | MCU |
| M2 | Telepresence Endpoint | Gateway |
| M3 | Telepresence Endpoint | Legacy Endpoint |
| M4 | Telepresence Endpoint | Telepresence Endpoint |
| M5 | Legacy Endpoint | MCU |
| M6 | Legacy Endpoint | Gateway |
| M7 | Legacy Endpoint | Legacy Endpoint |
| M8 | MCU | Gateway |
| M9 | MCU | MCU |

### 6.1.1	Telepresence endpoint

The telepresence endpoint is the logical point of final termination, and may consist of more than one physical device. The main functions of the telepresence endpoint include:

- Setting up call sessions through capability negotiation,

- Capturing, encoding, packetizing and sending local media streams,

- Receiving, de-packetizing, decoding and rendering remote media streams,

- Managing ongoing sessions,

- Releasing sessions / resources.

In contrast to a telepresence endpoint, an MCU may also send and receive media streams, but it cannot initiate or terminate media streams through capturing and/or rendering. There may be multiple forms of telepresence endpoints, such as: 3 camera/screen endpoints, single camera/screen endpoints, audio-only endpoints and etc.

Endpoint characteristics include:

- The number of main cameras, spatial relationships of cameras and capture capabilities, such as: resolution, frame rate and aspect ratio.

- Viewpoint and field of view for main cameras, including overlap regions, missing regions or whether images of cameras can be composited to a seamless image across the entire depth range. The number of presentation cameras and capture capabilities, such as: resolution, frame rate and aspect ratio.

- The video stream capabilities for data sharing such as: resolution, frame rate and aspect ratio.

- The number, type and size of main screens, spatial relationships of screens and screen capabilities, such as: resolution, frame rate, aspect ratio, and screen border width.

- The number and type of presentation screens, spatial relationship of presentation screens and screen capabilities.

- The number of microphones and the spatial relationships between the captured audio signals from those microphones.

- The number of loudspeakers, and the spatial relationships between the rendered audio signals.

- Display strategies for multipoint conference, such as the static or dynamic mapping of camera-to-display.

In general endpoint characteristics are transmitted through signalling streams in session initialization and used for capability negotiation to enable inter-operation of different telepresence endpoints. The characteristics of an endpoint may change during a session and may result in the need for additional media streams, modification of existing media streams or the removal of media streams.

When signalling its characteristics, an endpoint should take into consideration that it may be communicating with endpoints that have different capabilities and as such offer a range of characteristics to enhance interoperability between the endpoints. An endpoint should also offer characteristics that match the intended application. For example: if an application demands high resolution video then an endpoint would not offer low resolution video.

NOTE - [ITU-T H.245] defines a terminal as "any endpoint and may be a user's terminal or some other communication system such as an MCU or an information server". As such the term H.323 telepresence terminal is equivalent to the term "telepresence endpoint".

### 6.1.2	Multipoint control unit

The MCU is a device that connects two or more telepresence endpoints together to one single telepresence conference. An MCU can setup and control multiple videoconferencing call sessions. After receiving media streams from multiple endpoints or cascading MCUs and GWs, an MCU processes the media streams and sends the processed media stream to an endpoint, MCU or GW.

An MCU must support video and audio mixing and media adaptation for different types of endpoints and should also support control functions, for example, auxiliary stream token control, user selection of video sources and composition, automatic video switching based on talker detection and other user and administrative control functions.

An MCU should participate in the signalling negotiating the characteristics of the media in order to minimise the need for mixing or adapting media between endpoints where possible. This is to improve the QoS / QoE of the media.

### 6.1.3	Call and resource controller

A CRC is the resource management device for video conferencing and can provide resource management for endpoints, MCUs and GWs. The functions of a CRC include: address resolution, access control and bandwidth management. In H.323 networks this may be realised as a Gatekeeper and in SIP networks as a proxy server.

### 6.1.4	Gateway

A gateway connects various types of networks and provides interworking of signalling and media streams, such as interworking H.323 and SIP. It may also provide adaptation for media streams that have different characteristics e.g. codec format. A gateway allows telepresence and non-telepresence endpoints and other components residing in different networks (e.g. intra and inter operator networks, operator and enterprise networks, etc.) to have access to the same conference.

### 6.1.5	Management system

A management system mainly contains control and management functions that provide specific telepresence services, such as device management, conference scheduling and user authentication, etc. It provides a statistic collection and reporting function for information relating to telepresence conference sessions and system usage.

### 6.1.6	Legacy conferencing endpoints

A legacy conferencing endpoint can be legacy H.323 or non H.323 conferencing endpoint. In case of legacy H.323 endpoint, the main capabilities and functions are identical to the definition in [ITU-T H.323]. In case of non H.323 endpoint, it can be a legacy endpoint which uses non H.323 open standards (i.e. SIP). These kinds of endpoints may be a SIP hard phone and SIP soft phone (i.e. PC or Laptop computer). The legacy non H.323 endpoints can connect to other components (e.g. Call and Resource Controller) through a gateway which performs protocol conversion and stream adaptation.

### 6.1.7	Network

A signalling and transport network underlies the telepresence system and connects the various network elements within or across network boundaries (e.g. between private networks). The network is responsible for delivering signalling and media with an appropriate quality of service (QoS) in the telepresence system. The network elements via measurement of the network performance and/or through appropriate QoS mechanisms (e.g. packet prioritisation, resource reservation, media quality adaptation) should ensure an acceptable QoS for a telepresence experience. IPv4 or IPv6 network technologies may be applied to deliver this service.

The network may also be required to support mechanisms to secure telepresence session signalling and/or media flows. These mechanisms may be negotiated between network elements. These network elements may also require authentication in order to use network resources.

# 7	Conference control functions

Conference control functions are those functions used to manage the display mode of screens, control cameras used in telepresence systems, support addition/deletion of participants in the telepresence session, control composition policy, and switching of meeting room media. The conference control functions described in this clause relate to the telepresence system as a whole. The functions may be contained in one or more telepresence sub-system components. For example: the removal of an endpoint from a conference may be initiated by an endpoint but needs a MCUto change its media mix to remove the media.

Conference Control Functions are grouped into three main groups:

- Media Control – These functions relate to the negotiation and manipulation of the media streams.

- Meeting Arrangement – These functions relate to planning and running of a meeting.

- Endpoint management – These functions relate to interaction of endpoints with a conference and the control of local functions.

The control functions described in the sub-clauses are from a user perspective and provide an overview of call control functions available to telepresence systems. A telepresence system may not support all conference control functions.

## 7.1	Media control functions

### 7.1.1	Chair control

| Function | Description |
| --- | --- |
| Request chair control (makeMeChair) | Any endpoint can apply for chair control. Application is granted when there is no chair in the conference. If there is already a chair in the conference, whether the application is granted will depend on the decision of the current chair. Note - This is equivalent to the [ITU-T H.243] CCA command |
| Release chair control (cancelMakeMeChair) | Release chair control so that another endpoint/person may take the chair. Note - This is equivalent to the [ITU-T H.243] CCR command sent from terminal |
| Withdraw chair control (withdrawChairToken) | A previously chair controlled conference now has no chair control. Note - This is equivalent to the [ITU-T H.243] CCR command send from MCU |
| Floor Request (requestForFloor) | Request the floor Note - This is equivalent to [ITU-T H.243] CCK command |
| Display Floor Requests (floorRequested) | Displays the list of endpoints/persons applying for the floor. NOTE - [ITU-T H.243] provides a facility to forward each request for the floor to the chair. It does not provide a facility to display the list of outstanding floor requests. |

### 7.1.2	Media rendering

| Function | Description |
| --- | --- |
| Select default view | This function selects the default view for an endpoint. This may be a certain endpoint, round-robin of endpoints when there are multiple endpoints, etc. |
| Select broadcasting endpoint (MakeTerminalBroadcaster) | All endpoints (except the origin) in the conference receive media from the selected broadcasting endpoint. NOTE - This is equivalent to [ITU-T H.243] “Select Broadcaster” where the Chairman selects an endpoint/person’s video to be distributed to the entire conference, i.e. command VCB. |
| Cancel broadcasting endpoint (CancelMakeTerminalBroadcaster) | Media distribution from the broadcasting endpoint is cancelled and the conference reverts back to the previous mode of operation. NOTE - This is equivalent to [ITU-T H.243] “Cancel Broadcaster” where the media distribution is cancelled and the conference reverts to a normal operating mode, i.e. command Cancel-VCB. |
| Select endpoint and captures | Only the selected media captures from a selected endpoint are distributed in the conference. Media capture from the other endpoints is stopped, i.e. microphones are muted. |
| Select a composed image | Manually set a composed image from more than one media capture. |
| Voice controlled switch | Indicate that the media to distribute in the conference is based on a volume threshold, e.g. loudest endpoint's media is distributed. |
| Automatic composition | Allow the system to compose an image from more than one media capture based on pre-set criteria. |
| Set presentation status | Indicates that an endpoint is to send presentation media. |
| Remove presentation status | Stop an endpoint from sending presentation media. |
| Request for Visualization (BroadcastMyLogicalChannel) | An endpoint requests that its captures be distributed to the entire conference. NOTE - This is equivalent to the [ITU-T H.243] MCV command and is typically done when there is no chair. |
| Cancel Visualization (CancelBroadcastMyLogicalChannel) | An endpoint cancels its previously granted visualization request. NOTE - This is equivalent to the [ITU-T H.243] Cancel-MCV command. |

## 7.2	Meeting arrangement

| Function | Description |
| --- | --- |
| Schedule conference | Add conference information, invite participants (Note 1). |
| Start conference | Start the conference instance. Enable calling of endpoints. NOTE – For example [ITU-T H.323] Call Setup message, conferenceGoal=invite. |
| Extend conference | Extend the end time of the conference (Note 1). |
| End conference | End the conference. All sessions with endpoints cease. NOTE - This is equivalent to the [ITU-T H.243] CCK command. |
| Lock conference | No more endpoints can join the conference (Note 2). |
| Unlock conference | Allow new endpoints to join a conference (Note 2). |

NOTE 1 - There is no ITU-T Recommendation on conference scheduling.

NOTE 2 - [ITU-T H.243] does not have this facility.

## 7.3	Endpoint management

| Function | Description |
| --- | --- |
| Call endpoint | Call one endpoint which is in the conference list but has not joined. Call all endpoints which are in the conference list but have not joined. NOTE - [ITU-T H.243] provides a conference list, but it only includes endpoints which are already in the conference. |
| Disconnect endpoint | End a session with an endpoint which has already joined in the conference. Note: Equivalent to [ITU-T H.243] CCD command |
| Remove endpoint | Remove an endpoint from the conference list whether it has joined the conference or not. NOTE - [ITU-T H.243] provides a conference list, and a facility for removing endpoints from the conference. But the conference list only includes endpoints which are already in the conference. |
| Add endpoint | Add an endpoint to the conference list. NOTE - [ITU-T H.323] allows endpoints to be invited to an existing conference, but they are not added to the conference list until they are connected to the conference. |
| Mute | Mute and un-mute local or remote endpoint. |
| Set advertised captures | Set information associated with captures. Set which captures are advertised. |
| Far end camera control | Move remote camera |

# 8	Sub-system architectures

## 8.1	Endpoint architecture

The telepresence endpoint architecture is illustrated in Figure 4.

Figure 4: Endpoint architecture

A telepresence endpoint consists of the following components:

- A Video Capture Unit (VCU) that consists of multiple main cameras and presentation cameras (optional) in certain spatial relationships.

- A Video Render Unit (VRU) that consists of multiple screens for main video streams and screens (optional) for presentation in certain spatial relationships.

- An Audio Capture Unit (ACU) that consists of multiple microphones in certain spatial relationships.

- An Audio Render Unit (ARU) that consist of multiple loudspeakers in certain spatial relationships.

- A Telepresence Control Unit (TCU) that connects and controls other units. It includes: a Telepresence Media Codec (TMC), Telepresence Media Processor (TMP) and Telepresence Media Input/Output (TMIO) Control units.

- A Network Unit (NU), including: wired and wireless network interfaces and network switch, etc.

- An Interactive Unit (IU), including: touch control panels, PCs, etc.

### 8.1.1	Endpoint architecture description

#### 8.1.1.1	Video capture unit

A telepresence endpoint may have multiple video sources including main cameras, presentation cameras and auxiliary video stream/s from a PC or other device. Video sources connect through a telepresence media input/output control unit that can select and control multiple video sources.

Figure 5: Video capture unit architecture

Multiple main cameras may constitute a camera set for capturing images of participants at the telepresence endpoint. Each main camera capture images of participants in one region of the room corresponding to a particular spatial arrangement and the multiple images of cameras are combined to a wide view angle, high resolution view. Where spatial information is known the video capture unit shall support the carriage of this information. A camera set may use one of the following three models: the converging camera model, the parallel camera model and the con-optical centre camera model.

Figure 6: Camera models: converging camera model (left), parallel camera model (middle), con-optical centre camera model (right)

In the converging camera model, each of multiple cameras looks out directly in a different direction. The parallel camera model, in contrast to the converging camera model, all optical camera axes have the same direction. Multiple cameras capture multiple virtual images reflected by a mirror and the captured images almost have the same optical centre in the con-optical centre camera model. For the first two camera models, there may be overlap and missing regions between two images and individual images may not be able to be integrated seamlessly unless objects are extremely distant. While in the con-optical centre camera model, individual images can be stitched to a seamless image over the entire depth range. The composited image can be scaled to an image having a single camera's resolution for inter-operation with a legacy videoconferencing endpoint that does not support this capability.

Presentation cameras are used to capture images of documents or objects in a conference room. There may be various spatial placements for presentation cameras, for example, hanging the cameras from the ceiling to capture images of objects on a desk.

Auxiliary video streams from a PC or other devices are used for sharing data.

#### 8.1.1.2	Video render unit

A video render unit may comprise of one or more main screens and presentation screens. Main screens are primarily used to display the video streams captured by local and remote main cameras and can be LCD, PDP monitors or projectors. In a fully immersive system a main screen should have sufficient size to display actual-sized images. Multiple monitors are usually integrated to present a panoramic view, or a single projection screen and multiple projectors can also be used.

The telepresence media input/output control unit can dynamically arrange display patterns and contents for main screens and presentation screens in an automatic or manual way:

- Main screens may display the local and remote main video streams and presentation video streams with reproduction in a spatially correct manner based on the spatial information associated with the streams.

- Typically where a presentation screen does not have a camera capable of capturing the participant/s it should only display the local and remote presentation video streams, because eye contact cannot be achieved when a main video stream is presented on a presentation screen.

Figure 7: Video render unit architecture

#### 8.1.1.3	Audio capture unit

An audio capture unit may comprise of multiple microphones in some spatial placement for capturing audio signals at the telepresence endpoint. This spatial placement would largely be dependent on the telepresence endpoint configuration. For example: microphone placement may differ between single-row and multiple-row configurations. There may be no direct mapping between microphones and audio streams. Where spatial information is required the audio capture unit should support the carriage of this information.

Figure 8: Audio capture unit architecture

Microphones may connect through a media input/output control unit that can select and control multiple audio sources. Auxiliary audio streams from a PC or other devices also connect to an endpoint through the media input/output control unit. Audio signals from multiple microphones may be combined into one single audio stream for transmission. For example: the audio signal from each microphone may be coded as a separate channel using an applicable audio codec. These audio codings are then transported as an aggregated stream to the remote endpoint via a suitable transport such as RTP.

#### 8.1.1.4	Audio render unit

An audio render unit may comprise of multiple loudspeakers in some spatial placement for rendering audio signals of the remote telepresence endpoint. There are no direct mappings between loudspeakers, microphones and screens, but the spatial information of sound from the remote endpoint can be reproduced through the audio render unit. Both the sound and image positions of remote participants should coincide.

Loudspeakers should connect through the media input/output control unit that can select and control audio output dynamically.

Figure 9: Audio render unit architecture

#### 8.1.1.5	Telepresence control unit

A telepresence control unit connects other units in a telepresence endpoint. The main functions of the telepresence control unit include:

- To provide interfaces to other functional units in an endpoint, invoking other units to complete processing.

- To handle signalling processes, including: registration, authentication, session initialization, session control and session termination, capabilities negotiation for inter-operation, etc.

- To send and receive multiple media streams.

- To provide contacts and other interfaces.

- To provide device management and debug interfaces.

#### 8.1.1.6	Telepresence media codec

The telepresence media codec is a logical unit that be used for coding and decoding of multiple video and audio streams. Synchronization between video and audio streams is supported. The telepresence media codec can be implemented using two configurations, a centralized solution or distributed solution. A centralized solution uses a single device to code and decode multiple media streams. Synchronizing media streams for this configuration is simpler but the device requires higher processing capabilities. The distributed solution needs more than one device for coding and decoding of multiple media streams which results in more complicated synchronization strategies. However this configuration may use devices that have lower processing capabilities than that of the centralized solution.

#### 8.1.1.7	Telepresence media input/output control unit

The telepresence media input/output control unit is a logical unit that can select and control multiple input/output media streams that include main and auxiliary video, audio and text streams. Video and audio capture units, video and audio render units, PCs, text terminals and other audio endpoints are connected to the telepresence media input/output control unit.

#### 8.1.1.8	Telepresence media processor

The telepresence media processor is used for processing video and audio steams to provide the required audio and visual experience. The main functions of telepresence media processor include:

- Geometric correction for multiple camera images to get better image mosaic effect.

- Colour correction for making luminance and chrominance of multiple camera images uniform.

- Geometric correction for multiple video stream rendering, that adapt video outputs to various types of screens to produce a better display performance, for example: geometric correction for a curved projection screen.

- Colour correction for multiple video stream rendering for a better rendering performance.

- Pre-processing for audio signals captured by audio capture unit, such as audio de-noising and enhancement for multiple microphones, etc.

- Pre-processing for audio signals rendered by an audio render unit, for example, improving audio quality of loudspeakers.

- Incorporation of text information (e.g. captioning) into video streams prior to encoding or post decoding.

#### 8.1.1.9	Interactive Unit

The interactive unit provides an interface for users to receive information regarding a telepresence conference and to be able to control various functions:

- call control – For example: establishing and releasing a telepresence conference,

- media control – For example: selecting the media format and changing rendering properties such as display position,

- conference control – For example: managing the meeting via chair control,

- collaboration – For example: setting up and managing white boarding.

The interactive unit may be realised through one or more physical devices, for example: touch control panels, tablets etc. The goal of the interactive unit interface is to allow participants to control a telepresence conference with the minimum of input.

## 8.2	Multipoint control unit

The main functions of an MCU are basically identical to the definitions in [ITU-T H.323], as per the following descriptions.

Access functions:

- To support accesses of various types of endpoints that include: multi-screen telepresence endpoint, single-screen telepresence endpoint and audio endpoints.

- To support multi-image composition videoconferencing.

- To support single/multiple auxiliary media streams.

- To support MCU cascading connection. Multiple media channels should be supported and can be flexibly multiplexed in cascading connection mode.

- To assure QoS.

- To support automatic NAT traversal

- To support both IPv4/v6 connections.

Adaptation functions:

- Adaptation for media streams according to capabilities of endpoints, such as video transcoding, audio transcoding, etc.

- Adaptation for media streams according to endpoint characteristics, such as screen aspect ratio, etc.

- Utilising endpoint characteristics to determine cases where no adaptation of the media stream is necessary.

Control functions:

- To support voice switch and manual switch mode.

- To support switch based on endpoint or screens when an endpoint has multiple screens.

- To support conference chairman control, such as media selection, adding/deleting endpoints.

- To support far-end camera control.

- To support non-telepresence heterogeneous systems.

- To support spatial arrangement.

Security functions:

- To support authentication, signalling/media security.

## 8.3	Call and resource controller

The CRC is an optional function that provides call and resource control functions including (but not limited to):

- Address translation

- Admission control

- Bandwidth control

- Zone management

- Call authentication and authorisation

- Call management / routing

- Digit translation

- Call feature provision

Multiple call and resource controllers may be utilised in a telepresence system.

Examples of CRCs are H.323 gatekeepers and SIP Proxies.

## 8.4	Gateway

The main gateway functions are identical to those defined in [ITU-T H.323].

# 9	H.323 architecture

## 9.1	Mapping telepresence on a H.323 architecture

The telepresence system architecture is based on the existing [ITU-T H.323] system. A telepresence endpoint can be viewed as another kind of H.323 endpoint, as depicted in the Figure 10.

Figure 10: H.323 telepresence network model

Figure 11 illustrates the interfaces between the different components of a H.323 based multimedia system.

Figure 11: H.323 telepresence system architecture

The main interfaces (reference points) in H.323 telepresence system architecture are:

- A: Interface between legacy video conferencing endpoint and MCU, for transmitting conference call and control information, completing creation and release of media channels and media interaction. This relates to the generic telepresence system interface C7 (Table 1).

- B: Interface between legacy video conferencing endpoint and GK, for transmitting registration, authentication and call process information using RAS messages. This relates to the generic telepresence system interface C6 (Table 1).

- C: Interface between MCU and service system, for transmitting conference scheduling information, creating and ending conferences. This relates to the generic telepresence system interface MT2 (Table 2).

- D: Interface between MCU and GK, for registration, authentication, call process of MCU and transmitting resolving information of user addresses. This relates to the generic telepresence system interface C10 (Table 1).

- E: Interface between GKs, transmitting resolving information of user addresses using RAS messages, etc. This relates to the generic telepresence system interface C12 (Table 1).

- F: Interface between GK and service system, for transmitting GK's management and configuration information, user information, billing data, etc. This relates to the generic telepresence system interface MT1 (Table 2).

- G: Interface between legacy videoconferencing endpoints, for transmitting call signalling, media streams and related control information when a terminal has a peer-to-peer communication. This relates to the generic telepresence system interfaces C9 (Table 1) and M7 (Table 3).

- H: Interface between MCUs, for transmitting media streams between MCUs and information of MCU cascade connection and inter-operation. This relates to the generic telepresence system interfaces C14 (Table 1) and M9 (Table 3).

- I: Interface between GW and PSTN/ISDN network. J: Interface between GW and [ITU-T H.320] network, for inter-operation between H.323 and H.320 service network.

- K: Interface with other systems. This relates to the generic telepresence system interfaces C13 (Table 1) and M8 (Table 3).

- L: Interface between GW and MCU. This relates to the generic telepresence system interfaces C13 (Table 1) and M8 (Table 3).

- M: Interface between GW and GK. This relates to the generic telepresence system interfaces C11 (Table 1).

In order to support telepresence endpoints several new interfaces based on existing ones are introduced:

- A1: Interface between telepresence endpoint and MCU, for transmitting call and control information of telepresence conference, completing creation and release of media channels and exchanging video and audio streams between telepresence system and MCU. This relates to the generic telepresence system interfaces C2 (Table 1) and M1 (Table 3).

- B1: Interface between telepresence endpoint and GK, for transmitting registration, authentication and call process information using RAS messages. This relates to the generic telepresence system interface C1 (Table 1).

- G1: Interface between telepresence endpoints, for transmitting call signalling, media streams and related control information when a telepresence endpoint has a peer-to-peer communication with another. This relates to the generic telepresence system interfaces C5 (Table 1) and M4 (Table 3).

- G2: Interface between legacy video conference endpoint and telepresence endpoint, for transmitting call signalling, media streams and related control information when a telepresence endpoint has a peer-to-peer communication with a legacy video conference endpoint. This relates to the generic telepresence system interfaces C4 (Table 1) and M3 (Table 3).

### 9.1.1	Peer-to-peer communication

Figure 12 illustrates a peer-to-peer communication scenario for telepresence.

Figure 12: H.323 telepresence peer-to-peer communication

The communication between telepresence endpoints will have a single call control signalling channel supplemented by additional control signalling, one or more video streams and one or more audio streams.

The control signalling stream is used for controlling the whole calling process, for example: enabling/disabling the sending of auxiliary streams.

### 9.1.2	Multi-point communication

Figure 13 illustrates a multi-point communication scenario for telepresence.

Figure 13: H.323 telepresence multi-point communication

Different types of telepresence endpoints may connect to an MCU.

Telepresence endpoints send media streams according to the results of capability negotiation.

The MCU completes the adaptations for media streams, for example: adaptation for single channel and multi-channel audio streams, retransmission for multiple video streams and adaptation for conference control.

## 9.2	Call process

The call process of a H.323 based telepresence system follows the basic call phases as outlined in clause 8/[ITU-T H.323]. However there is additional functionality in the call phases in order to support telepresence. The additions are outlined below:

| Phase A: | An ability to negotiate the specific features related to the use of telepresence information is added. This allows an endpoint, gateway or MCU to indicate whether or not it supports telepresence. This is beneficial in order to determine early on in the call process whether a peer supports telepresence and allows endpoints to optimise their signalling, i.e. the endpoint wouldn’t send telepresence information if it is not used. |
| --- | --- |
| Phase B: | An ability to link the H.245 terminalCapabilities (i.e. media characteristics) with a set of telepresence information is added. This allows an endpoint, gateway or MCU to link a set of terminalCapabilities with a set of telepresence information. For example: it enables the characteristics of the capture media (e.g. spatial information) to be linked to a particular potential media stream. There is also an ability to transfer and negotiate the telepresence information (e.g. information regarding the media captures). |
| Phase C: | An ability to indicate how the H.245 logical channels (and thus the media) relate to a particular set of telepresence information is added. |
| Phase D: | The indication of the support of telepresence and the support telepresence information may be required for particular services, e.g. multipoint conferencing scenarios. |
| Phase E: | An ability to remove telepresence information from a media stream may be required in addition to normal release procedures. |

Figure 14 illustrates the call phases.

Figure 14: H.323 telepresence call processes

# 10	Telepresence room design

## 10.1	Room layouts

This clause details several example room layouts capturing and rendering room participants in a manner that facilitates a telepresence experience. Other configurations are possible.

In these examples where the multiple capture regions are utilized the boundaries between the regions may be convergent or appear to be parallel. These boundaries may have an impact on the apparent gap between renderings of the captures.

### 10.1.1	Single row seating three-screen telepresence system

The following figure depicts the room layout of a single row 3-screen telepresence system, with the following characteristics:

- Three main cameras and three main screens.

- Three regions for participants in the room, and each region contain two seats.

- Each main camera captures images of one region.

The missing/overlapped region of two captured images is located at the corner of the conference table and covered by borders of two main screens.

Figure 15: Three-screen telepresence room layout (single row)

### 10.1.2	Multi row seating three-screen telepresence system

The following figure depicts the room layout of a multi-row three-screen telepresence system, with the following characteristics:

- Three main cameras and three main screens.

- Three regions for participants in the room, and each region contains five seats.

- Each main camera captures images of one region.

The missing/overlapped region of two captured images is located at the corner of the conference table and is matched to the borders between two main screens.

Figure 16: Three-screen telepresence room layout (multi-row)

### 10.1.3	Multi row seating four-screen telepresence system

The following figure depicts the room layout of a multi-row 4-screen telepresence system, with the following characteristics:

- Four main cameras and four projectors.

- Four regions for participants in the room, and each region contains four or five seats.

- Each main camera captures images of one region.

- Three overhead microphones capturing multi-channel audio from the room

Each projected image shows the far end participants at their true size.  The display height is chosen so that standing individuals of normal height are displayed without cropping.  In order to increase the height, 4:3 aspect ratios are used.  Since projectors are used, there is no gap between the displayed images.

Main cameras are placed behind the projector screens, and are placed at eye level. The cameras are aligned to minimise gaps between captures. The seating is organised so that a person does not span a gap between captures.

The second row of participants is elevated about 25 cm above the first row.  This allows all participants to be easily seen.

Figure 17: Four-screen telepresence room layout (Multi row)

### 10.1.4	Single row seating six-screen telepresence system

The following figures depict the room layout of a 6-screen telepresence system, with the following characteristics:

- Three main cameras and six screens with narrow frame borders.

- Three screens in one row, two rows of screens.

- Main cameras placed between the screens of the bottom and top row.

- Three regions for participants in the room, and each region containing two or three seats.

- Each main camera captures images of one region.

- One wall projector displays a selected presentation/stream.

As can be seen in Figure 18, a set of three screens are arranged in one row. Each screen shows the far end participants at their true size. Three main cameras are placed at eye level and sandwiched between two rows of screens; each camera is placed in the centre of the bottom frame of the higher screen and in the centre of the top frame of the lower screen.

Camera 1

Camera 2

Camera 3

Screen 1

Screen 2

Screen 3

Screen 4

Screen 5

Screen 6

Figure 18: Screen and camera layout for six-screen telepresence

The use of two rows of screens allows for other display scenarios not possible with a single row of screens. This is particularly useful in scenarios with more than two end points in a call. For example the following scenarios are possible:

- The simultaneous display of two endpoints, with one endpoint shown in the first row and the other in the second row;

- Four adjacent screens, e.g. the left two and the middle two, displaying a scaled image of the current speaker, and the remaining two screens displaying in life size;

- All the screens displaying switched speakers, behaving like a switched video wall.

NOTE - this is possible in a single row. An additional row allows for more participants

- The upper row displaying active speakers while the lower row displays presentation and auxiliary information;

- The upper row displaying the local endpoint while the lower row displays the remote endpoint.

As shown in Figure 19, each main camera captures a region of two (or three) seats along a table. Presentation screens and microphones are mounted on the long table. An additional wall projector is provided to display desired streams/presentations upon user demand.

Since screens with narrow frame borders are used, the displayed images need more manipulations to obtain a continuous view. So image processing methods, such as cropping and scaling, are required to provide a better visual experience when interoperating with screens with broader frame borders.

NOTE - The dashed lines in Figure 19 represents objects that may be used when operating in a non-telepresence mode.

Camera

Projector

Seat

Screen

Microphone

Presentation screen

Region A

Region B

Region C

Figure 19: Six-screen telepresence room layout

## 10.2	Room characteristics

This clause discusses room design considerations when the room is used as part of a telepresence system. Many of the considerations apply for non-telepresence conferencing systems.

### 10.2.1	Room dimensions and shape

There are several factors that dictate the dimensions of a room associated with a telepresence system. The size of the room (width, depth and height) should be able to accommodate:

- Telepresence system hardware

- Furniture associated with telepresence participants, e.g. desks, chairs.

- Furniture associated with other participants in the room, e.g. chairs, couches.

- Ancillary furniture associated with the room, e.g. tables, cupboards.

- Extra devices such as additional monitors / document camera etc.

- Circulation areas for the participants to access the telepresence system and furniture.

- Allowance for access to the telepresence hardware or other services for maintenance.

- Allowance for fittings and services, e.g. lighting, HVAC controls, fire safety equipment.

The placement of the telepresence participants should take into consideration the design of the capture devices. Cameras have a certain depth of field and capture area in both the horizontal and vertical planes. Therefore there is a minimum and a maximum distance where a participant can be located.  Likewise microphones have a certain capture area.

The layout (horizontal and vertical) and the position of participants in relation to cameras and displays should be considered so that both direct and indirect gaze awareness is maximized.

It should also be recognized that rooms that are in excess of the size needed for the telepresence system with the above considerations present some problems. For example: a telepresence system may be accommodated in an auditorium. In this case even and consistent lighting can be a challenge. A larger room may also present acoustic problems.

The shape of the room also needs to be considered. Doors, windows, the building fabric (such as columns) will dictate where the telepresence system hardware can be placed. Asymmetric or curved walls and ceilings will have acoustic impacts as well as presenting issues with capture video. Doors and windows present acoustic issues in allowing outside sound into a room and reverberation. In addition windows present lighting issues.

Telepresence aims to provide a being there experience therefore the above needs to be considered so that a telepresence system can provide a pleasing experience for the participants.

### 10.2.2	Wall, floor and ceiling surfaces

The wall material, texture and colour influence both audio and video aspects of a telepresence system.

The type of wall material influences the acoustic properties of the room. Differing materials reflect and transmit sound at different rates. Materials should be chosen to maximise the amount of sound being absorbed. Where the wall material cannot be chosen it may be possible to apply a surface treatment to achieve the desired characteristics.

The wall texture and any patterning may influence video encoding. Walls with a "busy" pattern or texture may cause problems for video coding. Plain surfaces should be used.

The wall colour also affects the room lighting in terms of the amount of light reflected and the colour of the reflected light. In order to provide a being there experience the participants should be lit in a manner to allow the telepresence system to capture an accurate flesh tone.

Whilst floors are subject to the same considerations as wall with respect to reflected light, colour and sound transmission there are also other physical considerations. Floors interact directly with participants and as a result create sound. For example: a chair rolling across a surface. Therefore the floor material should also be chosen to minimise sound created through movement. Floors also need to provide a stable safe surface for any people, furniture or equipment in the room.

Ceilings are again subject to the same considerations as walls however given that most lighting is overhead the ceiling must be able to safely incorporate lighting.

### 10.2.3	Furniture

In a telepresence room, the tables and chairs are arranged in a way that each participant in normal seating position falls in the capture range of only one camera, and looks into that camera. The curve of the tables should be aligned with the arrangement of cameras and screens, to achieve a desirable experience.

It is suggested furniture captured by cameras should have a shape or curve like the table to have consistency in the whole telepresence room. Also, furniture with heavy texture or complicated pattern may make the video coding more complex and the system less effective. It is suggested to place any additional furniture beyond capture areas, and have clean and plain surfaced furniture with curves conforming to the capture areas. Decorations and adornments can be used to provide a sensation of depth, but their patterns and colours should be chosen carefully.

Furniture affects the acoustic and lighting properties of the room, in terms of sound reflection and light reflection, respectively.

In order to provide a more natural experience, peripheral equipment, such as computer cases, wireless routers, server cabinets, are suggested to be concealed in large-sized furniture (desk, cupboard, etc.), or integrated into the framework for screens and cameras . It is suggested that cables be hidden from the cameras, and best embedded underneath the floor or floor covering. Table-mounted microphones are suggested to be flat (not with gooseneck), and it is suggested to hang ceiling-mounted microphones above the upper-boundary of captures.

To provide a better experience, these factors should be considered when designing a telepresence room or introducing changes to a telepresence room.

### 10.2.4	Lighting

For a pleasing telepresence experience good lighting in terms of quantity and quality is needed for the environment where the telepresence endpoint is placed. A number of factors should be considered when designing and commissioning a telepresence room:

- Luminance distribution

- Illuminance

- Glare

- Directionality of light

- Colour aspect of the light and surfaces

- Flicker

- Daylight

- Maintenance

[ISO 8995] on the lighting of indoor work spaces provides further guidance on the above factors.

### 10.2.5	Acoustics

As discussed above the size and shape of a room including the materials that the walls, floors and ceiling are made from affect the acoustic properties of a telepresence room. Important parameters to consider are reverberation time, ambient noise and sound insulation. Reverberation time should be kept to a minimum as sound quality decreases as reverberation time increases. Ambient noise should be minimized in order to provide a pleasing audio experience. Sound insulation is also important to minimize both the sound admitted to and transmitted from the telepresence room. Minimizing the admitted sound will increase the telepresence experience. Minimizing the transmitted sound will increase the privacy of the meeting.

### 10.2.6	Heating ventilation and air conditioning (HVAC)

As the main goal of a telepresence system is to provide a pleasing experience for the participants, a telepresence room should provide a comfortable environment for them.  HVAC systems include temperate control and ventilation.  When designing these systems there needs to be several considerations:

- People loading: The amount of people expected in the room needs to be considered. Each person generates heat and needs a certain flow of air to be comfortable.

- Equipment loading: Electronic equipment generates heat and needs to be cooled for efficient operation.

- Noise: The movement of air around obstacles creates noise which contributes to the ambient noise of the room. HVAC and equipment cooling systems should be designed to minimize noise.

- Airflow: Air flow is important to evenly distribute the temperature throughout the room. HVAC systems should be designed to evenly distribute the temperature without causing localized areas of temperature or airflow that may cause discomfort to participants. There also should be adequate air flow surrounding equipment to enable cooling.

- Thermal Insulation: Due to acoustic treatments the walls, ceiling and floors in telepresence rooms may have a higher level of thermal insulation and less air leakage than in other parts of the building. This higher level of insulation and lower level of air leakage should be taken in account when designed HVAC systems.

- Utilization time: Telepresence rooms may be occupied at different times of the day or night. Consideration should be given to a HVAC system that provides an optimum environment whilst the room is occupied and offers an energy saving mode when not.

### 10.2.7	Power requirements

Telepresence system hardware and telepresence room electrical equipment (i.e. lighting) should be designed to minimize energy usage where possible. This minimizes heat generation and resulting HVAC load which results in saved costs for energy consumption. Standby modes with low energy consumption should be considered for telepresence systems and rooms when the telepresence endpoint is not in use.

The power should be of sufficient quality to allow the operation of sensitive telepresence system and computer hardware.

When calculating the power required for a telepresence room the following should be considered:

- The power consumption of the telepresence system hardware, including monitors, servers etc.

- The power consumption of supplementary devices such as projectors, extra displays.

- Power outlets associated with participants to allow them access to power for laptops, charging etc.

- Lighting power consumption.

- Any local HVAC system requirements.

- Power related to supplementary uses, e.g. kettles, lamps.

Power outlets should be provided in close proximity to both the telepresence system hardware and if required close to the participants.

### 10.2.8	Network connectivity

Telepresence rooms are required to provide some form of network connectivity appropriate for the telepresence system deployed to it. The size, the audio and video quality of the telepresence and resulting bandwidth to transport media flows will largely dictate the type of network connection.

Consideration should also be given to providing network connectivity for the room participants so that they can connect laptops or other internet enabled devices.

Network connectivity may also be needed for HVAC devices, e.g. thermostats.

Connections to the PSTN may be required.

Bibliography

[b-IETF RFC 5117]	IETF RFC 5117 (2008), RTP Topologies

[b-IETF RFC 7262]	IETF RFC 7262 (2014), Requirements for Telepresence Multi-Streams.

_________________