| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 232 (PLEN) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Sapporo, 30 June - 11 July 2014 |
| TD |  |  |
| Source: | Editor H.Sup.Term |  |
| Title: | Approval: H.Sup.13 (ex H.Sup.Term) "Gateway Control Protocol: Common ITU-T H.248 terminology – Release 1" (New) |  |

1.	Introduction

This document contains the draft new H.Sup.Term: "Gateway Control Protocol: Common ITU-T H.248 terminology – Release 1" for Approval.

CONTENTS

Page

1 Scope 4 2 References 4 3 Definitions 4 3.1 Terms defined elsewhere 4 3.2 Terms defined in this Supplement 4 4 Abbreviations and acronyms 5 5 Conventions 5 6 Handling of new terms 5 Bibliography 5

List of Tables

Page

Table 1 – Terms defined in this Supplement 4

List of Figures

Page

Figure 1 – Positioning of H.SuppTerm versus H.248.x-series of Recommendations 3

Draft New Supplement 13 to the ITU-T H-Series (ex H.Sup.Term)

Gateway Control Protocol:
Common ITU-T H.248 terminology – Release 1

Summary

This Supplement provides a central place for H.248 related terms that concern multiple H.248.x Recommendations, Supplements or the entire H.248.x-series. This Supplement is a living document.

Keywords

GCP, H.248

Introduction

All ITU-T Recommendations with respect to the ITU-T defined Gateway Control Protocol (GCP) are published within the H.248.x-series of Recommendations. The core protocol itself is defined by [ITU-T H.248.1]. Protocol extensions (in form of so called H.248 packages) and other material (such as profile guidelines) are the subject of self-contained Recommendations within the H.248.x-series [ITU-T H.248.x].

Basically, all of these GCP-related Recommendations (plus some Supplements) share the same terminology. The set of GCP terms were historically divided in two classes: general GCP terms and package specific GCP terms. The general terms with global GCP scope are subject of the core protocol specification [ITU-T H.248.1], whereas package specific terms are typically located in the concerned H.248.x-Recommendation (Figure 1).

Figure 1 – Positioning of H.SuppTerm versus H.248.x-series of Recommendations

When defining terms there are two basic issues:

- the terms may affect multiple H.248 documents, but not the entire series, leading to the question of the right place for such terminology; and

- the addition of new, global H.248 terms would imply a revision of H.248.1, which may not be desirable for a single term.

This Supplement acts as a central place for such GCP related terminology to alleviate the above issues.

# 1	Scope

In scope of this Supplement are GCP related terms that relate to multiple H.248 documents.

Out of scope of this Supplement are

- terms not related to H.248 technology,

- terms which affect a single H.248.x Recommendation.

# 2	References

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.x]	The ITU-T H.248.x-series of Recommendations (Gateway Control Protocol).

# 3	Definitions

## 3.1	Terms defined elsewhere

In addition to the basic GCP terms defined by clause 3.2/[ITU-T H.248.1], this Supplement uses the following terms defined elsewhere.

None.

## 3.2	Terms defined in this Supplement

NOTE - The terms in this clause are tabulated in order to maintain the usual alphabetical order when adding additional terms in future releases of this Supplement.

This Supplement defines the following terms (Table 1):

Table 1 – Terms defined in this Supplement

| Term | Definition |
| --- | --- |
| H.248 master-slave control relationship | The H.248 decomposed gateway follows a hierarchical control model in the network control plane (primarily related to call service control and media/bearer control functions). The control hierarchy constitutes a master-slave relationship, with the MGC as master and MG as the slave entity. NOTE 1 - [b-ITU-T Q.Sup31] Annexes C.2 or C.3 provide examples of functional-to-physical mapping models according to the H.248 master-slave control relationship. |
| MG autonomous mode | For a particular function, the MGC delegates some (but not all) service specific control decisions down to the MG level. The MG provides a local decision function. The MG autonomous control is limited to decisions with MG available information only. |
| MGC-strictly controlled mode | For a particular function, the control decisions are exclusively under MGC responsibility, the MG degenerates to a pure execution unit. |
| stream endpoint tuple (SEPT) | the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share the same StreamID value. The stream topology is given by the topology descriptor settings. |

# 4	Abbreviations and acronyms

This Supplement uses the following abbreviations and acronyms:

| GCP | Gateway Control Protocol |
| --- | --- |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SEPT | Stream Endpoint Tuple |

# 5	Conventions

None.

# 6	Handling of new terms

A new H.248.x-Recommendation under development identifies new terminology. Such terms are normally the subject of clause 3.2 "Definitions" according to the Recommendation template. When defining new terms it has to be decided whether a new term is only limited to the Recommendation itself, or whether multiple Recommendations could benefit from the new term. Where the term may benefit multiple Recommendations the term should be added to this Supplement. The new H.248.x-Recommendation may then refer these terms within clause 3.1 "Terms defined elsewhere".

Bibliography

[b-ITU-T Q.Sup31]	Technical Report TRQ.2141.0 (12/2000): Signalling requirements for the support of narrow-band services over broadband transport technologies - Capability set 2 (CS-2).
and
	Technical Report TRQ.2141.1 (11/2002): Signalling requirements for the support of narrowband services via broadband transport technologies - CS-2 signalling flows.

____________________