| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 155R1 (WP 1/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Sapporo, 30 June - 11 July 2014 |
| TD |  |  |
| Source: | Editor H.Sup.OpenFlow |  |
| Title: | H.Sup.OpenFlow: "Gateway Control Protocol: Protocol evaluation of OpenFlow versus ITU-T H.248" (New): Output draft Ed. 0.6 (Sapporo, 30 June - 11 July 2014) |  |

1.	Introduction

This document contains the editor's output draft ed. 0.6 for draft new H.Sup.OpenFlow: "Protocol evaluation – OpenFlow versus H.248".

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.Sup.OpenFlow Ed. 0.1 | 2013-10 (Geneva): 1st input based on "COM 16 – C 216" Location: http://itu.int/md/T13-SG16-C-0216/en |
| H.Sup.OpenFlow Ed. 0.2 | TD 115/WP1 (2013-10) Location: http://www.itu.int/md/meetingdoc.asp?lang=en&parent=T13-SG16-131028-TD-WP1-0115 |
| H.Sup.OpenFlow Ed. 0.3 | Input (AVD-4479) to WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/AVD-4479.zip |
| H.Sup.OpenFlow Ed. 0.4 | Output (TD-19) from WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/TD-19.zip |
| H.Sup.OpenFlow Ed. 0.5 | Input (TD 155/WP1) to SG16 Sapporo meeting, June/July 2013 Location: http://www.itu.int/md/T13-SG16-140630-TD-WP1-0155/en |
| H.Sup.OpenFlow Ed. 0.6 | Output (TD 155R1/WP1) from SG16 Sapporo meeting, June/July 2013 Location: |

CONTENTS

Page

1 Scope 6 2 References 7 3 Definitions 8 3.1 Terms defined elsewhere 8 3.2 Terms defined in this Recommendation 9 4 Abbreviations and acronyms 9 5 Conventions 10 6 Introduction 10 6.1 Brief protocol history 10 6.2 Terminology relations 11 6.3 Objectives in terms of network architecture 12 6.4 Relationship between controlling and controlled entities 13 6.5 Time scale of network control: call-dependent vs call-independent control actions 14 7 Packet forwarding model 15 7.1 OpenFlow processing model 15 7.2 Policy rule based description of packet operations 16 7.2.1 OpenFlow: basic policy rule format 16 7.2.2 OpenFlow: staged process model – concatenation of multiple policy rules 17 7.2.3 OpenFlow: ### 17 8 Stage 1 – Services (use cases) 17 9 Stage 2 – Protocol requirements 17 9.1 Decomposed network element architecture 18 9.2 Network function virtualization 18 9.3 Operations on traffic flows 19 9.3.1 General operations 19 9.3.2 Regarding policy rule conditions ("flow identification") 19 9.3.3 Regarding policy rule actions 21 9.4 High availability of services and networks 21 9.5 Public networks: support of regulation related requirements 22 10 Stage 3 – Protocol capabilities 22 10.1 Termination types: supported protocol stacks in forwarding plane 22 10.2 Connection models: IP mode of operation in forwarding plane 23 Appendix I Scope of OpenFlow – Use cases 26 I.1 Experimental prototyping (academic/research area) 26 I.2 Software defined networking (SDN) 26 I.3 Network functions virtualisation 26 I.4 Cloud oriented networking 26 I.5 Trill switching 26 I.6 Lx-VPNs (Layer x Virtual Private Network) 26 I.6.1 Layer 1 Virtual Private Network 26 I.6.2 Layer 2 Virtual Private Network 27 I.6.3 Layer 3 Virtual Private Network 27 I.7 Network element decomposition due to cost factor evolutions in network planes (user, control, management) 27 I.8 Network element decomposition due to high availability network solutions 27 I.9 Network performance management with respect to load balancing 27 I.10 Network element type "programmable Ethernet switch" 27 I.11 Network element type "programmable IP router" 27 I.12 Network element type "programmable firewall" 27 Appendix II Comparison H.248 and ForCES 28 II.1 ### 28 II.1.1 ### 28 Appendix III Comparison OpenFlow and ForCES 29 III.1 Scope of ForCES 29 III.2 Comparison OpenFlow and ForCES 29 III.3 Emulation of OpenFlow interface by ForCES 29 Bibliography 30

List of Tables

Page

Table 1 – Terminology relations between OpenFlow and H.248 11 Table 2 – Time scale of network control actions – Order and examples 14 Table 3 – Protocol requirements – H.248 vs OpenFlow – Relation controller to controlled network element 18 Table 4 – Protocol requirements – H.248 vs OpenFlow – Network function virtualization 18 Table 5 – Protocol requirements – H.248 vs OpenFlow – Flow operations 19 Table 6 – Flow identification – OpenFlow – Flow identifier elements 20 Table 7 – Policy rule actions – OpenFlow – Action types 21 Table 8 – Protocol requirements – H.248 vs OpenFlow – Redundancy support 21 Table 9 – Protocol requirements – H.248 vs OpenFlow – Load control and overload protection 22 Table 10 – Protocol capabilities – H.248 vs OpenFlow – Basic Capabilities 22 Table 11 – Protocol capabilities – H.248 vs OpenFlow – Termination types for packet-oriented bearer technologies 22 Table 12 – Protocol capabilities – H.248 vs OpenFlow – Termination types for Circuit-oriented bearer technologies 23 Table 13 – Protocol capabilities – H.248 vs OpenFlow – Connection topologies 23 Table 14 – Protocol capabilities – H.248 vs OpenFlow – IP connection models 24 Table 15 – Protocol capabilities – H.248 vs OpenFlow – Policy rule actions – Generic view 24 Table I.1 – OpenFlow use case "prototyping" 26

List of Figures

Page

Figure 1 – Decomposed control models – OpenFlow vs H.248 13 Figure 2 – Decomposed control models – ForCES vs H.248 13 Figure 3 – Packet ﬂow through the processing pipeline (see Fig. 2/[OPENFLOW SS]) 15 Figure 4 – Order of operations (within a layer) – Unidirectional packet bearer traffic models using the concept of policy rules 16 Figure 5 – OpenFlow: basic policy rule format (vs generic DPI policy rule format [ITU-T Y.2770]) 17 Figure II.1 – ForCES Architectural Diagram (see [b-IETF RFC 5810]/Figure 1) 28

Draft new Supplement H.Sup.OpenFlow

Gateway Control Protocol:
Protocol evaluation of OpenFlow versus ITU-T H.248

Summary

This Supplement ### to the H series of ITU-T Recommendations.

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

The "vertical" decomposition of network elements (in packet based communication infrastructures), driven by network evolution scenarios which will demand for "open control" interfaces between the packet transport, forwarding and routing domain and the overlay domain of application/service control, demands for technologies for controlling of packet flows in the most general sense.

There are a number of candidate technologies for such network solutions. Purpose of this document is an attempt in comparing H.248 with OpenFlow (as defined by [OPENFLOW SS]).

[…]

It has to be noted that an evaluation between H.248 and OpenFlow, - from overall protocol point of view -, is almost impossible due to the very narrowed scope of OpenFlow. OpenFlow was/is only considering on pure Ethernet switches with IP traffic as network elements, hence a packet technology dependent and tightly coupled protocol. H.248 is much more generic and covers all type of packet network entities related to packet switching, packet forwarding/routing, packet interworking, etc. H.248 as a protocol is thus much more powerful and may be considered as a superset with respect to OpenFlow. The capabilities of the OpenFlow protocol may be essentially also provided by H.248.

The results of any assessment are dependent on the considered context, which might be

- protocol Pi should satisfy the requirements given by network solution Nj (i.e., an evaluation against a specific, concrete set of required protocol capabilities);

- protocols Pi and Pj are compared with focus on the common set of capabilities (i.e., looking only at the intersection of Pi ∩ Pj);

- protocol Pi should be able to emulate the capabilities of protocol Pj (or vice versa) (i.e., an evaluation against a set of supported protocol capabilities); or

- any other objectives.

Architects of new networks would be interested in evaluation context 1. Technology replacement strategies would consider evaluation context 2. The scope of this technical report would be rather the third evaluation context.

[…]

The structure of this TR is as follows:

- protocol origins and histories (clause 6);

- packet forwarding models (clause 7);

- stage 1 (services, use cases) consideration (clause 8);

- stage 2 (protocol requirements) consideration (clause 9);

- stage 3 (protocol capabilities) consideration (clause 10);

and

- complementary information to OpenFlow (Appendix I);

- complementary, high-level assessment of H.248 vs ForCES (Appendix II) and

- reference to other work on OpenFlow vs ForCES.

{Editor's note (2013-11 meeting): clause 1 is not yet complete … }

# 2	References

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.10]	Recommendation ITU-T H.248.10 (2001), Gateway control protocol: Media gateway resource congestion handling package.

[ITU-T H.248.11]	Recommendation ITU-T H.248.11 (2013), Gateway control protocol: Media gateway overload control package.

[ITU-T H.248.21]	Recommendation ITU-T H.248.21 (2004), Gateway control protocol: Semi-permanent connection handling package.

[ITU-T H.248.32]	Recommendation ITU-T H.248.32 (2013), Gateway control protocol: Detailed congestion reporting package.

[ITU-T H.248.55]	Recommendation ITU-T H.248.55(2008), Gateway control protocol: Generic pull mode package.

[ITU-T H.248.64]	Recommendation ITU-T H.248.64 (2009), Gateway control protocol: IP router packages.

[ITU-T H.248.79]	Recommendation ITU-T H.248.79 (2012), Gateway control protocol: Guidelines for packet-based streams.

[ITU-T H.248.83]	Recommendation ITU-T H.248.83 (2012), Gateway control protocol: Media gateway instance package.

[ITU-T H.248.86]	Recommendation ITU-T H.248.86 (2014), Gateway control protocol: H.248 Support for deep packet inspection.

[ITU-T H.248.RTPMUX]	Recommendation ITU-T H.248.RTPMUX (201?), Gateway control protocol: H.248 support for RTP multiplexing.

[ITU-T Y.1221]	Recommendation ITU-T Y.1221 (06/2010), Traffic control and congestion control in IP-based networks..

[ITU-T Y.2770]	Recommendation ITU-T Y.2770 (11/2012), Requirements for deep packet inspection in next generation networks..

[IETF RFC 3654]	IETF RFC 3654 (2003), Requirements for Separation of IP Control and Forwarding.

[IETF RFC 5101]	IETF RFC 5101 (2008), Specification of the IP Flow Information Export (IPFIX) Protocol for the Exchange of Flow Information..

[IETF RFC 5810]	IETF RFC 5810 (2010), Forwarding and Control Element Separation (ForCES) – Protocol Specification.

[IETF RFC 5812]	IETF RFC 5810 (2012), Forwarding and Control Element Separation (ForCES) – Forwarding Element Model.

[OPENFLOW SS]	Open Networking Foundation (10/2013), OpenFlow Switch Specification Version 1.4.0 (Wire Protocol 0x05). 
URL: https://www.opennetworking.org/

{Editor's note (2013-11 meeting): just some initial references … check latest OpenFlow version in general …}

[OPENFLOW NF]	Open Networking Foundation (10/2013), OpenFlow Notification Framework / OpenFlow Management, Version 1.0.

[ETSI TS 183 025]	ETSI TS 183 025 V2.5.1 (2009-04), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); H.248 Non-Call Related Procedures and Management System Interaction.

[ETSI TR 183 068]	ETSI TR 183 068 V3.1.1 (2009-08), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[ETSI NFV-SWA]	Draft ETSI GS NFV-SWA 001 V0.2.0 (2014-05), Network Functions Virtualisation (NFV); Virtual Network Functions Architecture.

# 3	Definitions

## 3.1	Terms defined elsewhere

This supplement uses the following terms defined elsewhere:

3.1.1	flow [IETF RFC 5101]: A set of IP  packets passing an observation point in the network during a certain time interval. All packets belonging to a particular flow have a set of common properties. Each property is defined as the result of applying a function to the values of:

- one or more packet header fields (e.g., destination IP address), transport header fields (e.g., destination port number), or application header fields (e.g., RTP header fields [b-IETF RFC 3550]).

- one or more characteristics of the packet itself (e.g., number of MPLS labels, etc.).

- one or more of fields derived from packet treatment (e.g., next hop IP address, the output interface).

A packet is defined as belonging to a flow if it completely satisfies all the defined properties of the flow.

This definition covers the range from a flow containing all packets observed at a network interface to a flow consisting of just a single packet between two applications. It includes packets selected by a sampling mechanism.

NOTE – The above numbered listed items indicate flow properties in the categories of (1) "Protocol Control Information (PCI) of packets", (2) "Protocol Data Unit (PDU) properties of packets" and (3) "Local packet forwarding information".

3.1.2	flow descriptor (also known as flow level conditions) [ITU-T Y.2770]: Set of rule conditions that is used to identify a specific type of flow (according clause 3.1.3) from inspected traffic.

NOTE 1 – This definition of flow descriptor extends the definition in [b-ITU-T Y.2121] with additional elements as described in clause 3.

NOTE 2 – For further normative discussion of the flow descriptor as used in this Recommendation, see Annex A.

3.1.3	IPFIX flow identifier (IPFIX flow ID) [ITU-T Y.2770]: The set of values for the IPFIX flow keys, which is used in conjunction with the flow descriptor to identify a specific flow.

3.1.4	IPFIX flow key [ITU-T Y.2770]: Each of the information elements of the flow descriptor that is used in IPFIX-based flow identification processes (according to [IETF RFC 5101]).

NOTE – The IPFIX flow key definition is semantically consistent with the flow key definition specified in IPFIX [IETF RFC5101]. The only difference between the two terms is that the definition in this document is scoped to the flow descriptor.

## 3.2	Terms defined in this Recommendation

This supplement defines the following terms:

3.2.1	call-dependent (packet path node) control: the stimuli, which triggers a control action, originates in a call control function.

3.2.2	call-independent (packet path node) control: the stimuli, which triggers a control action, originates in a control/management function outside call control.

3.2.3	packet path node: network element in the packet (forwarding) path with regards to the transfer of user (data) plane traffic between the communication source and sink. E.g., an OpenFlow switch, an H.248 packet-to-packet gateway, an IP router, etc. The information from source to sink is carried by a (traffic) flow which follows a packet path (given by the network route from source to sink).

NOTE – A packet path node may be realized as integrated or decomposed network element.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| CE | Controlling entity (OF controller; H.248 MGC) |
| --- | --- |
| CNE | Controlled network element (OF switch; H.248 MG) |
| ET | Ethernet |
| ForCES | Forwarding and Control Element Separation |
| ICE | Interactive Connectivity Establishment |
| IMS | IP multimedia subsystem |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| HTTP | Hyper-text transfer protocol |
| LD | Local descriptor |
| MG | Media gateway |
| MGC | Media gateway controller |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NFV | Network Functions Virtualisation |
| NGN | Next generation network |
| NE | Network element |
| OF | OpenFlow |
| ONF | Open Networking Foundation (www.opennetworking.org) |
| PBB | Provider Backbone Bridges |
| RD | Remote descriptor |
| SDN | Software Defined Networking |
| SEP | (H.248) Stream endpoint |
| SIP | Session initiation protocol |
| STUN | Session Traversal Utilities for NAT |
| TCP | Transmission control protocol |
| UDP | User datagram protocol |
| VNF | Virtualized Network Functions |

# 5	Conventions

None.

# 6	Introduction

Modern network architectures follow the basic principle in separating "services" and "transport" as fundamentally described in the NGN (next generation network) reference model according to clause 7.1/[ITU-T Y.2011]. The "controller entity" belongs to the "service stratum", the "controlled entity" to the "transport stratum". The time scale of service control may vary from management plane actions till call/user/application/SLA/… individual control plane actions.

A brief background of the three control/management plane technologies OpenFlow, H.248 and ForCES is provided as well as their original purpose in network applications.

## 6.1	Brief protocol history

The basic idea of H.248 is the decomposition of a network element into control plane and user plane functions. The considered network element types cover circuit switches, frame, cell and packet switches, packet routers and gateways.

OpenFlow (and ForCES) share the same concept of control/user plane separation, but both are limited with respect to network element types in scope (Ethernet switches and IP routers respectively). Both protocols are also not that established and mature than H.248.

The H.248 technology is driven by the industry in terms of major manufactures and network operators, whereas OpenFlow (and ForCES) origin from academia. For instance, none of the major switch and router vendors offer OpenFlow or ForCES interfaces in their products (2012).

## 6.2	Terminology relations

Each control technology uses their specific terminology. Table 1 provides a rough comparison of OpenFlow specific terms and their relation in H.248.

Table 1 – Terminology relations between OpenFlow and H.248

| Term | OpenFlow | H.248 | Comments |
| --- | --- | --- | --- |
| Action: | an operation that forwards the packet to a port or modiﬁes the packet, such as decrementing the TTL ﬁeld. Actions may be speciﬁed as part of the instruction set associated with a ﬂow entry or in an action bucket associated with a group entry. Actions may be accumulated in the Action Set of the packet or applied immediately to the packet | relates to the concept of an "H.248 command" in the widest sense |  |
| Action Bucket: | a set of actions and associated parameters, deﬁned for groups. | The concept of a "bucket" could be related with the H.248concept of a "descriptor" (TBC). |  |
| Action Set | a set of actions associated with the packet that are accumulated while the packet is processed by each table and that are executed when the instruction set instructs the packet to exit the processing pipeline | relates to the concept of an "H.248 transaction" or "H.248 message" in the widest sense |  |
| Controller: | An entity interacting with the OpenFlow switch using the OpenFlow protocol. | MGC |  |
| Flow Entry: | an element in a ﬂow table used to match and process packets. It contains a set of match ﬁelds for matching packets, a priority for matching precedence, a set of counters to track packets, and a set of instructions to apply. |  |  |
| Flow Table: | A stage of the pipeline, contains ﬂow entries. |  |  |
| Group: | a list of action buckets and some means of choosing one or more of those buckets to apply on a per-packet basis. | relates to the concept of an "H.248 context" in the widest sense |  |
| Instruction: | Instructions are attached to a ﬂow entry and describe the OpenFlow processing that happen when a packet matches the ﬂow entry. An instruction either modiﬁes pipeline processing, such as direct the packet to another ﬂow table, or contains a set of actions to add to the action set, or contains a list of actions to apply immediately to the packet. | i.e., policy rule actions (see also [ITU-T H.248.79]) |  |
| Match Field: | a ﬁeld against which a packet is matched, including packet headers, the ingress port, and the metadata value. A match ﬁeld may be wildcarded (match any value) and in some cases bitmasked | i.e., an information element in policy rule conditions (see also [ITU-T H.248.79]) |  |
| Metadata: | a maskable register value that is used to carry information from one table to the next. |  |  |
| Meter: | a switch element that can measure and control the rate of packets. The meter trigger a meter band if the packet rate or byte rate passing through the meter exceed a predeﬁned threshold. If the meter band drops the packet, it is called a Rate Limiter. | a combination of - H.248.53 policer and - H.248 statistics |  |
| Outermost Tag: | the tag that appears closest to the beginning of a packet. |  |  |
| Packet: | an Ethernet frame, including header and payload. | Lx-PDU in general | OF is tightly coupled to ET! |
| Pipeline: | the set of linked ﬂow tables that provide matching, forwarding, and packet modiﬁcations in an OpenFlow switch. | could be modelled as a Context |  |
| Port: | where packets enter and exit the OpenFlow pipeline. May be a physical port, a logical port deﬁned by the switch, or a reserved port deﬁned by the OpenFlow protocol. | could be modelled as a Termination |  |
| Tag: | a header that can be inserted or removed from a packet via push and pop actions. | Lx-PCI in general |  |

## 6.3	Objectives in terms of network architecture

Figure 1 illustrates the high level decomposed control model for OpenFlow versus H.248.

It may be observed that both models largely overlap and consider identical scopes related to the network "packet forwarding plane" and the separation line between controlling and controlled entity.

For completeness, Figure 2 depicts the correspondent ForCES model versus H.248. There are actually similar observations and conclusions as in case of OpenFlow.

Figure 1 – Decomposed control models – OpenFlow vs H.248

Figure 2 – Decomposed control models – ForCES vs H.248

## 6.4	Relationship between controlling and controlled entities

The relationship between the controlling and controlled entity is asymmetrical, given by a master/slave or master/agent relation. Thus, there is not any peer-to-peer or client/server relation (with flexible client/server role assignments).

It may be concluded that OpenFlow, H.248 and ForCES follow a master/slave asymmetry, from perspective of a control/management plane protocol.

For H.248: H.248 inherently follows that control model as supports push as well as push mode control (see [ITU-T H.248.55].

## 6.5	Time scale of network control: call-dependent vs call-independent control actions

The fundamental procedures between the controlling and controlled entity are "control actions" in direction from controller (master) down to the controlled entity (slave). Control actions are normally "pushed" (push mode), i.e., the initial stimuli originates at controlling entity level, or could be also "pulled" (pull mode), i.e., the initial stimuli originates at the controlled entity.

There are a plethora of use cases, possibilities, service scenarios concerning the stimuli type behind such a control action. Table 2 indicates typical timescales between "high dynamical", call dependent packet path (node) control till "static", "semi-permanent", call-independent packet path (node) control.

Table 2 – Timescale of network control actions – Order and examples

| O. | Timescale of "control action" | Origin of stimuli for control action (examples) | H.248 | OpenFlow |
| --- | --- | --- | --- | --- |
| 1 | microseconds | event detection in packet flow (user plane), notified to controlling entity | Yes | ? |
| 2 | milliseconds | call control signalling (control plane) | Yes | ? |
| 3 | seconds | security policy signalling (control plane) related to filter updates | Yes | ? |
| 4 | 10's of seconds | security policy signalling (control plane) related to filter updates | Yes | ? |
| 5 | minutes | packet flow individual performance monitoring (control plane) (NOTE 1) | Yes | ? |
| 6 | 10's of minutes | packet flow individual configurations (management plane, NOTE 2). | Yes | Yes |
| 7 | hours | packet flow independent configurations related to "day time specific network traffic profiles", e.g. related to QoS engineering (control plane (NOTE 3), management plane). | Yes | Yes |
| 8 | days | packet flow independent configurations (e.g., driven by temporary routing decisions) | Yes | Yes |
| 9 | weeks | packet flow individual configurations as "semi-permanent" connection (management plane, NOTE 4). | Yes | Yes |
| NOTE 1 – E.g., either conditional reporting by controlled entity or/and auditing (e.g. interval based) by controlling entity. NOTE 2 – There are basically two options for triggering a "control action" via management plane: either via the management entity of the "controlling entity" (indirect) or via the management entity of the "controlled entity", - under the prerequisite of coordinated management entities in the management plane. In case of H.248, see e.g. illustration in Figure 1/[ETSI TS 183 025] and example data object (as managed/controlled object) in Annex B/[ETSI TS 183 025]. NOTE 3 – E.g., a centralized "QoS control server". NOTE 4 – See e.g., [ITU-T H.248.21] in case of H.248 related use case. |  |  |  |  |

For H.248: is basically applicable at all timescales as outlined in above Table 2. For instance, H.248-controlled deep packet inspection (DPI; [ITU-T H.248.86]) could be actually applied on all above time scales (see also Appendices I and IV in [ITU-T H.248.86]).

# 7	Packet forwarding model

## 7.1	OpenFlow processing model

Figure 3 provides a copy of the packet processing model of OpenFlow. The processing pipeline represents the common basic architecture: incoming packets are compared against a set of policy rule conditions, and the subsequent execution of associated policy rule action(s) in case of match events. The conditions are ordered by the usual concept of hierarchy and priorities. The performance objective of high packet throughput is addressed by the usual concept of a "staged pipeline", which represents a hierarchical internal packet path.

Figure 3 – Packet ﬂow through the processing pipeline (see Fig. 2/[OPENFLOW SS])

The H.248 MG internal packet processing pipeline is quite similar, some application specific models are illustrated in [ITU-T H.248.43] (with scope on filtering), [ITU-T H.248.53] (with scope on traffic policing) or [ITU-T H.248.86] (with scope on packet inspection). The "illustration of an IP processing model" with a mixture of filtering, QoS tagging, address translation, policing, etc is e.g. contained in Annex D/[ETSI TR 183 068].

## 7.2	Policy rule based description of packet operations

This is a general description technique, applicable for all kind of network element types, inclusive H.248 packet path entities, involved in any kind of packet processing (see also clause 7.3/[ITU-T H.248.79]).

Figure 4 – Order of operations (within a layer) – Unidirectional packet bearer traffic models using the concept of policy rules

### 7.2.1	OpenFlow: basic policy rule format

Figure 5 shows a comparison of the OpenFlow basic policy rule format and the generic DPI policy rule format of [ITU-T Y.2770].

Figure 5 – OpenFlow: basic policy rule format 
(vs generic DPI policy rule format [ITU-T Y.2770])

### 7.2.2	OpenFlow: staged process model – concatenation of multiple policy rules

The general principle is described in ### .

### 7.2.3	OpenFlow: ###

# 8	Stage 1 – Services (use cases)

{Editor's note (2013-11 meeting): stage 1 information are a place holder for the time being; not sure whether we go into detailed considerations  …}

# 9	Stage 2 – Protocol requirements

{Editor's note (2013-11 meeting): stage 2 information may be elaborated in order to understand better the history of the protocols. E.g. ForCES made following agenda for their comparison

- Protocol Requirement: Configuration of Modeled Elements

- Protocol Requirement: Support for Secure Communication

- Protocol Requirement: Scalability

- Protocol Requirement: Multihop

- Protocol Requirement: Message Priority

- Protocol Requirement: Reliability

- Protocol Requirement: Interconnect Independence

- Protocol Requirement: CE Redundancy or CE Failover

- Protocol Requirement: Packet Redirection/Mirroring

- Protocol Requirement: Topology Exchange

- Protocol Requirement: Dynamic Association

- Protocol Requirement: Command Bundling

- Protocol Requirement: Asynchronous Event Notification

- Protocol Requirement: Query Statistics

Protocol Requirement: Protection Against Denial of Service Attacks

…}

## 9.1	Decomposed network element architecture

See Table 3:

Table 3 – Protocol requirements – H.248 vs OpenFlow 
– Relation controller to controlled network element

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
| [R#1] | Single primary controller per switch/MG | Yes | Yes |  |
| [R#2] | Secondary Controller(s) as standby entities | Yes | Yes |  |
| [R#3] | Multiple primary controllers | Yes | Yes? (NOTE 1) |  |
| [R#4] | MG standalone mode | No | No | Temporary condition without any contact to a controller |
| NOTE 1 – The justification of that requirement is missing. It is also not clear how the parallel operation of multiple active controllers would be coordinated. |  |  |  |  |

## 9.2	Network function virtualization

The physical entity of the network functions "controller" and "controlled entity" could be basically virtualized. The virtualization of both network functions is normally outside the control protocol itself. However, the control protocol could provide inherent support for virtualized network functions, primarily in case of shared resources of the physical entity by multiple virtualized instances. H.248 is such a protocol, see clause 11.1 "Multiple Virtual MGs" in [ITU-T H.248.1]

See Table 4:

Table 4 – Protocol requirements – H.248 vs OpenFlow 
– Network function virtualization

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
| [R#1] | Virtualization of controlled entity | Yes | ? |  |
| [R#2] | Virtualization of controller | Yes | ? |  |

Further protocol support by H.248: see [ITU-T H.248.83].

## 9.3	Operations on traffic flows

The notion of traffic flow may be limited on packet flow, in which 'packet' represents generally a protocol data unit (PDU) at protocol layer Lx (i.e., 'packet' = Lx-PDU).

### 9.3.1	General operations

Table 5 – Protocol requirements – H.248 vs OpenFlow 
– Flow operations

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
| [R#1] | Flow termination | Yes | No | Protocol stack termination in CNE |
| [R#2] | Flow identification | Yes (NOTE 1) | No |  |
| [R#3] | Flow forwarding | Yes | Yes | Basic mode |
| [R#4] | Flow modification | Yes | Yes |  |
| [R#5] | Reporting of flow information | Yes | Yes |  |
| [R#6] |  |  |  |  |
| NOTE 1 – Scope of [ITU-T H.248.86] (by e.g. realizing IPFIX/Y.2770 flow identification service). |  |  |  |  |

### 9.3.2	Regarding policy rule conditions ("flow identification")

The match field has been expanding as the ONF specifications evolve - from a

- 10-tuple [McKowen-2008], to

- 12-tuple [OFS-1.0],

- 15-tuple [OFS-1.1.0], to

- 39-tuple [OFS-1.3] (14 required and 25 optional)

- and few additions in further versions.

Table 6 – Flow identification – OpenFlow – Flow identifier elements

[OPENFLOW SS]: Flow Match Fields

/* OXM Flow match field types for OpenFlow basic class. */ enum oxm_ofb_match_fields { OFPXMT_OFB_IN_PORT = 0, /* Switch input port. */ OFPXMT_OFB_IN_PHY_PORT = 1, /* Switch physical input port. */ OFPXMT_OFB_METADATA = 2, /* Metadata passed between tables. */ OFPXMT_OFB_ETH_DST = 3, /* Ethernet destination address. */ OFPXMT_OFB_ETH_SRC = 4, /* Ethernet source address. */ OFPXMT_OFB_ETH_TYPE = 5, /* Ethernet frame type. */ OFPXMT_OFB_VLAN_VID = 6, /* VLAN id. */ OFPXMT_OFB_VLAN_PCP = 7, /* VLAN priority. */ OFPXMT_OFB_IP_DSCP = 8, /* IP DSCP (6 bits in ToS field). */ OFPXMT_OFB_IP_ECN = 9, /* IP ECN (2 bits in ToS field). */ OFPXMT_OFB_IP_PROTO = 10, /* IP protocol. */ OFPXMT_OFB_IPV4_SRC = 11, /* IPv4 source address. */ OFPXMT_OFB_IPV4_DST = 12, /* IPv4 destination address. */ OFPXMT_OFB_TCP_SRC = 13, /* TCP source port. */ OFPXMT_OFB_TCP_DST = 14, /* TCP destination port. */ OFPXMT_OFB_UDP_SRC = 15, /* UDP source port. */ OFPXMT_OFB_UDP_DST = 16, /* UDP destination port. */ OFPXMT_OFB_SCTP_SRC = 17, /* SCTP source port. */ OFPXMT_OFB_SCTP_DST = 18, /* SCTP destination port. */ OFPXMT_OFB_ICMPV4_TYPE = 19, /* ICMP type. */ OFPXMT_OFB_ICMPV4_CODE = 20, /* ICMP code. */ OFPXMT_OFB_ARP_OP = 21, /* ARP opcode. */ OFPXMT_OFB_ARP_SPA = 22, /* ARP source IPv4 address. */ OFPXMT_OFB_ARP_TPA = 23, /* ARP target IPv4 address. */ OFPXMT_OFB_ARP_SHA = 24, /* ARP source hardware address. */ OFPXMT_OFB_ARP_THA = 25, /* ARP target hardware address. */ OFPXMT_OFB_IPV6_SRC = 26, /* IPv6 source address. */ OFPXMT_OFB_IPV6_DST = 27, /* IPv6 destination address. */ OFPXMT_OFB_IPV6_FLABEL = 28, /* IPv6 Flow Label */ OFPXMT_OFB_ICMPV6_TYPE = 29, /* ICMPv6 type. */ OFPXMT_OFB_ICMPV6_CODE = 30, /* ICMPv6 code. */ OFPXMT_OFB_IPV6_ND_TARGET = 31, /* Target address for ND. */ OFPXMT_OFB_IPV6_ND_SLL = 32, /* Source link-layer for ND. */ OFPXMT_OFB_IPV6_ND_TLL = 33, /* Target link-layer for ND. */ OFPXMT_OFB_MPLS_LABEL = 34, /* MPLS label. */ OFPXMT_OFB_MPLS_TC = 35, /* MPLS TC. */ OFPXMT_OFP_MPLS_BOS = 36, /* MPLS BoS bit. */ OFPXMT_OFB_PBB_ISID = 37, /* PBB I-SID. */ OFPXMT_OFB_TUNNEL_ID = 38, /* Logical Port Metadata. */ OFPXMT_OFB_IPV6_EXTHDR = 39, /* IPv6 Extension Header pseudo-field */ OFPXMT_OFB_PBB_UCA = 41, /* PBB UCA header field. */ };

The policy rule conditions of OF-based flow identification are basically limited to

- protocol layers L2 to L4 (similar as for "shallow packet inspection");

- L2 technology dependent ("Ethernet");

- IP L4 limited on a few transport protocols; and

- without support of any L4+ multiplexing (because the "top layer (L4) flow" granularity is limited to the usual "5-tuple" IP transport connection structure).

For instance: "RTP multiplexing" is not supported by OpenFlow, but in H.248 (see [ITU-T H.248.RTPMUX].

### 9.3.3	Regarding policy rule actions

See Table 7:

Table 7 – Policy rule actions – OpenFlow – Action types

[OPENFLOW SS]: Action Structure

enum ofp_action_type { OFPAT_OUTPUT = 0, /* Output to switch port. */ OFPAT_COPY_TTL_OUT = 11, /* Copy TTL "outwards" -- from next-to-outermost to outermost */ OFPAT_COPY_TTL_IN = 12, /* Copy TTL "inwards" -- from outermost to next-to-outermost */ OFPAT_SET_MPLS_TTL = 15, /* MPLS TTL */ OFPAT_DEC_MPLS_TTL = 16, /* Decrement MPLS TTL */ OFPAT_PUSH_VLAN = 17, /* Push a new VLAN tag */ OFPAT_POP_VLAN = 18, /* Pop the outer VLAN tag */ OFPAT_PUSH_MPLS = 19, /* Push a new MPLS tag */ OFPAT_POP_MPLS = 20, /* Pop the outer MPLS tag */ OFPAT_SET_QUEUE = 21, /* Set queue id when outputting to a port */ OFPAT_GROUP = 22, /* Apply group. */ OFPAT_SET_NW_TTL = 23, /* IP TTL. */ OFPAT_DEC_NW_TTL = 24, /* Decrement IP TTL. */ OFPAT_SET_FIELD = 25, /* Set a header field using OXM TLV format. */ OFPAT_PUSH_PBB = 26, /* Push a new PBB service tag (I-TAG) */ OFPAT_POP_PBB = 27, /* Pop the outer PBB service tag (I-TAG) */ OFPAT_EXPERIMENTER = 0xffff };

## 9.4	High availability of services and networks

High service and network availability is basically achieved by redundant network resources (Table 8) and load controlled resources (Table 9). H.248 got extensive requirements in both areas from day one, hence correspondent support in integral part of the protocol.

Table 8 – Protocol requirements – H.248 vs OpenFlow 
– Redundancy support

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
| [R#1] | Changeover procedures between primary and secondary controllers | Yes | ? |  |
| [R#2] | Changeover procedures between primary and secondary controlled entities | Yes | ? |  |
| NOTE 1 – #. |  |  |  |  |

NOTE –The "ServiceChange" based services in H.248 cover essentially above redundancy requirements.

Table 9 – Protocol requirements – H.248 vs OpenFlow 
– Load control and overload protection

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
| [R#3] | Overload control of "controlled entity" | Yes (NOTE 1) | No |  |
| [R#4] | Load control due to specific resource components of "controlled entity" | Yes (NOTE 2) | No |  |
| NOTE 1 – See [ITU-T H.248.11, H.248.10]. NOTE 2 – See [ITU-T H.248.32]. |  |  |  |  |

## 9.5	Public networks: support of regulation related requirements

LI, emergency, …

# 10	Stage 3 – Protocol capabilities

{Editor's note (2013-11 meeting): just some initial areas  …}

Table 10 – Protocol capabilities – H.248 vs OpenFlow – Basic Capabilities

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  |  | Yes | Yes |  |

## 10.1	Termination types: supported protocol stacks in forwarding plane

Table 11 – Protocol capabilities – H.248 vs OpenFlow 
– Termination types for packet-oriented bearer technologies

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | L2: Ethernet | Yes | Yes (NOTE 1) |  |
|  | L2: other than Ethernet | Yes | No |  |
|  | L3: IP (V4 and V6) | Yes | Yes (NOTE 1) |  |
|  | L3: other than IP | Yes | No |  |
|  | L4: IP transport protocols TCP, UDP, SCTP | Yes | Yes |  |
|  | L4: IP transport protocols DCCP, SCTP, … | Yes | No |  |
|  | Tunneled IP stacks | Yes | No | OpenFlow just IETF-BRM |
| NOTE 1 – The OpenFlow basic network element is an Ethernet switch! Thus, the notion of 'OpenFlow switch', rather than IP router or gateway etc. The OpenFlow protocol stack is fundamentally IP-over-Ethernet. |  |  |  |  |

Table 12 – Protocol capabilities – H.248 vs OpenFlow 
– Termination types for Circuit-oriented bearer technologies

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | circuit-switched bearer | Yes | No (NOTE 1) |  |
| NOTE 1 – OpenFlow is not considering any interworking between IP or Ethernet network domains with circuit switched networks. |  |  |  |  |

## 10.2	Connection models: IP mode of operation in forwarding plane

Table 13 – Protocol capabilities – H.248 vs OpenFlow 
– Connection topologies

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | pure Ethernet termination | Yes | No |  |
|  | pure IP termination | Yes | No | H.248: (IP, non-IP) topology |
|  | ET-to-ET (switched Ethernet) | Yes | Yes |  |
|  | IP-to-IP (in IP router mode) | Yes | Yes |  |
|  | IP-to-IP (in IP router mode plus local NAT) | Yes | No |  |
|  | IP-to-IP (in B2BIH mode) | Yes | No |  |
|  | IP-to-IP (with protocol version translation) | Yes | No | such as V4-to-V6 |
|  | tunnelled to untunnelled IP interworkign | Yes | No |  |
|  | IP multipoint topologies | Yes | No |  |
| NOTE 1 – # |  |  |  |  |

Table 14 – Protocol capabilities – H.248 vs OpenFlow 
– IP connection models

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | IPR native | Yes | Yes |  |
|  | IPR with NAT | Yes | No |  |
|  | B2BIH native | Yes | No | V4-to-V4 ¦ V6-to-V6 |
|  | B2BIH V4-to-V6 | Yes | No |  |
| NOTE 1 – # |  |  |  |  |

Table 15 – Protocol capabilities – H.248 vs OpenFlow 
– Policy rule actions – Generic view

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | Packet Forwarding (called Output in OF) | Yes | Yes |  |
|  | Packet Discarding (called Drop in OF) | Yes | Yes |  |
|  | Packet Scheduling (called Set-Queue in OF) | Yes | Yes | See clause 7.1.6/[ITU-T Y.1221] |
|  | Packet Marking | Yes | Yes (part of Set-Queue)? | See clause 7.1.4/[ITU-T Y.1221] |
|  | Traffic shaping | "Yes" (NOTE 1) | Yes (part of Set-Queue) | See clause 7.1.5/[ITU-T Y.1221] |
|  | Traffic policing | Yes | No | See clause 7.1.2/[ITU-T Y.1221] |
|  | Complete packet header removal or addition (called Push-Tag/Pop-Tag in OF) | Yes | Yes |  |
|  | Partial packet header modification (called Set-Field in OF) | Yes | Yes |  |
|  | Packet lifetime modification (called Change-TTL in OF) | No | Yes | Limited to IP and MPLS TTL. |
|  | Packet-, Flow- and Flow-aggregate-related Measurements |  |  |  |
| NOTE 1 – # |  |  |  |  |

Q1: how is NA(P)T realized?

Appendix I

Scope of OpenFlow – Use cases

(This appendix does not form an integral part of this Recommendation.)

This appendix provides example ##

## I.1	Experimental prototyping (academic/research area)

Table I.1 – OpenFlow use case "prototyping"

| Service description: |  |
| --- | --- |
| Reference(s): |  |
| Network architecture model: |  |
| Time scale of flow control/management: |  |
| Key protocol capabilities: |  |
| H.248 solution: |  |

## I.2	Software defined networking (SDN)

… network management level (traffic engineering)

automated management, self-management

## I.3	Network functions virtualisation (NFV)

[ETSI NFV-SWA] defines a virtualized network functions (VNF) architecture and informative Annex 1.4 decomposed control models with relevance for H.248. Exemplary use cases are described in Annex A.3 and Annex B.

{Editor's note (2014-07 meeting): use cases of Annex A.3/[ETSI NFV-SWA] may be adapted on an H.248 environment.  H.248 perspective. Use case of Annex B may be considered, too …}

## I.4	Cloud oriented networking

see article "OpenFlow as a Service"

## I.5	Trill switching

## I.6	Lx-VPNs (Layer x Virtual Private Network)

### I.6.1	Layer 1 Virtual Private Network

No

### I.6.2	Layer 2 Virtual Private Network

Ethernet based (e.g., PBB, VLAN)

### I.6.3	Layer 3 Virtual Private Network

MPLS based

## I.7	Network element decomposition due to cost factor evolutions in network planes (user, control, management)

## I.8	Network element decomposition due to high availability network solutions

## I.9	Network performance management with respect to load balancing

## I.10	Network element type "programmable Ethernet switch"

## I.11	Network element type "programmable IP router"

## I.12	Network element type "programmable firewall"

Appendix II

Comparison H.248 and ForCES

(This appendix does not form an integral part of this Recommendation.)

This appendix provides example ##

## II.1	###

### II.1.1	###

Figure II.1 – ForCES Architectural Diagram (see [b-IETF RFC 5810]/Figure 1)

Appendix III

Comparison OpenFlow and ForCES

(This appendix does not form an integral part of this Recommendation.)

The ForCES Fp interface could be principally realized also with H.248 (given by an H.248 Fp profile specification). Proponents of ForCES were invited for dispositions against OpenFlow. This appendix provides a brief summary.

## III.1	Scope of ForCES

See [b-IETF RFC 3654], [b-IETF RFC 3746], [b-IETF RFC 5810], [b-IETF RFC 5812].

## III.2	Comparison OpenFlow and ForCES

There is not yet any IETF approved evaluation available. [b-IETF hares] provides a private position.

## III.3	Emulation of OpenFlow interface by ForCES

[b-IETF haleplidis] provides an attempt in emulating OpenFlow by an extension to ForCES.

Bibliography

IETF working group ForCES (Forwarding and Control Element Separation):

[b-IETF RFC 3549]	IETF RFC 3549 (07/2003), Linux Netlink as an IP Services Protocol

[b-IETF RFC 3654]	IETF RFC 3654 (11/2003), Requirements for Separation of IP Control and Forwarding

[b-IETF RFC 3746]	IETF RFC 3746 (04/2004), Forwarding and Control Element Separation (ForCES) Framework

[b-IETF RFC 5810]	IETF RFC 5810 (03/2010), Forwarding and Control Element Separation (ForCES) Protocol Specification

[b-IETF RFC 5812]	IETF RFC 5812 (03/2010), Forwarding and Control Element Separation (ForCES) Forwarding Element Model

[b-IETF hares]	Private draft-hares-forces-vs-openflow (07/2012), Analysis of Comparisons between OpenFlow and ForCES
URL: http://tools.ietf.org/id/draft-hares-forces-vs-openflow-00.txt

[b-IETF haleplidis]	Private draft-haleplidis-forces-openflow-lib (07/2012), Forwarding and Control Element Separation (ForCES) OpenFlow Model Library
URL: http://tools.ietf.org/id/draft-haleplidis-forces-openflow-lib-01.txt

IETF working group IPFIX (IP Flow Information Export):

[b-IETF RFC 5556]	IETF RFC 5556 (05/2009), Transparent Interconnection of Lots of Links (TRILL): Problem and Applicability Statement

IETF working group IRS (Interface to the Routing System):

[b-IETF RFC 5556]	IETF RFC 5556 (05/2009), Transparent Interconnection of Lots of Links (TRILL): Problem and Applicability Statement

IETF working group TRILL (Transparent Interconnection of Lots of Links):

[b-IETF RFC 5556]	IETF RFC 5556 (05/2009), Transparent Interconnection of Lots of Links (TRILL): Problem and Applicability Statement

[b-IETF RFC 6325]	IETF RFC 6325 (07/2011), Routing Bridges (RBridges): Base Protocol Specification

____________________