| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4591 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.50 (Rev): STUN functional architecture and distributed STUN server function |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution likes to progress a discussion started some meetings ago concerning a "distributed STUN server function".

Background

The proposal at the last meeting was rejected:

| C.453 (Rev.1) | H.248.50 (Rev): distributed STUN server function | Alcatel-Lucent France |
| --- | --- | --- |

This Contribution was discussed. The changes in clause 10.1.4.2 were accepted however the other proposals were not accepted. Further work is needed to define the STUN server functionality and how it is allocated to network elements.

Thus, a functional architecture model for the STUN protocol is requested, which isn't unfortunately provided by the STUN specification (RFC 5389) itself.

# Functional model for the STUN protocol

NAT traversal using STUN is the concerted work of multiple protocols. We need to isolate the STUN protocol from the entire context. Figure A provides the starting point for such a model:

Figure A – Starting point "top level functional model"

Next, the STUN protocol (RFC 5389) represents a tool which might be used by different applications ("served user instances of STUN"), such as

  - ICE (RFC 5245)

  - SIP Outbound (RFC 5626)

  - NAT Behaviour Discovery (RFC 5780)

  - etc (see Figure B for a comprehensive inventory (status2014-10).

Figure B – Served user instance ICE, SIP-OUTBOUND, …

Thus, there are functions (or protocol services) which belong to the layer of the served user instance and not STUN itself, e.g., here the "address gathering function" and "connectivity check" function (belong to ICE, not to STUN).

We'd like to follow two approaches in developing a functional model for the STUN protocol:

1st 	Identification of "STUN service qualifiers" by analysis of the STUN message structure; and

2nd	Identification of "STUN service procedures" by analysis of the STUN protocol specification.

Figure C illustrates the STUN message structure:

Figure C – STUN message structure

Obviously, a good starting point might be to focus on the two "service elements" of supported methods and attributes by STUN (NOTE: the selection of one element only, - methods or attributes -, would lead to a coarse granular model concerning the underlying problem from H.248.50 perspective).

Below a copy of the current content of the IANA registry for STUN methods (Table 1) and STUN attributes (Table 2):

Table 1 – STUN methods (IANA registry 2014-10

| Value | Name | Reference |
| --- | --- | --- |
| 0x000 | Reserved | [IETF RFC 5389] |
| 0x001 | Binding | [IETF RFC 5389] |
| 0x002 | Reserved; was SharedSecret | [IETF RFC 5389] |
| 0x003 | Allocate | [IETF RFC 5766] |
| 0x004 | Refresh | [IETF RFC 5766] |
| 0x005 | Unassigned |  |
| 0x006 | Send | [IETF RFC 5766] |
| 0x007 | Data | [IETF RFC 5766] |
| 0x008 | CreatePermission | [IETF RFC 5766] |
| 0x009 | ChannelBind | [IETF RFC 5766] |
| 0x00A | Connect | [IETF RFC 6062] |
| 0x00B | ConnectionBind | [IETF RFC 6062] |
| 0x00C | ConnectionAttempt | [IETF RFC 6062] |

Ref.: http://www.iana.org/assignments/stun-parameters/stun-parameters.xhtml#stun-parameters-2

Table 2 – STUN attributes (IANA registry 2014-10

| Value | Name | Reference |
| --- | --- | --- |
| 0x0000 | Reserved | [IETF RFC 5389] |
| 0x0001 | MAPPED-ADDRESS | [IETF RFC 5389] |
| 0x0002 | Reserved; was RESPONSE-ADDRESS | [IETF RFC 5389] |
| 0x0003 | CHANGE-REQUEST | [IETF RFC 5780] |
| 0x0004 | Reserved; was SOURCE-ADDRESS | [IETF RFC 5389] |
| 0x0005 | Reserved; was CHANGED-ADDRESS | [IETF RFC 5389] |
| 0x0006 | USERNAME | [IETF RFC 5389] |
| 0x0007 | Reserved; was PASSWORD | [IETF RFC 5389] |
| 0x0008 | MESSAGE-INTEGRITY | [IETF RFC 5389] |
| 0x0009 | ERROR-CODE | [IETF RFC 5389] |
| 0x000A | UNKNOWN-ATTRIBUTES | [IETF RFC 5389] |
| 0x000B | Reserved; was REFLECTED-FROM | [IETF RFC 5389] |
| 0x000C | CHANNEL-NUMBER | [IETF RFC 5766] |
| 0x000D | LIFETIME | [IETF RFC 5766] |
| 0x000E-0x000F | Reserved |  |
| 0x0010 | Reserved (was BANDWIDTH) | [IETF RFC 5766] |
| 0x0011 | Reserved |  |
| 0x0012 | XOR-PEER-ADDRESS | [IETF RFC 5766] |
| 0x0013 | DATA | [IETF RFC 5766] |
| 0x0014 | REALM | [IETF RFC 5389] |
| 0x0015 | NONCE | [IETF RFC 5389] |
| 0x0016 | XOR-RELAYED-ADDRESS | [IETF RFC 5766] |
| 0x0017 | REQUESTED-ADDRESS-FAMILY | [IETF RFC 6156] |
| 0x0018 | EVEN-PORT | [IETF RFC 5766] |
| 0x0019 | REQUESTED-TRANSPORT | [IETF RFC 5766] |
| 0x001A | DONT-FRAGMENT | [IETF RFC 5766] |
| 0x001B-0x001F | Unassigned |  |
| 0x0020 | XOR-MAPPED-ADDRESS | [IETF RFC 5389] |
| 0x0021 | Reserved (was TIMER-VAL) | [IETF RFC 5766] |
| 0x0022 | RESERVATION-TOKEN | [IETF RFC 5766] |
| 0x0023 | Reserved |  |
| 0x0024 | PRIORITY | [IETF RFC 5245] |
| 0x0025 | USE-CANDIDATE | [IETF RFC 5245] |
| 0x0026 | PADDING | [IETF RFC 5780] |
| 0x0027 | RESPONSE-PORT | [IETF RFC 5780] |
| 0x0028-0x0029 | Reserved |  |
| 0x002A | CONNECTION-ID | [IETF RFC 6062] |
| 0x002B-0x002F | Unassigned |  |
| 0x0030 | Reserved |  |
| 0x0031-0x7FFF | Unassigned |  |
| 0x8000-0x8021 | Unassigned |  |
| 0x8022 | SOFTWARE | [IETF RFC 5389] |
| 0x8023 | ALTERNATE-SERVER | [IETF RFC 5389] |
| 0x8024 | Reserved |  |
| 0x8025 | Unassigned |  |
| 0x8026 | Reserved |  |
| 0x8027 | CACHE-TIMEOUT | [IETF RFC 5780] |
| 0x8028 | FINGERPRINT | [IETF RFC 5389] |
| 0x8029 | ICE-CONTROLLED | [IETF RFC 5245] |
| 0x802A | ICE-CONTROLLING | [IETF RFC 5245] |
| 0x802B | RESPONSE-ORIGIN | [IETF RFC 5780] |
| 0x802C | OTHER-ADDRESS | [IETF RFC 5780] |
| 0x802D | ECN-CHECK STUN | [IETF RFC 6679] |
| 0x802E-0xBFFF | Unassigned |  |
| 0xC000 | CISCO-STUN-FLOWDATA | [Dan_Wing] |
| 0xC001-0xFFFF | Unassigned |  |

Ref.: http://www.iana.org/assignments/stun-parameters/stun-parameters.xhtml#stun-parameters-4

Final step would be the visualization of both STUN service qualifiers with the "served user instance" model (from Figure B), see Figure D:

Figure D – STUN service qualifiers "Method" and "Attribute"

It may be observed that one method and 11 attributes are introduced by STUN itself, and on top of that there are additional nine methods and 22 attributes which are "application specific", i.e., dependent on the particular "STUN application".

As an interim result, it might be feasible in developing a general functional model for STUN by consideration of these service qualifiers, however, it seems to be still not appropriate for considered problem.

Our objective is the specific network model and STUN usage as outlined by Figure I.1/H.248.50, hence we may limit our considerations on the combination of "ICE/STUN" only. Figure E outlines the correspondent subset from Figure D, plus the additional indication of protocol procedures at STUN and ICE level:

Figure E – Functional model for STUN application "ICE"

There's now the question whether this model (Figure E) allows a sufficient differentiation between the two considered STUN functions ("STUN-based ICE procedures") of STUN-based address gathering and STUN-based connectivity checks. Table 3 provides an analysis:

Table 3 – STUN-based ICE procedures

| ICE level: | STUN level: |  |  |
| --- | --- | --- | --- |
| ICE procedure | a) STUN procedure(s) | b) STUN method(s) | c) STUN attribute(s) |
| 1) address gathering | Authentication & message-integrity mechanisms | Binding | e.g.: 1) MAPPED-ADDRESS 2) USERNAME 3) MESSAGE-INTEGRITY 8) XOR-MAPPED-ADDRESS |
| 2) connectivity checks | Authentication & message-integrity mechanisms | Binding | e.g.: 2) USERNAME 3) MESSAGE-INTEGRITY 12) PRIORITY 13) USE-CANDIDATE 14) ICE-CONTROLLED 15) ICE-CONTROLLING |
| Difference? | No | No | Yes |

It could be concluded that both STUN functions are fairly similar, only slightly differing with respect to the set of used STUN attributes. Hence, we are proposing to not follow further such a modelling approach (due to the difficulty in finding good names for the two STUN functions, beyond the names of the procedural names at ICE level).

Anyway, it is a matter of fact that the STUN server function is distributed across two different network elements. A way forward might be a view on the entire network solution (i.e., beyond the pure consideration of the STUN protocol and STUN application protocol). When looking at the boundary conditions at UE side it could be noted that the

- STUN-based address gathering (= step I in Figure I.1) occurs before any call control signalling (thus, UE has apriori knowledge about the network address of the contacted STUN server)

and the

- STUN-based connectivity checks (= step IV in Figure I.1) during the call establishment phase (thus, UE get the network address of the contacted STUN server dynamically assigned).

Observation:

Thus, we may differentiate both STUN server functions from an overall network solution perspective in

- call-independent STUN server functions and

- call-dependent STUN server functions.

And the MG-embedded STUN server function serves category (2) and MG remote, self-contained STUN server entities covers category (1).

# Conclusion

We may summarize:

- a particular STUN server (as a functional, physical or virtual entity) has a unique name given by a FQDN;

- a network domain is not limited to a single STUN server, rather may offer multiple STUN server entitiess;

- there are 
a) stand-alone STUN servers and 
b) H.248 MG-embedded STUN server functions (in context of H.248.50); 
the later one is termed as "MG act-as STUN server" function;

- ICE and STUN: there are two basic, STUN-based ICE procedures:
4.1)	STUN-based address gathering and
4.2)	STUN-based connectivity checks
and both STUN-based services could be provided by a single or different STUN server(s). This is transparent from UE (ICE agent) perspective (apart from the fact of different network address information).

- the new package-independent procedures in clause 10.1 represent a scenario with two STUN servers: a self-contained STUN server involved in the ICE address gathering phase and the "MG act-as STUN server" involved in the ICE connectivity checks.

Conclusions:

- there isn't any need to differentiate between the "STUN server" qualified as network element or qualified as protocol role;

- the internal modelling of the STUN protocol itself by identification of different protocol functions isn't straightforward, rather difficult, primarily due to the "tandem usage" of STUN in combination with a STUN application protocol;

- the STUN server function doesn't need to be decomposed in sub-functions for the considered problem here;

- it's rather the opposite: the ICE function may be decomposed in multiple, STUN-based sub-functions and different STUN server entities may be involved in these functions; and

- the differentiation criteria is "call-independent" and "call-dependent" STUN server function.

Below an attempt in resolving the issues in following sections:

- the STUN application-agnostic clause 6.1.5 and

- the STUN application-specific (ICE) clause 10.1.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against the input draft AVD-4592 to this meeting (AVD-4592).

Change proposal #1:

[Begin Proposed Correction]

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	NAT traversal [ITU-T Y.2111]: The operation of adapting the IP addresses so that the packets in the media flow can pass through a far-end (remote) NAT.

3.1.2	network address translation [ITU-T Y.2111]: The operation by which IP addresses are translated (mapped) from one address domain to another address domain.

3.1.3	pinhole [ITU-T H.248.37]: A configuration of two associated H.248 IP terminations within the same context, which allows/prohibits unidirectional forwarding of IP packets under specified conditions (e.g., address tuple).

3.1.4	symmetric NAT [IETF RFC 3489]: A symmetric NAT is one where all requests from the same internal IP address and port, to a specific destination IP address and port, are mapped to the same external IP address and port.

3.1.x1	STUN agent [IETF RFC 5389]: an entity that implements the STUN protocol. The entity can be either a STUN client or a STUN server.

3.1.x2	STUN client [IETF RFC 5389]: an entity that sends STUN requests and receives STUN responses. A STUN client can also send indications.

3.1.x3	STUN server [IETF RFC 5389]: an entity that receives STUN requests and sends STUN responses. A STUN server can also send indications.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

### 6.1.5	Note to "STUN server" capability

A network domain may provide one or multiple STUN server(s). A STUN server may be realized as stand-alone network element or as H.248 entity-embedded STUN server functions (in context of this Recommendation). For instance, the STUN agent provided by the MG is then called "MG act-as STUN server" function.

The STUN service(s) may be provided by a single or multiple STUN servers; typically dependent on the network configuration and/or STUN application.

The internal modeling of the STUN protocol in "service categories" (protocol functions) is difficult, however, STUN server functions could be classified in call-independent and call-dependent categories. It should be noted that such a classification scheme is not useful in general, but definetely suited in context of H.248 gateways.

For instance, the STUN application specific use cases (for STUN application 'ICE') according to clause 6.1.1 or Appendix I.1:

A stand-alone "STUN server" is involved during the STUN-based address gathering phase. And the H.248 MG as "STUN server" is involved during the STUN-based connectivity check phase. The first STUN server function is "call-independent" because actually happening before any call control signalling (from MGC perspective), and the second STUN server function is "call-dependent" because associated to call control signalling.

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 10	Package-independent NAT-T procedures

## 10.1	Support for MG terminated STUN-based connectivity checks

### 10.1.1	Overview

STUN-based connectivity checks can be originated or terminated by the MG. Clause 8.2 defines support for MG originated STUN connectivity checks. This clause covers network scenarios where the MG is requested to terminate the STUN-based connectivity check procedure of STUN application ICE (e.g., model according Figure 2). Appendix I.1 illustrates an example use case in more detail.

NOTE – A MG which only supports termination of STUN-based connectivity check procedures necessarily only supports ICE-lite.

{

### 10.1.2	Required MG support functions

…

[End Proposed Correction]

____________________