| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4599 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.Sup.13 (Rel. 2): Terminology related to H.248 related B2BUA entities |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes terminology related to H.248 related B2BUA entities.

1	Background

The discussion on this topic was started at the past meetings (see C-467, C-455, C-247), and is coupled with the IETF WG STRAW on their work on "media plane B2BUAs".

+-------+            +------------------+              +-----+

| Alice |            | Mediaplane B2BUA |              | Bob |

+-------+            +------------------+              +-----+

Question 1: Why is Q.3/16 required to clarify the B2BUA terminology subject?

Answer: The media plane B2BUA relates to a gateway interworking function (see also AVD-4611), which might be part of an integrated or decomposed gateway, but the IETF does NOT work on decomposed gateway architectures.

Question 2: Which H.248.x IWFs are related to media plane B2BUAs?

Answer:

- implicitly in H.248.64 related to "IP B2BUA"

- implicitly in H.248.48 / H.248.87 related to "RTCP B2BUA"

- implicitly in H.248.84 / H.248.89 related to "TCP B2BUAs" (TCP modes of operation)

- implicitly in H.248.88 related to "RTP B2BUAs" (RTP topologies)

- explicitly in H.248.78 related to "MSRP B2BUA" (B-ALG)

- explicitly in H.248.90 related to "TLS B2BUA"

- others

The IETF STRAW did publish a first taxonomy RFC 7092 on B2BUAs, which indicates explicitly that decomposed gateway architecture match that model:

Furthermore, the media plane B2BUA work is continueing, see e.g.

- media plane B2BUA "RTCP" (draft-ietf-straw-b2bua-rtcp);

- media plane B2BUA "DTLS-SRTP" (draft-ram-straw-b2bua-dtls-srtp); and

- media plane B2BUA "STUN" (draft-ietf-straw-b2bua-stun);

Again, all three B2BUA variants overlap already with existing work items of ITU-T Q.3/16 (see H.248.87, H.248.93 and H.248.50).

Question 3: Is it possible to clarify the relation between the IETF RFC 7092 B2BUA model and the ITU-T H.248 model of decomposed gateways?

Answer: Yes, and especially due to the explicit introduction of a media plane B2BUA function by the IETF.

See also STRAW email list, subject "[straw] b2bua-taxonomy: proposal to change "media-plane" to "signaling/media-plane"":
http://www.ietf.org/mail-archive/web/straw/current/msg00183.html 
http://www.ietf.org/mail-archive/web/straw/current/msg00195.html

2	Discussion

2.1	Problem statement

The discussion would be much simpler when the IETF would have defined a terminology for B2BUAs, and not a taxonomy (RFC 7092) only. Status are again the old definitions from RFC 3261 (2002), -

The following terms are defined in [RFC3261], Section 6.

B2BUA:   a SIP Back-to-Back User Agent, which is the logical

combination of a User Agent Server (UAS) and User Agent

Client (UAC).

UAS:     a SIP User Agent Server.

UAC:     a SIP User Agent Client.

-, which were (and still are) insufficient concerning the media plane aspects. Above terminology group is lacking some principle information such as "what is a user agent?" or "includes a user agent media plane functionality or not?".

What should be done from ITU-T Q.3/16 side?

- Define and agree on a H.248 gateway model which could be related with the IETF STRAW B2BUA model (RFC 7092).

- Define a hierarchical terminology for such a model.

Concerning list item (2), such a hierarchy would probably look like as follows, see Figure A:

Figure A1 – Hierarchical terminology framework in scope

The high level hierarchy from Figure A1 could be further detailed when considering more aspects from RFC 7092, see Figure A2:

Figure A2 – Hierarchical terminology framework of RFC 7092 in all details

Though the SIP as such is out of scope of ITU-T Q.3/16, we need to touch some aspects due to the fact that the H.248 MGC is tightly coupled to call control functions. Figure A3 summarizes the considered work scope:

Figure A3 –Terminology in scope of Q.3/16

Thus, the "B2BUA terminology" is based on "interworking terminology" (AVD-4611), which should be discussed first.

2.2	Development of an H.248 "B2BUA" gateway model

Figure B recalls the native peer-to-peer communication model between two IP based UEs using the SIP for application control:

Figure B – SIP peer-to-peer model

It may be observed that there's an implicit coverage of media plane aspects as well due to the SIP-embedded SDP block related to media information. However, it's a "media description" only, but not any definition of media endpoint behaviour as such. The SIP UA (RFC 3261) covers therefore only partially media plane functionality.

There are essentially three types of SIP IWFs of interest in this contribution:

- media-less SIP proxy,

- media-less SIP B2BUA and

- media-full SIP B2BUA.

Indeed, similar archictures were already discussed by ITU-T within the BICC type of networks, which are based on separated call control and media (bearer) planes, see Figure C:

Figure C – BICC model with decomposed (H.248) gateways

There were always integrated and decomposed gateway types in scope of the industry and subject of network deployments. Understanding the specifics of an decomposed (H.248) gateway implies firstly the consideration of the integrated gateway model, see next sub-clauses.

2.2.1	Integrated gateway model

The RFC 7092 "signaling/media plane B2BUA" comprises a media plane B2BUA function besides the pure SIP B2BUA, see Figure D:

Figure D – RFC 7092 B2BUA models for integrated gateways (SBCs), - with and without media plane B2BUA

Figure E illustrates the integrated gateway model within the end-to-end communication path between two SIP-based UEs:

Figure E – IETF RFC 7092 – Integrated gateway model for signaling/media plane B2BUA 
in end-to-end context

The integrated gateway is now decomposed according the H.248 defined functional distribution between H.248 MGC and H.248 MG, see next clause.

2.2.2	Decomposed gateway model (H.248)

The derivation of the decomposed gateway model (Figure F) is straightforward (based on Figure E):

Figure F – IETF RFC 7092 – Decomposed gateway model for signaling/media plane B2BUA 
in end-to-end context

The functional entity "signaling/media plane B2BUA" is constituted by the SIP B2BUA function and the MGC function.

3	Conclusion

We may derive "B2BUA terminology" based on above introduction, model development and relation to B2BUA concepts. The derivation would be part of a new Appendix in H.Sup13. An initial proposal is made by this contribution.

4	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against H.Sup.13 (Release 1; 07/2014).

#1:

[Begin Proposal]

# 1	Scope

In scope of this Supplement are GCP related terms that relate to multiple H.248 documents.

Out of scope of this Supplement are

- terms not related to H.248 technology,

- terms which affect a single H.248.x Recommendation.

## 1.1	Release 2

Release 2 adds further terms, in particular related to following areas:

- Back-to-back user agents (B2BUA) from H.248 perspective;

[End Proposed Correction]

#2:

[Begin Proposal]

# 2	References

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.x]	The ITU-T H.248.x-series of Recommendations (Gateway Control Protocol).

[ITU-T H.248.64]	Recommendation ITU-T H.248.64 (03/2013), Gateway Control Protocol: IP router packages.

[ITU-T Y.1251]		Recommendation ITU-T Y.1251 (08/2002), General architectural model for interworking.

[ITU-T Y.2902]		Recommendation ITU-T Y.2902 (11/2008), Carrier grade open environment components.

[IETF RFC 7092]	IETF RFC 7092 (12/2013), A Taxonomy of Session Initiation Protocol (SIP) Back-to-Back User Agents.

[End Proposed Correction]

#3:

[Begin Proposal]

# 3	Definitions

## 3.1	Terms defined elsewhere

In addition to the basic GCP terms defined by clause 3.2/[ITU-T H.248.1], this Supplement uses the following terms defined elsewhere.

3.1.1	back-to-back IP host (B2BIH) mode (clause 3.2.1/[ITU-T H.248.64]): A context of two or more IP-based terminations, where each termination appears to the rest of the network as an IP host (in contrast to an IP router).

## 3.2	Terms defined in this Supplement

NOTE - The terms in this clause are tabulated in order to maintain the usual alphabetical order when adding additional terms in future releases of this Supplement.

This Supplement defines the following terms (Table 1):

Table 1 – Terms defined in this Supplement

| Term | Definition |
| --- | --- |
| Back-to-back IP host (B2BIH) | provides a media plane B2BUA function with primary scope on IP protocol layer(s) L3 (and possibly L4) only, despite the fact that an IP host as such is terminating the entire protocol stack. NOTE – Example: the B2BIH mode (according [ITU-T H.248.64]) defines a specific media plane B2BUA function with scope on mode of operations as relevant for IP router/gateways. |
| Back-to-back user agent (B2BUA) | to be inserted from Appendix #5# |
| H.248 master-slave control relationship | The H.248 decomposed gateway follows a hierarchical control model in the network control plane (primarily related to call service control and media/bearer control functions). The control hierarchy constitutes a master-slave relationship, with the MGC as master and MG as the slave entity. NOTE 1 - [b-ITU-T Q.Sup31] Annexes C.2 or C.3 provide examples of functional-to-physical mapping models according to the H.248 master-slave control relationship. |
| Media plane B2BUA | to be inserted from Appendix #5# |
| MG autonomous mode | For a particular function, the MGC delegates some (but not all) service specific control decisions down to the MG level. The MG provides a local decision function. The MG autonomous control is limited to decisions with MG available information only. |
| MGC-strictly controlled mode | For a particular function, the control decisions are exclusively under MGC responsibility, the MG degenerates to a pure execution unit. |
| Proxy (derived from [ITU-T Y.2902]) | to be inserted from Appendix #5# |
| stream endpoint tuple (SEPT) | the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share the same StreamID value. The stream topology is given by the topology descriptor settings. |
| SIP B2BUA | to be inserted from Appendix #5# |

{Editor's note (2014-11): some of the new terms added to the table are developed in the new Appendices. Consistency between Table and Appendices should be cross-checked before approval of the new Supplement Release.}

[End Proposed Correction]

#4:

[Begin Proposal]

# 4	Abbreviations and acronyms

This Supplement uses the following abbreviations and acronyms:

| B2BIH | Back-to-back IP host |
| --- | --- |
| B2BUA | Back-to-back user agent |
| GCP | Gateway Control Protocol |
| IP | Internet Protocol |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| SCTP | Stream Control Transmission Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SEPT | Stream Endpoint Tuple |
| SIP | Session Initiation Protocol |
| UDP | User Datagram Protocol |

[End Proposed Correction]

#5:

[Begin Proposal]

Appendix #5#

Back-to-back user agents (B2BUA) from H.248 perspective

## #5#.1	Introduction

The back-to-back user agent (B2BUA) is an IETF concept and originates in the Session Initiation Protocol (SIP). [IETF RFC 7092] provides an extended B2BUA model by supplemental consideration of the media plane. The RFC indicates the option of decomposed gateway models as such, but any particular gateway control protocol itself is out of scope of the taxonomy. This Appendix provides such a mapping model for H.248.

## #5#.2	Models – Decomposed gateway and B2BUA entities

Figure #5#.1 illustrates an H.248 gateway embedded in an IP network with SIP as application control signalling protocol:

Figure #5#.1 – B2BUA entities and decomposed (H.248) gateway model 
– H.248 IP-IP gateway embedded in SIP network

Two back-to-back user agent (B2BUA) entities could be identified, tightly coupled to the SIP-based IP signalling path and the IP bearer path for media traffic:

- SIP B2BUA and

- Media plane B2BUA.

The SIP B2BUA is, strictly speaken, on top (and thus out of scope) of the H.248 MGC function (see Fig. #5#.1).

NOTE 1 – However, in many network (reference) models are both functions coupled ("melted"), due to various "session border controller/gateway) concepts, integrated gateway models, etc.

The media plane B2BUA exists in many H.248.x applications, see some examples in Figure #5#.2:

Figure #5#.2 – Media plane B2BUA functions as defined by ITU-T (examples)

H.248 terminology needs to be derived in order to relate H.248 gateways on existing and future B2BUA models. See e.g., [b-IETF b2bua-rtcp], [b-IETF b2bua-dtls] or [b-IETF b2bua-stun] for ongoing B2BUA discussions.

### #5#.3.2	Terminology

The B2BUA entity represents an hierarchical object, therefore a hierarchical terminology seems to be the simplest approach.

### #5#.3.2.1	First level terminology

Starting point:

| Agent: | An entity that acts on behalf of another entity. [ITU-T X.1252] |
| --- | --- |

Then

| User Agent: | An agent acting behalf of a person (user). NOTE - similar to [ITU-T H.764] |
| --- | --- |

Then we need to focus on protocol functions:

| Protocol User Agent: | An user agent acting as protocol terminator. Thus, a protocol user agent includes an (N)-connection-endpoint function for a (N)-protocol. NOTE 1 – see definitions in [ITU-T X.200]. NOTE 2 – When the "protocol" is of type "client/server", then there would be inherently a role (client or server) assignment to the (protocol)-connection-endpoint. Resulting in the two Protocol User Agent Client and Protocol User Agent Server instances. |
| --- | --- |

Then we need to consider the two network planes:

| Call control User Agent: | A protocol user agent acting on call control signalling level. |
| --- | --- |

and

| Media level User Agent: | A protocol user agent acting on media plane protocol (stacks). |
| --- | --- |

Finally we got the definition for a

| SIP User Agent: | A call control user agent acting on the SIP. |
| --- | --- |

and

| SIP User Agent Client¦Server: | A SIP user agent acting in protocol role client¦server. |
| --- | --- |

NOTE – Above definitions are consistent with the SIP UA semantic according to the "SIP RFC" [IETF RFC 3261], - excerpt from clause 6/[IETF RFC 3261]:

| (RFC 3261) User Agent (UA): | A logical entity that can act as both a user agent client and user agent server. |
| --- | --- |
| (RFC 3261) User Agent Client (UAC): | A user agent client is a logical entity that creates a new request, and then uses the client transaction state machinery to send it. The role of UAC lasts only for the duration of that transaction. In other words, if a piece of software initiates a request, it acts as a UAC for the duration of that transaction. If it receives a request later, it assumes the role of a user agent server for the processing of that transaction. |
| (RFC 3261) User Agent Server (UAS): | A user agent server is a logical entity that generates a response to a SIP request. The response accepts, rejects, or redirects the request. This role lasts only for the duration of that transaction. In other words, if a piece of software responds to a request, it acts as a UAS for the duration of that transaction. If it generates a request later, it assumes the role of a user agent client for the processing of that transaction. |

It may be concluded that the above introduced UA related definitions are simpler than the ones from [IETF RFC 3261] and are sufficient for the terminology as required for H.248 gateways.

#### #5#.3.2.2	Second level terminology

The next step requires a precise positioning of "B2BUA" versus "Proxy":

{Editor's note (2014-11): the proposals of this sub-clause is still work in progress because dependent on the terminology with respect to "interworking" (see AVD-4611) and "endpoints" (see AVD-4598).}

| B2BUA: | A logical concatenation of a pair of coupled Protocol User Agent Client and Protocol User Agent Server instances instances. FIXTHIS: further characteristics? |
| --- | --- |

| (N)-Proxy: | A logical concatenation of a pair of coupled (N)-connection-endpoints within the actual end-to-end (N)-association. The (N)-proxy does not replace the (N)-protocol, but may modify (N)-protocol control information (which represents a (N)-interworking function). The (N)-proxy may maintain state information across the two (N)-connection-endpoints. The two (N)-connection-endpoints are explicitly visible by their network-addresses from routing perspective. The (N)-proxy as network element is an intermediary node, authorized to work on behalf of the two (remote) (N)-end systems, including responding to protocol requests. |
| --- | --- |

#### #5#.3.2.3	Third level terminology

Finally, the term "media-relay B2BUA", "media-aware B2BUA" and "media-termination B2BUA" could be differentiated:

{Editor's note (2014-11): the proposals of this sub-clause is still work in progress because dependent on the terminology with respect to "media translation" (see AVD-4612).}

with base term:

| media-plane B2BUA: | a media-plane level interworking function (IWF) associated to and controlled by a call-control level control plane B2BUA instance. The IWF is usually located between bearer connection endpoints with identical or largely overlapping protocol stacks, but could be also any kind of H.248 network interworking or H.248 service interworking function. |
| --- | --- |

would be:

| media-relay B2BUA: | A media-plane B2BUA … FIXTHIS |
| --- | --- |
| media-aware B2BUA: | A media-plane B2BUA with a media aware IWF. |
| media-terminating B2BUA: | A media-plane B2BUA … FIXTHIS |

[End Proposed Correction]

#6:

[Begin Proposal]

Bibliography

[b-ITU-T Q.Sup31]	Technical Report TRQ.2141.0 (12/2000): Signalling requirements for the support of narrow-band services over broadband transport technologies - Capability set 2 (CS-2).
and
	Technical Report TRQ.2141.1 (11/2002): Signalling requirements for the support of narrowband services via broadband transport technologies - CS-2 signalling flows.

[b-IETF b2bua-rtcp]	IETF draft-ietf-straw-b2bua-rtcp (2014), Guidelines to support RTCP end-to-end in Back-to-Back User Agents (B2BUAs). 
URL: http://tools.ietf.org/wg/straw/draft-ietf-straw-b2bua-rtcp/

[b-IETF b2bua-dtls]	IETF draft-ram-straw-b2bua-dtls-srtp (2014), DTLS-SRTP Handling in Session Initiation Protocol (SIP) Back-to-Back User Agents (B2BUAs).

[b-IETF b2bua-stun]	IETF draft-ietf-straw-b2bua-stun (2014), Session Traversal Utilities for NAT (STUN) Message Handling for Session Initiation Protocol (SIP) Back-to-Back User Agents (B2BUAs).

[End Proposed Correction]

Backup material I: IETF STRAW email discussions

http://www.ietf.org/mail-archive/web/straw/current/msg00183.html

# [straw] comment to B2BUA terminology; RE: Draft on RTCP requirements deliverable

- From: "Schwarz, Albrecht (Albrecht)" <albrecht.schwarz at alcatel-lucent.com>

- To: Lorenzo Miniero <lorenzo at meetecho.com>, "straw at ietf.org" <straw at ietf.org>

- Cc: Sergio Garcia Murillo <sergio.garcia.murillo at gmail.com>, Victor Pascual <victor.pascual at quobis.com>

- Date: Mon, 21 Oct 2013 14:59:20 +0000

- In-reply-to: <20131020094346.5b6a9a02 at lminiero>

- References: <20131020094346.5b6a9a02 at lminiero>

- List-id: "Sip Traversal Required for Applications to Work \(STRAW\) working group discussion list" <straw.ietf.org>

Thanks for that effort Lorenzo!

Network PLANEs (such as user plane, control plane, management plane, signalling plane, data plane, media plane, etc) aim to separate characteristics of networks. Thus, different planes are NOT overlapping (because this would inherently violate a plane concept).

That said:

draft-ietf-straw-b2bua-taxonomy-02,

a description like: "A Media-plane B2BUA is one that operates at both the SIP and media

planes, not only on SIP messages but also SDP and potentially RTP/

RTCP or other media."

violates actually the plane concept!

My suggestion,

either remove the notion of 'plane' in "x-plane B2BUA" terms,

or use the following terminology:

There are two B2BUA types:

a) B2BUA with a "SIP B2BUA" and a "media-plane B2BUA"

b) B2BUA with a "SIP B2BUA" and without a "media-plane B2BUA"

The existing B2BUA terminology is confusing.

Regards,

Albrecht

-----Original Message-----

From: straw-bounces at ietf.org [mailto:straw-bounces at ietf.org] On Behalf Of Lorenzo Miniero

Sent: Sonntag, 20. Oktober 2013 09:44

To: straw at ietf.org

Cc: Sergio Garcia Murillo; Victor Pascual

Subject: [straw] Draft on RTCP requirements deliverable

Dear all,

we just published a draft aimed at addressing the RTCP requirements deliverable in the charter:

http://tools.ietf.org/html/draft-miniero-straw-b2bua-rtcp-00

The motivation for the draft also comes from implementation efforts we devoted in that sense, working on WebRTC gateways in particular. The idea is, considering these efforts, to also address the SRTP and STUN deliverables, but we decided to start writing something down on RTCP first and then maybe work on the others later on.

As most -00 it's still pretty rough, but most of the "juice" should be already there. We tried to follow the organization the taxonomy draft presents about B2BUAs handling media: of all of them, the media termination section is still quite sketchy, but the others already present some more meaningful content.

While working on the draft, Sergio also raised some valid points about whether or not we should also address RTP management in B2BUAs, especially when a B2BUA is terminating media. We decided, at least for now, to just address those aspects of RTP that have an impact on RTCP, since that's what the deliverable is about, but we felt it may be indeed worthwhile to also discuss potential RTP issues somewhere in STRAW, e.g., by describing the different things a B2BUA may/should (not) be doing as a media terminator. Is this something you also see a need for, and if so, would this require an additional deliverable or could it be part of existing ones?

Any feedback will be more than welcome!

Lorenzo

http://www.ietf.org/mail-archive/web/straw/current/msg00195.html

# Re: [straw] b2bua-taxonomy: proposal to change "media-plane" to "signaling/media-plane"

- From: "Schwarz, Albrecht (Albrecht)" <albrecht.schwarz at alcatel-lucent.com>

- To: "Hutton, Andrew" <andrew.hutton at unify.com>, Victor Pascual <victor.pascual.avila at gmail.com>, Christer Holmberg <christer.holmberg at ericsson.com>

- Cc: "hadriel.kaplan at oracle.com" <hadriel.kaplan at oracle.com>, "rai-ads at tools.ietf.org" <rai-ads at tools.ietf.org>, "straw at ietf.org" <straw at ietf.org>

- Date: Tue, 29 Oct 2013 08:11:47 +0000

- In-reply-to: <9F33F40F6F2CD847824537F3C4E37DDF17C2035A at MCHP04MSX.global-ad.net>

- References: <7594FB04B1934943A5C02806D1A2204B1C4F7129 at ESESSMB209.ericsson.se> <2571E680-7047-4DFB-8779-1D6F29703A3B at gmail.com> <9F33F40F6F2CD847824537F3C4E37DDF17C2035A at MCHP04MSX.global-ad.net>

- List-id: "Sip Traversal Required for Applications to Work \(STRAW\) working group discussion list" <straw.ietf.org>

The proposal would be an improvement, hence OK for me. Thanks, Albrecht From: straw-bounces at ietf.org [mailto:straw-bounces at ietf.org] On Behalf Of Hutton, Andrew Sent: Montag, 28. Oktober 2013 14:33 To: Victor Pascual; Christer Holmberg Cc: rai-ads at tools.ietf.org; hadriel.kaplan at oracle.com; straw at ietf.org Subject: Re: [straw] b2bua-taxonomy: proposal to change "media-plane" to "signaling/media-plane" Since the whole taxonomy is written in the context of SIP aware B2BUAâs I also am fine with the existing text but would not object to making it even clearer. Regards Andy From: straw-bounces at ietf.org [mailto:straw-bounces at ietf.org] On Behalf Of Victor Pascual Sent: 28 October 2013 06:24 To: Christer Holmberg Cc: hadriel.kaplan at oracle.com; rai-ads at tools.ietf.org; straw at ietf.org Subject: Re: [straw] b2bua-taxonomy: proposal to change "media-plane" to "signaling/media-plane" (As individual) I'm fine with the current text, but would have no objection if the WG decided to include the suggestion below. Thanks, -Victor On Oct 25, 2013, at 9:06 PM, Christer Holmberg <christer.holmberg at ericsson.com> wrote: (As co-chair) Hi, As you may have seen, Albrecht has raised concern regarding the "media-plane B2BUA" terminology in the b2bua-taxonomy document (it is also used in other STRAW documents). His issue has been that "media-plane" refers to actions taking place on the media-plane, while the b2bua-taxonomy document defines "media-plane B2BUA" as an entity that performs actions both on the signaling plane and the media plane. An easy way to address that would be to simply rename "media-plane B2BUA" to "signaling/media-plane B2BUA", or something similar. It would be a simple task to do (there should only 2-3 instances in the draft), and would not require us to move the b2bua-taxonomy document back to the WG (it is currently ongoing IESG review). Would anyone object to such change? Regards, Christer

Backup material II: 
Gateway mapping models for "signaling/media plane B2BUAs"

IETF RFC 7092 allows two interpretations, dependent whether the "SIP B2BUA" entity or the "signaling/media plane B2BUAs" entity is ther superior object over the other one.

Figure II.1 illustrates the 1st interpretation: a signaling/media plane B2BUA covers a SIP B2BUA plus a media plane B2BUA:

Figure II.1 –Gateway mapping model with SIP B2BUA limited on control plane only

Figure II.2 illustrates a 2nd interpretation: a SIP B2BUA covers a signaling plane B2BUA plus a media plane B2BUA:

Figure II.2 –Gateway mapping model with SIP B2BUA media plane coverage

The 2nd interpretation looks strange due to violation of plane principles. A control/signalling plane protocol (such as SIP) is limited on that particular plane. Any term definition

The 1st interpretation seems to be also consistent with the RFC 7092 description (note: there isn't any definition), see clause 3.2/RFC 7092:

A signaling/media plane B2BUA is one that operates at both the SIP and media planes and not only on SIP messages but also on SDP and potentially the Real-time Transport Protocol (RTP) / the Real-Time Control Protocol (RTCP) [RFC3550] or other media. […]

_________________

There are non-SIP UAs as well, see e.g. ITU-T. A much better definition could be e.g. following derivation,
a) Agent: An entity that acts on behalf of another entity. [ITU-T X.1252] 
b) User Agent: an agent acting behalf of a person (user). (similar to [ITU-T H.764]) 
c) Protocol User Agent: a user agent acting as protocol terminator. Thus, a protocol user agent includes an (N)-connection-endpoint function for protocol N..
d) Call control User Agent: a protocol user agent acting on call control signalling level.
e) Media level User Agent: a protocol user agent acting on media plane protocol(s).
f) SIP User Agent: a call control user agent acting on the SIP.
e) SIP User Agent Client|Server: a SIP user agent acting in protocol role client|server.