| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4600 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.Sup.13 (Rel. 2): Terminology related to TLS and DTLS |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

There might be further H.248 TLS enhancements in future which would need a refined TLS terminology. Correspondent living list material is suggested by this contribution.

Background

We removed following pending editor's notes from Draft H.248.90 before submitting this Draft for consent/approval at the last June/July SG16 Plenary:

{Editor's note #1 (2014-03 meeting): Terminology:
a) we need to revisit again terminology – "resumable TLS session", "resumed TLS session", "resuming TLS session", "presumable TLS session" and "TLS security context", "TLS security parameters" …
b) we need more syntax-oriented definitions for terms "TLS session" and "TLS connection", ("rather loosely described") in RFC 5246. E.g. something like (observation from AVD-4545):

| H.248 bearer concepts: "TLS session" vs "TLS connection" | Status: we are just using loosely the notion of "TLS session", which is sufficient in H.248.TLS when we could equate "TLS-enabled H.248 SEP" represents a "single TLS session". Question: the "TLS session resumption" capability actually demands for a consideration of "TLS connections", too. If not, semantics are fairly difficult … Proposal for consideration: add definitions to H.248.TLS … e.g. something like - TLS session: "a TLS session is constituted by the 6-tuple of {session identifier, peer certificate, compression method, cipher spec, master secret, is resumable}" (based on clause 7/RFC 5246) - TLS connection: "a TLS connection is constituted by the 15/21-tuple of {…}" (based on clause 6.1/RFC 5246) Such better bearer concepts would facilitate a) TLS session resumption semantics and b) (perhaps) improve "TLS profile" definition |
| --- | --- |

Background – RFC 5246, Appendix B "Glossary":

following terms should be revisited again concerning clause 3.1 and 3.2

| client | The application entity that initiates a TLS connection to a server. This may or may not imply that the client initiated the underlying transport connection. The primary operational difference between the server and client is that the server is generally authenticated, while the client is only optionally authenticated. |
| --- | --- |
| connection | A connection is a transport (in the OSI layering model definition) that provides a suitable type of service. For TLS, such connections are peer-to-peer relationships. The connections are transient. Every connection is associated with one session. |
| server | The server is the application entity that responds to requests for connections from clients. See also "client". |
| session | A TLS session is an association between a client and a server. Sessions are created by the handshake protocol. Sessions define a set of cryptographic security parameters that can be shared among multiple connections. Sessions are used to avoid the expensive negotiation of new security parameters for each connection. |
| session identifier | A session identifier is a value generated by a server that identifies a particular session. |

}

{Editor's note #2 (2014-03 meeting): Conditional service:
The capability of "TLS session resumption" is not generally supported by H.248 entities, rather conditional. The conditions for the first example are indicated, but not yet for the second example.}

# Discussion and resolution comments

The editor's notes are still valid, particularily when starting to extend the first release of H.248.90 (07/2014) concerning further TLS capabilities (TLS resumption capability, TLS profile auditing, etc).

Former C-484R1 did already provide an initial proposal of a TLS terminology framework, which, however, couldn't be completed in due time. The work could be completed in the meanwhile, resulting in a complete TLS terminology framework, with term definitions in sufficient detail for usage in the design of user equipment and network elements with TLS support (such as H.248 MGs). Figure A summarizes the resulting hierarchical terminology framework:

Figure A – (D)TLS terminology– Overview of hierarchical framework

The derivation of the hierachical terminology framework is described in the attached change proposal for H.Sup13. Figure B outlines the underlying object model for the TLS protocol:

Figure B – Object (static) model for the TLS protocol

The attached H.Sup13 update proposal is self-explanatory, however, a number of discussion topics are discussed upfront in more detail:

### Item #L.1: TLS protocol layering

| Motivation: | TLS is not a usual monolithic protocol layer, rather organized in two sub-layers: Lower TLS sub-layer: TLS record protocol Upper TLS sub-layer: TLS handshake, Change Cipher Spec and alert protocols |  |
| --- | --- | --- |
| Comment: | The TLS session / connection definition might be tightly associated to the sub-layering model? To be studied. |  |
| Resolution: | The OSI-BRM X.200 terminology could be more or less reused in case of a monolithic protocol layer. E.g., the definition of term "TLS connection" could then just refer to the X.200 term "(N)-connection" (with 'N' equal to 'TLS'). However, this isn't the case with TLS, hence we need to extend existing terms from OSI and IETF reference models. |  |

### Item #L.2: Iventory TLS glossary (Annex B/RFC 5246)

| Motivation: | connection A connection is a transport (in the OSI layering model definition) that provides a suitable type of service. For TLS, such connections are peer-to-peer relationships. The connections are transient. Every connection is associated with one session. session A TLS session is an association between a client and a server. Sessions are created by the handshake protocol. Sessions define a set of cryptographic security parameters that can be shared among multiple connections. Sessions are used to avoid the expensive negotiation of new security parameters for each connection. session identifier A session identifier is a value generated by a server that identifies a particular session. |  |
| --- | --- | --- |
| Comment: | - the overall description is usefull, but the glossary as such is too high level, insufficient for a useful terminology from H.248 perspective - in particular, if there are TLS connections and TLS sessions, then there are also TLS connection endpoints and TLS session endpoints, and what would be the "endpoint identifiers"? - furthermore: relation between "RFC 5246 ConnectionState" ("RFC 5246 SessionState") and "RFC 5246 Connection" ("RFC 5246 Session")? |  |
| Resolution: | The proposed TLS terminology framework (by this contribution) could resolve above questions and should be finally consistent with RFC 5246. |  |

### Item #L.3: Two options for the definition of "(N)-connection"

| Motivation: | There are two principal options: definition of "(N)-connection" based - on "(N)-connection-endpoint-identifiers"? or - on "(N)-connection-endpoint-state"? Rationale: RFC 5246 provides the explicit concepts of a "TLS connection state" and "TLS session state". |  |
| --- | --- | --- |
| Comment: | The used concept in communication networks is (a) rather than (b). But (a) leads to the question of suitable n-tuples as (N)-connection-endpoint-identifiers for a TLS session / connection? |  |
| Issue in RFC 5246: | The RFC should change - ConnectionState to ConnectionEndpointState and - SessionState to SessionEndpointState |  |
| Resolution: | The proposed TLS terminology framework (by this contribution) could resolve above questions and should be finally consistent with RFC 5246. |  |

### Item #L.4: "TLS session / connection" – connectionless or connection-oriented nature?

| Motivation: | Connectionless interpretation: - the pair of TLS endpoints may communicate to each other as soon as the complete information of "TLS endpoint identifiers" is available (at both sides) (note: like in case of UDP); - thus the "TLS session / connection" would be either existing or not (from an overall perspective); Connection-oriented interpretation: - the "TLS session / connection" may be modelled by a superior state machine, based on the two "TLS connection / session endpoint states"; - thus the "TLS session / connection state" would be e.g. idle, established, blocked, …; - and explicit bearer control procedures are required between both TLS endpoints (leading to endpoint state transitioning at each side). |  |
| --- | --- | --- |
| Comment: | First release of H.248.90 takes the connection-oriented interpretation due to the high-level concept of a TLS security session and the TLS handshake / alert / cipherchange protocols as bearer control procedures. The connection-oriented interpretation would be also the starting point when differentiating "TLS connection" and "TLS session", but this assumption still needs be confirmed for both. Status (2014-06): - TLS connection = connection-oriented - TLS session = connectionless |  |
| Issue in RFC 5246: | The RFC does not provide any explicit information on this topic. |  |
| Resolution: | The proposed TLS terminology framework (by this contribution) confirms the assumed mode of communication types connection-oriented and connectionless, which is finally consistent with the nature of resumable and non-resumable TLS session types. |  |

### Item #L.5: Relation between TLS session and TLS connection?

| Motivation: | There are basically three options from dynamic modeling perspective: O1) TLS session endpoint object derived from a (complete) TLS connection endpoint object? O2) TLS connection endpoint object derived from a (complete) TLS session endpoint object? O3) There's initially already a partial TLS session endpoint object and a partial TLS connection endpoint object available (due to default values, management actions or/and control plane signalling)? |  |
| --- | --- | --- |
| Comment: | All three modelling options were evaluated, with tendency to (O3). |  |
| Issue in RFC 5246: | Such kind of information is difficult to extract from the RFC, particularily also due to the fact that there wasn't any SNMP MIB for TLS defined by the IETF. |  |
| Resolution: | The proposed TLS terminology framework (by this contribution) confirms the assumed option 3. This requires the consideration of a dynamic, i.e., time-dependent object model. Such a model was evaluated and is described in new proposed subclause #6#.3. |  |

### Item #L.6: Nature of a TLS session (endpoint)?

| Motivation: | Nature of a TLS session, there are two fundamental types: - volatile (in case of transient (non-cached session data)) - semi-permanent (in case of resumable ...) |  |
| --- | --- | --- |
| Comment: | For instance, the very first TLS full handshake leads to a completion of the TLS session endpoint object information and the TLS connection endpoint object information. The TLS session endpoint object is then (e.g. after a completed TLS handshake procedure) either a) completely discarded (non-resumable session) b) cached, i.e., tagged with a "session id" and stored (resumable session) |  |
| Issue in RFC 5246: | More explicit text on that topic would be beneficial. |  |
| Resolution: | This question needs also the consideration a dynamic object model and could be resolved in context of above item #L.5. |  |

### Item #L.7: TLS connection endpoint object information?

| Motivation: | Proposed model of a TLS connection endpoint object, partioned into: - TLS connection endpoint identifer - TLS connection endpoint state machine - TLS protocol status "connection-level": "parameter-value" assignments [= RFC 5246 ConnectionState] |  |
| --- | --- | --- |
| Comment: | From dynamic modeling perspective would be two instances of object "TLS protocol status "connection-level"": - "current" (the object used for the operation of the active TLS connection) - "pending" (the background object modified by concurrent TLS handshake / change cipher spec / alert protocols) |  |
| Issue in RFC 5246: | Parameter ConnectionEnd (for the TLS role) in data structure SecurityParameters as part of the ConnectionState object is rather a session-level attribute than a connection-level item. See also item. |  |
| Resolution: | The proposed TLS terminology framework (by this contribution) confirms the internal modeling of a TLS connection endpoint object, in order to address the three associated aspects of: - identifier information, - state information and - TLS protocol parameter/value view. |  |

### Item #L.8: TLS session endpoint object information?

| Motivation: | Proposed model of a TLS session endpoint object, partioned into: - TLS session endpoint identifier - TLS role - TLS protocol status "session-level": "parameter-value" assignments [= RFC 5246 SessionState] |  |
| --- | --- | --- |
| Comment: | The TLS role characterization is moved to the session-level because there are not any role changes allowed by subsequent TLS bearer control procedures (like TLS resumption, TLS renegotiation). |  |
| Issue in RFC 5246: |  |  |
| Resolution: | The proposed TLS terminology framework (by this contribution) confirms the association of the TLS role as a characteristic of a TLS session endpoint. |  |

### Item #L.9: TLS endpoint and TLS endpoint identifier?

| Motivation: | The TLS endpoint frames a) TLS session endpoint(s) and b) associated TLS connection endpoint(s). There might be a ratio of a) 1:N between a TLS session endpoint and associated TLS connection endpoints and b) 1:M between a TLS endpoint and associated TLS session endpoints. A "TLS client" needs to contact a "TLS server", which is basically achieved on the servers' FQDN. Hence we'd like to use the FQDN of the IP host / hop as an element of the TLS endpoint name (identifier). NOTE - should be consistent to RFC 6066 |  |
| --- | --- | --- |
| Comment: | Such a modelling basis should satisfy in H.248 TLS endpoint instances tight to a) H.248 Root termination b) H.248 non-Root terminations and SEP level |  |
| Issue in RFC 5246: |  |  |
| Resolution: | The proposed TLS terminology framework (by this contribution) could resolve all above raised topics. The resolution requires the consideration of further concepts (and terms) such as "real system", "TLS protocol implementation", "TLS capable system", etc. |  |

### Item #L.10: Network view – Constitution of a TLS session?

| Motivation: | Reminder: there are two TLS session types from network perspective: 1) volatile or 2) semi-permanent ("cached") Hence, the constitution of a TLS session makes only sense in the 2nd case, - and then we would see following 3-tuple as TLS session identifier: - {SessionID or SessionTicket} (RFC 5246 / RFC 5077) - TLS endpoint identifier (client side) - TLS endpoint identifier (server side) |  |
| --- | --- | --- |
| Comment: | There isn't any need for global uniqueness of the {SessionID or SessionTicket} value assigments when using such a 3-tuple. |  |
| Issue in RFC 5246: |  |  |
| Resolution: | The proposed TLS terminology framework (by this contribution) could resolve above issue and should be finally consistent with RFC 5246 and RFC 5077. |  |

### Item #L.11: What is a "DTLS association"?

| Motivation: | There is an unexplained difference between RFC 5246 (TLS) and RFC 6347 (DTLS): the DTLS protocol specification uses the term (new / existing) (DTLS) "association" besides the terms "session" and "connection", - but why? |  |
| --- | --- | --- |
| Comment: | There are three options: O1) (DTLS) "association" an additional and different object versus (DTLS) "session" and (DTLS) "connection"? O2) (DTLS) "association" synonym to (DTLS) "session"? O3) (DTLS) "association" synonym to (DTLS) "connection"? |  |
| Issue in RFC 6347: | Independent of the assumed case the RFC should provide a clarification. The existing text is confusing and should be aligned with TLS. |  |
| Resolution: | We are tending to option O3. The DTLS protocol specification seems to be correct when replacing "association" by "connection" in RFC 6347. Hence, RFC 6347 should (and could) get rid of term "association", at least in context of the proposed (D)TLS terminology framework. |  |

### Item #L.12: Concepts of "TLS resumption" versus "TLS renegotiation"?

| Motivation: | RFC 5246 (TLS) indicates resumption and renegotiation at various places, hence both are representing two different concepts, however, it is fairly difficult to get a clear picture of both (because definitions are missint). |  |
| --- | --- | --- |
| Comment: | In order to get term definitions, it is necessary to consider time-dependent models because both procedures are not memory-less, rather dependent on the history of previous procedures executed on TLS session endpoint and TLS connection endpoint objects. |  |
| Issue in RFC 6347: | RFC could be definetely improved, particularily on TLS renegotiation. |  |
| Resolution: | Firstly, we need definitions for terms on TLS session endpoint and TLS connection endpoint object (see previous items). Then we need to consider following permutations: Under the condition of an 1) {existing ¦ not existing} TLS connection endpoint object will be a 2) {full ¦ abbreviated} TLS handshake procedure executed under the condition of an 3) {semi-permanent ¦ transient} TLS session endpoint object. Thus, there are eight (theoretical) combinations which are resulting in scenarios with different TLS resumption and TLS renegotiation characteristics, hence we are able to show that both concepts are useful but rather disjoint, - and we are able to provide definition proposals for both terms. |  |

# Conclusions

All open questions are addressed and could be resolved based on the proposed TLS terminology framework by this contribution.

The proposed term definitions are unambiguous and should provide a sufficient level of information in order to use them in the area of protocol engineering (such as H.248 protocol development). There are not any further open issues to our knowledge.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against H.Sup.13 (Release 1; 07/2014).

#1:

[Begin Proposal]

# 1	Scope

In scope of this Supplement are GCP related terms that relate to multiple H.248 documents.

Out of scope of this Supplement are

- terms not related to H.248 technology,

- terms which affect a single H.248.x Recommendation.

## 1.1	Release 2

Release 2 adds further terms, in particular related to following areas:

- the Transport Level Security (TLS) protocol (see [ITU-T H.248.90]) and the Datagram Transport Level Security (DTLS) protocol (see [ITU-T H.248.93]);

{Editor's note (2014-11): whether the TLS|DTLS terms are finally documented in H.Sup13 or H.248.90 is still open. The work is initially anchored in H.Sup13 due to some DTLS-specific terms. Furthermore, an other alternative might be finally a mix of H.Sup13 and H.248.90. A decision will be made either befor approval of H.Sup.13 Release 2 or when H.248.90 will be revised/extended.}

[End Proposed Correction]

#2:

[Begin Proposal]

# 2	References

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.x]	The ITU-T H.248.x-series of Recommendations (Gateway Control Protocol).

[ITU-T H.248.90]	Recommendation ITU-T H.248.90 (07/2014), Gateway Control Protocol: H.248 packages for control of transport security using TLS.

[ITU-T H.248.93]	Recommendation ITU-T H.248.90 (07/2014), Gateway Control Protocol: H.248 packages for control of transport security using DTLS.

[ITU-T X.213]	Recommendation ITU-T X.213 (10/2001), Information technology – Open Systems Interconnection – Network service definition.

[ITU-T X.214]	Recommendation ITU-T X.214 (11/1995), Information technology – Open Systems Interconnection – Transport service definition.

[ITU-T X.650]	Recommendation ITU-T X.650 (10/1996), Information technology – Open Systems Interconnection – Basic Reference Model: Naming and addressing.

[IETF RFC 5077]	IETF RFC 5077 (01/2008), Transport Layer Security (TLS) Session Resumption without Server-Side State.

[IETF RFC 5246]	IETF RFC 5246 (2006), The Transport Layer Security (TLS) Protocol Version 1.2.

[IETF RFC 6066]	IETF RFC 6066 (01/2011), Transport Layer Security (TLS) Extensions: Extension Definitions.

[End Proposed Correction]

#3:

[Begin Proposal]

# 3	Definitions

## 3.1	Terms defined elsewhere

In addition to the basic GCP terms defined by clause 3.2/[ITU-T H.248.1], this Supplement uses the following terms defined elsewhere.

None.

## 3.2	Terms defined in this Supplement

NOTE - The terms in this clause are tabulated in order to maintain the usual alphabetical order when adding additional terms in future releases of this Supplement.

This Supplement defines the following terms (Table 1):

Table 1 – Terms defined in this Supplement

| Term | Definition |
| --- | --- |
| H.248 master-slave control relationship | The H.248 decomposed gateway follows a hierarchical control model in the network control plane (primarily related to call service control and media/bearer control functions). The control hierarchy constitutes a master-slave relationship, with the MGC as master and MG as the slave entity. NOTE 1 - [b-ITU-T Q.Sup31] Annexes C.2 or C.3 provide examples of functional-to-physical mapping models according to the H.248 master-slave control relationship. |
| MG autonomous mode | For a particular function, the MGC delegates some (but not all) service specific control decisions down to the MG level. The MG provides a local decision function. The MG autonomous control is limited to decisions with MG available information only. |
| MGC-strictly controlled mode | For a particular function, the control decisions are exclusively under MGC responsibility, the MG degenerates to a pure execution unit. |
| stream endpoint tuple (SEPT) | the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share the same StreamID value. The stream topology is given by the topology descriptor settings. |

{Editor's note (2014-11): some of the new terms added to the table are developed in the new Appendix. Consistency between Table and Appendix should be cross-checked before approval of the new Supplement Release.}

Preliminary Table 1b – Terms defined for (D)TLS

| Term | Definition |
| --- | --- |
| Data transfer ready TLS connection: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Open system: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Real system: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Resumable TLS client session endpoint state: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Resumable TLS server session endpoint state: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Resumable TLS session: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Resumed TLS session: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Semi-permanent TLS client session endpoint state: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Semi-permanent TLS server session endpoint state: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Semi-permanent TLS session: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Set of Fully Qualified Domain Names: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Set of semi-permanent TLS client session endpoint states: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Set of semi-permanent TLS server session endpoint states: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Sets of IP addresses, L4 ports, transport addresses and transport Protocols: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Time aspects: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS abbreviated handshake: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS capable system: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS client connection endpoint: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS client role: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS client session endpoint state derivation operation and transient and semi-permanent TLS client session endpoint states: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS client session endpoint state: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS connection: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS connection endpoint: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS connection endpoint identifier: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS connection endpoint state machine: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS endpoint: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS full handshake: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS message: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS message sequence: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS protocol implementation: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS protocol status "connection level": | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS server connection endpoint: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS server role: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS server session endpoint state derivation operation and transient and semi-permanent TLS server session endpoint state: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS server session endpoint state: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS session identifier: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS session renegotiation: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| TLS session resumption: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Transient TLS client session endpoint state: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Transient TLS server session endpoint state: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |
| Transient TLS session: | ### FIXTHIS: insert definition from Appendix as soon as stable #### |

[End Proposed Correction]

#4:

[Begin Proposal]

# 4	Abbreviations and acronyms

This Supplement uses the following abbreviations and acronyms:

| A | (Network) Address in general, IP Address |
| --- | --- |
| BRM | Basic Reference Model |
| DTLS | Datagram Transport Layer Security |
| EP | Endpoint |
| FSM | Finite State Machine |
| FQDN | Fully Qualified Domain Name |
| GCP | Gateway Control Protocol |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MIB | Management Information Base |
| L(A) | (Network) Address of Local (connection endpoint) |
| LD(A) | (Network) Address of Local Destination (connection endpoint) |
| LD(A,P) | (Network) Address and (Transport) Port of Local Destination (connection endpoint) |
| LS(A) | (Network) Address of Local Source (connection endpoint) |
| LS(A,P) | (Network) Address and (Transport) Port of Local Source (connection endpoint) |
| OS | Open System |
| OSI | Open System Interconnection |
| P | (Transport / L4) Port in general, (IP Transport Protocol) Port |
| R(A) | (Network) Address of Remote (connection endpoint) |
| RD(A) | (Network) Address of Remote Destination (connection endpoint) |
| RD(A,P) | (Network) Address and (Transport) Port of Remote Destination (connection endpoint) |
| RL | (TLS) Record Layer |
| RS(A) | (Network) Address of Remote Source (connection endpoint) |
| RS(A,P) | (Network) Address and (Transport) Port of Remote Source (connection endpoint) |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SEPT | Stream Endpoint Tuple |
| SNMP | Simple Network Management Protocol |
| TLS | Transport Layer Security |

[End Proposed Correction]

#5:

[Begin Proposal]

Appendix #6#

Terminology for TLS and DTLS

## #6#.1	Introduction

### #6#.1.1	Motivation from IETF RFC side

The definition of the TLS protocol [IETF RFC 5246] is slightly unusual in the area of protocol specifications, looks more like a software description with a high-level data model, perhaps written after an implementation. The RFC does not provide explicit definitions for the main terms, rather providing a glossary in Appendix B/[IETF RFC 5246]. The Glossary itself provides descriptions of the main terms, but not any definitions. At least not definitions at the detailed level as required for protocol specifications, which implies a precise linkage to objects as e.g. used within protocol and service data units and protocol control information elements.

Without any doubt, TLS itself is a pretty successful, mature and well-proven technology. The production of (D)TLS term definition implies hence reverse engineering of (D)TLS RFCs.

### #6#.1.2	Motivation from ITU-T H.248 side

The first release of [ITU-T H.248.90] and [ITU-T H.248.93] provides basic H.248 support for (D)TLS. There was not any need for a precise distinction between a ((D)TLS)-session and a ((D)TLS)-connection, rather the abstracted model of a H.248 TLS bearer session (connection) is sufficient in these Recommendations.

However, the support of following TLS-related H.248 procedures demands for a more precise terminology:

- support of TLS session resumption (see Appendix VII/[ITU-T H.248.90]),

- semantic of H.248 AuditCapabilities procedures on H.248 package elements (primarily related to TLS profile concept);

- semantic of H.248 AuditValue procedures on H.248 package elements (primarily related to TLS profile concept) and

- security considerations for H.248 controlled ((D)TLS)-connections.

### #6#.1.3	Scope

Following proceeding was applied for the derivation of terminology:

- object modelling – static models: the terms are related to protocol(s), hence, the majority of them represents data objects (or protocol data units);

- object modelling – dynamic models: the lifetime of (N)-associations (with N equal to protocol 'TLS' and 'DTLS') is relevant for some terms, which are potential subject of object modification operations,

and finally

- hierarchical terminology.

Thus, the terminology concept is firstly illustrated by some models, before the actual terms as such are defined.

### #6#.1.4	Conventions

### #6#.1.4.1	Term notation

There are two equivalent notations used for terms:

- the OSI-BRM syntax "(N)-<name1>-<name2>-<…>" with N for a particular protocol or protocol layer, e.g., generic (N)-connection-endpoint such as the protocol specific (TLS)-connection-endpoint, and

- the colloquial syntax (without brackets and dashes) as used in most ITU-T Recommendations, e.g. TLS connection endpoint.

Furthermore, the two protocols TLS and DTLS are largely overlapping: common attributes are qualified with prefix '(D)TLS'.

#### #6#.1.4.2	Object type "TLS connection endpoint"

Table #6#.1 provides some synonym adjectives which are used interchangeable in this Appendix:

Table #6#.1 – Synonym adjectives

| IETF RFC TLS wording |  |  | SDP Offer/Answer wording |  |
| --- | --- | --- | --- | --- |
| current | = |  | actual |  |
| pending | = | latent |  |

## #6#.2	Static models

### #6#.2.1	Starting point: model for an IP transport connection protocol (TCP)

Figure #6#.1 illustrates the hierarchical object model (and therefore also hierarchical vocabulary) considering the example of the well defined terminology for TCP:

Figure #6#.1 – Object (and terminology) model for TCP

Observations:

- the [ITU-T X.200] Open System Interconnection (OSI) model provides the base terms (such as (N)-protocol, (N)-connection-endpoint, (N)-connection-endpoint-identifier);

- the TCP mode of communication is of type connection-oriented, hence the existence of an object "TCP connection endpoint state machine";

- differentiation between (TCP)-connection-endpoint-identifier, (TCP)-connection-endpoint-state-machine and (TCP)-protocol-status;

- the TCP)-protocol-status object is required for the visualization of protocol parameters and value assignments (as required for terminology), which are beyond the scope of objects (TCP)-connection-endpoint-identifier and (TCP)-connection-endpoint-state-machine;

- hierarchical setting of open system (OS) versus (TCP)-protocol-implementation versus (TCP)-connection-endpoint due to the underlying cardinality of 1-to-K ratio ("there might be multiple (N)-protocol-implementations per OS etc");

allowing finally the constitution of

- a (TCP)-connection (given by the 5-tuple of {R(A), R(P), L(A), L(P), "TCP"}).

The same proceeding is now applied to protocols TLS and DTLS, but the object model is more complicated due to the additional protocol-specific services of "resumption" and "renegotiation".

### #6#.2.2	Static model for protocol TLS

Figure #6#.1 illustrates the hierarchical, time-independent object model for TLS:

Figure #6#.2 – Object (static) model for the TLS protocol

Additional observations (beyond "TCP"):

- there is an additional level with respect to (N)-associations: the (TLS)-session level besides the (TLS)-connection;

- mode of communication: the (TLS)-connection is also connection-oriented (as TCP), however, the (TLS)-session represents a connectionless concept;

- the connectionless nature of the (TLS)-session relates to a transient character of the underyling protocol objects, which implies the consideration of dynamic aspects, too;

- a comprehensive TLS terminology needs to take into account some TLS extensions such as "TLS session ticket" [IETF RFC 5077] and "TLS server name" [IETF RFC 6066] due to their impact on objects (and terms) (TLS)-endpoint and (TLS)-session-endpoint-identifier.

The dynamic character of objects (TLS)-session and (TLS)-connection is illustrated in the next clause #6#.3, before introducing the terminology derivation (in clause #6#.4).

## #6#.3	Dynamic models

### #6#.3.1	Lifetime of TLS sessions / connections: use case (Sa) "transient TLS session"

The (TLS)-connection is the actual communication association used for the transfer of TLS traffic. Figure #6#.3 illustrates the dynamic model by consideration of plane-specific traffic activities (which affect TLS objects) for the case of transient (TLS)-sessions.

Conventions used in Figure #6#.3 to Figure #6#.7:

- scope on open system "H.248 MG" (see Figure #6#.2), which terminates the TLS protocol (as subject of an TLS-enabled H.248 stream endpoint);

- traffic activity is indicated at following interfaces:

I1) management plane, i.e., the OAM interface of the H.248 MG (with its element management system);

I2a) control plane "key exchange server signalling", an optional interface in case that the H.248 MG is directly connected to such a centralized server (per TLS domain);

I2b) control plane "call control signalling", i.e., the call control signalling activity at H.248 MGC level;

NOTE – The H.248 interface (I2c), - control plane "gateway control signalling" -, is omitted because not relevant from (D)TLS terminology development point of view. H.248 signalling activity is expected to happen within the time window of call control signalling (I2b) activity.

I3) data plane, i.e., the TLS message exchange with the remote TLS endpoint;

- the objects (TLS)-session-endpoint and (TLS)-connection-endpoint comprise a set of TLS protocol parameters and value assignments, which are abstracted by the metric "information completeness" (and an abstracted value range from zero to 100 %) in the drawings;

- the TLS protocol differentiates between two (TLS)-connection-endpoint objects from implementation perspective, which representing the used one (called "current" or "actual") for TLS communication and the pending one ("latent") modified by ongoing TLS control procedures.

Overview of selected and illustrated use cases in this clause (Table #6#.2):

Table #6#.2 – Dynamic models – Illustrated use cases and demonstrated TLS capabilities

| Use case |  | TLS resumption | TLS renegotiation |
| --- | --- | --- | --- |
| Sa | transient TLS session | No | No |  |
| Sb | semi-permanent TLS session | Yes | No |  |
| Sc | renegotiation using a transient TLS session | No | Yes |  |  |  |
| Sd | renegotiation using a semi-permanent TLS session | Yes | Yes |  |  |  |
| Se | lifetime of a semi-permanent session ends | Yes | Yes |  |  |  |

The variety of use cases is due to the fact of the two different concepts of "TLS resumption" and "TLS renegotiation" (see clause #6#.4 "Terminology").

Figure #6#.3 – Lifetime of TLS sessions / connections – Case (Sa) "transient TLS session"

The various steps of Figure #6#.3 are described in Table #6#.3. The first two steps (1, 2) are not related to call-dependent procedures.

Table #6#.3 – Lifetime of TLS sessions / connections – Case (Sa) "transient TLS session"

| Step |  | Time |  | Traffic activity |  | Status TLS session / TLS connection |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Initialization of network element H.248. MG (or open system in general): |  |  |  |
| 1 |  | t1 - t2 |  | A) management activities, such as configuration management related to a SNMP "TLS MIB" (e.g., setting of a default TLS protocol version). NOTE – Protocol parameter value assignments might be alternatively subject of default value definitions or/and the registration of an H.248 profile (with support of the H.248 tlscn package) by the MG with the MGC. |  | There is sufficient protocol information available (after t2) in order to start an initial TLS handshake with a remote TLS endpoint, of course under the condition that the TLS client knows the network address of the TLS server. |  |
| 2 | t3 – t4 | B) the H.248 MG might be optionally connected to a centralized key exchange server (dependent on the security architecture of the TLS domain). There might be key exchange server signalling activities related e.g. to the update of TLS cipher suites. | The minimum protocol information of the previous step is complemented (or modified) by additional, security related information. |
| Call-dependent TLS activities: |  |  |  |
| 3 | t5 – t8 | C) end-to-end call control signalling inclusive the declaration and negotiation of TLS capabilities | Not yet any change because pure control plane activity so far. H.248 gateway control signalling is triggered immediately after t5 (by call control), and before t6. |
| 4 | t6 – t7 | D) TLS full handshake procedure, either initiated by the local or remote TLS endpoint. E.g., an outgoing TLS full handshake (acting as TLS connection establishment request) would be triggered by the MG when receiving an H.248 command.request from the MGC. | Missing information is exchanged or existing information is updated during the TLS capability negotiation phase with the remote TLS endpoint. There is 100% completeness after the successful TLS full handshake procedure (t7). |
| TLS protocol internal activities: |  |  |  |
| 5 | t7 | - | The TLS full handshake was executed on the initial TLS connection endpoint object Oc1, whereas the handshake procedure was updating the latent TLS connection endpoint object Oc2. The TLS ChangeCipherSpec message triggers the overwriting of the actual by the latent TLS connection endpoint object (Oc1, Oc2) information (at t7). |
| 6 | t7 - t7' | - | The TLS session endpoint object Os1 is deleted. There isn't any TLS session existing anymore after t7'! |
| 7 | t8 - … |  | TLS-based data transfer (during active call phase). |  | The TLS connection endpoint object information (Oc1) remains unmodified (because there aren't any TLS control procedures related to update of information (NOTE 1). |  |
| NOTE 1 – The TLS protocol specification is silent how implementations should handle the pending TLS connection endpoint object Oc2. |  |  |  |  |  |  |

It may be noted that the TLS terminology framework needs to consider already information modeling before the first contact (by the first TLS message sent for a full handshake) with a remote TLS endpoint.

### #6#.3.2	Lifetime of TLS sessions / connections: use case (Sb) "semi-permanent TLS session"

Figure #6#.4 shows a case where different TLS connection endpoint objects are related to the same semi-permanent TLS session endpoint object.

Figure #6#.4 – Lifetime of TLS sessions / connections – Case (Sb) "semi-permanent TLS session"

The steps 1-4 from Figure #6#.4 are the same than for the previous use case and are described in Table #6#.3. The additional steps in Figure #6#.4 are described in Table #6#.4.

Table #6#.4 – Lifetime of TLS sessions / connections – Case (Sb) "semi-permanent TLS session"

| Step | Time | Traffic activity | Status TLS session / TLS connection |
| --- | --- | --- | --- |
| Steps 1 to 4: see Table #6#.3. |  |  |  |
| TLS protocol internal activities: |  |  |  |
| 5 | t7 | - | The TLS full handshake was executed on the initial TLS connection endpoint objects Oc1, whereas the handshake procedure was updating the pending TLS connection endpoint object Oc2. The TLS ChangeCipherSpec message triggers the overwriting of the actual by the latent TLS connection endpoint object information (at t7). The semi-permanent TLS session endpoint object Os1 is still maintained in memory, leaving the option of future "TLS session resumption" procedures (called "cached" in the RFC). |
| 6 | t8-... | TLS-based data transfer (during active call phase) using TLS connection endpoint object Oc1. | The TLS connection endpoint object information (Oc1) remains unmodified, because there isn’t any TLS control procedure related to update of information. |
| 7 | t9-t12 | E) end-to-end call control signalling for a different call, inclusive the declaration and negotiation of TLS capabilities | Not yet any change because pure control plane activity so far. H.248 gateway control signalling is triggered immediately after t9 (by call control), and before t10. |
| 8 | t10-t11 | F) TLS abbreviated handshake procedure, either initiated by the local or remote TLS endpoint. E.g., an outgoing TLS abbreviated handshake (acting as TLS connection establishment request) would be triggered by the MG when receiving an H.248 command.request from the MGC. | The TLS abbreviated handshake was executed on the TLS connection endpoint object Oc3 using the existing TLS session endpoint object Os1 (TLS session resumption), whereas the handshake procedure was updating the pending TLS connection endpoint object Oc4. Missing information is exchanged or existing information is updated during the TLS capability negotiation phase with the remote TLS endpoint. The TLS session endpoint object Os1 is not updated during or after the successful TLS abbreviated handshake procedure. |
| 9 | t11 | - | TLS ChangeCipherSpec message triggers the overwriting of the actual by the latent TLS connection endpoint object information (Oc3, Oc4) at t11. The TLS session endpoint object Os1 remains unchanged in memory (NOTE 1). |
| 10 | t12 - ... | TLS-based data transfer (during active call phase). | The TLS connection endpoint object information (Oc3) remains unmodified (because there aren't any TLS control procedures related to update of information. |
| NOTE 1: Leaving the option of future "TLS session resumption" procedures. |  |  |  |

### #6#.3.3	Lifetime of TLS sessions / connections: use case (Sc) "renegotiation using a transient TLS session"

Figure #6#.5 shows a case where a full handshake is used during the TLS renegotiation procedures (e.g. because TLS session resumption is not supported by both TLS endpoints).

Figure #6#.5– Lifetime of TLS sessions / connections: use case (Sc) "renegotiation using a transient TLS session"

The steps 1-7 from Figure #6#.5 are the same than for the use case Sa "transient TLS session" described in Table #6#.3. The additional steps in Figure #6#.5 are described in Table #6#.5.

Table #6#.5 – Lifetime of TLS sessions / connections – Case (Sc) "renegotiation using a transient TLS session"

| Step | Time | Traffic activity | Status TLS session / TLS connection |
| --- | --- | --- | --- |
| Steps 1 to 8: see Table #6#.3. |  |  |  |
| TLS protocol internal activities: |  |  |  |
| 9 | t9-t10 | E) A TLS renegotiation using a TLS full handshake procedure is performed, either initiated by the local or remote TLS endpoint. E.g., an outgoing TLS full handshake would be triggered by the MG when the condition for a renegotiation is met. | The TLS full handshake was executed on the actual TLS connection endpoint object Oc1, whereas the handshake procedure was updating the latent TLS connection endpoint object Oc2. # Missing information is exchanged or existing information is updated during the TLS capability negotiation phase with the remote TLS endpoint. There is 100% completeness of the TLS session endpoint object Os2 after the successful TLS full handshake procedure (t10). |
| 10 | t10 | - | # TLS ChangeCipherSpec message triggers the overwriting of the actual by the latent TLS connection endpoint object information (Oc1, Oc2) at t10. |
| 11 | 10-t10’ | - | The TLS session endpoint object is deleted. There isn't any TLS session Oc2 existing anymore after t10'! |
| 12 | t10-... | TLS-based data transfer (during active call phase). | The TLS connection endpoint object information (Oc1) remains unmodified (because there aren't any TLS control procedures related to update of information. |

### #6#.3.4	Lifetime of TLS sessions / connections: use case (Sd) "renegotiation using a semi-permanent TLS session"

Figure #6#.6 highlights the differences to the previous use case of resumption using a transient TLS session (Sc):

Figure #6#.6– Lifetime of TLS sessions / connections: use case (Sd) "renegotiation using a semi-permanent TLS session"

The steps 1-6 from Figure #6#.6 are the same than for use case Sb "semi-permanent TLS session" and are described in Table #6#.4. The additional steps in Figure #6#.6 are described in Table #6#.6.

Table #6#.6 – Lifetime of TLS sessions / connections – Case (Sd) "renegotiation using a semi-permanent TLS session"

| Step | Time | Traffic activity | Status TLS session / TLS connection |
| --- | --- | --- | --- |
| # Steps 1 to 6: see Table #6#.4. |  |  |  |
| TLS protocol internal activities: |  |  |  |
| 7 | t9-t10 | E) A TLS renegotiation using a TLS abbreviated handshake procedure is performed, either initiated by the local or remote TLS endpoint. E.g., an outgoing TLS full handshake would be triggered by the MG when the condition for a renegotiation is met. | The TLS abbreviated handshake using the TLS session endpoint object Os1 (TLS session resumption) was executed on the actual TLS connection endpoint object Oc1, whereas the handshake procedure was updating the latent TLS connection endpoint object Oc2. Missing information is exchanged or existing information is updated during the TLS capability negotiation phase with the remote TLS endpoint. The TLS session endpoint object Os1 remains unmodified. |
| 8 | t10-... | TLS-based data transfer (during active call phase) using TLS connection endpoint object Oc1. | The TLS connection endpoint object information (Oc1) remains unmodified, because there isn’t any TLS control procedure related to update of information. |

### #6#.3.5	Lifetime of TLS sessions / connections: use case (Se) "lifetime of a semi-permanent session ends"

Figure #6#.7 shows a case where the lifetime of a semi-permanent session ends and thus is removed from the cache.

The steps 1-8 from Figure #6#.7 are the same as for the use case Sa "renegotiation using a semi-permanent TLS session" described in Table #6#.6. The additional steps in Figure #6#.7 are described in Table #6#.7.

Figure #6#.7– Lifetime of TLS sessions / connections: use case (Se) "lifetime of a semi-permanent session ends"

Table #6#.7 – Lifetime of TLS sessions / connections – Case (Se) "lifetime of a semi-permanent session ends"

| Step | Time | Traffic activity | Status TLS session / TLS connection |
| --- | --- | --- | --- |
| Steps 1 to 8: see Table #6#.6. |  |  |  |
| 9 | t11-t11' | - | Either on the local or on the remote TLS endpoint a condition occurs which leads to the end of the lifetime of the TLS session endpoint object Os1. It is removed from the cache and deleted. |
| 10 | t12-t13 | E) A TLS renegotiation using a TLS full handshake procedure is performed, either initiated by the local or remote TLS endpoint. E.g., an outgoing TLS full handshake would be triggered by the MG when the condition for a renegotiation is met. | The TLS full handshake was executed on the actual TLS connection endpoint object Oc1, whereas the handshake procedure was updating the latent TLS connection endpoint object Oc2. Missing information is exchanged or existing information is updated during the TLS capability negotiation phase with the remote TLS endpoint. There is 100% completeness of the TLS session endpoint object Os2 after the successful TLS full handshake procedure (t13). |
| 11 | t13-... | TLS-based data transfer (during active call phase) using TLS connection endpoint object Oc1. | The TLS connection endpoint object information (Oc1) remains unmodified, because there isn’t any TLS control procedure related to update of information. |

## #6#.4	Terminology

### #6#.4.1	Overview – hierarchical framework

Figure #6#.5 summarizes the main relationships between the terms as defined by subsequent clause #6#.4.2:

Figure #6#.5 – (D)TLS terminology– Overview of hierarchical framework

It should be emphasized that Figure #6#.5 indicates only the major dependencies between related terms.

### #6#.4.2	Composition of term definitions

Figure #6#.2 indicated the possibility of multiple TLS protocol implementations per Open System. This is a valid assumption for H.248 IP media gateways, particularily when supporting multihomed IP host interfaces and IP connectivity with multiple TLS domains. Hence, the definition of a real system would be the starting point,

| Real system (clause 4.1.1/[ITU-T X.200]): | A set of one or more computers, the associated software, peripherals, terminals, human operators, physical processes, information transfer means, etc., that forms an autonomous whole capable of performing information processing and/or information transfer. |
| --- | --- |

which allows to define term

| TLS protocol implementation: | A (software or hardware) based implementation of the TLS protocol as specified in [IETF RFC 5246] and is always associated to a real system. |
| --- | --- |

Thus, the realization of an H.248 MG represents a real system and the support of TLS-enabled H.248 terminations/stream endpoints implies at least one TLS protocol implementation on the MG, leading to the definition of a

| TLS capable system: | A real system which provides one or multiple TLS protocol implementations. |
| --- | --- |

Next, the top-level (protocol) object within a TLS capable system is a

| TLS endpoint: | An instance of a TLS protocol implementation with exactly one local IP (transport) address. A TLS endpoint is housing one or multiple TLS session endpoints, which are acting either as TLS client or as TLS server (i.e., "TLS client session endpoint" and "TLS server session endpoint") when executing the TLS handshake protocol. |
| --- | --- |
|  | TLS endpoint identifier: Discussion: The FQDN behind the IP (transport) address could be used as part of an TLS endpoint name, i.e., as part of a TLS endpoint identifier (which is consistent with the "TLS server endpoint name" according to [IETF RFC 6066]). However, it needs to be considered that there might be multiple TLS endpoints per real system (e.g., due to multiple TLS protocol implementations in parallel) which share the IP interfaces of the real system, hence: Definition: Each TLS endpoint being identified by a finite set of IP transport addresses, where two TLS endpoints are distinct if and only if their associated sets of IP transport addresses are disjoint ("a certain transport address is associated to at most one TLS endpoint" and "each TLS endpoint is associated to at least one transport address"). |

{Editor's note (2014-11): this definition provides a more detailed characteristic of a TLS endpoint than the initial high-level term definition in clause 3.2.5/[ITU-T H.248.90]: "TLS endpoint: Either a TLS-client or a TLS-server". Thus, the initial definition would be obseleted (or extended) by above definition.}

Next, the concept of a (N)-connection and (N)-connection-endpoint of the OSI-BRM, - see e.g.

| (N)-connection-endpoint (clause 5.3.1.3/[ITU-T X.200]): | A terminator at one end of an (N)-connection within an (N)-service-access-point. |
| --- | --- |

-, needs to be refined due to the internal multilayering of the TLS protocol. A TLS connection endpoint could be unambigously defined by following term:

| TLS connection endpoint: | A part of an instance of a TLS protocol implementation, which is able to send and receive TLS messages, and which is associated to exactly one 8-tuple being composed of - a creation point in time tc, - a destruction point in time td, - a (non-empty) local IP address AL, - a (non-empty) local L4 port PL, - an empty or non-empty remote IP address AR, - an empty or non-empty remote L4 port PR, - a (non-empty) L4 transport protocol T, and - the protocol string "TLS". NOTE 1 – If FQDNs are used as TLS endpoint identifiers, then an additional requirement on the TLS connection endpoint could be that AL is one of the IP addresses associated to the TLS endpoint’s FQDN. |
| --- | --- |

Following three objects are also part of a TLS connection endpoint object:

| TLS protocol status "connection level": | The 8-tuple parameter-value assignment at a particular point in time of the TLS connection endpoint. |
| --- | --- |

Correspondent to the (N)-connection-endpoint-identifier of the OSI-BRM (see [ITU-T X.650] clause 5.4.1.5/[ITU-T X.200]) would be a

| TLS connection endpoint identifier: | The local IP transport address and indication of "TLS/L4" protocol stack, i.e., the 4-tuple of {AL, PL, T, "TLS"} from the TLS connection endpoint. NOTE 1 – Parameter "L4" is required because TLS is a L4 independent protocol, hence a TLS capable system could offer multiple TLS endpoints with different L4 protocols. |
| --- | --- |

And due to the connection-oriented mode of communication at TLS connection level:

| TLS connection endpoint state machine: | The state model associated to the connection-oriented TLS connection concept. NOTE 1 – [IETF RFC 5246] itself does not provide an explicit description of a state model. NOTE 2 – Annex A.3/[ITU-T H.248.90] defines a simplified state model, sufficient for H.248 control purposes. |
| --- | --- |

NOTE to [IETF RFC 5246], clause 6.1 "TLS connection state" concept, which represents the "the operating environment of the TLS Record Protocol": this RFC term is not defined in this Appendix, but if some relation would be required, then there would be following proposal with respect to an connection endpoint object:
TLS connection endpoint state: the actual information represented by an TLS connection endpoint object, i.e., the information contained in objects TLS protocol status "connection level", TLS connection endpoint identifier and TLS connection endpoint state machine.

Thus, based on TLS connection endpoint objects could be the TLS connection derived:

| TLS connection: | A cooperative relationship among a pair of of TLS capable systems, represented by a TLS client connection endpoint and a TLS server connection endpoint (NOTE 1). A TLS connection allows the execution of an establishment procedure given by either a TLS full handshake or a TLS abbreviated handshake (on request of the TLS served user instance at TLS client side). Thus, the mode of communication of TLS connection could be considered as connection-oriented (i.e., a TLS connection is statefull, e.g., at the top-level by the 2-state model {Idle, Established}). NOTE 1 – Thus, formally the TLS connection is a set of two 8-tuples. NOTE 2 – The definition is based on the concepts of an (N)-association and (N)-connection according to [ITU-T X.200]. The definition is consistent and not redefining the explanatory description of a "connection" in the TLS glossary, as contained in Appendix B/[IETF RFC 5246]. NOTE 3 – An interim network element (such as an IP router) could identify the traffic flow of a particular TLS connection on basis of the 6-tuple of {IP@ (client side), IP@ (server side), L4 port (client side), L4 port (server side), protocol 'L4', protocol 'TLS'}. Thus, a TLS connection is constituted by such a 6-tuple. |
| --- | --- |

As next step, TLS protocol procedures needs to be considered in order to describe the information exchange and update of local and remote TLS connection and session endpoint objects. Starting with existing term

| TLS message (clause 3.2.5/[ITU-T H.248.90]): | A unit of TLS-RL user data (clause 3.2.12/[ITU-T H.248.90]) as produced by the TLS handshake protocol, the TLS alert protocol or the TLS change cipher spec protocol. |
| --- | --- |

The TLS protocol is asymmetrical and of type client/server, thus, the two communicating TLS connection endpoints are not equal, there are rather two role specific endpoint types:

| TLS client connection endpoint: | A TLS connection endpoint which sends a TLS message containing a ClientHello handshake structure (NOTE 1) to another TLS connection endpoint. NOTE 1 – The TLS ClientHello handshake structure is defined in [IETF RFC 5246]. |
| --- | --- |
| TLS server connection endpoint: | A TLS connection endpoint which (receives a TLS message containing a ClientHello handshake structure and subsequently) sends a TLS message containing a ServerHello handshake structure (NOTE 1) back to that other TLS connection endpoint. NOTE 1 – The TLS ServerHello handshake structure is defined in [IETF RFC 5246]. |

The endpoint associated role behaviour could be defined as follows:

| TLS client role: | A TLS connection endpoint is said to assume the TLS client role, if it is a TLS client connection endpoint. |
| --- | --- |

{Editor's note (2014-11): this definition is consistent with the TLS client definition according to clause 3.2.2/[ITU-T H.248.90]:

3.2.2	TLS client [IETF RFC 5246]: the TLS entity that initiates a TLS security session to a TLS server [IETF RFC 5246].

NOTE – The primary operational difference between the TLS server and TLS client is that the TLS server is generally authenticated, while the TLS client is only optionally authenticated.

}

and

| TLS server role: | A TLS connection endpoint is said to assume the TLS server role, if it is a TLS server connection endpoint. |
| --- | --- |

{Editor's note (2014-11): this definition is consistent with the TLS server definition according to clause 3.2.11/[ITU-T H.248.90]:

3.2.11	TLS server [IETF RFC 5246]: the TLS entity that responds to requests for security sessions from TLS clients [IETF RFC 5246].

}

The next step would be the consideration of the TLS protocol procedures for establishment and renewing of the communication associations. Starting with term

| TLS message sequence: | A finite sequence of TLS messages. |
| --- | --- |

there would be following two handshake procedure types:

| TLS full handshake: | A TLS message sequence where the first TLS message contains ClientHello structure and if the sequence constitutes a successful TLS full handshake message flow as specified in clause 7.3, Figure 1 of [IETF RFC 5246]. |
| --- | --- |
| TLS abbreviated handshake: | A TLS message sequence where the first TLS message contains ClientHello structure and if the sequence constitutes a successful TLS abbreviated handshake message flow as specified in clause 7.3, Figure 2 of [IETF RFC 5246]. |

The execution of such a procedure leads to an established TLS connection, i.e., ready for data transfer:

| Data transfer ready TLS connection: | A TLS connection after the successful execution of a TLS full handshake or a TLS abbreviated handshake procedures. NOTE 1 – The notion of "Data transfer ready" refers to a specific state of the TLS connection and is synonym to the notion of "Established". The general term "Data transfer ready" matches both connectionless and connection-oriented type of connections, see [ITU-T X.213 and X.214]. |
| --- | --- |

Next step would be the identification of objects related to the TLS session level, starting with the session endpoint state information model:

| TLS client session endpoint state: | The 17-tuple of following parameter-value pairs, as partially described in [IETF RFC 5246]: TLS protocol parameter: Type: - version: ProtocolVersion - prf_algorithm: PRFAlgorithm - bulk_cipher_algorithm: BulkCipherAlgorithm - cipher_type: CipherType - enc_key_length: uint8 - block_length: unit8 - fixed_iv_length: unit8 - record_iv_length: unit8 - mac_algorithm: MACAlgorithm - mac_length: unit8 - mac_key_length: unit8 - compression_algorithm: CompressionMethod - master_secret: opaque[48] - session_id: SessionID (NOTE 1) Other parameters: Type: - creation time tc: time (NOTE 2) - destruction time td: time - server_address: IPaddress (NOTE 3) NOTE 1 – A TLS client session endpoint state is semi-permanent if and only if its session_id value is non-empty. Hence it is transient if and only if its session_id value is empty. NOTE 2 – While a semi-permanent TLS client session endpoint state’s creation point in time correlates with the corresponding TLS full handshake’s end time (which is thus a point in time at which both TLS connection endpoints, which exchange the TLS full handshake messages, do still exist), this semi-permanent TLS client session endpoint state’s destruction point in time is independent of the destruction points in time of these two TLS connection endpoints. Especially, the semi-permanent TLS client session endpoint state may still exist after one or even both TLS connection endpoints are already destroyed. NOTE 3 – A TLS protocol implementation may add further information to a TLS client session endpoint state, like e.g. the associated TLS server endpoint’s source IP address. This additional information may be used by a TLS client connection endpoint in order to decide if an already stored semi-permanent TLS client session endpoint state may be used (may be "resumed") for the establishment of a new TLS connection towards a destination transport address of another TLS endpoint. |
| --- | --- |
| TLS server session endpoint state: | The 16-tuple of following parameter-value pairs, as partially described in [IETF RFC 5246]: TLS protocol parameter: Type: - version: ProtocolVersion - prf_algorithm: PRFAlgorithm - bulk_cipher_algorithm: BulkCipherAlgorithm - cipher_type: CipherType - enc_key_length: uint8 - block_length: unit8 - fixed_iv_length: unit8 - record_iv_length: unit8 - mac_algorithm: MACAlgorithm - mac_length: unit8 - mac_key_length: unit8 - compression_algorithm: CompressionMethod - master_secret: opaque[48] - session_id: SessionID (NOTE 1) Other parameters (NOTE 3): Type: - creation time tc: time (NOTE 2) - destruction time td: time NOTE 1 – A TLS server session endpoint state is semi-permanent if and only if its session_id value is non-empty. Hence it is transient if and only if its session_id value is empty. NOTE 2 – While a semi-permanent TLS server session endpoint state’s creation point in time correlates with the corresponding TLS full handshake’s end time (which is thus a point in time at which both TLS connection endpoints, which exchange the TLS full handshake messages, do still exist), this semi-permanent TLS server session endpoint state’s destruction point in time is independent of the destruction points in time of these two TLS connection endpoints. Especially, the semi-permanent TLS server session endpoint state may still exist after one or even both TLS connection endpoints are already destroyed. NOTE 3 – Difference between the TLS server session endpoint state versus the TLS server session endpoint: the 17th parameter "server address" is of course missing at server side. |

Above state information is generated by specific TLS operations:

| TLS client session endpoint state derivation operation: | The algorithm implemented in the TLS client session endpoint which takes as input a TLS full handshake and which calculates the n-tuple of parameter-value pairs as defined for the TLS client session endpoint state. |
| --- | --- |
| TLS server session endpoint state derivation operation: | The algorithm implemented in the TLS server session endpoint which takes as input a TLS full handshake and which calculates the n-tuple of parameter-value pairs as defined for the TLS server session endpoint state. |

The TLS server endpoint determines the nature of the TLS session (semi-permanent or transient) by a correspondent setting of parameter SessionID in the ServerHello structure, and the TLS client endpoint decides if it actually tries to resume a server side semi-permanent TLS server session state:

| Semi-permanent TLS client session endpoint state: | A TLS client session endpoint state if its session_id value is non-empty. |
| --- | --- |
| Semi-permanent TLS server session endpoint state: | A TLS server session endpoint state if its session_id value is non-empty. |
| Transient TLS client session endpoint state: | A TLS client session endpoint state if its session_id value is empty. |
| Transient TLS server session endpoint state: | A TLS server session endpoint state if its session_id value is empty. |

Following terms are required in order to address cardinality due to the possibility of multiple TLS sessions per TLS endpoint:

| Set of semi-permanent TLS client session endpoint states: | Multiple semi-permanent TLS client session endpoint states with different state information (NOTE 1). NOTE 1 – Inequality is given when at least one n-tuple parameter (of the TLS client session endpoint state) has a different value assignment. |
| --- | --- |
| Set of semi-permanent TLS server session endpoint states: | Multiple semi-permanent TLS server session endpoint states with different state information (NOTE 1). NOTE 1 – Inequality is given when at least one n-tuple parameter (of the TLS server session endpoint state) has a different value assignment. |

which allows finally the identification of following two TLS session types:

| Semi-permanent TLS session: | The pair of a semi-permanent TLS client session endpoint state and a semi-permanent TLS server session endpoint state, coupled by the TLS full handshake procedure which was executed across the associated TLS connection. |
| --- | --- |
| Transient TLS session: | The pair of a transient TLS client session endpoint state and a transient TLS server session endpoint state, coupled by the TLS full handshake procedure which was executed across the associated TLS connection. The transient TLS client session endpoint state information and transient TLS server session endpoint state information is immediately deleted again after the successful TLS full handshake procedure. |

which leads to the overal term of a

| TLS session: | A semi-permanent TLS session or transient TLS session. |
| --- | --- |
|  | Hierachical terminology expanded plus complementary characteristics: Thus, a TLS session is a transformed TLS connection after the successful execution of a TLS (full or abbreviated) handshake procedure, constituted by the pair of TLS client session state and a TLS server session state. A TLS session is consequently an association between the two TLS session endpoints. The mode of communication of TLS session could be considered as connectionless (i.e., a TLS session is either existing or not). ## The nature of a TLS session (from TLS endpoint perspective) is either volatile (i.e., the local TLS session information would be immediately discarded after a TLS handshake procedure) or semi-permanent (in case of resumable TLS sessions). ## Notably, a TLS session may still exist after one or even both TLS connection endpoints, which did exchange the TLS full handshake messages from which the related TLS session states were derived from, are already destroyed. ## The TLS role (client or server) is a TLS session level characteristic. NOTE 1 – The definition is consistent and not redefining the explanatory description of a "session" in the TLS glossary, as contained in Appendix B/[IETF RFC 5246]. NOTE 2 – An interim network element (such as an IP router) could identify the traffic flow of a particular TLS session on basis of the inspection of TLS server session endpoint state and TLS client session endpoint state information in the TLS message flow. |

| TLS session identifier: | A TLS protocol parameter representing the identification allocated to a particular semi-permanent TLS session. NOTE 1 – The definition is consistent and not redefining the explanatory description of a "session identifier" in the TLS glossary, as contained in Appendix B/[IETF RFC 5246]. ## NOTE 2 – This definition may need to be extended by additional consideration of the session ticket based TLS session resumption mechanism as defined in [IETF RFC 5077]. There would be then two TLS protocol parameters (session_id and SessionTicket) as possible identifiers (both protocol parameters are mutually exclusive). |
| --- | --- |

Following terms could be consequently precised based on above terminology baseline:

| Resumable TLS client session endpoint state: | ## Synonym to a semi-permanent TLS client session endpoint state. |
| --- | --- |
| ## Resumable TLS server session endpoint state: | ## Synonym to a semi-permanent TLS server session endpoint state. |

| Resumable TLS session: | ## Synonym to a semi-permanent TLS session. |
| --- | --- |
|  | Expanded term: ## A TLS session where the TLS server session state as well as the TLS client session state are both resumable. A TLS session state is called resumable, if its session_id value is non-empty. |
| ## Resumed TLS session: | A resumable TLS session after the successful execution of a TLS abbreviated handshake procedure. ## NOTE 1 – The TLS session identifier value (of the resumed TLS session) is identical to the previous value of the resumable TLS session. NOTE 2 – The TLS connection, which is established with the TLS abbreviated handshake based on the existing resumable TLS session, may be created while the original TLS connection, which was established via the TLS full handshake from which the resumable TLS client and server session endpoint states and hence the resumable TLS session were derived, is still established, or only after this original TLS connection is already terminated. NOTE 3 – Several new TLS connections may be established using TLS abbreviated handshakes based on the same resumable TLS session. If the first TLS connection is established using an TLS abbreviated handshake based on a resumable TLS session, then we may say that this TLS session becomes a resumed TLS session at this first TLS abbreviated handshake's end time. |

Finally, the two TLS procedures related to renegotiation and resumption could be defined as follows:

| TLS session renegotiation: | A TLS session level concept which leads, - by the execution of a TLS handshake procedure (full or abbreviated) -, to the update of the TLS protocol status "connection-level" information of an Data transfer ready TLS connection, for the purpose of the establishment of new cryptographic parameters. |
| --- | --- |
|  | NOTE 1 – Belongs to the category of bearer connection modification procedures. NOTE 2 – The initial TLS full handshake or TLS abbreviated handshake is not regarded as a TLS session renegotiation as this handshake is executed on a TLS connection which is in the state Idle. After this initial handshake is finished the TLS connection state is changed to Data transfer ready, and thus all following TLS full handshakes or TLS abbreviated handshakes on that TLS connection are regarded as TLS session renegotiation. NOTE 3 – The TLS connection state remains in Data transfer ready during and after the TLS session renegotiation. NOTE 4 – Term naming of "TLS session renegotiation": alternatives of "TLS renegotiation" and "TLS connection renegotiation" were also considered, but finally dropped because also not perfectly matching the underlying protocol procedure and their effect on protocol object information. |
| TLS session resumption: | A TLS session level concept which represents the execution of a TLS abbreviated handshake procedure on an existing TLS session, i.e., a semi-permanent TLS session, for the purpose of either deriving a further Data transfer ready TLS connection or updating of an existing Data transfer ready TLS connection. |

Hence, both procedural terms describe slightly different effects, but there are specific scenarios where both overlap ("the case of an TLS abbreviated handshake in order to update an existing Data transfer ready TLS connection").

## #6#.5	Formal description technique (FDT) based "TLS terminology framework"

## #6#.5.1		FDT using a mathematical-oriented pseudo language

The FDT provides a more detailed description concerning the dynamic aspects by explicit consideration of timing (i.e., time-dependent term definitions).

{Editor's note (2014-11): clause #6#.5 is a placeholder and related to the material contained in AVD-4614. It is still open whether a formal decription is really required or beneficia. Thus, clause #6#.5 might be deleted again before the approval of this Supplement.}

[End Proposed Correction]

____________________

Remember: H.248.90 release 1 could live with the abstraction of an "H.248 TLS security session" (aka "H.248 TLS bearer connection") as a merged entity of a "RFC 5246 TLS connection / RFC 5246 TLS session".

A "TLS object" represents in reality a data object, i.e. a data structure (with an internal hierarchy), which provides some information. However, a fairly abstracted model is sufficient for the considersations in this section.