| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4614 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | Backup-material to AVD-4600: Formal description of "TLS terminology framework" |  |  |  |
| Purpose: | Information |  |  |  |

Summary

This contribution supplements AVD-4600 by providing the formal description framework for TLS terminology.

This contribution updates the initial formal description as contained in C-484R1 from last meeting.

The formal description technique uses a mathematical-oriented pseudo language.

Whether this text is finally used for H.Sup13 is still open (see proposed Editor's Note in AVD-4600).

Thus, this contribution does not request any action and is for information only.

|  | 1 | Name | 01(33) | 28/04/99 | 01(33) | ALCATEL |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Definitions of TLS Related Terms

Table of Contents:

1	Introduction	2

1.1	Summary	2

1.2	References	2

2	Definition of TLS Related Terms	2

2.1	Sets of IP Addresses, L4 Ports, Transport Addresses and Transport Protocols	2

2.2	Set of Fully Qualified Domain Names	3

2.3	Time Aspects	3

2.4	Real System	3

2.5	TLS Protocol Implementation	3

2.6	TLS Capable System	3

2.7	TLS Endpoint	3

2.8	TLS Connection Endpoint	4

2.9	TLS Connection	5

2.10	TLS Message	5

2.11	TLS Client Connection Endpoint	6

2.12	TLS Server Connection Endpoint	7

2.13	TLS Client Role	7

2.14	TLS Server Role	7

2.15	TLS Message Sequence	7

2.16	TLS Full Handshake	7

2.17	TLS Abbreviated Handshake	9

2.18	Data Transfer Ready TLS Connection	10

2.19	TLS Client Session Endpoint State Derivation Operation and Transient and Semi-Permanent TLS Client Session Endpoint States	10

2.20	TLS Server Session Endpoint State Derivation Operation and Transient and Semi-Permanent TLS Server Session Endpoint State	12

2.21	Set of Semi-Permanent TLS Client Session Endpoint States	13

2.22	Set of Semi-Permanent TLS Server Session Endpoint States	14

2.23	Semi-Permanent TLS Session	14

2.24	Transient TLS Session	15

2.25	Resumable TLS Client Session Endpoint State	15

2.26	Resumable TLS Server Session Endpoint State	16

2.27	Resumable TLS Session	16

2.28	Resumed TLS Session	16

3	Open Points	17

Abstract:

This document defines TLS relates terms and uses a rather formal approach based on some basic mathematical concepts.

# Introduction

## Summary

Formal description of TLS terminology as used as baseline for AVD-4600.

## References

- ITU-T X.200 (7/94), Information Technology  –  Open Systems Interconnection  – Basic Reference Model:  The Basic Model

- IETF RFC 5246, The Transport Layer Security (TLS) Protocol Version 1.2

- IETF RFC 5077, Transport Layer Security (TLS) Session Resumption without Server-Side State

- IETF RFC 1035, DOMAIN NAMES - IMPLEMENTATION AND SPECIFICATION

- IETF RFC 791, INTERNET PROTOCOL, DARPA INTERNET PROGRAM PROTOCOL SPECIFICATION

- IETF RFC 2460, Internet Protocol, Version 6 (IPv6) Specification

- IANA Registry “Assigned Internet Protocol Numbers”, 
http://www.iana.org/assignments/protocol-numbers/protocol-numbers.xhtml

# Definition of TLS Related Terms

## Sets of IP Addresses, L4 Ports, Transport Addresses and Transport Protocols

We write AIP4 when referring to the set of IPv4 addresses as defined in RFC 791 [5], with equality of IPv4 addresses defined based on their 32 bit representations.

Similarly, let AIP6 be the set of IPv6 addresses as defined in RFC 2460 [6] , with equality of IPv6 addresses defined based on their 128 bit representations.

We set AIP≔ AIP4∪AIP6 and call this the set of IP addresses.

Additionally, we call AP≔0, ⋯, 216-1⊆N the set of L4 ports.

AIP4 may be considered as subset of 0, ⋯,2554, and AIP6 may be considered as a subset of 0, ⋯,216-18.

If talking of a transport address, then we refer to a pair of an IPv4 or an IPv6 address and a layer 4 (typically TCP or UDP or SCTP) port. We write AT for the set of transport addresses, that is we set

AT≔AIP×AP

We use the symbol P for the set of L4 transport protocol keywords as IANA registered on [7], such that e.g. TLS, UDP, SCTP∈P.

## Set of Fully Qualified Domain Names

We use AFQDN when referring to the set of fully qualified domain names as specified in RFC 1035 [4], where two fully qualified domain names are considered to be equal, if and only if they are identical as ASCII character strings.

## Time Aspects

Some of the following definitions utilize specific points in time. In such cases the time is modelled based on real numbers, and specific points in time are denoted as real number valued variables, like e.g. t0∈R.

## Real System

We use the definition of a real system R as defined in ITU-T X.200 [1].

We assume that for any two real systems R1 and R2 we can decide whether they are equal (R1=R2) or not equal (R1≠R2), and that for any object X we can decide, if X is a real system or not. That is, we assume that there is a set of real systems.

## TLS Protocol Implementation

A TLS protocol implementation is a (software or hardware) based implementation of the TLS protocol as specified in RFC 5246 [2] and is always associated to a real system.

## TLS Capable System

Let R be a real system. If R incorporates a TLS protocol layer implementation, then R is called a TLS capable system. A given TLS capable system R may be associated to a finite number greater than or equal to one of TLS protocol implementations. Each TLS capable system R is always associated to a creation point in time tc,R∈R, and we write Rtc,R when explicitly referring to R’s creation point in time.

For each real system R we assume that we can decide if R is a TLS capable system or not, that is that there is a set of TLS capable systems, which is a subset of the set of real systems.

Additionally we assume that at any specific point in time t0∈R with t0≥tc,R a TLS capable system R supports at least one and at most a finite number of local IP addresses. We write AIPR,t0 when referring to R’s set of IP addresses at point in time t0 and hence assume that AIPR,t0 is a non-empty and finite subset of AIP.

If Rtc,R is a TLS capable system created at tc,R, then R supports at most a finite number of local transport addresses in AT for any tc,R≤t∈R.

## TLS Endpoint

Let Rtc,R be a TLS capable system created at tc,R∈R. A TLS endpoint E on R is an instance of a TLS protocol implementation, which is associated to Rtc,R, and which is additionally associated to a unique identifier fE, see Note 3. Each TLS endpoint E is always associated to a creation point in time tc,R≤tc,E∈R, and we write Etc,E when explicitly referring to E’s creation point in time.

If E1tc,E1 is a TLS endpoint with identifier fE1created at tc,E1 on TLS capable system R1tc,R1, and if E2tc,E2 is a TLS endpoint with identifier fE2created at tc,E2 on TLS capable system R2tc,R2, then we say that E1tc,E1 is equal to E2tc,E2, and write E1tc,E1=E2tc,E2, if and only if fE1=fE2. We additionally require that E1tc,E1=E2tc,E2 implies tc,E1=tc,E2 and R1tc,R1=R2tc,R2.

Thus we require that a TLS endpoint is uniquely identified by its identifier. One option may be to use fully qualified domain names (FQDN) (see [4]) as such identifiers. However, this might not be sufficient in cases where a single homed IP host is assigned just one FQDN but supports more than just one TLS endpoint. In such cases a set of L4 addresses (ports) may additionally be required to uniquely identify a TLS endpoint.

Some of the subsequent definitions describe properties of TLS endpoints and concepts associated to one or also two such TLS endpoints. In some cases these properties or concepts are also associated with one or more specific points in time. In all such cases these points in time are always assumed to be later than (and thus greater than) the creation point(s) in time of the related TLS endpoint(s).

## TLS Connection Endpoint

Let R be a TLS capable system and Etc,E be a TLS endpoint on R created at tc,E∈R. A TLS connection endpoint p on Etc,E is a part of an instance of a TLS protocol implementation, which is associated to Etc,E, and which is able to send and receive TLS messages (see Section 2.10), and which is associated to exactly one 8-tuple being composed of

A creation point in time tc,E<tc∈R

A destruction point in time tc<td∈R

A (non-empty) local IP address al∈AIPR,tc

A (non-empty) local L4 port pl∈AP

An empty or non-empty remote IP address ar∈AIP

An empty or non-empty remote L4 port pr∈AP

A (non-empty) L4 transport protocol P∈P

The protocol string “TLS”

If FQDNs are used as TLS endpoint identifiers, then an additional requirement on the TLS connection endpoint could be that al is one of the IP addresses associated to the TLS endpoint’s FQDN.

We write

p[tc, td]

and say that p[tc,td] is a TLS connection endpoint on Etc,E existing in [tc,td] when emphasizing that this TLS connection endpoint only exists in the time interval from tc (included) till td (also included).

Thus we consider a TLS connection endpoint to be an element of a subset of R×R×AIP×AP×AIP∪""×AP∪""×P×"TLS" and may write ptc, td=tc,td,al,pl,ar,pr,P,"TLS", where ar or pr are allowed to be equal to “” (the empty string).

Two TLS connection endpoints p1[tc,1, td,1] and p2[tc,2, td,2] on Etc,E are said to be equal, in symbols

p1[tc,1, td,1]  = p2[tc,2, td,2]

if and only if they are equal as elements of R×R×AIP×AP×AIP∪""×AP∪""×P×"TLS".

With this equality relation on TLS connection endpoints on Etc,E we define for any tc,R<t0∈R

PE[t0] ∶= ptc, td  ptc, td is TLS connection endpoint on Etc,E existing in tc,td and  tc≤t0≤td

and call this the set of all TLS connection endpoints on Etc,E existing at t0.

The set PE[t0] is a subset of R×R×AIP×AP×AIP∪""×AP∪""×P×"TLS" and is finite for each point in time t0 because Etc,E is able to only support a finite number of local transport addresses at t0 as per the definition of Etc,E as a TLS endpoint on a TLS capable system R.

## TLS Connection

Let p1tc,1,td,1=tc,1,td,1,al,1,pl,1,ar,1,pr,1,P1,"TLS" be a TLS connection endpoint on a TLS endpoint E1tc,E1, and p2tc,2,td,2=tc,2,td,2,al,2,pl,2,ar,2,pr2,P2,"TLS" be a TLS connection endpoint on a TLS endpoint E2tc,E2. The 2-element set p1tc,1,td,1, p2tc,2,td,2 is called a TLS connection between p1tc,1,td,1 and p2tc,2,td,2 if and only if

al,1=ar,2∧al,2=ar,1∧pl,1=pr,2∧pl,2=pr,1∧P1=P2∧tc,1<td,2∧tc,2<td,1

Thus, formally the TLS connection is a set of two 8-tuples tc,1,td,1,al,1,pl,1,ar,1,pr,1,P1,"TLS",tc,2,td,2,al,2,pl,2,ar,2,pr2,P2,"TLS" with the additional condition that the coordinates of these two 8-tuples are related as defined above, such that we could also have defined a TLS connection as being a 10-tuple tc,1,td,1,tc,2,td,2,al,pl,ar,pr,P,"TLS". However, here we stick to the definition of a TLS connection being a set of two TLS endpoints with above mentioned additional conditions.

The conditions tc,1<td,1, tc,1<td,2, tc,2<td,2 and tc,2<td,1 imply that maxtc,1,tc,2<mintd,1,td,2, such that we can say that the TLS connection between p1tc,1,td,1 and p2tc,2,td,2 exists in the time interval maxtc,1,tc,2,mintd,1,td,2.

Defining a TLS connection as a set of two TLS connection endpoints rather than as a pair of two TLS connection endpoints has the advantage that the order of the two TLS connection endpoints is not significant when focusing on the TLS connection.

## TLS Message

Let E1tc,E1 and E2tc,E2 be TLS endpoints and p1tc,1, td,1 and p2tc,2, td,2 be two TLS connection endpoints on E1tc,E1 and E2tc,E2, respectively, such that p1tc,1,td,1, p2tc,2,td,2 is a TLS connection. A TLS message m is a finite sequence of information elements, which is generated either by the TLS handshake protocol layer instance, or by the TLS change cipher spec protocol layer instance, or by the TLS alert protocol layer instance of E1tc,E1, and which is sent by p1tc,1, td,1 to p2tc,2, td,2. In this case we call p1[tc,1, td,1] the source TLS connection endpoint and p2[tc,2, td,2] the destination TLS connection endpoint of m. Thus, a TLS message always consists either of a TLS handshake layer Handshake structure, or of a TLS change cipher spec layer ChangeCipherSpec structure,  or of a TLS alert layer Alert structure as defined in RFC 5246 [2].

This definition excludes application layer data fragments being sent by p1’s TLS record layer instance to p2’s TLS record layer instance.

m is sent from p1tc,1, td,1’s local transport address to p2tc,2, td,2’s local transport address, such that the L4 packet, which carries m, contains p1tc,1, td,1’s local transport address as source IP address and source L4 port, and contains p2tc,2, td,2’s local transport address as destination IP address and destination L4 port.

Each TLS message is always associated to a transmission point in time ts∈R with tc,1≤ts≤td,1 and to a reception point in time tr∈R with ts<tr and tc,2≤tr≤td,2.

These timing conditions immediately imply that tc,1<td,2, that is that p1 has been created before p2 is destroyed (which is actually to be expected).

We write

mts,tr

and say that mts,tr is a TLS message sent at ts and received at tr.

If we want to emphasize mts,tr’s source and destination TLS connection endpoints, then we also write

mts,tr : p1tc,1, td,1⇢ p2[tc,2, td,2]

mts,tr : p1tc,1, td,1⇢ p2[tc,2, td,2] always implies ts<tr, and tc,1≤ts≤td,1, and tc,2≤tr≤td,2.

Two TLS messages m1ts,1,tr,1 and m2ts,2,tr,2 are called equal, if there exists a TLS connection endpoint p1tc,1, td,1 on E1tc,E1 and a TLS connection endpoint p2tc,2, td,2 on E2tc,E2, such that p1tc,1, td,1,p2tc,2, td,2 is a TLS connection, and such that both m1ts,1,tr,1 and m2ts,2,tr,2 are sent from p1[tc,1, td,1] to p2[tc,2, td,2], and such that m1’s and m2’s finite bit vector representations are equal, and such that ts,1=ts,2 and tr,1=tr,2. In this case we write

m1ts,1,tr,1=m2ts,2,tr,2

A TLS message’s finite bit vector representation can be derived from Appendix A of RFC 5246 [2].

## TLS Client Connection Endpoint

Let Etc,E be a TLS endpoint. A TLS client connection endpoint p on Etc,E is a TLS connection endpoint on Etc,E, which sends a TLS message m containing a ClientHello handshake structure to another TLS connection endpoint at least once at some point in time during its existence. The TLS ClientHello handshake structure is defined in RFC 5246 [2]. We define Pc,Etc,E∶=∅, and for tc,R<t0∈R we set

Pc,Et0 ∶= ptc,td∈PEt0  ptc,td is TLS client connection endpoint on Etc,E

and call this the set of all TLS client connection endpoints on Etc,E existing at t0.

A TLS client connection endpoint does never send a TLS ServerHello handshake structure.

## TLS Server Connection Endpoint

Let Etc,E be a TLS capable system. A TLS server connection endpoint p on R is a TLS connection endpoint on Etc,E, which sends a TLS message m containing a ServerHello handshake structure to another TLS connection endpoint at least once at some point in time during its existence. The TLS ServerHello handshake structure is defined in RFC 5246 [2]. We define Ps,Etc,E∶=∅, and for tc,R<t0∈R we set

Ps,Et0 ∶= ptc,td∈PEt0  ptc,td is TLS server connection endpoint on Etc,E

And call this the set of all TLS server endpoints on Etc,E existing at t0.

A TLS server connection endpoint does never send a TLS ClientHello handshake structure.

## TLS Client Role

A TLS connection endpoint p is said to assume the TLS client role, if it is a TLS client connection endpoint.

## TLS Server Role

A TLS connection endpoint p is said to assume the TLS server role, if it is a TLS server connection endpoint.

## TLS Message Sequence

Let p1tc,1,td,1 be a TLS client connection endpoint and p2tc,2,td,2 be a TLS server connection endpoint such that p1tc,1,td,1, p2tc,2,td,2 is a TLS connection. A finite sequence m1ts,1,tr,1,  ⋯, mnts,n,tr,n with n≥1 is called a TLS message sequence associated to p1tc,1,td,1, p2tc,2,td,2, if mits,i,tr,i is a TLS message for each 1≤i≤n and if each mits,i,tr,i is either sent by p1tc,1,td,1 to  p2tc,2,td,2, or by p2tc,2,td,2 to p1tc,1,td,1.

## TLS Full Handshake

Let E1tc,E1 and E2tc,E2 be TLS endpoints, and let p1tc,1,td,1 be a TLS client connection endpoint on E1tc,E1 and p2tc,2,td,2 be a TLS server connection endpoint on E2tc,E2 such that p1tc,1,td,1, p2tc,2,td,2 is a TLS connection, and let m1ts,1,tr,1,⋯, mnts,n,tr,n be a TLS message sequence associated to p1tc,1,td,1, p2tc,2,td,2. If message m1 contains a ClientHello structure and is sent by p1tc,1,td,1 to  p2tc,2,td,2, and if m1ts,1,tr,1,  ⋯, mnts,n,tr,n constitutes a successful TLS full handshake message flow as specified in Section 7.3, Figure 1 of RFC 5246 [2], then m1ts,1,tr,1,  ⋯, mnts,n,tr,n is called a TLS full handshake between p1tc,1,td,1 and p2tc,2,td,2, and is denoted as

hfp1tc,1,td,1, p2tc,2,td,2ts,te∶=m1ts,1,tr,1,⋯, mnts,n,tr,n

Here we set

ts∶=ts,1 and te∶=tr,n

and call ts the TLS full handshake’s start time, and te the TLS full handshake’s end time.

This definition implies that n≥8, as a TLS full handshake at least consists of following TLS messages: ClientHello, ServerHello, ServerHelloDone, ClientKeyExchange, ChangeCipherSpec, Finished, ChangeCipherSpec, Finished.

This definition implies that ts,2>tr,1 (because p2 sends its ServerHello message only after it has received and processed p1’s ClientHello message). (We assume that processing a received TLS message takes a finite period of time of length greater than zero.)

This definition also implies that ts,i ≤ts,j for all 2≤i<j≤n.

This definition additionally implies that ts<te (due to ts,1<tr,1<ts,2≤ts,n<tr,n).

As already noted in Section 2.9, this definition also implies that tc,1<td,2 and also that tc,2<td,1, as the full handshake also contains at least one TLS message sent by p2tc,2,td,2 to p1tc,1,td,1. Thus this definition implies that p1tc,1,td,1 is not destroyed before p2tc,2,td,2 is created, and vice versa that p2tc,2,td,2 is not destroyed before p1tc,1,td,1 is created. Hence there is some point in time t0 at which both TLS connection endpoints exist simultaneously.

If hfp1tc,1,td,1, p2tc,2,td,2ts,te=m1ts,1,tr,1,  ⋯, mnts,n,tr,n is a TLS full handshake, then message m2ts,2,tr,2 contains a ServerHello structure as defined in Section 7.4.1.3 of RFC 5246 [2]. In this case m2’s ServerHello.session_id value may or may not be empty. If this value is non-empty, then we require that (i) it is pairwise different from the session_id values of all TLS server session endpoint states associated to E2 existing at ts,2, and (ii) that additionally this ServerHello.session_id value is also different from all session id values which might be used by TLS full handshakes, which are ongoing but not yet completed at ts,2, between any other TLS server connection endpoint on E2 existing at ts,2. (See Section 2.22 for a definition of TLS server session endpoint states associated to a TLS endpoint and existing at a certain point in time.) This requirement goes beyond the related requirement in Section 7.4.1.3 of RFC 5246 [2], as in case of a full handshake there it is only required that the TLS server connection endpoint adds a session_id value (if this is non-empty) to its ServerHello message, which is unequal to the session_id value in the received ClientHello message. Here we additionally require the replied non-empty ServerHello.session_id value being unique among all TLS server session endpoint states stored on the TLS server connection endpoint’s TLS endpoint at the corresponding point in time the ServerHello message is sent in order to guarantee that a received ClientHello.session_id value always refers to at most one existing TLS server session endpoint state associated to this TLS server connection endpoint’s TLS endpoint.

If p1tc,1,td,1 and p2tc,2,td,2 are two TLS client connection endpoints on a TLS endpoint E1tc,E1, and if p3tc,3,td,3 and p4tc,4,td,4 are two TLS server connection endpoints on a TLS endpoint E2tc,E2, and if hf,1(p1tc,1,td,1,  p3tc,3,td,3)ts,1,te,1= m1,1[ts,1,1,tr,1,1],  ⋯, m1,nts,1,n,tr,1,n is a TLS full handshake between p1tc,1,td,1 and p3tc,3,td,3, and if hf,2(p2tc,2,td,2,  p4tc,4,td,4)ts,2,te,2= m2,1[ts,2,1,tr,2,1],  ⋯, m2,kts,2,k,tr,2,k is a TLS full handshake between p2tc,2,td,2 and p4tc,4,td,4, then we say that hf,1(p1tc,1,td,1,  p3tc,3,td,3)ts,1,te,1 is equal to hf,2(p2tc,2,td,2,  p4tc,4,td,4)ts,2,te,2 and write

hf,1p1tc,1,td,1, p3tc,3,td,3ts,1,te,1= hf,2p2tc,2,td,2, p4tc,4,td,4ts,2,te,2

if and only if p1tc,1,td,1= p2tc,2,td,2 and p3tc,3,td,3= p4tc,4,td,4 and n = k and m1,its,1,i,tr,1,i= m2,its,2,i,tr,2,i for all i ∈1, ⋯, n.

This implies that also ts,1=ts,2 and te,1=te,2.

## TLS Abbreviated Handshake

Let E1tc,E1 and E2tc,E2 be TLS endpoints, and let p1tc,1,td,1 be a TLS client connection endpoint on E1tc,E1 and p2tc,2,td,2 be a TLS server endpoint on E2tc,E2 such that p1tc,1,td,1, p2tc,2,td,2 is a TLS connection, and let m1ts,1,tr,1, ⋯, mnts,n,tr,n be a TLS message sequence associated to pctc,1,td,1, pstc,2,td,2. If message m1 contains a ClientHello structure and is sent by p1tc,1,td,1 to  p2tc,2,td,2, and if m1ts,1,tr,1,  ⋯, mnts,n,tr,n constitutes a successful TLS abbreviated handshake message flow as specified in Section 7.3, Figure 2 of RFC 5246 [2], then m1ts,1,tr,1,  ⋯, mnts,n,tr,n is called a TLS abbreviated handshake between p1tc,1,td,1 and p2tc,2,td,2, and is denoted as

hap1tc,1,td,1, p2tc,2,td,2ts,te∶=m1ts,1,tr,1,⋯, mnts,n,tr,n

Here we set

ts∶=ts,1 and te∶=tr,n

and call ts the TLS abbreviated handshake’s start time, and te the TLS abbreviated handshake’s end time.

We define equality of TLS abbreviated handshakes in a similar way as equality of TLS full handshakes.

If hap1tc,1,td,1, p2tc,2,td,2ts,te=m1ts,1,tr,1,  ⋯, mnts,n,tr,n is a TLS abbreviated handshake, then, according to Section 7.4.1.3 of RFC 5246 [2], message m1’s  ClientHello structure contains a non-empty session_id value, and m2 contains a ServerHello structure, whose session_id value is also non-empty and is equal to m1’s ClientHello.session_id value. Due to the uniqueness of the session_id values of all TLS server session endpoint states associated to E2 (according to the definition of a TLS full handshake), in this case there exists exactly one TLS server session endpoint state es associated to E2 and existing at ts,2, such that m1.ClientHello.session_id=m2.ServerHello.session_id=es.session_id. 
Additionally, in order for the TLS client connection endpoint p1tc,1,td,1 on E1 to be able to add this session_id value to its generated ClientHello message, there needs to exist a TLS client session endpoint state ec associated to E1 and existing at ts,1, such that m1.ClientHello.session_id=ec.session_id.
In case of such a TLS abbreviated handshake we therefore always have ec.session_id=m1.ClientHello.session_id=m2.ServerHello.session_id=es.session_id.
See Section 2.21 for a definition of the TLS client session endpoint states associated to a TLS endpoint at a certain point in time, and see Section 2.22 for a definition of the TLS server session endpoint states associated to a TLS endpoint existing at a certain point in time.

## Data Transfer Ready TLS Connection

Let p1tc,1,td,1 be a TLS client connection endpoint on a TLS endpoint E1tc,E1, and p2tc,2,td,2 be a TLS server connection endpoint on a TLS endpoint E2tc,E2, such that p1tc,1,td,1, p2tc,2,td,2 is TLS connection between p1tc,1,td,1 and p2tc,2,td,2. According to Note 9 this TLS connection exists in the time interval maxtc,1,tc,2,mintd,1,td,2. Additionally, let t0∈maxtc,1,tc,2,mintd,1,td,2. We say that p1tc,1,td,1, p2tc,2,td,2 is a data transfer ready TLS connection at t0, if there exists an n≥ 2 and a TLS message sequence m1ts,1,tr,1,  ⋯, mnts,n,tr,n associated to p1tc,1,td,1, p2tc,2,td,2, such that m1ts,1,tr,1,  ⋯, mnts,n,tr,n is either a TLS full handshake or a TLS abbreviated handshake, and such that t0≥tr,n, that is, such that t0 is greater than or equal to this full or abbreviated handshake’s end time.

## TLS Client Session Endpoint State Derivation Operation and Transient and Semi-Permanent TLS Client Session Endpoint States

Let E1tc,E1 and E2tc,E2 be TLS endpoints, and p1tc,1,td,1 be a TLS connection endpoint on E1tc,E1 assuming the TLS client role, and p2tc,2,td,2 be a TLS connection endpoint on E2tc,E2 assuming the TLS server role, and p1tc,1,td,1, p2tc,2,td,2 be a TLS connection between p1tc,1,td,1 and p2tc,2,td,2. Furthermore, let 0<Δt∈R be a positive real number.

The TLS client session endpoint state derivation operation associated to E1tc,E1 and Δt is denoted as δc,E1,Δt and is an algorithm implemented on E1tc,E1, which takes as input a TLS full handshake hfp1tc,1,td,1, p2tc,2,td,2ts,te between p1tc,1,td,1 and p2tc,2,td,2, and which calculates the following n-tuple of parameter-value pairs, as partially described in RFC 5246 [2].

version: ProtocolVersion

prf_algorithm: PRFAlgorithm

bulk_cipher_algorithm: BulkCipherAlgorithm

cipher_type: CipherType

enc_key_length: uint8

block_length: unit8

fixed_iv_length: unit8

record_iv_length: unit8

mac_algorithm: MACAlgorithm

mac_length: unit8

mac_key_length: unit8

compression_algorithm: CompressionMethod

master_secret: opaque[48]

session_id: SessionID

Creation time tc∈R

Destruction time td∈R

server_address: IPaddress (see Note 30)

The n-tuple of these attribute-value pairs is called a TLS client session endpoint state associated to E1tc,E1 and p1tc,1,td,1, p2tc,2,td,2 and is denoted as sc.

δc,E1,Δt assigns the creation point in time tc and the destruction point in time td to sc as follows:

It always sets the creation point in time tc to be equal to the full handshake’s end time, that is it sets tc≔te.

If checks if the full handshake’s ServerHello.session_id is non-empty or empty.

If the full handshake’s ServerHello.session_id is non-empty, then it calculates the destruction point in time td as td∶=te+Δt.

If the full handshake’s ServerHello.session_id is empty, then it sets the destruction point in time td as td∶=te, such that then td=tc=te.

We write sctc,td when emphasizing the TLS client session endpoint state’s creation and destruction times.

With these definitions in place we may thus write

sctc,td = δc,E1,Δthfp1tc,1,td,1, p2tc,2,td,2ts,tetc,td

and have always tc=te, and td=tc+Δt or td=tc.

We additionally require that each TLS client session endpoint state associated to E1tc,E1 is always the result of E1’s TLS client session endpoint state derivation operation being applied to a corresponding TLS full handshake, that is that for each TLS client session endpoint state sctc,td associated to E1tc,E1 there exist TLS connection endpoints p1tc,1,td,1 and p2tc,2,td,2 and a TLS full handshake hfp1tc,1,td,1, p2tc,2,td,2ts,te such that

sctc,td=δc,E1,Δthfp1tc,1,td,1, p2tc,2,td,2ts,tetc,td

If sctc,td is a TLS client session endpoint state associated to E1tc,E1, and if td>tc, then we call sctc,td a semi-permanent TLS client session endpoint state. Else, if td=td, then we call sctc,td a transient TLS client session endpoint state.

If sctc,td is a transient TLS client session endpoint state (such that tc=td), then we may also use sctc as abbreviation of sctc,td and say that sctc is a transient TLS client session endpoint state existing at tc.

We thus require that the TLS client session endpoint state derivation operation is executed at the corresponding TLS full handshake’s end time.

The value of Δt may depend on local policy rules implemented on E1tc,E1 as e.g. described in Appendix F.1.4 in RFC 5246 [2].

While a semi-permanent TLS client session endpoint state’s creation point in time correlates with the corresponding TLS full handshake’s end time (which is thus a point in time at which both TLS connection endpoints, which exchange the TLS full handshake messages, do still exist), this semi-permanent TLS client session endpoint state’s destruction point in time is independent of the destruction points in time of these two TLS connection endpoints. Especially, the semi-permanent TLS client session endpoint state may still exist after one or even both TLS connection endpoints are already destroyed.

A TLS client session endpoint state is semi-permanent if and only if its session_id value is non-empty. Hence it is transient if and only if its session_id value is empty.

A TLS protocol implementation may add further information to a TLS client session endpoint state, like e.g. the associated TLS server endpoint’s source IP address. This additional information may be used by a TLS client connection endpoint in order to decide if an already stored semi-permanent TLS client session endpoint state may be used (may be “resumed”) for the establishment of a new TLS connection towards a destination transport address of another TLS endpoint.

## TLS Server Session Endpoint State Derivation Operation and Transient and Semi-Permanent TLS Server Session Endpoint State

Let E1tc,E1 and E2tc,E2 be TLS endpoints, and p1tc,1,td,1 be a TLS connection endpoint on E1tc,E1 assuming the TLS client role, and p2tc,2,td,2 be a TLS connection endpoint on E2tc,E2 assuming the TLS server role, and p1tc,1,td,1, p2tc,2,td,2 be a TLS connection between p1tc,1,td,1 and p2tc,2,td,2. Furthermore, let 0<Δt∈R be a positive real number.

The TLS server session endpoint state derivation operation associated to E2tc,E2 and Δt is denoted as δs,E2,Δt and is an algorithm implemented on E2tc,E2, which takes as input a TLS full handshake hfp1tc,1,td,1, p2tc,2,td,2ts,te between p1tc,1,td,1 and p2tc,2,td,2, and which calculates the following n-tuple of parameter-value pairs as described in RFC 5246 [2].

version: ProtocolVersion

prf_algorithm: PRFAlgorithm

bulk_cipher_algorithm: BulkCipherAlgorithm

cipher_type: CipherType

enc_key_length: uint8

block_length: unit8

fixed_iv_length: unit8

record_iv_length: unit8

mac_algorithm: MACAlgorithm

mac_length: unit8

mac_key_length: unit8

compression_algorithm: CompressionMethod

master_secret: opaque[48]

session_id: SessionID

Creation time tc∈R

Destruction time td∈R

The n-tuple of these attribute-value pairs is called a TLS server session endpoint state associated to E2tc,E2 and p1tc,1,td,1, p2tc,2,td,2 and is denoted as ss.

δs,E2,Δt assigns a creation point in time tc and a destruction point in time td to ss as follows:

It always sets the creation point in time tc to be equal to the full handshake’s end time, that is it sets tc≔te.

If checks if the full handshake’s ServerHello.session_id is non-empty or empty.

If the full handshake’s ServerHello.session_id is non-empty, then it calculates the destruction point in time td as td∶=te+Δt.

If the full handshake’s ServerHello.session_id is empty, then it sets the destruction point in time td as td∶=te, such that then td=tc=te.

We write sstc,td when emphasizing the TLS server session endpoint state’s creation and destruction times.

With these definitions in place we may thus write

sstc,td = δs,E2,Δthfp1tc,1,td,1, p2tc,2,td,2ts,tetc,td

and have always tc=te and td=tc+Δt.

We additionally require that each TLS server session endpoint state associated to E2tc,E2 is always the result of E2’s TLS server session endpoint state derivation operation being applied to a corresponding TLS full handshake, that is that for each TLS server session endpoint state sstc,td associated to E2tc,E2 there exist TLS connection endpoints p1tc,1,td,1 and p2tc,2,td,2 and a TLS full handshake hfp1tc,1,td,1, p2tc,2,td,2ts,te such that

sstc,td=δs,E2,Δthfp1tc,1,td,1, p2tc,2,td,2ts,tetc,td

If sstc,td is a TLS server session endpoint state associated to E2tc,E2, and if td>tc, then we call sstc,td a semi-permanent TLS server session endpoint state. Else, if td=td, then we call sstc,td a transient TLS server session endpoint state.

If sstc,td is a transient TLS server session endpoint state (such that tc=td), then we may also use sstc as abbreviation of sstc,td and say that sstc is a transient TLS server session endpoint state existing at tc.

We thus require that the TLS server session endpoint state derivation operation is executed at the corresponding TLS full handshake’s end time.

The value of Δt may depend on local policy rules implemented on E2tc,E2 as e.g. described in Appendix F.1.4 in RFC 5246 [2].

While a semi-permanent TLS server session endpoint state’s creation point in time correlates with the corresponding TLS full handshake’s end time (which is thus a point in time at which both TLS connection endpoints, which exchange the TLS full handshake messages, do still exist), this semi-permanent TLS server session endpoint state’s destruction point in time is independent of the destruction points in time of these two TLS connection endpoints. Especially, the semi-permanent TLS server session endpoint state may still exist after one or even both TLS connection endpoints are already destroyed.

A TLS server session endpoint state is semi-permanent if and only if its session_id value is non-empty. Hence it is transient if and only if its session_id value is empty.

## Set of Semi-Permanent TLS Client Session Endpoint States

Let Etc,E be a TLS endpoint created at tc,E. If sc,1tc,1,td,1 and sc,2tc,2,td,2 are two semi-permanent TLS client session endpoint states associated to Etc,E, then we say that  sc,1tc,1,td,1 is equal to sc,2tc,2,td,2, and write

sc,1tc,1,td,1= sc,2tc,2,td,2

if and only if sc,1tc,1,td,1 and sc,2tc,2,td,2 are equal as n-tuples.

This definition of equality of semi-permanent TLS client session endpoint states sc,1tc,1,td,1 and sc,2tc,2,td,2 associated to a TLS endpoint Etc,E especially implies that tc,1=tc,2 and td,1=td,2.

With this equality relation on semi-permanent TLS client session endpoint states associated to E we define Sc,Etc,E∶=∅ and for any tc,E<t0∈R

Sc,Et0 ∶=sctc,td  sctc,td is semi-permanent TLS client session endpoint state associated to E and tc≤t0≤td

and call this the set of semi-permanent TLS client session endpoint states associated to Etc,E existing at t0.

## Set of Semi-Permanent TLS Server Session Endpoint States

Let Etc,E be a TLS endpoint created at tc,R. If ss,1tc,1,td,1 and ss,2tc,2,td,2 are two semi-permanent TLS server session endpoint states associated to Etc,E, then we say that  ss,1tc,1,td,1 is equal to ss,2tc,2,td,2, and write

ss,1tc,1,td,1= ss,2tc,2,td,2

if and only if ss,1tc,1,td,1 and ss,2tc,2,td,2 are equal as n-tuples.

This definition of equality of semi-permanent TLS server session endpoint states ss,1tc,1,td,1 and ss,2tc,2,td,2 associated to a TLS endpoint Etc,E especially implies that tc,1=tc,2 and td,1=td,2.

With this equality relation on semi-permanent TLS server session endpoint states associated to E we define Ss,Etc,E∶=∅ and for any tc,E<t0∈R

Ss,Et0 ∶=sstc,td  sstc,td is semi-permanent TLS server session endpoint state associated to E and tc≤t0≤td

and call this the set of semi-permanent TLS server session endpoint states associated to Etc,E existing at t0.

Sections 7.3 and 7.4.1.3 of RFC 5246 [2] talk about a TLS server’s “session cache”. The set of semi-permanent TLS server session endpoint states may be thought of such a “session cache”.

## Semi-Permanent TLS Session

Let E1tc,E1 and E2tc,E2 be TLS endpoints, maxtc,E1,tc,E2<t0∈R, and let Sc,E1t0 be the set of all semi-permanent TLS client session endpoint states associated to E1tc,E1 existing at t0, and let Ss,E2t0 be the set of all semi-permanent TLS server session endpoint states associated to E2tc,E2 also existing at t0. Further let sctc,sc,td,sc ∈ Sc,E1t0 be a semi-permanent TLS client session endpoint state associated to E1tc,E1 existing at t0, and let sstc,ss,td,ss ∈ Ss,E2t0 be a semi-permanent TLS server session endpoint state associated to E2tc,E2 also existing at t0. The pair sctc,sc,td,sc, sstc,ss,td,ss is called a Semi-Permanent TLS session between E1tc,E1 and E2tc,E2 if there exists a TLS client connection endpoint pctc,pc,td,pc on E1tc,E1 and a TLS server connection endpoint pstc,ps,td,ps on E2tc,E2 such that pctc,pc,td,pc, pstc,ps,td,ps is a TCP connection between pctc,pc,td,pc and pstc,ps,td,ps, and such that there exists a TLS full handshake hfpctc,pc,td,pc, pstc,ps,td,psts,te between pctc,pc,td,pc and pstc,ps,td,ps, and such that

sctc,sc,td,sc= δc,E1,Δt1hfpctc,pc,td,pc, pstc,ps,td,psts,tetc,sc,td,sc

and

sstc,ss,td,ss= δs,E2,Δt2hfpctc,pc,td,pc, pstc,ps,td,psts,tetc,ss,td,ss

If sctc,sc,td,sc, sstc,ss,td,ss is a semi-permanent TLS session with 
sctc,sc,td,sc= δc,E1,Δt1hfpctc,pc,td,pc, pstc,ps,td,psts,tetc,sc,td,sc and 
sstc,ss,td,ss= δs,E2,Δt2hfpctc,pc,td,pc, pstc,ps,td,psts,tetc,ss,td,ss, 
then, as tc,sc=te=tc,ss, we can consider te as semi-permanent TLS session creation point in time. However, as Δt1 and Δt2 are independent (each TLS endpoint may define its TLS client and session endpoint state derivation operation’s Δt values independently) we cannot assume that always td,sc=td,ss. Instead, we may consider mintd,sc,td,ss as the semi-permanent TLS session’s destruction point in time. We have tc,sc=tc,ss<mintd,sc,td,ss and may thus say that the semi-permanent TLS session sctc,sc,td,sc, sstc,ss,td,ss exists in the time interval tc,sc,mintd,sc,td,ss.

Similar to the involved semi-permanent TLS client and server session endpoint states, a semi-permanent TLS session may still exist after one or even both TLS connection endpoints are already destroyed, which did exchange the TLS full handshake messages from which the related semi-permanent TLS session endpoint states were derived from.

## Transient TLS Session

Let E1tc,E1 and E2tc,E2 be TLS endpoints, maxtc,E1,tc,E2<t0∈R, let sct0 be a transient TLS client session endpoint state existing at t0 associated to E1tc,E1, and let sst0 be a transient TLS server session endpoint state existing at t0 associated to E2tc,E2. The pair sct0,sst0 is called a transient TLS session between E1tc,E1 and E2tc,E2, if there exists a TLS client connection endpoint pctc,pc,td,pc on E1tc,E1 and a TLS server connection endpoint pstc,ps,td,ps on E2tc,E2 such that pctc,pc,td,pc, pstc,ps,td,ps is a TCP connection between pctc,pc,td,pc and pstc,ps,td,ps, and such that there exists a TLS full handshake hfpctc,pc,td,pc, pstc,ps,td,psts,te between pctc,pc,td,pc and pstc,ps,td,ps, and such that

sct0=sct0,t0= δc,E1,Δt1hfpctc,pc,td,pc, pstc,ps,td,psts,tet0,t0

and

sst0=sst0,t0= δs,E2,Δt2hfpctc,pc,td,pc, pstc,ps,td,psts,tet0,t0

In this case we also say that sct0,sst0 is a transient TLS session between E1tc,E1 and E2tc,E2 at t0.

## Resumable TLS Client Session Endpoint State

Let Etc,E be a TLS endpoint. A semi-permanent TLS client session endpoint state associated to Etc,E existing at a time tc,E<t0∈R is also called a resumable TLS client session endpoint state associate to Etc,E existing at t0.

This definition explicitly excludes the session ticket based TLS session resumption mechanism as defined in RFC 5077 [3]. It is for further study how the definition of a resumable TLS client session endpoint state might be modified in order to also model session ticket based TLS session resumption.

## Resumable TLS Server Session Endpoint State

Let Etc,E be a TLS endpoint. A semi-permanent TLS server session endpoint state associated to Etc,E existing at a time tc,E<t0∈R is also called a resumable TLS server session endpoint state associate to Etc,E existing at t0.

As in case of the definition of the resumable TLS client session endpoint state, this definition explicitly excludes the session ticket based TLS session resumption mechanism as defined in RFC 5077 [3]. It is for further study how the definition of a resumable TLS server session endpoint state might be modified in order to also model session ticket based TLS session resumption.

## Resumable TLS Session

A semi-permanent TLS session is also called a resumable TLS session.

## Resumed TLS Session

Let E1tc,E1 and E2tc,E2 be TLS endpoints, maxtc,E1,tc,E2<t0∈R, and let Sc,E1t0 be the set of all semi-permanent TLS client session endpoint states associated to E1tc,E1 existing at t0, and let Ss,E2t0 be the set of all semi-permanent TLS server session endpoint states associated to E2tc,E2 also existing at t0. Further let sctc,sc,td,sc ∈ Sc,E1t0 be a semi-permanent TLS client session endpoint state associated to E1tc,E1 existing at t0, and let sstc,ss,td,ss ∈ Ss,E2t0 be a semi-permanent TLS server session endpoint state associated to E2tc,E2 also existing at t0 such that  sctc,sc,td,sc, sstc,ss,td,ss is as resumable TLS session. As per Note 38 this resumable TLS session exists in the time interval tc,sc,mintd,sc,td,ss. If t1∈tc,sc,mintd,sc,td,ss, and if there exists a TLS server connection endpoint pstc,ps,td,ps on E2tc,E2, and if there exists a TLS client connection endpoint pctc,pc,td,pc on a TLS endpoint E3tc,E3 with tc,E3<t1 such that pctc,pc,td,pc,pstc,ps,td,ps is a TLS connection, and if there additionally exists a TLS abbreviated handshake hapctc,pc,td,pc,  pstc,ps,td,psts,te=m1ts,1,tr,1,m2ts,2,tr,2 ,⋯, mnts,n,tr,n such that m1’s ClientHello structure contains the session_id value of sc and such that m2’2 ServerHello structure contains the session_id value of ss, and such that also t1=tr,n, then we say that sctc,sc,td,sc, sstc,ss,td,ss is a TLS session, which is resumed at t1.

In this case the two session_id values are equal, that is it is m1.ClientHello.session_id= sc.session_id=ss.session_id=m2.ServerHello.session_id.

Due to t1=tr,n we consider the TLS session as being resumed at the abbreviated handshake’s end time.

In above definition we do not require that E3tc,E3=E1tc,E1. This definition allows E3 and E1 to be unequal. However, if E3 is actually unequal to E1, then we assume that the semi-permanent TLS client session endpoint state sc was transferred or copied from E1 to E3 prior to the transmission of the abbreviated handshake’s first ClientHello message from E3’s TLS client connection endpoint pc to E2’s TLS server connection endpoint ps.

As already noted in Section 2.17 after the definition of the abbreviated handshake, the TLS client connection endpoint involved in the abbreviated handshake copies the related TLS client session endpoint state’s session_id value into its ClientHello.session_id attribute, such that we may assume that this TLS client session endpoint state exists at the abbreviated handshake’s start time ts. In order for the TLS server endpoint to accept the received session_id value, the related TLS server session endpoint state needs to exist at the message m1’s reception point in time, which typically is “shortly” after m1’s transmission point in time, that is “shortly” after the abbreviated handshake’s start time. As this TLS server session endpoint state was created at the same point in time as the related TLS client session endpoint state was also created, we may assume that the TLS server session endpoint state also exists at the abbreviated handshake’s start time.

The TLS connection, which is established with the abbreviated handshake based on the existing resumable TLS session, may be created while the original TLS connection, which was established via the TLS full handshake from which the resumable TLS client and server session endpoint states and hence the resumable TLS session were derived, is still established, or only after this original TLS connection is already terminated.

Several new TLS connections may be established using abbreviated handshakes based on the same resumable TLS session. If the first TLS connection is established using an abbreviated handshake based on a resumable TLS session, then we may say that this TLS session becomes a resumed TLS session at this first abbreviated handshake’s end time.

# Open Points

RFC 5246 [2], Section 7.2.2:

Error handling in the TLS Handshake protocol is very simple.  When an error is detected, the detecting party sends a message to the other party.  Upon transmission or receipt of a fatal alert message, both parties immediately close the connection.  Servers and clients MUST forget any session-identifiers, keys, and secrets associated with a failed connection.  Thus, any connection terminated with a fatal alert MUST NOT be resumed.

The definition of a resumable TLS server session endpoint state should be modified in order to also accommodate this condition. Also the definition of a resumable TLS client session endpoint state may need to be changed correspondingly.

Potential options:

Definition of a fatal alert on a TLS connection (transmission or reception of a fatal alert message by one of the TLS connection’s endpoints.)

Based on this, definition of a fatal alert associated to a TLS session

Or, definition of a fatal alert associated to a TLS client and server session endpoint state

Based on this, either modification of a TLS (client or server) session endpoint state’s destruction point in time (at fatal alert transmission or reception point in time), or introduction of a TLS (client and server) session endpoint state property “is resumable”, which changes from “true” to “false” at fatal alert transmission or reception point in time.

RFC 5077 [3]:

Session resumption based on session ticket; modification of definitions of

Resumable TLS (client and server) session endpoint states

Resumable TLS session

Resumed TLS session

Others?