| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4618 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.50 (Rev): Living list item #5.3 - "Happy Eyeballs Extension for ICE" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution aims to progress the discussion about the ICE capability of "IPv4/IPv6 dual stack fairness", formally called "happy eyeballs extension for ICE".

1	Introduction

This pending topic is anchored in the living list around two years:

### L.5.3	Item #5.3: IETF document “Happy Eyeballs Extension for ICE”

| Reference: | draft-reddy-mmusic-ice-happy-eyeballs-00 (2012-10-04) “Happy Eyeballs Extension for ICE” URL: http://tools.ietf.org/id/draft-reddy-mmusic-ice-happy-eyeballs-00.txt |
| --- | --- |
| Motivation: | - related to dual stack hosts (the notion of “happy eyeballs” refers to RFC 6555), i.e., candidate selection in parallel for IPv4 and IPv6 media path options (rather than serial …) - reduced delay … - no ICE syntactical changes, just algorithmic extensions (TBD) |
| H.248.50 impact? | - ### TO BE CHECKED |

2	Discussion and summary

The related technology is in the meanwhile termed as "ICE IPv4/IPv6 Dual Stack Fairness" in IETF. There will be only algorithmic changes within an ICE entity, but not any syntactical ICE protocol extensions.

The technology could be basically supported by the "H.248.50 toolkit". There'll be changes at all place where the ICE "address prioritization" step is touched. Attached change proposals provide such indications from our perspective.

The MGC or MG could be in charge of the ICE "address prioritization" step, dependent on the underlying network solution related to ICE. When that task would be subject of the MG, then the MG needs to know which algorithm concerning the calculation of priority values needs to be used. This contribution makes the assumption that such information could be part of an H.248 profile specification, also due to the fact that information about the address prioritization algorithm is out of scope of call control signaling (rather a network operator policy).

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against the input draft AVD-4592 to this meeting (AVD-4592).

Change proposal #1:

[Begin Proposed Correction]

### 6.5.3	ICE-controlled STUN/TURN

See clause 8. The ICE (interactive connectivity establishment) methodology defines a "superior" protocol that uses the STUN/TURN mechanism and other protocols. The use of ICE is also tightly coupled with SIP and in particular with SIP/SDP Offer/Answer procedures. ICE is executed between a pair of ICE agents. There are two device types concerning the embodiment of ICE agents with respect to their location in the IP network infrastructure:

- terminal-embedded ICE agent: user equipment with ICE agent function and SIP UA function;

- gateway-embedded ICE agent: a decomposed H.248 gateway provides the ICE agent function in a distributed manner (with the SIP signalling function located at MGC level).

This clause describes the ICE process from the perspective of a generic ICE agent, i.e., it does not differentiate between the above two embodiments (but indicating possible H.248 impact for the gateway-provided ICE agent).

The execution of ICE may be basically divided into several consecutive phases:

1)	Gather addresses

- Purpose: Address gathering.

- This phase may be based on STUN and/or TURN procedures as required.

- Used ITU-T H.248.50 packages: None.

2)	Prioritizing addresses

- Purpose: optimize final IP bearer path (routing).

- Function provided either by

- MG, in case of FIXTHIS; or

- MGC, in case of FIXTHIS; or

- external (SIP) server, in case of FIXTHIS;

- Used ITU-T H.248.50 packages: FIXTHIS.

- Address prioritization rules: The basic rules for address prioritization are described in clause 4.1.2.1/[IETF RFC 5245]. The purpose of ICE IPv4/IPv6 dual stack fairness support may demand for other prioritization algorithms, see [b-IETF ice-dualstack]. The applicable prioritization rules should be explicitly part of an H.248 profile specification in case that an H.248 entity is responsible for that ICE function..

3)	Call/session control signalling: generate SDP Offer (for SIP signalling)

- Purpose: generate SDP Offer content (if originating side).

- Multiple SDP attributes may be involved: FIXTHIS.

NOTE – The SDP attribute values may be requested from MGC and provided by MG, in case of network scenario FIXTHIS.

- Used ITU-T H.248.50 packages: None (due to application control protocol related function).

4)	Call/session control signalling: Negotiations – Offering and Answering (initiate and accept SIP messages

–	Purpose: Address advertisement.

–	This phase is typically related to the initiation of a SIP INVITE (thus, out of scope of this Recommendation).

NOTE – The peer user (called party, invited party) may also start a correspondent "address gathering" procedure (based on STUN and/or TURN as required). These remote procedures would not be visible for the local MGC/MG entities.

–	The peer (SIP) user may reply with a SIP OK (thus, out of scope of this Recommendation).

–	Used ITU-T H.248.50 packages: None (due to application control protocol related function).

5)	Connectivity checks

–	This phase is again based on STUN and/or TURN procedures as required.

–	The connectivity checks are based on the same 4-tuple(s) as (later) used for the ITU-T H.248 stream.

–	The STUN/TURN messages may be thus multiplexed into the RTP/RTCP packet flow of the checked ITU-T H.248 stream (of an ITU-T H.248 RTP termination).

–	Used ITU-T H.248.50 packages: mgastuns, ostuncc.

6)	Completing ICE procedures

–	One agent assumes ICE controlling role, other agent assumes ICE controlled role.

–	Used ITU-T H.248.50 packages: FIXTHIS.

{Editor’s note (2013-01 meeting): aspect of “Gateway Free Operation (GrFO)” – there might be ICE variants which may conclude that the routed IP bearer path could possibly finally bypass the MG instance, which was initially involved in above ICE procedural steps. The initially created H.248 context would be released again, the MG is deallocated (which may be termed as GrFO, similar to TrFO). 
In case of GrFO, then the next step would be different. 
Note: GrFO could also lead to just an “MG transparent forwarding packet forwarding mode”, i.e., the initial IP bearer path would be still routed via the MG, but agnostic packet forwarding …
Contributions are solicited.}

7)	Bearer connection: check/selection of media streams

–	Used ITU-T H.248.50 packages: None.

The outlined, high level ICE process is not detailed concerning possible differences from ICE mode perspective. The ICE mode impact on gateway-embedded ICE agents is described in Appendix II.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

## 7.1	STUN base package

|  | Package name: | STUN Base |
| --- | --- | --- |
|  | Package ID: | stunb (0x00bd) |
|  | Description: | This package describes the mapping between transport addresses defined in the ITU-T H.248 local descriptor and the list position when used for STUN purposes. |
|  | Version: | 1 |
|  | Extends: | None |

{Contributor’s note (2014-11 meeting): the package property concerning "address correlations" is not impacted by [b-IETF ice-dualstack] in our understanding, - to be confirmed. If yes, then no changes to clause 7.1.}

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

## 7.2	MG STUN client package

|  | Package name: | MG STUN Client |
| --- | --- | --- |
|  | ### Package ID: | mgstunc (0x00be) |
|  | ### Description: | This package enables an MGC to determine the mapped IP address and port that will be routed back to the media component that sent the request. This package applies to both [IETF RFC 3489] and [IETF RFC 5389]. |
|  | ### Version: | 1 |
|  | ### Extends: | None |

### 7.2.1	Properties

#### 7.2.1.1	STUN address

{Contributor’s note (2014-11 meeting): the package property concerning "STUN address" is not impacted by [b-IETF ice-dualstack] in our understanding, - to be confirmed. If yes, then no changes to clause 7.2.}

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

# 8	ICE support

{Editor’s note (2012-09 meeting): this section is apparently structured along individual SDP elements related to ICE. Thus, there’s a prerequisite that the reader is fully familiar with ICE. Readability might be enhanced by introducing briefly the concerned SDP elements and/or provide a list or subclause structure. 
Contributions are solicited.}

The support of ICE implies that the SDP CANDIDATE attribute is supported. This can facilitate address gathering and description of candidates. However, due to the nature of the ITU-T H.248 connection model, several procedural changes are required.

Address gathering

The MGC may request an MG to perform address discovery through the use of the wildcarding mechanism on the CANDIDATE attribute. In this case the MG is responsible for STUN server discovery. Fully specifying the CANDIDATE attribute will not result in any action. The candidate-attribute is specified by the following ABNF [IETF RFC 5245]:

candidate-attribute = "candidate" ":" foundation SP component-id SP

transport SP

priority SP

connection-address SP     ;from [IETF RFC 4566]

port     ;port from [IETF RFC 4566]

SP cand-type

[SP rel-addr]

[SP rel-port]

*(SP extension-att-name SP

extension-att-value)

NOTE 1 – Clause 4.5/[IETF RFC 6544] defines an extended ABNF grammar for the SDP "a=candidate:" attribute for TCP.

NOTE 2 – The priority value is the result of a particular address prioritization algorithm (see clause 4.1.2.1/[IETF RFC 5245] for basic rules and [b-IETF ice-dualstack] rule related to ICE IPv4/IPv6 dual stack fairness).

The MGC shall either include values for, or wildcard mandatory fields. Including values provides a way of narrowing down the selection. For example:

If the MGC wanted a particular candidate type it could specify:

a=candidate:1 1 UDP  $ $ typ host

a=candidate:2 1 UDP  $ $ srflx raddr  $ rport $

and the host address and server reflexive address would be returned.

If the MGC wanted all candidate types it could specify:

a=candidate:$ $ $ $ $ $ typ *

and the local, server reflexive address and relay addresses would be returned.

If a request results in multiple candidates then these shall be returned in multiple "a=" lines.

As per clause 4.3 of [IETF RFC 5245], if the MG utilizes RTCP, the MGC and MG MUST encode the RTCP candidate using the "a=rtcp" attribute as defined in [IETF RFC 3605]. If RTCP is not in use, the MGC and MG MUST signal that by using "b=RS:0" and "b=RR:0" as defined in [IETF RFC 3556].

If the MGC requires a MG to allocate a port for RTCP, the MGC will send the required information to request an RTCP port (e.g., "a=" line in SDP, a=rtcp $).

A further complication is provided by the selection of the "In-Use candidate" in the SDP media (m=) and connection (c=) lines. In the ITU-T H.248 local descriptor this is a local transport address in the MG. Whereas in ICE this may be another address. Therefore, in order to construct an SDP offer, the MGC shall determine the "In-Use candidate" from the CANDIDATE attributes returned in the response.

The remote-candidate-att is related to the SDP (SIP) offer and provides the identity of the remote candidates that the offerer wishes the answerer to use in its answers. It is assumed that the MGC (acting as the offerer) will populate this attribute based on candidate information from the MG.

Similarly the ICE SDP "ice-pwd-att" attribute may be wildcarded or provided.

If the MGC asks for candidates, it should provide or ask for credentials in order for the MG to be able to perform the necessary signalling.

If MGC provides credentials, the MGC will send the required "a=" lines in SDP. For example:

a=ice-pwd:asd88fgpdd777uzjYhagZg

a=ice-ufrag:8hhY

If the MGC asks the MG to provide the credentials, the MGC will send the required "a=" lines in SDP. For example:

a=ice-pwd:$

a=ice-ufrag:$

If MGC does not supply the necessary "a=" lines in SDP, the MG may determine these credentials and return them to the MGC in a reply message. This is in order for the MG to be able to send a correct STUN message.

The use of the "a=ice-lite" attribute indicates whether or not the agent implements a lite version of ICE (see also clause 6.1.6 and Appendix III). As the agent resides in the MG this information needs to be provided to the MGC in order for the information to be communicated to a peer. Given the structure of the "a=ice-lite" attribute the MGC is unable to perform a CHOOSE wildcard operation on the attribute. As such where the MG implements a "lite" version of ICE it shall include the "a=ice-lite" attribute in any command response containing the candidate attribute.

{Editor’s note (2012-09 meeting): this clause is a placeholder (item #1) for providing use case details on ICE full and ICE lite and impact on H.248 entities. 
Contributions are solicited.}

Where a binary encoding of ITU-T H.248 is used, the ICE-related attributes may be realized through the use of [ITU-T H.248.1] Annex C.11 properties. In order to facilitate multiple values the "sub-list of" form should be used.

{Editor’s note (2012-09 meeting): this clause is a placeholder related to ICE control mode details (see item #2). 
Contributions are solicited.}

Connectivity checking

In order for an MG to perform some ICE operations, the remote candidate list is required. This remote candidate list can be provided to the MG in the form of candidate attributes being placed in the appropriate remote descriptors. The MGC may be required to reformat the SDP answer/offer received (i.e., via SIP) in order for it to be applicable to ITU-T H.248 structures. The setting of the candidate attributes in the remote descriptor will not in itself generate any ICE operation such as Connectivity Checking or keep alive mechanisms. ITU-T H.248 packages are used to trigger these operations.

[End Proposed Correction]

Change proposal #5:

[Begin Proposed Correction]

## 8.2	Originate STUN continuity check package

…

### 8.2.6	Procedures

#### 8.2.6.1	Connectivity check procedures for UDP

When the "Send Connectivity Check" (scc) signal is sent, the MGC should initiate the STUN continuity check procedures as outlined in clause 7 of [IETF RFC 5245]. As the ICE agent functionality is split between a MGC and MG and the MG is responsible for sending the connectivity checks, the agent role (controlled or controlling, see clause 5.2 of [IETF RFC 5245]) should be provided. In order for the MGC to obtain the result of the connectivity check, it shall set the "Connectivity Check Result" (ccr) event.

The usage of the "Send Connectivity Check" signal shall clear the results of any previous connectivity checking on the Termination/Stream.

If the ccr event is set and the scc signal is received, the MG shall apply the procedures of clause 7.1.2 of [IETF RFC 5245] to form and prioritize (NOTE 1) a checklist.

NOTE 1 – The calculation of the priority value follows basically clause 4.1.2.1/[IETF RFC 5245], though, there might be other policies such as [b-IETF ice-dualstack].

Once the procedures of clause 7.1.3 of [IETF RFC 5245] have been completed, the MG shall then notify the MGC of the result. The MG shall deem the procedures of clause 5.8 of [IETF RFC 5245] completed once all checks have reached the state "succeeded" or "failed". In order to uniquely identify the checks on the termination, the MG shall provide the StreamID, GroupID, foundation of the local candidate, foundation of the remote candidate and the component-id. If the control role of this candidate pair changes, an optional flag "Rc" is included in the end. The MG shall maintain the results of the checking for the lifetime of the Termination/Stream or until they are cleared by a subsequent "Send Connectivity Check" signal.

Where there is only one stream or group the respective ids default to 1.

If there are no successful candidate pairs for one of the particular component-ids in the list, then the connectivity check for that component-id has failed.

If there are more than one successful candidate pairs for a particular component-id in the list, only the candidate pair having the highest priority will be reported. The "Sub-list of" shall be in the same order as the checklist in order to maintain relative priority.

Depending on its connection role, "controlled" or "controlling", the MGC may use the result in the ObservedEventsDescriptor parameter ccr/ctp to determine which transport pair to use for the media connection.

If a new peer reflexive candidate is discovered as per the procedure in clause 7.1.3.2.1 of [IETF RFC 5245], and if the "New Peer Reflexive Candidate" (nprc) event is set, the MG will notify the MGC with the new peer reflexive candidate. In order to identify this peer reflexive candidate, the MG shall provide the StreamID, GroupID, Foundation, Component-id and the transport IP address and port number of this peer reflexive candidate. If the MGC requires that the peer reflexive candidate be paired with other remote candidates besides the one in the valid pair that will be generated, the MGC may generate an updated offer which includes the peer reflexive candidate. This will cause it to be paired with all other remote candidates. The list of the candidate pairs is updated in this case.

When a new peer reflexive candidate is discovered, the Connectivity Checks procedure continues. If the MGC updates the local candidates via the SDP in the ITU-T H.248 message, the MG will continue with any connectivity checks that are still in progress. It will not perform checks on the new candidates. The MGC shall send the "Send Additional Connectivity Check" (sacc) signal to perform checking of the new candidates.

In some kinds of conditions, where the list of the connectivity check candidate pairs is changed (i.e., a new stream is added, an existing stream is modified or a new peer reflexive candidate is discovered) additional connectivity check procedures may be initiated. The MGC may send the "Send Additional Connectivity Check" signal to initiate a connectivity check procedure on any candidate/transport pair not previously checked. In this procedure, the MG shall only perform checks not already performed. Another example is where a new stream is added after the connectivity checks for the existing stream have finished. In this case, the additional connectivity checks will only check the candidate pairs of the newly added stream.

#### 8.2.6.2	Connectivity check procedures for TCP

This package may be also applied for TCP based; the procedures of previous clause 8.2.6.1 are applicable under consideration of TCP specific aspects according to clause 7/[IETF RFC 6544].

[End Proposed Correction]

Change proposal #6:

[Begin Proposed Correction]

Bibliography

[b-ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control protocol: NAT-traversal for peer-to-peer services.

[b-ITU-T H.248.89]	Recommendation ITU-T H.248.89 (10/2014), Gateway control protocol: TCP support packages.

[b-ITU-T H.248.90]	Recommendation ITU-T H.248.90 (10/2014), Gateway control protocol: H.248 packages for control of transport security using TLS.

[b-ITU-T H.248.93]	Recommendation ITU-T H.248.93 (10/2014), Gateway control protocol: H.248 packages for control of transport security using DTLS.

[b-IETF RFC 4571]	IETF RFC 4571 (07/2006), Framing Real-time Transport Protocol (RTP) and RTP Control Protocol (RTCP) Packets over Connection-Oriented Transport.

[b-IETF RFC 5626]	IETF RFC 5626 (10/2009), Managing Client-Initiated Connections in the Session Initiation Protocol (SIP).

NOTE 1 – Two keep-alive techniques: CRLF keep-alive technique for TCP and SCTP, and STUN keep-alive technique for UDP.

[b-IETF RFC 6223]	IETF RFC 6223 (04/2011), Indication of Support for Keep-Alive.

{Editor’s note (2013-01 meeting): possible further “keep alive” reference:
“Indication of support for reverse keep-alive” 
(draft-holmberg-sipcore-rkeep). Contributions solicited.}

[b-IETF RFC 6724]	IETF RFC 6724 (09/2012), Default Address Selection for Internet Protocol Version 6 (IPv6).

[b-IETF ice-lite]	IETF draft-rescorla-mmusic-ice-lite (02/2007), Implementing Interactive Connectivity Establishment (ICE) in Lite Mode.
URL: http://tools.ietf.org/html/draft-rescorla-mmusic-ice-lite-00

[b-IETF ice-dualstack]	IETF draft-martinsen-mmusic-ice-dualstack-fairness (##/201#), ICE IPv4/IPv6 Dual Stack Fairness.

[End Proposed Correction]

Change proposal #7:

[Begin Proposed Correction]

[End Proposed Correction]

____________________