| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4619 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.77 (Rev): New srtp package version 2 - Additional key management scheme(s) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides an upgrade proposal of the srtp package in order to support key management for SRTP using DTLS-SRTP besides the existing support of SDES.

1	Introduction

[H.248.WEBRTC] provides requirements for DTLS-SRTP, see input draft AVD-4594:

### 8.6.2	Requirements related to media security

R-8.6.2/1: The H.248 WEBRTC gateway is required to support media security for RTP using SRTP, according to [ITU-T H.248.77].

{Editor’s note (2014-03 meeting): the specific H.248.77 use cases should be detailed …}

R-8.6.2/2: The H.248 WEBRTC gateway is required to support the media-path coupled SRTP key exchange (using DTLS-SRTP [IETF RFC 5763]) according to requirement [R-8.6.1/2].

R-8.6.2/3: The H.248 WEBRTC gateway can optionally support the signalling-path coupled SRTP key exchange, i.e., the "SDES" (SDP security descriptions) related method according to [ITU-T H.248.77].

It could be concluded that a H.248 WEBRTC gateway requires an H.248.77 based indication (by the MGC to the MG) concerning the applied key management for the SRTP media traffic.

2	Discussion

2.1	Multiple key management schemes for SRTP?

In general, there are a plethora of key management schemes for SRTP, see

IETF RFC 7202 (04/2014), Securing the RTP Framework: Why RTP Does Not Mandate a Single Media Security Solution.

Therefore it might be quite common that a particular H.248 media gateway (or H.248 profile) with SRTP support may need to handle in parallel multiple SRTP key management schemes.

2.2	Use cases for H.248 gateways

Example A: SRTP-based IP applications for

- WEBRTC calls with DTLS-SRTP based key management and

- non-WEBRTC calls (e.g., NGN or IMS VoIP calls) with SDES based key management.

or even

Example B: SRTP-based IP applications for

- WEBRTC calls with DTLS-SRTP based key management (due to IETF default)

- WEBRTC calls with SDES based key management (as optional agreement with WEBRTC client) and

- non-WEBRTC calls (e.g., NGN or IMS VoIP calls) with SDES based key management.

Observations:

There are multiple combinations already in case of two key management schemes in parallel. The MGC must be able to call-individual indicated the key management scheme in use.

2.3	H.248 signalling

There are basically two methods (as already discussed for multiple H.248 packages in the past), either

- H.248 Property-based, using H.248.77 srtp/km codepoints as an explicit, direct and unambiguous signalling method, or

- SDP-based or any other signalling elements.

The 2nd approach isn't further evaluated here, primarily because

- there are multiple SDP codepoints for the indication of a correspondent transport protocol stack (RFC 5764);

- H.248 descriptor level: TerminationState required, but no LD/RD level; and

- the general issue in overloading semantics of other signalling elements.

The 1st approach looks fairly straightforward by just adding explicit codepoint for further key management schemes. The srtp/km codepoint would then "profile" the set of relevant H.248.77 srtp package procedures.

3	Conclusion

The codepoint space for the indication of SRTP key management schemes should be extended. Another SRTP key management (than SDES) results in slightly different H.248 procedures, plus eventually additional package elements. Thus, it is yet unclear whether

- the upversioning of the srtp package?,

- the extension of the srtp package? or

- new package(s) (per key management scheme)?

should be considered for H.248.77.

This contribution provides an initial proposal for a skeleton with regards to DTLS-SRTP support only.

4	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against published Recommendation H.248.77 (09/2010) http://www.itu.int/rec/T-REC-H.248.77-201009-I/en .

Change proposal #1:

[Begin Proposed Correction]

# 1	Scope

The secure real-time transport protocol (SRTP) is an RTP profile that provides confidentiality, message authentication and replay protection to RTP and RTCP sessions. The secure RTP package allows a MGC to control the use of SRTP by a MG. This package is defined in detail in clause 6.

By itself, the secure RTP package is incomplete, as it does not provide procedures for key management. Instead, it is designed to rely on existing key-management schemes. Clause 7 provides procedures for the use of one such key-management scheme: SDP security descriptions.

Several reasons exist why this Recommendation is required, in addition to the existing (usually SDP-based) SRTP key-management schemes. The most significant of which are listed below:

–	Most existing SDP key-management schemes rely on the SDP offer/answer model (see [b-IETF RFC 3264]). However, the offer/answer model is not used in ITU-T H.248 as it does not fit the nature of the connection between an ITU-T H.248 MGC and a MG.

–	Existing SDP key-management schemes do not contain procedures relating to parameter overspecification and wildcarding, which are unique to ITU-T H.248.

–	The limited lifetime of SRTP master keys calls for mechanisms for handling master key expiry. The existing mechanisms cannot be used in ITU-T H.248.

–	The SRTP package allows explicit control over the key-management scheme employed, allowing easy interoperability with, and migration to future schemes.

–	The SRTP package allows an MGC to audit the SRTP capabilities of an MG through the use of the packages descriptor and the properties of the new package.

–	The SRTP package allows an MGC to collect statistics regarding the number of security violations encountered by the MG, and the volume of SRTP traffic it processed.

The scope of the (09/2010) release of this Recommendation is limited to use-cases in which a MG applies SRTP procedures, as described in clause 3.3 of [IETF RFC 3711], to the SRTP packets it sends and receives. Use-cases in which the MG handles SRTP packets without using those procedures (e.g., transparent forwarding, storage in encrypted form, etc.) are intentionally left out of this Recommendation.

Release (##/2015) extends the scope of further key management schemes for SRTP, such as DTLS-SRTP according to [b-IETF RFC 5763] and [IETF RFC 5764].

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2005), Gateway control protocol: Version 3, including its Amendment 1 (2008) and Amendment 2 (2009).

[ITU-T H.248.8]	Recommendation ITU-T H.248.8 (2007), Gateway control protocol: Error code and service change reason description.

[ITU-T H.248.47]	Recommendation ITU-T H.248.47 (2008), Gateway control protocol: Statistic conditional reporting package.

[ITU-T H.248.49]	Recommendation ITU-T H.248.49 (2007), Gateway control protocol: Session description protocol RFC and capabilities packages.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 3711]	IETF RFC 3711 (2004), The Secure Real-time Transport Protocol (SRTP).

[IETF RFC 4568]	IETF RFC 4568 (2006), Session Description Protocol (SDP) Security Descriptions for Media Streams.

[IETF RFC 5764]	IETF RFC 5764 (2010), Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP).

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

AES	Advanced Encryption Standard

DTLS	Datagram Transport Layer Security

HMAC	Keyed-Hash Message Authentication Code

IP	Internet Protocol

IPSec	IP Security

ISDN	Integrated Services Digital Network

L1	Layer 1 (of the Open Systems Interconnection model – the physical layer)

L2	Layer 2 (of the Open Systems Interconnection model – the data link layer)

MG	Media Gateway

MGC	Media Gateway Controller

MKI	Master Key Identifier

NAPT	Network Address and Port Translation

NAT	Network Address Translation

PSTN	Public Switched Telephone Network

ROC	Rollover Counter

RTCP	RTP Control Protocol

RTP	Real-time Transport Protocol

SDP	Session Description Protocol

SHA1	Secure Hash Algorithm 1

SRTCP	Secure RTCP

SRTP	Secure RTP

SSRC	Synchronization Source

TDM	Time Division Multiplexing

UDP	User Datagram Protocol

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

# 6	Secure RTP package

Package Name:	Secure RTP

Package ID:	srtp (0x0107)

Description:	This package defines elements that allow the MGC to control an MG's use of the SRTP profile.
Version 2 adds support for DTLS-SRTP [IETF RFC 5764].

Version:	2

Extends:	None

## 6.1	Properties

### 6.1.1	Supported Encryption Transforms

Property Name:	Supported Encryption Transforms

Property ID:	set (0x0001)

Description:	This property declares the set of encryption transforms that can be used by SRTP sessions.

Type:	Sub-list of Enumeration

Possible values:	Each item in the list can be one of:

"NULL" (0x0000): The NULL Cipher

"AES_CM_128" (0x0001): AES in counter mode with a 128-bit key

"AES_CM_192" (0x0002): AES in counter mode with a 192-bit key

"AES_CM_256" (0x0003): AES in counter mode with a 256-bit key

"AES_F8_128" (0x0004): AES in f8 mode with a 128-bit key

"AES_F8_192" (0x0005): AES in f8 mode with a 192-bit key

"AES_F8_256" (0x0006): AES in f8 mode with a 256-bit key

Default:	Provisioned

Defined in:	TerminationState

Characteristics:	ReadOnly

### 6.1.2	Supported Authentication Transforms

…

### 6.1.3	Key Management Scheme

Property Name:	Key Management Scheme

Property ID:	km (0x0003)

Description:	This property controls the key management scheme that will be used for supplying the SRTP parameters and keys

Type:	Enumeration

Possible values:	"None" (0x0000): No key management will be used.

"SDES" (0x0001): SDP security descriptions [IETF RFC 4568]

"DTLS-SRTP" (0x0002): DTLS-SRTP [IETF RFC 5764]

Default:	"None", unless provisioned otherwise

Defined in:	TerminationState

Characteristics:	Read/Write

### 6.1.4	Key Lifetime Expiry Behaviour

…

## 6.2	Events

…

{Editor's note (2014-11): any impact on events by DTLS-SRTP based key management?}

## 6.3	Signals

None.

## 6.4	Statistics

…

{Editor's note (2014-11): new statistics for DTLS-SRTP based key management?}

## 6.5	Error codes

None.

## 6.6	Procedures

...

{Editor's note (2014-11): any impact on procedures by DTLS-SRTP based key management? For instance: 
procedure: 	IF srtp/km = "DTLS-SRTP 
	THEN srtp/set = only {NULL, AES_CM_128} }

[End Proposed Correction]

Change proposal #5:

[Begin Proposed Correction]

Bibliography

[b-IETF RFC 3264]	IETF RFC 3264 (2002), An Offer/Answer Model with Session Description Protocol (SDP).

[b-IETF RFC 4567]	IETF RFC 4567 (2006), Key Management Extensions for Session Description Protocol (SDP) and Real Time Streaming Protocol (RTSP).

[b-IETF RFC 5763]	IETF RFC 5763 (05/2010), Framework for Establishing a Secure Real-time Transport Protocol (SRTP) Security Context Using Datagram Transport Layer Security (DTLS).

[End Proposed Correction]

____________________