| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4622 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | H.248.78: XMPP and MSRP interworking |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This Contribution proposes the addition of an annex to H.248.78 for the purpose of describing the interworking between XMPP and MSRP on decomposed gateways utilising H.248.

2.	Discussion

The IETF “SIP to XMPP” (STOX) working group has been developing a series of documents describing interworking between SIP and XMPP. A focus of the work has been interworking the messaging aspects of XMPP and SIP.

An issue for consideration is the support of XMPP via H.248. The STOX chat draft [draft-ietf-stox-chat] describes the interworking between SIP/MSRP [IETF RFC4975] and XMPP [IETF RFC6121] for ‘chat’. However this draft does not take into consideration an architecture where a decomposed XMPP-to-MSRP gateway is utilised.

One important aspect of the support of XMPP and SIP/MSRP interworking is in which element (MGC or MG) the respective protocols are terminated. From RFC7081” CUSAX: Combined Use of the Session Initiation Protocol (SIP) and the Extensible Messaging and Presence Protocol (XMPP)” it can be seen that XMPP is basically a peer protocol to SIP. The SIP protocol is terminated at a MGC. As such it would seem logical to terminate the XMPP protocol at the MGC also. With respect to MSRP ITU-T H.248.78 already specifies that MSRP is a bearer level application protocol and is thus terminated at an MG. Therefore the chat messaging information received at the MGC must be communicated to the MG and vice versa.

There are basically two options.

- Terminate the XMPP signalling at the MGC and apply the XMPP to MSRP interworking in the MGC and send MSRP messages over H.248.

- Terminate the XMPP signalling at the MG and apply the XMPP to MSRP interworking in the MG and send XMPP messages over H.248 to the MGC to trigger call control messages (i.e. SIP) .

The use of option 2) presents a number of co-ordination issues. The main issue is that XMPP combines the messaging with ‘state’ information whereas both SIP and MSRP must be used to achieve the same functionality. Clause 4/[draft-ietf-stox-chat] provides an example interworking. It can be seen that a single XMPP message results in both SIP messages and MSRP messages being sent. There is also XMPP information that needs to be used in SIP messages (see Table 1/ [draft-ietf-stox-chat]). Terminating XMPP messages in the MG would lead to complications whereby the MG would have to co-ordinate with the MGC to initiate the SIP signalling. This is not seen as practical due to the extra signalling that would be required.

Option 1) presents a more practical approach whereby the application interworking logic is maintained at the MG. The XMPP is able to initiate SIP messages with a minimal interaction with the MG. The MGC is also able to utilise an existing solution as defined by H.248.78 for the transport of bearer level signalling protocols. The signalling of MSRP is already described by H.248.78.

As such it is proposed to add an Annex to H.248.78 documenting the interworking of XMPP and SIP/MSRP where a decomposed H.248 gateway is used.

3.	Proposal

It is proposed to accept the following changes. The changes are marked up against TD 148r1/WP1 from the Sapporo, July Q3/16 Rapporteurs meeting. For readability the proposed new Annex A is not marked up.

References:

[draft-ietf-stox-chat]	http://tools.ietf.org/html/draft-ietf-stox-chat-08

Draft revised Recommendation ITU-T H.248.78

Gateway control protocol: Bearer-level application level gateway

AAP Summary

Recommendation ITU-T H.248.78 defines packages that allow the support of bearer-level application gateway (B-ALG) functions in various service configurations. A B-ALG belongs to the category of protocol layer 4+ NAT traversal support functions. This Revision adds a further H.248 package which allows a particular B-ALG type, entirely resident in the media gateway itself. It also adds an annex that describes Extensible Messaging and Presence Protocol (XMPP) to MSRP interworking utilising mechanisms defined in this Recommendation.

Summary

Recommendation ITU-T H.248.78 defines a package that allows bearer-level application protocol (such as real time streaming protocol (RTSP) or message session relay protocol (MSRP)) messages received by a media gateway (MG) to be communicated to a media gateway controller (MGC). This allows the MGC to perform application-level gateway functionality such as network address translation on the messages and then forward the messages to the MG for sending to the destination.

The (03/2013) revision incorporates an update to correctly refer to the "Message Session Relay Protocol".

This revision of Recommendation ITU-T H.248.78 adds a further H.248 package for support of MG located application-level gateway functionality. It also adds an annex that describes Extensible Messaging and Presence Protocol (XMPP) to MSRP interworking utilising mechanisms defined in this Recommendation.

# 1	Scope

In a split media gateway controller (MGC) and media gateway (MG) environment, call-level application level gateways (ALG) are common in order to translate address information (i.e., network address and port translation (NAPT)) between two signalling domains. Typically, the application signalling traverses the MGC. ITU-T H.248 is therefore not involved in these translations other than to set the necessary media transport parameters on applicable terminations.

Increasingly bearer-level application control signalling is being used which instead traverses the MG. This bearer-level application signalling typically is used within a session that has been established by the MGC; thus, some coordination between the two levels is needed.

This Recommendation provides functionality that allows the MGC and MG to coordinate bearer-level application level gateway functionality.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.69]	Recommendation ITU-T H.248.69 (2009), Gateway control protocol: Packages for interworking between MSRP and H.248.

[IETF RFC 2326]	IETF RFC 2326 (1998), Real Time Streaming Protocol (RTSP).

[IETF RFC 3261]	IETF RFC 3261 (2002), SIP: Session Initiation Protocol.

[IETF RFC 4975]	IETF RFC 4975 (2007), The Message Session Relay Protocol (MSRP).

[IETF RFC 6121]	IETF RFC 6121 (2011), Extensible Messaging and Presence Protocol (XMPP): Instant Messaging and Presence.

[IETF RFC 7247]	IETF RFC 7247 (2014), Interworking between the Session Initiation Protocol (SIP) and the Extensible Messaging and Presence Protocol (XMPP): Architecture, Addresses, and Error Handling.

[IETF RFC 7248]	IETF RFC 7248 (2014), Interworking between the Session Initiation Protocol (SIP) and the Extensible Messaging and Presence Protocol (XMPP): Presence.

[IETF STOX CHAT]	IETF draft-ietf-stox-chat-08 (xxxx), Interworking between the Session Initiation Protocol (SIP) and the Extensible Messaging and Presence Protocol (XMPP): One-to-One Text Chat Sessions).

[IETF STOX GROUPCHAT] IETF draft-ietf-stox-groupchat-05 (xxxx), Interworking between the Session Initiation Protocol (SIP) and the Extensible Messaging and Presence Protocol (XMPP): Groupchat.

[IETF STOX IM]	IETF draft-ietf-stox-im-10 (xxxx), Interworking between the Session Initiation Protocol (SIP) and the Extensible Messaging and Presence Protocol (XMPP): Instant Messaging.

[IETF STOX MEDIA]	draft-ietf-stox-media-04 (xxxx), Interworking between the Session Initiation Protocol (SIP) and the Extensible Messaging and Presence Protocol (XMPP): Media Sessions.

[ETSI TS 123 228]	ETSI TS 123 228 (2010), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; IP Multimedia Subsystem (IMS); Stage 2 (3GPP TS 23.228 version 9.4.0 Release 9).

…

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| ALG | Application-Level Gateway |
| --- | --- |
| B2B | Back to Back |
| B-ALG | Bearer Level Application-Level Gateway |
| DNS | Domain Name System |
| GW | Gateway |
| HTML | Hypertext Mark-up Language |
| HTTP | Hypertext Transfer Protocol |
| IP | Internet Protocol |
| IPv4 | Internet Protocol Version 4 |
| IPv6 | Internet Protocol Version 6 |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MSRP | Message Session Relay Protocol |
| NAPT | Network Address and Port Translation |
| OSI | Open Systems Interconnection |
| RTSP | Real-Time Streaming Protocol |
| S2X | SIP to XMPP |
| SCTP | Stream Control Transport Protocol |
| SDP | Session Description Protocol |
| SIP | Session Initiation Protocol |
| TCP | Transmission Control Protocol |
| UDP | User Datagram Protocol |
| X2S | XMPP to SIP |
| XMPP | Extensible Messaging and Presence Protocol |

Annex A

XMPP to MSRP interworking utilising ITU-T H.248.78

# A.1 Scope

This Annex describes the interworking between the Extensible Messaging and Presence Protocol (XMPP) [IETF RFC6121] and the Message Session Relay Protocol (MSRP) [IETF RFC4975].

[IETF RFC 7247] provides an architecture for mapping between XMPP and SIP [IETF RFC3261] and its associated messaging extensions. It defines two types of logical gateway functions: a “XMPP-to-SIP gateway” and  a “SIP-to-XMPP gateway” that covers protocol translation in terms of flow from a XMPP based system to a SIP based system and vice versa. The RFC indicates that these logical functions could occur in one physical entity. The RFC considers the gateway functions as monolithic entities and does not describe situations where the gateway functions could be decomposed into a MGC and MG. This Annex provides additional information on the interwork when the gateway functions are decomposed.

In addition to [IETF RFC 7247] several RFCs describe in further detail the interwork of different functionalities offered by SIP and XMPP. These include:

- Presence [IETF RFC 7248]

- Instant Messaging [IETF STOX IM]

- One-to-One Text Chat Sessions [IETF STOX CHAT]

- Groupchat [IETF STOX GROUPCHAT]

- Media Sessions [IETF STOX MEDIA]

When considering the allocation of functionality in a decomposed gateway typically control (e.g. call control) functionality is allocated to a MGC and bearer/media related functionality is allocated to a MG.

As [IETF RFC 7248], [IETF STOX IM] are only related to SIP and XMPP messages the interworking described in those RFCs is contained in the MGC. No involvement of the MG is necessary thus no interaction with H.248 is needed. These RFCs are not considered further by this Annex.

[IETF STOX MEDIA] discusses interwork between SIP and XMPP when they are used to establish media flows. This interwork is relevant to decomposed gateways as the XMPP and SIP signalling is processed at the MGC and the media is processed on the MG. The RFC describes the mapping of XMPP to SIP and SDP (which is used to describe the characteristics of the media). As H.248 uses SDP to describe the characteristics of the media the SDP defined in the RFC can be used to derive the H.248 messages. Whilst the XMPP and SIP messages will result in the need for H.248 signalling to establish and release media this Annex does not describe these message flows.

[IETF STOX CHAT] and [IETF STOX GROUPCHAT] describes the interwork between XMPP and SIP where MSRP is used for messaging in the SIP network. MSRP is considered an application level bearer protocol and thus may be terminated at an MG. Therefore there is interworking between XMPP and SIP/MSRP involving both the MGC and MG. Thus there is an interaction with H.248. This Annex further describes this interaction.

# A.2 Architectural Assumptions

Figure 1 / [IETF RFC 7247] provides a possible gateway deployment architecture showing the SIP to XMPP gateway (S2X GW) and the XMPP to SIP gateway (X2S GW) being implemented in a monolithic way.

Figure 1 below shows an alternate architecture where the gateway functions for XMPP to SIP/MSRP (and vice-versa) interworking are implemented in a decomposed H.248 controlled gateway.

Figure 1 – XMPP to SIP/MSRP interworking architecture.

The S2X GW and X2S GW encompass the procedures as described by [IETF STOX CHAT]. The MGC provides interworking between SIP and XMPP messages. As a result of the interworking the MGC establishes a MSRP bearer on the MG. MSRP messages are carried across the H.248 interface. The “MGC Controlled Bearer Level ALG” (mcbalg) package in clause 7 is used to transport the MSRP messages between the MGC and MG and vice versa.

# A.3 Procedures

[IETF STOX CHAT] provides procedures for mapping  XMPP chat sessions using message stanzas of type "chat" as  specified in [IETF RFC 6121] and  MSRP chat sessions using the SIP INVITE and SEND request types as specified in [IETF RFC 4975]. In order to interwork XMPP and SIP/MSRP the MGC shall follow the procedures defined in [IETF STOX CHAT] with the following additions:

- When the MGC receives either:

  - a SIP INVITE with SDP indicating a media type “message” and protocol “TCP/MSRP”, or

  - a XMPP message with type ‘chat’ that results in the sending of an SIP INVITE with SDP indicating a media type “message” and a protocol indicating the use of MSRP (e.g. TCP/MSRP).

It shall add a Termination/Stream to a Context with sufficient information in the Local/Remote Descriptors to initiate a MSRP bearer connection.

- To allow MSRP messages to be received from the MG the MGC shall set the “Detect bearer level message” (mcbalg/det) Event with a protocol filter parameter indicating that MSRP shall be detected and reported. The message filter parameter should be set to “*” so that all MSRP messages are reported.

- If the MGC receives an XMPP message with type ‘chat’ and once the MSRP bearer has been established, the MGC shall use the “Send bearer level message” (mcbalg/sblm) Signal to send MSRP messages (resulting from the MGC applying the [IETF STOX CHAT] interworking procedures) to the MG. The MG then forwards the MSRP messages across the MSRP bearer connection.

- The MG for MSRP messages received on the MSRP bearer, shall forward the message via the Detect Bearer level message” (mcbalg/sblm) ObservedEvent to the MGC. The MGC shall then apply the interworking procedures from [IETF STOX CHAT] to produce an appropriate XMPP message.

- The MGC on reception of a SIP BYE or a XMPP message indicating a gone chat state which results in the sending of a SIP BYE, shall remove the MSRP Termination/Stream.

# A.4 Example

The following example is based on the order of events are described by figure 1/[IETF STOX CHAT]. It describes additional events (H1 to H12) when the interworking function is decomposed across a H.248 media gateway. In the example MSRP is transported over a TCP bearer connection. The method for carrying MSRP messages via H.248 is independent of what transport protocol MSRP utilises. A flow for clause 5/[IETF STOX CHAT] is not detailed as the same basic H.248 messages and method of transporting MSRP messages across H.248 is used.

## A.4.1 XMPP to MSRP

Figure 2 – XMPP to MSRP order of events in a H.248 controlled gateway

The messages related to F1 to F16 are as per clause 4/[IETF STOX CHAT].

On reception of the XMPP message (F2) the MGC determines that a H.248 Termination/Stream needs to be instantiated on the MG. The MGC sends a H.248 Add.req (H1) requesting that a bearer be added for a MSRP media stream. The MGC requests local address information (e.g. IP Address and Port):

MGC to MG:

MEGACO/3 [123.123.123.4]:55555

Transaction = 10000 {

Context = $ {

Add = T$ {

Media {

Stream = 1 {

Local {

v=0

c=IN IP4 $

m=message $ TCP/MSRP *

a=accept-types:text/plain

a=path:msrp://s2x.example.net:12763/kjhd37s2s20w2a;tcp

}

}

}

}

}

}

The address information is returned via an Add.rep (H2).

MEGACO/3 [125.125.125.111]:55555

Reply = 10000 {

Context = 1234 {

Add=A4445{

Media {

Stream = 1 {

Local {

v=0

c=IN IP4 x2s.example.com

m=message 7654 TCP/MSRP *

a=accept-types:text/plain

a=path:msrp://s2x.example.net:12763/kjhd37s2s20w2a;tcp

}

}

}

The address information is used by the MGC to construct the SIP INVITE (F3).

On reception of the SIP 200 OK (F6) the MGC uses the provided address information and sends a H.248 Modify.req to the MG to update the Remote Descriptor with the far end transport address. In the Modify the MGC also includes a request to establish the underlying MSRP transport as well as setting events related to the underlying transport (e.g. to detect when the bearer is established). It also includes the “Detect bearer level message” (mcbalg/det) Event with a protocol filter parameter indicating that MSRP is to be detected and reported:

MGC to MG:

MEGACO/3 [123.123.123.4]:55555

Transaction = 10001 {

Context = 1234 {

Modify = T1 {

Media {

Stream = 1 {

Remote {

v=0

c=IN IP4 s2x.example.net

m=message 12763 TCP/MSRP *

a=accept-types:text/plain

a=path:msrp://s2x.example.net:12763/kjhd37s2s20w2a;tcp

}

}

},

Events = 2223 {

mcbalg/det{pf=2855},

tcpbcc/BNCChange{type=[Est,Rel]}

},

Signals {

tcpbcc/EstBNC

}

}

}

}

The MG then responds with a H.248 Modify.rep (H4) and the MSRP bearer begins establishment.

On completion of the MSRP bearer establishment the MG notifies the MGC that bearer establishment has completed via a H.248 Notify.req (H5). The MGC acknowledges the receipt of the message via a H.248 Notify.rep (H6).

As the MSRP bearer has been established the MGC forwards the MSRP SEND message derived from the XMPP message (F2) to the MG via a H.248 Modify.req with the “Send bearer level message” (mcbalg/sblm) Signal with the MSRP SEND message as the message content parameter value:

MGC to MG:

MEGACO/3 [123.123.123.4]:55555

Transaction = 10002 {

Context = 1234 {

Modify = T1 {

Signals {

mcbalg/sblm{stream=1,mc=

"MSRP a786hjs2 SEND

From-Path: msrp://x2s.example.com:7654/jshA7weztas;tcp

To-Path: msrp://s2x.example.net:12763/kjhd37s2s20w2a;tcp

Message-ID: 54C6F4F1-A39C-47D6-8718-FA65B3D0414A

Byte-Range: 1-25/25

Content-Type: text/plain

Art thou not Romeo, and a Montague?

-------a786hjs2$

"}

}

}

}

The MG then sends the MSRP SEND message to the user (F9). The MG acknowledges the receipt of the Modify command via a H.248 Modify.rep.

On receipt of a MSRP SEND (F10) from the user, the MG forwards the MSRP SEND message to the MGC via a H.248 Notify.req (H9) with a Detect Bearer level message” (mcbalg/sblm) ObservedEvent. The message content parameter contains the detected MSRP message:

MEGACO/3 [125.125.125.111]:55555

Transaction = 10003 {

Context = 1234 {

Notify = T1 {ObservedEvents =2223 {

mcbalg/det{ stream=1, mc=

"MSRP di2fs53v SEND

To-Path: msrp://x2s.example.com:7654/jshA7weztas;tcp

From-Path: msrp://s2x.example.net:12763/kjhd37s2s20w2a;tcp

Message-ID: 6480C096-937A-46E7-BF9D-1353706B60AA

Byte-Range: 1-25/25

Failure-Report: no

Content-Type: text/plain

Neither, fair saint, if either thee dislike.

-------di2fs53v$"}}

}

}

}

The MGC acknowledges the receipt of the message via a H.248 Notify.rep (H10). The MGC then uses the received MSRP message and maps it to an appropriate XMPP message (F11).

The MGC on the receipt of a SIP BYE (F14) will remove the MSRP Termination/Stream via a H.248 Subtract.req (H11). This will have the effect of removing the MSRP bearer. The MG will acknowledge the removal via a H.248 Subtract.rep.

___________________