| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4623 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | H.248.STGROUP: Updates |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This Contribution proposes updates to H.248.STGROUP to address a number of open editor’s notes.

2.	Discussion

Draft Recommendation H.248.STGROUP after the Sapporo Q3/16 meeting appears to technically stable. There are however a number of editor’s notes that need to be addressed before the draft Recommendation is ready for Consent. This contribution proposes a number of updates to address the editor’s notes as well as items highlighted that need further text.

The individual changes are discussed in editor’s notes below.

3.	Proposal

It is proposed to accept the following changes. The changes are marked up against TD 181/WP1 from the Sapporo, July Q3/16 Rapporteurs meeting.

Draft new Recommendation ITU-T H.248.xx (ex H.248.STGROUP)

Gateway Control Protocol: 
H.248 Stream grouping and aggregation

AAP Summary

This Recommendation discusses how H.248 streams may be grouped according to a particular semantic and how if necessary these grouped source streams may be aggregated into a single multiplexed transport stream. It defines a H.248 package that allows an explicit grouping semantic to be assigned to a group of streams. It also introduces the concept and support for a new type of H.248 stream, the “aggregation stream”. This allows H.248 to better support multiple media streams.

Summary

This Recommendation discusses how H.248 streams may be grouped according to a particular semantic and how if necessary these grouped source streams may be aggregated into a single multiplexed transport stream. It defines a H.248 package that allows an explicit grouping semantic to be assigned to a group of streams. It also introduces the concept and support for a new type of H.248 stream, the “aggregation stream”.

The ability to group streams and to provide an explicit semantic allows the media gateway controller to control the processing of multiple streams. The aggregation stream allows  existing H.248 Descriptors to be applied to source streams as well as the aggregated stream. That means Signals, Events and Statistics may be applied to aggregated stream with benefits including for example: the ability to report cumulative Statistics rather than multiple individual statistics, report a single Event rather multiple events on the source streams. The ability to group streams and to provide an explicit semantic

# 1	Scope

As described by the H.248 connection model (clause 6/[ITU-T H.248.1) a termination sources and/or sinks one or more streams. In a multimedia conference, a termination can be multimedia and sources or sinks multiple media streams. The H.248 protocol uses the stream concept to model individual media flows. That is typically there is one media (and optional control flow) per H.248 stream. The characteristics of each stream is described through the use of the local control, local and remote descriptors. Each termination contains a group of streams. H.248 makes no assumptions as to the relationship between the streams on a termination other than indicating that streams exiting a termination shall be synchronised with each other (clause 7.1.4/ITU-T H.248.1).

Certain network technologies may require a media gateway (MG) to have knowledge of how two or more streams relate to each other beyond simply knowing that they shall be synchronised. Where there is a multiplicity of streams there may be a different relation between different sets of streams. For the purposes of this Recommendation the relation between streams is defined as the “grouping semantic”. Depending on the grouping semantic the MG may be required to apply certain processing to the media related to the streams. For example: one semantic may indicate that the media is to be duplicated on separate streams, another semantic may indicate media is only sent on one of two streams at a time. Some grouping semantics require that media flows from multiple streams are aggregated into a single flow.

H.248.1 does not support a means to describe grouping semantics. This Recommendation defines the “media grouping package” that allows a media gateway controller (MGC)  to group media from two or more H.248 streams according to a particular semantic and optionally to aggregate the media from the grouped streams according to the semantic.

The grouping framework in [IETF RFC5888] can in principle be applied to any transport. It is the actual grouping stream semantics that imposes limitations. i.e. BUNDLE may only be applied to IP transports. As such the “media grouping package” and the concept of a H.248 aggregation stream can be in principle applied to any transport. The majority of defined stream semantics focus on based transports. As such this Recommendation describes the semantics with respect to IP. Whilst some semantics may be used for any type of transport the use of the semantics for these transport are not covered in this Recommendation.

{Contributor’s note: The grouping framework in RFC5888 can in principle be applied to any transport. It is the actual grouping semantic that imposes limitations. i.e. BUNDLE may only be applied to IP transports. The mgroup package and the concept of an aggregation stream can in principle be applied to any transport. Again it is the semantic that drives the behaviour. It is proposed to capture this information in the text and delete the editor’s note.}

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.7]	Recommendation ITU-T H.248.7 (2004), Gateway Control Protocol: Generic Announcement package.

[ITU-T H.248.47]	Recommendation ITU-T H.248.47 (2008), Gateway Control Protocol: Statistic conditional reporting package.

[ITU-T H.248.58]	Recommendation ITU-T H.248.58 (2008), Gateway Control Protocol: Packages for application level H.248 statistics.

[ITU-T H.248.61]	Recommendation ITU-T H.248.61 (2013), Gateway Control Protocol: Packages for network level ITU-T H.248 statistics.

[ITU-T H.248.65]	Recommendation ITU-T H.248.65 (2009), Gateway control protocol: Support of the resource reservation protocol.

[IETF RFC 3524]	IETF RFC 3524 (2003), Mapping of Media Streams to Resource Reservation Flows.

[IETF RFC 4566]	IETF RFC 4566 (2006), SDP: Session Description Protocol.

[IETF RFC 4588]	IETF RFC 4588 (2006), RTP Retransmission Payload Format.

[IETF RFC 5583]	IETF RFC 5583 (2009), Signaling Media Decoding Dependency in the Session Description Protocol (SDP).

[IETF RFC 5888]	IETF RFC 5888 (2010), The Session Description Protocol (SDP) Grouping Framework.

[IETF RFC 5956]	IETF RFC 5956 (2010), Forward Error Correction Grouping Semantics in the Session Description Protocol.

[IETF draft-ietf-mmusic-sdp-bundle-negotiation]	IETF draft-ietf-mmusic-sdp-bundle-negotiation (201X), Negotiating Media Multiplexing Using the Session Description Protocol (SDP).

[IETF GRPTAX]	IETF draft-ietf-avtext-rtp-grouping-taxonomy (201X), A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources.

[IETF RTPMUX]	IETF draft-ietf-avtcore-rtp-multi-stream (201X), Sending Multiple Media Streams in a Single RTP Session.

{Contributor’s note: The title is correct. The note can be deleted.}

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1		media sink [IETF GRPTAX]: The Media Sink receives a Source Stream (Section 2.1.5) that contains, usually periodically, sampled media data together with associated synchronization information.  Depending on application, this Source Stream then needs to be transformed into a Raw Stream (Section 2.1.3) that is sent in synchronization with the output from other Media Sinks to a Media Render (Section 2.1.29).  The media sink  may also be connected with a Media Source (Section 2.1.4) and be used as part of a conceptual Media Source.

3.1.2		media source [IETF GRPTAX]: A Media Source is the logical source of a reference clock synchronized, time progressing, digital media stream, called a Source Stream.  This transformation takes one or more Raw Streams and provides a Source Stream as output.  This output has been synchronized with some reference clock, even if just a system local wall clock.

3.1.4		Stream [ITU-T H.248.1]: Bidirectional media or control flow received/sent by a media gateway as part of a call or conference.

3.1.5		Stream Semantics: [IETF RFC 5888] the purpose for which the media streams are grouped.

{Contributor’s note: It is proposed to include the above definitions for the above terms.}

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Aggregation: The MG internal process of combining two or more source streams on entering the H.248 context in the MG into one single stream, called aggregation stream. The way the source streams are combined is dependent on the grouping stream semantics defined for the source streams. It may consist of selecting one of the source streams, applying a reparation procedure to one of the source streams based on the information from another source stream, etc.

3.2.2	Aggregation stream: a stream that sources media to two or more streams and/ or sinks media from two or more streams which are related to each other with a certain grouping semantics. The aggregation stream exists only in the interior of the context. Within one context a stream may be defined as an aggregation stream in one termination and as a normal stream in another termination.

3.2.3	Deaggregation: the inverse process of  aggregation. The way source streams are generated from an aggregation stream is dependent of the grouping semantics. It may consist on delivering media to only one of them at a time, providing a replica, generating repairing or redundancy information, etc.

3.2.4	Group: Streams that share the same stream semantic are part of the same group. A stream may be part of several groups. Whether the streams in a group are aggregated or not depends on the stream semantic. [IETF RFC 5888] is utilised to group the streams and to provide the semantics.

3.2.5	Multiplexed stream: A transport stream external to the MG where multiple media streams are combined into one transport stream.

3.2.6	Source stream: each one of the streams external to the context that sources media to an aggregation stream when entering the context or which receives media from the aggregation stream when leaving the context. Also, each of the streams defined as belonging to a media group.

# {Contributor’s note: It is proposed to adopt these definitions to address the editor’s note in clause 5.}

…

# 5	Conventions

This Recommendation uses the terms “Grouping”, “Aggregation” and “Multiplexing”. As per the definitions:

- Grouping describes the process using H.248 to group streams together that share a common stream semantic.

- Aggregation describes the process of using H.248 to indicate that the media of several streams is aggregated in a single stream.

- Multiplexing is the process used by the transport to carry the aggregated streams.

{Editor’s note: Further work needs to occur on the definitions of / conventions for the terms for aggregation / grouping and multiplexing.}

{Contributor’s note: It is proposed to add the above text and to delete the editor’s note.}

# 6	H.248.1 defined stream grouping,aggregation and multiplexing

This clause describes the existing H.248.1 mechanisms related to stream grouping, aggregation and multiplexing.

## 6.1	Reserve Group

Within a single stream H.248.1 allows multiple sets of local and/or remote parameters to be specified. This is accomplished through the use of the ReserveGroup attribute (clause 7.1.7.1.2/[ITU-T H.248.1]). This allows a MGC to request that a MG reserve resources for functions described in each group. Media packets may be sent or received according to any of the groups although only a single alternative may be active at any given time. The Reserve Group concept does not describe any grouping semantic other than an implicit semantic indicating that the different groups may only be sent one at a time As such the Reserve Group concept does not lead to aggregated streams. Clause 8.6.3 describes several grouping semantics where the reserve group concept may be used.

## 6.2	Stream multiplexing using transport address information

{Contributor’s note: It is proposed to add text to clause 6.1 and 6.2 that in conjunction with the definitions and conventions should clarify the use of the terms as well as the relation of the existing H.248 procedures to these concepts. It is proposed to delete the note.}

Typically a single media stream is assigned to a single H.248 Stream endpoint by specifying a unique address/port combination in the Local Descriptor per H.248 Stream on a Termination along with the associated media properties. In general to indicate the use of a multiplexed media stream where multiple streams are sent using the same transport address, the MGC requests the same address/port in the Local Descriptor for all H.248 Streams that are to make up the multiplexed media stream.

As such when the same transport address/port is used for several streams there is an implicit grouping semantic which effectively aggregates several streams resulting in a multiplexed transport.

For text based H.248, SDP is used for each Local Descriptor. In order to request to the same address/port the following conditions must be must:

- The value of the c-line must be the same for each Local Descriptor

- The value of m-line [IETF RFC 4566] <port> field is the same for each Local Descriptor.

A MGC may fully specify the address/port information for multiple H.248 Streams in a single Termination in an Add.req command. However in many cases the MGC will wildcard “choose” this information, in order for the MG to provide the address/port. In the case where wildcarding is used the MGC can:

- request the address/port information for one of the H.248 Streams involved in the multiplex and then use the same address/port information in subsequent Mod.req/s for additional H.248 Streams, or

- request address/port information for all the H.248 Streams involved in the multiplex and then select the address/port from one of the H.248 Streams and then send a subsequent Modify.request to modify the address/port information. This has the disadvantage over option a) that some resources may have been allocated to the previous address/port and these allocations would have to be changed.

Figure 1 shows example syntax for a Termination (T1) utilising two RTP Streams and Termination (T2) utilising one RTP Stream for two H.248 Streams.

| Two H.248 Streams, Two RTP Streams | Two H.248 Streams, One RTP Stream |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10003 { Context = 1 { Add = T1 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2223 RTP/AVP 34 } } } } } } | MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T2 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2222 RTP/AVP 34 } } } } } } |

Figure 1 – Example single and multiplexed RTP streams

On reception of an Add, Modify or Move request command with several Local Descriptors from the same Termination containing the same address/port information the MG shall treat the associated H.248 Streams as a multiplexed media stream. Multiplexing is applied according to the type or profile of the stream. For example, in the case that a media profile RTP/AVP is given RTP multiplexing is applied. Each media stream component of the overall multiplexed RTP Stream is coded according to the information Local Descriptor / H.248 Stream for that particular media.

RTP multiplexing is described by [IETF RTPMUX].

{Contributor’s note: The correct reference would be http://tools.ietf.org/html/draft-ietf-avtcore-rtp-multi-stream-04. It is proposed to add this reference. An alternative would be to remove this reference as this is a general section.}

This multiplexed media stream may contain more than one media type.

Multiple multiplexed media streams are also possible on a Termination.

Individual components of the media multiplex can be modified by changing the Local and/or Remote Descriptors of the related H.248 Stream. If the address/port information of a H.248 Stream is changed to be different from the other components of the multiplexed media stream then the media defined by the H.248 Stream will longer be part of the multiplex and it will be its own media stream.

Clause 8.6.3 describes several grouping semantics where the use of transport address for multiplexing may be used.

# 7	Further issues with existing H.248 stream grouping mechanisms

This clause describes issues associated with the stream grouping described in clause 6.

…

Bibliography

[b-IETF RFC 4091]	IETF RFC 4091 (07/2005), The Alternative Network Address Types (ANAT) Semantics for the Session Description Protocol (SDP) Grouping Framework.

___________________