| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4624 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | H.248.STGROUP: Duplication Grouping Semantics |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This Contribution proposes updates to H.248.STGROUP to add the Duplication Grouping “DUP” Semantic.

2.	Discussion

Draft Recommendation H.248.STGROUP in clause 8.6.3 indicates the MG Behaviour with respect to the currently standardised group semantics. In addition to the semantics in the clause RFC7104 “Duplication Grouping Semantics in the Session Description Protocol” defines the DUP semantic that indicates that the MG should duplicate packets and send them in separate redundant streams. It is proposed to add a new clause 8.6.3.9 describing the use of DUP in H.248 controlled gateways.

3.	Proposal

It is proposed to accept the following changes. The changes are marked up against TD 181/WP1 from the Sapporo, July Q3/16 Rapporteurs meeting.

Draft new Recommendation ITU-T H.248.xx (ex H.248.STGROUP)

Gateway Control Protocol: 
H.248 Stream grouping and aggregation

…

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.7]	Recommendation ITU-T H.248.7 (2004), Gateway Control Protocol: Generic Announcement package.

[ITU-T H.248.47]	Recommendation ITU-T H.248.47 (2008), Gateway Control Protocol: Statistic conditional reporting package.

[ITU-T H.248.58]	Recommendation ITU-T H.248.58 (2008), Gateway Control Protocol: Packages for application level H.248 statistics.

[ITU-T H.248.61]	Recommendation ITU-T H.248.61 (2013), Gateway Control Protocol: Packages for network level ITU-T H.248 statistics.

[ITU-T H.248.65]	Recommendation ITU-T H.248.65 (2009), Gateway control protocol: Support of the resource reservation protocol.

[IETF RFC 3524]	IETF RFC 3524 (2003), Mapping of Media Streams to Resource Reservation Flows.

[IETF RFC 4566]	IETF RFC 4566 (2006), SDP: Session Description Protocol.

[IETF RFC 4588]	IETF RFC 4588 (2006), RTP Retransmission Payload Format.

[IETF RFC 5583]	IETF RFC 5583 (2009), Signaling Media Decoding Dependency in the Session Description Protocol (SDP).

[IETF RFC 5888]	IETF RFC 5888 (2010), The Session Description Protocol (SDP) Grouping Framework.

[IETF RFC 5956]	IETF RFC 5956 (2010), Forward Error Correction Grouping Semantics in the Session Description Protocol.

[IETF RFC 7104]	IETF RFC 7104 (2014), Duplication Grouping Semantics in the Session Description Protocol.

[IETF draft-ietf-mmusic-sdp-bundle-negotiation]	IETF draft-ietf-mmusic-sdp-bundle-negotiation (201X), Negotiating Media Multiplexing Using the Session Description Protocol (SDP).

{Editor’s note: Title has changed to change}

…

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| AMR | Adaptive Multi Rate |
| --- | --- |
| ANAT | Alternative Network Address Types |
| DDP | Decoding Dependency |
| DUP | Duplication |
| ID | Identity |
| FEC | Forward Error Correction |
| FEC-FR | Forward Error Correction Flow Repair |
| ID | Identity |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| LD | Local descriptor |
| LS | Lip Synchronization |
| MG | Media gateway |
| MGC | Media gateway controller |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NGN | Next generation network |
| PCM | Pulse Code Modulation |
| RD | Remote descriptor |
| RSVP | Resource Reservation Protocol |
| RTCP | RTP Control Protocol |
| RTP | Real Time Protocol |
| SDP | Session description protocol |
| SRF | Single Reservation Flow |
| SEP | (H.248) Stream endpoint |
| SEPP | (H.248) Stream endpoint pair |
| SIP | Session initiation protocol |
| UDP | User datagram protocol |
| WEBRTC | Real-Time Communication in WEB-browsers (as work item in W3C) |

…

### 8.6.3	MG Behaviour

The behaviour of the MG with respect to the grouping is largely dependent on the semantic associated with the grouping, e.g. the value of semantic in the mgroup/groupse properties.

The SDP parameter IANA registry (http://www.iana.org/assignments/sdp-parameters/) contains a list of defined SDP group semantics. In general the MG should adopt the behaviour defined by the RFC (or other standard) that defines the semantic taking into consideration H.248 protocol behaviour. The following sub-clauses discuss several semantics.

…

#### 8.6.3.8	Multiplexing Negotiation Using Session Description Protocol (BUNDLE)

The “BUNDLE” semantic is defined in [IETF draft-ietf-mmusic-sdp-bundle-negotiation]. It used to describe that two or more RTP streams use a single 5-tuple (i.e. multiplex multiple RTP streams onto a single 5-tuple). As [IETF draft-ietf-mmusic-sdp-bundle-negotiation] only describes the use of BUNDLE for IP transports it shall not be used with Streams using non-IP transports.

As described in clause 7 it is possible to request that a MG use a multiplexed RTP stream however there are a number of deficiencies.

Firstly the MGC is unaware if the MG supports RTP multiplexing. In order to determine if the MG supports RTP multiplexing the MGC may audit the media grouping package to determine if the “BUNDLE” semantic is supported (see clause 8.6.1).

Secondly there is no way to wildcard CHOOSE the address/port information from several H.248 Streams and request that a single transport connection (i.e. multiplexed RTP stream) be used. The media grouping package contains the “group semantics” property that, when including the BUNDLE semantic allows the MGC to indicate that two or more H.248 Streams are multiplexed together. MGs that support the media grouping package should check the “group semantics” property when processing wildcarding requests for multiple H.248 Streams. If the H.248 Streams match the association given by this property then the same value should be returned for those H.248 Streams.

##### 8.6.3.8.1	Directionality

BUNDLE is symmetric in nature. If it is used, it applies to source and destination.

In practice, however, the required behaviour from the MG specific to this semantic, is related to the Local Descriptor, i.e. to the received media. The use of the same address and port for multiple streams in the Remote Descriptor is usually not problematic in the MG, also when BUNDLE is not used.

Therefore the use of the direction parameter in the mgroup/groupse property is superfluous in case of the BUNDLE semantics.

### 8.6.3.9	Duplication (DUP)

### The “DUP” semantic is defined in [IETF RFC 7104]. It indicates that the MG should duplicate packets and send them in separate redundant streams. The RFC allows the “DUP” semantic to be used within a single m-line where multiple redundant streams use the same IP destination address. For an example see clause 4.1/[IETF RFC 7104]). For this usage a MGC can use the SDP to indicating the grouping rather than using the mgroup package.

### The RFC also allows the “DUP” semantic to be used across m-lines where the redundant streams have different destination addresses. For an example see clause 4.2/[IETF RFC 7104]. For this usage the MGC shall use the mgroup/groupse to assign the DUP semantic to the relevant streams. Where a gateway is required to interwork between a redundant and non-redundant media stream the MGC shall use an aggregation stream to aggregate/de-aggregate the main and redundant streams.

##### 8.6.3.8.1	Directionality

The “DUP” semantic implicitly describes sending behaviour. Thus the DUP semantic is associated with the sending direction. It is possible that the whilst a Termination sends redundant media streams that it receives a single stream in return. It is likely that these redundant stream would have a stream mode of send only so setting the DUP semantic with “both” direction would have little effect given the overriding stream mode. However if the main stream as set with the DUP semantic with a “both” direction it would be problematic as the stream mode would be send/receive. However there would be no redundant receive stream.

___________________