| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4628 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | H.TPS-AV: Living List Item Discussion |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This Contribution proposes updates to H.TPS-AV to address the living list items.

2.	Discussion

H.TPS-AV currently has several living list items that need to be addressed before the document is ready to be put forward for Consent. This Contribution discusses several living list items and proposes resolutions for these items. For a discussion of the items and the proposed changes see the Contributor’s notes below.

3.	Proposal

It is proposed to accept the following changes. Changes are marked up against TD 175r2/WP1 from the July 2014 Q.5/16 Rapporteurs’ meeting in Sapporo.

Living List Items:

- All parameters that correspond to IETF CLUE information elements need to be periodically checked for accuracy and updated as needed.

{Contributor’s note: Contributions have previously been received on this issue. No need for describing the number of displays was identified. It is proposed to remove this item. The removal of the item would not prevent people submitting contributions identifying additional parameters.}

{Contributor’s note: Clause 7.1.2.7.1 indicates in the references methods for signalling the Max MBPS Alternate methods for signalling are also mentioned in the “reference” clause in many other parameters. The signalling method is also mentioned in Appendix I on the parameter signalling analysis. Therefore the Contributors believe that this item has been addressed and can be removed.}

{Contributor’s note: The Data Model draft is now intended on being a standard track RFC and thus is a normative description of the CLUE XML. Therefore the references to the CLUE Data model can remain. The version number of the reference should be updated from v3 to v7. It is proposed to delete this living list item.}

{Contributor’s note: This has now been removed from the CLUE data.model. This living list item can be deleted.}

{Contributor’s note: These have now been removed from the CLUE data model. This living list item can be deleted.}

Draft new ITU-T H.TPS-AV

Audio/video parameters for telepresence systems

…

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.TPS-Reqs]	ITU-T Recommendation X.yyy (date), Definitions, requirements, and use cases for Telepresence Systems.

[F/H.TPS-Arch]	ITU-T Recommendation X.yyy (date), Telepresence System Architecture.

[ITU-R BT.1361]	ITU-R Recommendation BT.1361 (02/1998), Worldwide unified colorimetry and related characteristics of future television and imaging systems.

[ITU-R BT.2020]	ITU-R Recommendation BT.2020 (08/2012), Parameter values for ultra-high definition television systems for production and international programme exchange.

[ITU-R BT.601]		ITU-R Recommendation BT.601-7 (03/2011), Studio encoding parameters of digital television for standard 4:3 and wide screen 16:9 aspect ratios.

[ITU-R BT.709]	ITU-R Recommendation BT.709-5 (04/2002), Parameter values for the HDTV standards for production and international programme exchange.

[ITU-T G.122]	ITU-T Recommendation G.122 (03/1993), Influence of national systems on stability and talker echo in international connections.

[ITU-T H.241]	ITU-T Recommendation H.241 (02/2012), Extended video procedures and control signals for ITU-T H.300-series terminals.

[ITU-T H.245]	ITU-T Recommendation H.245 (05/2011), Control protocol for multimedia communication.

[ITU-T H.264]	ITU-T Recommendation H.264 (01/2012), Advanced video coding for generic audiovisual services.

[ITU-T H.323]	ITU-T Recommendation H.323 (12/2009), Packet-based multimedia communications systems.

[ITU-T P.56]	ITU-T Recommendation P.56 (12/2011), Objective measurement of active speech level.

[ITU-T P.300]	ITU-T Recommendation P.300 (11/2001), Transmission performance of group audio terminals (GATs).

[ITU-T P.501]	ITU-T Recommendation P.501 (01/2012), Test signals for use in telephonometry.

[ITU-T Q.931]	ITU-T Recommendation Q.931 (05/1998), ISDN user-network interface layer 3 specification for basic call control.

[IETF CLUE DM]		IETF draft-ietf-clue-data-model-schema-07, An XML schema for the CLUE Data Model.

[IETF CLUE FW]		IETF draft-ietf-clue-framework-15, Framework for Telepresence Multi-Streams.

…

7.1.1 Captured Media Type

| Identity: | CapturedMediaType |
| --- | --- |
| Description: | This parameter is a mandatory field specifying the media type of the capture ("audio", "video", "text",...). |
| Format: | Enumeration. |
| Possible Values: | Audio, Video, Text… |
| Default: | - |
| Reference: | See the 'CapturedMedia' attributes in [IETF CLUE DM] for further information. |
| Interface: | - |
| Supplementary Information: | The possible values of this parameter are TBD. |

…

7.1.1.12.2 Area Scale

| Identity: | AreaScale |
| --- | --- |
| Description: | This parameter indicates the type of scale that is being applied to the spatial information parameters |
| Format: | Enumeration |
| Possible Values: | "mm" The metric system with the millimetre unit is being applied. "unknown" An unknown scale is being used that is the same for all captures in the scene. This may be used to indicate relative distances. "No scale" This indicates that the scale may be different for each capture in a scene. |
| Default: | None. An endpoint must indicate what scale it uses using one of the above values. {Contributor’s note: The use of “scale” is indicated in both [IETF CLUE FW] and [IETF CLUE DM]. It is proposed to capture the mandatory nature of the parameter as part of the default text. It is proposed to remove the editor’s note.} |
| Reference: | See "Area Scale" in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | - |

…

### I.1.1	General Parameters

| Parameter | Need for signalling at session initiation | Need for signalling during session | Remarks |
| --- | --- | --- | --- |
| Captured Media Type | Y | Y | See the ' CapturedMedia ' attributes in [IETF CLUE DM]. |
| Scene Description | Y | Y | See the “Description" attribute in 7.2.1 of [IETF CLUE FW]. |
| Capture Set Description | Y | Y | See the “Description" attribute in 7.2.2 of [IETF CLUE FW]. |
| Language | Y | N | See "Language" in [IETF CLUE FW]. |
| Priority | Y | Y | See "Priority" in [IETF CLUE FW]. |
| Embedded text | Y | Y | See "Boolean" in [IETF CLUE FW]. |
| Related to | Y | Y | See "Related to" in [IETF CLUE FW]. |
| Presentation | Y | Y | See "Presentation" in [IETF CLUE FW] |
| Person Information | Y | Y | As per clause 7.1.1.11/[IETF CLUE FW]. |
| Person Type | Y | Y | As per clause 7.1.1.12/[IETF CLUE FW]. |
| Scene Information | Y | Y | As per clause 7.3.1.1/[IETF CLUE FW]. |
| Capture description | Y | Y | See the “Description" attribute in 7.1.1 of [IETF CLUE FW]. |
| Area scale | Y | N | See "Area Scale" in [IETF CLUE FW]. |
| Mobility of capture | Y | N | See "Mobility of capture" in [IETF CLUE FW]. |
| View | Y | Y | See "View" in [IETF CLUE FW]. |
| Overall maximum bit-rate | Y | N | See "maxGroupBandwidth" in [IETF CLUE FW]. |
| Max capture encodings | Y | N | This parameter aligns with the "Max Capture Encodings" attribute in [IETF CLUE FW]. |

____________________