| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4631 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co. Ltd. |  |  |  |
| Title: | H.TPS-AV: Signalling audio level |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution addresses an Editor’s note in Table I.1.3 of draft Rec. H.TPS-AV concerning signalling audio level, and proposes revised text.

# Introduction

In Table I.1.3 of draft Rec. H.TPS-AV there is an Editor’s note in the Remarks column of the row “Audio Level” which reads:

{Editors’s note: Is there a need to send an “audio level” parameter at session initiation? No audio level in H.245.  Audio level may be signalled via RTP using IETF RFC 6464.}

This contribution discusses the issues related to audio level and whether and how it should be signalled, and proposes a response to address this Editor’s note.

# Discussion

Thinking about audio level a little deeper, it seems that what we have really been talking about up to now in H.TPS-AV is the nominal audio level (-26 dBov). In principle there is no need to signal the nominal audio level since it is determined by the sending loudness rating parameter. Essentially, a sending-side having a default sending loudness rating of  (+ 13–Fs) will produce an audio level of approximately -26 dBov (assuming the use of a codec having a +3 dBm0 overload point). Signalling of nominal audio level has never been done in conventional telephony or audio conferencing, and as noted in the Editor’s note, it is not specified in Rec. H.245. Given these considerations, it is proposed that audio level in clause 7.1.3.4 is renamed as nominal audio level, but is not signalled.

At the same time, following the rationale of RFC6464 “A Real-time Transport Protocol (RTP) Header Extension for Client-to-Mixer Audio Level Indication”, there may be a need to signal the actual audio level of an audio stream as it varies as a function of time, to obviate the need for an MCU or end-point to have to decode the audio signal to perform this function, which could save some computing power in the case of a large number of audio streams. This can be defined as a new parameter “dynamic audio level”.

This new parameter can be signalled using RFC6464, although it should be noted that RFC6464  is not a complete solution for signalling audio level since it is clearly stated in section 3 that the audio level header extension only carries the level of the audio in the RTP payload of the packet with which it is associated, with no long-term averaging or smoothing applied. This is not enough for characterising the level of speech, which requires some form of syllabic averaging, for example following ITU-T Rec. P.56. To be consistent with RFC6464, possible values of dynamic audio level are from -127 to 0 dBov, although the practical range will be from about -86 to -10. It is noted that RFC6464 reserves the value of -127 to indicate digital silence.

# Conclusion

It is proposed to accept and incorporate the revised text below. Changes are marked against the output version of H.TPS-AV from the July 2014 Q5/16 meeting in Sapporo (TD 175 R2 (WP 1/16)).

Begin proposed changes

__________________________________________________

#### 7.1.3.4	Nominal Audio Level

| Identity: | NominalAudioLevel |
| --- | --- |
| Description: | This parameter indicates the nominal audio level sent in the Telepresence audio stream. |
| Format: | Numeric, in dBov |
| Possible Values: | -26 |
| Default: | -26 |
| Reference: | - |
| Interface: | - |

Telepresence systems shall send audio to other telepresence systems with anominal audio level of NominalAudioLevel dBov as measured by method B of [ITU-T P.56], which may be implemented using the tool sv-p56.c in the Software Tools Library [ITU-T G.191].

When a telepresence system connects to non-telepresence devices, it should use the NominalAudioLevel expected by that device.  If this is unknown, it should use the Telepresence NominalAudioLevel.

#### 7.1.3.5	Dynamic Audio Level

| Identity: | DynamicAudioLevel |
| --- | --- |
| Description: | This parameter indicates the actual audio level sent in the Telepresence audio stream as it varies as a function of time. |
| Format: | Numeric, in dBov |
| Possible Values: | -127 to 0 |
| Default: | None |
| Reference: | - |
| Interface: | - |

Dynamic audio level shall be measured using method B of  [ITU-T P.56], which may be implemented using the tool sv-p56.c in the Software Tools Library [ITU-T G.191].

7.1.3.6	Sending Loudness rating

……………………………………

7.1.3.7	Weighted Microphone Loudspeaker Coupling Loss

……………………………………

Appendix I

Parameter signalling analysis

…………………………………

### I.1.3	Audio parameters

………………………………….

| Nominal Audio Level | N | N | Not signalled. |
| --- | --- | --- | --- |
| Dynamic Audio Level | Y | Y | May be signalled via RTP using RFC6464. |

__________________________________________________

End proposed changes

__________________