| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4634 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.SCTP updates |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes a number of changes in the H.248.SCTP draft.

# Discussion

1. During the last SG16 Q3 meeting in Sapporo it was agreed to avoid the use of the term “control plane” to refer to the vertical interface between MGC and MG. The text in H.248.TCP, H.248.TLS and H.248.DTLS was modified accordingly, but there was one occurrence left in H.248.SCTP.

The following proposal aligns the text in H.248.SCTP with the text in the other consented Recommendations.

2. The draft states that this Recommendation is not dependent on any SCTP transport mode and that it focuses on native SCTP/IP.

At the moment however the uses cases that are contemplated in this Recommendation use the DTLS/UDP/IP stack. The use of a particular stack can have an impact on the way SCTP is used. For example, DTLS transport will preclude the use of multi-homing. The use of the DCEP protocol may impose also some requirements on the H.248 solution or benefit from specific features from such solution.

It is proposed to modify the indicated statements as it is not clear at this stage that the H.248 solution in this Recommendation will cover all transport modes.

3. The description of the interworking scenarios repeatedly uses the expression “originating/terminating the DTLS/UDP tunnel in tunnelled SCTP via DTLS/UDP” which is hard to read. The wording can be simplified. Likewise for “DTLS-based transport security in transport secured SCTP”.

4. A list of interworking scenarios was introduced in the last SG16 meeting, considering the different transport protocol stacks at both sides of the MG. It was acknowledged that some classification of the use cases would be desirable.

This contribution proposes three main groups:

- SCTP to non-SCTP interworking scenarios

- SCTP to SCTP interworking scenarios with change of protocol transport

- SCTP to SCTP interworking scenarios without change of protocol transport

Scenarios belonging to the first two groups involve SCTP protocol termination at the H.248 SEP (with the possible exception of SCTP/DTLS/UDP/IP to SCTP/IP case). Scenarios belonging to the third group may correspond to transparent forwarding of messages within an end-to-end SCTP association, DTLS session, or SCTP association tunnelled in DTLS/UDP or they may involve some interworking function at some layer (SCTP, DTLS, or SCTP user part) without excluding SCTP protocol termination.

Furthermore, it is proposed to refer to all these cases as “interworking scenarios” and reserve the name “use cases” for the different applications using the SCTP bearers which are considered in this Recommendation (CLUE and WebRTC).

5. The #2.1.2 interworking scenario, DTLS over SCTP to DTLS* over SCTP interworking (i.e. two different DTLS security domains) requires necessarily certain level of application awareness in the MG processing with DTLS being the application from SCTP point of view, i.e. the SCTP user part layer. Thus it does not seem correct to say that there is a variation of this scenario which is application aware.

Likewise, the #1.1.2 scenario, an end-to-end SCTP association over IP layer, represents primarily a case of  transparent forwarding of SCTP messages through the MG. Therefore it does not seem correct to present the clause 6.2.2.1 “SCTP-to-SCTP transparent forwarding” as a variation of the #1.1.2 scenario.

6. In Appendix II there is a note stating that it is expected that only a primary path will exist in case of SCTP over DTLS. This will necessarily be the case since the DTLS transport does not allow multi-homing. Therefore a firm statement can be given.

# Conclusion

In order to address the discussion points described in clause 2, the following changes are proposed.

[Begin Correction]

## 5.3	SCTP bearer

The notation of SCTP bearer connection relates to a SCTP association in the network user plane. There might be also an SCTP association between MGC and MG in case of a SCTP-based H.248 transport mode [ITU-T H.248.67] which is out of the scope of this Recommendation.

{Editor's note (2014-03): it is yet open whether we need to consider
a) "unmultiplexed SCTP bearers" only (i.e., a 1:1 relation of SCTP Stream to SCTP association from H.248 perspective), or/and
b) "multiplexed SCTP bearers" (i.e., there might be following model: an H.248 "multi-stream-to-termination structure" with "H.248 termination = SCTP association" and "H.248 SEP = SCTP Stream" and multiple SCTP Streams).
Such aspects may impact above convention on "SCTP bearer". }

[End Correction]

[Begin Correction]

## 6.1	Use cases related to SCTP transport modes

The SCTP protocol could be used in various transport protocol stacks  ortransport modes (Figure 3).

Figure 3 – SCTP transport modes

This Recommendation focuses on tunneled SCTP via DTLS/UDP.

Contributor’s note: the use cases covered by this Recommendation at this stage use the SCTP/DTLS/UDP transport. If other cases are identified which use other transports, the statement above can be modified.

[End Correction]

[Begin Correction]

### 6.2.1	High-level interworking scenarios categories

The notion "IP-to-IP" (briefly "IP-IP") indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])).

Each H.248 stream endpoint (SEP) may be associated with a non-SCTP or different SCTP protocol stack variants, leading to various bearer network connection scenarios (see Figure 4):

- #1.1: an H.248 IP-IP MG located in the middle of an end-to-end SCTP bearer connection, there are two sub-variants:

  - #1.1.1: the end-to-end SCTP bearer connection is divided in two separate SCTP associations ("SCTP proxy mode");

  - #1.1.2: the end-to-end SCTP bearer connection relates to a single SCTP association ("SCTP relay mode");

- Editor’s note: a classification of all use cases is desired.

- #1.2: an H.248 IP-IP MG terminating/originating the DTLS-based secured transport  over SCTP (stack "DTLS/SCTP/IP") and interworking to native SCTP;

- #1.3: an H.248 IP-IP MG terminating/originating the "DTLS/UDP" tunnel with encapsulated SCTP (stack "SCTP/DTLS/UDP/IP") and interworking to native SCTP;

- #1.4: an H.248 IP-IP MG terminating/originating the  native SCTP transport (stack SCTP/IP) and interworking with other, non-SCTP based IP transport protocols;

- #2.1: an H.248 IP-IP MG located in the middle of an end-to-end transport secured DTLS/SCTP bearer connection, there are two sub-variants:

  - #2.1.1: transparent forwarding of DTLS/SCTP packets;

  - #2.1.2: DTLS/SCTP to DTLS*/SCTP forwarding between different security domains (i.e., there are two DTLS sessions with not necessarily equal security settings);

- #2.2: an H.248 IP-IP MG terminating/originating the "DTLS/UDP" tunnel with encapsulated SCTP (stack "SCTP/DTLS/UDP/IP") at one side and terminating/originating DTLS-based secured transport over SCTP (stack "DTLS/SCTP/IP") at the other side and interworking between both;

- #2.3: an H.248 IP-IP MG terminating/originating DTLS-based secured transport  over SCTP (stack "DTLS/SCTP/IP") and interworking to other, non-SCTP based IP transport protocols;

- #3.1: an H.248 IP-IP MG located in the middle of an end-to-end tunneled SCTP via DTLS/UDP (stack "SCTP/DTLS/UDP/IP") bearer connection;

NOTE – E.g., following scenario given by "MSRP over WEBRTC data channels (see [b-IETF msrp-over-webrtc], [b-IETF data-channel-sdpneg])" and a DTLS encapsulation of SCTP based on [b-IETF sctp-dtls-encaps].

- #3.2: an H.248 IP-IP MG terminating/originating the "DTLS/UDP" tunnel with encapsulated SCTP (stack "SCTP/DTLS/UDP/IP") and interworking to other, non-SCTP based IP transport protocols;

Figure 4 – Bearer connection network use cases with H.248 IP-IP gateways

Application protocol perspective:

The application protocol relates to the upper layer (user part) protocol(s) on top of the SCTP transport. Most of above scenarios are basically of type "application-agnostic", but there might be also scenarios with "application-aware" type of H.248 signalling (see also clause 6.2.2.2).

[End Correction]

For clarity reasons, the classification of interworking scenarios is shown as a separate change proposal to the terminology and wording changes from above proposal in spite of being related to the same clause (it would be difficult for the reader to identify the terminology/wording changes otherwise).

The proposed text below reproduces all changes from above and proposes a classification of the use cases.

[Begin Correction]

### 6.2.1	High-level interworking scenarios categories

The notion "IP-to-IP" (briefly "IP-IP") indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])).

Each H.248 stream endpoint (SEP) may be associated with a non-SCTP or different SCTP protocol stack variants, leading to various bearer network connection scenarios (see Figure 4):

SCTP to non-SCTP interworking:

- #1.1: an H.248 IP-IP MG terminating/originating the  native SCTP transport (stack SCTP/IP) and interworking with other, non-SCTP based IP transport protocols;

- #1.2: an H.248 IP-IP MG terminating/originating DTLS-based secured transport  over SCTP (stack "DTLS/SCTP/IP") and interworking to other, non-SCTP based IP transport protocols;

- #1.3: an H.248 IP-IP MG terminating/originating the "DTLS/UDP" tunnel with encapsulated SCTP (stack "SCTP/DTLS/UDP/IP") and interworking to other, non-SCTP based IP transport protocols;

The three cases above require realization of an SCTP endpoint, i.e. full protocol termination.

SCTP to SCTP interworking with transport mode change:

- #2.1: an H.248 IP-IP MG terminating/originating the DTLS-based secured transport  over SCTP (stack "DTLS/SCTP/IP") and interworking to native SCTP;

- #2.2: an H.248 IP-IP MG terminating/originating the "DTLS/UDP" tunnel with encapsulated SCTP (stack "SCTP/DTLS/UDP/IP") and interworking to native SCTP;

- #2.3: an H.248 IP-IP MG terminating/originating the "DTLS/UDP" tunnel with encapsulated SCTP (stack "SCTP/DTLS/UDP/IP") at one side and terminating/originating DTLS-based secured transport over SCTP (stack "DTLS/SCTP/IP") at the other side and interworking between both;

The change of transport mode in the cases above does not per se require a realization of an SCTP endpoint, i.e. termination of the SCTP protocol. However, depending on the type of interworking through the context at the user part layer or upper layers, an SCTP end point realization may be required. This is the case at least in 2.1 and 2.3. In these cases, SCTP user messages can not be transparently forwarded and separate flow control at each end point is pertinent.

SCTP to SCTP with no transport mode change:

- #3.1: an H.248 IP-IP MG located in the middle of an end-to-end SCTP bearer connection, there are two sub-variants:

  - #3.1.1: the end-to-end SCTP bearer connection is divided in two separate SCTP associations ("SCTP proxy mode");

  - #3.1.2: the end-to-end SCTP bearer connection relates to a single SCTP association ("SCTP relay mode");

- #3.2: an H.248 IP-IP MG located in the middle of an end-to-end transport secured DTLS/SCTP bearer connection, there are two sub-variants:

  - #3.2.1: transparent forwarding of DTLS/SCTP packets;

  - #3.2.2: DTLS/SCTP to DTLS*/SCTP forwarding between different security domains (i.e., there are two DTLS sessions with not necessarily equal security settings);

- #3.3: an H.248 IP-IP MG located in the middle of an end-to-end tunneled SCTP via DTLS/UDP (stack "SCTP/DTLS/UDP/IP") bearer connection;

NOTE – E.g., following interworking scenario given by "MSRP over WEBRTC data channels (see [b-IETF msrp-over-webrtc], [b-IETF data-channel-sdpneg])" and a DTLS encapsulation of SCTP based on [b-IETF sctp-dtls-encaps].

Case 3.1.1 implies realization of SCTP end points at both sides of the context. Case 3.2.2 will also normally require realization of an SCTP end point. The rest of the cases under this header may represent transparent forwarding of SCTP messages through the MG and thus not require any SCTP related H.248 control procedures.

Figure 4 – Bearer connection network scenarios with H.248 IP-IP gateways

Contributor’s note: numbering of interworking scenarios in figure should be modified according to new classification.

Application protocol perspective:

The application protocol relates to the upper layer protocol(s) on top of the SCTP transport (see Figure 3). Most of above scenarios are basically of type "application-agnostic", but there might be also scenarios with "application-aware" type of H.248 signalling (see also clause 6.2.2.2).

[End Correction]

[Begin Correction]

Contributor’s note: Former #1.1.2 (now #3.1.2) implies primarily transparent forwarding of SCTP messages. If some case with non-transparent behaviour is identified, this should rather be presented as the variation case, not the other way.

[End Correction]

[Begin Correction]

#### 6.2.2.2	"SCTP-to-SCTP application aware processing"

The #3.2.2 scenario implies application awareness since the DTLS, being the SCTP user part, requires bearer control procedures. There exist variations of use case #3.1.1 and #3.3where  the MG may also be required to be application aware in case of additional operations on the upper protocol layer(s) which imply knowledge about the carried application. Such MG operations might be related to the detection of an event, the generation of statistics, the modification of protocol data units, etc .

[End Correction]

[Begin Correction]

## 6.3	Bearer connection network use cases with H.248 "SCTP to non-SCTP" gateways

Relates to scenarios #1.1, #1.2 and #1.3.

Example: WebRTC data channel interworking as illustrated in Appendix I.3/[ITU-T H.248.93]; all that examples relate to use case #1.3.

[End Correction]

[Begin Correction]

- 6.4	Supported use cases by this Recommendation

The first release of this Recommendation provides H.248 support for use cases related to following communication services in network environments with H.248 gateway involvement (Table 2):

Table 2 – Supported use cases by this Recommendation

| Communication service | H.248 MG type | Bearer connection network use case(s) |
| --- | --- | --- |
| WEBRTC (Web-based Real-Time Communication) stack component "data channel" | H.248 WebRTC gateway (see [b-ITU-T H.248.WEBRTC]) | Scenarios: #2.2 and #1.3 |
| Telepresence stack component "data channel" | H.248 media server | Scenarios: To be completed |

[End Correction]

[Begin Correction]

### 7.2.2	Model for a context with two SCTP terminations

Figure 7 represents the general H.248 (SCTP, SCTP) connection model, i.e., a two-termination-context with both terminations enabled for SCTP. The type of interworking function (IWF) determines the particular MG behaviour concerning SCTP handling.

Figure 7 – Two-termination context with context-level SCTP scope

[End Correction]

[Begin Correction]

### II.1.1	Network model for interworking to non-SCTP based data transport

Figure II.1 illustrates a "half call" model with focus on an H.248 gateway and a peered user equipment (UE). An H.248 IP-IP gateway is located between access and core network level. The data channel (DC) uses the protocol stack segment SCTP/DTLS/UDP/IP as transport. The data channel is terminated in the H.248 MG (see Figure II.1), which implies the termination of protocols 'DTLS' and 'SCTP'.

Figure II.1 – Network model for H.248 IP access gateways (for half call only) and user plane stacks

Editor’s note: The SCTP association, SCTP stream and DC channel boxes in the figure above should be depicted in a different way not to give the impression they are different protocol layers. Maybe tt could be three boxes at the same level, or two boxes inside an SCTP association box.

Both protocols, DTLS and SCTP, are of type connection-oriented, hence there are correspondent bearer control protocol (BCP) procedures for the establishment of firstly a DTLS session and then subsequently a SCTP association.

There is only a primary SCTP path used, thus, not any secondary SCTP path prepared due to the fact of the underlying DTLS/UDP tunnel which does not allow multi-homing.

There will be initial application control signalling (such as SIP based call control signalling), which triggers the MGC to request DTLS and SCTP bearer establishment from the MG (see next sub-clauses).

[End Correction]

__________________